3GPP TSG-RAN5 Meeting #2024-TTCN email	R5s240149

# Online, , 8th Dec 2023 - 31st Dec 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 3367 | rev | - | Current version: | 17.9.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to NR RRC test case 8.1.1.2.3 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies UK Ltd |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2024-02-06 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In the current TTCN implementation of TC 8.1.1.2.3, the SS transmits Close UE Test Loop Mode B message twice in preamble, first time with IP PDU Delay is set to 0s, and second time it is set to 5s. Based on the implemention of loopback mode, some UEs may ignore the second Close UE test loop message as and as a result the IP PDU Delay will be set to 0s, whereas the testcase requirement is 5s. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The function call f_NR5GC_Preamble(v_NGC_NASCellA, STATE_IDLE_1A, TEST_LOOPModeB_ON) isreplaced with f_NR5GC_Preamble(v_NGC_NASCellA, STATE_IDLE_1A, TESTModeB_ON). |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may fail the test case |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.1.1.2.3.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Table of Contents

Table of Contents	2

1	Overview	3

2	Corrections required.	3

2.1	Change 1	3

- Overview

This document lists all the changes needed to correct issues in the ATS iwd-TTCN3-B2022-09_D23wk49 related to the title of this CR.

Contact:	Mohit Kanchan
	mohit.kanchan@keysight.com

- Corrections required.

  - Change 1

| Function name | f_TC_8_1_1_2_3_NR5GC |
| --- | --- |
| Reason for change | In the current TTCN implementation of TC 8.1.1.2.3, the SS transmits Close UE Test Loop Mode B message twice in preamble, first time with IP PDU Delay is set to 0s, and second time it is set to 5s. Based on the implemention of loopback mode, some UEs may ignore the second Close UE test loop message as and as a result the IP PDU Delay will be set to 0s, whereas the testcase requirement is 5s. |
| Summary of change | The function call f_NR5GC_Preamble(v_NGC_NASCellA, STATE_IDLE_1A, TEST_LOOPModeB_ON) isreplaced with f_NR5GC_Preamble(v_NGC_NASCellA, STATE_IDLE_1A, TESTModeB_ON). |
| TTCN module | RRC_ConnEst_NR5GC.ttcn |
| MCC160 Comment | Accepted |

Before Change:

function f_TC_8_1_1_2_3_NR5GC() runs on NR5GC_PTC { // RRC connection establishment / RRC Reject with wait time var Octet_Type v_IP_PDU_delay := '05'O; f_NR5GC_Init(NR_1); //Create and configure cell f_NR_CellConfig_Def(nr_Cell1); f_NR5GC_Preamble(nr_Cell1, STATE_IDLE_1A, TEST_LOOPModeB_ON); f_NR5GC_RRC_ConnectedState3N_Def(nr_Cell1, TEST_LOOPModeB_ON, -, v_IP_PDU_delay); f_NR_TestBody_Set(true); fl_TC_8_1_1_2_3_TestBody(); f_NR_TestBody_Set(false); f_NR_Postamble(nr_Cell1, STATE_IDLE_1A); // @sic R5s221102 sic@ }

After Change:

function f_TC_8_1_1_2_3_NR5GC() runs on NR5GC_PTC { // RRC connection establishment / RRC Reject with wait time var Octet_Type v_IP_PDU_delay := '05'O; f_NR5GC_Init(NR_1); //Create and configure cell f_NR_CellConfig_Def(nr_Cell1); f_NR5GC_Preamble(nr_Cell1, STATE_IDLE_1A, TESTModeB_ON); f_NR5GC_RRC_ConnectedState3N_Def(nr_Cell1, TEST_LOOPModeB_ON, -, v_IP_PDU_delay); f_NR_TestBody_Set(true); fl_TC_8_1_1_2_3_TestBody(); f_NR_TestBody_Set(false); f_NR_Postamble(nr_Cell1, STATE_IDLE_1A); // @sic R5s221102 sic@ }