3GPP TSG-RAN5 Meeting #2024-TTCN email	R5s240158

# Online, , 8th Dec 2023 - 31st Dec 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 3374 | rev | - | Current version: | 17.9.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | NR5GC FR1 : Addition of Rel-17 eNS test case 10.1.8.4 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ANRITSU LTD |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI17_Test, eNS_Ph2-UEConTest |  |  |  |  |  | Date: |  |  | 2024-02-07 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | To add test case 10.1.8.4 to the 5G/NR5GC ATS for FR1 path. |  |  |  |  |  |  |  |  |
| Summary of change: |  | This document lists all changes applied to test case 10.1.8.4 required for approval. See detailed change description for further information |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test case will not be added to ATS |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 10.1.8.4.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR… |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

1.1	Verification Test Summary	3

2	Corrections required	3

2.1	Change 1	3

2.2	Change 2	5

3	Branches executed	13

4	Execution Log Files	13

4.1	Samsung Shannon S5E9945ERD	13

5	References	13

# Overview

This document lists all the essential changes needed to correct problems in the TTCN implementation of test case 10.1.8.4 which is part of the 5G NR5GC test suite.

With these changes applied the test case can be demonstrated to run with one or more UEs (see section 4). Execution log files are provided as evidence.

Contact:	Kalahasti, Narendra

Narendra.Kalahasti@anritsu.com

## Verification Test Summary

Test Case: 	10.1.8.4

ATS Version:	iwd-TTCN3-B2022-09_D23wk49

System Simulator used:	Anritsu Protocol Conformance Test System ME7834NR

UE used:	Samsung Shannon S5E9945ERD

Verification Status:	PASS

# Corrections required

## Change 1

| Function name | function f_TC_10_1_8_4_NR5GC () |
| --- | --- |
| Reason for change | After finishing step16, UE is in connected mode, So postamble should be updated accordingly on connected state. |
| Summary of change | Modified postamble step with STATE_CONNECTED_3A. |
| TTCN module | GSM_NR5GC |
| MCC160 Comment | Accepted |

Before Change

function f_TC_10_1_8_4_NR5GC() runs on NR5GC_PTC {// @sic R5-233401 sic@ //NSAC / 5GSM message not forwarded / Back-off timer var NR_CellId_Type v_NGC_NASCellA := f_NR5GC_MapNASCell(ngc_CellA, NG_AllCellsOnSamePLMN); f_NR5GC_Init_NAS(NR_1); //Create and configure cells f_NR_CellConfig_Def(v_NGC_NASCellA); //Preamble f_NR5GC_Preamble (v_NGC_NASCellA, STATE_CONNECTED_3A); f_NR_TestBody_Set(true); //Main test body fl_TC_10_1_8_4_Body(); f_NR_TestBody_Set(false); // Finish f_NR_Postamble(v_NGC_NASCellA, STATE_IDLE_1A); }

After Change

function f_TC_10_1_8_4_NR5GC() runs on NR5GC_PTC {// @sic R5-233401 sic@ //NSAC / 5GSM message not forwarded / Back-off timer var NR_CellId_Type v_NGC_NASCellA := f_NR5GC_MapNASCell(ngc_CellA, NG_AllCellsOnSamePLMN); f_NR5GC_Init_NAS(NR_1); //Create and configure cells f_NR_CellConfig_Def(v_NGC_NASCellA); //Preamble f_NR5GC_Preamble (v_NGC_NASCellA, STATE_CONNECTED_3A); f_NR_TestBody_Set(true); //Main test body fl_TC_10_1_8_4_Body(); f_NR_TestBody_Set(false); // Finish f_NR_Postamble(v_NGC_NASCellA, STATE_CONNECTED_3A); }

## Change 2

| Function name | function fl_TC_10_1_8_4_Body () |
| --- | --- |
| Reason for change | - According to 24.501, Table 8.2.11.1.1: DL NAS TRANSPORT message content, IEI should be set to ‘58’O when there is some value for GMM cause. - AT step 10 PDU session going to be released was established at step 8, So need to fetch second PDU_Session_ID from v_PDUSessionInfoList. |
| Summary of change | - Updated GMM cause IEI with ‘58’O. - Modified v_PDUSessionInfoList[1].SessionId with second session. |
| TTCN module | GSM_NR5GC |
| MCC160 Comment | - Accepted - Accepted conditional to prose CR agreement at RAN5#102 Additional change: v_PDU_SessionId value is assigned at step 10 and will be used to release PDU at step 16, latest PDU established. This change is already reflected in prose CR submitted along with this CR document. |

Before Change

function fl_TC_10_1_8_4_Body() runs on NR5GC_PTC { var NR_CellId_Type v_NGC_NASCellA := f_NR5GC_MapNASCell(ngc_CellA, NG_AllCellsOnSamePLMN); var GSM_MobilityInfo_Type v_GSM_MobilityInfo; var NR_RadioBearerId_Type v_SRB := tsc_NR_RbId_SRB2; var charstring v_NSSAI := f_ConvertS_NSSAI_ForATCommand (cs_S_NSSAI_SST1eMBB); var template (value) GPRS_Timer3 v_BackOffTimer := cs_GPRS_Timer2_3_IEI('37'O, '101'B, '00101'B);//5 mins var template (value) PayloadContainerType v_PayloadContainerType := cs_PayloadContainerType ('0001'B); // @sic R5-236457 sic@ var template (value) GMM_GSM_Cause v_GMM_Cause := cs_GMM_GSM_Cause (omit, '01000101'B); // @sic R5-236457 sic@ var template (value) NG_NAS_DL_Pdu_Type v_PduToSend; var octetstring v_EncodedNasPdu; var O1_Type v_PDU_SessionId; // @sic R5-236457 sic@ var PDUSessionInfoList_Type v_PDUSessionInfoList; var NR_SRB_COMMON_IND v_ReceivedAsp; timer t_Wait; timer t_T3585Max := f_NR_SetTimerToleranceMax(v_NGC_NASCellA, nasTimer, 300.0); //@siclog "Steps 1" siclog@ //Cause the UE to request establishment of PDU session with S_NSSAI. f_UT_RequestPDUSession(UT, f_NR5GC_MobileInfo_GetNoOfSessions()+1, -, -, -, v_NSSAI); //@siclog "Steps 2" siclog@ SRB.receive(car_NR_SRB_NasPdu_IND(v_NGC_NASCellA, v_SRB, cr_NG_NAS_IndWithPiggybacking(tsc_SHT_IntegrityProtected_Ciphered))) -> value v_ReceivedAsp; if (not f_Check_NG_PDUSessionEstablishmentReq(v_GSM_MobilityInfo, v_ReceivedAsp.Signalling.Nas[0].Pdu, ?,?,-,-, -,-,-,-, -,-,-,-, cs_S_NSSAI_SST1eMBB_WithIEI, //S-NSSAI specific ? )) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "PDU Session Establishment Message Failed"); } //@siclog "Step 3" siclog@ // @sic R5-236457 sic@ v_PDU_SessionId := v_ReceivedAsp.Signalling.Nas[0].Pdu.PiggybackedPduList[0].Msg.pdu_Session_Establishment_Request.pduSessionId.sessionId; //PDU SESSION ESTABLISHMENT REQUEST message received in Step 2 is included in the payload container v_EncodedNasPdu := bit2oct(encvalue(v_ReceivedAsp.Signalling.Nas[0].Pdu.PiggybackedPduList[0].Msg.pdu_Session_Establishment_Request)); v_PduToSend := f_Get_NG_DLNASTransport (v_PayloadContainerType, omit, cs_PayloadContainer(omit, v_EncodedNasPdu), v_PDU_SessionId, -, v_GMM_Cause, v_BackOffTimer, omit); SRB.send(cas_NR_SRB_NasPdu_REQ(v_NGC_NASCellA, v_SRB, -, cs_NG_NAS_Request(tsc_SHT_IntegrityProtected_Ciphered, v_PduToSend.Msg))); //@siclog "Step 4" siclog@ t_T3585Max.start; t_Wait.start(60.0); alt{ [] SRB.receive(car_NR_SRB_NasPdu_IND(v_NGC_NASCellA, v_SRB, cr_NG_NAS_IndWithPiggybacking(tsc_SHT_IntegrityProtected_Ciphered))) -> value v_ReceivedAsp { t_Wait.stop; if (not f_Check_NG_PDUSessionEstablishmentReq(v_GSM_MobilityInfo, v_ReceivedAsp.Signalling.Nas[0].Pdu)) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Received PDU Session Establishment REQ"); } } []t_Wait.timeout { f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 4"); } } //@siclog "Step 5" siclog@ //Cause the UE to request establishment of PDU session with same S-NSSAI which was used in Step 1. f_UT_RequestPDUSession(UT, f_NR5GC_MobileInfo_GetNoOfSessions()+1, -, -, -, v_NSSAI, -, -, -, -, -, -, LOCAL_CNF_REQUIRED); // @sic R5s230789 sic@ //@siclog "Step 6" siclog@ t_Wait.start(15.0); alt{ [] SRB.receive(car_NR_SRB_NasPdu_IND(v_NGC_NASCellA, v_SRB, cr_NG_NAS_IndWithPiggybacking(tsc_SHT_IntegrityProtected_Ciphered))) -> value v_ReceivedAsp { t_Wait.stop; if (not f_Check_NG_PDUSessionEstablishmentReq(v_GSM_MobilityInfo, v_ReceivedAsp.Signalling.Nas[0].Pdu)) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Received PDU Session Establishment REQ"); } } []t_Wait.timeout { f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 6"); } } //@siclog "Steps 7" siclog@ //Cause the UE to request establishment of PDU session without S-NSSAI. f_UT_RequestPDUSession(UT, f_NR5GC_MobileInfo_GetNoOfSessions()+1); //@siclog "Step 8 - 9" siclog@ f_NR5GC_PDUSessionEstablishment (v_NGC_NASCellA, 1);// @sic R5-233403 sic@ f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 8"); v_PDUSessionInfoList := f_NR5GC_MobileInfo_GetSessionInfoList(); v_PDU_SessionId := v_PDUSessionInfoList[0].SessionId; //@siclog "Step 10" siclog@ //The generic test procedure in TS 38.508-1 Table 4.9.21.2.2-1 of Procedure for PDU Session Release is performed. f_NR5GC_PDUSessionRelease(v_NGC_NASCellA, v_PDU_SessionId); //@siclog "Step 11" siclog@ //T3585 + 10% tolerance expired (330 seconds) t_T3585Max.timeout; //@siclog "Step 12" siclog@ //Cause the UE to request establishment of PDU session with same S-NSSAI which was used in Step 1. f_UT_RequestPDUSession(UT, f_NR5GC_MobileInfo_GetNoOfSessions()+1, -, -, -, v_NSSAI); //@siclog "Step 13" siclog@ //Void //@siclog "Steps 14 - 15" siclog@ f_NR5GC_PDUSessionEstablishment (v_NGC_NASCellA, 1, -, -, cs_S_NSSAI_SST1eMBB_WithIEI); f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 14 - 15"); v_PDUSessionInfoList := f_NR5GC_MobileInfo_GetSessionInfoList(); v_PDU_SessionId := v_PDUSessionInfoList[0].SessionId; //@siclog "Step 16" siclog@ //The generic test procedure in TS 38.508-1 Table 4.9.21.2.2-1 of Procedure for PDU Session Release is performed. f_NR5GC_PDUSessionRelease(v_NGC_NASCellA, v_PDU_SessionId); }//End of test body of TC_10_1_8_4

After Change

function fl_TC_10_1_8_4_Body() runs on NR5GC_PTC { var NR_CellId_Type v_NGC_NASCellA := f_NR5GC_MapNASCell(ngc_CellA, NG_AllCellsOnSamePLMN); var GSM_MobilityInfo_Type v_GSM_MobilityInfo; var NR_RadioBearerId_Type v_SRB := tsc_NR_RbId_SRB2; var charstring v_NSSAI := f_ConvertS_NSSAI_ForATCommand (cs_S_NSSAI_SST1eMBB); var template (value) GPRS_Timer3 v_BackOffTimer := cs_GPRS_Timer2_3_IEI('37'O, '101'B, '00101'B);//5 mins var template (value) PayloadContainerType v_PayloadContainerType := cs_PayloadContainerType ('0001'B); // @sic R5-236457 sic@ var template (value) GMM_GSM_Cause v_GMM_Cause := cs_GMM_GSM_Cause ('58'O, '01000101'B); // @sic R5-236457 sic@ var template (value) NG_NAS_DL_Pdu_Type v_PduToSend; var octetstring v_EncodedNasPdu; var O1_Type v_PDU_SessionId; // @sic R5-236457 sic@ var PDUSessionInfoList_Type v_PDUSessionInfoList; var NR_SRB_COMMON_IND v_ReceivedAsp; timer t_Wait; timer t_T3585Max := f_NR_SetTimerToleranceMax(v_NGC_NASCellA, nasTimer, 300.0); //@siclog "Steps 1" siclog@ //Cause the UE to request establishment of PDU session with S_NSSAI. f_UT_RequestPDUSession(UT, f_NR5GC_MobileInfo_GetNoOfSessions()+1, -, -, -, v_NSSAI); //@siclog "Steps 2" siclog@ SRB.receive(car_NR_SRB_NasPdu_IND(v_NGC_NASCellA, v_SRB, cr_NG_NAS_IndWithPiggybacking(tsc_SHT_IntegrityProtected_Ciphered))) -> value v_ReceivedAsp; if (not f_Check_NG_PDUSessionEstablishmentReq(v_GSM_MobilityInfo, v_ReceivedAsp.Signalling.Nas[0].Pdu, ?,?,-,-, -,-,-,-, -,-,-,-, cs_S_NSSAI_SST1eMBB_WithIEI //S-NSSAI specific )) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "PDU Session Establishment Message Failed"); } //@siclog "Step 3" siclog@ // @sic R5-236457 sic@ v_PDU_SessionId := v_ReceivedAsp.Signalling.Nas[0].Pdu.PiggybackedPduList[0].Msg.pdu_Session_Establishment_Request.pduSessionId.sessionId; //PDU SESSION ESTABLISHMENT REQUEST message received in Step 2 is included in the payload container v_EncodedNasPdu := bit2oct(encvalue(v_ReceivedAsp.Signalling.Nas[0].Pdu.PiggybackedPduList[0].Msg.pdu_Session_Establishment_Request)); v_PduToSend := f_Get_NG_DLNASTransport (v_PayloadContainerType, omit, cs_PayloadContainer(omit, v_EncodedNasPdu), v_PDU_SessionId, -, v_GMM_Cause, v_BackOffTimer, omit); SRB.send(cas_NR_SRB_NasPdu_REQ(v_NGC_NASCellA, v_SRB, -, cs_NG_NAS_Request(tsc_SHT_IntegrityProtected_Ciphered, v_PduToSend.Msg))); //@siclog "Step 4" siclog@ t_T3585Max.start; t_Wait.start(60.0); alt{ [] SRB.receive(car_NR_SRB_NasPdu_IND(v_NGC_NASCellA, v_SRB, cr_NG_NAS_IndWithPiggybacking(tsc_SHT_IntegrityProtected_Ciphered))) -> value v_ReceivedAsp { t_Wait.stop; if (not f_Check_NG_PDUSessionEstablishmentReq(v_GSM_MobilityInfo, v_ReceivedAsp.Signalling.Nas[0].Pdu)) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Received PDU Session Establishment REQ"); } } []t_Wait.timeout { f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 4"); } } //@siclog "Step 5" siclog@ //Cause the UE to request establishment of PDU session with same S-NSSAI which was used in Step 1. f_UT_RequestPDUSession(UT, f_NR5GC_MobileInfo_GetNoOfSessions()+1, -, -, -, v_NSSAI, -, -, -, -, -, -, LOCAL_CNF_REQUIRED); // @sic R5s230789 sic@ //@siclog "Step 6" siclog@ t_Wait.start(15.0); alt{ [] SRB.receive(car_NR_SRB_NasPdu_IND(v_NGC_NASCellA, v_SRB, cr_NG_NAS_IndWithPiggybacking(tsc_SHT_IntegrityProtected_Ciphered))) -> value v_ReceivedAsp { t_Wait.stop; if (not f_Check_NG_PDUSessionEstablishmentReq(v_GSM_MobilityInfo, v_ReceivedAsp.Signalling.Nas[0].Pdu)) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Received PDU Session Establishment REQ"); } } []t_Wait.timeout { f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 6"); } } //@siclog "Steps 7" siclog@ //Cause the UE to request establishment of PDU session without S-NSSAI. f_UT_RequestPDUSession(UT, f_NR5GC_MobileInfo_GetNoOfSessions()+1); //@siclog "Step 8 - 9" siclog@ f_NR5GC_PDUSessionEstablishment (v_NGC_NASCellA, 1);// @sic R5-233403 sic@ f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 8"); v_PDUSessionInfoList := f_NR5GC_MobileInfo_GetSessionInfoList(); v_PDU_SessionId := v_PDUSessionInfoList[1].SessionId; //@siclog "Step 10" siclog@ //The generic test procedure in TS 38.508-1 Table 4.9.21.2.2-1 of Procedure for PDU Session Release is performed. f_NR5GC_PDUSessionRelease(v_NGC_NASCellA, v_PDU_SessionId); //@siclog "Step 11" siclog@ //T3585 + 10% tolerance expired (330 seconds) t_T3585Max.timeout; //@siclog "Step 12" siclog@ //Cause the UE to request establishment of PDU session with same S-NSSAI which was used in Step 1. f_UT_RequestPDUSession(UT, f_NR5GC_MobileInfo_GetNoOfSessions()+1, -, -, -, v_NSSAI); //@siclog "Step 13" siclog@ //Void //@siclog "Steps 14 - 15" siclog@ f_NR5GC_PDUSessionEstablishment (v_NGC_NASCellA, 1, -, -, cs_S_NSSAI_SST1eMBB_WithIEI); f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 14 - 15"); v_PDUSessionInfoList := f_NR5GC_MobileInfo_GetSessionInfoList(); <<COMMENTED OUT>> //v_PDU_SessionId := v_PDUSessionInfoList[0].SessionId; <<COMMENTED OUT>> //@siclog "Step 16" siclog@ //The generic test procedure in TS 38.508-1 Table 4.9.21.2.2-1 of Procedure for PDU Session Release is performed. f_NR5GC_PDUSessionRelease(v_NGC_NASCellA, v_PDU_SessionId); }//End of test body of TC_10_1_8_4

# Branches executed

The test case was executed on NR band n78 using NR ciphering nea2 and integrity algorithm nia2

# Execution Log Files

## Samsung Shannon S5E9945ERD

The Samsung Shannon S5E9945ERD UE passed this test case on Anritsu Protocol Conformance Test System ME7834NR. The documentation below is enclosed as evidence of the successful test case run [1]:

- Test case execution log file:

TC_10_1_8_4_LOG.html

- PICS/PIXIT parameter file:

TC_10_1_8_4_PIXIT.xml

# References

| [1] | R5s240159: NR5GC FR1: Supporting information for addition of Rel-17 eNS test case 10.1.8.4 |
| --- | --- |