3GPP TSG-RAN5 Meeting #2024-TTCN email	R5s240160

# Online, , 8th Dec 2023 - 31st Dec 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 37.571-4 | CR | 0197 | rev | - | Current version: | 17.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to f_POS_NR_CheckCapabilities |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2024-02-09 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In Table 9.3.1.1.3.3-5 from 3GPP 37.571-2 it is mentioned that the presence of the IE otdoa-ProvideCapabilities depends on the pics pc_OTDOA_onNR. However, current implementation for NR5GC POS test cases also compares the presence of otdoa-ProvideCapabilities with pc_OTDOA, which is meant for LTE test cases. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The OTDOA check in f_POS_NR_CheckCapabilities is reworked to use only pc_OTDOA_onNR |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE can fail this test case |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.3.1.1, 9.3.4.1, 9.3.4.2, 9.3.4.3, 9.3.4.4, 9.3.1.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	2

2	Corrections required	2

2.1	Change 1	2

# Overview

This document lists all the essential changes needed to correct problems in the iwd-TTCN3-B2022-09_D23wk49 related to the title of this CR.

Contact: 	Parikshit Bhise

Parikshit.bhise@rohde-schwarz.com

# Corrections required

## Change 1

| Function name | f_POS_NR_CheckCapabilities |
| --- | --- |
| Reason for change | In Table 9.3.1.1.3.3-5 from 3GPP 37.571-2 it is mentioned that the presence of the IE otdoa-ProvideCapabilities depends on the pics pc_OTDOA_onNR. However, current implementation for NR5GC POS test cases also compares the presence of otdoa-ProvideCapabilities with pc_OTDOA, which is meant for LTE test cases. |
| Summary of change | The OTDOA check in f_POS_NR_CheckCapabilities is reworked to use only pc_OTDOA_onNR |
| TTCN module | NR5GC_Positioning_Functions.ttcn |
| MCC160 Comment | Accepted. |

Before

<<SKIPPED CODE>>

function f_POS_NR_CheckCapabilities (LPP_Message p_LppMessage, boolean p_CheckEpduCap) runs on NR_BASE_PTC

{

var octetstring v_Octetstring;

//CommonIEs

if (pc_LPP_MsgSegmentation) {

if (not isvalue(p_LppMessage.lpp_MessageBody.c1.provideCapabilities.criticalExtensions.c1.provideCapabilities_r9.commonIEsProvideCapabilities)) {

f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Error in Common capabilities PICS");

}

else {

if (not match(p_LppMessage.lpp_MessageBody.c1.provideCapabilities.criticalExtensions.c1.provideCapabilities_r9.commonIEsProvideCapabilities, cr_CommonIEsProvideCapabilities)) {

f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Error in LPP-Segmentation capabilities");

}

}

}

//AGNSS

if (isvalue(p_LppMessage.lpp_MessageBody.c1.provideCapabilities.criticalExtensions.c1.provideCapabilities_r9.a_gnss_ProvideCapabilities) and (pc_UEB_AGNSS or pc_UEA_AGNSS)) {

f_POS_NR_CheckCapabilities_Agnss(p_LppMessage.lpp_MessageBody.c1.provideCapabilities.criticalExtensions.c1.provideCapabilities_r9.a_gnss_ProvideCapabilities);

}

else if (isvalue(p_LppMessage.lpp_MessageBody.c1.provideCapabilities.criticalExtensions.c1.provideCapabilities_r9.a_gnss_ProvideCapabilities) or pc_UEB_AGNSS or pc_UEA_AGNSS) {

f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Error in A-GNSS capabilities PICS");

}

//OTDOA

if (isvalue(p_LppMessage.lpp_MessageBody.c1.provideCapabilities.criticalExtensions.c1.provideCapabilities_r9.otdoa_ProvideCapabilities)) {

if (not (p_LppMessage.lpp_MessageBody.c1.provideCapabilities.criticalExtensions.c1.provideCapabilities_r9.otdoa_ProvideCapabilities.otdoa_Mode[0]== '1'B) and pc_OTDOA_onNR) { //@sic R5-225313 sic@

f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "UE reported OTDOA UE-Assisted mode not supported");

}

else if((p_LppMessage.lpp_MessageBody.c1.provideCapabilities.criticalExtensions.c1.provideCapabilities_r9.otdoa_ProvideCapabilities.otdoa_Mode[0]== '1'B) and pc_OTDOA_onNR){ //@sic R5-225313 sic@

f_POS_NR_OTDOA_CheckEutraBand(p_LppMessage.lpp_MessageBody.c1.provideCapabilities.criticalExtensions.c1.provideCapabilities_r9.otdoa_ProvideCapabilities);

}

}

else if (pc_OTDOA) {

f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Error in OTDOA capabilities PICS");

}

<<SKIPPED CODE>>

After

<<SKIPPED CODE>>

function f_POS_NR_CheckCapabilities (LPP_Message p_LppMessage, boolean p_CheckEpduCap) runs on NR_BASE_PTC

{

var octetstring v_Octetstring;

//CommonIEs

if (pc_LPP_MsgSegmentation) {

if (not isvalue(p_LppMessage.lpp_MessageBody.c1.provideCapabilities.criticalExtensions.c1.provideCapabilities_r9.commonIEsProvideCapabilities)) {

f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Error in Common capabilities PICS");

}

else {

if (not match(p_LppMessage.lpp_MessageBody.c1.provideCapabilities.criticalExtensions.c1.provideCapabilities_r9.commonIEsProvideCapabilities, cr_CommonIEsProvideCapabilities)) {

f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Error in LPP-Segmentation capabilities");

}

}

}

//AGNSS

if (isvalue(p_LppMessage.lpp_MessageBody.c1.provideCapabilities.criticalExtensions.c1.provideCapabilities_r9.a_gnss_ProvideCapabilities) and (pc_UEB_AGNSS or pc_UEA_AGNSS)) {

f_POS_NR_CheckCapabilities_Agnss(p_LppMessage.lpp_MessageBody.c1.provideCapabilities.criticalExtensions.c1.provideCapabilities_r9.a_gnss_ProvideCapabilities);

}

else if (isvalue(p_LppMessage.lpp_MessageBody.c1.provideCapabilities.criticalExtensions.c1.provideCapabilities_r9.a_gnss_ProvideCapabilities) or pc_UEB_AGNSS or pc_UEA_AGNSS) {

f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Error in A-GNSS capabilities PICS");

}

//OTDOA

if (isvalue(p_LppMessage.lpp_MessageBody.c1.provideCapabilities.criticalExtensions.c1.provideCapabilities_r9.otdoa_ProvideCapabilities) and pc_OTDOA_onNR) {

if(p_LppMessage.lpp_MessageBody.c1.provideCapabilities.criticalExtensions.c1.provideCapabilities_r9.otdoa_ProvideCapabilities.otdoa_Mode[0]== '1'B){

f_POS_NR_OTDOA_CheckEutraBand(p_LppMessage.lpp_MessageBody.c1.provideCapabilities.criticalExtensions.c1.provideCapabilities_r9.otdoa_ProvideCapabilities);

}

else{

f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "UE reported OTDOA UE-Assisted mode not supported");

}

}

else if (isvalue(p_LppMessage.lpp_MessageBody.c1.provideCapabilities.criticalExtensions.c1.provideCapabilities_r9.otdoa_ProvideCapabilities) or pc_OTDOA_onNR) {

f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Error in OTDOA capabilities PICS");

}

<<SKIPPED CODE>>