3GPP TSG-RAN5 Meeting #2024-TTCN email	R5s240163

# Online, , 8th Dec 2023 - 31st Dec 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 3379 | rev | - | Current version: | 17.9.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction of Rel-17 eNS test case 10.1.8.2 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ANRITSU LTD |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI17_Test, eNS_Ph2-UEConTest |  |  |  |  |  | Date: |  |  | 2024-02-14 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | At step9 and step26, in using tsc_RandomAPN will cause to set PDU type as none. So on further step at f_NR5GC_PDUSessionEstablishment need to send DNN as well to correctly evaluate tsc_RandomAPN as PDU type |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added cs_DNN(-, f_DomainName_Encode(valueof(tsc_RandomAPN))) on function f_NR5GC_PDUSessionEstablishment as argument. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test may fail |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 10.1.8.2.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR… |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

1.1	Verification Test Summary	3

2	Corrections required	3

2.1	Change 1	5

# Overview

This document lists all the changes needed to correct issues in the ATS iwd-TTCN3-B2022-09_D23wk49 related to the title of this CR.

Contact:	Kalahasti, Narendra
	Narendra.Kalahasti@anritsu.com

# Corrections required

## Change 1

| Function name | function fl_TC_10_1_8_2_Body () |
| --- | --- |
| Reason for change | At step9 and step26, in using tsc_RandomAPN will cause to set PDU type as none which leads to the test failure. In fuction f_NR5GC_PDUSessionEstablishment, it need to send DNN as well to correctly evaluate tsc_RandomAPN as internet type. |
| Summary of change | Added cs_DNN(-, f_DomainName_Encode(valueof(tsc_RandomAPN))) on function f_NR5GC_PDUSessionEstablishment as argument. |
| TTCN module | GSM_NR5GC.ttcn |
| MCC160 Comment | Accepted |

Before Change

function fl_TC_10_1_8_2_Body() runs on NR5GC_PTC { ------skipping code------ //@siclog "Step 8" siclog@ f_NR5GC_SwitchOnAndResetIMS(); //@siclog "Step 9" siclog@ f_NR5GC_Registration(v_NGC_NASCellA); // @sic R5s230782 sic@ if (pc_noOf_PDUsSameConnection > 0) // @sic R5s230782 sic@ { f_NR5GC_RRC_Idle_Steps16_20(v_NGC_NASCellA, -, TESTMode_OFF); f_NR5GC_TriggerSinglePDUSession(v_NGC_NASCellA, f_NR5GC_MobileInfo_GetNoOfSessions()+1, -, tsc_RandomAPN, cs_S_NSSAI_SST1eMBB); }else { if(pc_noOf_PDUsNewConnection > 0 or pc_noOf_PDUsSameConnection == 0) { //Anritsu TTCN CR R5s240104 if (pc_noOf_PDUsNewConnection == 0 and pc_noOf_PDUsSameConnection == 0) { f_NR_RRCRelease(v_NGC_NASCellA); f_UT_RequestPDUSession(UT, f_NR5GC_MobileInfo_GetNoOfSessions()+1, tsc_RandomAPN, -, -, v_NSSAI); v_ReceivedMsg := f_NR_RRC_ConnEst_DefWithNas (v_NGC_NASCellA, tsc_NR_RRC_TI_Def, ?); // @sic R5s200489 sic@ if (not f_Check_NG_ServiceReqMsg (v_ReceivedMsg.Pdu, cs_NAS_KeySetIdentifier_lv(f_NR_SecurityKSIamf_Get (), tsc_NasKsi_NativeSecurityContext), ?, '0000'B)) { // @sic R5s210189 sic@ f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Service Request Message Failed"); } f_NR_RRC_ActivateSecurity (v_NGC_NASCellA, valueof(v_ReceivedMsg.SecurityProtection.NasCount)); // Use NasCount returned from Service Req SRB.send (cas_NR_SRB_NasPdu_REQ(v_NGC_NASCellA, tsc_NR_RbId_SRB1, -, cs_NG_NAS_Request(tsc_SHT_IntegrityProtected_Ciphered, f_Get_NG_ServiceAcceptMsg(v_ReceivedMsg.Pdu.Msg.service_Request.pduSessionStatus)))); f_NR5GC_PDUSessionEstablishment (v_NGC_NASCellA, 1,//Anritsu TTCN CR R5s240104 tsc_NR_RbId_SRB1, // @sic R5-206291 sic@ omit, // @sic R5s210134 sic@ *, *, crs_SSC_Mode(-, '001'B), omit, omit, cs_Session_AMBR ('06'O, '05'O, '0004'O, '05'O, '0004'O), cs_S_NSSAI_SST1eMBB_WithIEI, omit); } } } //@siclog "Step 9a1 - 9a2" siclog@ // @sic R5-233405 sic@ ------skipping code------ //@siclog "Steps 24 - 25" siclog@ f_NR5GC_PDUSessionRelease(v_NGC_NASCellA, v_PDUSessionIdToRelease); //@siclog "Steps 26" siclog@ //Cause the UE to request establishment of PDU session with same S-NSSAI which was used in Step 1. f_UT_RequestPDUSession(UT, f_NR5GC_MobileInfo_GetNoOfSessions()+1, tsc_RandomAPN, -, -, v_NSSAI); //AT command to trigger a PDU session WITH S-NSSAI // @sic R5s230782 sic@ //@siclog "Steps 27 - 28" siclog@ f_NR5GC_PDUSessionEstablishment (v_NGC_NASCellA, 1, -, -, cs_S_NSSAI_SST1eMBB_WithIEI); f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 27 - 28"); } //End of test body of TC_10_1_8_2

After Change

function fl_TC_10_1_8_2_Body() runs on NR5GC_PTC { ------skipping code------ //@siclog "Step 8" siclog@ f_NR5GC_SwitchOnAndResetIMS(); //@siclog "Step 9" siclog@ f_NR5GC_Registration(v_NGC_NASCellA); // @sic R5s230782 sic@ if (pc_noOf_PDUsSameConnection > 0) // @sic R5s230782 sic@ { f_NR5GC_RRC_Idle_Steps16_20(v_NGC_NASCellA, -, TESTMode_OFF); f_NR5GC_TriggerSinglePDUSession(v_NGC_NASCellA, f_NR5GC_MobileInfo_GetNoOfSessions()+1, -, tsc_RandomAPN, cs_S_NSSAI_SST1eMBB); }else { if(pc_noOf_PDUsNewConnection > 0 or pc_noOf_PDUsSameConnection == 0) { //Anritsu TTCN CR R5s240104 if (pc_noOf_PDUsNewConnection == 0 and pc_noOf_PDUsSameConnection == 0) { f_NR_RRCRelease(v_NGC_NASCellA); f_UT_RequestPDUSession(UT, f_NR5GC_MobileInfo_GetNoOfSessions()+1, tsc_RandomAPN, -, -, v_NSSAI); v_ReceivedMsg := f_NR_RRC_ConnEst_DefWithNas (v_NGC_NASCellA, tsc_NR_RRC_TI_Def, ?); // @sic R5s200489 sic@ if (not f_Check_NG_ServiceReqMsg (v_ReceivedMsg.Pdu, cs_NAS_KeySetIdentifier_lv(f_NR_SecurityKSIamf_Get (), tsc_NasKsi_NativeSecurityContext), ?, '0000'B)) { // @sic R5s210189 sic@ f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Service Request Message Failed"); } f_NR_RRC_ActivateSecurity (v_NGC_NASCellA, valueof(v_ReceivedMsg.SecurityProtection.NasCount)); // Use NasCount returned from Service Req SRB.send (cas_NR_SRB_NasPdu_REQ(v_NGC_NASCellA, tsc_NR_RbId_SRB1, -, cs_NG_NAS_Request(tsc_SHT_IntegrityProtected_Ciphered, f_Get_NG_ServiceAcceptMsg(v_ReceivedMsg.Pdu.Msg.service_Request.pduSessionStatus)))); f_NR5GC_PDUSessionEstablishment (v_NGC_NASCellA, 1,//Anritsu TTCN CR R5s240104 tsc_NR_RbId_SRB1, // @sic R5-206291 sic@ omit, // @sic R5s210134 sic@ *, *, crs_SSC_Mode(-, '001'B), cs_DNN(-, f_DomainName_Encode(valueof(tsc_RandomAPN))), omit, cs_Session_AMBR ('06'O, '05'O, '0004'O, '05'O, '0004'O), cs_S_NSSAI_SST1eMBB_WithIEI, omit); } } } //@siclog "Step 9a1 - 9a2" siclog@ // @sic R5-233405 sic@ -------skipping code------- //@siclog "Steps 24 - 25" siclog@ f_NR5GC_PDUSessionRelease(v_NGC_NASCellA, v_PDUSessionIdToRelease); //@siclog "Steps 26" siclog@ //Cause the UE to request establishment of PDU session with same S-NSSAI which was used in Step 1. f_UT_RequestPDUSession(UT, f_NR5GC_MobileInfo_GetNoOfSessions()+1, tsc_RandomAPN, -, -, v_NSSAI); //AT command to trigger a PDU session WITH S-NSSAI // @sic R5s230782 sic@ //@siclog "Steps 27 - 28" siclog@ f_NR5GC_PDUSessionEstablishment (v_NGC_NASCellA, 1, -, -, cs_S_NSSAI_SST1eMBB_WithIEI,-,-, cs_DNN(-, f_DomainName_Encode(valueof(tsc_RandomAPN)))); f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 27 - 28"); } //End of test body of TC_10_1_8_2