3GPP TSG-RAN5 Meeting #2024-TTCN email	R5s240218

# Online, , 8th Dec 2023 - 31st Dec 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.523-3 | CR | 4793 | rev | - | Current version: | 18.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of LTE NAS RACS testcase 9.2.5.2 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ANRITSU LTD |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI16_Test, RACS-UEConTest |  |  |  |  |  | Date: |  |  | 2024-02-20 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | To add test case 9.2.5.2 to the LTE_A_PRO ATS. |  |  |  |  |  |  |  |  |
| Summary of change: |  | This document lists all changes applied to test case 9.2.5.2 required for approval. See detailed change description for further information |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test case will not be added to ATS |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.2.5.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS36.523-1 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Table of Contents

Table of Contents	2

1	Overview	3

1.1	Verification Test Summary	3

2	Corrections required	3

2.1	Change 1	3

3	Branches executed	5

4	Execution Log Files	5

4.1	Samsung Shannon S5E9945ERD UE	5

5	References	5

- Overview

This document lists all the essential changes needed to correct problems in the TTCN implementation of test case 9.2.5.2 which is part of the LTE_A_PRO test suite.

With these changes applied the test case can be demonstrated to run with one or more UEs (see section 4). Execution log files are provided as evidence.

Contact:	Kalahasti, Narendra

Narendra.Kalahasti@anritsu.com

  - Verification Test Summary

Test Case: 	9.2.5.2

ATS Version:	iwd-TTCN3-B2022-09_D23wk49

System Simulator used:	Anritsu Protocol Conformance Test System ME7834NR

UE used:	Samsung Shannon S5E9945ERD UE

Verification Status:	PASS

- Corrections required

  - Change 1

| Testcase name | fl_TC_9_2_5_2_Body |
| --- | --- |
| Reason for change | - UE remains in Switch off state post Step2, it is necessary to add UT command for Switch on. - With UE switched off and change of USIM at Step2, it is possible UE can send attach request without security header protection. - According to cl. 5.3.20 od TS 24.301 “A UE radio capability ID can be either manufacturer-assigned or network-assigned.” UE with manufacturer-assigned UE Radio Capability ID can include UE Radio Capability availability inidication in the attach request at Step3, hence it is proposed to request actual ID using NAS Security Mode command and validate if UE reports Network-assigned ID. |
| Summary of change | - Add missing UT command for UE Switch on. - Allow SS to receive attach request with no security header protection. - Validate the UE radio capability ID received in the NAS Security mode complete message instead of checking UE radio capability availability indication in the Attach request message. |
| TTCN module | LTE_A_PRO\9_2\NAS_RACS.ttcn |
| MCC160 Comment | - Accepted. - Accepted. - Rejected, this change is inconsistent with the R5-241690 agreed at RAN5#102. |

Before change:

function fl_TC_9_2_5_2_Body() runs on EUTRA_PTC

{

var NAS_MSG_Indication_Type v_NAS_Ind;

//@siclog "Step 1" siclog@

//If possible (see ICS) switch off is performed or the USIM is removed.Otherwise the power is removed.

f_EUTRA_UE_Detach_SwitchOffUe(eutra_CellA, RRC_IDLE);

//@siclog "Step 2" siclog@

//The UE is brought back to operation with USIM A inserted.

f_UT_USIM_Insert(UT, "36.523-1 Table 9.2.5.2.3.1-1");

f_UT_AutomaticPLMN_Select_WithSwitchOnOff (UT);

//@siclog "Step 3" siclog@

//Check: Does the UE transmit an ATTACH REQUEST message without including UE radio capability availability indication?

v_NAS_Ind:= f_EUTRA_RRC_ConnEst_DefWithNas(eutra_CellA,

tsc_RRC_TI_Def,

cr_EstablishmentCause_Any,

cr_NAS_IndicationWithPiggybacking(tsc_SHT_IntegrityProtected,

cr_508_ATTACH_REQUEST(f_GetEAttachType(NORMAL), f_GetAdditionalUpdateType(NORMAL)),

cr_508_PDN_CONNECTIVITY_REQUEST(tsc_PdnRequest_InitialAttach,

f_GetPdnType())));

if (match (v_NAS_Ind.Pdu.Msg.aTTACH_REQUEST.ueRadioCapIdAvailability.availability, '000'B)) { //prose CR needed.

f_EUTRA_PreliminaryPass(__FILE__, __LINE__, "Step 3");

}

//@siclog "Step 4-17" siclog@

//The attach procedure is completed by executing steps 5 to 18 of the UE registration procedure in TS 36.508 sub clause 4.5.2.3.

f_EUTRA_InitialRegistration_Step5_18b1(eutra_CellA, STATE2_IDLEUPDATE, NORMAL, v_NAS_Ind);

} //End of fl_TC_9_2_5_2_Body

After change:

function fl_TC_9_2_5_2_Body() runs on EUTRA_PTC

{

var NAS_MSG_Indication_Type v_NAS_Ind;

//@siclog "Step 1" siclog@

//If possible (see ICS) switch off is performed or the USIM is removed.Otherwise the power is removed.

f_EUTRA_UE_Detach_SwitchOffUe(eutra_CellA, RRC_IDLE);

//@siclog "Step 2" siclog@

//The UE is brought back to operation with USIM A inserted.

f_UT_USIM_Insert(UT, "36.523-1 Table 9.2.5.2.3.1-1");

f_UT_AutomaticPLMN_Select_WithSwitchOnOff (UT);

f_EUTRA_SwitchOnUEandStartIP(eutra_CellA);

//@siclog "Step 3" siclog@

//Check: Does the UE transmit an ATTACH REQUEST message without including UE radio capability availability indication?

v_NAS_Ind:= f_EUTRA_RRC_ConnEst_DefWithNas(eutra_CellA,

tsc_RRC_TI_Def,

cr_EstablishmentCause_Any,

cr_NAS_IndicationWithPiggybacking((tsc_SHT_NoSecurityProtection, tsc_SHT_IntegrityProtected),

cr_508_ATTACH_REQUEST(f_GetEAttachType(NORMAL), f_GetAdditionalUpdateType(NORMAL)),

cr_508_PDN_CONNECTIVITY_REQUEST(tsc_PdnRequest_InitialAttach,

f_GetPdnType())));

//@siclog "Step 4-5" siclog@

//Steps 5-6 of the generic procedure for UE registration specified in TS 36.508 subclause 4.5.2.3 are performed.

f_EUTRA_InitialRegistration_Step5_6(eutra_CellA, v_NAS_Ind, false);

v_AuthParams := f_EUTRA_Security_Get();

//@siclog "Step 6" siclog@

//The SS transmits a NAS SECURITY MODE COMMAND message requesting UE radio capability ID.

//siclog "Step 7" siclog@

//Check: Does the UE transmit a NAS SECURITY MODE COMPLETE message including UE radio capability ID 1?

if (ispresent(v_NAS_Ind.Pdu.Msg.aTTACH_REQUEST.ueAddSecurityCap)) {

v_AuthParams.NAS_SecurityCap5G := v_NAS_Ind.Pdu.Msg.aTTACH_REQUEST.ueAddSecurityCap;

}

v_NasCountUL:= f_EUTRA_NAS_ActivateSecurity(eutra_CellA,v_AuthParams,-,true,true, v_NAS_Ind.Pdu.Msg.aTTACH_REQUEST.ueRadioCapIdAvailability);

v_UERadioCapId := f_EUTRA_MobileInfo_GetRxdRACS_Id();

if (isvalue(v_UERadioCapId)) {

if (match(valueof(v_UERadioCapId), cs_UERadioCapId('66'O, '00?????????'O))) {

f_EUTRA_PreliminaryPass(__FILE__, __LINE__, "Step 7");

} else {

f_EUTRA_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 7: UE reported Network-assigned UE radio capability ID");

}

} else {

f_EUTRA_PreliminaryPass(__FILE__, __LINE__, "Step 7");

}

//@siclog "Steps 8-17" siclog@

//Steps 9-18 of the generic procedure for UE registration specified in TS 36.508 subclause 4.5.2.3 are performed.

f_EUTRA_InitialRegistration_Step9_18b1(eutra_CellA,v_NAS_Ind,v_NasCountUL);

} //End of fl_TC_9_2_5_2_Body

- Branches executed

The test case implementation was executed with Integrity and ciphering enabled using LTE FDD band 4.

- Execution Log Files

  - Samsung Shannon S5E9945ERD UE

The Samsung Shannon S5E9945ERD UE passed this test case on Anritsu Protocol Conformance Test System ME7834NR. The documentation below is enclosed as evidence of the successful test case run [1]:

- Test case execution log file:

TC_9_2_5_2_LOG.xml

- PICS/PIXIT parameter file:

TC_9_2_5_2 _PIXIT.txt

- References

| [1] | R5s240219: Supporting information for addition of LTE NAS RACS testcase 9.2.5.2 |
| --- | --- |