3GPP TSG-RAN5 Meeting #2024-TTCN email	R5s240227

# Online, , 8th Dec 2023 - 31st Dec 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 3384 | rev | - | Current version: | 17.9.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to function f_EUTRA38_MultiPDN_EstAddPDNs_DefaultActivated |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2024-03-04 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In the current TTCN implementation of this function, f_EUTRA38_MultiPDN_GetPDNType(v_PDNConnReq.accessPointName) returns PDN Type as IMS_DNN even if the PDN connectivity REQ indicates the request type as emergency, and if there is no APN provided in the PDN Connectivity REQ. This then results in the timer v_WaitForUserplaneSignalling being set to waitForIMS. For a testcase like 11.1.7, this introduces a delay of 10s between the UE transmitting ACK (34.229-1 Annex C.22 Step 5) and receiving Activate DedicatedEPS Bearer Context REQ because when the function fl_TC_11_1_7_NR5GC_EUTRA_TestBody --> f_EUTRA38_MultiPDN_EstablishAdditionalPDNs --> f_EUTRA38_MultiPDN_EstAddPDNs_DefaultActivated --> f_EUTRA_DelayForUserPlaneSignalling is called. Some UE implementations release the IMS call because of this delay (by sending a BYE), which results in testcase failure. This needs to be addressed. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Updated the implementation of function f_EUTRA38_MultiPDN_EstAddPDNs_DefaultActivated. Please see below. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may fail the test case. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 11.1.7.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	3

1	Overview	4

2	Corrections required	4

2.1	Function f_EUTRA38_MultiPDN_EstAddPDNs_DefaultActivated	4

# Overview

This document lists all the essential changes needed to correct problems in the iwd-TTCN3-B2022-09_D23wk49 related to the title of this CR.

Contact:	Parikshit Bhise

Parikshit.bhise@rohde-schwarz.com

# Corrections required

## Function f_EUTRA38_MultiPDN_EstAddPDNs_DefaultActivated

| Function name | f_EUTRA38_MultiPDN_EstAddPDNs_DefaultActivated |
| --- | --- |
| Reason for change | In the current TTCN implementation of this function, f_EUTRA38_MultiPDN_GetPDNType(v_PDNConnReq.accessPointName) returns PDN Type as IMS_DNN even if the PDN connectivity REQ indicates the request type as emergency, and if there is no APN provided in the PDN Connectivity REQ. This then results in the timer v_WaitForUserplaneSignalling being set to waitForIMS. For a testcase like 11.1.7, this introduces a delay of 10s between the UE transmitting ACK (34.229-1 Annex C.22 Step 5) and receiving Activate DedicatedEPS Bearer Context REQ because when the function fl_TC_11_1_7_NR5GC_EUTRA_TestBody --> f_EUTRA38_MultiPDN_EstablishAdditionalPDNs --> f_EUTRA38_MultiPDN_EstAddPDNs_DefaultActivated --> f_EUTRA_DelayForUserPlaneSignalling is called. Some UE implementations release the IMS call because of this delay (by sending a BYE), which results in testcase failure. This needs to be addressed. |
| Summary of change | Updated the TTCN implementation to 1) check if the IE AccessPointName is present in the PDN Connectivity REQ, and based on that and RequestType, assigned the value to IE AccessPointName. Please see below. |
| TTCN module | Eutra38_NR5GC_InitialRegistration.ttcn |
| MCC160 Comment | Accepted in principle, but got it from the DRB Info as this information has already been stored |

Before Change:

<<SKIPPED CODE>> v_SRB := tsc_SRB2; // SRB2 will have been configured by now, even if not earlier a_EUTRA38_MultiPDN_ConnectToAdditionalPDN_Step4(p_CellId, v_EpsBearerId); // If the UE has just moved from NR, don't need to wait for IMS signalling if (not isvalue(p_PduInfoFromNR)) { // @sic R5s230015 sic@ select (f_EUTRA38_MultiPDN_GetPDNType(v_PDNConnReq.accessPointName)) { // @sic R5-206296 sic@ case (IMS_DNN) { v_WaitForUserplaneSignalling := waitForIMS; } case (Internet_DNN) { v_WaitForUserplaneSignalling := dontWaitForIMS; } } } } } // Now all PDNs on same connection have been established, wait for any signalling on user plane to finish before releasing connection f_EUTRA_DelayForUserPlaneSignalling (v_WaitForUserplaneSignalling); return v_SRB; <<SKIPPED CODE>>

After Change:

<<SKIPPED CODE>> v_SRB := tsc_SRB2; // SRB2 will have been configured by now, even if not earlier a_EUTRA38_MultiPDN_ConnectToAdditionalPDN_Step4(p_CellId, v_EpsBearerId); if( not ispresent(v_PDNConnReq.accessPointName)) { //WA#11_1_7 if(v_PDNConnReq.requestType.typeValue == '100'B){ v_PDNConnReq.accessPointName := valueof(cs_AccessPointName(f_DomainName_EncodeLabels({"sos"}))); } } // If the UE has just moved from NR, don't need to wait for IMS signalling if (not isvalue(p_PduInfoFromNR)) { // @sic R5s230015 sic@ select (f_EUTRA38_MultiPDN_GetPDNType(v_PDNConnReq.accessPointName)) { // @sic R5-206296 sic@ case (IMS_DNN) { v_WaitForUserplaneSignalling := waitForIMS; } case (Internet_DNN) { v_WaitForUserplaneSignalling := dontWaitForIMS; } } } } } // Now all PDNs on same connection have been established, wait for any signalling on user plane to finish before releasing connection f_EUTRA_DelayForUserPlaneSignalling (v_WaitForUserplaneSignalling); return v_SRB; <<SKIPPED CODE>>

#### MCC160 Implementation:

function f_EUTRA38_MultiPDN_EstAddPDNs_DefaultActivated(EUTRA_CellId_Type p_CellId,

integer p_ExpectedNoOfNewPDNs,

boolean p_FailWhenK_LessN := true,

integer p_SRB := tsc_SRB2,  // @sic R5s210341 sic@

template (omit) NasCount_Type p_NasCountUL := omit, // Only needed when called after TAU

B3_Type p_RequestType := '001'B,

template AccessPointName p_AccessPointName := cr_AccessPointName_Any,

boolean p_N1disabled := false,

template (omit) EUTRA_NR_PduSessionInfoList_Type p_PduInfoFromNR := omit

) runs on EUTRA_Multi5GS_PTC return integer

{

var SRB_COMMON_IND v_ReceivedAsp;

var NAS_MSG_Indication_Type v_NasInd;

var PDN_CONNECTIVITY_REQUEST v_PDNConnReq;

var DelayForUserPlaneSignalling_Type v_WaitForUserplaneSignalling := noDelay; // Default: no IP signalling possible

var EPS_BearerIdentity v_EpsBearerId;

var integer v_L_NumOfReqsReceivedInParallel := 0;

var integer v_K_NumOfPDNsProcessed := 0;

var boolean v_EndLoop := false;

var boolean v_RxdMsg;

var integer v_SRB := p_SRB;

var B3_Type v_PDNType := f_GetPdnType();

timer t_Wait := 10.0;

while ((v_K_NumOfPDNsProcessed < p_ExpectedNoOfNewPDNs) and not v_EndLoop) {

v_RxdMsg := false;

t_Wait.start;

if (f_EUTRA38_MsgInDefault_GetNumOfPDNConnectivityReq() > v_L_NumOfReqsReceivedInParallel) {

t_Wait.stop;

v_L_NumOfReqsReceivedInParallel := v_L_NumOfReqsReceivedInParallel +1;

v_ReceivedAsp := f_EUTRA38_MsgInDefault_GetOnePDNConnectivityReq(v_L_NumOfReqsReceivedInParallel);

v_RxdMsg := true;

} else { // no outstanding msgs, so must wait for next one

alt {

[] SRB.receive(car_SRB_NasPdu_IND(p_CellId, v_SRB, cr_NAS_Indication(tsc_SHT_IntegrityProtected_Ciphered, cr_PDNConnReqAdditionalPDN(v_PDNType, p_AccessPointName, p_RequestType))))  // @sic R5-216188 sic@

-> value v_ReceivedAsp    /* NOTE: acc. to the prose the ESM information transfer flag is still "Not present or any allowed value"

*       nevertheless if the UE sets the ESM information transfer flag to true this may cause issues

*       as the generic procedure does not handle any requested ESM information transfer */

{

t_Wait.stop;

v_RxdMsg := true;

}

[] t_Wait.timeout

{ // @sic R5-206291 sic@

if (p_FailWhenK_LessN and (v_K_NumOfPDNsProcessed < p_ExpectedNoOfNewPDNs)) {

f_EUTRA_SetVerdictFailOrInconc(__FILE__, __LINE__, "Established " & int2str(v_K_NumOfPDNsProcessed) & "PDNs instead of " & int2str(p_ExpectedNoOfNewPDNs));

} else if (not p_FailWhenK_LessN) { // New message not received in time, so just end the loop

v_EndLoop := true;

}

}

}

}

if (v_RxdMsg) { // this will be false if either just triggered a new PDN, or about to end the loop

// Update K now as latest msg is about to be processed

v_K_NumOfPDNsProcessed := v_K_NumOfPDNsProcessed +1;

v_NasInd := v_ReceivedAsp.Signalling.Nas[0];

v_PDNConnReq := v_NasInd.Pdu.Msg.pDN_CONNECTIVITY_REQUEST;

v_EpsBearerId := f_EUTRA38_MultiPDN_EstablishAdditionalPDN_Step2(p_CellId, v_PDNConnReq, (v_SRB != tsc_SRB2), p_NasCountUL, p_PduInfoFromNR, p_N1disabled);  // @sic R5-227443, R5s230015 sic@

a_EUTRA_IdleUpdated_ConnectToAdditionalPDN_Step3(p_CellId);

v_SRB := tsc_SRB2;  // SRB2 will have been configured by now, even if not earlier

a_EUTRA38_MultiPDN_ConnectToAdditionalPDN_Step4(p_CellId, v_EpsBearerId);

// If the UE has just moved from NR, don't need to wait for IMS signalling

if (not isvalue(p_PduInfoFromNR)) { // @sic R5s230015 sic@

v_ExistingDRBList := f_EUTRA38_MobileInfo_GetDRBInfoList();

// get details of bearer just activated

v_Index := f_EUTRA38_DRBInfo_FindEntry(v_ExistingDRBList, (hex2int(v_EpsBearerId) -4));

select (v_ExistingDRBList[v_Index].APN ) { // @sic R5-206296 sic@

case (IMS_DNN) {

v_WaitForUserplaneSignalling := waitForIMS;

}

case (Internet_DNN) {

v_WaitForUserplaneSignalling := dontWaitForIMS;

}

}

}

}

}

// Now all PDNs on same connection have been established, wait for any signalling on user plane to finish before releasing connection

f_EUTRA_DelayForUserPlaneSignalling (v_WaitForUserplaneSignalling);

return v_SRB;

}