3GPP TSG-RAN5 Meeting #2024-TTCN email	R5s240229

# Online, , 8th Dec 2023 - 31st Dec 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 3385 | rev | - | Current version: | 17.9.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to NR5GC testcase 8.1.5.8.1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2024-03-05 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In the current TTCN implementation of 23wk49, function TC_8_1_5_8_1_NR5GC_TestBody at steps 7-8 , Procedure delay is checked for UECapabilityEnquiry and Information. The expectation is that UE sends UECapabilityInformation as a whole message using NR_ImcsValue_Type tsc_NR_DefaultGrant_Imcs := 9; Which is provided as part of the function f_NR_ULGrantConfiguration_Start. But if UE is having bigger UECapabilityInformation which can not be accommodated with NR_ImcsValue_Type tsc_NR_DefaultGrant_Imcs := 9; then SS will end up receiving UECapabilityInformation in segments which will fail the case due to procedure delay condition not met . Hence it is proposed to have the periodic UL grant configurable for receiving UECapabilityInformation message. |  |  |  |  |  |  |  |  |
| Summary of change: |  | periodic UL grant configurable for receiving UECapabilityInformation message is made configurable using Imcs value. Imcs value is set as 12 for the current requirement |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may fail the test case |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.1.5.8.1.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	3

1	Overview	3

2	Corrections required	3

2.1	Change to the Function f_TC_8_1_5_8_1_NR5GC_TestBody	3

2.2	Change to the Function f_NR5GC_RRC_ProcedureDelay	5

# Overview

This document lists all the essential changes needed to correct problems in the iwd-TTCN3-B2022-09_D23wk49 related to the title of this CR.

Contact: 	Parikshit Bhise

Parikshit.bhise@rohde-schwarz.com

# Corrections required

## Change to the Function f_TC_8_1_5_8_1_NR5GC_TestBody

| Function name | f_TC_8_1_5_8_1_NR5GC_TestBody |
| --- | --- |
| Reason for change | In the current TTCN implementation of 23wk49, function TC_8_1_5_8_1_NR5GC_TestBody at steps 7-8 , Procedure delay is checked for UECapabilityEnquiry and Information. The expectation is that UE sends UECapabilityInformation as a whole message using NR_ImcsValue_Type tsc_NR_DefaultGrant_Imcs := 9; Which is provided as part of the function f_NR_ULGrantConfiguration_Start. But if UE is having bigger UECapabilityInformation which can not be accommodated with NR_ImcsValue_Type tsc_NR_DefaultGrant_Imcs := 9; then SS will end up receiving UECapabilityInformation in segments which will fail the case due to procedure delay condition not met . Hence it is proposed to have the periodic UL grant configurable for receiving UECapabilityInformation message. |
| Summary of change | periodic UL grant configurable for receiving UECapabilityInformation message is made configurable using Imcs value. Imcs value is set as 12 for the current requirement |
| TTCN module | NR5GC\8_1_5\RRC_Others_NR5GC.ttcn |
| MCC160 Comment | See change 2.2 |

Before Change:

function f_TC_8_1_5_8_1_NR5GC_TestBody() runs on NR5GC_PTC { <<SKIPPED CODE>> //@siclog "Step 7 - 8" siclog@ //The SS transmits a UECapabilityEnquiry message. //The SS starts transmitting a periodic DCI from (80*2^numerology+1+TDL) slots after step 7 to schedule PUSCH. //Check: Does the UE transmit a UECapabilityInformation message within (80*2^numerology+1+TDL+K2) slots after successful completion of step 7? v_EncodeCapRequestNR := bit2oct(encvalue( cs_38508_UECapabilityRequestFilterNR({ cs_NR_BandInformationNR(f_NR_CellInfo_GetBandIndicator(nr_Cell1)) }) )); f_NR5GC_RRC_ProcedureDelay(nr_Cell1, cas_NR_SRB1_RrcPdu_REQ(nr_Cell1, -, cs_38508_UECapabilityEnquiry(tsc_NR_RRC_TI_Def, nr, v_EncodeCapRequestNR)), car_NR_SRB1_RrcPdu_IND(nr_Cell1, cr_38508_UECapabilityInformationAny(tsc_NR_RRC_TI_Def)), 80, "Step 8", cs_NR_FreqDomainSchedulExplicit(-, v_NR_ResourceAllocation.Lrbs) //DCI with the maximum grant allowed by the carrier bandwidth @sic R5-217815 sic@ ); <<SKIPPED CODE>> }

After Change:

function f_TC_8_1_5_8_1_NR5GC_TestBody() runs on NR5GC_PTC { <<SKIPPED CODE>> //@siclog "Step 7 - 8" siclog@ //The SS transmits a UECapabilityEnquiry message. //The SS starts transmitting a periodic DCI from (80*2^numerology+1+TDL) slots after step 7 to schedule PUSCH. //Check: Does the UE transmit a UECapabilityInformation message within (80*2^numerology+1+TDL+K2) slots after successful completion of step 7? v_EncodeCapRequestNR := bit2oct(encvalue( cs_38508_UECapabilityRequestFilterNR({ cs_NR_BandInformationNR(f_NR_CellInfo_GetBandIndicator(nr_Cell1)) }) )); f_NR5GC_RRC_ProcedureDelay(nr_Cell1, cas_NR_SRB1_RrcPdu_REQ(nr_Cell1, -, cs_38508_UECapabilityEnquiry(tsc_NR_RRC_TI_Def, nr, v_EncodeCapRequestNR)), car_NR_SRB1_RrcPdu_IND(nr_Cell1, cr_38508_UECapabilityInformationAny(tsc_NR_RRC_TI_Def)), 80, "Step 8", cs_NR_FreqDomainSchedulExplicit(-, v_NR_ResourceAllocation.Lrbs), //DCI with the maximum grant allowed by the carrier bandwidth @sic R5-217815 sic@ -, //WA#WI=1237198 12 //WA#WI=1237198 ); <<SKIPPED CODE>> }

## Change to the Function f_NR5GC_RRC_ProcedureDelay

| Function name | f_NR5GC_RRC_ProcedureDelay |
| --- | --- |
| Reason for change | NR_ImcsValue_Type tsc_NR_DefaultGrant_Imcs := 9; is provided as part of the function f_NR_ULGrantConfiguration_Start. But if UE is having bigger UECapabilityInformation which can not be accommodated with NR_ImcsValue_Type tsc_NR_DefaultGrant_Imcs := 9; then SS will end up receiving UECapabilityInformation in segments which will fail the case due to procedure delay condition not met . Hence it is proposed to have the periodic UL grant configurable for receiving UECapabilityInformation message. |
| Summary of change | periodic UL grant configurable for receiving UECapabilityInformation message is made configurable using Imcs value. Imcs value is set as 12 for the current requirement |
| TTCN module | Common\NR5GC\NR5GC_RRCSteps.ttcn |
| MCC160 Comment | Accepted in principle, but instead use ImcsValue=28 to cope with the biggest possible grant. |

Before Change:

function f_NR5GC_RRC_ProcedureDelay(NR_CellId_Type p_NR_CellId, template (value) NR_SRB_COMMON_REQ p_SRB_COMMON_REQ, template (present) NR_SRB_COMMON_IND p_SRB_COMMON_IND, integer p_Nms, charstring p_TestcaseStep, template (value) NR_FreqDomainSchedulExplicit_Type p_FreqDomainSchedul := cs_NR_FreqDomainSchedulExplicitUL_Def, boolean p_NRDC_PduSession := false ) runs on NR5GC_PTC return NR_SRB_COMMON_IND { //@sic R5s230158: added p_NRDC_PduSession for TC_8_2_6_2_2 sic@ <<SKIPPED CODE >> //T3 configure SS with periodic UL Grant at T3 f_NR_ULGrantConfiguration_Start(p_NR_CellId, cs_TimingInfo_NR(v_T3), -, p_FreqDomainSchedul, -, cs_UL_GrantConfig_ContPeriodic(v_GrantPeriodicity)); <<SKIPPED CODE >> }

After Change:

function f_NR5GC_RRC_ProcedureDelay(NR_CellId_Type p_NR_CellId, template (value) NR_SRB_COMMON_REQ p_SRB_COMMON_REQ, template (present) NR_SRB_COMMON_IND p_SRB_COMMON_IND, integer p_Nms, charstring p_TestcaseStep, template (value) NR_FreqDomainSchedulExplicit_Type p_FreqDomainSchedul := cs_NR_FreqDomainSchedulExplicitUL_Def, boolean p_NRDC_PduSession := false, template (value) NR_ImcsValue_Type p_ImcsValue := tsc_NR_DefaultGrant_Imcs ) runs on NR5GC_PTC return NR_SRB_COMMON_IND { << SKIPPED CODE>> //T3 configure SS with periodic UL Grant at T3 f_NR_ULGrantConfiguration_Start(p_NR_CellId, cs_TimingInfo_NR(v_T3), p_ImcsValue, p_FreqDomainSchedul, -, cs_UL_GrantConfig_ContPeriodic(v_GrantPeriodicity)); //WA#WI=1237198 <<SKIPPED CODE >> }

MCC160 proposed implementation

function f_NR5GC_RRC_ProcedureDelay(NR_CellId_Type p_NR_CellId, template (value) NR_SRB_COMMON_REQ p_SRB_COMMON_REQ, template (present) NR_SRB_COMMON_IND p_SRB_COMMON_IND, integer p_Nms, charstring p_TestcaseStep, template (value) NR_FreqDomainSchedulExplicit_Type p_FreqDomainSchedul := cs_NR_FreqDomainSchedulExplicitUL_Def, boolean p_NRDC_PduSession := false ) runs on NR5GC_PTC return NR_SRB_COMMON_IND { << SKIPPED CODE>> //T3 configure SS with periodic UL Grant at T3 f_NR_ULGrantConfiguration_Start(p_NR_CellId, cs_TimingInfo_NR(v_T3), 28, p_FreqDomainSchedul, -, cs_UL_GrantConfig_ContPeriodic(v_GrantPeriodicity)); //WA#WI=1237198 <<SKIPPED CODE >> }