3GPP TSG-RAN5 Meeting #2024-TTCN email	R5s240231

# Online, , 8th Dec 2023 - 31st Dec 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 3386 | rev | - | Current version: | 17.9.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to function fl_EUTRA38_IRAT_InitListOfFrequencies |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2024-03-07 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | - Use of dl_CarrierFreq parameter will not give the correct value for Eutran Cell band greater than 64 in use. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Replaced dl_CarrierFreq parameter with dl_CarrierFreq_r9 which takes care of all bands as per its initialization inside funtion fl_EUTRA_InitFrequencyDef_f1Tof4 and f_EUTRA_InitFrequencyLowMidHigh with dl_CarrierFreq or dl_CarrierFreq_v9e0 based on band is smaller or greater than 64. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may fail the test cases for Eutra band > 64. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | NR5GC IRAT test cases. |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

2	Corrections required	4

2.1	Function fl_EUTRA38_IRAT_InitListOfFrequencies ()	4

# Overview

This document lists all the essential changes needed to correct problems in the iwd-TTCN3-B2022-09_D23wk49 related to the title of this CR.

Contact:	Parikshit Bhise

parikshit.bhise@rohde-schwarz.com

# Corrections required

## Function fl_EUTRA38_IRAT_InitListOfFrequencies ()

| Function name | fl_EUTRA38_IRAT_InitListOfFrequencies |
| --- | --- |
| Reason for change | 1. Use of dl_CarrierFreq parameter will not give the correct value for Eutran Cell band greater than 64 in use. |
| Summary of change | 1. Replaced dl_CarrierFreq parameter with dl_CarrierFreq_r9 which takes care of all bands as per its initialization inside funtion fl_EUTRA_InitFrequencyDef_f1Tof4 and f_EUTRA_InitFrequencyLowMidHigh with dl_CarrierFreq or dl_CarrierFreq_v9e0 based on band is smaller or greater than 64. |
| TTCN module | .. Common\EUTRA38_IRAT_CommonFunctions.ttcn |
| MCC160 Comment | Accepted |

Before Change:

function fl_EUTRA38_IRAT_InitListOfFrequencies () runs on EUTRA_PTC return template (value) EUTRA_NR_CoOrd_SysInfo_Type { var template (value) CoOrd_EUTRASysInfo_List_Type v_CoOrd_EUTRASysInfo_List; var Frequency_fList_Type v_Frequency := f_EUTRA_InitFrequency_f1Tof4(f_EUTRA_CellInfo_GetBand(eutra_Cell1), f_EUTRA_CellInfo_GetDL_ChBandwidth(eutra_Cell1), f_EUTRA_CellInfo_GetUL_ChBandwidth(eutra_Cell1)); var integer v_DL_Bandwidth := f_ConvertDL_BandwidthToInteger(f_EUTRA_CellInfo_GetDL_ChBandwidth(eutra_Cell1)); v_CoOrd_EUTRASysInfo_List[0] := cs_CoOrdEUTRASysInfo(v_Frequency.f1.dl_CarrierFreq, v_DL_Bandwidth); // f1 if (v_Frequency.f2.dl_CarrierFreq != tsc_DummyFrequency) { // f2 defined for the band v_CoOrd_EUTRASysInfo_List[1] := cs_CoOrdEUTRASysInfo(v_Frequency.f2.dl_CarrierFreq, v_DL_Bandwidth); // f2 if (v_Frequency.f3.dl_CarrierFreq != tsc_DummyFrequency) { // f3 defined for the band v_CoOrd_EUTRASysInfo_List[2] := cs_CoOrdEUTRASysInfo(v_Frequency.f3.dl_CarrierFreq, v_DL_Bandwidth); // f3 if (v_Frequency.f4.dl_CarrierFreq != tsc_DummyFrequency) { // f4 defined for the band v_CoOrd_EUTRASysInfo_List[3] := cs_CoOrdEUTRASysInfo(v_Frequency.f4.dl_CarrierFreq, v_DL_Bandwidth); // f4 } } } return cs_EUTRA_NR_CoOrdSysInfo_Eutra(v_CoOrd_EUTRASysInfo_List) }

After Change:

function fl_EUTRA38_IRAT_InitListOfFrequencies () runs on EUTRA_PTC return template (value) EUTRA_NR_CoOrd_SysInfo_Type { var template (value) CoOrd_EUTRASysInfo_List_Type v_CoOrd_EUTRASysInfo_List; var Frequency_fList_Type v_Frequency := f_EUTRA_InitFrequency_f1Tof4(f_EUTRA_CellInfo_GetBand(eutra_Cell1), f_EUTRA_CellInfo_GetDL_ChBandwidth(eutra_Cell1), f_EUTRA_CellInfo_GetUL_ChBandwidth(eutra_Cell1)); var integer v_DL_Bandwidth := f_ConvertDL_BandwidthToInteger(f_EUTRA_CellInfo_GetDL_ChBandwidth(eutra_Cell1)); //WA#WI=1258243 v_CoOrd_EUTRASysInfo_List[0] := cs_CoOrdEUTRASysInfo(v_Frequency.f1.dl_CarrierFreq_r9, v_DL_Bandwidth); // f1 if (v_Frequency.f2.dl_CarrierFreq_r9 != tsc_DummyFrequency) { // f2 defined for the band v_CoOrd_EUTRASysInfo_List[1] := cs_CoOrdEUTRASysInfo(v_Frequency.f2.dl_CarrierFreq_r9, v_DL_Bandwidth); // f2 if (v_Frequency.f3.dl_CarrierFreq_r9 != tsc_DummyFrequency) { // f3 defined for the band v_CoOrd_EUTRASysInfo_List[2] := cs_CoOrdEUTRASysInfo(v_Frequency.f3.dl_CarrierFreq_r9, v_DL_Bandwidth); // f3 if (v_Frequency.f4.dl_CarrierFreq_r9 != tsc_DummyFrequency) { // f4 defined for the band v_CoOrd_EUTRASysInfo_List[3] := cs_CoOrdEUTRASysInfo(v_Frequency.f4.dl_CarrierFreq_r9, v_DL_Bandwidth); // f4 } } } return cs_EUTRA_NR_CoOrdSysInfo_Eutra(v_CoOrd_EUTRASysInfo_List) }