3GPP TSG-RAN5 Meeting #2024-TTCN email	R5s240238

# Online, , 8th Dec 2023 - 31st Dec 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 3388 | rev | - | Current version: | 17.9.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to NR5GC testcase 8.1.6.1.4.2 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI16_Test, NR_SON_MDT-UEConTest |  |  |  |  |  | Date: |  |  | 2024-03-13 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | After Step 14, the UE is in connected state, and hence the timer T3540 is running. This needs to be accommodated while calling the postamble function. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Instead of calling function f_NR_Postamble(nr_Cell1, STATE_CONNECTED_3A, tsc_NR_RbId_SRB1); , called f_NR_Postamble(nr_Cell1, STATE_CONNECTED_3A_T3540, tsc_NR_RbId_SRB1); for postamble. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE will fail this test case. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.1.6.1.4.2.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Table of Contents

Table of Contents	2

1	Overview	2

1	Corrections required	3

1.1	Change 1	3

- Overview

This document lists all the essential changes needed to correct problems in the iwd-TTCN3-B2022-09_D23wk49 related to the title of this CR.

Contact:	Parikshit Bhise

Parikshit.Bhise@rohde-schwarz.com

- Corrections required

  - Change 1

| Function name | f_TC_8_1_6_1_4_2_NR5GC() |
| --- | --- |
| Reason for change | After Step 14, the UE is in connected state, and hence the timer T3540 is running. This needs to be accommodated while calling the postamble function. |
| Summary of change | Instead of calling function f_NR_Postamble(nr_Cell1, STATE_CONNECTED_3A, tsc_NR_RbId_SRB1); , called f_NR_Postamble(nr_Cell1, STATE_CONNECTED_3A_T3540, tsc_NR_RbId_SRB1); for postamble. |
| TTCN module | RRC_MDT_IntraNR_CEF_NR5GC.ttcn |
| MCC160 Comment | Accepted |

Before Change:

function f_TC_8_1_6_1_4_2_NR5GC() runs on NR5GC_PTC { //Connection Establishment Failure / Logging and reporting / RRC Resume f_NR5GC_Init(NR_1); // @sic R5-217920, R5s211598 sic@ //Create and configure cell f_NR_CellConfig_Def(nr_Cell1); f_NR5GC_Preamble(nr_Cell1, STATE_INACTIVE_2A, -, false); // @sic R5-217920, R5s211598 sic@ f_NR_TestBody_Set(true); fl_TC_8_1_6_1_4_2_TestBody(); f_NR_TestBody_Set(false); f_NR_Postamble(nr_Cell1, STATE_CONNECTED_3A, tsc_NR_RbId_SRB1); // @sic R5-217920, R5s211598 sic@ }

After Change:

function f_TC_8_1_6_1_4_2_NR5GC() runs on NR5GC_PTC { //Connection Establishment Failure / Logging and reporting / RRC Resume f_NR5GC_Init(NR_1); // @sic R5-217920, R5s211598 sic@ //Create and configure cell f_NR_CellConfig_Def(nr_Cell1); f_NR5GC_Preamble(nr_Cell1, STATE_INACTIVE_2A, -, false); // @sic R5-217920, R5s211598 sic@ f_NR_TestBody_Set(true); fl_TC_8_1_6_1_4_2_TestBody(); f_NR_TestBody_Set(false); f_NR_Postamble(nr_Cell1, STATE_CONNECTED_3A_T3540, tsc_NR_RbId_SRB1); // @sic R5-217920, R5s211598 sic@ );