3GPP TSG-RAN5 Meeting #2024-TTCN email	R5s240243

# Online, , 8th Dec 2023 - 31st Dec 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 3390 | rev | - | Current version: | 17.9.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to NR5GC testcase 7.1.1.9.1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2024-03-13 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | TTCN will send a Status PDU by calling function f_NR_TxSTATUS_PDU if receive UE loop back step1 data in step4Aa. When Initialization of RLC variables, use constant value of AM_SN_Size =18 which not consider the suitation of Redcap device that should use 12 bit SN. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add condition to decide what SN to use before step4Aa. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE can fail the test cases |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.1.1.9.1.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	3

1	Overview	3

2	Corrections required	3

2.1 Change 1	3

# Overview

This document lists all the essential changes needed to correct problems in the iwd-TTCN3-B2022-09_D23wk49 related to the title of this CR.

Contact: 	Parikshit Bhise

Parikshit.bhise@rohde-schwarz.com

# Corrections required

## 2.1 Change 1

| Function name | function f_TC_7_1_1_9_1_Step4Aato9A |
| --- | --- |
| Reason for change | TTCN will send a Status PDU by calling function f_NR_TxSTATUS_PDU if receive UE loop back step1 data in step4Aa. When Initialization of RLC variables, use constant value of AM_SN_Size =18 which not consider the suitation of Redcap device that should use 12 bit SN. |
| Summary of change | Add condition to decide what SN to use before step4Aa. |
| TTCN module | MAC_NR5GC.ttcn |
| MCC160 Comments | Accepted, see proposed implementation below. |

Before Change:

| function fl_TC_7_1_1_9_1_NR_TestBody(DRB_Identity p_NR_DRB_Id, template (value) NR_RadioBearerList_Type p_NR_RadioBearerList) runs on NR5GC_PTC { //@sic R5-206320 R5-213132 sic@ var B6_Type v_LCID := int2bit( f_NR_LogicalChannelId_DRB(p_NR_DRB_Id),6); var template (value) NR_RLC_BearerToAddModList_Type v_RLC_BearerConfigList; var template (value) NR_RLC_BearerToAddModList_Type v_NR_RLC_BearerToAddModList; //@sic R5s201457 sic@ var RLC_Config.am.ul_AM_RLC.t_PollRetransmit v_T_PollRetransmit := f_NR_UL_AM_T_PollRetransmit(f_NR_CellInfo_GetIsFR1(nr_Cell1)); var RLC_Config.am.dl_AM_RLC.t_Reassembly v_T_Reassembly:= f_NR_DL_A <Code Skipped> // @siclog "Step 4a - Void" WA#WI=1221582 //Initialization of RLC variables v_RLC_Rec := f_NR_InitRLC_Record(); v_RLC_Rec.AM_RX_Next := 1; // @siclog "Step 4Aa" siclog@ //@sic R5-232641 R5s230370 sic@ t_Watchdog.start(1.0); alt { [] DRB.receive(car_NR_DRB_COMMON_IND_RLC_PDUList(nr_Cell1, p_NR_DRB_Id, cr_TimingInfo_Any, ?)) { f_NR_TxSTATUS_PDU(v_RLC_Rec, p_NR_DRB_Id); // @siclog "UE has looped back step1 data" siclog@ v_IsDataRecvd := true; t_Watchdog.stop; } [] t_Watchdog.timeout { } } |  |
| --- | --- |

After Change:

function fl_TC_7_1_1_9_1_NR_TestBody(DRB_Identity p_NR_DRB_Id, template (value) NR_RadioBearerList_Type p_NR_RadioBearerList) runs on NR5GC_PTC { //@sic R5-206320 R5-213132 sic@ var B6_Type v_LCID := int2bit( f_NR_LogicalChannelId_DRB(p_NR_DRB_Id),6); var template (value) NR_RLC_BearerToAddModList_Type v_RLC_BearerConfigList; var template (value) NR_RLC_BearerToAddModList_Type v_NR_RLC_BearerToAddModList; //@sic R5s201457 sic@ var RLC_Config.am.ul_AM_RLC.t_PollRetransmit v_T_PollRetransmit := f_NR_UL_AM_T_PollRetransmit(f_NR_CellInfo_GetIsFR1(nr_Cell1)); var RLC_Config.am.dl_AM_RLC.t_Reassembly v_T_Reassembly:= f_NR_DL_A <Code Skipped> // @siclog "Step 4a - Void" WA#WI=1221582 //Initialization of RLC variables v_RLC_Rec := f_NR_InitRLC_Record(); v_RLC_Rec.AM_RX_Next := 1; if(f_NR_38508_SetSNFieldLengthAM() == size12){ v_RLC_Rec.AM_SN_Size :=12; }else{ v_RLC_Rec.AM_SN_Size :=18; } // @siclog "Step 4Aa" siclog@ //@sic R5-232641 R5s230370 sic@ t_Watchdog.start(1.0); alt { [] DRB.receive(car_NR_DRB_COMMON_IND_RLC_PDUList(nr_Cell1, p_NR_DRB_Id, cr_TimingInfo_Any, ?)) { f_NR_TxSTATUS_PDU(v_RLC_Rec, p_NR_DRB_Id); // @siclog "UE has looped back step1 data" siclog@ v_IsDataRecvd := true; t_Watchdog.stop; } [] t_Watchdog.timeout { } }

## MCC160 proposed implementation

function f_TC_7_1_1_9_1_Step4Aato9A ( DRB_Identity p_NR_DRB_Id,

B6_Type p_LCID,

template (value) NR_RadioBearerList_Type p_NR_RadioBearerList,

boolean p_StopGrant:= true

) runs on NR_BASE_PTC return boolean

{ //@sic R5-206320 sic@

//@sic R5s230904: renamed function f_TC_7_1_1_9_1_Step5to9A -> f_TC_7_1_1_9_1_Step4Aato9A sic@

var NR_MAC_SDU_Type v_EncodedRlcPdu1;

var NR_UplinkBWP_Type v_NR_UplinkBWP := f_NR_CellInfo_GetUplinkBWP(nr_Cell1, tsc_NR_BWP_Id);

var NR_ResourceAllocation_Type v_NR_ResourceAllocation;

var default v_IgnoreNRSR:= null; //@sic R5-204418 sic@

var template (value) NR_HarqProcessConfig_Type v_NR_HarqProcessConfig_DL := { SpecificSubset := {0,2,3,4,6,7 }};//@sic R5s211710 sic@

var boolean v_IsDataRecvd := false;

var NR_RLC_SS_State_Type v_RLC_Rec;

timer t_Watchdog;

// Initialization of RLC variables

v_RLC_Rec := f_NR_InitRLC_Record();

if (pc_supportOfRedCap_r17) { v_RLC_Rec.AM_SN_Size := 12; } // @sic R5s240243 sic@

v_RLC_Rec.AM_RX_Next := 1;

// @siclog "Step 4Aa" siclog@

//@sic R5-232641 R5s230370 sic@

t_Watchdog.start(1.0);

alt {

[] DRB.receive(car_NR_DRB_COMMON_IND_RLC_PDUList(nr_Cell1, p_NR_DRB_Id,  cr_TimingInfo_Any, ?))

{

f_NR_TxSTATUS_PDU(v_RLC_Rec, p_NR_DRB_Id);

// @siclog "UE has looped back step1 data" siclog@

v_IsDataRecvd := true;

t_Watchdog.stop;

}

[] t_Watchdog.timeout

{ }

}

…

return v_IsDataRecvd;

}