3GPP TSG-RAN5 Meeting #2024-TTCN email	R5s240257

# Online, , 8th Dec 2023 - 31st Dec 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.523-3 | CR | 4799 | rev | - | Current version: | 18.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to NBIOT testcase 22.5.6 (for NB-NTN UE) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI13_Test, NB_IOT-UEConTest |  |  |  |  |  | Date: |  |  | 2024-03-15 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In the current TTCN implementation of this testcase, timer of 0.5s is started after Step 44 so that the ATTACH ACCEPT is successfully transmitted before changing the cell power level. This value works for normal NBIOT device, but for NB-NTN device, this is not enough because of the Koffset. As a result, a NB-NTN device my not get the Attach ACCEPT at Step 44 successully. This needs to be addressed. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Updated the TTCN to accommodate both normal NBIOT and NTN-NBIOT device behavior. Please see below. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Conformant UE may failed the test case. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 22.5.6 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Table of Contents

Table of Contents	2

1	Overview	3

2	Corrections required	3

2.1	Change 1	3

# Overview

This document lists all the changes needed to correct issues in the ATS iwd-TTCN3-B2022-09_D23wk49 related to the title of this CR.

Contact:  Bhise Parikshit
parikshit.bhise@rohde-schwarz.com

# Corrections required

## Change 1

| Function name | fl_TC_22_5_6_Body() |
| --- | --- |
| Reason for change | In the current TTCN implementation of this testcase, timer of 0.5s is started after Step 44 so that the ATTACH ACCEPT is successfully transmitted before changing the cell power level. This value works for normal NBIOT device, but for NB-NTN device, this is not enough because of the Koffset. As a result, a NB-NTN device my not get the Attach ACCEPT at Step 44 successully. This needs to be addressed. |
| Summary of change | Based on whether pc_NB_ntn_Connectivity_EPC or pc_NB_ntn_only_Connectivity_EPC is set to TRUE, started the timer for 0.5s or 1.5s Please see below. |
| TTCN module | NBIOT_NAS_Attach.ttcn |
| MCC160 Comments | Accepted, but will be implemented slightly different to ensure the new timer applies only when the TC is run the NTN environment. |

function fl_TC_22_5_6_Body() runs on NBIOT_PTC { ……………………… //@siclog "Step 44" siclog@ fl_TC_22_5_6_AttachAccept(nbiot_Cell51, v_NAS_Ind, cs_TAIListNonConsecutive_lv(f_Asn2Nas_PlmnId (v_Guti_Params_Cell51.PLMN_Identity), {bit2oct(f_NBIOT_CellInfo_GetTAC (nbiot_Cell50))}), // @sic R5s170770 sic@ f_GutiParameters2MobileIdentity(tsc_IEI_Guti, v_Guti1_Params)); // @sic R5s170770 sic@ f_Delay(0.5); // @sic R5s180246 sic@ //@siclog "Step 45" siclog@ //SS adjusts the cell power levels according to row T4 in table 22.5.6.3-1. v_CellPowerList := { // @sic R5-174464 sic@ cs_NB_CellPower(nbiot_Cell50, tsc_ServingCellRS_EPRE), cs_NB_CellPower(nbiot_Cell51, tsc_NonSuitableNBIOTCellRS_EPRE) }; f_NBIOT_SetCellPowerList(v_CellPowerList); //@siclog "Step 46- 51" siclog@ …………………… }

After Change:

function fl_TC_22_5_6_Body() runs on NBIOT_PTC { ……………………… //@siclog "Step 44" siclog@ fl_TC_22_5_6_AttachAccept(nbiot_Cell51, v_NAS_Ind, cs_TAIListNonConsecutive_lv(f_Asn2Nas_PlmnId (v_Guti_Params_Cell51.PLMN_Identity), {bit2oct(f_NBIOT_CellInfo_GetTAC (nbiot_Cell50))}), // @sic R5s170770 sic@ f_GutiParameters2MobileIdentity(tsc_IEI_Guti, v_Guti1_Params)); // @sic R5s170770 sic@ if (pc_NB_ntn_Connectivity_EPC or pc_NB_ntn_only_Connectivity_EPC) { // WI=1263631 f_Delay(1.5); } else { f_Delay(0.5); // @sic R5s180246 sic@ } //@siclog "Step 45" siclog@ //SS adjusts the cell power levels according to row T4 in table 22.5.6.3-1. v_CellPowerList := { // @sic R5-174464 sic@ cs_NB_CellPower(nbiot_Cell50, tsc_ServingCellRS_EPRE), cs_NB_CellPower(nbiot_Cell51, tsc_NonSuitableNBIOTCellRS_EPRE) }; f_NBIOT_SetCellPowerList(v_CellPowerList); //@siclog "Step 46- 51" siclog@ …………………………………. }

MCC160 alternative implemenation:

function fl_TC_22_5_6_Body() runs on NBIOT_PTC { ……………………… //@siclog "Step 44" siclog@ fl_TC_22_5_6_AttachAccept(nbiot_Cell51, v_NAS_Ind, cs_TAIListNonConsecutive_lv(f_Asn2Nas_PlmnId (v_Guti_Params_Cell51.PLMN_Identity), {bit2oct(f_NBIOT_CellInfo_GetTAC (nbiot_Cell50))}), // @sic R5s170770 sic@ f_GutiParameters2MobileIdentity(tsc_IEI_Guti, v_Guti1_Params)); // @sic R5s170770 sic@ if (f_NBIOT_MobileInfo_GetNTNSatelliteType() != ntn_None) { // WI=1263631 f_Delay(1.5); } else { f_Delay(0.5); // @sic R5s180246 sic@ } //@siclog "Step 45" siclog@ //SS adjusts the cell power levels according to row T4 in table 22.5.6.3-1. v_CellPowerList := { // @sic R5-174464 sic@ cs_NB_CellPower(nbiot_Cell50, tsc_ServingCellRS_EPRE), cs_NB_CellPower(nbiot_Cell51, tsc_NonSuitableNBIOTCellRS_EPRE) }; f_NBIOT_SetCellPowerList(v_CellPowerList); //@siclog "Step 46- 51" siclog@ …………………………………. }