3GPP TSG-RAN5 Meeting #2024-TTCN email	R5s240268

# Online, , 8th Dec 2023 - 31st Dec 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 3403 | rev | - | Current version: | 17.9.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to NR MAC test case 7.1.1.6.1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2024-03-26 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | During testing on 24wk12 IWD for TC 7.1.1.6.1 with the MCC160 proposed implementation to test TP4, it is found that the TC is failing both for ENDC and NR5GC. Specifically, the MCC160 implementation does not follow the SPS pattern and multiple chipsets are sending a DTX at Step 11 |  |  |  |  |  |  |  |  |
| Summary of change: |  | To properly test TP4, the following changes are required as was originally highlighted in TTCN CR R5s230810: - Introduce Steps 11a1 – 11a3: If pc_dl_SchedulingOffset_PDSCH_TypeA is TRUE, adjust the timing of v_TimingDL_CRNTI to make PDSCH coincide with the next SPS transmission slot and satisfy TP4. Since k1 is being changed to 8, the timing of v_TimingUL_5_CRNTI is also adjusted to be the same as the SPS HARQ slot. - Introduce Steps 11b1 – 11b2: If pc_dl_SchedulingOffset_PDSCH_TypeA is FALSE, then a normal DL SPS transmission is made instead of a C-RNTI transmission. This is done so that the timing of succeeding steps do not need to be modified. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may fail the test case. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.1.1.6.1.ENDC, 7.1.1.6.1.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	3

1	Overview	4

2	Corrections required	4

2.1	Change 1	4

# Overview

This document lists all the changes needed to correct issues in the ATS iwd-TTCN3-B2022-09_D24wk12 related to the title of this CR.

Contact:  Bhise Parikshit
parikshit.bhise@rohde-schwarz.com

# Corrections required

## Change 1

| Function name | function f_TC_7_1_1_6_1_NR_TestBody1 |
| --- | --- |
| Reason for change | During testing on 24wk12 IWD for TC 7.1.1.6.1 with the MCC160 proposed implementation to test TP4, it is found that the TC is failing both for ENDC and NR5GC. Specifically, the MCC160 implementation does not follow the SPS pattern and multiple chipsets are sending a DTX at Step 11 |
| Summary of change | To properly test TP4, the following changes are required as was originally highlighted in TTCN CR R5s230810 Change 4: - Introduce Steps 11a1 – 11a3: If pc_dl_SchedulingOffset_PDSCH_TypeA is TRUE, adjust the timing of v_TimingDL_CRNTI to make PDSCH coincide with the next SPS transmission slot and satisfy TP4. Since k1 is being changed to 8, the timing of v_TimingUL_5_CRNTI is also adjusted to be the same as the SPS HARQ slot. - Introduce Steps 11b1 – 11b2: If pc_dl_SchedulingOffset_PDSCH_TypeA is FALSE, then a normal DL SPS transmission is made instead of a C-RNTI transmission. This is done so that the timing of succeeding steps do not need to be modified. |
| TTCN module | MAC_SPS_TC_Common_NR.ttcn |
| MCC160 Comments | Accepted. |

Before Change:

function f_TC_7_1_1_6_1_NR_TestBody1(DRB_Identity p_NR_DRB_Id) runs on NR_BASE_PTC return SubFrameTiming_Type { const integer const_SPS_Period :=80; // period in milli seconds const integer const_HARQ_Id := 1; //arbitrarily selected in steps 17 and 20 var NR_MAC_SDU_Type v_EncodedRlcPdu1; var template (value) NR_MAC_SDU_SubPDU_List_Type v_SDU_SubPDUListDL; var template (value) NR_MAC_PDUList_Type v_MAC_PDUListDL; var SubFrameTiming_Type v_TimingDL; var SubFrameTiming_Type v_TimingDL_CRNTI; var SubFrameTiming_Type v_TimingUL_1; var SubFrameTiming_Type v_TimingUL_2; var SubFrameTiming_Type v_TimingUL_3; var SubFrameTiming_Type v_TimingUL_4; var SubFrameTiming_Type v_TimingUL_5_CRNTI; var SubFrameTiming_Type v_TimingUL_6; var SubFrameTiming_Type v_TimingUL_7; var SubFrameTiming_Type v_TimingUL_8; var SubFrameTiming_Type v_TimingUL_9; var SubFrameTiming_Type v_TimingUL_10; var SubFrameTiming_Type v_TimingUL_11; var integer v_Step11_Time; <<SKIPPED CODE>> //@siclog "Step 11a1, 11a2 11b1" siclog@ //C-RNTI grant auto transferred by SS //@sic R5s221197 R5s230088 R5-237339 sic@ if (pc_dl_SchedulingOffset_PDSCH_TypeA) { v_Step11_Time :=const_SPS_Period-4; //Set K1=8 v_DciDlInfo.ResoureAssignment.TimeDomain.Index := int2bit(1, 4); //@sic R5s240155 sic@ f_NR_SS_CommonCellConfig(nr_Cell1, cas_NR_DciDlInfo_REQ(nr_Cell1, cs_TimingInfo_Now, v_DciDlInfo)); } else { v_Step11_Time :=const_SPS_Period; } v_TimingDL_CRNTI := f_SubFrameTiming_AddMilliSeconds(v_TimingDL,v_Step11_Time); v_TimingUL_5_CRNTI := f_SubFrameTiming_AddMilliSeconds(v_TimingUL_4,const_SPS_Period); <<SKIPPED CODE>> // REMOVED PDCP SDU =36B +3B PDCP Header+ 1B RLC UM Header + 2B MAC Header =42B=336 bits (an allowed TB size as per 38.523-3 Annex B) v_EncodedRlcPdu1 := f_NR_RLC_UMD_FullPDU_Encvalue(f_NR_PDCP_PDU_Encvalue(f_NR_ReturnDefaultPDCP_PDU (3, crs_NR_PDCP_SDU_36B, 1)));//@sic R5-227518 sic@ v_SDU_SubPDUListDL := { cs_NR_MAC_SDU_SubPDU_LI8(tsc_LCID_SCG_Bearer, v_EncodedRlcPdu1) }; v_MAC_PDUListDL := { cs_NR_DL_MAC_PDU_SDU_SubPDU_Padding(v_SDU_SubPDUListDL) }; DRB.send(cas_NR_DRB_COMMON_REQ_MAC_PDUList(nr_Cell1, p_NR_DRB_Id, cs_TimingInfo_NR( v_TimingDL_CRNTI), v_MAC_PDUListDL)); //@siclog "Step 12-13 void " siclog@ //@siclog "Step 14 " siclog@ //@sic R5-237339 sic@ v_TimingDL := f_SubFrameTiming_AddMilliSeconds(v_TimingDL,2*const_SPS_Period); v_TimingUL_6 := f_SubFrameTiming_AddMilliSeconds(v_TimingUL_4,2*const_SPS_Period); // PDCP SDU =38B +3B PDCP Header+ 1B RLC UM Header + 2B MAC Header =44B=352 bits (an allowed TB size as per 38.523-3 Annex B) v_EncodedRlcPdu1 := f_NR_RLC_UMD_FullPDU_Encvalue(f_NR_PDCP_PDU_Encvalue(f_NR_ReturnDefaultPDCP_PDU (4, crs_NR_PDCP_SDU_38B, 1)));//@sic R5-227518 sic@ v_SDU_SubPDUListDL := { cs_NR_MAC_SDU_SubPDU_LI8(tsc_LCID_SCG_Bearer, v_EncodedRlcPdu1) }; v_MAC_PDUListDL := { cs_NR_DL_MAC_PDU_SDU_SubPDU_Padding(v_SDU_SubPDUListDL) }; DRB.send(cads_NR_DRB_COMMON_REQ_MAC_PDUList_NoCRNTI_Grant(nr_Cell1, p_NR_DRB_Id, cs_TimingInfo_NR( v_TimingDL), v_MAC_PDUListDL)); <<SKIPPED CODE>>

After Change:

function f_TC_7_1_1_6_1_NR_TestBody1(DRB_Identity p_NR_DRB_Id) runs on NR_BASE_PTC return SubFrameTiming_Type { const integer const_SPS_Period :=80; // period in milli seconds const integer const_HARQ_Id := 1; //arbitrarily selected in steps 17 and 20 var NR_MAC_SDU_Type v_EncodedRlcPdu1; var template (value) NR_MAC_SDU_SubPDU_List_Type v_SDU_SubPDUListDL; var template (value) NR_MAC_PDUList_Type v_MAC_PDUListDL; var SubFrameTiming_Type v_TimingDL; var SubFrameTiming_Type v_TimingDL_CRNTI; var SubFrameTiming_Type v_TimingDL_Step11b; //WA# var SubFrameTiming_Type v_TimingUL_1; var SubFrameTiming_Type v_TimingUL_2; var SubFrameTiming_Type v_TimingUL_3; var SubFrameTiming_Type v_TimingUL_4; var SubFrameTiming_Type v_TimingUL_5_CRNTI; var SubFrameTiming_Type v_TimingUL_6; var SubFrameTiming_Type v_TimingUL_7; var SubFrameTiming_Type v_TimingUL_8; var SubFrameTiming_Type v_TimingUL_9; var SubFrameTiming_Type v_TimingUL_10; var SubFrameTiming_Type v_TimingUL_11; var integer v_Step11_Time; <<SKIPPED CODE>> //@siclog "Step 11a1, 11a2 11b1" siclog@ //C-RNTI grant auto transferred by SS //@sic R5s221197 R5s230088 R5-237339 sic@ //WA# if (pc_dl_SchedulingOffset_PDSCH_TypeA) { //@siclog "Step 11a1-11a2" siclog@ select(v_SCS) { case(kHz15) { v_TimingDL_CRNTI := f_SubFrameTiming_AddMilliSeconds(v_TimingDL,const_SPS_Period/2 - 4); // C-RNTI ActTime with k0=4 -- k1 needs to be adjusted to 8 v_TimingUL_5_CRNTI := f_SubFrameTiming_AddMilliSeconds(v_TimingUL_4,const_SPS_Period/2); // Receive C-RNTI HARQ with same timing as SPS HARQ //v_TimingUL_5_CRNTI.Symbol := valueof(cs_SymbolTimingInfo_Any); // REMOVED //v_TimingUL_5_CRNTI.Subframe.Number := v_TimingDL_CRNTI.Subframe.Number + 7; //FDD K1 =7 // REMOVED } case(kHz30) { v_TimingDL_CRNTI := f_SubFrameTiming_AddMilliSeconds(v_TimingDL,const_SPS_Period/2 - 2); // C-RNTI ActTime with k0=4 -- k1 needs to be adjusted to 8 v_TimingUL_5_CRNTI := f_SubFrameTiming_AddMilliSeconds(v_TimingUL_4,const_SPS_Period/2); // Receive C-RNTI HARQ with same timing as SPS HARQ //v_TimingUL_5_CRNTI.Symbol := valueof(cs_SymbolTimingInfo_Any); // REMOVED //v_TimingUL_5_CRNTI.Subframe.Number := v_TimingDL_CRNTI.Subframe.Number + 2; //TDD K1 =5 // REMOVED //v_TimingUL_5_CRNTI.Slot.SlotOffset.Numerology1 := 1; // REMOVED } case(kHz120) { v_TimingDL_CRNTI := f_SubFrameTiming_AddMilliSeconds(v_TimingDL,const_SPS_Period/2 - 1); // C-RNTI ActTime with k0=4 -- k1 needs to be adjusted to 8 v_TimingDL_CRNTI.Slot.SlotOffset.Numerology3 := 5; v_TimingUL_5_CRNTI := f_SubFrameTiming_AddMilliSeconds(v_TimingUL_4,const_SPS_Period/2); // Receive C-RNTI HARQ with same timing as SPS HARQ //v_TimingUL_5_CRNTI.Symbol := valueof(cs_SymbolTimingInfo_Any); // REMOVED //v_TimingUL_5_CRNTI.Subframe.Number := v_TimingDL_CRNTI.Subframe.Number + 1; //FR2 K1 =8 // REMOVED } case else { FatalError(__FILE__, __LINE__, "Invalid SubCarrierSapcing"); } } //Set K1=8 v_DciDlInfo.ResoureAssignment.TimeDomain.Index := int2bit(1, 4); //@sic R5s240155 sic@ f_NR_SS_CommonCellConfig(nr_Cell1, cas_NR_DciDlInfo_REQ(nr_Cell1, cs_TimingInfo_Now, v_DciDlInfo)); // PDCP SDU =36B +3B PDCP Header+ 1B RLC UM Header + 2B MAC Header =42B=336 bits (an allowed TB size as per 38.523-3 Annex B) v_EncodedRlcPdu1 := f_NR_RLC_UMD_FullPDU_Encvalue(f_NR_PDCP_PDU_Encvalue(f_NR_ReturnDefaultPDCP_PDU (3, crs_NR_PDCP_SDU_36B, 1)));//@sic R5-227518 sic@ v_SDU_SubPDUListDL := { cs_NR_MAC_SDU_SubPDU_LI8(tsc_LCID_SCG_Bearer, v_EncodedRlcPdu1) }; v_MAC_PDUListDL := { cs_NR_DL_MAC_PDU_SDU_SubPDU_Padding(v_SDU_SubPDUListDL) }; DRB.send(cas_NR_DRB_COMMON_REQ_MAC_PDUList(nr_Cell1, p_NR_DRB_Id, cs_TimingInfo_NR( v_TimingDL_CRNTI), v_MAC_PDUListDL)); } else { //@siclog "Step 11b1" siclog@ v_TimingDL_Step11b := f_SubFrameTiming_AddMilliSeconds(v_TimingDL,const_SPS_Period/2); // Normal SPS transmission v_TimingUL_5_CRNTI := f_SubFrameTiming_AddMilliSeconds(v_TimingUL_4,const_SPS_Period/2); // Receive HARQ with same timing as SPS HARQ // PDCP SDU =36B +3B PDCP Header+ 1B RLC UM Header + 2B MAC Header =42B=336 bits (an allowed TB size as per 38.523-3 Annex B) v_EncodedRlcPdu1 := f_NR_RLC_UMD_FullPDU_Encvalue(f_NR_PDCP_PDU_Encvalue(f_NR_ReturnDefaultPDCP_PDU (3, crs_NR_PDCP_SDU_36B, 1)));//@sic R5-227518 sic@ v_SDU_SubPDUListDL := { cs_NR_MAC_SDU_SubPDU_LI8(tsc_LCID_SCG_Bearer, v_EncodedRlcPdu1) }; v_MAC_PDUListDL := { cs_NR_DL_MAC_PDU_SDU_SubPDU_Padding(v_SDU_SubPDUListDL) }; DRB.send(cads_NR_DRB_COMMON_REQ_MAC_PDUList_NoCRNTI_Grant(nr_Cell1, p_NR_DRB_Id, cs_TimingInfo_NR( v_TimingDL_Step11b), v_MAC_PDUListDL)); } <<SKIPPED CODE>> //@siclog "Step 12-13 void " siclog@ //@siclog "Step 14 " siclog@ //@sic R5-237339 sic@ v_TimingDL := f_SubFrameTiming_AddMilliSeconds(v_TimingDL,const_SPS_Period+const_SPS_Period/2); //WA# v_TimingUL_6 := f_SubFrameTiming_AddMilliSeconds(v_TimingUL_4,const_SPS_Period+const_SPS_Period/2); //WA# // PDCP SDU =38B +3B PDCP Header+ 1B RLC UM Header + 2B MAC Header =44B=352 bits (an allowed TB size as per 38.523-3 Annex B) v_EncodedRlcPdu1 := f_NR_RLC_UMD_FullPDU_Encvalue(f_NR_PDCP_PDU_Encvalue(f_NR_ReturnDefaultPDCP_PDU (4, crs_NR_PDCP_SDU_38B, 1)));//@sic R5-227518 sic@ v_SDU_SubPDUListDL := { cs_NR_MAC_SDU_SubPDU_LI8(tsc_LCID_SCG_Bearer, v_EncodedRlcPdu1) }; v_MAC_PDUListDL := { cs_NR_DL_MAC_PDU_SDU_SubPDU_Padding(v_SDU_SubPDUListDL) }; DRB.send(cads_NR_DRB_COMMON_REQ_MAC_PDUList_NoCRNTI_Grant(nr_Cell1, p_NR_DRB_Id, cs_TimingInfo_NR( v_TimingDL), v_MAC_PDUListDL)); <<SKIPPED CODE>>

## Additional changes Based in Anritsu Feedback.

| Function name | function f_TC_7_1_1_6_1_NR_TestBody1 |
| --- | --- |
| Reason for change | [Anritsu] Step 11a1: To better align with the prose and ensure the SS is configured at the right time, the update of K0 and K1 for Step 11a1 should have the same ActTime as the C-RNTI DL Grant. [Anritsu] Step 26-27: When undoing the k0 and k1 update, there is no need to wait for 100ms after Step 26-27 MAC PDU is sent. To ensure that k0 and k1 is reset to default promptly after Step 26-27, it is proposed to reduce the current time from 100ms to 50ms after v_TimingDL. |
| Summary of change | - [Anritsu] Change Step 11a1 timing from ActivateNow to v_TimingDL_CRNTI - [Anritsu] Reduce the wait in the timing of rest to default DCI configuration after Step 26-27 |
| TTCN module | MAC_SPS_TC_Common_NR.ttcn |

Before Change:

function f_TC_7_1_1_6_1_NR_TestBody1(DRB_Identity p_NR_DRB_Id) runs on NR_BASE_PTC return SubFrameTiming_Type { const integer const_SPS_Period :=80; // period in milli seconds const integer const_HARQ_Id := 1; //arbitrarily selected in steps 17 and 20 var NR_MAC_SDU_Type v_EncodedRlcPdu1; var template (value) NR_MAC_SDU_SubPDU_List_Type v_SDU_SubPDUListDL; var template (value) NR_MAC_PDUList_Type v_MAC_PDUListDL; var SubFrameTiming_Type v_TimingDL; var SubFrameTiming_Type v_TimingDL_CRNTI; var SubFrameTiming_Type v_TimingUL_1; var SubFrameTiming_Type v_TimingUL_2; var SubFrameTiming_Type v_TimingUL_3; var SubFrameTiming_Type v_TimingUL_4; var SubFrameTiming_Type v_TimingUL_5_CRNTI; var SubFrameTiming_Type v_TimingUL_6; var SubFrameTiming_Type v_TimingUL_7; var SubFrameTiming_Type v_TimingUL_8; var SubFrameTiming_Type v_TimingUL_9; var SubFrameTiming_Type v_TimingUL_10; var SubFrameTiming_Type v_TimingUL_11; var integer v_Step11_Time; <<SKIPPED CODE>> //@siclog "Step 11a1, 11a2 11b1" siclog@ //C-RNTI grant auto transferred by SS //@sic R5s221197 R5s230088 R5-237339 sic@ if (pc_dl_SchedulingOffset_PDSCH_TypeA) { v_Step11_Time :=const_SPS_Period-4; //Set K1=8 v_DciDlInfo.ResoureAssignment.TimeDomain.Index := int2bit(1, 4); //@sic R5s240155 sic@ f_NR_SS_CommonCellConfig(nr_Cell1, cas_NR_DciDlInfo_REQ(nr_Cell1, cs_TimingInfo_Now, v_DciDlInfo)); } else { v_Step11_Time :=const_SPS_Period; } v_TimingDL_CRNTI := f_SubFrameTiming_AddMilliSeconds(v_TimingDL,v_Step11_Time); v_TimingUL_5_CRNTI := f_SubFrameTiming_AddMilliSeconds(v_TimingUL_4,const_SPS_Period); <<SKIPPED CODE>> // REMOVED PDCP SDU =36B +3B PDCP Header+ 1B RLC UM Header + 2B MAC Header =42B=336 bits (an allowed TB size as per 38.523-3 Annex B) v_EncodedRlcPdu1 := f_NR_RLC_UMD_FullPDU_Encvalue(f_NR_PDCP_PDU_Encvalue(f_NR_ReturnDefaultPDCP_PDU (3, crs_NR_PDCP_SDU_36B, 1)));//@sic R5-227518 sic@ v_SDU_SubPDUListDL := { cs_NR_MAC_SDU_SubPDU_LI8(tsc_LCID_SCG_Bearer, v_EncodedRlcPdu1) }; v_MAC_PDUListDL := { cs_NR_DL_MAC_PDU_SDU_SubPDU_Padding(v_SDU_SubPDUListDL) }; DRB.send(cas_NR_DRB_COMMON_REQ_MAC_PDUList(nr_Cell1, p_NR_DRB_Id, cs_TimingInfo_NR( v_TimingDL_CRNTI), v_MAC_PDUListDL)); //@siclog "Step 12-13 void " siclog@ //@siclog "Step 14 " siclog@ //@sic R5-237339 sic@ v_TimingDL := f_SubFrameTiming_AddMilliSeconds(v_TimingDL,2*const_SPS_Period); v_TimingUL_6 := f_SubFrameTiming_AddMilliSeconds(v_TimingUL_4,2*const_SPS_Period); // PDCP SDU =38B +3B PDCP Header+ 1B RLC UM Header + 2B MAC Header =44B=352 bits (an allowed TB size as per 38.523-3 Annex B) v_EncodedRlcPdu1 := f_NR_RLC_UMD_FullPDU_Encvalue(f_NR_PDCP_PDU_Encvalue(f_NR_ReturnDefaultPDCP_PDU (4, crs_NR_PDCP_SDU_38B, 1)));//@sic R5-227518 sic@ v_SDU_SubPDUListDL := { cs_NR_MAC_SDU_SubPDU_LI8(tsc_LCID_SCG_Bearer, v_EncodedRlcPdu1) }; v_MAC_PDUListDL := { cs_NR_DL_MAC_PDU_SDU_SubPDU_Padding(v_SDU_SubPDUListDL) }; DRB.send(cads_NR_DRB_COMMON_REQ_MAC_PDUList_NoCRNTI_Grant(nr_Cell1, p_NR_DRB_Id, cs_TimingInfo_NR( v_TimingDL), v_MAC_PDUListDL)); // @siclog "Step 16-17" siclog@ v_TimingDL := f_SubFrameTiming_AddMilliSeconds(v_TimingDL,const_SPS_Period/2); f_NR_SS_L1_TestModeConfig(nr_Cell1, cas_NR_L1_TestModeCRC_CSRNTI_REQ(nr_Cell1, cs_TimingInfo_NR( v_TimingDL), Error1AndNormal)); v_TimingDL := f_SubFrameTiming_AddMilliSeconds(v_TimingDL,const_SPS_Period/2); v_TimingUL_7 := f_SubFrameTiming_AddMilliSeconds(v_TimingUL_6,const_SPS_Period); // PDCP SDU =38B +3B PDCP Header+ 1B RLC UM Header + 2B MAC Header =44B=352 bits (an allowed TB size as per 38.523-3 Annex B) v_EncodedRlcPdu1 := f_NR_RLC_UMD_FullPDU_Encvalue(f_NR_PDCP_PDU_Encvalue(f_NR_ReturnDefaultPDCP_PDU (5, crs_NR_PDCP_SDU_38B, 1)));//@sic R5-227518 sic@ v_SDU_SubPDUListDL := { cs_NR_MAC_SDU_SubPDU_LI8(tsc_LCID_SCG_Bearer, v_EncodedRlcPdu1) }; v_MAC_PDUListDL := { cs_NR_DL_MAC_PDU_SDU_SubPDU_Padding(v_SDU_SubPDUListDL) }; f_NR_DL_SpsAssignmentConfiguration(nr_Cell1,cs_TimingInfo_NR( v_TimingDL),v_N_PRB,v_I_MCS); DRB.send(cads_NR_DRB_COMMON_REQ_MAC_PDUList_NoCRNTI_Grant(nr_Cell1, p_NR_DRB_Id, cs_TimingInfo_NR( v_TimingDL), v_MAC_PDUListDL,cs_NR_HarqProcessAssignment_Explicit(const_HARQ_Id))); //@siclog "Step 19-20" siclog@ v_TimingDL := f_SubFrameTiming_AddMilliSeconds(v_TimingDL,const_SPS_Period/2); v_TimingUL_8 := f_SubFrameTiming_AddMilliSeconds(v_TimingUL_7,const_SPS_Period/2); f_NR_DL_SpsAssignmentConfiguration(nr_Cell1,cs_TimingInfo_NR( v_TimingDL),v_N_PRB,v_I_MCS,true);//NDI=1 DRB.send(cads_NR_DRB_COMMON_REQ_MAC_PDUList_NoCRNTI_Grant(nr_Cell1, p_NR_DRB_Id, cs_TimingInfo_NR( v_TimingDL), v_MAC_PDUListDL,cs_NR_HarqProcessAssignment_Explicit(const_HARQ_Id))); //@siclog "Step 22 " siclog@ v_TimingDL := f_SubFrameTiming_AddMilliSeconds(v_TimingDL,const_SPS_Period/2-10); // 10 ms earlier v_TimingUL_9 := f_SubFrameTiming_AddMilliSeconds(v_TimingUL_8,const_SPS_Period/2-10); f_NR_SpsCgDeactivate(nr_Cell1, cs_TimingInfo_NR( v_TimingDL),cs_NR_SpsCgDeactivate_Pdcch_DL); //@siclog "Step 24 " siclog@ v_TimingDL := f_SubFrameTiming_AddMilliSeconds(v_TimingDL,10+const_SPS_Period); v_TimingUL_10 := f_SubFrameTiming_AddMilliSeconds(v_TimingUL_9,10+const_SPS_Period); // PDCP SDU =38B +3B PDCP Header+ 1B RLC UM Header + 2B MAC Header =44B=352 bits (an allowed TB size as per 38.523-3 Annex B) v_EncodedRlcPdu1 := f_NR_RLC_UMD_FullPDU_Encvalue(f_NR_PDCP_PDU_Encvalue(f_NR_ReturnDefaultPDCP_PDU (6, crs_NR_PDCP_SDU_38B,1)));//@sic R5-227518 sic@ v_SDU_SubPDUListDL := { cs_NR_MAC_SDU_SubPDU_LI8(tsc_LCID_SCG_Bearer, v_EncodedRlcPdu1) }; v_MAC_PDUListDL := { cs_NR_DL_MAC_PDU_SDU_SubPDU_Padding(v_SDU_SubPDUListDL) }; DRB.send(cads_NR_DRB_COMMON_REQ_MAC_PDUList_NoCRNTI_Grant(nr_Cell1, p_NR_DRB_Id, cs_TimingInfo_NR( v_TimingDL), v_MAC_PDUListDL)); //@siclog "Step 26-27 " siclog@ v_TimingDL := f_SubFrameTiming_AddMilliSeconds(v_TimingDL,const_SPS_Period); v_TimingUL_11 := f_SubFrameTiming_AddMilliSeconds(v_TimingUL_10,const_SPS_Period); f_NR_DL_SpsAssignmentConfiguration(nr_Cell1,cs_TimingInfo_NR( v_TimingDL),v_N_PRB,v_I_MCS); DRB.send(cads_NR_DRB_COMMON_REQ_MAC_PDUList_NoCRNTI_Grant(nr_Cell1, p_NR_DRB_Id, cs_TimingInfo_NR( v_TimingDL), v_MAC_PDUListDL)); //@sic R5-237339 R5s240155 sic@ if (pc_dl_SchedulingOffset_PDSCH_TypeA) //Default DCI configuraton. { //undo Set K1=8 f_NR_SS_CellConfig_DciDlInfo_NoRetransmission(nr_Cell1,cs_TimingInfo_NR(f_SubFrameTiming_AddMilliSeconds(v_TimingDL,100))); } // @siclog "Step 3" siclog@ SYSIND.receive(car_NR_SYSTEM_UL_HARQ_IND_Timing(nr_Cell1, ack, cr_TimingInfo_NR( v_TimingUL_1) )); f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 3"); // @siclog "Step 5" siclog@ SYSIND.receive(car_NR_SYSTEM_UL_HARQ_IND_Timing(nr_Cell1, ack, cr_TimingInfo_NR( v_TimingUL_2) )); f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 5"); // @siclog "Step 8" siclog@ SYSIND.receive(car_NR_SYSTEM_UL_HARQ_IND_Timing(nr_Cell1, ack, cr_TimingInfo_NR( v_TimingUL_3) )); f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 8"); <<SKIPPED CODE>>

After Change:

function f_TC_7_1_1_6_1_NR_TestBody1(DRB_Identity p_NR_DRB_Id) runs on NR_BASE_PTC return SubFrameTiming_Type { const integer const_SPS_Period :=80; // period in milli seconds const integer const_HARQ_Id := 1; //arbitrarily selected in steps 17 and 20 var NR_MAC_SDU_Type v_EncodedRlcPdu1; var template (value) NR_MAC_SDU_SubPDU_List_Type v_SDU_SubPDUListDL; var template (value) NR_MAC_PDUList_Type v_MAC_PDUListDL; var SubFrameTiming_Type v_TimingDL; var SubFrameTiming_Type v_TimingDL_CRNTI; var SubFrameTiming_Type v_TimingDL_Step11b; //WA# var SubFrameTiming_Type v_TimingUL_1; var SubFrameTiming_Type v_TimingUL_2; var SubFrameTiming_Type v_TimingUL_3; var SubFrameTiming_Type v_TimingUL_4; var SubFrameTiming_Type v_TimingUL_5_CRNTI; var SubFrameTiming_Type v_TimingUL_6; var SubFrameTiming_Type v_TimingUL_7; var SubFrameTiming_Type v_TimingUL_8; var SubFrameTiming_Type v_TimingUL_9; var SubFrameTiming_Type v_TimingUL_10; var SubFrameTiming_Type v_TimingUL_11; var integer v_Step11_Time; <<SKIPPED CODE>> //@siclog "Step 11a1, 11a2 11b1" siclog@ //C-RNTI grant auto transferred by SS //@sic R5s221197 R5s230088 R5-237339 sic@ //WA# if (pc_dl_SchedulingOffset_PDSCH_TypeA) { //@siclog "Step 11a1-11a2" siclog@ select(v_SCS) { case(kHz15) { v_TimingDL_CRNTI := f_SubFrameTiming_AddMilliSeconds(v_TimingDL,const_SPS_Period/2 - 4); // C-RNTI ActTime with k0=4 -- k1 needs to be adjusted to 8 v_TimingUL_5_CRNTI := f_SubFrameTiming_AddMilliSeconds(v_TimingUL_4,const_SPS_Period/2); // Receive C-RNTI HARQ with same timing as SPS HARQ //v_TimingUL_5_CRNTI.Symbol := valueof(cs_SymbolTimingInfo_Any); // REMOVED //v_TimingUL_5_CRNTI.Subframe.Number := v_TimingDL_CRNTI.Subframe.Number + 7; //FDD K1 =7 // REMOVED } case(kHz30) { v_TimingDL_CRNTI := f_SubFrameTiming_AddMilliSeconds(v_TimingDL,const_SPS_Period/2 - 2); // C-RNTI ActTime with k0=4 -- k1 needs to be adjusted to 8 v_TimingUL_5_CRNTI := f_SubFrameTiming_AddMilliSeconds(v_TimingUL_4,const_SPS_Period/2); // Receive C-RNTI HARQ with same timing as SPS HARQ //v_TimingUL_5_CRNTI.Symbol := valueof(cs_SymbolTimingInfo_Any); // REMOVED //v_TimingUL_5_CRNTI.Subframe.Number := v_TimingDL_CRNTI.Subframe.Number + 2; //TDD K1 =5 // REMOVED //v_TimingUL_5_CRNTI.Slot.SlotOffset.Numerology1 := 1; // REMOVED } case(kHz120) { v_TimingDL_CRNTI := f_SubFrameTiming_AddMilliSeconds(v_TimingDL,const_SPS_Period/2 - 1); // C-RNTI ActTime with k0=4 -- k1 needs to be adjusted to 8 v_TimingDL_CRNTI.Slot.SlotOffset.Numerology3 := 5; v_TimingUL_5_CRNTI := f_SubFrameTiming_AddMilliSeconds(v_TimingUL_4,const_SPS_Period/2); // Receive C-RNTI HARQ with same timing as SPS HARQ //v_TimingUL_5_CRNTI.Symbol := valueof(cs_SymbolTimingInfo_Any); // REMOVED //v_TimingUL_5_CRNTI.Subframe.Number := v_TimingDL_CRNTI.Subframe.Number + 1; //FR2 K1 =8 // REMOVED } case else { FatalError(__FILE__, __LINE__, "Invalid SubCarrierSapcing"); } } //Set K1=8 and K0=4 v_DciDlInfo.ResoureAssignment.TimeDomain.Index := int2bit(1, 4); //@sic R5s240155 sic@ f_NR_SS_CommonCellConfig(nr_Cell1, cas_NR_DciDlInfo_REQ(nr_Cell1, cs_TimingInfo_NR( v_TimingDL_CRNTI) , v_DciDlInfo)); // PDCP SDU =36B +3B PDCP Header+ 1B RLC UM Header + 2B MAC Header =42B=336 bits (an allowed TB size as per 38.523-3 Annex B) v_EncodedRlcPdu1 := f_NR_RLC_UMD_FullPDU_Encvalue(f_NR_PDCP_PDU_Encvalue(f_NR_ReturnDefaultPDCP_PDU (3, crs_NR_PDCP_SDU_36B, 1)));//@sic R5-227518 sic@ v_SDU_SubPDUListDL := { cs_NR_MAC_SDU_SubPDU_LI8(tsc_LCID_SCG_Bearer, v_EncodedRlcPdu1) }; v_MAC_PDUListDL := { cs_NR_DL_MAC_PDU_SDU_SubPDU_Padding(v_SDU_SubPDUListDL) }; DRB.send(cas_NR_DRB_COMMON_REQ_MAC_PDUList(nr_Cell1, p_NR_DRB_Id, cs_TimingInfo_NR( v_TimingDL_CRNTI), v_MAC_PDUListDL)); } else { //@siclog "Step 11b1" siclog@ v_TimingDL_Step11b := f_SubFrameTiming_AddMilliSeconds(v_TimingDL,const_SPS_Period/2); // Normal SPS transmission v_TimingUL_5_CRNTI := f_SubFrameTiming_AddMilliSeconds(v_TimingUL_4,const_SPS_Period/2); // Receive HARQ with same timing as SPS HARQ // PDCP SDU =36B +3B PDCP Header+ 1B RLC UM Header + 2B MAC Header =42B=336 bits (an allowed TB size as per 38.523-3 Annex B) v_EncodedRlcPdu1 := f_NR_RLC_UMD_FullPDU_Encvalue(f_NR_PDCP_PDU_Encvalue(f_NR_ReturnDefaultPDCP_PDU (3, crs_NR_PDCP_SDU_36B, 1)));//@sic R5-227518 sic@ v_SDU_SubPDUListDL := { cs_NR_MAC_SDU_SubPDU_LI8(tsc_LCID_SCG_Bearer, v_EncodedRlcPdu1) }; v_MAC_PDUListDL := { cs_NR_DL_MAC_PDU_SDU_SubPDU_Padding(v_SDU_SubPDUListDL) }; DRB.send(cads_NR_DRB_COMMON_REQ_MAC_PDUList_NoCRNTI_Grant(nr_Cell1, p_NR_DRB_Id, cs_TimingInfo_NR( v_TimingDL_Step11b), v_MAC_PDUListDL)); } <<SKIPPED CODE>> //@siclog "Step 12-13 void " siclog@ //@siclog "Step 14 " siclog@ //@sic R5-237339 sic@ v_TimingDL := f_SubFrameTiming_AddMilliSeconds(v_TimingDL,const_SPS_Period+const_SPS_Period/2); //WA# v_TimingUL_6 := f_SubFrameTiming_AddMilliSeconds(v_TimingUL_4,const_SPS_Period+const_SPS_Period/2); //WA# // PDCP SDU =38B +3B PDCP Header+ 1B RLC UM Header + 2B MAC Header =44B=352 bits (an allowed TB size as per 38.523-3 Annex B) v_EncodedRlcPdu1 := f_NR_RLC_UMD_FullPDU_Encvalue(f_NR_PDCP_PDU_Encvalue(f_NR_ReturnDefaultPDCP_PDU (4, crs_NR_PDCP_SDU_38B, 1)));//@sic R5-227518 sic@ v_SDU_SubPDUListDL := { cs_NR_MAC_SDU_SubPDU_LI8(tsc_LCID_SCG_Bearer, v_EncodedRlcPdu1) }; v_MAC_PDUListDL := { cs_NR_DL_MAC_PDU_SDU_SubPDU_Padding(v_SDU_SubPDUListDL) }; DRB.send(cads_NR_DRB_COMMON_REQ_MAC_PDUList_NoCRNTI_Grant(nr_Cell1, p_NR_DRB_Id, cs_TimingInfo_NR( v_TimingDL), v_MAC_PDUListDL)); // @siclog "Step 16-17" siclog@ v_TimingDL := f_SubFrameTiming_AddMilliSeconds(v_TimingDL,const_SPS_Period/2); f_NR_SS_L1_TestModeConfig(nr_Cell1, cas_NR_L1_TestModeCRC_CSRNTI_REQ(nr_Cell1, cs_TimingInfo_NR( v_TimingDL), Error1AndNormal)); v_TimingDL := f_SubFrameTiming_AddMilliSeconds(v_TimingDL,const_SPS_Period/2); v_TimingUL_7 := f_SubFrameTiming_AddMilliSeconds(v_TimingUL_6,const_SPS_Period); // PDCP SDU =38B +3B PDCP Header+ 1B RLC UM Header + 2B MAC Header =44B=352 bits (an allowed TB size as per 38.523-3 Annex B) v_EncodedRlcPdu1 := f_NR_RLC_UMD_FullPDU_Encvalue(f_NR_PDCP_PDU_Encvalue(f_NR_ReturnDefaultPDCP_PDU (5, crs_NR_PDCP_SDU_38B, 1)));//@sic R5-227518 sic@ v_SDU_SubPDUListDL := { cs_NR_MAC_SDU_SubPDU_LI8(tsc_LCID_SCG_Bearer, v_EncodedRlcPdu1) }; v_MAC_PDUListDL := { cs_NR_DL_MAC_PDU_SDU_SubPDU_Padding(v_SDU_SubPDUListDL) }; f_NR_DL_SpsAssignmentConfiguration(nr_Cell1,cs_TimingInfo_NR( v_TimingDL),v_N_PRB,v_I_MCS); DRB.send(cads_NR_DRB_COMMON_REQ_MAC_PDUList_NoCRNTI_Grant(nr_Cell1, p_NR_DRB_Id, cs_TimingInfo_NR( v_TimingDL), v_MAC_PDUListDL,cs_NR_HarqProcessAssignment_Explicit(const_HARQ_Id))); //@siclog "Step 19-20" siclog@ v_TimingDL := f_SubFrameTiming_AddMilliSeconds(v_TimingDL,const_SPS_Period/2); v_TimingUL_8 := f_SubFrameTiming_AddMilliSeconds(v_TimingUL_7,const_SPS_Period/2); f_NR_DL_SpsAssignmentConfiguration(nr_Cell1,cs_TimingInfo_NR( v_TimingDL),v_N_PRB,v_I_MCS,true);//NDI=1 DRB.send(cads_NR_DRB_COMMON_REQ_MAC_PDUList_NoCRNTI_Grant(nr_Cell1, p_NR_DRB_Id, cs_TimingInfo_NR( v_TimingDL), v_MAC_PDUListDL,cs_NR_HarqProcessAssignment_Explicit(const_HARQ_Id))); //@siclog "Step 22 " siclog@ v_TimingDL := f_SubFrameTiming_AddMilliSeconds(v_TimingDL,const_SPS_Period/2-10); // 10 ms earlier v_TimingUL_9 := f_SubFrameTiming_AddMilliSeconds(v_TimingUL_8,const_SPS_Period/2-10); f_NR_SpsCgDeactivate(nr_Cell1, cs_TimingInfo_NR( v_TimingDL),cs_NR_SpsCgDeactivate_Pdcch_DL); //@siclog "Step 24 " siclog@ v_TimingDL := f_SubFrameTiming_AddMilliSeconds(v_TimingDL,10+const_SPS_Period); v_TimingUL_10 := f_SubFrameTiming_AddMilliSeconds(v_TimingUL_9,10+const_SPS_Period); // PDCP SDU =38B +3B PDCP Header+ 1B RLC UM Header + 2B MAC Header =44B=352 bits (an allowed TB size as per 38.523-3 Annex B) v_EncodedRlcPdu1 := f_NR_RLC_UMD_FullPDU_Encvalue(f_NR_PDCP_PDU_Encvalue(f_NR_ReturnDefaultPDCP_PDU (6, crs_NR_PDCP_SDU_38B,1)));//@sic R5-227518 sic@ v_SDU_SubPDUListDL := { cs_NR_MAC_SDU_SubPDU_LI8(tsc_LCID_SCG_Bearer, v_EncodedRlcPdu1) }; v_MAC_PDUListDL := { cs_NR_DL_MAC_PDU_SDU_SubPDU_Padding(v_SDU_SubPDUListDL) }; DRB.send(cads_NR_DRB_COMMON_REQ_MAC_PDUList_NoCRNTI_Grant(nr_Cell1, p_NR_DRB_Id, cs_TimingInfo_NR( v_TimingDL), v_MAC_PDUListDL)); //@siclog "Step 26-27 " siclog@ v_TimingDL := f_SubFrameTiming_AddMilliSeconds(v_TimingDL,const_SPS_Period); v_TimingUL_11 := f_SubFrameTiming_AddMilliSeconds(v_TimingUL_10,const_SPS_Period); f_NR_DL_SpsAssignmentConfiguration(nr_Cell1,cs_TimingInfo_NR( v_TimingDL),v_N_PRB,v_I_MCS); DRB.send(cads_NR_DRB_COMMON_REQ_MAC_PDUList_NoCRNTI_Grant(nr_Cell1, p_NR_DRB_Id, cs_TimingInfo_NR( v_TimingDL), v_MAC_PDUListDL)); //@sic R5-237339 R5s240155 sic@ if (pc_dl_SchedulingOffset_PDSCH_TypeA) //Default DCI configuraton. { //undo Set K1=8 f_NR_SS_CellConfig_DciDlInfo_NoRetransmission(nr_Cell1,cs_TimingInfo_NR(f_SubFrameTiming_AddMilliSeconds(v_TimingDL,50))); } // @siclog "Step 3" siclog@ SYSIND.receive(car_NR_SYSTEM_UL_HARQ_IND_Timing(nr_Cell1, ack, cr_TimingInfo_NR( v_TimingUL_1) )); f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 3"); // @siclog "Step 5" siclog@ SYSIND.receive(car_NR_SYSTEM_UL_HARQ_IND_Timing(nr_Cell1, ack, cr_TimingInfo_NR( v_TimingUL_2) )); f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 5"); // @siclog "Step 8" siclog@ SYSIND.receive(car_NR_SYSTEM_UL_HARQ_IND_Timing(nr_Cell1, ack, cr_TimingInfo_NR( v_TimingUL_3) )); f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 8"); <<SKIPPED CODE>>

## [Anritsu] Change 2

| Function name | function f_TC_7_1_1_6_1_NR5GC |
| --- | --- |
| Reason for change | Since SPS-Config is still active on SS, the Step 29 RRC Reconfiguration may be transmitted via DL SPS scheduling and addressed via CS-RNTI. In this scenario, once UE applies the Reconfiguration and stops receiving as per stored SPS assignment, UE will not see the RLC Status PDU for RRC Reconfiguration Complete, leading to unexpected RLC retransmissions. To prevent this and transmit Step 29 via C-RNTI, it is proposed to do an SPS local release before the Step 29 RRC Reconfiguration. |
| Summary of change | Perform SPS LocalRelease before Step 29. To prevent a race condition with the adjacent RadioBearerList config, an explicit ActTime is set. |
| TTCN module | MAC_SPS_NR5GC.ttcn |

Before Change:

function f_TC_7_1_1_6_1_NR5GC() runs on NR5GC_PTC { // Correct handling of DL assignment / Semi-persistent case var DRB_Identity v_DRBId; var template (value) PhysicalCellGroupConfig v_PhysicalCellGroupConfig ; var template (value) CellGroupConfig v_CellGroupConfig; var template (value) SpCellConfig v_SpCellConfig ; var template (value) NR_DownlinkBWP_List_Type v_NR_DownlinkBWP_List; var template (omit) RRCReconfiguration_v1530_IEs v_V1530Ext; var template (value) NR_RLC_BearerToAddModList_Type v_RLC_BearerConfigList; var template (value) NR_RadioBearerList_Type v_SS_Drb_ConfigList; var SubFrameTiming_Type v_TimingDL; var SubFrameTiming_Type v_TimingDL2; var template (value) ServingCellConfig v_ServingCellConfig; //@sic R5s221197 sic@ var template (value) NR_DownlinkBWP_Type v_DownlinkBWP ; //Init Cell parameters f_NR5GC_Init(NR_1); //@sic R5-297339 sic@ v_DownlinkBWP := f_NR_CellInfo_GetDownlinkBWP(nr_Cell1, tsc_NR_BWP_Id); v_DownlinkBWP.Pdsch.ConfigCommon.R15.pdsch_TimeDomainAllocationList := cs_NR_PDSCH_TimeDomainResourceAllocationList_71161; f_NR_CellInfo_SetDownlinkBWP(nr_Cell1, v_DownlinkBWP, tsc_NR_BWP_Id); f_NR_CellConfig_Def(nr_Cell1); //@sic R5-297339 sic@ if (pc_dl_SchedulingOffset_PDSCH_TypeA) { f_NR_CellInfo_SetSIB1_TimeDomainAllocation(nr_Cell1, cs_NR_PDSCH_TimeDomainResourceAllocationList_71161); } //Create and configure NR cell1 f_NR_CellConfig_Def(nr_Cell1); //@sic R5-297339 sic@ if (pc_dl_SchedulingOffset_PDSCH_TypeA) { v_NR_DownlinkBWP_List := f_NR_CellInfo_GetDownlinkBWP_List(nr_Cell1); v_NR_DownlinkBWP_List[0].Pdsch.ConfigCommon.R15.pdsch_TimeDomainAllocationList := cs_NR_PDSCH_TimeDomainResourceAllocationList_71161; //v_NR_DownlinkBWP_List[0].Sps.R15 := cs_SPS_Config_711161; f_NR_CellInfo_SetDownlinkBWP_List(nr_Cell1, v_NR_DownlinkBWP_List); f_NR_SS_CommonCellConfig(nr_Cell1, cads_NR_DownlinkBWP_ConfigCommon_REQ(nr_Cell1, -, v_NR_DownlinkBWP_List)); } // Get NR capabilities sent during EUTRA preamble f_NR5GC_Preamble (nr_Cell1, STATE_IDLE_1A, TESTModeA_ON); //Set null cipher Algorithm f_NR_AS_CipheringAlgorithm_Set(nea0); //Retrieve DRB Id of the first PDU session v_DRBId := f_NR_GetDefaultDRB_ForFirstPDUSession(); //Initialise for No Header Manipulation DL only. v_SS_Drb_ConfigList := {cs_NR_SS_RadioBearer_MAC_TestMode(v_DRBId, cs_NR_MAC_TestModeInfo_NormalMode)}; v_RLC_BearerConfigList := f_NR_ReconfigureRLC_UM (v_DRBId, -, -, f_NR_DL_UM_T_Reassembly(f_NR_CellInfo_GetIsFR1(nr_Cell1))); //Configure f_NR5GC_RRC_ConnectedState3N_Def(nr_Cell1, TEST_LOOPModeA_ON, cs_UE_TestLoopModeA_NR_LB_Setup_1DRB(0, v_DRBId), -, //p_IpPduDelayTime v_RLC_BearerConfigList, -, //MAC_CellGroupConfig -, v_SS_Drb_ConfigList ); f_NR_TestBody_Set(true); //@sic step 0A-0B sic@ //@sic R5s221197 sic@ v_ServingCellConfig := f_NR_GetServingCellConfig(nr_Cell1); v_SpCellConfig := f_NR_GetSpCellConfigDef(nr_Cell1, v_ServingCellConfig); v_SpCellConfig.spCellConfigDedicated.initialDownlinkBWP.sps_Config := {setup := cs_SPS_Config_711161}; v_NR_DownlinkBWP_List := f_NR_CellInfo_GetDownlinkBWP_List(nr_Cell1); v_NR_DownlinkBWP_List[0].Sps.R15 := cs_SPS_Config_711161; v_PhysicalCellGroupConfig := f_NR_CellInfo_GetPhysicalCellGroupConfig(nr_Cell1); v_PhysicalCellGroupConfig.cs_RNTI := { setup := tsc_RNTI_Value_CS_RNTI}; v_CellGroupConfig := cs_38508_CellGroupConfig (tsc_NR_CellGroupId_MCG , omit, omit, omit, v_PhysicalCellGroupConfig, v_SpCellConfig ); v_V1530Ext := cs_NR_RRCReconfiguration_v1530_IEs(bit2oct(encvalue(v_CellGroupConfig))); f_NR_SendRRCReconfiguration (nr_Cell1, -,-, v_V1530Ext); f_NR_SS_ServingCellConfig(nr_Cell1, -, cs_NR_ServingCellConfig_SpCell(cs_NR_SS_SpCellConfig(omit, omit, omit, v_PhysicalCellGroupConfig))); f_NR_CellInfo_SetDownlinkBWP_List(nr_Cell1, v_NR_DownlinkBWP_List); f_NR_SS_CommonCellConfig(nr_Cell1, cads_NR_DownlinkBWP_ConfigCommon_REQ(nr_Cell1, -, v_NR_DownlinkBWP_List)); // @siclog "Steps 1 - 28" siclog@ v_SS_Drb_ConfigList := {cs_NR_SS_RadioBearer_MAC_TestMode(v_DRBId, cs_NR_MAC_TestModeInfo_NoHeaderManipulationDL_UL)}; f_NR_SS_CommonRadioBearerConfig(nr_Cell1, v_SS_Drb_ConfigList); v_TimingDL := f_TC_7_1_1_6_1_NR_TestBody1(v_DRBId); //@siclog steps 29-30 siclog@ v_SS_Drb_ConfigList := {cs_NR_SS_RadioBearer_MAC_TestMode(v_DRBId, cs_NR_MAC_TestModeInfo_NormalMode)}; f_NR_SS_CommonRadioBearerConfig(nr_Cell1, v_SS_Drb_ConfigList); v_SpCellConfig.spCellConfigDedicated.initialDownlinkBWP.sps_Config := {release := NULL}; v_NR_DownlinkBWP_List[0].Sps := omit; v_CellGroupConfig := cs_38508_CellGroupConfig (tsc_NR_CellGroupId_MCG , omit, omit, omit, omit, v_SpCellConfig ); v_V1530Ext := cs_NR_RRCReconfiguration_v1530_IEs(bit2oct(encvalue(v_CellGroupConfig))); f_NR_SendRRCReconfiguration (nr_Cell1, -,-, v_V1530Ext); //@siclog steps 31-32 siclog@ v_SS_Drb_ConfigList := {cs_NR_SS_RadioBearer_MAC_TestMode(v_DRBId, cs_NR_MAC_TestModeInfo_NoHeaderManipulationDL_UL)}; f_NR_SS_CommonRadioBearerConfig(nr_Cell1, v_SS_Drb_ConfigList); f_TC_7_1_1_6_1_NR_TestBody2(v_DRBId,v_TimingDL,10); //10*80 =800 ms is assumed to be sufficient to perfor RRC Reconfiguration //@sic R5s230088 sic@ f_NR_CellInfo_SetDownlinkBWP_List(nr_Cell1, v_NR_DownlinkBWP_List); f_NR_SS_CommonCellConfig(nr_Cell1, cads_NR_DownlinkBWP_ConfigCommon_REQ(nr_Cell1, -, v_NR_DownlinkBWP_List)); f_NR_TestBody_Set(false); //Postamble f_NR_ULGrantConfiguration_Start(nr_Cell1); // Def UL Grant Config // to configure SS to stop report reception of HARQ ACK or NACK f_NR_SS_SystemIndCtrlConfig(nr_Cell1, cds_NR_SystemIndCtrl_UL_HARQ(disable)); // configure SS Mac IN NORMAL TEST MODE. v_SS_Drb_ConfigList := {cs_NR_SS_RadioBearer_MAC_TestMode(v_DRBId, cs_NR_MAC_TestModeInfo_NormalMode)}; f_NR_SS_CommonRadioBearerConfig(nr_Cell1, v_SS_Drb_ConfigList); //@sic R5s230130 sic@ //f_NR5GC_OpenUE_TestLoopMode_Deactivate_TestMode(nr_Cell1); f_NR_Postamble(nr_Cell1, STATE_CONNECTED_3A); };

After Change:

function f_TC_7_1_1_6_1_NR5GC() runs on NR5GC_PTC { // Correct handling of DL assignment / Semi-persistent case var DRB_Identity v_DRBId; var template (value) PhysicalCellGroupConfig v_PhysicalCellGroupConfig ; var template (value) CellGroupConfig v_CellGroupConfig; var template (value) SpCellConfig v_SpCellConfig ; var template (value) NR_DownlinkBWP_List_Type v_NR_DownlinkBWP_List; var template (omit) RRCReconfiguration_v1530_IEs v_V1530Ext; var template (value) NR_RLC_BearerToAddModList_Type v_RLC_BearerConfigList; var template (value) NR_RadioBearerList_Type v_SS_Drb_ConfigList; var SubFrameTiming_Type v_TimingDL; var template (value) ServingCellConfig v_ServingCellConfig; //@sic R5s221197 sic@ var template (value) NR_DownlinkBWP_Type v_DownlinkBWP ; //Init Cell parameters f_NR5GC_Init(NR_1); //@sic R5-297339 sic@ v_DownlinkBWP := f_NR_CellInfo_GetDownlinkBWP(nr_Cell1, tsc_NR_BWP_Id); v_DownlinkBWP.Pdsch.ConfigCommon.R15.pdsch_TimeDomainAllocationList := cs_NR_PDSCH_TimeDomainResourceAllocationList_71161; f_NR_CellInfo_SetDownlinkBWP(nr_Cell1, v_DownlinkBWP, tsc_NR_BWP_Id); f_NR_CellConfig_Def(nr_Cell1); //@sic R5-297339 sic@ if (pc_dl_SchedulingOffset_PDSCH_TypeA) { f_NR_CellInfo_SetSIB1_TimeDomainAllocation(nr_Cell1, cs_NR_PDSCH_TimeDomainResourceAllocationList_71161); } //Create and configure NR cell1 f_NR_CellConfig_Def(nr_Cell1); //@sic R5-297339 sic@ if (pc_dl_SchedulingOffset_PDSCH_TypeA) { v_NR_DownlinkBWP_List := f_NR_CellInfo_GetDownlinkBWP_List(nr_Cell1); v_NR_DownlinkBWP_List[0].Pdsch.ConfigCommon.R15.pdsch_TimeDomainAllocationList := cs_NR_PDSCH_TimeDomainResourceAllocationList_71161; //v_NR_DownlinkBWP_List[0].Sps.R15 := cs_SPS_Config_711161; f_NR_CellInfo_SetDownlinkBWP_List(nr_Cell1, v_NR_DownlinkBWP_List); f_NR_SS_CommonCellConfig(nr_Cell1, cads_NR_DownlinkBWP_ConfigCommon_REQ(nr_Cell1, -, v_NR_DownlinkBWP_List)); } // Get NR capabilities sent during EUTRA preamble f_NR5GC_Preamble (nr_Cell1, STATE_IDLE_1A, TESTModeA_ON); //Set null cipher Algorithm f_NR_AS_CipheringAlgorithm_Set(nea0); //Retrieve DRB Id of the first PDU session v_DRBId := f_NR_GetDefaultDRB_ForFirstPDUSession(); //Initialise for No Header Manipulation DL only. v_SS_Drb_ConfigList := {cs_NR_SS_RadioBearer_MAC_TestMode(v_DRBId, cs_NR_MAC_TestModeInfo_NormalMode)}; v_RLC_BearerConfigList := f_NR_ReconfigureRLC_UM (v_DRBId, -, -, f_NR_DL_UM_T_Reassembly(f_NR_CellInfo_GetIsFR1(nr_Cell1))); //Configure f_NR5GC_RRC_ConnectedState3N_Def(nr_Cell1, TEST_LOOPModeA_ON, cs_UE_TestLoopModeA_NR_LB_Setup_1DRB(0, v_DRBId), -, //p_IpPduDelayTime v_RLC_BearerConfigList, -, //MAC_CellGroupConfig -, v_SS_Drb_ConfigList ); f_NR_TestBody_Set(true); //@sic step 0A-0B sic@ //@sic R5s221197 sic@ v_ServingCellConfig := f_NR_GetServingCellConfig(nr_Cell1); v_SpCellConfig := f_NR_GetSpCellConfigDef(nr_Cell1, v_ServingCellConfig); v_SpCellConfig.spCellConfigDedicated.initialDownlinkBWP.sps_Config := {setup := cs_SPS_Config_711161}; v_NR_DownlinkBWP_List := f_NR_CellInfo_GetDownlinkBWP_List(nr_Cell1); v_NR_DownlinkBWP_List[0].Sps.R15 := cs_SPS_Config_711161; v_PhysicalCellGroupConfig := f_NR_CellInfo_GetPhysicalCellGroupConfig(nr_Cell1); v_PhysicalCellGroupConfig.cs_RNTI := { setup := tsc_RNTI_Value_CS_RNTI}; v_CellGroupConfig := cs_38508_CellGroupConfig (tsc_NR_CellGroupId_MCG , omit, omit, omit, v_PhysicalCellGroupConfig, v_SpCellConfig ); v_V1530Ext := cs_NR_RRCReconfiguration_v1530_IEs(bit2oct(encvalue(v_CellGroupConfig))); f_NR_SendRRCReconfiguration (nr_Cell1, -,-, v_V1530Ext); f_NR_SS_ServingCellConfig(nr_Cell1, -, cs_NR_ServingCellConfig_SpCell(cs_NR_SS_SpCellConfig(omit, omit, omit, v_PhysicalCellGroupConfig))); f_NR_CellInfo_SetDownlinkBWP_List(nr_Cell1, v_NR_DownlinkBWP_List); f_NR_SS_CommonCellConfig(nr_Cell1, cads_NR_DownlinkBWP_ConfigCommon_REQ(nr_Cell1, -, v_NR_DownlinkBWP_List)); // @siclog "Steps 1 - 28" siclog@ v_SS_Drb_ConfigList := {cs_NR_SS_RadioBearer_MAC_TestMode(v_DRBId, cs_NR_MAC_TestModeInfo_NoHeaderManipulationDL_UL)}; f_NR_SS_CommonRadioBearerConfig(nr_Cell1, v_SS_Drb_ConfigList); v_TimingDL := f_TC_7_1_1_6_1_NR_TestBody1(v_DRBId); v_TimingDL2 := f_SubFrameTiming_AddMilliSeconds(v_TimingDL,30); //30ms later f_NR_SpsCgDeactivate(nr_Cell1, cs_TimingInfo_NR( v_TimingDL2),cs_NR_SpsCgDeactivate_LclRel); //@siclog steps 29-30 siclog@ v_SS_Drb_ConfigList := {cs_NR_SS_RadioBearer_MAC_TestMode(v_DRBId, cs_NR_MAC_TestModeInfo_NormalMode)}; f_NR_SS_CommonRadioBearerConfig(nr_Cell1, v_SS_Drb_ConfigList); v_SpCellConfig.spCellConfigDedicated.initialDownlinkBWP.sps_Config := {release := NULL}; v_NR_DownlinkBWP_List[0].Sps := omit; v_CellGroupConfig := cs_38508_CellGroupConfig (tsc_NR_CellGroupId_MCG , omit, omit, omit, omit, v_SpCellConfig ); v_V1530Ext := cs_NR_RRCReconfiguration_v1530_IEs(bit2oct(encvalue(v_CellGroupConfig))); f_NR_SendRRCReconfiguration (nr_Cell1, -,-, v_V1530Ext); //@siclog steps 31-32 siclog@ v_SS_Drb_ConfigList := {cs_NR_SS_RadioBearer_MAC_TestMode(v_DRBId, cs_NR_MAC_TestModeInfo_NoHeaderManipulationDL_UL)}; f_NR_SS_CommonRadioBearerConfig(nr_Cell1, v_SS_Drb_ConfigList); f_TC_7_1_1_6_1_NR_TestBody2(v_DRBId,v_TimingDL,10); //10*80 =800 ms is assumed to be sufficient to perfor RRC Reconfiguration //@sic R5s230088 sic@ f_NR_CellInfo_SetDownlinkBWP_List(nr_Cell1, v_NR_DownlinkBWP_List); f_NR_SS_CommonCellConfig(nr_Cell1, cads_NR_DownlinkBWP_ConfigCommon_REQ(nr_Cell1, -, v_NR_DownlinkBWP_List)); f_NR_TestBody_Set(false); //Postamble f_NR_ULGrantConfiguration_Start(nr_Cell1); // Def UL Grant Config // to configure SS to stop report reception of HARQ ACK or NACK f_NR_SS_SystemIndCtrlConfig(nr_Cell1, cds_NR_SystemIndCtrl_UL_HARQ(disable)); // configure SS Mac IN NORMAL TEST MODE. v_SS_Drb_ConfigList := {cs_NR_SS_RadioBearer_MAC_TestMode(v_DRBId, cs_NR_MAC_TestModeInfo_NormalMode)}; f_NR_SS_CommonRadioBearerConfig(nr_Cell1, v_SS_Drb_ConfigList); //@sic R5s230130 sic@ //f_NR5GC_OpenUE_TestLoopMode_Deactivate_TestMode(nr_Cell1); f_NR_Postamble(nr_Cell1, STATE_CONNECTED_3A); };

## [Anritsu] Change 3

| Function name | function f_TC_7_1_1_6_1_ENDC_NR |
| --- | --- |
| Reason for change | Same as Change 2 |
| Summary of change | Same as Change 2 |
| TTCN module | MAC_SPS_ENDC_NR.ttcn |

Before Change:

function f_TC_7_1_1_6_1_ENDC_NR() runs on ENDC_NR_PTC {// Correct handling of DL assignment / Semi-persistent case var DRB_Identity v_DRBId :=tsc_NR_DRB2; var template (value) PhysicalCellGroupConfig v_PhysicalCellGroupConfig ; //@sic R5s230087 sic@ var template (value) NR_RadioBearerList_Type v_SS_Drb_ConfigList := {cs_NR_SS_RadioBearer_MAC_TestMode(tsc_NR_DRB2, cs_NR_MAC_TestModeInfo_NormalMode)}; var template (value) CellGroupConfig v_CellGroupConfig; var template (value) SpCellConfig v_SpCellConfig ; var template (value) NR_DownlinkBWP_List_Type v_NR_DownlinkBWP_List; var SubFrameTiming_Type v_TimingDL; var template (value) ServingCellConfig v_ServingCellConfig; //@sic R5s221195 sic@ var template (value) NR_DownlinkBWP_Type v_DownlinkBWP ; //Init Cell parameters f_ENDC_NR_Init(); //@sic R5-297339 R5s240155 sic@ v_DownlinkBWP := f_NR_CellInfo_GetDownlinkBWP(nr_Cell1, tsc_NR_BWP_Id); v_DownlinkBWP.Pdsch.ConfigCommon.R15.pdsch_TimeDomainAllocationList := cs_NR_PDSCH_TimeDomainResourceAllocationList_71161; f_NR_CellInfo_SetDownlinkBWP(nr_Cell1, v_DownlinkBWP, tsc_NR_BWP_Id); if (pc_dl_SchedulingOffset_PDSCH_TypeA) { f_NR_CellInfo_SetSIB1_TimeDomainAllocation(nr_Cell1, cs_NR_PDSCH_TimeDomainResourceAllocationList_71161); } //Create and configure NR cell1 f_NR_CellConfig_Def(nr_Cell1); // Get NR capabilities sent during EUTRA preamble f_NR_ENDC_PreambleOnEUTRA(nr_Cell1, MCG_Only); f_NR_AS_CipheringAlgorithm_Set(nea0); //Configure on SS the SCG DRB in No MAC header manipulation in DL and RLC and PDCP in transparent mode f_NR_ENDC_ConfigAM_UM(nr_Cell1, 0, 1, v_SS_Drb_ConfigList); // Wait for Trigger from EUTRA indicating start of Test Body f_EUTRA_NR_WaitForCoOrd_Trigger(EUTRA); f_NR_TestBody_Set(true); //@siclog steps 0A-0B siclog@ v_ServingCellConfig := f_NR_GetServingCellConfig(nr_Cell1); v_SpCellConfig := f_NR_GetSpCellConfigDef(nr_Cell1, v_ServingCellConfig); v_SpCellConfig.spCellConfigDedicated.initialDownlinkBWP.sps_Config := {setup := cs_SPS_Config_711161}; v_NR_DownlinkBWP_List := f_NR_CellInfo_GetDownlinkBWP_List(nr_Cell1); v_NR_DownlinkBWP_List[0].Sps.R15 := cs_SPS_Config_711161; //@sic R5-297339 R5s240155 sic@ if (pc_dl_SchedulingOffset_PDSCH_TypeA) { v_NR_DownlinkBWP_List[0].Pdsch.ConfigCommon.R15.pdsch_TimeDomainAllocationList := cs_NR_PDSCH_TimeDomainResourceAllocationList_71161; } f_NR_CellInfo_SetDownlinkBWP_List(nr_Cell1, v_NR_DownlinkBWP_List); v_PhysicalCellGroupConfig := f_NR_CellInfo_GetPhysicalCellGroupConfig(nr_Cell1); v_PhysicalCellGroupConfig.cs_RNTI := { setup := tsc_RNTI_Value_CS_RNTI}; //@sic R5s221195 R5-297339 sic@ if (pc_dl_SchedulingOffset_PDSCH_TypeA) { v_SpCellConfig.reconfigurationWithSync.spCellConfigCommon.downlinkConfigCommon.initialDownlinkBWP.pdsch_ConfigCommon.setup.pdsch_TimeDomainAllocationList := cs_NR_PDSCH_TimeDomainResourceAllocationList_71161; } else { v_SpCellConfig.reconfigurationWithSync := omit; } v_SpCellConfig.rlf_TimersAndConstants := omit; v_CellGroupConfig := cs_38508_CellGroupConfig (tsc_NR_CellGroupId_SCG ,//@sic R5s221195 sic@ omit, omit, omit, v_PhysicalCellGroupConfig, v_SpCellConfig ); //@sic R5s221195 sic@ f_NR_SendRRCReconfigurationContentsToEUTRA (omit, v_CellGroupConfig); f_NR_SS_ServingCellConfig(nr_Cell1, -, cs_NR_ServingCellConfig_SpCell(cs_NR_SS_SpCellConfig(omit, omit, omit, v_PhysicalCellGroupConfig))); f_NR_SS_CommonCellConfig(nr_Cell1, cads_NR_DownlinkBWP_ConfigCommon_REQ(nr_Cell1, -, v_NR_DownlinkBWP_List)); //@sic R5s230087 sic@ v_SS_Drb_ConfigList :={cs_NR_SS_RadioBearer_MAC_TestMode(tsc_NR_DRB2, cs_NR_MAC_TestModeInfo_NoHeaderManipulationDL_UL)}; f_NR_SS_CommonRadioBearerConfig(nr_Cell1, v_SS_Drb_ConfigList); // @siclog "Steps 1 - 28" siclog@ v_TimingDL := f_TC_7_1_1_6_1_NR_TestBody1(v_DRBId); //@siclog steps 29-30 siclog@ if (pc_dl_SchedulingOffset_PDSCH_TypeA) { v_SpCellConfig.reconfigurationWithSync := omit; } v_SpCellConfig.spCellConfigDedicated.initialDownlinkBWP.sps_Config := {release := NULL}; v_NR_DownlinkBWP_List[0].Sps := omit; v_CellGroupConfig := cs_38508_CellGroupConfig (tsc_NR_CellGroupId_SCG , //@sic R5s221195 sic@ omit, omit, omit, omit, v_SpCellConfig ); f_NR_SendRRCReconfigurationContentsToEUTRA (omit, v_CellGroupConfig); //@siclog steps 31-32 siclog@ f_TC_7_1_1_6_1_NR_TestBody2(v_DRBId,v_TimingDL,10); //10*80 =800 ms is assumed to be sufficient to perfor RRC Reconfiguration f_NR_CellInfo_SetDownlinkBWP_List(nr_Cell1, v_NR_DownlinkBWP_List); f_NR_SS_CommonCellConfig(nr_Cell1, cads_NR_DownlinkBWP_ConfigCommon_REQ(nr_Cell1, -, v_NR_DownlinkBWP_List)); f_NR_TestBody_Set(false); // Send Trigger to EUTRA indicating Finish of Test Body f_EUTRA_NR_SendCoOrd(EUTRA, cms_EUTRA_NR_Trigger); //Wait from coordination AFTER the connection is released in E-UTRA f_EUTRA_NR_WaitForCoOrd_Trigger(EUTRA, "Postamble"); //Postamble f_NR_ReleaseAllCells(); };

After Change:

import from Common4G5G_Timing all; … function f_TC_7_1_1_6_1_ENDC_NR() runs on ENDC_NR_PTC {// Correct handling of DL assignment / Semi-persistent case var DRB_Identity v_DRBId :=tsc_NR_DRB2; var template (value) PhysicalCellGroupConfig v_PhysicalCellGroupConfig ; //@sic R5s230087 sic@ var template (value) NR_RadioBearerList_Type v_SS_Drb_ConfigList := {cs_NR_SS_RadioBearer_MAC_TestMode(tsc_NR_DRB2, cs_NR_MAC_TestModeInfo_NormalMode)}; var template (value) CellGroupConfig v_CellGroupConfig; var template (value) SpCellConfig v_SpCellConfig ; var template (value) NR_DownlinkBWP_List_Type v_NR_DownlinkBWP_List; var SubFrameTiming_Type v_TimingDL; var SubFrameTiming_Type v_TimingDL2; var template (value) ServingCellConfig v_ServingCellConfig; //@sic R5s221195 sic@ var template (value) NR_DownlinkBWP_Type v_DownlinkBWP ; //Init Cell parameters f_ENDC_NR_Init(); //@sic R5-297339 R5s240155 sic@ v_DownlinkBWP := f_NR_CellInfo_GetDownlinkBWP(nr_Cell1, tsc_NR_BWP_Id); v_DownlinkBWP.Pdsch.ConfigCommon.R15.pdsch_TimeDomainAllocationList := cs_NR_PDSCH_TimeDomainResourceAllocationList_71161; f_NR_CellInfo_SetDownlinkBWP(nr_Cell1, v_DownlinkBWP, tsc_NR_BWP_Id); if (pc_dl_SchedulingOffset_PDSCH_TypeA) { f_NR_CellInfo_SetSIB1_TimeDomainAllocation(nr_Cell1, cs_NR_PDSCH_TimeDomainResourceAllocationList_71161); } //Create and configure NR cell1 f_NR_CellConfig_Def(nr_Cell1); // Get NR capabilities sent during EUTRA preamble f_NR_ENDC_PreambleOnEUTRA(nr_Cell1, MCG_Only); f_NR_AS_CipheringAlgorithm_Set(nea0); //Configure on SS the SCG DRB in No MAC header manipulation in DL and RLC and PDCP in transparent mode f_NR_ENDC_ConfigAM_UM(nr_Cell1, 0, 1, v_SS_Drb_ConfigList); // Wait for Trigger from EUTRA indicating start of Test Body f_EUTRA_NR_WaitForCoOrd_Trigger(EUTRA); f_NR_TestBody_Set(true); //@siclog steps 0A-0B siclog@ v_ServingCellConfig := f_NR_GetServingCellConfig(nr_Cell1); v_SpCellConfig := f_NR_GetSpCellConfigDef(nr_Cell1, v_ServingCellConfig); v_SpCellConfig.spCellConfigDedicated.initialDownlinkBWP.sps_Config := {setup := cs_SPS_Config_711161}; v_NR_DownlinkBWP_List := f_NR_CellInfo_GetDownlinkBWP_List(nr_Cell1); v_NR_DownlinkBWP_List[0].Sps.R15 := cs_SPS_Config_711161; //@sic R5-297339 R5s240155 sic@ if (pc_dl_SchedulingOffset_PDSCH_TypeA) { v_NR_DownlinkBWP_List[0].Pdsch.ConfigCommon.R15.pdsch_TimeDomainAllocationList := cs_NR_PDSCH_TimeDomainResourceAllocationList_71161; } f_NR_CellInfo_SetDownlinkBWP_List(nr_Cell1, v_NR_DownlinkBWP_List); v_PhysicalCellGroupConfig := f_NR_CellInfo_GetPhysicalCellGroupConfig(nr_Cell1); v_PhysicalCellGroupConfig.cs_RNTI := { setup := tsc_RNTI_Value_CS_RNTI}; //@sic R5s221195 R5-297339 sic@ if (pc_dl_SchedulingOffset_PDSCH_TypeA) { v_SpCellConfig.reconfigurationWithSync.spCellConfigCommon.downlinkConfigCommon.initialDownlinkBWP.pdsch_ConfigCommon.setup.pdsch_TimeDomainAllocationList := cs_NR_PDSCH_TimeDomainResourceAllocationList_71161; } else { v_SpCellConfig.reconfigurationWithSync := omit; } v_SpCellConfig.rlf_TimersAndConstants := omit; v_CellGroupConfig := cs_38508_CellGroupConfig (tsc_NR_CellGroupId_SCG ,//@sic R5s221195 sic@ omit, omit, omit, v_PhysicalCellGroupConfig, v_SpCellConfig ); //@sic R5s221195 sic@ f_NR_SendRRCReconfigurationContentsToEUTRA (omit, v_CellGroupConfig); f_NR_SS_ServingCellConfig(nr_Cell1, -, cs_NR_ServingCellConfig_SpCell(cs_NR_SS_SpCellConfig(omit, omit, omit, v_PhysicalCellGroupConfig))); f_NR_SS_CommonCellConfig(nr_Cell1, cads_NR_DownlinkBWP_ConfigCommon_REQ(nr_Cell1, -, v_NR_DownlinkBWP_List)); //@sic R5s230087 sic@ v_SS_Drb_ConfigList :={cs_NR_SS_RadioBearer_MAC_TestMode(tsc_NR_DRB2, cs_NR_MAC_TestModeInfo_NoHeaderManipulationDL_UL)}; f_NR_SS_CommonRadioBearerConfig(nr_Cell1, v_SS_Drb_ConfigList); // @siclog "Steps 1 - 28" siclog@ v_TimingDL := f_TC_7_1_1_6_1_NR_TestBody1(v_DRBId); v_TimingDL2 := f_SubFrameTiming_AddMilliSeconds(v_TimingDL,30); //30ms later f_NR_SpsCgDeactivate(nr_Cell1, cs_TimingInfo_NR( v_TimingDL2),cs_NR_SpsCgDeactivate_LclRel); //@siclog steps 29-30 siclog@ if (pc_dl_SchedulingOffset_PDSCH_TypeA) { v_SpCellConfig.reconfigurationWithSync := omit; } v_SpCellConfig.spCellConfigDedicated.initialDownlinkBWP.sps_Config := {release := NULL}; v_NR_DownlinkBWP_List[0].Sps := omit; v_CellGroupConfig := cs_38508_CellGroupConfig (tsc_NR_CellGroupId_SCG , //@sic R5s221195 sic@ omit, omit, omit, omit, v_SpCellConfig ); f_NR_SendRRCReconfigurationContentsToEUTRA (omit, v_CellGroupConfig); //@siclog steps 31-32 siclog@ f_TC_7_1_1_6_1_NR_TestBody2(v_DRBId,v_TimingDL,10); //10*80 =800 ms is assumed to be sufficient to perfor RRC Reconfiguration f_NR_CellInfo_SetDownlinkBWP_List(nr_Cell1, v_NR_DownlinkBWP_List); f_NR_SS_CommonCellConfig(nr_Cell1, cads_NR_DownlinkBWP_ConfigCommon_REQ(nr_Cell1, -, v_NR_DownlinkBWP_List)); f_NR_TestBody_Set(false); // Send Trigger to EUTRA indicating Finish of Test Body f_EUTRA_NR_SendCoOrd(EUTRA, cms_EUTRA_NR_Trigger); //Wait from coordination AFTER the connection is released in E-UTRA f_EUTRA_NR_WaitForCoOrd_Trigger(EUTRA, "Postamble"); //Postamble f_NR_ReleaseAllCells(); };

## [Anritsu] Change 4

| Function name | function f_TC_7_1_1_6_1_NR_TestBody2 |
| --- | --- |
| Reason for change | Since Change 2 and 3 are performing SPS LocalRelease, the SPS-Config needs to be reactivated to be able to transmit Step 31 MAC PDU using DL SPS. Comment: The use of f_NR_DL_SpsAssignmentConfiguration() would lead to the transmission of a DL SPS Grant DCI (CS-RNTI), which is not mentioned in prose. Since the RRC is no longer configured for SPS, the MAC shall ignore this DCI. This preserves the relevant TP5: (5) with { UE in RRC_Connected state with DRB established and stored DL SPS grant to receive MAC PDU at slot z+n*[semiPersistSchedIntervalDL] } ensure that { when { UE receives a RRC Message including sps-Configuration with sps-ConfigurationDL set as ‘disable’ and hence resulting in DL SPS grant deactivation } then { UE deletes the stored sps-Configuration DL parameters and stops receiving DL MAC PDU’s as per stored SPS assignment in slot z+n*[semiPersistSchedIntervalDL] } } |
| Summary of change | Reactivate DL SPS to enable transmission of Step 31 MAC PDU via CS-RNTI |
| TTCN module | MAC_SPS_TC_Common_NR.ttcn |

Before Change:

function f_TC_7_1_1_6_1_NR_TestBody2(DRB_Identity p_NR_DRB_Id, SubFrameTiming_Type p_TimingDL, integer p_X) runs on NR_BASE_PTC { const integer const_SPS_Period :=80; // period in milli seconds var NR_MAC_SDU_Type v_EncodedRlcPdu1; var template (value) NR_MAC_SDU_SubPDU_List_Type v_SDU_SubPDUListDL; var template (value) NR_MAC_PDUList_Type v_MAC_PDUListDL; var SubFrameTiming_Type v_TimingDL := p_TimingDL; var SubFrameTiming_Type v_TimingUL_1; var SubcarrierSpacing v_SCS := f_NR_CellInfo_GetSCS(nr_Cell1); // K1 = dl-DataToUL-ACK =110 = 8 slots, TS 38.508-1 Table 4.6.3-112; timer t_Watchdog := 1.0; v_TimingDL := f_SubFrameTiming_AddMilliSeconds(v_TimingDL,p_X * const_SPS_Period); v_TimingUL_1:= v_TimingDL; select(v_SCS) { case(kHz15) { v_TimingDL.Subframe.Number := 1; v_TimingUL_1.Subframe.Number := 9; } case(kHz30) { v_TimingDL.Subframe.Number := 0; v_TimingDL.Slot.SlotOffset.Numerology1 := 0; //@sic R5s221197 sic@ v_TimingUL_1.Subframe.Number := 4; v_TimingUL_1.Slot.SlotOffset.Numerology1 := 0; } case(kHz120) { v_TimingDL.Subframe.Number := 0; v_TimingDL.Slot.SlotOffset.Numerology3 := 1; v_TimingUL_1.Subframe.Number := 1; v_TimingUL_1.Slot.SlotOffset.Numerology3 := 1; } case else { FatalError(__FILE__, __LINE__, "Invalid SubCarrierSapcing"); } } f_NR_SS_SystemIndCtrlConfig(nr_Cell1, cds_NR_SystemIndCtrl_UL_HARQ(enable)); // @siclog "Step 31" siclog@ // PDCP SDU =38B +3B PDCP Header+ 1B RLC UM Header + 2B MAC Header =44B=352 bits (an allowed TB size as per 38.523-3 Annex B) v_EncodedRlcPdu1 := f_NR_RLC_UMD_FullPDU_Encvalue(f_NR_PDCP_PDU_Encvalue(f_NR_ReturnDefaultPDCP_PDU (7, crs_NR_PDCP_SDU_38B, 1)));//@sic R5-227518 sic@ v_SDU_SubPDUListDL := { cs_NR_MAC_SDU_SubPDU_LI8(tsc_LCID_SCG_Bearer, v_EncodedRlcPdu1) }; v_MAC_PDUListDL := { cs_NR_DL_MAC_PDU_SDU_SubPDU_Padding(v_SDU_SubPDUListDL) }; DRB.send(cads_NR_DRB_COMMON_REQ_MAC_PDUList_NoCRNTI_Grant(nr_Cell1, p_NR_DRB_Id, cs_TimingInfo_NR( v_TimingDL), v_MAC_PDUListDL)); v_TimingDL := f_SubFrameTiming_AddMilliSeconds(v_TimingDL,10); //10ms later f_NR_SpsCgDeactivate(nr_Cell1, cs_TimingInfo_NR( v_TimingDL),cs_NR_SpsCgDeactivate_LclRel); // @siclog "Step 32" siclog@ t_Watchdog.start; alt { [] SYSIND.receive(car_NR_SYSTEM_UL_HARQ_IND_Timing(nr_Cell1, ? , cr_TimingInfo_NR( v_TimingUL_1) )) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 32"); t_Watchdog.stop; } []t_Watchdog.timeout { f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 32"); } } f_NR_SS_SystemIndCtrlConfig(nr_Cell1, cds_NR_SystemIndCtrl_UL_HARQ(disable)); };

After Change:

function f_TC_7_1_1_6_1_NR_TestBody2(DRB_Identity p_NR_DRB_Id, SubFrameTiming_Type p_TimingDL, integer p_X) runs on NR_BASE_PTC { const integer const_SPS_Period :=80; // period in milli seconds var NR_MAC_SDU_Type v_EncodedRlcPdu1; var template (value) NR_MAC_SDU_SubPDU_List_Type v_SDU_SubPDUListDL; var template (value) NR_MAC_PDUList_Type v_MAC_PDUListDL; var SubFrameTiming_Type v_TimingDL := p_TimingDL; var SubFrameTiming_Type v_TimingUL_1; var SubcarrierSpacing v_SCS := f_NR_CellInfo_GetSCS(nr_Cell1); var integer v_I_MCS; // Modulation and Coding Scehme Index var integer v_N_PRB; // Number of physical resource Blocks // K1 = dl-DataToUL-ACK =110 = 8 slots, TS 38.508-1 Table 4.6.3-112; timer t_Watchdog := 1.0; // Re-nitialise Nprb and Imcs for TBsize=352 if ( f_NR_CellInfo_GetIsFR1(nr_Cell1)) {v_N_PRB := 7; v_I_MCS:= 3;} //TS 38.508-1 Table 4.6.3-51 Table 4.6.3-51 =1, so 38.523-3 Table B.1.1.2-3 else {v_N_PRB :=11 ; v_I_MCS:= 0}//TS 38.508-1 Table 4.6.3-51 Table 4.6.3-51 =0, so 38.523-3 Table B.1.1.1-3 v_TimingDL := f_SubFrameTiming_AddMilliSeconds(v_TimingDL,p_X * const_SPS_Period); v_TimingUL_1:= v_TimingDL; select(v_SCS) { case(kHz15) { v_TimingDL.Subframe.Number := 1; v_TimingUL_1.Subframe.Number := 9; } case(kHz30) { v_TimingDL.Subframe.Number := 0; v_TimingDL.Slot.SlotOffset.Numerology1 := 0; //@sic R5s221197 sic@ v_TimingUL_1.Subframe.Number := 4; v_TimingUL_1.Slot.SlotOffset.Numerology1 := 0; } case(kHz120) { v_TimingDL.Subframe.Number := 0; v_TimingDL.Slot.SlotOffset.Numerology3 := 1; v_TimingUL_1.Subframe.Number := 1; v_TimingUL_1.Slot.SlotOffset.Numerology3 := 1; } case else { FatalError(__FILE__, __LINE__, "Invalid SubCarrierSapcing"); } } f_NR_SS_SystemIndCtrlConfig(nr_Cell1, cds_NR_SystemIndCtrl_UL_HARQ(enable)); // @siclog "Step 31" siclog@ f_NR_DL_SpsAssignmentConfiguration(nr_Cell1,cs_TimingInfo_NR( v_TimingDL),v_N_PRB,v_I_MCS); // PDCP SDU =38B +3B PDCP Header+ 1B RLC UM Header + 2B MAC Header =44B=352 bits (an allowed TB size as per 38.523-3 Annex B) v_EncodedRlcPdu1 := f_NR_RLC_UMD_FullPDU_Encvalue(f_NR_PDCP_PDU_Encvalue(f_NR_ReturnDefaultPDCP_PDU (7, crs_NR_PDCP_SDU_38B, 1)));//@sic R5-227518 sic@ v_SDU_SubPDUListDL := { cs_NR_MAC_SDU_SubPDU_LI8(tsc_LCID_SCG_Bearer, v_EncodedRlcPdu1) }; v_MAC_PDUListDL := { cs_NR_DL_MAC_PDU_SDU_SubPDU_Padding(v_SDU_SubPDUListDL) }; DRB.send(cads_NR_DRB_COMMON_REQ_MAC_PDUList_NoCRNTI_Grant(nr_Cell1, p_NR_DRB_Id, cs_TimingInfo_NR( v_TimingDL), v_MAC_PDUListDL)); v_TimingDL := f_SubFrameTiming_AddMilliSeconds(v_TimingDL,10); //10ms later f_NR_SpsCgDeactivate(nr_Cell1, cs_TimingInfo_NR( v_TimingDL),cs_NR_SpsCgDeactivate_LclRel); // @siclog "Step 32" siclog@ t_Watchdog.start; alt { [] SYSIND.receive(car_NR_SYSTEM_UL_HARQ_IND_Timing(nr_Cell1, ? , cr_TimingInfo_NR( v_TimingUL_1) )) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 32"); t_Watchdog.stop; } []t_Watchdog.timeout { f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 32"); } } f_NR_SS_SystemIndCtrlConfig(nr_Cell1, cds_NR_SystemIndCtrl_UL_HARQ(disable)); };