3GPP TSG-RAN5 Meeting #2024-TTCN email	R5s240277

# Online, , 8th Dec 2023 - 31st Dec 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.523-3 | CR | 4804 | rev | - | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to NBIOT testcase 22.1.1 (for NTN) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI13_Test, NB_IOT-UEConTest |  |  |  |  |  | Date: |  |  | 2024-03-28 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Current test case presents to issues to be fixed: - According to the Table 22.1.1.3.2-1 in 36.523-1, the TTCN code does not take the Koffset of NTN into account between step 16a4C and step 16a5, add delay 1s to ensure the UE get the RRC Release message. - According to the Table 22.1.1.3-5 in 36.523-1, the IE of “Extended protocol configuration options” can be omit. So need to consider this situation in f_NBIOT_MobileInfo_GetExtdPCO(). |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Added 1s delay between step 16a4C and step 16a5. - Assigned omit as default. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may fail the test case. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 22.1.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Table of Contents

Table of Contents	2

1	Overview	2

2	Corrections required	2

2.1	Correction to function f_TC_22_1_1_Module3	2

2.2	Correction to function f_NBIOT_MobileInfo_GetExtdPCO	3

# Overview

This document lists all the essential changes needed to correct problems in the iwd-TTCN3-B2022-09_D24wk12 related to the title of this CR.

Contact: 	Juan Garcia Martin

Juan.GarciaMartin@rohde-schwarz.com

# Corrections required

## Correction to function f_TC_22_1_1_Module3

| Function name | f_TC_22_1_1_Module3 |
| --- | --- |
| Reason for change | According to the Table 22.1.1.3.2-1 in 36.523-1, the TTCN code does not take the Koffset of NTN into account between step 16a4C and step 16a5, add delay 1s to ensure the UE get the RRC Release message. |
| Summary of change | Added 1s delay between step 16a4C and step 16a5. |
| TTCN module | NBIOT_General_CP |
| MCC160 Comment | Rejected as not needed, the NTN delay is already being considered inside f_NBIOT_RRC_ConnectionRelease_Common, it’s been implemented as per R5s230706 and R5-237436 |

Before Change:

<<SKIPPED CODE>> //@siclog "Step 16a4C" siclog@ //The SS transmits a CP-ACK encapsulated in a Downlink NAS transport message to the UE. v_NAS_TxMessageContainer_SMS := cs_NAS_MsgContainer_SMS (f_SMS_PDU_Encvalue(cs_CP_ACK_PDU (cs_MT_TI0_FromSS))); SRB.send(cas_NB_SRB_NasPdu_REQ(nbiot_Cell50, tsc_SRB1bis, cs_TimingInfo_Now, cs_NAS_Request (tsc_SHT_IntegrityProtected_Ciphered, cs_SMS_DL_NAS_Transport (tsc_SHT_NoSecurityProtection, v_NAS_TxMessageContainer_SMS)))); //@siclog "Step 16a5" siclog@ //The SS transmits an RRCConnectionRelease-NB message. f_NBIOT_RRC_ConnectionRelease (nbiot_Cell50); //@siclog "Step 16a6" siclog@ //Wait for the time set in the CLOSE UE TEST LOOP to expire. t_LoopDelay.timeout; <<SKIPPED CODE>>

After Change:

<<SKIPPED CODE>> //@siclog "Step 16a4C" siclog@ //The SS transmits a CP-ACK encapsulated in a Downlink NAS transport message to the UE. v_NAS_TxMessageContainer_SMS := cs_NAS_MsgContainer_SMS (f_SMS_PDU_Encvalue(cs_CP_ACK_PDU (cs_MT_TI0_FromSS))); SRB.send(cas_NB_SRB_NasPdu_REQ(nbiot_Cell50, tsc_SRB1bis, cs_TimingInfo_Now, cs_NAS_Request (tsc_SHT_IntegrityProtected_Ciphered, cs_SMS_DL_NAS_Transport (tsc_SHT_NoSecurityProtection, v_NAS_TxMessageContainer_SMS)))); //@siclog "Step 16a5" siclog@ //The SS transmits an RRCConnectionRelease-NB message. if (pc_NB_ntn_Connectivity_EPC or pc_NB_ntn_only_Connectivity_EPC) { f_Delay(1.0); } f_NBIOT_RRC_ConnectionRelease (nbiot_Cell50); //@siclog "Step 16a6" siclog@ //Wait for the time set in the CLOSE UE TEST LOOP to expire. t_LoopDelay.timeout; <<SKIPPED CODE>>

## Correction to function f_NBIOT_MobileInfo_GetExtdPCO

| Function name | f_NBIOT_MobileInfo_GetExtdPCO |
| --- | --- |
| Reason for change | According to the Table 22.1.1.3-5 in 36.523-1, the IE of “Extended protocol configuration options” can be omit. So need to consider this situation in f_NBIOT_MobileInfo_GetExtdPCO |
| Summary of change | Assigned omit as default. |
| TTCN module | NBIOT_CellInfo |
| MCC160 Comment | Accepted but just need to return global variable. Don’t need if statement |

Before Change:

function f_NBIOT_MobileInfo_GetExtdPCO(integer p_PDN := 0) runs on NBIOT_PTC return ExtdProtocolConfigOptions { return vc_NBIOT_Global.MobileInfo.PDNs[p_PDN].protocolConfigurationOptions; }

After Change:

function f_NBIOT_MobileInfo_GetExtdPCO(integer p_PDN := 0) runs on NBIOT_PTC return template (omit) ExtdProtocolConfigOptions { var template (omit) ExtdProtocolConfigOptions p_PCO := omit; if (isvalue(vc_NBIOT_Global.MobileInfo.PDNs[p_PDN].protocolConfigurationOptions)) { p_PCO := vc_NBIOT_Global.MobileInfo.PDNs[p_PDN].protocolConfigurationOptions; } return p_PCO; }