3GPP TSG-RAN5 Meeting #2024-TTCN email	R5s240278

# Online, , 8th Dec 2023 - 31st Dec 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 3408 | rev | - | Current version: | 17.10.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to NR5GC test case 6.1.2.13 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2024-03-28 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | - As per the agreed prose R5-241498 preamble part UE has to be in NR RRC Idle mode (state 1N-A) on NR Cell 1 according to TS 38.508-1 [4] Table 4.4A.2-1. But, it seems unwanted steps are still left over in 24wk12 delivery. - After Step 4 UE would be camped on NR Cell 12 but postamble step uses NR Cell 1. - At Step 4 SS has to change the SIB1 of NR Cell 12 according to Table 6.1.2.13.3.3-1 to set the cellReservedForOperatorUse as notReserved but this step was missing in current TTCN implementation. Note: A Prose CR will also be raised to specify this step explicitly in prose step 4 so that it is followed in implementation correctly. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Removed the UE switch off(f_NR5GC_SwitchOnAndResetIMS), f_NR5GC_Registration, f_NR5GC_RRC_Idle_Steps16_20 and f_NR_RRCRelease() steps as these are not needed now as per the agreed prose. - Replaced NR Cell 1 with NR Cell 12 in f_NR_Postamble(). - Added f_NR_ModifySysinfo to update the SIB1 of NR Cell 12 without enabling send Short Message indication as it would ensure UE would follow Step 5 after the expiry of timer 300 seconds from Step 1 as SIB1 would be updated to reflect the change meanwhile. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may fail the test case. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.1.2.13.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS/TR 38.523-1 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | A Prose CR will be raised to support this TTCN CR. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	3

1	Overview	3

2	Corrections required	3

2.1	Correction to function f_TC_6_1_2_13_NR5GC	3

2.2	Correction to function f_TC_6_1_2_13_NR5GC_TestBody	5

# Overview

This document lists all the essential changes needed to correct problems in the iwd-TTCN3-B2022-09_D24wk12 related to the title of this CR.

Contact: 	Juan Garcia Martin

Juan.GarciaMartin@rohde-schwarz.com

# Corrections required

## Correction to function f_TC_6_1_2_13_NR5GC

| Function name | f_TC_6_1_2_13_NR5GC |
| --- | --- |
| Reason for change | - As per the agreed prose R5-241498 preamble part UE has to be in NR RRC Idle mode (state 1N-A) on NR Cell 1 according to TS 38.508-1 [4] Table 4.4A.2-1. But, it seems unwanted steps are still left over in 24wk12 delivery. - After Step 4 UE would be camped on NR Cell 12 but postamble step uses NR Cell 1. |
| Summary of change | - Removed the UE switch off(f_NR5GC_SwitchOnAndResetIMS), f_NR5GC_Registration, f_NR5GC_RRC_Idle_Steps16_20 and f_NR_RRCRelease() steps as these are not needed now as per the agreed prose. - Replaced NR Cell 1 with NR Cell 12 in f_NR_Postamble(). |
| TTCN module | Idle_CellReSelection_NR5GC |
| MCC160 Comment | 1.- Accepted 2.- Accepted |

Before Change:

function f_TC_6_1_2_13_NR5GC() runs on NR5GC_PTC {// Cell reselection using cell status and cell reservations / Access Identity 0, 1, 2 and 12 to 14 - cellReservedForOperatorUse // @sic R5-197020 R5-204416 R5-205366 sic@ f_NR5GC_Init(NR_4); // System information combination NR-4 as defined in TS 38.508-1 clause 4.4.3.1.2 is used in NR Cells f_NR_CellInfo_SetTAC (nr_Cell12, int2bit(2,24)); // NR Cell 1 and NR Cell 12 with different tracking areas f_NR_CellInfo_SetPLMN(nr_Cell12, tsc_NR_HPLMN_Def); f_NR_CellInfo_SetCellReservedForOperatorUse(nr_Cell12, -, reserved ); // Table 6.1.2.13.3.3-1: SIB1 for NR Cell 12 //Set maximum cell power level for the Cells to be used f_NR_CellInfo_InitMaxReferencePower(nr_Cell12, -80, -82); // Create and configure cell f_NR_CellConfig_Def(nr_Cell1); f_NR_CellConfig_Def(nr_Cell12); // Preamble: The UE is in state 1N-A on NR Cell 1(serving cell) according to TS 38.508-1 Table 4.4A.2-1 f_NR5GC_Preamble(nr_Cell1, STATE_IDLE_1A); f_NR_SetCellPower (nr_Cell1, -88, -91); //f_NR_UE_DeRegisterOnSwitchOff (nr_Cell1, STATE_IDLE_1A); // @sic R5-241498 sic@ f_NR5GC_SwitchOnAndResetIMS(); //(fully) switch on the UE // Preamble: The UE is in state 1N-A on NR Cell 1(serving cell) according to TS 38.508-1 Table 4.4A.2-1 f_NR5GC_Registration(nr_Cell1); f_NR5GC_RRC_Idle_Steps16_20 (nr_Cell1); // @sic R5s200751 sic@ f_NR_RRCRelease(nr_Cell1); // procedure done on preamble so release connection to complete f_NR_TestBody_Set(true); f_TC_6_1_2_13_NR5GC_TestBody(); f_NR_TestBody_Set(false); // Switch/Power off UE f_NR_Postamble(nr_Cell1, STATE_IDLE_1A); } //end of f_TC_6_1_2_13_NR5GC

After Change:

function f_TC_6_1_2_13_NR5GC() runs on NR5GC_PTC {// Cell reselection using cell status and cell reservations / Access Identity 0, 1, 2 and 12 to 14 - cellReservedForOperatorUse // @sic R5-197020 R5-204416 R5-205366 sic@ f_NR5GC_Init(NR_4); // System information combination NR-4 as defined in TS 38.508-1 clause 4.4.3.1.2 is used in NR Cells f_NR_CellInfo_SetTAC (nr_Cell12, int2bit(2,24)); // NR Cell 1 and NR Cell 12 with different tracking areas f_NR_CellInfo_SetPLMN(nr_Cell12, tsc_NR_HPLMN_Def); f_NR_CellInfo_SetCellReservedForOperatorUse(nr_Cell12, -, reserved ); // Table 6.1.2.13.3.3-1: SIB1 for NR Cell 12 //Set maximum cell power level for the Cells to be used f_NR_CellInfo_InitMaxReferencePower(nr_Cell12, -80, -82); // Create and configure cell f_NR_CellConfig_Def(nr_Cell1); f_NR_CellConfig_Def(nr_Cell12); // Preamble: The UE is in state 1N-A on NR Cell 1(serving cell) according to TS 38.508-1 Table 4.4A.2-1 f_NR5GC_Preamble(nr_Cell1, STATE_IDLE_1A); f_NR_SetCellPower (nr_Cell1, -88, -91); //f_NR_UE_DeRegisterOnSwitchOff (nr_Cell1, STATE_IDLE_1A); // @sic R5-241498 sic@ // f_NR5GC_SwitchOnAndResetIMS(); //(fully) switch on the UE WA#WA=1268716 // Preamble: The UE is in state 1N-A on NR Cell 1(serving cell) according to TS 38.508-1 Table 4.4A.2-1 //f_NR5GC_Registration(nr_Cell1); //f_NR5GC_RRC_Idle_Steps16_20 (nr_Cell1); // @sic R5s200751 sic@ //f_NR_RRCRelease(nr_Cell1); // procedure done on preamble so release connection to complete f_NR_TestBody_Set(true); f_TC_6_1_2_13_NR5GC_TestBody(); f_NR_TestBody_Set(false); // Switch/Power off UE f_NR_Postamble(nr_Cell12, STATE_IDLE_1A); } //end of f_TC_6_1_2_13_NR5GC

## Correction to function f_TC_6_1_2_13_NR5GC_TestBody

| Function name | f_TC_6_1_2_13_NR5GC_TestBody |
| --- | --- |
| Reason for change | At Step 4 SS has to change the SIB1 of NR Cell 12 according to Table 6.1.2.13.3.3-1 to set the cellReservedForOperatorUse as notReserved but this step was missing in current TTCN implementation. Note: A Prose CR will also be raised to specify this step explicitly in prose step 4 so that it is followed in implementation correctly. |
| Summary of change | Added f_NR_ModifySysinfo to update the SIB1 of NR Cell 12 without enabling Send Short Message indication as it would ensure UE would follow Step 5 after the expiry of timer 300 seconds from Step 1 as SIB1 would be updated to reflect the change meanwhile. |
| TTCN module | Idle_CellReSelection_NR5GC |
| MCC160 Comment | Accepted |

Before Change:

function f_TC_6_1_2_13_NR5GC_TestBody() runs on NR5GC_PTC { // @sic R5-241498 sic@ var integer v_T1_Cell12_RS_EPRE_FR1 := -80; var integer v_T1_Cell12_RS_EPRE_FR2 := -82; var template (value) NR_CellPowerList_Type v_CellPowerList_AtT1; timer t_Wait_300 := 300.0; v_CellPowerList_AtT1:={ cs_NR_CellPower(nr_Cell12, v_T1_Cell12_RS_EPRE_FR1, v_T1_Cell12_RS_EPRE_FR2) }; // @siclog "Step 1" siclog@ // The SS adjusts the SS/PBCH EPRE levels according to row "T1" in table 6.1.2.13.3.2-1/2. f_NR_SetCellPowerList(v_CellPowerList_AtT1); t_Wait_300.start; // @siclog "Step 2" siclog@ // Check: Does the UE initiate a random access procedure on NR Cell 12 within the next 120s? f_NR_CheckNoRRCSetupReq(nr_Cell12, 120.0); // @siclog "Step 3" siclog@ // Check: Does the test result of generic test procedure in TS 38.508-1 Table 4.9.4.2.2-1 indicate that check UE is in state NR RRC_IDLE on NR Cell 1? f_NR5GC_CheckRRCIdle(nr_Cell1); // @siclog "Step 4" siclog@ // The SS adjusts the SS/PBCH EPRE levels according to row "T2" in table 6.1.2.13.3.2-1/2 f_NR_CellInfo_SetCellReservedForOperatorUse(nr_Cell12); // Table 6.1.2.13.3.3-1: SIB1 for NR Cell 12 // @siclog "Step 5" siclog@ // Check: Does the test result of generic test procedure in TS 38.508-1 subclause 4.9.5.2.2-1 indicate that the UE is camped on NR Cell 12 after 300 seconds from Step 1? // NOTE: The UE shall exclude the barred cell 12 as a candidate for cell selection/reselection for 300 seconds after Step 1. t_Wait_300.timeout; // reselection shall not happen before expiry of 300s timer f_NR5GC_MobilityRegistration (nr_Cell12); f_NR_PreliminaryPass(__FILE__, __LINE__, "Test Case 6.1.2.13 Step 5"); }//end of f_TC_6_1_2_13_NR5GC_TestBody

After Change:

function f_TC_6_1_2_13_NR5GC_TestBody() runs on NR5GC_PTC { // @sic R5-241498 sic@ var integer v_T1_Cell12_RS_EPRE_FR1 := -80; var integer v_T1_Cell12_RS_EPRE_FR2 := -82; var template (value) NR_CellPowerList_Type v_CellPowerList_AtT1; timer t_Wait_300 := 300.0; v_CellPowerList_AtT1:={ cs_NR_CellPower(nr_Cell12, v_T1_Cell12_RS_EPRE_FR1, v_T1_Cell12_RS_EPRE_FR2) }; // @siclog "Step 1" siclog@ // The SS adjusts the SS/PBCH EPRE levels according to row "T1" in table 6.1.2.13.3.2-1/2. f_NR_SetCellPowerList(v_CellPowerList_AtT1); t_Wait_300.start; // @siclog "Step 2" siclog@ // Check: Does the UE initiate a random access procedure on NR Cell 12 within the next 120s? f_NR_CheckNoRRCSetupReq(nr_Cell12, 120.0); // @siclog "Step 3" siclog@ // Check: Does the test result of generic test procedure in TS 38.508-1 Table 4.9.4.2.2-1 indicate that check UE is in state NR RRC_IDLE on NR Cell 1? f_NR5GC_CheckRRCIdle(nr_Cell1); // @siclog "Step 4" siclog@ // The SS adjusts the SS/PBCH EPRE levels according to row "T2" in table 6.1.2.13.3.2-1/2 f_NR_CellInfo_SetCellReservedForOperatorUse(nr_Cell12); // Table 6.1.2.13.3.3-1: SIB1 for NR Cell 12 f_NR_ModifySysinfo(nr_Cell12, false); // Modify Sysinfo in SS // @siclog "Step 5" siclog@ // Check: Does the test result of generic test procedure in TS 38.508-1 subclause 4.9.5.2.2-1 indicate that the UE is camped on NR Cell 12 after 300 seconds from Step 1? // NOTE: The UE shall exclude the barred cell 12 as a candidate for cell selection/reselection for 300 seconds after Step 1. t_Wait_300.timeout; // reselection shall not happen before expiry of 300s timer f_NR5GC_MobilityRegistration (nr_Cell12); f_NR_PreliminaryPass(__FILE__, __LINE__, "Test Case 6.1.2.13 Step 5"); }//end of f_TC_6_1_2_13_NR5GC_TestBody