3GPP TSG-RAN5 Meeting #2024-TTCN email	R5s240280

# Online, , 8th Dec 2023 - 31st Dec 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 3409 | rev | - | Current version: | 17.10.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to NR5GC test case 8.1.1.1a.3 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI17_Test, NR_UE_pow_sav_enh_plus_CT-UEConTest |  |  |  |  |  | Date: |  |  | 2024-03-28 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Following issues has been found and needs to be corrected: - In function f_TC_8_1_1_1a_3_NR5GC, f_NR_CellInfo_SetSIB1_PEI_Config_r17 is not assuring to not include subgroupsNumForUEID-r17 contents as indicated in Table 8.1.1.1a.3.3.3-3 cs_SubgroupConfig_r17 information. - In function fl_TC_8_1_1_1a_3_TestBody, call for f_NR_CellInfo_SetSIB1_PEI_Config_r17 is introduced in the place where RRCSetup is supposed to go. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Corrected function f_TC_8_1_1_1a_3_NR5GC to not include subgroupsNumForUEID-r17 in SIB1 as indicated in Table 8.1.1.1a.3.3.3-3. - Called f_NR_RRCSetup_Def function at Step 3. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may fail the test case |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.1.1.1a.3.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	2

2	Corrections required	2

2.1	Correction to function f_TC_8_1_1_1a_3_NR5GC	2

2.2	Correction to function fl_TC_8_1_1_1a_3_TestBody	3

# Overview

This document lists all the essential changes needed to correct problems in the iwd-TTCN3-B2022-09_D24wk12 related to the title of this CR.

Contact: 	Juan Garcia Martin

Juan.GarciaMartin@rohde-schwarz.com

# Corrections required

## Correction to function f_TC_8_1_1_1a_3_NR5GC

| Function name | f_TC_8_1_1_1a_3_NR5GC |
| --- | --- |
| Reason for change | In function f_TC_8_1_1_1a_3_NR5GC, f_NR_CellInfo_SetSIB1_PEI_Config_r17 is not assuring to not include subgroupsNumForUEID-r17 contents as indicated in Table 8.1.1.1a.3.3.3-3 cs_SubgroupConfig_r17 information. |
| Summary of change | Corrected function f_TC_8_1_1_1a_3_NR5GC to not include subgroupsNumForUEID-r17 in SIB1 as indicated in Table 8.1.1.1a.3.3.3-3. |
| TTCN module | RRC_PEI_NR5GC |
| MCC160 Comment | Accepted |

Before Change:

function f_TC_8_1_1_1a_3_NR5GC() runs on NR5GC_PTC { //Paging Early Indication without Subgrouping / RRC_IDLE f_NR5GC_Init(NR_1); //SIB1 will not include subgroupsNumForUEID-r17 as indicated in Table 8.1.1.1a.3.3.3-3 f_NR_CellInfo_SetSIB1_PEI_Config_r17(nr_Cell1); //Create and configure cell f_NR_CellConfig_Def(nr_Cell1); f_NR5GC_Preamble(nr_Cell1, STATE_IDLE_1A); f_NR_TestBody_Set(true); fl_TC_8_1_1_1a_3_TestBody(); f_NR_TestBody_Set(false); f_NR_Postamble(nr_Cell1, STATE_CONNECTED_3A); }

After Change:

function f_TC_8_1_1_1a_3_NR5GC() runs on NR5GC_PTC { //Paging Early Indication without Subgrouping / RRC_IDLE f_NR5GC_Init(NR_1); //SIB1 will not include subgroupsNumForUEID-r17 as indicated in Table 8.1.1.1a.3.3.3-3 f_NR_CellInfo_SetSIB1_PEI_Config_r17(nr_Cell1, cs_SubgroupConfig_r17(-, omit)); //Create and configure cell f_NR_CellConfig_Def(nr_Cell1); f_NR5GC_Preamble(nr_Cell1, STATE_IDLE_1A); f_NR_TestBody_Set(true); fl_TC_8_1_1_1a_3_TestBody(); f_NR_TestBody_Set(false); f_NR_Postamble(nr_Cell1, STATE_CONNECTED_3A); }

## Correction to function fl_TC_8_1_1_1a_3_TestBody

| Function name | fl_TC_8_1_1_1a_3_TestBody |
| --- | --- |
| Reason for change | In function fl_TC_8_1_1_1a_3_TestBody, call for f_NR_CellInfo_SetSIB1_PEI_Config_r17 is introduced in the place where RRCSetup is supposed to go. |
| Summary of change | Called f_NR_RRCSetup_Def function at Step 3. |
| TTCN module | RRC_PEI_NR5GC |
| MCC160 Comment | Accepted |

Before Change:

function fl_TC_8_1_1_1a_3_TestBody() runs on NR5GC_PTC { /* @sic R5s240109 sic@ */ var NG_NAS_MSG_Indication_Type v_ReceivedMsg; //@siclog "Step 1" siclog@ //The SS transmits a Paging message f_NR_Paging_Def(nr_Cell1, RRC_IDLE); //@siclog "Step 2" siclog@ //Check: Does the UE transmit an RRCSetupRequest message? f_NR_RRC_SetupRequest_Def(nr_Cell1, mt_Access); f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 2"); //@siclog "Step 3" siclog@ //The SS transmits an RRCSetup message. f_NR_CellInfo_SetSIB1_PEI_Config_r17(nr_Cell1, cs_SubgroupConfig_r17(-, omit)); //@siclog "Step 4" siclog@ //The UE transmit an RRCSetupComplete message including SERVICE REQUEST to confirm the successful completion of the connection establishment. v_ReceivedMsg := f_NR_RRCSetupComplete_Def(nr_Cell1, cr_38508_RRCSetupComplete(tsc_NR_RRC_TI_Def), tsc_SHT_IntegrityProtected); //@siclog "Step 5-8" siclog@ //Steps 5 to 8 of the NR RRC_CONNECTED procedure in TS 38.508-1 Table 4.5.4.2-3 are executed to successfully complete the service request procedure. f_NR5GC_RRC_ConnectedState3N_Steps5_8(nr_Cell1, v_ReceivedMsg); }

After Change:

function fl_TC_8_1_1_1a_3_TestBody() runs on NR5GC_PTC { /* @sic R5s240109 sic@ */ var NG_NAS_MSG_Indication_Type v_ReceivedMsg; //@siclog "Step 1" siclog@ //The SS transmits a Paging message f_NR_Paging_Def(nr_Cell1, RRC_IDLE); //@siclog "Step 2" siclog@ //Check: Does the UE transmit an RRCSetupRequest message? f_NR_RRC_SetupRequest_Def(nr_Cell1, mt_Access); f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 2"); //@siclog "Step 3" siclog@ //The SS transmits an RRCSetup message. f_NR_RRCSetup_Def (nr_Cell1); //@siclog "Step 4" siclog@ //The UE transmit an RRCSetupComplete message including SERVICE REQUEST to confirm the successful completion of the connection establishment. v_ReceivedMsg := f_NR_RRCSetupComplete_Def(nr_Cell1, cr_38508_RRCSetupComplete(tsc_NR_RRC_TI_Def), tsc_SHT_IntegrityProtected); //@siclog "Step 5-8" siclog@ //Steps 5 to 8 of the NR RRC_CONNECTED procedure in TS 38.508-1 Table 4.5.4.2-3 are executed to successfully complete the service request procedure. f_NR5GC_RRC_ConnectedState3N_Steps5_8(nr_Cell1, v_ReceivedMsg; }