3GPP TSG-RAN5 Meeting #2024-TTCN email	R5s240283

# Online, , 8th Dec 2023 - 31st Dec 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 3411 | rev | - | Current version: | 17.10.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to NR5GC test case 7.1.1.8.1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2024-03-28 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The BWP switching at Steps 5, 9, 16 and 20 are being done with immediate activation time. There is not enough time for the RLC STATUS PDUs to be transmitted on the BWP where the loopback of data happened, before the BWP switch occurs, leading to a situation where the BWP has been switched and UE has not received the RLC STATUS PDU leading it to re-transmit the same data on the UL again. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added a delay of 100ms before the BWP switch at Steps 5, 9, 16 and 20 to ensure the RLC STATUS PDU is received by the UE. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may fail the test case. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.1.1.8.1.ENDC, 7.1.1.8.1.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	2

2	Corrections required	2

2.1	Correction to function f_TC_7_1_1_8_1_NR_CommonTestBody	2

# Overview

This document lists all the essential changes needed to correct problems in the iwd-TTCN3-B2022-09_D24wk12 related to the title of this CR.

Contact: 	Juan Garcia Martin

Juan.GarciaMartin@rohde-schwarz.com

# Corrections required

## Correction to function f_TC_7_1_1_8_1_NR_CommonTestBody

| Function name | f_TC_7_1_1_8_1_NR_CommonTestBody |
| --- | --- |
| Reason for change | The BWP switching at Steps 5, 9, 16 and 20 are being done with immediate activation time. There is not enough time for the RLC STATUS PDUs to be transmitted on the BWP where the loopback of data happened, before the BWP switch occurs, leading to a situation where the BWP has been switched and UE has not received the RLC STATUS PDU leading it to re-transmit the same data on the UL again |
| Summary of change | Added a delay of 100ms before the BWP switch at Steps 5, 9, 16 and 20 to ensure the RLC STATUS PDU is received by the UE. |
| TTCN module | MAC_BWP_Common_NR |
| MCC160 | Accepted in primciple as improvement but we never use delays unless they are specified in the TC prose. Instead, it is proposed to use activation time when reconfiguring the SS. See MCC160 proposed implementation below. The delay in step 20 needs to be set t0 1200ms. |

Before Change:

function f_TC_7_1_1_8_1_NR_CommonTestBody(NR_CellId_Type p_NR_CellId, DRB_Identity p_DrbId, template (value) NR_ResourceAllocation_Type p_ResourceAllocationUL) runs on NR_BASE_PTC { //@sic R5-213459 R5-227417 sic@ <<SKIPPED CODE>> // @siclog "Steps 3-3A" siclog@ fl_TC_7_1_1_8_1_ReceiveData(p_NR_CellId, p_DrbId, {v_RLC_SDU1}, true, v_SN); //@sic R5-231578 R5s230220 sic@ f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 3"); v_SN := v_SN + 1; if (v_BWP_SameNumerology_upto4) { //@sic R5s230634 sic@ f_NR_SS_ConfigureBWPs(p_NR_CellId, -, tsc_NR_BWP_Id_FirstActive, tsc_NR_BWP_Id_2, tsc_NR_BWP_Id_2, tsc_NR_BWP_Id_2); } else { f_NR_SS_ConfigureBWPs(p_NR_CellId, -, tsc_NR_BWP_Id_FirstActive, tsc_NR_BWP_Id_Initial, tsc_NR_BWP_Id_Initial, tsc_NR_BWP_Id_Initial); } // @siclog "Step 5" siclog@ v_TimingInfoDL := f_TC_7_1_1_8_1_NR_GetNextSendOccasion_DL(p_NR_CellId); v_RLC_SDU1 := fl_TC_7_1_1_8_1_SendData(p_NR_CellId, p_DrbId, cs_TimingInfo_NR(v_TimingInfoDL), v_SN); <<SKIPPED CODE>> // @siclog "Steps 7-7A" siclog@ fl_TC_7_1_1_8_1_ReceiveData(p_NR_CellId, p_DrbId, {v_RLC_SDU1}, true, v_SN); //@sic R5-231578 R5s230220 sic@ f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 7"); v_SN := v_SN + 1; //@sic R5s230220 change 9.2 sic@ if (v_BWP_SameNumerology_upto4) { //@sic R5-2134599 sic@ f_NR_SS_ConfigureBWPs_DL(p_NR_CellId, -, tsc_NR_BWP_Id_2); } else { f_NR_SS_ConfigureBWPs_DL(p_NR_CellId, -, tsc_NR_BWP_Id_Initial); } // @siclog "Step 9" siclog@ v_TimingInfoDL := f_TC_7_1_1_8_1_NR_GetNextSendOccasion_DL(p_NR_CellId, 300); v_RLC_SDU1 := fl_TC_7_1_1_8_1_SendData(p_NR_CellId, p_DrbId, cs_TimingInfo_NR(v_TimingInfoDL), v_SN); <<SKIPPED CODE>> // @siclog "Steps 13D-13E" siclog@ fl_TC_7_1_1_8_1_ReceiveData(p_NR_CellId, p_DrbId, {v_RLC_SDU1}, true, v_SN); //@sic R5-231578 R5s230220 sic@ f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 13D"); f_NR_SS_SystemIndCtrlConfig(p_NR_CellId, cds_NR_SystemIndCtrl_RachPreamble(disable)); //@sic R5s230634 sic@ v_SN := v_SN + 1; //@sic R5s230634 sic@ f_NR_SS_ConfigureBWPs(p_NR_CellId, -, tsc_NR_BWP_Id_Initial, tsc_NR_BWP_Id_1, tsc_NR_BWP_Id_1, tsc_NR_BWP_Id_1); // @siclog "Step 16" siclog@ v_TimingInfoDL := f_TC_7_1_1_8_1_NR_GetNextSendOccasion_DL(p_NR_CellId); //@sic R5s230220 change 10 sic@ v_TimingInfoConfig := f_SubFrameTiming_AddMilliSeconds(v_TimingInfoDL, 100); // 100ms after step 16 v_RLC_SDU1 := fl_TC_7_1_1_8_1_SendData(p_NR_CellId, p_DrbId, cs_TimingInfo_NR(v_TimingInfoDL), v_SN); v_SN := v_SN + 1; //@sic R5s230220 change 10 sic@ <<SKIPPED CODE>> // @siclog "Steps 19-19A" siclog@ fl_TC_7_1_1_8_1_ReceiveData(p_NR_CellId, p_DrbId, {v_RLC_SDU1, v_RLC_SDU2}, true, v_SN); //@sic R5-231578 R5s230220 sic@ v_SN := v_SN + 1; //@sic R5-216169 R5s230220 R5s230634 sic@ if (v_BWP_SameNumerology_upto4 ) { f_NR_SS_ConfigureBWPs(p_NR_CellId, -, tsc_NR_BWP_Id_2, tsc_NR_BWP_Id_2, tsc_NR_BWP_Id_2, tsc_NR_BWP_Id_2); } else { f_NR_SS_ConfigureBWPs(p_NR_CellId, -, tsc_NR_BWP_Id_Initial, tsc_NR_BWP_Id_Initial, tsc_NR_BWP_Id_Initial, tsc_NR_BWP_Id_Initial); } // @siclog "Step 20" siclog@ v_TimingInfoDL := f_SubFrameTiming_AddMilliSeconds(v_TimingInfoDL, 1400); // 1400ms after step 17 => 1000ms after step 18 v_RLC_SDU1 := fl_TC_7_1_1_8_1_SendData(p_NR_CellId, p_DrbId, cs_TimingInfo_NR(v_TimingInfoDL), v_SN); <<SKIPPED CODE>>

After Change:

function f_TC_7_1_1_8_1_NR_CommonTestBody(NR_CellId_Type p_NR_CellId, DRB_Identity p_DrbId, template (value) NR_ResourceAllocation_Type p_ResourceAllocationUL) runs on NR_BASE_PTC { //@sic R5-213459 R5-227417 sic@ <<SKIPPED CODE>> // @siclog "Steps 3-3A" siclog@ fl_TC_7_1_1_8_1_ReceiveData(p_NR_CellId, p_DrbId, {v_RLC_SDU1}, true, v_SN); //@sic R5-231578 R5s230220 sic@ f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 3"); v_SN := v_SN + 1; f_Delay (0.1); if (v_BWP_SameNumerology_upto4) { //@sic R5s230634 sic@ f_NR_SS_ConfigureBWPs(p_NR_CellId, -, tsc_NR_BWP_Id_FirstActive, tsc_NR_BWP_Id_2, tsc_NR_BWP_Id_2, tsc_NR_BWP_Id_2); } else { f_NR_SS_ConfigureBWPs(p_NR_CellId, -, tsc_NR_BWP_Id_FirstActive, tsc_NR_BWP_Id_Initial, tsc_NR_BWP_Id_Initial, tsc_NR_BWP_Id_Initial); } // @siclog "Step 5" siclog@ v_TimingInfoDL := f_TC_7_1_1_8_1_NR_GetNextSendOccasion_DL(p_NR_CellId); v_RLC_SDU1 := fl_TC_7_1_1_8_1_SendData(p_NR_CellId, p_DrbId, cs_TimingInfo_NR(v_TimingInfoDL), v_SN); <<SKIPPED CODE>> // @siclog "Steps 7-7A" siclog@ fl_TC_7_1_1_8_1_ReceiveData(p_NR_CellId, p_DrbId, {v_RLC_SDU1}, true, v_SN); //@sic R5-231578 R5s230220 sic@ f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 7"); v_SN := v_SN + 1; f_Delay (0.1); //@sic R5s230220 change 9.2 sic@ if (v_BWP_SameNumerology_upto4) { //@sic R5-2134599 sic@ f_NR_SS_ConfigureBWPs_DL(p_NR_CellId, -, tsc_NR_BWP_Id_2); } else { f_NR_SS_ConfigureBWPs_DL(p_NR_CellId, -, tsc_NR_BWP_Id_Initial); } // @siclog "Step 9" siclog@ v_TimingInfoDL := f_TC_7_1_1_8_1_NR_GetNextSendOccasion_DL(p_NR_CellId, 300); v_RLC_SDU1 := fl_TC_7_1_1_8_1_SendData(p_NR_CellId, p_DrbId, cs_TimingInfo_NR(v_TimingInfoDL), v_SN); <<SKIPPED CODE>> // @siclog "Steps 13D-13E" siclog@ fl_TC_7_1_1_8_1_ReceiveData(p_NR_CellId, p_DrbId, {v_RLC_SDU1}, true, v_SN); //@sic R5-231578 R5s230220 sic@ f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 13D"); f_NR_SS_SystemIndCtrlConfig(p_NR_CellId, cds_NR_SystemIndCtrl_RachPreamble(disable)); //@sic R5s230634 sic@ v_SN := v_SN + 1; f_Delay (0.1); //@sic R5s230634 sic@ f_NR_SS_ConfigureBWPs(p_NR_CellId, -, tsc_NR_BWP_Id_Initial, tsc_NR_BWP_Id_1, tsc_NR_BWP_Id_1, tsc_NR_BWP_Id_1); // @siclog "Step 16" siclog@ v_TimingInfoDL := f_TC_7_1_1_8_1_NR_GetNextSendOccasion_DL(p_NR_CellId); //@sic R5s230220 change 10 sic@ v_TimingInfoConfig := f_SubFrameTiming_AddMilliSeconds(v_TimingInfoDL, 100); // 100ms after step 16 v_RLC_SDU1 := fl_TC_7_1_1_8_1_SendData(p_NR_CellId, p_DrbId, cs_TimingInfo_NR(v_TimingInfoDL), v_SN); v_SN := v_SN + 1; //@sic R5s230220 change 10 sic@ <<SKIPPED CODE>> // @siclog "Steps 19-19A" siclog@ fl_TC_7_1_1_8_1_ReceiveData(p_NR_CellId, p_DrbId, {v_RLC_SDU1, v_RLC_SDU2}, true, v_SN); //@sic R5-231578 R5s230220 sic@ v_SN := v_SN + 1; f_Delay (0.1); //@sic R5-216169 R5s230220 R5s230634 sic@ if (v_BWP_SameNumerology_upto4 ) { f_NR_SS_ConfigureBWPs(p_NR_CellId, -, tsc_NR_BWP_Id_2, tsc_NR_BWP_Id_2, tsc_NR_BWP_Id_2, tsc_NR_BWP_Id_2); } else { f_NR_SS_ConfigureBWPs(p_NR_CellId, -, tsc_NR_BWP_Id_Initial, tsc_NR_BWP_Id_Initial, tsc_NR_BWP_Id_Initial, tsc_NR_BWP_Id_Initial); } // @siclog "Step 20" siclog@ v_TimingInfoDL := f_SubFrameTiming_AddMilliSeconds(v_TimingInfoDL, 1300); // 1300ms after step 17 => 1000ms after step 18 v_RLC_SDU1 := fl_TC_7_1_1_8_1_SendData(p_NR_CellId, p_DrbId, cs_TimingInfo_NR(v_TimingInfoDL), v_SN); <<SKIPPED CODE>>

MCC160 proposed implementation

function f_TC_7_1_1_8_1_NR_CommonTestBody(NR_CellId_Type p_NR_CellId,

DRB_Identity p_DrbId,

template (value) NR_ResourceAllocation_Type p_ResourceAllocationUL) runs on NR_BASE_PTC

{ //@sic R5-213459 R5-227417 sic@

var NR_PhysicalParameters_Type v_NR_PhysicalParameters;

var template (value) NR_RachProcedureConfig_Type v_NR_RachProcedure;

var template (value) NR_PDCCH_Order_Type v_PDCCH_Order;

var BWP_Id v_BWP_Id_PDCCH_Order;

var SubFrameTiming_Type v_TimingInfoDL;

var SubFrameTiming_Type v_TimingInfoSS;     //@sic R5s240283 sic@

var SubFrameTiming_Type v_TimingInfoUL;

var SubFrameTiming_Type v_TimingInfoConfig;

var octetstring v_RLC_SDU1;

var octetstring v_RLC_SDU2;

var integer v_RA_PreambleIndex;

var integer v_SN := 0;

var boolean v_BWP_SameNumerology_upto4 := fl_NR_ReturnBWP_SameNumerologyUpto4_Capability(p_NR_CellId); //@sic R5-231578 sic@

var SubFrameTiming_Type v_TimingInfoUL_SS;

// @siclog "Step 1" siclog@

v_TimingInfoDL := f_TC_7_1_1_8_1_NR_GetNextSendOccasion_DL(p_NR_CellId);

v_RLC_SDU1 := fl_TC_7_1_1_8_1_SendData(p_NR_CellId, p_DrbId, cs_TimingInfo_NR(v_TimingInfoDL), v_SN);

// @siclog "Step 2" siclog@

v_TimingInfoUL := f_NR_GetNextSendOccasion_ULafterDL(p_NR_CellId, v_TimingInfoDL, 100);            // 100ms after step 1

f_NR_OneULGrantTransmission(p_NR_CellId, cs_TimingInfo_NR(v_TimingInfoUL), p_ResourceAllocationUL);

// @siclog "Steps 3-3A" siclog@

fl_TC_7_1_1_8_1_ReceiveData(p_NR_CellId, p_DrbId, {v_RLC_SDU1}, true, v_SN); //@sic R5-231578 R5s230220 sic@

f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 3");

v_SN := v_SN + 1;

//@sic R5s240283 sic@

v_TimingInfoSS := f_NR_GetNextSendOccasion_DL(p_NR_CellId);

if (v_BWP_SameNumerology_upto4) { //@sic  R5s230634 sic@

f_NR_SS_ConfigureBWPs(p_NR_CellId, cs_TimingInfo_NR(v_TimingInfoSS), tsc_NR_BWP_Id_FirstActive, tsc_NR_BWP_Id_2, tsc_NR_BWP_Id_2, tsc_NR_BWP_Id_2);

} else {

f_NR_SS_ConfigureBWPs(p_NR_CellId, cs_TimingInfo_NR(v_TimingInfoSS), tsc_NR_BWP_Id_FirstActive, tsc_NR_BWP_Id_Initial, tsc_NR_BWP_Id_Initial, tsc_NR_BWP_Id_Initial);

}

// @siclog "Step 5" siclog@

v_TimingInfoDL := f_TC_7_1_1_8_1_NR_GetNextSendOccasion_DL(p_NR_CellId);

v_RLC_SDU1 := fl_TC_7_1_1_8_1_SendData(p_NR_CellId, p_DrbId, cs_TimingInfo_NR(v_TimingInfoDL), v_SN);

// @siclog "Step 6" siclog@

v_TimingInfoUL := f_NR_GetNextSendOccasion_ULafterDL(p_NR_CellId, v_TimingInfoDL, 100);    // 100ms after step 5

f_NR_OneULGrantTransmission(p_NR_CellId, cs_TimingInfo_NR(v_TimingInfoUL), p_ResourceAllocationUL);

// @siclog "Steps 7-7A" siclog@

fl_TC_7_1_1_8_1_ReceiveData(p_NR_CellId, p_DrbId, {v_RLC_SDU1}, true, v_SN); //@sic R5-231578 R5s230220 sic@

f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 7");

v_SN := v_SN + 1;

//@sic R5s240283 sic@

v_TimingInfoSS := f_NR_GetNextSendOccasion_DL(p_NR_CellId);

//@sic R5s230220 change 9.2 sic@

if (v_BWP_SameNumerology_upto4) {  //@sic R5-2134599 sic@

f_NR_SS_ConfigureBWPs_DL(p_NR_CellId, cs_TimingInfo_NR(v_TimingInfoSS), tsc_NR_BWP_Id_2);

} else {

f_NR_SS_ConfigureBWPs_DL(p_NR_CellId, cs_TimingInfo_NR(v_TimingInfoSS), tsc_NR_BWP_Id_Initial);

}

// @siclog "Step 9" siclog@

v_TimingInfoDL := f_TC_7_1_1_8_1_NR_GetNextSendOccasion_DL(p_NR_CellId, 300);

v_RLC_SDU1 := fl_TC_7_1_1_8_1_SendData(p_NR_CellId, p_DrbId, cs_TimingInfo_NR(v_TimingInfoDL), v_SN);

//@sic R5s230220 change 9.2 sic@

v_TimingInfoUL_SS := f_NR_GetNextSendOccasion_ULafterDL(p_NR_CellId, v_TimingInfoDL, 90);              // 90 ms after step 9 - reconfigure UL/DL BWP in SS

v_TimingInfoUL := f_NR_GetNextSendOccasion_ULafterDL(p_NR_CellId, v_TimingInfoDL, 100);                // 100ms after step 9

if (v_BWP_SameNumerology_upto4) {  //@sic R5-2134599  R5s230634 sic@

f_NR_SS_ConfigureBWPs( p_NR_CellId, cs_TimingInfo_NR(v_TimingInfoUL_SS), tsc_NR_BWP_Id_3, tsc_NR_BWP_Id_3, tsc_NR_BWP_Id_2, tsc_NR_BWP_Id_3);

} else {

f_NR_SS_ConfigureBWPs(p_NR_CellId, cs_TimingInfo_NR(v_TimingInfoUL_SS), tsc_NR_BWP_Id_1, tsc_NR_BWP_Id_1, tsc_NR_BWP_Id_Initial, tsc_NR_BWP_Id_1);

}

// @siclog "Step 10" siclog@

f_NR_OneULGrantTransmission(p_NR_CellId, cs_TimingInfo_NR(v_TimingInfoUL), p_ResourceAllocationUL);

// @siclog "Steps 11-11AA" siclog@

fl_TC_7_1_1_8_1_ReceiveData(p_NR_CellId, p_DrbId, {v_RLC_SDU1}, true, v_SN); //@sic R5-231578 R5s230220 sic@

f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 11");

// configure CBRA CRNTI Based RACH procedure

//@sic R5s230220 change 10 sic@

v_NR_PhysicalParameters := f_NR_CellInfo_GetPhysicalParameters(p_NR_CellId);

v_NR_RachProcedure := cs_NR_RachProcedureConfig_CBRA_CrntiBased(v_NR_PhysicalParameters, p_ResourceAllocationUL, -, tsc_NR_ENDC_Msg3Grant_Def, cs_NR_AssignedBWPs_InitialBWP);

f_NR_SS_RachProcedureConfig(p_NR_CellId, v_NR_RachProcedure);

v_SN := v_SN + 1;

//@sic R5-2134599  R5s230634 sic@

if (v_BWP_SameNumerology_upto4) {

v_BWP_Id_PDCCH_Order := tsc_NR_BWP_Id_3;

} else {

v_BWP_Id_PDCCH_Order := tsc_NR_BWP_Id_1;

}

// @sic R5s230733 sic@

// @siclog "Step 11A" siclog@

v_TimingInfoDL := f_TC_7_1_1_8_1_NR_GetNextSendOccasion_DL(p_NR_CellId);

v_RLC_SDU1 := fl_TC_7_1_1_8_1_SendData(p_NR_CellId, p_DrbId, cs_TimingInfo_NR(v_TimingInfoDL), v_SN);

// Enable PRACH reception indication

f_NR_SS_SystemIndCtrlConfig(p_NR_CellId, cds_NR_SystemIndCtrl_RachPreamble(enable));

// @siclog "Step 12" siclog@

v_TimingInfoDL := f_SubFrameTiming_AddMilliSeconds(v_TimingInfoDL, 100);                            // 100ms after step 9

v_RA_PreambleIndex := 0;                                                                            // => contention-based random access

v_PDCCH_Order := cs_NR_PDCCH_Order(f_NR_CellInfo_GetSSB_Index(p_NR_CellId), v_RA_PreambleIndex);

f_NR_SS_CommonCellConfig(p_NR_CellId, cads_NR_DciTriggerPDCCHOrder_REQ(p_NR_CellId, cs_TimingInfo_NR(v_TimingInfoDL), v_PDCCH_Order, cs_NR_AssignedBWPs_DedicatedBWP(v_BWP_Id_PDCCH_Order)));

// @sic R5s230733 sic@

v_TimingInfoConfig := f_SubFrameTiming_AddMilliSeconds(v_TimingInfoDL, 10);

f_NR_SS_ConfigureBWPs(p_NR_CellId, cs_TimingInfo_NR(v_TimingInfoConfig), tsc_NR_BWP_Id_Initial, tsc_NR_BWP_Id_Initial, tsc_NR_BWP_Id_Initial, tsc_NR_BWP_Id_Initial);

// @siclog "Step 13" siclog@

SYSIND.receive(car_NR_PRACH_Preamble_IND(p_NR_CellId));

f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 13");

// @siclog "Step 13A - 13C" siclog@

// steps are done automatically according to configuration of the RACH procedure

// @siclog "Steps 13D-13E" siclog@

fl_TC_7_1_1_8_1_ReceiveData(p_NR_CellId, p_DrbId, {v_RLC_SDU1}, true, v_SN); //@sic R5-231578 R5s230220 sic@

f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 13D");

f_NR_SS_SystemIndCtrlConfig(p_NR_CellId, cds_NR_SystemIndCtrl_RachPreamble(disable)); //@sic R5s230634 sic@

v_SN := v_SN + 1;

//@sic R5s240283 sic@

v_TimingInfoSS := f_NR_GetNextSendOccasion_DL(p_NR_CellId);

//@sic  R5s230634 sic@

f_NR_SS_ConfigureBWPs(p_NR_CellId, cs_TimingInfo_NR(v_TimingInfoSS), tsc_NR_BWP_Id_Initial, tsc_NR_BWP_Id_1, tsc_NR_BWP_Id_1, tsc_NR_BWP_Id_1);

// @siclog "Step 16" siclog@

v_TimingInfoDL := f_TC_7_1_1_8_1_NR_GetNextSendOccasion_DL(p_NR_CellId);   //@sic R5s230220 change 10 sic@

v_TimingInfoConfig := f_SubFrameTiming_AddMilliSeconds(v_TimingInfoDL, 100);                        // 100ms after step 16

v_RLC_SDU1 := fl_TC_7_1_1_8_1_SendData(p_NR_CellId, p_DrbId, cs_TimingInfo_NR(v_TimingInfoDL), v_SN);

v_SN := v_SN + 1;  //@sic R5s230220 change 10 sic@

f_NR_SS_ConfigureBWPs_DL(p_NR_CellId, cs_TimingInfo_NR(v_TimingInfoConfig), tsc_NR_BWP_Id_1);

// @siclog "Step 17" siclog@

v_TimingInfoDL := f_SubFrameTiming_AddMilliSeconds(v_TimingInfoDL, 200);                            // 200ms after step 16 @sic R5s230220 R5-231578 sic@

v_RLC_SDU2 := fl_TC_7_1_1_8_1_SendData(p_NR_CellId, p_DrbId, cs_TimingInfo_NR(v_TimingInfoDL), v_SN);

// @siclog "Step 18" siclog@

v_TimingInfoUL := f_NR_GetNextSendOccasion_ULafterDL(p_NR_CellId, v_TimingInfoDL, 200);                // 200ms after step 17 @sic R5s230220 R5-231578 sic@

f_NR_OneULGrantTransmission(p_NR_CellId, cs_TimingInfo_NR(v_TimingInfoUL), p_ResourceAllocationUL);

// @siclog "Steps 19-19A" siclog@

fl_TC_7_1_1_8_1_ReceiveData(p_NR_CellId, p_DrbId, {v_RLC_SDU1, v_RLC_SDU2}, true, v_SN); //@sic R5-231578 R5s230220 sic@

v_SN := v_SN + 1;

//@sic R5-216169 R5s230220 R5s230634 R5s240283 sic@

v_TimingInfoSS := f_NR_GetNextSendOccasion_DL(p_NR_CellId);

if (v_BWP_SameNumerology_upto4 ) {

f_NR_SS_ConfigureBWPs(p_NR_CellId, cs_TimingInfo_NR(v_TimingInfoSS), tsc_NR_BWP_Id_2, tsc_NR_BWP_Id_2, tsc_NR_BWP_Id_2, tsc_NR_BWP_Id_2);

} else {

f_NR_SS_ConfigureBWPs(p_NR_CellId, cs_TimingInfo_NR(v_TimingInfoSS), tsc_NR_BWP_Id_Initial, tsc_NR_BWP_Id_Initial, tsc_NR_BWP_Id_Initial, tsc_NR_BWP_Id_Initial);

}

// @siclog "Step 20" siclog@

//The SS waits 1000 ms from step 18 to ensure that the bwp-InactivityTimer expired and then transmits a valid MAC PDU containing RLC PDU in the BWP with defaultDownlinkBWP-Id (= Downlink BWP#2 IF pc_bwp_sameNumerology_upto4_FR1_TDD and band under test is FR1 TDD ELSE BWP#0 IF pc_bwp_sameNumerology_upto2_FR1_TDD and band under test is FR1 TDD).

v_TimingInfoDL := f_SubFrameTiming_AddMilliSeconds(v_TimingInfoDL, 1200);                           // 1200ms after step 17 => 1000ms after step 18 @sic R5s240283 sic@

v_RLC_SDU1 := fl_TC_7_1_1_8_1_SendData(p_NR_CellId, p_DrbId, cs_TimingInfo_NR(v_TimingInfoDL), v_SN);

// @siclog "Step 21" siclog@

v_TimingInfoUL := f_NR_GetNextSendOccasion_ULafterDL(p_NR_CellId, v_TimingInfoDL, 100);                // 100ms after step 20

f_NR_OneULGrantTransmission(p_NR_CellId, cs_TimingInfo_NR(v_TimingInfoUL), p_ResourceAllocationUL);

// @siclog "Steps 22-22A" siclog@

fl_TC_7_1_1_8_1_ReceiveData(p_NR_CellId, p_DrbId, {v_RLC_SDU1}, true, v_SN); //@sic R5-231578 R5s230220 sic@

f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 22");

//DL and UL BWP already configured on tsc_NR_BWP_Id_2/tsc_NR_BWP_Id_Initial @sic R5s230733 sic@

}