3GPP TSG-RAN5 Meeting #2024-TTCN email	R5s240284

# Online, , 8th Dec 2023 - 31st Dec 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 3412 | rev | - | Current version: | 17.10.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to NR5GC Processing Delay test cases |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2024-04-02 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Several issues has been found related to Processing Delay test cases: - In the current TTCN implementation of the function f_TC_8_1_5_8_1_NR5GC_TestBody in 24wk12,Table 8.1.5.8.1.3.3-11: Physical layer parameters for DCI format 1_0 (Steps 14a5 and 25, Table 8.1.5.8.1.3.2-3) Is not implemented. In addition to this , it is also proposed to extend this table 8.1.5.8.1.3.3-11 to cover step 3 where RRCSetupRequest procedure is checked for procedure delay. An associated Prose CR will be raised for this requirement. - In the current TTCN implementation of the function f_TC_8_1_5_8_2_x_NR5GC_TestBody , Table 8.1.5.8.2.1.3.3-7: Physical layer parameters for DCI format 1_0 (Steps 6a5, Table 8.1.5.8.2.1.3.3-5). Changes are required for 8.1.5.8.2.1.NR5GC, 8.1.5.8.2.2.NR5GC and 8.1.5.8.2.3.NR5GC test cases. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Implemented and added Table 8.1.5.8.1.3.3-11 to Steps 3, 14a5 and 25 of function f_TC_8_1_5_8_1_NR5GC_TestBody. Added new parameter to function f_NR5GC_RRC_ProcedureDelay so it can be distinguished between DCI_1_1 procedures and DCI_1_0 Procedures - Implemented and added Table 8.1.5.8.2.1.3.3-7 to Steps 6a5 of function f_TC_8_1_5_8_2_x_NR5GC_TestBody. Changes will apply to 8.1.5.8.2.1.NR5GC, 8.1.5.8.2.2.NR5GC and 8.1.5.8.2.3.NR5GC test cases. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may fail the test cases. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.1.5.8.1.NR5GC, 8.1.5.8.2.1.NR5GC, 8.1.5.8.2.2.NR5GC and 8.1.5.8.2.3.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | x |  | Test specifications |  |  |  | TS/TR 38.523-1 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | An associated Prose is raised to cover the requirement of “extending table 8.1.5.8.1.3.3-11 to cover step 3 RRCSetupRequest procedure”. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

1	Overview	3

2	Corrections required:	3

2.1	Change to function f_TC_8_1_5_8_1_NR5GC_TestBody	3

2.2	Change to function f_TC_8_1_5_8_2_x_NR5GC_TestBody	14

2.3	Change to function f_NR5GC_RRC_ProcedureDelay	17

2.4	Change to function f_NR5GC_RRC_ProcedureDelayWithRACH	19

- Overview

This document lists all the essential changes needed to correct problems in the iwd-TTCN3-B2022-09_D24wk12 related to the title of this CR.

Contact: 	Juan Garcia Martin

Juan.GarciaMartin@rohde-schwarz.com

# Corrections required:

## Change to function f_TC_8_1_5_8_1_NR5GC_TestBody

| Function name | f_TC_8_1_5_8_1_NR5GC_TestBody |
| --- | --- |
| Reason for change | - In the current TTCN implementation of the function f_TC_8_1_5_8_1_NR5GC_TestBody in 24wk12,Table 8.1.5.8.1.3.3-11: Physical layer parameters for DCI format 1_0 (Steps 14a5 and 25, Table 8.1.5.8.1.3.2-3) Is not implemented. - In addition to this , it is also proposed to extend this table 8.1.5.8.1.3.3-11 to cover step 3 where RRCSetupRequest procedure is checked for procedure delay. An associated Prose CR will be raised for this requirement. |
| Summary of change | - Implemented and added Table 8.1.5.8.1.3.3-11 to Steps 3, 14a5 and 25 of function f_TC_8_1_5_8_1_NR5GC_TestBody. - Added new parameter to function f_NR5GC_RRC_ProcedureDelay so it can be distinguished between DCI_1_1 procedures and DCI_1_0 Procedures |
| TTCN module | RRC_Others_NR5GC |
| MCC160 Comment | - Accepted, see MCC160 proposed implementation below - Postponed, conditional to prose CR agreement |

Before Change:

function f_TC_8_1_5_8_1_NR5GC_TestBody() runs on NR5GC_PTC { var RNTI_Value v_RNTI_Value; var PhysCellId v_PhysCellId; var ShortMAC_I v_ShortMAC_I; var NR_PhysicalParameters_Type v_NR_PhysicalParameters; var NR_SecurityParams_Type v_SecurityParams; var octetstring v_EncodeCapRequestNR; var template (value) DRB_CountMSB_InfoList v_CountList; var template (value) RadioBearerConfig v_RadioBearerConfig; var template (value) RadioBearerConfig v_RadioBearerConfigResume;//@sic R5s220683 sic@ var template (value) CellGroupConfig v_CellGroupConfig; var NR_RadioBearerList_Type v_SS_Config; var template (value) RRCReconfiguration_v1530_IEs v_V1530Ext; var template (value) NR_RRC_MSG_Request_Type v_RrcMsg; var NR_SRB_COMMON_IND v_ReceivedNR_SRB_COMMON_IND; var template (omit) octetstring v_EncodedSCG_Config := omit; var NR_DRBInfoList_Type v_ExistingDRBs; var template (value) NR_RachProcedureConfig_Type v_NR_RachProcedureConfig; var integer v_Max_nPRB := f_NR_CellInfo_GetCarrierBandwidth_DL(nr_Cell1); var NR_ResourceAllocation_Type v_NR_ResourceAllocation := {v_Max_nPRB, 28}; var template DRB_CountInfoList v_CountList_UE; var template (value) NG_NAS_MSG_RequestList_Type v_ServerAcceptMsg; var template (value) NR_CellPowerList_Type v_CellPowerList_AtT0; var template (value) NR_CellPowerList_Type v_CellPowerList_AtT1; //@sic R5s220683 sic@ var template (omit) DRB_ToAddModList v_DRB_ToAddModList := omit; var integer i; timer t_Wait; v_CellPowerList_AtT0 :={ cs_NR_CellPower(nr_Cell2, -96, tsc_NR_SuitableIntraFreqCellSSS_EPRE_FR2)//@sic R5-237355 sic@ }; v_CellPowerList_AtT1:={ cs_NR_CellPower(nr_Cell1, tsc_NR_NonSuitableOffCellSSS_EPRE, tsc_NR_NonSuitableOffCellSSS_EPRE),//@sic R5-237355 sic@ cs_NR_CellPower(nr_Cell2, tsc_NR_ServingCellSSS_EPRE_FR1, tsc_NR_ServingCellSSS_EPRE_FR2)//@sic R5-237355 sic@ }; f_NR_SetCellPowerList(v_CellPowerList_AtT0); //@siclog "Step 1" siclog@ //The SS transmits a Paging message including a matched ng-5G-S-TMSI. f_NR_Paging_Def(nr_Cell1, RRC_IDLE); //@siclog "Step 2" siclog@ //The UE transmits an RRCSetupRequest message. f_NR_RRC_SetupRequest_Def(nr_Cell1); //@siclog "Steps 3 - 4" siclog@ //The SS transmits an RRCSetup message. //The SS starts transmitting a periodic DCI from (10*2^numerology+1+TDL) slots after step 3 to schedule PUSCH. //Check: Does the UE transmit an RRCSetupComplete message within (10*2^numerology+1+TDL+K2) slots15 (FDD)/18 (TDD) subframes after successful completion of step 3? v_ReceivedNR_SRB_COMMON_IND := f_NR5GC_RRC_ProcedureDelay(nr_Cell1, cas_NR_SRB0_RrcPdu_REQ(nr_Cell1,cs_TimingInfo_Now,f_NR_38508_RRCSetup(nr_Cell1, tsc_NR_RRC_TI_Def)), car_NR_SRB1_RrcNasPdu_IND(nr_Cell1, cr_38508_RRCSetupComplete(tsc_NR_RRC_TI_Def, ?,- , *), cr_NG_NAS_IndWithPiggybacking(tsc_SHT_IntegrityProtected)), 10, "Step 4" ); //@siclog "Step 5 - 6" siclog@ //The SS transmits a SecurityModeCommand message. //The SS starts transmitting a periodic DCI from (5*2^numerology+1+TDL) slots after step 5 to schedule PUSCH. //Check: Does the UE transmit a SecurityModeComplete message within (5*2^numerology+1+TDL+K2) 10 (FDD)/13 (TDD) subframes slots after successful completion of step 5? v_SecurityParams := f_NR_Security_Get(); v_SecurityParams := f_NR_Authentication_InitAS (v_SecurityParams, v_ReceivedNR_SRB_COMMON_IND.Signalling.Nas[0].SecurityProtection.NasCount); v_SecurityParams.AS_Ciphering.ActTimeList := f_NR_RRC_CipherActTime_GetInitial(nr_Cell1); f_NR_SS_RRC_EnableIntProt_CiphULandDL(nr_Cell1, v_SecurityParams.AS_Integrity, v_SecurityParams.AS_Ciphering); f_NR5GC_RRC_ProcedureDelay(nr_Cell1, cas_NR_SRB1_RrcPdu_REQ(nr_Cell1, -, cs_38508_RRCSecurityModeCommand(tsc_NR_RRC_TI_Def, cs_38508_SecurityAlgorithmConfig(v_SecurityParams.AS_Ciphering.Algorithm, v_SecurityParams.AS_Integrity.Algorithm))), car_NR_SRB1_RrcPdu_IND(nr_Cell1, cr_38508_RRCSecurityModeComplete(tsc_NR_RRC_TI_Def)), 5, "Step 6" ); f_NR_Security_Set(v_SecurityParams); //@siclog "Step 7 - 8" siclog@ //The SS transmits a UECapabilityEnquiry message. //The SS starts transmitting a periodic DCI from (80*2^numerology+1+TDL) slots after step 7 to schedule PUSCH. //Check: Does the UE transmit a UECapabilityInformation message within (80*2^numerology+1+TDL+K2) slots after successful completion of step 7? v_EncodeCapRequestNR := bit2oct(encvalue( cs_38508_UECapabilityRequestFilterNR({ cs_NR_BandInformationNR(f_NR_CellInfo_GetBandIndicator(nr_Cell1)) }) )); f_NR5GC_RRC_ProcedureDelay(nr_Cell1, cas_NR_SRB1_RrcPdu_REQ(nr_Cell1, -, cs_38508_UECapabilityEnquiry(tsc_NR_RRC_TI_Def, nr, v_EncodeCapRequestNR)), car_NR_SRB1_RrcPdu_IND(nr_Cell1, cr_38508_UECapabilityInformationAny(tsc_NR_RRC_TI_Def)), 80, "Step 8", cs_NR_FreqDomainSchedulExplicit(-, v_NR_ResourceAllocation.Lrbs) //DCI with the maximum grant allowed by the carrier bandwidth @sic R5-217815 sic@ ); //@siclog "Step 9 - 10" siclog@ //The SS transmits an RRCReconfiguration message to establish SRB2 and DRB #n for the UE f_NR_BuildRRCReconfigExistingDRBs (v_RadioBearerConfig, v_CellGroupConfig, v_SS_Config, nr_Cell1); v_SecurityParams := f_NR_Security_Get(); v_V1530Ext := cs_NR_RRCReconfiguration_v1530_IEs(bit2oct(encvalue(v_CellGroupConfig))); //Reconfigure DRB in SS - existing DRBs are not reconfigured during release f_NR_SS_CommonRadioBearerConfig(nr_Cell1, v_SS_Config, cs_TimingInfo_Now); // Configure PDCP security v_SecurityParams.AS_Ciphering.ActTimeList := f_NR_RRC_CipherActTime_GetCurrent(nr_Cell1, false); f_NR_SS_RRC_EnableIntProt_CiphULandDL(nr_Cell1, v_SecurityParams.AS_Integrity, v_SecurityParams.AS_Ciphering); v_ServerAcceptMsg := {cs_NG_NAS_Request(tsc_SHT_IntegrityProtected_Ciphered, f_Get_NG_ServiceAcceptMsg(v_ReceivedNR_SRB_COMMON_IND.Signalling.Nas[0].Pdu.Msg.service_Request.pduSessionStatus))}; v_RrcMsg := cs_NR_RRC_MSG_Request_DCCH(cs_38508_RRCReconfiguration(tsc_NR_RRC_TI_Def, v_RadioBearerConfig, -, -, v_V1530Ext)); f_NR5GC_RRC_ProcedureDelay(nr_Cell1, cas_NR_RrcNasPduList_REQ(nr_Cell1, tsc_NR_RbId_SRB1, cs_TimingInfo_Now, v_RrcMsg, v_ServerAcceptMsg, -, -, cs_RlcBearerRouting_NR(nr_Cell1)), car_NR_SRB_RrcPdu_IND(nr_Cell1, tsc_NR_RbId_SRB1, cr_38508_RRCReconfigurationComplete), 10, "Step 10" ); //@siclog "Step 11 - 12" siclog@ //The SS transmits a CounterCheck message. //The SS starts transmitting a periodic DCI from (5*2^numerology+1+TDL) slots after step 11 to schedule PUSCH. //Check: Does the UE transmit a CounterCheckResponse message within (5*2^numerology+1+TDL+K2)10 (FDD)/13 (TDD) subframes slots after successful completion of step 11? v_ExistingDRBs := f_NR5GC_MobileInfo_GetDRBInfoList(); v_CountList := {cs_38508_DRB_CountMSB_Info(v_ExistingDRBs[0].DRB_Id, 1, 1)}; v_CountList_UE := {cr_DRB_CountInfo(v_ExistingDRBs[0].DRB_Id)}; f_NR5GC_RRC_ProcedureDelay(nr_Cell1, cas_NR_SRB1_RrcPdu_REQ(nr_Cell1, -, cs_38508_CounterCheck(tsc_NR_RRC_TI_Def, v_CountList)), car_NR_SRB1_RrcPdu_IND(nr_Cell1, cr_38508_CounterCheckResponse(tsc_NR_RRC_TI_Def, v_CountList_UE)), 5, "Step 12" ); //@siclog "Step 13" siclog@ //Void //@siclog "Step 14a1-14a7" siclog@ if ( pc_inactiveState ){ //@siclog "Step 14a1" siclog@ //The SS transmits an RRCRelease message with suspendConfig. f_NR_RRC_InactiveDef(nr_Cell1); //@siclog "Step 14a2" siclog@ //Wait 1s to let UE enter RRC_INACTIVE state. f_Delay(1.0); //@siclog "Step 14a3" siclog@ //The SS transmits a Paging message including a matched ng-5G-S-TMSI. f_NR_Paging_Def(nr_Cell1, RRC_INACTIVE); v_CellGroupConfig := f_NR_GetSCGConfigResume(nr_Cell1); v_EncodedSCG_Config := bit2oct(encvalue(v_CellGroupConfig)); //@sic R5s220683 sic@ for (i:= 0; i < lengthof(v_ExistingDRBs); i :=i+1) { v_DRB_ToAddModList[i] := cs_NR_DRB_ToAddMod(omit, v_ExistingDRBs[i].DRB_Id, true_, -, omit); } v_RadioBearerConfigResume := cs_38508_RadioBearerConfigDef( {cs_NR_SRB_ToAddMod_ReestablishPDCP(tsc_SRB2)}, v_DRB_ToAddModList); //@siclog "Step 14a4" siclog@ //The UE transmits an RRCResumeRequest message SRB.receive(car_NR_SRB0_RrcPdu_IND(nr_Cell1, cr_38508_RRCResumeRequest(tsc_NR_ShortI_RNTI_Value1, mt_Access))); f_NR_BuildRRCReconfigExistingDRBs (v_RadioBearerConfig, v_CellGroupConfig, v_SS_Config, nr_Cell1); f_NR_SS_CommonRadioBearerConfig(nr_Cell1, v_SS_Config); //Activate security on newly configured DRB f_NR_SS_AS_ActivateSecurity_NoRefresh(nr_Cell1, cs_TimingInfo_Now); //@siclog "Step 14a5" siclog@ //The SS transmits an RRCResume message for the UE //@sic R5-237357 sic@ if(pc_reducedCP_Latency){ //@siclog "Step 14a5Aa1 - 14a5Aa2" siclog@ //The SS starts transmitting a periodic DCI from (10* 2 2^numerology +1+TDL) slots after step 14a5 to schedule PUSCH. //Check: Does the UE transmit an RRCResumeComplete message within (6*2^numerology+1+TDL+K2) slots after successful completion of step 14a5? f_NR5GC_RRC_ProcedureDelayWithRACH( nr_Cell1, cas_NR_SRB1_RrcPdu_REQ(nr_Cell1, cs_TimingInfo_Now, cs_38508_RRCResume(tsc_NR_RRC_TI_Def, v_EncodedSCG_Config, v_RadioBearerConfigResume)),//@sic R5s220683 sic@ car_NR_SRB1_RrcPdu_IND(nr_Cell1, cr_38508_RRCResumeComplete), 6, "Step 14a5Aa2" ); } else { //@siclog "Step 14a5Ab1 - 14a5Ab2" siclog@ //The SS starts transmitting a periodic DCI from n slots after step 14a5 to schedule PUSCH. //Check: Does the UE transmit an RRCResumeComplete message within (10* 2 2^numerology +1+TDL+K2)slots after successful completion of step 14a5? f_NR5GC_RRC_ProcedureDelayWithRACH( nr_Cell1, cas_NR_SRB1_RrcPdu_REQ(nr_Cell1, cs_TimingInfo_Now, cs_38508_RRCResume(tsc_NR_RRC_TI_Def, v_EncodedSCG_Config, v_RadioBearerConfigResume)),//@sic R5s220683 sic@ car_NR_SRB1_RrcPdu_IND(nr_Cell1, cr_38508_RRCResumeComplete), 10, "Step 14a5Ab2" ); } v_NR_RachProcedureConfig := f_NR_InitialiseRachProcedure(nr_Cell1); f_NR_SS_RachProcedureConfig(nr_Cell1, v_NR_RachProcedureConfig); //@siclog "Step 14a6-14a7" siclog@ //Void } //@siclog "Step 14-21" siclog@ //Void //@siclog "Step 22" siclog@ //The SS re-adjusts the SS levels according to row "T1" in table 8.1.5.8.1.3.2-1/2 in order that the radio link quality of NR Cell 1 is degraded, and NR Cell 2 is suitable for camping f_NR_SetCellPowerList(v_CellPowerList_AtT1); //@siclog "Step 23" siclog@ //Wait 1s to let T310 expire t_Wait.start (1.0);//@sic R5s220683 sic@ //@siclog "Step 24" siclog@ //The UE transmits an RRCReestablishmentRequest message on Cell 2 v_RNTI_Value := f_NR_CellInfo_GetRNTI(nr_Cell1); v_PhysCellId := f_NR_CellInfo_GetPhysicalCellId(nr_Cell1); v_NR_PhysicalParameters := f_NR_CellInfo_GetPhysicalParameters(nr_Cell2); f_NR_CellInfo_SetRNTI(nr_Cell2, tsc_C_RNTI_Value4); f_NR_SS_C_RNTI_Config(nr_Cell2, tsc_C_RNTI_Value4, -, tsc_NoCnfReq, tsc_FollowOnFlag); f_NR_SS_CommonCellConfig(nr_Cell2, cads_NR_RachProcedure_DcchDtchToCSS_Config_REQ(nr_Cell2, cs_TimingInfo_Now, tsc_CnfReq, cs_NR_RachProcedureConfig_RRCReestablishment(v_NR_PhysicalParameters, f_NR_CellInfo_GetRNTI(nr_Cell2), 256), //@sic R5-225418, R5s230476 R5s230884 sic@ cs_NR_SearchSpaceDlDciAssignment_CSSType1(v_NR_PhysicalParameters), cs_NR_SearchSpaceUlDciAssignmentCSS(v_NR_PhysicalParameters, cs_NR_DciFormat_0_0_Params))); v_ShortMAC_I := f_NR_Calculate_ShortMAC(nr_Cell1, nr_Cell2); //@sic R5s220683 sic@ alt{ []SRB.receive(car_NR_SRB0_RrcPdu_IND(nr_Cell2, cr_38508_RRCReestablishmentRequest(cr_NR_ReestabUE_Identity (v_RNTI_Value, v_PhysCellId, v_ShortMAC_I), otherFailure))) { t_Wait.stop; } []t_Wait.timeout { SRB.receive(car_NR_SRB0_RrcPdu_IND(nr_Cell2, cr_38508_RRCReestablishmentRequest(cr_NR_ReestabUE_Identity (v_RNTI_Value, v_PhysCellId, v_ShortMAC_I), otherFailure))); } } << SKIPPED CODE >>

After Change:

function f_TC_8_1_5_8_1_NR5GC_TestBody() runs on NR5GC_PTC { var RNTI_Value v_RNTI_Value; var PhysCellId v_PhysCellId; var ShortMAC_I v_ShortMAC_I; var NR_PhysicalParameters_Type v_NR_PhysicalParameters; var NR_SecurityParams_Type v_SecurityParams; var octetstring v_EncodeCapRequestNR; var template (value) DRB_CountMSB_InfoList v_CountList; var template (value) RadioBearerConfig v_RadioBearerConfig; var template (value) RadioBearerConfig v_RadioBearerConfigResume;//@sic R5s220683 sic@ var template (value) CellGroupConfig v_CellGroupConfig; var NR_RadioBearerList_Type v_SS_Config; var template (value) RRCReconfiguration_v1530_IEs v_V1530Ext; var template (value) NR_RRC_MSG_Request_Type v_RrcMsg; var NR_SRB_COMMON_IND v_ReceivedNR_SRB_COMMON_IND; var template (omit) octetstring v_EncodedSCG_Config := omit; var NR_DRBInfoList_Type v_ExistingDRBs; var template (value) NR_RachProcedureConfig_Type v_NR_RachProcedureConfig; var integer v_Max_nPRB := f_NR_CellInfo_GetCarrierBandwidth_DL(nr_Cell1); var NR_ResourceAllocation_Type v_NR_ResourceAllocation := {v_Max_nPRB, 28}; var template DRB_CountInfoList v_CountList_UE; var template (value) NG_NAS_MSG_RequestList_Type v_ServerAcceptMsg; var template (value) NR_CellPowerList_Type v_CellPowerList_AtT0; var template (value) NR_CellPowerList_Type v_CellPowerList_AtT1; //@sic R5s220683 sic@ var template (omit) DRB_ToAddModList v_DRB_ToAddModList := omit; var integer i; timer t_Wait; v_NR_PhysicalParameters := f_NR_CellInfo_GetPhysicalParameters(nr_Cell2); var template (value) NR_RachProcedureConfig_Type p_RachProcedureConfig := cs_NR_RachProcedureConfig_RRCReestablishment(v_NR_PhysicalParameters, f_NR_CellInfo_GetRNTI(nr_Cell2), 256); v_CellPowerList_AtT0 :={ cs_NR_CellPower(nr_Cell2, -96, tsc_NR_SuitableIntraFreqCellSSS_EPRE_FR2)//@sic R5-237355 sic@ }; v_CellPowerList_AtT1:={ cs_NR_CellPower(nr_Cell1, tsc_NR_NonSuitableOffCellSSS_EPRE, tsc_NR_NonSuitableOffCellSSS_EPRE),//@sic R5-237355 sic@ cs_NR_CellPower(nr_Cell2, tsc_NR_ServingCellSSS_EPRE_FR1, tsc_NR_ServingCellSSS_EPRE_FR2)//@sic R5-237355 sic@ }; f_NR_SetCellPowerList(v_CellPowerList_AtT0); v_NR_RachProcedureConfig := f_NR_InitialiseRachProcedure(nr_Cell1); if (f_NR_CellInfo_GetIsFDD(nr_Cell1)){ v_NR_RachProcedureConfig.RachProcedureList[0].ContentionResolution.Msg4_Based.SearchSpaceAndDci.DciInfo.Format.Format_1_0.PdschHarqTimingIndicator.Value := '010'B; } f_NR_SS_RachProcedureConfig(nr_Cell1, v_NR_RachProcedureConfig); //@siclog "Step 1" siclog@ //The SS transmits a Paging message including a matched ng-5G-S-TMSI. f_NR_Paging_Def(nr_Cell1, RRC_IDLE); //@siclog "Step 2" siclog@ //The UE transmits an RRCSetupRequest message. f_NR_RRC_SetupRequest_Def(nr_Cell1); //@siclog "Steps 3 - 4" siclog@ //The SS transmits an RRCSetup message. //The SS starts transmitting a periodic DCI from (10*2^numerology+1+TDL) slots after step 3 to schedule PUSCH. //Check: Does the UE transmit an RRCSetupComplete message within (10*2^numerology+1+TDL+K2) slots15 (FDD)/18 (TDD) subframes after successful completion of step 3? v_ReceivedNR_SRB_COMMON_IND := f_NR5GC_RRC_ProcedureDelay(nr_Cell1, cas_NR_SRB0_RrcPdu_REQ(nr_Cell1,cs_TimingInfo_Now,f_NR_38508_RRCSetup(nr_Cell1, tsc_NR_RRC_TI_Def)), car_NR_SRB1_RrcNasPdu_IND(nr_Cell1, cr_38508_RRCSetupComplete(tsc_NR_RRC_TI_Def, ?,- , *), cr_NG_NAS_IndWithPiggybacking(tsc_SHT_IntegrityProtected)), 10, "Step 4",-,-,true); //@siclog "Step 5 - 6" siclog@ //The SS transmits a SecurityModeCommand message. //The SS starts transmitting a periodic DCI from (5*2^numerology+1+TDL) slots after step 5 to schedule PUSCH. //Check: Does the UE transmit a SecurityModeComplete message within (5*2^numerology+1+TDL+K2) 10 (FDD)/13 (TDD) subframes slots after successful completion of step 5? v_SecurityParams := f_NR_Security_Get(); v_SecurityParams := f_NR_Authentication_InitAS (v_SecurityParams, v_ReceivedNR_SRB_COMMON_IND.Signalling.Nas[0].SecurityProtection.NasCount); v_SecurityParams.AS_Ciphering.ActTimeList := f_NR_RRC_CipherActTime_GetInitial(nr_Cell1); f_NR_SS_RRC_EnableIntProt_CiphULandDL(nr_Cell1, v_SecurityParams.AS_Integrity, v_SecurityParams.AS_Ciphering); f_NR5GC_RRC_ProcedureDelay(nr_Cell1, cas_NR_SRB1_RrcPdu_REQ(nr_Cell1, -, cs_38508_RRCSecurityModeCommand(tsc_NR_RRC_TI_Def, cs_38508_SecurityAlgorithmConfig(v_SecurityParams.AS_Ciphering.Algorithm, v_SecurityParams.AS_Integrity.Algorithm))), car_NR_SRB1_RrcPdu_IND(nr_Cell1, cr_38508_RRCSecurityModeComplete(tsc_NR_RRC_TI_Def)), 5, "Step 6" ); f_NR_Security_Set(v_SecurityParams); //@siclog "Step 7 - 8" siclog@ //The SS transmits a UECapabilityEnquiry message. //The SS starts transmitting a periodic DCI from (80*2^numerology+1+TDL) slots after step 7 to schedule PUSCH. //Check: Does the UE transmit a UECapabilityInformation message within (80*2^numerology+1+TDL+K2) slots after successful completion of step 7? v_EncodeCapRequestNR := bit2oct(encvalue( cs_38508_UECapabilityRequestFilterNR({ cs_NR_BandInformationNR(f_NR_CellInfo_GetBandIndicator(nr_Cell1)) }) )); f_NR5GC_RRC_ProcedureDelay(nr_Cell1, cas_NR_SRB1_RrcPdu_REQ(nr_Cell1, -, cs_38508_UECapabilityEnquiry(tsc_NR_RRC_TI_Def, nr, v_EncodeCapRequestNR)), car_NR_SRB1_RrcPdu_IND(nr_Cell1, cr_38508_UECapabilityInformationAny(tsc_NR_RRC_TI_Def)), 80, "Step 8", cs_NR_FreqDomainSchedulExplicit(-, v_NR_ResourceAllocation.Lrbs) //DCI with the maximum grant allowed by the carrier bandwidth @sic R5-217815 sic@ ); //@siclog "Step 9 - 10" siclog@ //The SS transmits an RRCReconfiguration message to establish SRB2 and DRB #n for the UE f_NR_BuildRRCReconfigExistingDRBs (v_RadioBearerConfig, v_CellGroupConfig, v_SS_Config, nr_Cell1); v_SecurityParams := f_NR_Security_Get(); v_V1530Ext := cs_NR_RRCReconfiguration_v1530_IEs(bit2oct(encvalue(v_CellGroupConfig))); //Reconfigure DRB in SS - existing DRBs are not reconfigured during release f_NR_SS_CommonRadioBearerConfig(nr_Cell1, v_SS_Config, cs_TimingInfo_Now); // Configure PDCP security v_SecurityParams.AS_Ciphering.ActTimeList := f_NR_RRC_CipherActTime_GetCurrent(nr_Cell1, false); f_NR_SS_RRC_EnableIntProt_CiphULandDL(nr_Cell1, v_SecurityParams.AS_Integrity, v_SecurityParams.AS_Ciphering); v_ServerAcceptMsg := {cs_NG_NAS_Request(tsc_SHT_IntegrityProtected_Ciphered, f_Get_NG_ServiceAcceptMsg(v_ReceivedNR_SRB_COMMON_IND.Signalling.Nas[0].Pdu.Msg.service_Request.pduSessionStatus))}; v_RrcMsg := cs_NR_RRC_MSG_Request_DCCH(cs_38508_RRCReconfiguration(tsc_NR_RRC_TI_Def, v_RadioBearerConfig, -, -, v_V1530Ext)); f_NR5GC_RRC_ProcedureDelay(nr_Cell1, cas_NR_RrcNasPduList_REQ(nr_Cell1, tsc_NR_RbId_SRB1, cs_TimingInfo_Now, v_RrcMsg, v_ServerAcceptMsg, -, -, cs_RlcBearerRouting_NR(nr_Cell1)), car_NR_SRB_RrcPdu_IND(nr_Cell1, tsc_NR_RbId_SRB1, cr_38508_RRCReconfigurationComplete), 10, "Step 10" ); //@siclog "Step 11 - 12" siclog@ //The SS transmits a CounterCheck message. //The SS starts transmitting a periodic DCI from (5*2^numerology+1+TDL) slots after step 11 to schedule PUSCH. //Check: Does the UE transmit a CounterCheckResponse message within (5*2^numerology+1+TDL+K2)10 (FDD)/13 (TDD) subframes slots after successful completion of step 11? v_ExistingDRBs := f_NR5GC_MobileInfo_GetDRBInfoList(); v_CountList := {cs_38508_DRB_CountMSB_Info(v_ExistingDRBs[0].DRB_Id, 1, 1)}; v_CountList_UE := {cr_DRB_CountInfo(v_ExistingDRBs[0].DRB_Id)}; f_NR5GC_RRC_ProcedureDelay(nr_Cell1, cas_NR_SRB1_RrcPdu_REQ(nr_Cell1, -, cs_38508_CounterCheck(tsc_NR_RRC_TI_Def, v_CountList)), car_NR_SRB1_RrcPdu_IND(nr_Cell1, cr_38508_CounterCheckResponse(tsc_NR_RRC_TI_Def, v_CountList_UE)), 5, "Step 12" ); //@siclog "Step 13" siclog@ //Void //@siclog "Step 14a1-14a7" siclog@ if ( pc_inactiveState ){ //@siclog "Step 14a1" siclog@ //The SS transmits an RRCRelease message with suspendConfig. f_NR_RRC_InactiveDef(nr_Cell1); //@siclog "Step 14a2" siclog@ //Wait 1s to let UE enter RRC_INACTIVE state. f_Delay(1.0); //@siclog "Step 14a3" siclog@ //The SS transmits a Paging message including a matched ng-5G-S-TMSI. f_NR_Paging_Def(nr_Cell1, RRC_INACTIVE); v_CellGroupConfig := f_NR_GetSCGConfigResume(nr_Cell1); v_EncodedSCG_Config := bit2oct(encvalue(v_CellGroupConfig)); //@sic R5s220683 sic@ for (i:= 0; i < lengthof(v_ExistingDRBs); i :=i+1) { v_DRB_ToAddModList[i] := cs_NR_DRB_ToAddMod(omit, v_ExistingDRBs[i].DRB_Id, true_, -, omit); } v_RadioBearerConfigResume := cs_38508_RadioBearerConfigDef( {cs_NR_SRB_ToAddMod_ReestablishPDCP(tsc_SRB2)}, v_DRB_ToAddModList); //@siclog "Step 14a4" siclog@ //The UE transmits an RRCResumeRequest message SRB.receive(car_NR_SRB0_RrcPdu_IND(nr_Cell1, cr_38508_RRCResumeRequest(tsc_NR_ShortI_RNTI_Value1, mt_Access))); f_NR_BuildRRCReconfigExistingDRBs (v_RadioBearerConfig, v_CellGroupConfig, v_SS_Config, nr_Cell1); f_NR_SS_CommonRadioBearerConfig(nr_Cell1, v_SS_Config); //Activate security on newly configured DRB f_NR_SS_AS_ActivateSecurity_NoRefresh(nr_Cell1, cs_TimingInfo_Now); //@siclog "Step 14a5" siclog@ //The SS transmits an RRCResume message for the UE //@sic R5-237357 sic@ if(pc_reducedCP_Latency){ //@siclog "Step 14a5Aa1 - 14a5Aa2" siclog@ //The SS starts transmitting a periodic DCI from (10* 2 2^numerology +1+TDL) slots after step 14a5 to schedule PUSCH. //Check: Does the UE transmit an RRCResumeComplete message within (6*2^numerology+1+TDL+K2) slots after successful completion of step 14a5? f_NR5GC_RRC_ProcedureDelayWithRACH( nr_Cell1, cas_NR_SRB1_RrcPdu_REQ(nr_Cell1, cs_TimingInfo_Now, cs_38508_RRCResume(tsc_NR_RRC_TI_Def, v_EncodedSCG_Config, v_RadioBearerConfigResume)),//@sic R5s220683 sic@ car_NR_SRB1_RrcPdu_IND(nr_Cell1, cr_38508_RRCResumeComplete), 6, "Step 14a5Aa2" ); } else { //@siclog "Step 14a5Ab1 - 14a5Ab2" siclog@ //The SS starts transmitting a periodic DCI from n slots after step 14a5 to schedule PUSCH. //Check: Does the UE transmit an RRCResumeComplete message within (10* 2 2^numerology +1+TDL+K2)slots after successful completion of step 14a5? f_NR5GC_RRC_ProcedureDelayWithRACH( nr_Cell1, cas_NR_SRB1_RrcPdu_REQ(nr_Cell1, cs_TimingInfo_Now, cs_38508_RRCResume(tsc_NR_RRC_TI_Def, v_EncodedSCG_Config, v_RadioBearerConfigResume)),//@sic R5s220683 sic@ car_NR_SRB1_RrcPdu_IND(nr_Cell1, cr_38508_RRCResumeComplete), 10, "Step 14a5Ab2" ); } v_NR_RachProcedureConfig := f_NR_InitialiseRachProcedure(nr_Cell1); f_NR_SS_RachProcedureConfig(nr_Cell1, v_NR_RachProcedureConfig); //@siclog "Step 14a6-14a7" siclog@ //Void } //@siclog "Step 14-21" siclog@ //Void //@siclog "Step 22" siclog@ //The SS re-adjusts the SS levels according to row "T1" in table 8.1.5.8.1.3.2-1/2 in order that the radio link quality of NR Cell 1 is degraded, and NR Cell 2 is suitable for camping f_NR_SetCellPowerList(v_CellPowerList_AtT1); //@siclog "Step 23" siclog@ //Wait 1s to let T310 expire t_Wait.start (1.0);//@sic R5s220683 sic@ //@siclog "Step 24" siclog@ //The UE transmits an RRCReestablishmentRequest message on Cell 2 v_RNTI_Value := f_NR_CellInfo_GetRNTI(nr_Cell1); v_PhysCellId := f_NR_CellInfo_GetPhysicalCellId(nr_Cell1); v_NR_PhysicalParameters := f_NR_CellInfo_GetPhysicalParameters(nr_Cell2); f_NR_CellInfo_SetRNTI(nr_Cell2, tsc_C_RNTI_Value4); f_NR_SS_C_RNTI_Config(nr_Cell2, tsc_C_RNTI_Value4, -, tsc_NoCnfReq, tsc_FollowOnFlag); if (f_NR_CellInfo_GetIsFDD(nr_Cell2)) { p_RachProcedureConfig.RachProcedureList[0].ContentionResolution.Msg4_Based.SearchSpaceAndDci.DciInfo.Format.Format_1_0.PdschHarqTimingIndicator.Value := '010'B; } f_NR_SS_CommonCellConfig(nr_Cell2, cads_NR_RachProcedure_DcchDtchToCSS_Config_REQ(nr_Cell2, cs_TimingInfo_Now, tsc_CnfReq, /*cs_NR_RachProcedureConfig_RRCReestablishment(v_NR_PhysicalParameters, f_NR_CellInfo_GetRNTI(nr_Cell2), 256)*/p_RachProcedureConfig, //@sic R5-225418, R5s230476 R5s230884 sic@ cs_NR_SearchSpaceDlDciAssignment_CSSType1(v_NR_PhysicalParameters), cs_NR_SearchSpaceUlDciAssignmentCSS(v_NR_PhysicalParameters, cs_NR_DciFormat_0_0_Params))); v_ShortMAC_I := f_NR_Calculate_ShortMAC(nr_Cell1, nr_Cell2); //@sic R5s220683 sic@ alt{ []SRB.receive(car_NR_SRB0_RrcPdu_IND(nr_Cell2, cr_38508_RRCReestablishmentRequest(cr_NR_ReestabUE_Identity (v_RNTI_Value, v_PhysCellId, v_ShortMAC_I), otherFailure))) { t_Wait.stop; } []t_Wait.timeout { SRB.receive(car_NR_SRB0_RrcPdu_IND(nr_Cell2, cr_38508_RRCReestablishmentRequest(cr_NR_ReestabUE_Identity (v_RNTI_Value, v_PhysCellId, v_ShortMAC_I), otherFailure))); } } << SKIPPED CODE >>

MCC160 proposed implementation

function f_TC_8_1_5_8_1_NR5GC_TestBody() runs on NR5GC_PTC

{

var RNTI_Value v_RNTI_Value;

var PhysCellId v_PhysCellId;

var ShortMAC_I v_ShortMAC_I;

var NR_PhysicalParameters_Type v_NR_PhysicalParameters := f_NR_CellInfo_GetPhysicalParameters(nr_Cell2);  //@sic R5s240284 sic@

var NR_SecurityParams_Type v_SecurityParams;

var octetstring v_EncodeCapRequestNR;

var template (value) DRB_CountMSB_InfoList v_CountList;

var template (value) RadioBearerConfig v_RadioBearerConfig;

var template (value) RadioBearerConfig v_RadioBearerConfigResume;//@sic R5s220683 sic@

var template (value) CellGroupConfig v_CellGroupConfig;

var NR_RadioBearerList_Type v_SS_Config;

var template (value) RRCReconfiguration_v1530_IEs v_V1530Ext;

var template (value) NR_RRC_MSG_Request_Type v_RrcMsg;

var NR_SRB_COMMON_IND v_ReceivedNR_SRB_COMMON_IND;

var template (omit) octetstring v_EncodedSCG_Config := omit;

var NR_DRBInfoList_Type v_ExistingDRBs;

var template (value) NR_RachProcedureConfig_Type v_NR_RachProcedureConfig := f_NR_InitialiseRachProcedure(nr_Cell1); //@sic R5s240284 sic@

var integer v_Max_nPRB := f_NR_CellInfo_GetCarrierBandwidth_DL(nr_Cell1);

var NR_ResourceAllocation_Type v_NR_ResourceAllocation := {v_Max_nPRB, 28};

var template DRB_CountInfoList v_CountList_UE;

var template (value) NG_NAS_MSG_RequestList_Type v_ServerAcceptMsg;

var template (value) NR_CellPowerList_Type v_CellPowerList_AtT0;

var template (value) NR_CellPowerList_Type v_CellPowerList_AtT1;

//@sic R5s220683 sic@

var template (omit) DRB_ToAddModList v_DRB_ToAddModList := omit;

var integer i;

var SubcarrierSpacing v_SCS := f_NR_CellInfo_GetSCS(nr_Cell1);   //@sic R5s240284 sic@

var template (value) NR_RachProcedureConfig_Type v_RachProcedureConfigReestablishment := cs_NR_RachProcedureConfig_RRCReestablishment(v_NR_PhysicalParameters, f_NR_CellInfo_GetRNTI(nr_Cell2), 256);   //@sic R5s240284 sic@

timer t_Wait;

v_CellPowerList_AtT0 :={

cs_NR_CellPower(nr_Cell2, -96, tsc_NR_SuitableIntraFreqCellSSS_EPRE_FR2)//@sic R5-237355 sic@

};

v_CellPowerList_AtT1:={

cs_NR_CellPower(nr_Cell1, tsc_NR_NonSuitableOffCellSSS_EPRE, tsc_NR_NonSuitableOffCellSSS_EPRE),//@sic R5-237355 sic@

cs_NR_CellPower(nr_Cell2, tsc_NR_ServingCellSSS_EPRE_FR1, tsc_NR_ServingCellSSS_EPRE_FR2)//@sic R5-237355 sic@

};

f_NR_SetCellPowerList(v_CellPowerList_AtT0);

//@sic R5s240284 sic@

v_NR_RachProcedureConfig.RachProcedureList[0].ContentionResolution.Msg4_Based.SearchSpaceAndDci.DciInfo.Format.Format_1_0.PdschHarqTimingIndicator.Value := f_NR_DciFormat_1_0_RRCDelayPdschHarqTimingIndicator(v_SCS);

f_NR_SS_RachProcedureConfig(nr_Cell1, v_NR_RachProcedureConfig);

//@siclog "Step 1" siclog@

//The SS transmits a Paging message including a matched ng-5G-S-TMSI.

f_NR_Paging_Def(nr_Cell1, RRC_IDLE);

//@siclog "Step 2" siclog@

//The UE transmits an RRCSetupRequest message.

f_NR_RRC_SetupRequest_Def(nr_Cell1);

//@siclog "Steps 3 - 4" siclog@

//The SS transmits an RRCSetup message.

//The SS starts transmitting a periodic DCI from (10*2^numerology+1+TDL) slots after step 3 to schedule PUSCH.

//Check: Does the UE transmit an RRCSetupComplete message within (10*2^numerology+1+TDL+K2) slots15 (FDD)/18 (TDD) subframes after successful completion of step 3?

v_ReceivedNR_SRB_COMMON_IND := f_NR5GC_RRC_ProcedureDelay(nr_Cell1,

cas_NR_SRB0_RrcPdu_REQ(nr_Cell1,cs_TimingInfo_Now,f_NR_38508_RRCSetup(nr_Cell1, tsc_NR_RRC_TI_Def)),

car_NR_SRB1_RrcNasPdu_IND(nr_Cell1, cr_38508_RRCSetupComplete(tsc_NR_RRC_TI_Def, ?,- , *), cr_NG_NAS_IndWithPiggybacking(tsc_SHT_IntegrityProtected)),

10, "Step 4" , -, -,

true);  //@sic R5s240284 sic@

//@siclog "Step 5 - 6" siclog@

//The SS transmits a SecurityModeCommand message.

//The SS starts transmitting a periodic DCI from (5*2^numerology+1+TDL) slots after step 5 to schedule PUSCH.

//Check: Does the UE transmit a SecurityModeComplete message within (5*2^numerology+1+TDL+K2) 10 (FDD)/13 (TDD) subframes slots after successful completion of step 5?

v_SecurityParams := f_NR_Security_Get();

v_SecurityParams := f_NR_Authentication_InitAS (v_SecurityParams, v_ReceivedNR_SRB_COMMON_IND.Signalling.Nas[0].SecurityProtection.NasCount);

v_SecurityParams.AS_Ciphering.ActTimeList := f_NR_RRC_CipherActTime_GetInitial(nr_Cell1);

f_NR_SS_RRC_EnableIntProt_CiphULandDL(nr_Cell1,

v_SecurityParams.AS_Integrity,

v_SecurityParams.AS_Ciphering);

f_NR5GC_RRC_ProcedureDelay(nr_Cell1,

cas_NR_SRB1_RrcPdu_REQ(nr_Cell1,

-,

cs_38508_RRCSecurityModeCommand(tsc_NR_RRC_TI_Def,

cs_38508_SecurityAlgorithmConfig(v_SecurityParams.AS_Ciphering.Algorithm,

v_SecurityParams.AS_Integrity.Algorithm))),

car_NR_SRB1_RrcPdu_IND(nr_Cell1, cr_38508_RRCSecurityModeComplete(tsc_NR_RRC_TI_Def)),

5, "Step 6" );

f_NR_Security_Set(v_SecurityParams);

//@siclog "Step 7 - 8" siclog@

//The SS transmits a UECapabilityEnquiry message.

//The SS starts transmitting a periodic DCI from (80*2^numerology+1+TDL) slots after step 7 to schedule PUSCH.

//Check: Does the UE transmit a UECapabilityInformation message within (80*2^numerology+1+TDL+K2) slots after successful completion of step 7?

v_EncodeCapRequestNR := bit2oct(encvalue( cs_38508_UECapabilityRequestFilterNR({ cs_NR_BandInformationNR(f_NR_CellInfo_GetBandIndicator(nr_Cell1)) }) ));

f_NR5GC_RRC_ProcedureDelay(nr_Cell1,

cas_NR_SRB1_RrcPdu_REQ(nr_Cell1, -, cs_38508_UECapabilityEnquiry(tsc_NR_RRC_TI_Def, nr, v_EncodeCapRequestNR)),

car_NR_SRB1_RrcPdu_IND(nr_Cell1, cr_38508_UECapabilityInformationAny(tsc_NR_RRC_TI_Def)),

80, "Step 8",

cs_NR_FreqDomainSchedulExplicit(-, v_NR_ResourceAllocation.Lrbs)  //DCI with the maximum grant allowed by the carrier bandwidth  @sic R5-217815 sic@

);

//@siclog "Step 9 - 10" siclog@

//The SS transmits an RRCReconfiguration message to establish SRB2 and DRB #n for the UE

f_NR_BuildRRCReconfigExistingDRBs (v_RadioBearerConfig, v_CellGroupConfig, v_SS_Config, nr_Cell1);

v_SecurityParams := f_NR_Security_Get();

v_V1530Ext := cs_NR_RRCReconfiguration_v1530_IEs(bit2oct(encvalue(v_CellGroupConfig)));

//Reconfigure DRB in SS - existing DRBs are not reconfigured during release

f_NR_SS_CommonRadioBearerConfig(nr_Cell1, v_SS_Config, cs_TimingInfo_Now);

// Configure PDCP security

v_SecurityParams.AS_Ciphering.ActTimeList := f_NR_RRC_CipherActTime_GetCurrent(nr_Cell1, false);

f_NR_SS_RRC_EnableIntProt_CiphULandDL(nr_Cell1,

v_SecurityParams.AS_Integrity,

v_SecurityParams.AS_Ciphering);

v_ServerAcceptMsg := {cs_NG_NAS_Request(tsc_SHT_IntegrityProtected_Ciphered, f_Get_NG_ServiceAcceptMsg(v_ReceivedNR_SRB_COMMON_IND.Signalling.Nas[0].Pdu.Msg.service_Request.pduSessionStatus))};

v_RrcMsg := cs_NR_RRC_MSG_Request_DCCH(cs_38508_RRCReconfiguration(tsc_NR_RRC_TI_Def, v_RadioBearerConfig, -, -, v_V1530Ext));

f_NR5GC_RRC_ProcedureDelay(nr_Cell1,

cas_NR_RrcNasPduList_REQ(nr_Cell1, tsc_NR_RbId_SRB1, cs_TimingInfo_Now, v_RrcMsg, v_ServerAcceptMsg, -, -, cs_RlcBearerRouting_NR(nr_Cell1)),

car_NR_SRB_RrcPdu_IND(nr_Cell1, tsc_NR_RbId_SRB1, cr_38508_RRCReconfigurationComplete),

10, "Step 10" );

//@siclog "Step 11 - 12" siclog@

//The SS transmits a CounterCheck message.

//The SS starts transmitting a periodic DCI from (5*2^numerology+1+TDL) slots after step 11 to schedule PUSCH.

//Check: Does the UE transmit a CounterCheckResponse message within (5*2^numerology+1+TDL+K2)10 (FDD)/13 (TDD) subframes slots after successful completion of step 11?

v_ExistingDRBs := f_NR5GC_MobileInfo_GetDRBInfoList();

v_CountList := {cs_38508_DRB_CountMSB_Info(v_ExistingDRBs[0].DRB_Id, 1, 1)};

v_CountList_UE := {cr_DRB_CountInfo(v_ExistingDRBs[0].DRB_Id)};

f_NR5GC_RRC_ProcedureDelay(nr_Cell1,

cas_NR_SRB1_RrcPdu_REQ(nr_Cell1, -, cs_38508_CounterCheck(tsc_NR_RRC_TI_Def, v_CountList)),

car_NR_SRB1_RrcPdu_IND(nr_Cell1, cr_38508_CounterCheckResponse(tsc_NR_RRC_TI_Def, v_CountList_UE)),

5, "Step 12" );

//@siclog "Step 13" siclog@

//Void

//@siclog "Step 14a1-14a7" siclog@

if ( pc_inactiveState ){

//@siclog "Step 14a1" siclog@

//The SS transmits an RRCRelease message with suspendConfig.

f_NR_RRC_InactiveDef(nr_Cell1);

//@siclog "Step 14a2" siclog@

//Wait 1s to let UE enter RRC_INACTIVE state.

f_Delay(1.0);

//@siclog "Step 14a3" siclog@

//The SS transmits a Paging message including a matched ng-5G-S-TMSI.

f_NR_Paging_Def(nr_Cell1, RRC_INACTIVE);

v_CellGroupConfig := f_NR_GetSCGConfigResume(nr_Cell1);

v_EncodedSCG_Config := bit2oct(encvalue(v_CellGroupConfig));

//@sic R5s220683 sic@

for (i:= 0; i < lengthof(v_ExistingDRBs); i :=i+1) {

v_DRB_ToAddModList[i] := cs_NR_DRB_ToAddMod(omit, v_ExistingDRBs[i].DRB_Id, true_, -, omit);

}

v_RadioBearerConfigResume := cs_38508_RadioBearerConfigDef( {cs_NR_SRB_ToAddMod_ReestablishPDCP(tsc_SRB2)}, v_DRB_ToAddModList);

//@siclog "Step 14a4" siclog@

//The UE transmits an RRCResumeRequest message

SRB.receive(car_NR_SRB0_RrcPdu_IND(nr_Cell1, cr_38508_RRCResumeRequest(tsc_NR_ShortI_RNTI_Value1, mt_Access)));

f_NR_BuildRRCReconfigExistingDRBs (v_RadioBearerConfig, v_CellGroupConfig, v_SS_Config, nr_Cell1);

f_NR_SS_CommonRadioBearerConfig(nr_Cell1, v_SS_Config);

//Activate security on newly configured DRB

f_NR_SS_AS_ActivateSecurity_NoRefresh(nr_Cell1, cs_TimingInfo_Now);

//@siclog "Step 14a5" siclog@

//The SS transmits an RRCResume message for the UE

//@sic R5-237357 sic@

if(pc_reducedCP_Latency){

//@siclog "Step 14a5Aa1 - 14a5Aa2" siclog@

//The SS starts transmitting a periodic DCI from (10* 2 2^numerology +1+TDL) slots after step 14a5 to schedule PUSCH.

//Check: Does the UE transmit an RRCResumeComplete message within (6*2^numerology+1+TDL+K2) slots after successful completion of step 14a5?

f_NR5GC_RRC_ProcedureDelayWithRACH( nr_Cell1,

cas_NR_SRB1_RrcPdu_REQ(nr_Cell1, cs_TimingInfo_Now, cs_38508_RRCResume(tsc_NR_RRC_TI_Def, v_EncodedSCG_Config, v_RadioBearerConfigResume)),//@sic R5s220683 sic@

car_NR_SRB1_RrcPdu_IND(nr_Cell1, cr_38508_RRCResumeComplete),

6, "Step 14a5Aa2" );

} else {

//@siclog "Step 14a5Ab1 - 14a5Ab2" siclog@

//The SS starts transmitting a periodic DCI from n slots after step 14a5 to schedule PUSCH.

//Check: Does the UE transmit an RRCResumeComplete message within (10* 2 2^numerology +1+TDL+K2)slots after successful completion of step 14a5?

f_NR5GC_RRC_ProcedureDelayWithRACH( nr_Cell1,

cas_NR_SRB1_RrcPdu_REQ(nr_Cell1, cs_TimingInfo_Now, cs_38508_RRCResume(tsc_NR_RRC_TI_Def, v_EncodedSCG_Config, v_RadioBearerConfigResume)),//@sic R5s220683 sic@

car_NR_SRB1_RrcPdu_IND(nr_Cell1, cr_38508_RRCResumeComplete),

10, "Step 14a5Ab2" );

}

v_NR_RachProcedureConfig := f_NR_InitialiseRachProcedure(nr_Cell1);

f_NR_SS_RachProcedureConfig(nr_Cell1, v_NR_RachProcedureConfig);

//@siclog "Step 14a6-14a7" siclog@

//Void

}

//@siclog "Step 14-21" siclog@

//Void

//@siclog "Step 22" siclog@

//The SS re-adjusts the SS levels according to row "T1" in table 8.1.5.8.1.3.2-1/2 in order that the radio link quality of NR Cell 1 is degraded, and NR Cell 2 is suitable for camping

f_NR_SetCellPowerList(v_CellPowerList_AtT1);

//@siclog "Step 23" siclog@

//Wait 1s to let T310 expire

t_Wait.start (1.0);//@sic R5s220683 sic@

//@siclog "Step 24" siclog@

//The UE transmits an RRCReestablishmentRequest message on Cell 2

v_RNTI_Value := f_NR_CellInfo_GetRNTI(nr_Cell1);

v_PhysCellId := f_NR_CellInfo_GetPhysicalCellId(nr_Cell1);

f_NR_CellInfo_SetRNTI(nr_Cell2, tsc_C_RNTI_Value4);

f_NR_SS_C_RNTI_Config(nr_Cell2, tsc_C_RNTI_Value4, -, tsc_NoCnfReq, tsc_FollowOnFlag);

//@sic R5s240284 sic@

v_RachProcedureConfigReestablishment.RachProcedureList[0].ContentionResolution.Msg4_Based.SearchSpaceAndDci.DciInfo.Format.Format_1_0.PdschHarqTimingIndicator.Value := f_NR_DciFormat_1_0_RRCDelayPdschHarqTimingIndicator(v_SCS);

f_NR_SS_CommonCellConfig(nr_Cell2, cads_NR_RachProcedure_DcchDtchToCSS_Config_REQ(nr_Cell2, cs_TimingInfo_Now, tsc_CnfReq,

v_RachProcedureConfigReestablishment,  //@sic R5-225418, R5s230476 R5s230884 sic@

cs_NR_SearchSpaceDlDciAssignment_CSSType1(v_NR_PhysicalParameters),

cs_NR_SearchSpaceUlDciAssignmentCSS(v_NR_PhysicalParameters, cs_NR_DciFormat_0_0_Params)));

v_ShortMAC_I := f_NR_Calculate_ShortMAC(nr_Cell1, nr_Cell2);

//@sic R5s220683 sic@

alt{

[]SRB.receive(car_NR_SRB0_RrcPdu_IND(nr_Cell2, cr_38508_RRCReestablishmentRequest(cr_NR_ReestabUE_Identity (v_RNTI_Value, v_PhysCellId, v_ShortMAC_I), otherFailure)))

{

t_Wait.stop;

}

[]t_Wait.timeout

{

SRB.receive(car_NR_SRB0_RrcPdu_IND(nr_Cell2, cr_38508_RRCReestablishmentRequest(cr_NR_ReestabUE_Identity (v_RNTI_Value, v_PhysCellId, v_ShortMAC_I), otherFailure)));

}

}

//@siclog "Step 25 - 26 " siclog@

//The SS transmits an RRCReestablishment message on slot #i in the radio frame,where: i = 1 for micro=0,1  i = 21 for micro=3

//The SS starts transmitting a periodic DCI from (10*2^numerology+1+TDL) slots after step 25 to schedule PUSCH. IF UE transmits a Random Access Preamble, then SS sends RAR. (Note 1, Note 2)

//Check: Does the UE transmit an RRCReestablishmentComplete message within (10*2^numerology+1+TDL+K2) slots after successful completion of step 25? (Note 1, Note 2, Note 3)

//@sic R5-217815 R5-217830 sic@

f_NR_SS_AS_ActivateSecurity_Current(nr_Cell2, 0, -, true);

f_NR5GC_RRC_ProcedureDelayWithRACH(nr_Cell2,

cas_NR_SRB1_RrcPdu_REQ(nr_Cell2, cs_TimingInfo_Now, cs_38508_RRCReestablishment_Def(tsc_NR_RRC_TI_Def, 0)),

car_NR_SRB1_RrcPdu_IND(nr_Cell2, cr_38508_RRCReestablishmentComplete),

10, "Step 26");

//@siclog "Step 26A - 26B" siclog@

//The SS transmits an RRCReconfiguration message to resume existing radio bearer.

//The UE transmit an RRCReconfigurationComplete message.

f_NR5GC_RRCReestablishment_Steps8To13(nr_Cell1, nr_Cell2);

} //end of f_TC_8_1_5_8_1_NR5GC_TestBody

## Change to function f_TC_8_1_5_8_2_x_NR5GC_TestBody

| Function name | f_TC_8_1_5_8_2_x_NR5GC_TestBody |
| --- | --- |
| Reason for change | In the current TTCN implementation of the function f_TC_8_1_5_8_2_x_NR5GC_TestBody , Table 8.1.5.8.2.1.3.3-7: Physical layer parameters for DCI format 1_0 (Steps 6a5, Table 8.1.5.8.2.1.3.3-5) is not implemented |
| Summary of change | Table 8.1.5.8.2.1.3.3-7: Physical layer parameters for DCI format 1_0 is implemented at Steps 6a5 of the function f_TC_8_1_5_8_2_x_NR5GC_TestBody |
| TTCN module | RRC_Others_CA_NR5GC |
| MCC160 Comment | Accepted, the values need to be changed for all SCS. See MCC160 proposed implementation below. |

Before Change:

function f_TC_8_1_5_8_2_x_NR5GC_TestBody(NR_CellId_Type p_PCell, NR_CellId_Type p_SCell) runs on NR5GC_PTC { var template (value) CellGroupConfig v_SCGConfigCA := f_NR_GetSCGConfig_SCellAdd(p_SCell, tsc_NR_SCellIndex_1); var template (value) CellGroupConfig v_SCGConfig; var template (value) RRCReconfiguration_v1530_IEs v_V1530Ext := cs_NR_RRCReconfiguration_v1530_IEs(bit2oct(encvalue(v_SCGConfigCA))); var template (value) RadioBearerConfig v_RadioBearerConfig; var template (omit) DRB_ToAddModList v_DRB_ToAddModList := omit; var NR_DRBInfoList_Type v_ExistingDRBs := f_NR5GC_MobileInfo_GetDRBInfoList(); var template (value) NR_RachProcedureConfig_Type v_NR_RachProcedureConfig := f_NR_InitialiseRachProcedure(p_PCell); var integer i; var SubFrameTiming_Type v_TimingInfo; //@sic R5s210842 change 1 sic@ var SubFrameTiming_Type v_TimingInfo2; //@sic R5s210842 change 1 sic@ var template (value) NR_RadioBearerList_Type v_SS_Config := f_NR5GC_GetRadioBearerList(v_ExistingDRBs); //@sic R5s220835 sic@ // @siclog "Steps 1-3" siclog@ // The SS transmits an RRCReconfiguration message with sCellToAddModList to add SCell(Cell 3) // The SS starts transmitting a periodic DCI from (16*2microsec+1+TDL) slots after step 1 to schedule PUSCH. // Check: Does the UE transmit an RRCReconfigurationComplete message within (16*2microsec+1+TDL+K2) slots after successful completion of step 1? (Note 1, Note 2, Note 3) //@sic R5s210842 change 1 sic@ v_TimingInfo := f_NR_GetNextSendOccasion_DL(p_PCell,500); if (f_NR_CellInfo_GetSCS(p_PCell) != kHz15) { //@sic R5s230375 R5s230510 sic@ v_TimingInfo2 := f_SubFrameTiming_AddMilliSeconds(v_TimingInfo, 5); // 5 ms later } else { //Allow more time for FDD/TDD SCS=15kHz - k1=7 - reconfigure SS after UL HARQ feedback is received by SS v_TimingInfo2 := f_SubFrameTiming_AddMilliSeconds(v_TimingInfo, 10); // 10 ms later } f_NR_ConfigPCell_AddRelSCells(p_PCell, {p_SCell}, tsc_PCellIndex_0, cs_TimingInfo_NR(v_TimingInfo2), f_NR_CellInfo_GetRNTI(p_PCell)); f_NR_ConfigSCell(p_SCell,v_SCGConfigCA, cs_TimingInfo_NR(v_TimingInfo2)); //@sic R5s220365 sic@ f_NR5GC_RRC_ProcedureDelay(p_PCell, cas_NR_SRB1_RrcPdu_REQ(p_PCell, cs_TimingInfo_NR(v_TimingInfo), cs_38508_RRCReconfiguration(tsc_NR_RRC_TI_Def, -, -, -, v_V1530Ext), crs_MacBearerRouting(p_PCell)), //@sic R5s221132 sic@ car_NR_SRB_RrcPdu_IND(p_PCell, tsc_NR_RbId_SRB1, cr_38508_RRCReconfigurationComplete, - , crs_MacBearerRouting(p_PCell)), 16, //converted in slots inside the function "Step 3"); f_NR_ULGrantConfiguration_Start(p_PCell); //@sic R5s220365 sic@ // @siclog "Steps 4-5" siclog@ //The SS transmits an RRCReconfiguration message to release SCell(Cell 3) //UE transmits an RRCReconfigurationComplete message f_NR5GC_Rel_1SCell(p_PCell, p_SCell); if ( pc_inactiveState ) { // @siclog "Step6a1" siclog@ // IF pc_inactiveState THEN the SS transmits an RRCRelease message with suspendConfig. f_NR_RRC_InactiveDef(p_PCell); // @siclog "Step6a2" siclog@ //Wait 1s to let UE enter RRC_INACTIVE state. f_Delay(1.0); // @siclog "Step6a3" siclog@ // The SS transmits a Paging message including a matched ng-5G-S-TMSI. f_NR_Paging_Def(p_PCell, RRC_INACTIVE); << SKIPPED CODE >>

After Change:

function f_TC_8_1_5_8_2_x_NR5GC_TestBody(NR_CellId_Type p_PCell, NR_CellId_Type p_SCell) runs on NR5GC_PTC { var template (value) CellGroupConfig v_SCGConfigCA := f_NR_GetSCGConfig_SCellAdd(p_SCell, tsc_NR_SCellIndex_1); var template (value) CellGroupConfig v_SCGConfig; var template (value) RRCReconfiguration_v1530_IEs v_V1530Ext := cs_NR_RRCReconfiguration_v1530_IEs(bit2oct(encvalue(v_SCGConfigCA))); var template (value) RadioBearerConfig v_RadioBearerConfig; var template (omit) DRB_ToAddModList v_DRB_ToAddModList := omit; var NR_DRBInfoList_Type v_ExistingDRBs := f_NR5GC_MobileInfo_GetDRBInfoList(); var template (value) NR_RachProcedureConfig_Type v_NR_RachProcedureConfig := f_NR_InitialiseRachProcedure(p_PCell); var integer i; var SubFrameTiming_Type v_TimingInfo; //@sic R5s210842 change 1 sic@ var SubFrameTiming_Type v_TimingInfo2; //@sic R5s210842 change 1 sic@ var template (value) NR_RadioBearerList_Type v_SS_Config := f_NR5GC_GetRadioBearerList(v_ExistingDRBs); //@sic R5s220835 sic@ // @siclog "Steps 1-3" siclog@ // The SS transmits an RRCReconfiguration message with sCellToAddModList to add SCell(Cell 3) // The SS starts transmitting a periodic DCI from (16*2microsec+1+TDL) slots after step 1 to schedule PUSCH. // Check: Does the UE transmit an RRCReconfigurationComplete message within (16*2microsec+1+TDL+K2) slots after successful completion of step 1? (Note 1, Note 2, Note 3) //@sic R5s210842 change 1 sic@ v_TimingInfo := f_NR_GetNextSendOccasion_DL(p_PCell,500); if (f_NR_CellInfo_GetSCS(p_PCell) != kHz15) { //@sic R5s230375 R5s230510 sic@ v_TimingInfo2 := f_SubFrameTiming_AddMilliSeconds(v_TimingInfo, 5); // 5 ms later } else { //Allow more time for FDD/TDD SCS=15kHz - k1=7 - reconfigure SS after UL HARQ feedback is received by SS v_TimingInfo2 := f_SubFrameTiming_AddMilliSeconds(v_TimingInfo, 10); // 10 ms later } f_NR_ConfigPCell_AddRelSCells(p_PCell, {p_SCell}, tsc_PCellIndex_0, cs_TimingInfo_NR(v_TimingInfo2), f_NR_CellInfo_GetRNTI(p_PCell)); f_NR_ConfigSCell(p_SCell,v_SCGConfigCA, cs_TimingInfo_NR(v_TimingInfo2)); //@sic R5s220365 sic@ f_NR5GC_RRC_ProcedureDelay(p_PCell, cas_NR_SRB1_RrcPdu_REQ(p_PCell, cs_TimingInfo_NR(v_TimingInfo), cs_38508_RRCReconfiguration(tsc_NR_RRC_TI_Def, -, -, -, v_V1530Ext), crs_MacBearerRouting(p_PCell)), //@sic R5s221132 sic@ car_NR_SRB_RrcPdu_IND(p_PCell, tsc_NR_RbId_SRB1, cr_38508_RRCReconfigurationComplete, - , crs_MacBearerRouting(p_PCell)), 16, //converted in slots inside the function "Step 3"); f_NR_ULGrantConfiguration_Start(p_PCell); //@sic R5s220365 sic@ // @siclog "Steps 4-5" siclog@ //The SS transmits an RRCReconfiguration message to release SCell(Cell 3) //UE transmits an RRCReconfigurationComplete message f_NR5GC_Rel_1SCell(p_PCell, p_SCell); if ( pc_inactiveState ) { // @siclog "Step6a1" siclog@ // IF pc_inactiveState THEN the SS transmits an RRCRelease message with suspendConfig. f_NR_RRC_InactiveDef(p_PCell); // @siclog "Step6a2" siclog@ //Wait 1s to let UE enter RRC_INACTIVE state. f_Delay(1.0); v_NR_RachProcedureConfig := f_NR_InitialiseRachProcedure(nr_Cell1); if (f_NR_CellInfo_GetIsFDD(nr_Cell1)){ v_NR_RachProcedureConfig.RachProcedureList[0].ContentionResolution.Msg4_Based.SearchSpaceAndDci.DciInfo.Format.Format_1_0.PdschHarqTimingIndicator.Value := '010'B; } f_NR_SS_RachProcedureConfig(nr_Cell1, v_NR_RachProcedureConfig); // @siclog "Step6a3" siclog@ // The SS transmits a Paging message including a matched ng-5G-S-TMSI. f_NR_Paging_Def(p_PCell, RRC_INACTIVE); << SKIPPED CODE >>

MCC160 proposed implementation

/*

* @desc      Return PDSCH-to-HARQ_feedback timing indicator as according to 38.523-1 Table 8.1.5.8.2.1.3.3-7 and Table 8.1.5.8.1.3.3-11

*/

function f_NR_DciFormat_1_0_RRCDelayPdschHarqTimingIndicator(SubcarrierSpacing p_SCS) return B3_Type

{ //@sic R5s240284 sic@

var B3_Type v_Value;

select (p_SCS) {

case(kHz15)  { v_Value := '010'B; }   /* 3 slots */

case(kHz30)  { v_Value := '110'B; }   /* 7 slots */

case(kHz120) { v_Value := '010'B; }   /* 3 slots */

case else    { FatalError(__FILE__, __LINE__, "Invalid SubcarrierSpacing"); }

}

return v_Value;

}

function f_TC_8_1_5_8_2_x_NR5GC_TestBody(NR_CellId_Type     p_PCell,

NR_CellId_Type     p_SCell) runs on NR5GC_PTC

{

var template (value) CellGroupConfig v_SCGConfigCA := f_NR_GetSCGConfig_SCellAdd(p_SCell, tsc_NR_SCellIndex_1);

var template (value) CellGroupConfig v_SCGConfig;

var template (value) RRCReconfiguration_v1530_IEs v_V1530Ext := cs_NR_RRCReconfiguration_v1530_IEs(bit2oct(encvalue(v_SCGConfigCA)));

var template (value) RadioBearerConfig v_RadioBearerConfig;

var template (omit) DRB_ToAddModList v_DRB_ToAddModList := omit;

var NR_DRBInfoList_Type v_ExistingDRBs := f_NR5GC_MobileInfo_GetDRBInfoList();

var template (value) NR_RachProcedureConfig_Type v_NR_RachProcedureConfig := f_NR_InitialiseRachProcedure(p_PCell);

var integer i;

var SubFrameTiming_Type v_TimingInfo;    //@sic R5s210842 change 1 sic@

var SubFrameTiming_Type v_TimingInfo2;   //@sic R5s210842 change 1 sic@

var template (value) NR_RadioBearerList_Type v_SS_Config := f_NR5GC_GetRadioBearerList(v_ExistingDRBs);        //@sic R5s220835 sic@

var SubcarrierSpacing v_SCS := f_NR_CellInfo_GetSCS(p_PCell);   //@sic R5s240283 sic@

// @siclog "Steps 1-3" siclog@

// The SS transmits an RRCReconfiguration message with sCellToAddModList to add SCell(Cell 3)

// The SS starts transmitting a periodic DCI from (16*2microsec+1+TDL) slots after step 1 to schedule PUSCH.

// Check: Does the UE transmit an RRCReconfigurationComplete message within (16*2microsec+1+TDL+K2) slots after successful completion of step 1? (Note 1, Note 2, Note 3)

//@sic R5s210842 change 1 sic@

v_TimingInfo := f_NR_GetNextSendOccasion_DL(p_PCell,500);

if (v_SCS != kHz15) {                             //@sic R5s230375 R5s230510 sic@

v_TimingInfo2 := f_SubFrameTiming_AddMilliSeconds(v_TimingInfo, 5);   // 5 ms later

} else {

//Allow more time for FDD/TDD SCS=15kHz - k1=7 - reconfigure SS after UL HARQ feedback is received by SS

v_TimingInfo2 := f_SubFrameTiming_AddMilliSeconds(v_TimingInfo, 10); // 10 ms later

}

f_NR_ConfigPCell_AddRelSCells(p_PCell, {p_SCell}, tsc_PCellIndex_0, cs_TimingInfo_NR(v_TimingInfo2), f_NR_CellInfo_GetRNTI(p_PCell));

f_NR_ConfigSCell(p_SCell,v_SCGConfigCA, cs_TimingInfo_NR(v_TimingInfo2));

//@sic R5s220365 sic@

f_NR5GC_RRC_ProcedureDelay(p_PCell,

cas_NR_SRB1_RrcPdu_REQ(p_PCell, cs_TimingInfo_NR(v_TimingInfo), cs_38508_RRCReconfiguration(tsc_NR_RRC_TI_Def, -, -, -, v_V1530Ext), crs_MacBearerRouting(p_PCell)), //@sic R5s221132 sic@

car_NR_SRB_RrcPdu_IND(p_PCell, tsc_NR_RbId_SRB1, cr_38508_RRCReconfigurationComplete, - , crs_MacBearerRouting(p_PCell)),

16,   //converted in slots inside the function

"Step 3");

f_NR_ULGrantConfiguration_Start(p_PCell);   //@sic R5s220365 sic@

// @siclog "Steps 4-5" siclog@

//The SS transmits an RRCReconfiguration message to release SCell(Cell 3)

//UE transmits an RRCReconfigurationComplete message

f_NR5GC_Rel_1SCell(p_PCell, p_SCell);

if ( pc_inactiveState ) {

// @siclog "Step6a1" siclog@

// IF pc_inactiveState THEN the SS transmits an RRCRelease message with suspendConfig.

f_NR_RRC_InactiveDef(p_PCell);

// @siclog "Step6a2" siclog@

//Wait 1s to let UE enter RRC_INACTIVE state.

f_Delay(1.0);

// @sic R5s240284 sic@

// Set PDSCH-to-HARQ_feedback timing indicator in DCI 1_0 as per Table 8.1.5.8.1.3.3-11

v_NR_RachProcedureConfig.RachProcedureList[0].ContentionResolution.Msg4_Based.SearchSpaceAndDci.DciInfo.Format.Format_1_0.PdschHarqTimingIndicator.Value := f_NR_DciFormat_1_0_RRCDelayPdschHarqTimingIndicator(v_SCS);

f_NR_SS_RachProcedureConfig(nr_Cell1, v_NR_RachProcedureConfig);

// @siclog "Step6a3" siclog@

// The SS transmits a Paging message including a matched ng-5G-S-TMSI.

f_NR_Paging_Def(p_PCell, RRC_INACTIVE);

// @siclog "Step6a4" siclog@

// The UE transmits an RRCResumeRequest message.

f_NR_RRC_ResumeRequest_Def(p_PCell);

v_SCGConfig := f_NR_GetSCGConfigResume(p_PCell);                          // condition RESUME

v_SCGConfig.mac_CellGroupConfig := v_SCGConfigCA.mac_CellGroupConfig;     // condition SCell_add

v_SCGConfig.sCellToAddModList  := v_SCGConfigCA.sCellToAddModList;        // condition SCell_add

for (i:=0; i < lengthof(v_ExistingDRBs); i:=i+1) {

v_DRB_ToAddModList[i] := cs_NR_DRB_ToAddMod(omit, v_ExistingDRBs[i].DRB_Id, true_, -, omit);

}

v_RadioBearerConfig := cs_38508_RadioBearerConfigDef({cs_NR_SRB_ToAddMod_ReestablishPDCP(tsc_SRB2)}, v_DRB_ToAddModList);

f_NR_SS_CommonRadioBearerConfig(p_PCell, v_SS_Config, cs_TimingInfo_Now);  //@sic R5s220835 sic@

//@sic R5s210620 R5s210842 change 1 sic@

v_TimingInfo := f_NR_GetNextSendOccasion_DL(p_PCell,500);

if (v_SCS != kHz15) {   //@sic R5s230510 sic@

v_TimingInfo2 := f_SubFrameTiming_AddMilliSeconds(v_TimingInfo, 5); // 5 ms later

} else {

//Allow more time for FDD/TDD SCS=15kHz - k1=7 - reconfigure SS after UL HARQ feedback is received by SS

v_TimingInfo2 := f_SubFrameTiming_AddMilliSeconds(v_TimingInfo, 10); // 10 ms later

}

f_NR_ConfigPCell_AddRelSCells(p_PCell, {p_SCell}, tsc_PCellIndex_0, cs_TimingInfo_NR(v_TimingInfo2), f_NR_CellInfo_GetRNTI(p_PCell));

f_NR_ConfigSCell(p_SCell,v_SCGConfigCA, cs_TimingInfo_NR(v_TimingInfo2));

// @siclog "Steps 6a5 - 6a7" siclog@

//The SS transmits an RRCResume message with sCellToAddModList to add SCell(Cell 3)

//The SS starts transmitting a periodic DCI from (16*2micros+1+TDL) slots after step 6a5 to schedule PUSCH.(Note 1, Note 2)

//Check: Does the UE transmit an RRCResumeComplete message within (16*2micros+1+TDL+K2) slots after successful completion of step 6a5? (Note 1, Note 2, Note 3)

//@sic R5-235291 R5-237424 sic@

f_NR5GC_RRC_ProcedureDelayWithRACH(p_PCell,

cas_NR_SRB1_RrcPdu_REQ(p_PCell, cs_TimingInfo_NR(v_TimingInfo), cs_38508_RRCResume(tsc_NR_RRC_TI_Def, bit2oct(encvalue(v_SCGConfig)), v_RadioBearerConfig), crs_MacBearerRouting(p_PCell)),

car_NR_SRB1_RrcPdu_IND(p_PCell, cr_38508_RRCResumeComplete, -, crs_MacBearerRouting(p_PCell)),

16,                  //converted in slots inside the function

"Step 6a7");

f_NR_ULGrantConfiguration_Start(p_PCell);   //@sic R5s220365 sic@

//DRBs are not reconfigured in SS: that is ok as they are not used

v_NR_RachProcedureConfig := f_NR_InitialiseRachProcedure(nr_Cell1);   //@sic R5s240284 sic@

f_NR_SS_RachProcedureConfig(p_PCell, v_NR_RachProcedureConfig);

// @siclog "Steps 6a8 - 6a9" siclog@

//@sic R5-214618 sic@

f_NR5GC_Rel_1SCell(p_PCell, p_SCell);   //Release CA so that MAC bearer routing is disabled

}

}

## Change to function f_NR5GC_RRC_ProcedureDelay

| Function name | f_NR5GC_RRC_ProcedureDelay |
| --- | --- |
| Reason for change | In the current TTCN implementation of the function f_TC_8_1_5_8_2_x_NR5GC_TestBody , Table 8.1.5.8.2.1.3.3-7: Physical layer parameters for DCI format 1_0 (Steps 6a5, Table 8.1.5.8.2.1.3.3-5) is not implemented. |
| Summary of change | New parameter InitialAccess is added in the function f_NR5GC_RRC_ProcedureDelay to set the K1 value as “3” for FDD scenarios in procedure delay cases involving DCI_1_0 specific procedure |
| TTCN module | NR5GC_RRCSteps |
| MCC160 Comment | Postponed, conditional to prose CR agreement. K1 shall be set for all SCS values. See MCC160 proposed implementation. |

Before Change:

function f_NR5GC_RRC_ProcedureDelay(NR_CellId_Type p_NR_CellId, template (value) NR_SRB_COMMON_REQ p_SRB_COMMON_REQ, template (present) NR_SRB_COMMON_IND p_SRB_COMMON_IND, integer p_Nms, charstring p_TestcaseStep, template (value) NR_FreqDomainSchedulExplicit_Type p_FreqDomainSchedul := cs_NR_FreqDomainSchedulExplicitUL_Def, boolean p_NRDC_PduSession := false ) runs on NR5GC_PTC return NR_SRB_COMMON_IND { //@sic R5s230158: added p_NRDC_PduSession for TC_8_2_6_2_2 sic@ var integer v_T1Slots; var integer v_T2Slots; var integer v_T3Slots; var integer v_T4Slots; var SubFrameTiming_Type v_T1; // Send DL Msg var SubFrameTiming_Type v_T2; // HARQ ACK receive var SubFrameTiming_Type v_T3; // Start UL Grant start var SubFrameTiming_Type v_T4; // UL RRC PDU received var SubFrameTiming_Type v_Tnow; var integer v_SlotsNumberPerSubframe := f_NR_CellInfo_GetSlotsNumberPerSubframe(p_NR_CellId); var SubcarrierSpacing v_SCS := f_NR_CellInfo_GetSCS(p_NR_CellId); var integer v_Nslots := p_Nms*v_SlotsNumberPerSubframe + 1; //@sic R5s210842 change 2.1 sic@ var integer v_Delta1; var integer v_Delta2; var integer v_GrantPeriodicity; var integer v_DL_RetransmissionCnt := 0; var NR_SYSTEM_IND v_SYS_IND; var boolean v_T2Flag := false; var boolean v_T4Flag := false; var integer v_K1; var integer v_K2 := 4; var NR_SRB_COMMON_IND v_ReceivedNR_SRB_COMMON_IND; var integer v_MaxNumberSlots := 1024 * 10 * v_SlotsNumberPerSubframe; // Maximum number per SFN complete cycle var template (value) NR_SRB_COMMON_REQ v_SRB_COMMON_REQ := p_SRB_COMMON_REQ; var template (omit) GSM_MobilityInfo_Type v_GSM_MobilityInfo := f_Get_PDUSessionForDNNType(f_NR5GC_MobileInfo_GetSessionInfoList(), Internet_DNN); //@sic R5s230158 sic@ timer t_Watchdog; //@sic R5s210842 sic@ if (f_TimingInfo_IsNow(v_SRB_COMMON_REQ.Common.TimingInfo)) { v_T1 := f_NR_GetNextSendOccasion_DL(p_NR_CellId); v_SRB_COMMON_REQ.Common.TimingInfo.SubFrame := v_T1; } else { v_T1 := valueof(v_SRB_COMMON_REQ.Common.TimingInfo.SubFrame); } v_T1Slots := f_NR_ConvertTiming_To_TimingInSlots(v_T1, v_SlotsNumberPerSubframe); // Initialise GrantPeriodicity, Delta1, K1 if (f_NR_CellInfo_GetIsFDD(p_NR_CellId) and not(f_NR_CellInfo_GetHalfDuplexFDD(p_NR_CellId))){ //@sic R5-232191 sic@ v_GrantPeriodicity := 1; // Grant repeated in every slot v_Delta1 := 0; v_K1 := 7; } else { // TDD << SKIPPED CODE >>

After Change:

function f_NR5GC_RRC_ProcedureDelay(NR_CellId_Type p_NR_CellId, template (value) NR_SRB_COMMON_REQ p_SRB_COMMON_REQ, template (present) NR_SRB_COMMON_IND p_SRB_COMMON_IND, integer p_Nms, charstring p_TestcaseStep, template (value) NR_FreqDomainSchedulExplicit_Type p_FreqDomainSchedul := cs_NR_FreqDomainSchedulExplicitUL_Def, boolean p_NRDC_PduSession := false, boolean InitialAccess := false ) runs on NR5GC_PTC return NR_SRB_COMMON_IND { //@sic R5s230158: added p_NRDC_PduSession for TC_8_2_6_2_2 sic@ var integer v_T1Slots; var integer v_T2Slots; var integer v_T3Slots; var integer v_T4Slots; var SubFrameTiming_Type v_T1; // Send DL Msg var SubFrameTiming_Type v_T2; // HARQ ACK receive var SubFrameTiming_Type v_T3; // Start UL Grant start var SubFrameTiming_Type v_T4; // UL RRC PDU received var SubFrameTiming_Type v_Tnow; var integer v_SlotsNumberPerSubframe := f_NR_CellInfo_GetSlotsNumberPerSubframe(p_NR_CellId); var SubcarrierSpacing v_SCS := f_NR_CellInfo_GetSCS(p_NR_CellId); var integer v_Nslots := p_Nms*v_SlotsNumberPerSubframe + 1; //@sic R5s210842 change 2.1 sic@ var integer v_Delta1; var integer v_Delta2; var integer v_GrantPeriodicity; var integer v_DL_RetransmissionCnt := 0; var NR_SYSTEM_IND v_SYS_IND; var boolean v_T2Flag := false; var boolean v_T4Flag := false; var integer v_K1; var integer v_K2 := 4; var NR_SRB_COMMON_IND v_ReceivedNR_SRB_COMMON_IND; var integer v_MaxNumberSlots := 1024 * 10 * v_SlotsNumberPerSubframe; // Maximum number per SFN complete cycle var template (value) NR_SRB_COMMON_REQ v_SRB_COMMON_REQ := p_SRB_COMMON_REQ; var template (omit) GSM_MobilityInfo_Type v_GSM_MobilityInfo := f_Get_PDUSessionForDNNType(f_NR5GC_MobileInfo_GetSessionInfoList(), Internet_DNN); //@sic R5s230158 sic@ timer t_Watchdog; //@sic R5s210842 sic@ if (f_TimingInfo_IsNow(v_SRB_COMMON_REQ.Common.TimingInfo)) { v_T1 := f_NR_GetNextSendOccasion_DL(p_NR_CellId); v_SRB_COMMON_REQ.Common.TimingInfo.SubFrame := v_T1; } else { v_T1 := valueof(v_SRB_COMMON_REQ.Common.TimingInfo.SubFrame); } v_T1Slots := f_NR_ConvertTiming_To_TimingInSlots(v_T1, v_SlotsNumberPerSubframe); // Initialise GrantPeriodicity, Delta1, K1 if (f_NR_CellInfo_GetIsFDD(p_NR_CellId) and not(f_NR_CellInfo_GetHalfDuplexFDD(p_NR_CellId))){ //@sic R5-232191 sic@ v_GrantPeriodicity := 1; // Grant repeated in every slot v_Delta1 := 0; if(not InitialAccess) { v_K1 := 7; } else { v_K1 := 3; } } else { // TDD << SKIPPED CODE >>

MCC160 proposed implementation

function f_NR5GC_RRC_ProcedureDelay(NR_CellId_Type                        p_NR_CellId,

template (value) NR_SRB_COMMON_REQ    p_SRB_COMMON_REQ,

template (present) NR_SRB_COMMON_IND  p_SRB_COMMON_IND,

integer                               p_Nms,

charstring                            p_TestcaseStep,

template (value) NR_FreqDomainSchedulExplicit_Type p_FreqDomainSchedul := cs_NR_FreqDomainSchedulExplicitUL_Def,

boolean p_NRDC_PduSession := false,

boolean p_InitialAccess := false

) runs on NR5GC_PTC return NR_SRB_COMMON_IND

{ //@sic R5s230158: added p_NRDC_PduSession for TC_8_2_6_2_2 R5s250284: added p_InitialAccess sic@

var integer v_T1Slots;

var integer v_T2Slots;

var integer v_T3Slots;

var integer v_T4Slots;

var SubFrameTiming_Type v_T1; // Send DL Msg

var SubFrameTiming_Type v_T2; // HARQ ACK receive

var SubFrameTiming_Type v_T3; // Start UL Grant start

var SubFrameTiming_Type v_T4; // UL RRC PDU received

var SubFrameTiming_Type v_Tnow;

var integer v_SlotsNumberPerSubframe := f_NR_CellInfo_GetSlotsNumberPerSubframe(p_NR_CellId);

var SubcarrierSpacing v_SCS := f_NR_CellInfo_GetSCS(p_NR_CellId);

var integer v_Nslots := p_Nms*v_SlotsNumberPerSubframe + 1;  //@sic R5s210842 change 2.1 sic@

var integer v_Delta1;

var integer v_Delta2;

var integer v_GrantPeriodicity;

var integer v_DL_RetransmissionCnt := 0;

var NR_SYSTEM_IND v_SYS_IND;

var boolean v_T2Flag := false;

var boolean v_T4Flag := false;

var integer v_K1;

var integer v_K2 := 4;

var NR_SRB_COMMON_IND v_ReceivedNR_SRB_COMMON_IND;

var integer v_MaxNumberSlots := 1024 * 10 * v_SlotsNumberPerSubframe;     // Maximum number per SFN complete cycle

var template (value) NR_SRB_COMMON_REQ v_SRB_COMMON_REQ := p_SRB_COMMON_REQ;

var template (omit) GSM_MobilityInfo_Type v_GSM_MobilityInfo := f_Get_PDUSessionForDNNType(f_NR5GC_MobileInfo_GetSessionInfoList(), Internet_DNN);  //@sic R5s230158 sic@

timer t_Watchdog;

//@sic R5s210842 sic@

if (f_TimingInfo_IsNow(v_SRB_COMMON_REQ.Common.TimingInfo)) {

v_T1 := f_NR_GetNextSendOccasion_DL(p_NR_CellId);

v_SRB_COMMON_REQ.Common.TimingInfo.SubFrame := v_T1;

} else {

v_T1 := valueof(v_SRB_COMMON_REQ.Common.TimingInfo.SubFrame);

}

v_T1Slots := f_NR_ConvertTiming_To_TimingInSlots(v_T1, v_SlotsNumberPerSubframe);

// Initialise GrantPeriodicity, Delta1, K1

if (f_NR_CellInfo_GetIsFDD(p_NR_CellId) and not(f_NR_CellInfo_GetHalfDuplexFDD(p_NR_CellId))){ //@sic R5-232191 sic@

v_GrantPeriodicity := 1; // Grant repeated in every slot

v_Delta1 := 0;

v_K1 := 7;

} else { // TDD

select(v_SCS) {

case(kHz15) {

v_GrantPeriodicity := 5; //Grant repeated every 5 slots

v_Delta1 := (5 - (v_T1Slots + v_Nslots)) mod 5;

v_K1 := 7;

}

case(kHz30) {

v_GrantPeriodicity := 10; //Grant repeated every 10 slots

v_Delta1 := (4 - (v_T1Slots + v_Nslots) mod 10) mod 10;

v_K1 := 5;

}

case(kHz120) {

v_GrantPeriodicity := 5; //Grant repeated every 5 slots

v_Delta1 := (5- (v_T1Slots + v_Nslots) mod 5) mod 5;

v_K1 := 5;

}

case else { FatalError(__FILE__, __LINE__, "Invalid SCS"); }

}

}

if (p_InitialAccess) { //For RRCSetup DCI 1_0 is used - Set K1 as per Table 8.1.5.8.1.3.3-11

v_K1 := bit2int(f_NR_DciFormat_1_0_RRCDelayPdschHarqTimingIndicator(v_SCS)) + 1;

}

//Calculate T3

v_T3Slots := v_T1Slots + v_Nslots + v_Delta1;  //@sic R5s210842 change 2 sic@

v_T3 := f_NR_ConvertTimingInSlots_To_Timing(v_T3Slots, v_SCS, v_SlotsNumberPerSubframe);

//T1: send DL Msg

SRB.send(v_SRB_COMMON_REQ);

//Enable HARQ ack/nack reporting

f_NR_SS_SystemIndCtrlConfig(p_NR_CellId, cds_NR_SystemIndCtrl_UL_HARQ(enable), cs_TimingInfo_NR(v_T1)); //@sic R5s210842 change 2.4 sic@

//T3 configure SS with periodic UL Grant at T3

//@sic R5s240229 sic@

f_NR_ULGrantConfiguration_Start(p_NR_CellId, cs_TimingInfo_NR(v_T3), 28, p_FreqDomainSchedul, -, cs_UL_GrantConfig_ContPeriodic(v_GrantPeriodicity));

// Start WatchDog timer

v_Tnow := f_NR_GetCurrentTiming(p_NR_CellId);

t_Watchdog.start (int2float(f_SubFrameTiming_Duration(v_Tnow, v_T1) + p_Nms + 50) / 1000.0);  /* 50ms added to be at the safe side */

…

return v_ReceivedNR_SRB_COMMON_IND;

}

## Change to function f_NR5GC_RRC_ProcedureDelayWithRACH

| Function name | f_NR5GC_RRC_ProcedureDelayWithRACH |
| --- | --- |
| Reason for change | In the current TTCN implementation of the function f_TC_8_1_5_8_2_x_NR5GC_TestBody , Table 8.1.5.8.2.1.3.3-7: Physical layer parameters for DCI format 1_0 (Steps 6a5, Table 8.1.5.8.2.1.3.3-5) is not implemented. |
| Summary of change | K1 is set as 3 as per the tables 8.1.5.8.1.3.3-11: Physical layer parameters for DCI format 1_0 and Table 8.1.5.8.2.1.3.3-7: Physical layer parameters for DCI format 1_0 . |
| TTCN module | NR5GC_RRCSteps |
| MCC160 Comment | Accepted, implemented for all SCS. See MCC160 proposed implementation. |

Before Change:

function f_NR5GC_RRC_ProcedureDelayWithRACH(NR_CellId_Type p_NR_CellId, template (value) NR_SRB_COMMON_REQ p_SRB_COMMON_REQ, template (present) NR_SRB_COMMON_IND p_SRB_COMMON_IND, integer p_Nms, charstring p_TestcaseStep ) runs on NR_BASE_PTC return NR_SRB_COMMON_IND { //@sic R5-237419 renamed function to f_NR5GC_RRC_ProcedureDelayWithRACH sic@ var integer v_T1Slots; var integer v_T2Slots; var integer v_T3Slots; var integer v_T4Slots; var SubFrameTiming_Type v_T1; // Send DL Msg var SubFrameTiming_Type v_T2; // HARQ ACK receive var SubFrameTiming_Type v_T3; // Start UL Grant start var SubFrameTiming_Type v_T4; // UL RRC PDU received var SubFrameTiming_Type v_Tnow; var integer v_SlotsNumberPerSubframe := f_NR_CellInfo_GetSlotsNumberPerSubframe(p_NR_CellId); var SubcarrierSpacing v_SCS := f_NR_CellInfo_GetSCS(p_NR_CellId); var integer v_Nslots := p_Nms*v_SlotsNumberPerSubframe + 1; //@sic R5s210842 change 2.1 sic@ var integer v_Delta1; var integer v_Delta2; var integer v_GrantPeriodicity; var integer v_DL_RetransmissionCnt := 0; var NR_SYSTEM_IND v_SYS_IND; var boolean v_T2Flag := false; var boolean v_T4Flag := false; var integer v_K1; var integer v_K2 := 4; var NR_SRB_COMMON_IND v_ReceivedNR_SRB_COMMON_IND; var integer v_MaxNumberSlots := 1024 * 10 * v_SlotsNumberPerSubframe; // Maximum number per SFN complete cycle var boolean v_RACH_ProcedureStarted := false; var integer v_DeltaTDLmax := 0; var template (value) NR_SRB_COMMON_REQ v_SRB_COMMON_REQ := p_SRB_COMMON_REQ; timer t_Watchdog; // The SS transmits an RRCReestablishment/RRCResume message on slot #i in the radio frame,where: // i = 1 for micro=0,1 // i = 21 for micro=3 //@sic R5s240042 sic@ if (f_TimingInfo_IsNow(v_SRB_COMMON_REQ.Common.TimingInfo)) { v_T1 := f_NR_GetNextSendOccasion_DL(p_NR_CellId); v_T1.SFN.Number := (v_T1.SFN.Number + 1) mod 1024; } else { v_T1 := valueof(v_SRB_COMMON_REQ.Common.TimingInfo.SubFrame); } select (v_SCS) { case (kHz15) { v_T1.Subframe.Number := 1; v_T1.Slot.SlotOffset.Numerology0 := true; if (f_NR_CellInfo_GetIsFDD(p_NR_CellId)) { if (p_Nms == 6) { //pc_reducedCP_Latency @sic R5-237357 sic@ v_T1.Subframe.Number := 1; } else { v_T1.Subframe.Number := 5; } } } case (kHz30) { v_T1.Subframe.Number := 0; v_T1.Slot.SlotOffset.Numerology1 := 1; } case (kHz120) { v_T1.Subframe.Number := 2; v_T1.Slot.SlotOffset.Numerology3 := 5; if (p_Nms == 6) { //pc_reducedCP_Latency @sic R5-237357 sic@ v_T1.Subframe.Number := 5; } } case else { FatalError(__FILE__, __LINE__, "Invalid SCS");} } v_SRB_COMMON_REQ.Common.TimingInfo.SubFrame := v_T1; v_T1Slots := f_NR_ConvertTiming_To_TimingInSlots(v_T1, v_SlotsNumberPerSubframe); // Initialise GrantPeriodicity, Delta1, K1 if (f_NR_CellInfo_GetIsFDD(p_NR_CellId)){ v_GrantPeriodicity := 1; // Grant repeated in every slot v_Delta1 := 0; v_K1 := 7; v_DeltaTDLmax := 7; if (f_NR_CellInfo_GetHalfDuplexFDD(p_NR_CellId)) { select (p_Nms) { //@sic R5-232191 R5-237429 R5s240042 sic@ case (10, 16) {v_DeltaTDLmax := 2;} case (6) {v_DeltaTDLmax := 6;} case else { FatalError(__FILE__, __LINE__, "Invalid Nsf"); } } } << SKIPPED CODE >>

After Change:

function f_NR5GC_RRC_ProcedureDelayWithRACH(NR_CellId_Type p_NR_CellId, template (value) NR_SRB_COMMON_REQ p_SRB_COMMON_REQ, template (present) NR_SRB_COMMON_IND p_SRB_COMMON_IND, integer p_Nms, charstring p_TestcaseStep ) runs on NR_BASE_PTC return NR_SRB_COMMON_IND { //@sic R5-237419 renamed function to f_NR5GC_RRC_ProcedureDelayWithRACH sic@ var integer v_T1Slots; var integer v_T2Slots; var integer v_T3Slots; var integer v_T4Slots; var SubFrameTiming_Type v_T1; // Send DL Msg var SubFrameTiming_Type v_T2; // HARQ ACK receive var SubFrameTiming_Type v_T3; // Start UL Grant start var SubFrameTiming_Type v_T4; // UL RRC PDU received var SubFrameTiming_Type v_Tnow; var integer v_SlotsNumberPerSubframe := f_NR_CellInfo_GetSlotsNumberPerSubframe(p_NR_CellId); var SubcarrierSpacing v_SCS := f_NR_CellInfo_GetSCS(p_NR_CellId); var integer v_Nslots := p_Nms*v_SlotsNumberPerSubframe + 1; //@sic R5s210842 change 2.1 sic@ var integer v_Delta1; var integer v_Delta2; var integer v_GrantPeriodicity; var integer v_DL_RetransmissionCnt := 0; var NR_SYSTEM_IND v_SYS_IND; var boolean v_T2Flag := false; var boolean v_T4Flag := false; var integer v_K1; var integer v_K2 := 4; var NR_SRB_COMMON_IND v_ReceivedNR_SRB_COMMON_IND; var integer v_MaxNumberSlots := 1024 * 10 * v_SlotsNumberPerSubframe; // Maximum number per SFN complete cycle var boolean v_RACH_ProcedureStarted := false; var integer v_DeltaTDLmax := 0; var template (value) NR_SRB_COMMON_REQ v_SRB_COMMON_REQ := p_SRB_COMMON_REQ; timer t_Watchdog; // The SS transmits an RRCReestablishment/RRCResume message on slot #i in the radio frame,where: // i = 1 for micro=0,1 // i = 21 for micro=3 //@sic R5s240042 sic@ if (f_TimingInfo_IsNow(v_SRB_COMMON_REQ.Common.TimingInfo)) { v_T1 := f_NR_GetNextSendOccasion_DL(p_NR_CellId); v_T1.SFN.Number := (v_T1.SFN.Number + 1) mod 1024; } else { v_T1 := valueof(v_SRB_COMMON_REQ.Common.TimingInfo.SubFrame); } select (v_SCS) { case (kHz15) { v_T1.Subframe.Number := 1; v_T1.Slot.SlotOffset.Numerology0 := true; if (f_NR_CellInfo_GetIsFDD(p_NR_CellId)) { if (p_Nms == 6) { //pc_reducedCP_Latency @sic R5-237357 sic@ v_T1.Subframe.Number := 1; } else { v_T1.Subframe.Number := 5; } } } case (kHz30) { v_T1.Subframe.Number := 0; v_T1.Slot.SlotOffset.Numerology1 := 1; } case (kHz120) { v_T1.Subframe.Number := 2; v_T1.Slot.SlotOffset.Numerology3 := 5; if (p_Nms == 6) { //pc_reducedCP_Latency @sic R5-237357 sic@ v_T1.Subframe.Number := 5; } } case else { FatalError(__FILE__, __LINE__, "Invalid SCS");} } v_SRB_COMMON_REQ.Common.TimingInfo.SubFrame := v_T1; v_T1Slots := f_NR_ConvertTiming_To_TimingInSlots(v_T1, v_SlotsNumberPerSubframe); // Initialise GrantPeriodicity, Delta1, K1 if (f_NR_CellInfo_GetIsFDD(p_NR_CellId)){ v_GrantPeriodicity := 1; // Grant repeated in every slot v_Delta1 := 0; v_K1 := 3; v_DeltaTDLmax := 7; if (f_NR_CellInfo_GetHalfDuplexFDD(p_NR_CellId)) { select (p_Nms) { //@sic R5-232191 R5-237429 R5s240042 sic@ case (10, 16) {v_DeltaTDLmax := 2;} case (6) {v_DeltaTDLmax := 6;} case else { FatalError(__FILE__, __LINE__, "Invalid Nsf"); } } } << SKIPPED CODE >>

MCC160 proposed implementation

function f_NR5GC_RRC_ProcedureDelayWithRACH(NR_CellId_Type                        p_NR_CellId,

template (value) NR_SRB_COMMON_REQ    p_SRB_COMMON_REQ,

template (present) NR_SRB_COMMON_IND  p_SRB_COMMON_IND,

integer                               p_Nms,

charstring                            p_TestcaseStep

) runs on NR_BASE_PTC return NR_SRB_COMMON_IND

{  //@sic R5-237419 renamed function to f_NR5GC_RRC_ProcedureDelayWithRACH sic@

var integer v_T1Slots;

var integer v_T2Slots;

var integer v_T3Slots;

var integer v_T4Slots;

var SubFrameTiming_Type v_T1; // Send DL Msg

var SubFrameTiming_Type v_T2; // HARQ ACK receive

var SubFrameTiming_Type v_T3; // Start UL Grant start

var SubFrameTiming_Type v_T4; // UL RRC PDU received

var SubFrameTiming_Type v_Tnow;

var integer v_SlotsNumberPerSubframe := f_NR_CellInfo_GetSlotsNumberPerSubframe(p_NR_CellId);

var SubcarrierSpacing v_SCS := f_NR_CellInfo_GetSCS(p_NR_CellId);

var integer v_Nslots := p_Nms*v_SlotsNumberPerSubframe + 1;  //@sic R5s210842 change 2.1 sic@

var integer v_Delta1;

var integer v_Delta2;

var integer v_GrantPeriodicity;

var integer v_DL_RetransmissionCnt := 0;

var NR_SYSTEM_IND v_SYS_IND;

var boolean v_T2Flag := false;

var boolean v_T4Flag := false;

var integer v_K1;

var integer v_K2 := 4;

var NR_SRB_COMMON_IND v_ReceivedNR_SRB_COMMON_IND;

var integer v_MaxNumberSlots := 1024 * 10 * v_SlotsNumberPerSubframe;     // Maximum number per SFN complete cycle

var boolean v_RACH_ProcedureStarted := false;

var integer v_DeltaTDLmax := 0;

var template (value) NR_SRB_COMMON_REQ v_SRB_COMMON_REQ := p_SRB_COMMON_REQ;

timer t_Watchdog;

// The SS transmits an RRCReestablishment/RRCResume message on slot #i in the radio frame,where:

// i = 1 for micro=0,1

// i = 21 for micro=3

//@sic R5s240042 sic@

if (f_TimingInfo_IsNow(v_SRB_COMMON_REQ.Common.TimingInfo)) {

v_T1 := f_NR_GetNextSendOccasion_DL(p_NR_CellId);

v_T1.SFN.Number := (v_T1.SFN.Number + 1) mod 1024;

} else {

v_T1 := valueof(v_SRB_COMMON_REQ.Common.TimingInfo.SubFrame);

}

select (v_SCS) {

case (kHz15) {

v_T1.Subframe.Number := 1;

v_T1.Slot.SlotOffset.Numerology0 := true;

if (f_NR_CellInfo_GetIsFDD(p_NR_CellId)) {

if (p_Nms == 6) { //pc_reducedCP_Latency @sic R5-237357 sic@

v_T1.Subframe.Number := 1;

} else {

v_T1.Subframe.Number := 5;

}

}

}

case (kHz30) {

v_T1.Subframe.Number := 0;

v_T1.Slot.SlotOffset.Numerology1 := 1;

}

case (kHz120) {

v_T1.Subframe.Number := 2;

v_T1.Slot.SlotOffset.Numerology3 := 5;

if (p_Nms == 6) { //pc_reducedCP_Latency @sic R5-237357 sic@

v_T1.Subframe.Number := 5;

}

}

case else { FatalError(__FILE__, __LINE__, "Invalid SCS");}

}

v_SRB_COMMON_REQ.Common.TimingInfo.SubFrame := v_T1;

//@sic R5s240284 sic@

v_K1 := bit2int(f_NR_DciFormat_1_0_RRCDelayPdschHarqTimingIndicator(v_SCS)) + 1;

v_T1Slots := f_NR_ConvertTiming_To_TimingInSlots(v_T1, v_SlotsNumberPerSubframe);

// Initialise GrantPeriodicity, Delta1, K1

if (f_NR_CellInfo_GetIsFDD(p_NR_CellId)){

v_GrantPeriodicity := 1; // Grant repeated in every slot

v_Delta1 := 0;

// v_K1 := 7;

v_DeltaTDLmax := 7;

if (f_NR_CellInfo_GetHalfDuplexFDD(p_NR_CellId)) {

select (p_Nms) { //@sic R5-232191 R5-237429 R5s240042 sic@

case (10, 16) {v_DeltaTDLmax := 2;}

case (6)  {v_DeltaTDLmax := 6;}

case else { FatalError(__FILE__, __LINE__, "Invalid Nsf"); }

}

}

} else { // TDD

select(v_SCS) {

case(kHz15) {

v_GrantPeriodicity := 5; //Grant repeated every 5 slots

v_Delta1 := (5 - (v_T1Slots + v_Nslots)) mod 5;

// v_K1 := 7;

select (p_Nms) { //@sic R5-237429 R5s240042 sic@

case (10, 16) {v_DeltaTDLmax := 7;}

case (6)  {v_DeltaTDLmax := 11;}

case else { FatalError(__FILE__, __LINE__, "Invalid Nsf"); }

}

}

case(kHz30) {

v_GrantPeriodicity := 10; //Grant repeated every 10 slots

v_Delta1 := (4 - (v_T1Slots + v_Nslots) mod 10) mod 10;

// v_K1 := 5;

select (p_Nms) { //@sic R5-237429 R5s240042 sic@

case (10, 16) {v_DeltaTDLmax := 17;}

case (6)  {v_DeltaTDLmax := 25;}

case else { FatalError(__FILE__, __LINE__, "Invalid Nsf"); }

}

}

case(kHz120) {

v_GrantPeriodicity := 5; //Grant repeated every 5 slots

v_Delta1 := (5- (v_T1Slots + v_Nslots) mod 5) mod 5;

// v_K1 := 5;

select (p_Nms) { //@sic R5-237429 R5s240042 sic@

case (10, 16) {v_DeltaTDLmax := 32;}

case (6)  {v_DeltaTDLmax := 24;}

case else { FatalError(__FILE__, __LINE__, "Invalid Nsf"); }

}

}

case else { FatalError(__FILE__, __LINE__, "Invalid SCS"); }

}

}

……

return v_ReceivedNR_SRB_COMMON_IND;

}