3GPP TSG-RAN5 Meeting #2024-TTCN email	R5s240285

# Online, , 8th Dec 2023 - 31st Dec 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.523-3 | CR | 4806 | rev | - | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to NBIOT MAC Testcases (for NTN) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI13_Test, NB_IOT-UEConTest |  |  |  |  |  | Date: |  |  | 2024-04-02 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | In several NBIOT MAC testcases listed below, f_NBIOT_MobileInfo_GetNTNSatelliteType is called before vc_NBIOT_Global.MobileInfo.NTN_SatelliteType is set in f_NBIOT_Init. This will cause a runtime error as there are variables not yet initialized. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Moved instances of f_NBIOT_MobileInfo_GetNTNSatelliteType and f_NBIOT_GetNPDCCH_PeriodBundle (which contains a call to f_NBIOT_MobileInfo_GetNTNSatelliteType) to after f_NBIOT_Init is called. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may fail the test cases. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 22.3.1.2, 22.3.1.3, 22.3.1.4, 22.3.1.6 & 22.3.1.6a |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Table of Contents

1	Overview	2

2	Corrections Required:	2

2.1	Correction to function f_TC_22_3_1_2_NBIOT	2

2.2	Correction to function f_TC_22_3_1_3_NBIOT	4

2.3	Correction to function f_TC_22_3_1_4_NBIOT	5

2.4	Correction to function f_TC_22_3_1_6_NBIOT	6

2.5	Correction to function f_TC_22_3_1_6a_NBIOT	8

- Overview

This document lists all the essential changes needed to correct problems in the iwd-TTCN3-B2022-09_D24wk12 related to the title of this CR.

Contact: 	Juan Garcia Martin

Juan.GarciaMartin@rohde-schwarz.com

- Corrections Required:

  - Correction to function f_TC_22_3_1_2_NBIOT

| Function name | f_TC_22_3_1_2_NBIOT |
| --- | --- |
| Reason for change | Unintialized value error is present due to f_NBIOT_MobileInfo_GetNTNSatelliteType being called before f_NBIOT_Init functrion. f_NBIOT_GetNPDCCH_PeriodBundle also calls f_NBIOT_MobileInfo_GetNTNSatelliteType and gives the same error. |
| Summary of change | Moved occurrence of f_NBIOT_GetNPDCCH_PeriodBundle and f_NBIOT_MobileInfo_GetNTNSatelliteType to after f_NBIOT_L2_Preamble_State2B_NB which calls f_NBIOT_Init. |
| TTCN module | NBIOT_MAC_Testcases |
| MCC160 Comment | Accepted |

Before

function f_TC_22_3_1_2_NBIOT() runs on NBIOT_PTC

{

var NBIOT_CellId_Type v_CellId := nbiot_Cell1;

var NBIOT_IDLEUPDATED_STATE_Type v_TestLoopMode := STATE2A_NB_TESTLOOP_ModeG;

var template (value) MAC_MainConfig_NB_r13 v_MAC_MainConfig := cs_MAC_MainConfig_NB_SRB;

var template (value) DL_CCCH_Message_NB v_RrcConnSetup;

var NB_RLC_CountsInfoList_Type v_RLC_CountsInfoListByRef;                                   // RLC status to be maintained by Send/Receive functions

var EUTRA_ASN1_C_RNTI_Type v_C_RNTI;

var template (value) TimingInfo_Type v_TimingInfo;

var template (value) MAC_PDU_Type v_MAC_PDU;

var template (value) NB_DciDlInfo_Type v_DciDlInfo;

var MAC_SDUList_Type v_MAC_SDUList;

var SubFrameTiming_Type v_TimingStart;

var SubFrameTiming_Type v_TimingStart_Step21;

var SubFrameTiming_Type v_TimingStart_Step24;

var integer v_RlcSduSizeDL;

var integer v_TBS;

var integer v_Imcs;

var integer v_Isf;

var integer v_RepetitionNumber;

var integer v_IncrementVTS;

var integer v_TestLoopModeRepetitions := 0;                                       // according to test case preamble

var NB_SYSTEM_IND v_NB_SYSTEM_IND;

var template (value) NB_RarList_Type v_NB_RarList;

var template (value) NB_RachProcedureList_Type v_RachProcedureList;

var integer v_RAPID;

var integer v_RA_PreambleIndex;

var boolean v_NtnEnvironment;

var integer v_NpdcchPeriodBundle := f_NBIOT_GetNPDCCH_PeriodBundle(nbiot_Cell1); //@sic R5-241564 sic@

timer t_Timer;

if(f_NBIOT_MobileInfo_GetNTNSatelliteType()!= ntn_None)

{//@sic R5-241564 sic@

v_NtnEnvironment := true;

}

else

{

v_NtnEnvironment := false;

}

v_MAC_MainConfig.timeAlignmentTimerDedicated_r13 := sf5120;

v_RrcConnSetup := f_NBIOT_508_RRCConnectionSetup(-, -, v_MAC_MainConfig);

f_NBIOT_L2_Preamble_State2B_NB(v_RLC_CountsInfoListByRef, v_CellId, TransparentMode_RLC_MAC, v_TestLoopMode, v_RrcConnSetup, v_TestLoopModeRepetitions);

f_NBIOT_MAC_CPMode_SendMacPdu(v_CellId, -, cs_MAC_PDU_TimingAdvance_0);    /* @sic R5s170886 change 1: ensure timeAlignmentTimer(5.12s) not to time out before step 20 sic@ */

f_Delay(0.5);                                                              /* @sic R5s170886 change 1: to be sure that TA is acknowledged before HARQ_IndMode gets enabled sic@ */

f_NBIOT_SS_ConfigUL_HARQ_IndMode(v_CellId, enable);

v_C_RNTI := f_NBIOT_CellInfo_GetC_RNTI(v_CellId);

<< SKIPPED CODE >>

After

<< SKIPPED CODE >>

function f_TC_22_3_1_2_NBIOT() runs on NBIOT_PTC

{

var NBIOT_CellId_Type v_CellId := nbiot_Cell1;

var NBIOT_IDLEUPDATED_STATE_Type v_TestLoopMode := STATE2A_NB_TESTLOOP_ModeG;

var template (value) MAC_MainConfig_NB_r13 v_MAC_MainConfig := cs_MAC_MainConfig_NB_SRB;

var template (value) DL_CCCH_Message_NB v_RrcConnSetup;

var NB_RLC_CountsInfoList_Type v_RLC_CountsInfoListByRef;                                   // RLC status to be maintained by Send/Receive functions

var EUTRA_ASN1_C_RNTI_Type v_C_RNTI;

var template (value) TimingInfo_Type v_TimingInfo;

var template (value) MAC_PDU_Type v_MAC_PDU;

var template (value) NB_DciDlInfo_Type v_DciDlInfo;

var MAC_SDUList_Type v_MAC_SDUList;

var SubFrameTiming_Type v_TimingStart;

var SubFrameTiming_Type v_TimingStart_Step21;

var SubFrameTiming_Type v_TimingStart_Step24;

var integer v_RlcSduSizeDL;

var integer v_TBS;

var integer v_Imcs;

var integer v_Isf;

var integer v_RepetitionNumber;

var integer v_IncrementVTS;

var integer v_TestLoopModeRepetitions := 0;                                       // according to test case preamble

var NB_SYSTEM_IND v_NB_SYSTEM_IND;

var template (value) NB_RarList_Type v_NB_RarList;

var template (value) NB_RachProcedureList_Type v_RachProcedureList;

var integer v_RAPID;

var integer v_RA_PreambleIndex;

var boolean v_NtnEnvironment;

var integer v_NpdcchPeriodBundle;

timer t_Timer;

v_MAC_MainConfig.timeAlignmentTimerDedicated_r13 := sf5120;

v_RrcConnSetup := f_NBIOT_508_RRCConnectionSetup(-, -, v_MAC_MainConfig);

f_NBIOT_L2_Preamble_State2B_NB(v_RLC_CountsInfoListByRef, v_CellId, TransparentMode_RLC_MAC, v_TestLoopMode, v_RrcConnSetup, v_TestLoopModeRepetitions);

if(f_NBIOT_MobileInfo_GetNTNSatelliteType()!= ntn_None)

{//@sic R5-241564 sic@

v_NtnEnvironment := true;

}

else

{

v_NtnEnvironment := false;

}

v_NpdcchPeriodBundle := f_NBIOT_GetNPDCCH_PeriodBundle(nbiot_Cell1);

f_NBIOT_MAC_CPMode_SendMacPdu(v_CellId, -, cs_MAC_PDU_TimingAdvance_0);    /* @sic R5s170886 change 1: ensure timeAlignmentTimer(5.12s) not to time out before step 20 sic@ */

f_Delay(0.5);                                                              /* @sic R5s170886 change 1: to be sure that TA is acknowledged before HARQ_IndMode gets enabled sic@ */

f_NBIOT_SS_ConfigUL_HARQ_IndMode(v_CellId, enable);

v_C_RNTI := f_NBIOT_CellInfo_GetC_RNTI(v_CellId);

<< SKIPPED CODE >>

  - Correction to function f_TC_22_3_1_3_NBIOT

| Function name | f_TC_22_3_1_3_NBIOT |
| --- | --- |
| Reason for change | Unintialized value error is present due to f_NBIOT_MobileInfo_GetNTNSatelliteType being called before f_NBIOT_Init functrion. f_NBIOT_GetNPDCCH_PeriodBundle also calls f_NBIOT_MobileInfo_GetNTNSatelliteType and gives the same error. |
| Summary of change | Moved occurrence of f_NBIOT_GetNPDCCH_PeriodBundle to after f_NBIOT_L2_Preamble_State2B_NB which calls f_NBIOT_Init. |
| TTCN module | NBIOT_MAC_Testcases |
| MCC160 Comment | Accepted |

Before

function f_TC_22_3_1_3_NBIOT() runs on NBIOT_PTC

{

var NBIOT_CellId_Type v_CellId := nbiot_Cell1;

var NBIOT_IDLEUPDATED_STATE_Type v_TestLoopMode := STATE2A_NB_TESTLOOP_ModeG;

var NB_RLC_CountsInfoList_Type v_RLC_CountsInfoListByRef;                         // RLC status to be maintained by Send/Receive functions

var template (value) RLC_SDU_Type v_RlcSdu;

var template (value) RLC_SDUList_Type v_RlcSduList;

var EUTRA_ASN1_C_RNTI_Type v_C_RNTI;

var NB_L2_DATA_IND v_NB_L2_DATA_IND;

var TimingInfo_Type v_Timestamp;

var SubFrameTiming_Type v_SearchSpaceForULTransmission;

var template (value) TimingInfo_Type v_TimingInfo;

var template (value) NB_DciUlInfo_Type v_DciUlInfo;

var template (value) NB_UL_TransRetransmissionList_Type v_TransRetransmissionList;

var SubFrameTiming_Type v_TimingStart;

var template (present) MAC_PDU_Type v_MAC_PDU_RX;

var template (present) MAC_SDUList_Type v_MAC_SDUList_RX;

var template (value) NB_DciUlInfo_Type v_ULGrant;

var integer v_IncrementVRR;

var integer v_NpdcchPeriodBundle := f_NBIOT_GetNPDCCH_PeriodBundle(nbiot_Cell1); //@sic R5-241564 sic@

f_NBIOT_L2_Preamble_State2B_NB(v_RLC_CountsInfoListByRef, v_CellId, TransparentMode_RLC_MAC, v_TestLoopMode);

<< SKIPPED CODE >>

After

function f_TC_22_3_1_3_NBIOT() runs on NBIOT_PTC

{

var NBIOT_CellId_Type v_CellId := nbiot_Cell1;

var NBIOT_IDLEUPDATED_STATE_Type v_TestLoopMode := STATE2A_NB_TESTLOOP_ModeG;

var NB_RLC_CountsInfoList_Type v_RLC_CountsInfoListByRef;                         // RLC status to be maintained by Send/Receive functions

var template (value) RLC_SDU_Type v_RlcSdu;

var template (value) RLC_SDUList_Type v_RlcSduList;

var EUTRA_ASN1_C_RNTI_Type v_C_RNTI;

var NB_L2_DATA_IND v_NB_L2_DATA_IND;

var TimingInfo_Type v_Timestamp;

var SubFrameTiming_Type v_SearchSpaceForULTransmission;

var template (value) TimingInfo_Type v_TimingInfo;

var template (value) NB_DciUlInfo_Type v_DciUlInfo;

var template (value) NB_UL_TransRetransmissionList_Type v_TransRetransmissionList;

var SubFrameTiming_Type v_TimingStart;

var template (present) MAC_PDU_Type v_MAC_PDU_RX;

var template (present) MAC_SDUList_Type v_MAC_SDUList_RX;

var template (value) NB_DciUlInfo_Type v_ULGrant;

var integer v_IncrementVRR;

var integer v_NpdcchPeriodBundle;

f_NBIOT_L2_Preamble_State2B_NB(v_RLC_CountsInfoListByRef, v_CellId, TransparentMode_RLC_MAC, v_TestLoopMode);

v_NpdcchPeriodBundle := f_NBIOT_GetNPDCCH_PeriodBundle(nbiot_Cell1);

<< SKIPPED CODE >>

  - Correction to function f_TC_22_3_1_4_NBIOT

| Function name | f_TC_22_3_1_4_NBIOT |
| --- | --- |
| Reason for change | Unintialized value error is present due to f_NBIOT_MobileInfo_GetNTNSatelliteType being called before f_NBIOT_Init functrion. f_NBIOT_GetNPDCCH_PeriodBundle also calls f_NBIOT_MobileInfo_GetNTNSatelliteType and gives the same error. |
| Summary of change | Moved occurrence of f_NBIOT_GetNPDCCH_PeriodBundle and f_NBIOT_MobileInfo_GetNTNSatelliteType to after f_NBIOT_L2_Preamble_State2B_NB which calls f_NBIOT_Init. |
| TTCN module | NBIOT_MAC_Testcases |
| MCC160 Comment | Accepted |

Before

function f_TC_22_3_1_4_NBIOT() runs on NBIOT_PTC

{

var NBIOT_CellId_Type v_CellId := nbiot_Cell1;

var NBIOT_IDLEUPDATED_STATE_Type v_TestLoopMode := STATE2A_NB_TESTLOOP_ModeG;

var RetxBSR_Timer_NB_r13 v_RetxBSR_Timer_Preamble := pp16;                  /* @sic R5-176882, R5s170821 change 7: pp4 -> pp16 sic@ */

var PeriodicBSR_Timer_NB_r13 v_PeriodicBSR_Timer := pp64;                   /* @sic R5-176882, R5s170821 change 7: pp4 -> pp64 sic@ */

var RetxBSR_Timer_NB_r13 v_RetxBSR_Timer_Step21C := pp64;                   /* @sic R5-176882, R5s170821 change 7 sic@ */

var PeriodicBSR_Timer_NB_r13 v_PeriodicBSR_Timer_Step21C := pp16;           /* @sic R5-176882, R5s170821 change 7 sic@ */

var template (value) MAC_MainConfig_NB_r13 v_MAC_MainConfig := cs_MAC_MainConfig_NB_SRB;

var template (value) DL_CCCH_Message_NB v_RrcConnSetup;

var NB_RLC_CountsInfoList_Type v_RLC_CountsInfoListByRef;                          // RLC status to be maintained by Send/Receive functions

var SubFrameTiming_Type v_SubFrameTiming;

var template (value) TimingInfo_Type v_TimingInfo;

var template (value) RLC_SDUList_Type v_RlcSduList;

var template (value) RLC_SDUList_Type v_RlcSduList_Step5C;

var template (value) RLC_SDUList_Type v_RlcSduList_Step7;

var template (present) MAC_SDUList_Type v_MAC_SDUList_RX;

var template (present) MAC_PDU_Type v_MAC_PDU_RX;

var integer v_NoOfNpdcchPeriods;

var integer v_BSR_Index_Min;

var template (value) NB_RarList_Type v_NB_RarList;

var template (value) NB_RachProcedureList_Type v_RachProcedureList;

var float v_RetxBSRTimerValueMin;

var integer i;

var integer v_NpdcchPeriodBundle := f_NBIOT_GetNPDCCH_PeriodBundle(nbiot_Cell1); //@sic R5-241564 sic@

var float v_SchedulingOffset;

if(f_NBIOT_MobileInfo_GetNTNSatelliteType()!= ntn_None)

{//@sic R5-241564 sic@

v_SchedulingOffset := int2float(v_NpdcchPeriodBundle);

}

else

{

v_SchedulingOffset := int2float(3 * v_NpdcchPeriodBundle);

}

v_MAC_MainConfig.ul_SCH_Config_r13.periodicBSR_Timer_r13 := v_PeriodicBSR_Timer;

v_MAC_MainConfig.ul_SCH_Config_r13.retxBSR_Timer_r13 := v_RetxBSR_Timer_Preamble;

v_RrcConnSetup := f_NBIOT_508_RRCConnectionSetup(-, -, v_MAC_MainConfig);

f_NBIOT_L2_Preamble_State2B_NB(v_RLC_CountsInfoListByRef, v_CellId, TransparentMode_RLC_MAC, v_TestLoopMode, v_RrcConnSetup);

f_NBIOT_SS_ConfigRachPreambleIndMode(v_CellId, enable);                     /* @sic R5-176882, R5s170821 sic@ */

<< SKIPPED CODE >>

After

function f_TC_22_3_1_4_NBIOT() runs on NBIOT_PTC

{

var NBIOT_CellId_Type v_CellId := nbiot_Cell1;

var NBIOT_IDLEUPDATED_STATE_Type v_TestLoopMode := STATE2A_NB_TESTLOOP_ModeG;

var RetxBSR_Timer_NB_r13 v_RetxBSR_Timer_Preamble := pp16;                  /* @sic R5-176882, R5s170821 change 7: pp4 -> pp16 sic@ */

var PeriodicBSR_Timer_NB_r13 v_PeriodicBSR_Timer := pp64;                   /* @sic R5-176882, R5s170821 change 7: pp4 -> pp64 sic@ */

var RetxBSR_Timer_NB_r13 v_RetxBSR_Timer_Step21C := pp64;                   /* @sic R5-176882, R5s170821 change 7 sic@ */

var PeriodicBSR_Timer_NB_r13 v_PeriodicBSR_Timer_Step21C := pp16;           /* @sic R5-176882, R5s170821 change 7 sic@ */

var template (value) MAC_MainConfig_NB_r13 v_MAC_MainConfig := cs_MAC_MainConfig_NB_SRB;

var template (value) DL_CCCH_Message_NB v_RrcConnSetup;

var NB_RLC_CountsInfoList_Type v_RLC_CountsInfoListByRef;                          // RLC status to be maintained by Send/Receive functions

var SubFrameTiming_Type v_SubFrameTiming;

var template (value) TimingInfo_Type v_TimingInfo;

var template (value) RLC_SDUList_Type v_RlcSduList;

var template (value) RLC_SDUList_Type v_RlcSduList_Step5C;

var template (value) RLC_SDUList_Type v_RlcSduList_Step7;

var template (present) MAC_SDUList_Type v_MAC_SDUList_RX;

var template (present) MAC_PDU_Type v_MAC_PDU_RX;

var integer v_NoOfNpdcchPeriods;

var integer v_BSR_Index_Min;

var template (value) NB_RarList_Type v_NB_RarList;

var template (value) NB_RachProcedureList_Type v_RachProcedureList;

var float v_RetxBSRTimerValueMin;

var integer i;

var integer v_NpdcchPeriodBundle;

var float v_SchedulingOffset;

v_MAC_MainConfig.ul_SCH_Config_r13.periodicBSR_Timer_r13 := v_PeriodicBSR_Timer;

v_MAC_MainConfig.ul_SCH_Config_r13.retxBSR_Timer_r13 := v_RetxBSR_Timer_Preamble;

v_RrcConnSetup := f_NBIOT_508_RRCConnectionSetup(-, -, v_MAC_MainConfig);

f_NBIOT_L2_Preamble_State2B_NB(v_RLC_CountsInfoListByRef, v_CellId, TransparentMode_RLC_MAC, v_TestLoopMode, v_RrcConnSetup);

v_NpdcchPeriodBundle := f_NBIOT_GetNPDCCH_PeriodBundle(nbiot_Cell1);

if(f_NBIOT_MobileInfo_GetNTNSatelliteType()!= ntn_None)

{//@sic R5-241564 sic@

v_SchedulingOffset := int2float(v_NpdcchPeriodBundle);

}

else

{

v_SchedulingOffset := int2float(3 * v_NpdcchPeriodBundle);

}

f_NBIOT_SS_ConfigRachPreambleIndMode(v_CellId, enable);                     /* @sic R5-176882, R5s170821 sic@ */

<< SKIPPED CODE >>

  - Correction to function f_TC_22_3_1_6_NBIOT

| Function name | f_TC_22_3_1_6_NBIOT |
| --- | --- |
| Reason for change | Unintialized value error is present due to f_NBIOT_MobileInfo_GetNTNSatelliteType being called before f_NBIOT_Init functrion. f_NBIOT_GetNPDCCH_PeriodBundle also calls f_NBIOT_MobileInfo_GetNTNSatelliteType and gives the same error. |
| Summary of change | Moved occurrence of f_NBIOT_GetNPDCCH_PeriodBundle and f_NBIOT_MobileInfo_GetNTNSatelliteType to after f_NBIOT_L2_Preamble_State2B_NB which calls f_NBIOT_Init. |
| TTCN module | NBIOT_MAC_Testcases |
| MCC160 Comment | Accepted |

Before

function f_TC_22_3_1_6_NBIOT() runs on NBIOT_PTC

{

var NBIOT_CellId_Type v_CellId := nbiot_Cell1;

var NBIOT_IDLEUPDATED_STATE_Type v_TestLoopMode := STATE2A_NB_TESTLOOP_ModeG;

var NB_RLC_CountsInfoList_Type v_RLC_CountsInfoListByRef;                         // RLC status to be maintained by Send/Receive functions

var SubFrameTiming_Type v_SubFrameTiming;

var template (value) TimingInfo_Type v_TimingInfo;

var Table_22_3_1_6_3_2__2_Entry_Type v_TestData;

var RLC_SDU_Type v_RlcSdu;

var RLC_SDUList_Type v_RlcSduList;

var integer v_RlcSduSize;

var integer v_Start := 0;

var integer i;

var integer v_NpdcchPeriodBundle := f_NBIOT_GetNPDCCH_PeriodBundle(nbiot_Cell1); //@sic R5-241564 sic@

var float v_SchedulingOffset;

if(f_NBIOT_MobileInfo_GetNTNSatelliteType()!= ntn_None)

{//@sic R5-241564 sic@

v_SchedulingOffset := int2float(v_NpdcchPeriodBundle);

}

else

{

v_SchedulingOffset := int2float(3 * v_NpdcchPeriodBundle);

}

f_NBIOT_L2_Preamble_State2B_NB(v_RLC_CountsInfoListByRef, v_CellId, TransparentMode_RLC_MAC, v_TestLoopMode);

<< SKIPPED CODE >>

After

function f_TC_22_3_1_6_NBIOT() runs on NBIOT_PTC

{

var NBIOT_CellId_Type v_CellId := nbiot_Cell1;

var NBIOT_IDLEUPDATED_STATE_Type v_TestLoopMode := STATE2A_NB_TESTLOOP_ModeG;

var NB_RLC_CountsInfoList_Type v_RLC_CountsInfoListByRef;                         // RLC status to be maintained by Send/Receive functions

var SubFrameTiming_Type v_SubFrameTiming;

var template (value) TimingInfo_Type v_TimingInfo;

var Table_22_3_1_6_3_2__2_Entry_Type v_TestData;

var RLC_SDU_Type v_RlcSdu;

var RLC_SDUList_Type v_RlcSduList;

var integer v_RlcSduSize;

var integer v_Start := 0;

var integer i;

var integer v_NpdcchPeriodBundle;

var float v_SchedulingOffset;

f_NBIOT_L2_Preamble_State2B_NB(v_RLC_CountsInfoListByRef, v_CellId, TransparentMode_RLC_MAC, v_TestLoopMode);

v_NpdcchPeriodBundle := f_NBIOT_GetNPDCCH_PeriodBundle(nbiot_Cell1);

if(f_NBIOT_MobileInfo_GetNTNSatelliteType()!= ntn_None)

{//@sic R5-241564 sic@

v_SchedulingOffset := int2float(v_NpdcchPeriodBundle);

}

else

{

v_SchedulingOffset := int2float(3 * v_NpdcchPeriodBundle);

}

<< SKIPPED CODE >>

  - Correction to function f_TC_22_3_1_6a_NBIOT

| Function name | f_TC_22_3_1_6a_NBIOT |
| --- | --- |
| Reason for change | Unintialized value error is present due to f_NBIOT_MobileInfo_GetNTNSatelliteType being called before f_NBIOT_Init functrion. f_NBIOT_GetNPDCCH_PeriodBundle also calls f_NBIOT_MobileInfo_GetNTNSatelliteType and gives the same error. |
| Summary of change | Moved occurrence of f_NBIOT_GetNPDCCH_PeriodBundle and f_NBIOT_MobileInfo_GetNTNSatelliteType to after f_NBIOT_L2_Preamble_State2B_NB which calls f_NBIOT_Init. |
| TTCN module | NBIOT_MAC_Testcases |
| MCC160 Comment | Accepted |

Before

function f_TC_22_3_1_6a_NBIOT() runs on NBIOT_PTC

{

var NBIOT_CellId_Type v_CellId := nbiot_Cell1;

var NBIOT_IDLEUPDATED_STATE_Type v_TestLoopMode := STATE2A_NB_TESTLOOP_ModeG;

var NB_RLC_CountsInfoList_Type v_RLC_CountsInfoListByRef;                         // RLC status to be maintained by Send/Receive functions

var SubFrameTiming_Type v_SubFrameTiming;

var template (value) TimingInfo_Type v_TimingInfo;

var Table_22_3_1_6_3_2__2_Entry_Type v_TestData;

var RLC_SDU_Type v_RlcSdu;

var RLC_SDUList_Type v_RlcSduList;

var integer v_RlcSduSize;

var integer v_Start := 0;

var integer i;

var integer v_NpdcchPeriodBundle := f_NBIOT_GetNPDCCH_PeriodBundle(nbiot_Cell1); //@sic R5-241564 sic@

var float v_SchedulingOffset;

if(f_NBIOT_MobileInfo_GetNTNSatelliteType()!= ntn_None)

{//@sic R5-241564 sic@

v_SchedulingOffset := int2float(2 * v_NpdcchPeriodBundle);

}

else

{

v_SchedulingOffset := int2float(3 * v_NpdcchPeriodBundle);

}

<< SKIPPED CODE >>

After

function f_TC_22_3_1_6a_NBIOT() runs on NBIOT_PTC

{

var NBIOT_CellId_Type v_CellId := nbiot_Cell1;

var NBIOT_IDLEUPDATED_STATE_Type v_TestLoopMode := STATE2A_NB_TESTLOOP_ModeG;

var NB_RLC_CountsInfoList_Type v_RLC_CountsInfoListByRef;                         // RLC status to be maintained by Send/Receive functions

var SubFrameTiming_Type v_SubFrameTiming;

var template (value) TimingInfo_Type v_TimingInfo;

var Table_22_3_1_6_3_2__2_Entry_Type v_TestData;

var RLC_SDU_Type v_RlcSdu;

var RLC_SDUList_Type v_RlcSduList;

var integer v_RlcSduSize;

var integer v_Start := 0;

var integer i;

var integer v_NpdcchPeriodBundle;

var float v_SchedulingOffset;

f_NBIOT_L2_Preamble_State2B_NB(v_RLC_CountsInfoListByRef, v_CellId, TransparentMode_RLC_MAC, v_TestLoopMode);

v_NpdcchPeriodBundle := f_NBIOT_GetNPDCCH_PeriodBundle(nbiot_Cell1);

if(f_NBIOT_MobileInfo_GetNTNSatelliteType()!= ntn_None)

{//@sic R5-241564 sic@

v_SchedulingOffset := int2float(2 * v_NpdcchPeriodBundle);

}

else

{

v_SchedulingOffset := int2float(3 * v_NpdcchPeriodBundle);

}

<< SKIPPED CODE >>