3GPP TSG-RAN5 Meeting #2024-TTCN email	R5s240286

# Online, , 8th Dec 2023 - 31st Dec 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 3413 | rev | - | Current version: | 17.10.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to NR5GC test case 7.1.1.8.3 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI17_Test, NR_redcap_plus_ARCH-UEConTest |  |  |  |  |  | Date: |  |  | 2024-04-02 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | It has been observed that current call to f_NR_SS_SystemIndCtrlConfig() while disabling UL_Harq Ack sometimes allows (due to some delay in disabling it) to entertain the Harq Ack reception of rrcSetupComplete RLC Ack message which may lead the test case failure due to unexpected rrc pdu message. So, it should be called as soon as possible once rrcSetupComplete has been received. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Moved the call of f_NR_SS_SystemIndCtrlConfig() disabling UL_Harq Ack just after the reception of rrcSetupComplete. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may fail the test case. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.1.1.8.3.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

1	Overview	2

2	Corrections required:	2

2.1	Change to function f_TC_7_1_1_8_3_NR_TestBody	2

# Overview

This document lists all the essential changes needed to correct problems in the iwd-TTCN3-B2022-09_D24wk12 related to the title of this CR.

Contact: 	Juan Garcia Martin

Juan.GarciaMartin@rohde-schwarz.com

- Corrections required:

  - Change to function f_TC_7_1_1_8_3_NR_TestBody

| Function name | f_TC_7_1_1_8_3_NR_TestBody |
| --- | --- |
| Reason for change | It has been observed that current call to f_NR_SS_SystemIndCtrlConfig() while disabling UL_Harq Ack sometimes allows (due to some delay in disabling it) to entertain the Harq Ack reception of rrcSetupComplete RLC Ack message which may lead the test case failure due to unexpected rrc pdu message. So, it should be called as soon as possible once rrcSetupComplete has been received. |
| Summary of change | Moved the call of f_NR_SS_SystemIndCtrlConfig() disabling UL_Harq Ack just after the reception of rrcSetupComplete. |
| TTCN module |  |
| MCC160 Comment | Accepted – implemented as proposed |

Before Change:

<< SKIPPED CODE >> // @siclog "Step 5" siclog@ // Check: Does the UE transmit an RRCSetupRequest message by setting LCID to 35 // @siclog "Step 6" siclog@ // The SS transmits an RRCSetup message. // SS configured to do it // @siclog "Step 7" siclog@ // Check: Does the UE transmit HARQ ACK for step 6 on PUCCH configured by initialUplinkBWP-RedCap? alt { // RRCSetupRequest [not v_RRCSetupRequest_Received] SRB.receive (car_NR_SRB0_RrcPdu_IND(nr_Cell2, cr_38508_RRCSetupRequest)) -> value v_SRB_COMMON_IND { v_RRCSetupRequest_Received := true; v_TimingRRCSetupRequest := valueof(v_SRB_COMMON_IND.Common.TimingInfo.SubFrame); if (not (v_RRCSetupRequestComplete_Received and v_LCID_Received and v_UL_HARQ_Received)) { //Other events not all received repeat; // => wait for other events } } //RRCSetupComplete [not v_RRCSetupRequestComplete_Received] SRB.receive (car_NR_SRB1_RrcNasPdu_IND(nr_Cell2, cr_38508_RRCSetupComplete(tsc_NR_RRC_TI_Def), cr_NG_NAS_IndWithPiggybacking(tsc_SHT_IntegrityProtected), omit)) -> value v_SRB_COMMON_IND { v_RRCSetupRequestComplete_Received := true; v_NasInd := valueof (v_SRB_COMMON_IND.Signalling.Nas[0]); if (not (v_RRCSetupRequest_Received and v_LCID_Received and v_UL_HARQ_Received)) { //Other events not all received repeat; // => wait for other events } } // SysInd LCID [not v_LCID_Received] SYSIND.receive (cr_NR_SystemIndicationUL_SRB0_LCID_IND(nr_Cell2, cr_TimingInfo_Any, 35)) -> value v_SYSTEM_IND { v_LCID_Received := true; if (not (v_RRCSetupRequest_Received and v_RRCSetupRequestComplete_Received and v_UL_HARQ_Received)) { //Other events not all received repeat; // => wait for other events } } // SysInd UL HARQ [not v_UL_HARQ_Received] SYSIND.receive (car_NR_SYSTEM_UL_HARQ_IND(nr_Cell2, ack)) -> value v_SYSTEM_IND { v_UL_HARQ_Received := true; v_Timing_UL_HARQ := valueof(v_SYSTEM_IND.Common.TimingInfo.SubFrame); //Check: Does the UE transmit HARQ ACK for step 6 on PUCCH configured by initialUplinkBWP-RedCap? f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 7"); if (not (v_RRCSetupRequest_Received and v_RRCSetupRequestComplete_Received and v_LCID_Received)) { //Other events not all received repeat; // => wait for other events } } } //Check timing of events f_NR_RacingCond_CheckTwoEvents(nr_Cell2, v_TimingRRCSetupRequest, v_Timing_UL_HARQ); //Check: Does the UE transmit an RRCSetupRequest message by setting LCID to 35? if (v_RRCSetupRequest_Received and v_LCID_Received) { f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 5"); } else { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 5"); } // Disable HARQ reception indication f_NR_SS_SystemIndCtrlConfig(nr_Cell2, cds_NR_SystemIndCtrl_UL_HARQ(disable)); f_NR_SS_SystemIndCtrlConfig(nr_Cell2, cds_NR_SystemIndCtrl_UL_SRB0_LCID(disable)); // @siclog "Steps 8-13a1" siclog@ f_NR5GC_RRC_ConnectedState3N_Steps5_8(nr_Cell2, v_NasInd); } << SKIPPED CODE >>

After Change:

<< SKIPPED CODE >> // @siclog "Step 5" siclog@ // Check: Does the UE transmit an RRCSetupRequest message by setting LCID to 35 // @siclog "Step 6" siclog@ // The SS transmits an RRCSetup message. // SS configured to do it // @siclog "Step 7" siclog@ // Check: Does the UE transmit HARQ ACK for step 6 on PUCCH configured by initialUplinkBWP-RedCap? alt { // RRCSetupRequest [not v_RRCSetupRequest_Received] SRB.receive (car_NR_SRB0_RrcPdu_IND(nr_Cell2, cr_38508_RRCSetupRequest)) -> value v_SRB_COMMON_IND { v_RRCSetupRequest_Received := true; v_TimingRRCSetupRequest := valueof(v_SRB_COMMON_IND.Common.TimingInfo.SubFrame); if (not (v_RRCSetupRequestComplete_Received and v_LCID_Received and v_UL_HARQ_Received)) { //Other events not all received repeat; // => wait for other events } } //RRCSetupComplete [not v_RRCSetupRequestComplete_Received] SRB.receive (car_NR_SRB1_RrcNasPdu_IND(nr_Cell2, cr_38508_RRCSetupComplete(tsc_NR_RRC_TI_Def), cr_NG_NAS_IndWithPiggybacking(tsc_SHT_IntegrityProtected), omit)) -> value v_SRB_COMMON_IND { v_RRCSetupRequestComplete_Received := true; << MOVED CODE >> if(v_UL_HARQ_Received) { f_NR_SS_SystemIndCtrlConfig(nr_Cell2, cds_NR_SystemIndCtrl_UL_HARQ(disable)); } << MOVED CODE >> v_NasInd := valueof (v_SRB_COMMON_IND.Signalling.Nas[0]); if (not (v_RRCSetupRequest_Received and v_LCID_Received and v_UL_HARQ_Received)) { //Other events not all received repeat; // => wait for other events } } // SysInd LCID [not v_LCID_Received] SYSIND.receive (cr_NR_SystemIndicationUL_SRB0_LCID_IND(nr_Cell2, cr_TimingInfo_Any, 35)) -> value v_SYSTEM_IND { v_LCID_Received := true; if (not (v_RRCSetupRequest_Received and v_RRCSetupRequestComplete_Received and v_UL_HARQ_Received)) { //Other events not all received repeat; // => wait for other events } } // SysInd UL HARQ [not v_UL_HARQ_Received] SYSIND.receive (car_NR_SYSTEM_UL_HARQ_IND(nr_Cell2, ack)) -> value v_SYSTEM_IND { v_UL_HARQ_Received := true; v_Timing_UL_HARQ := valueof(v_SYSTEM_IND.Common.TimingInfo.SubFrame); //Check: Does the UE transmit HARQ ACK for step 6 on PUCCH configured by initialUplinkBWP-RedCap? f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 7"); if (not (v_RRCSetupRequest_Received and v_RRCSetupRequestComplete_Received and v_LCID_Received)) { //Other events not all received repeat; // => wait for other events } } } //Check timing of events f_NR_RacingCond_CheckTwoEvents(nr_Cell2, v_TimingRRCSetupRequest, v_Timing_UL_HARQ); //Check: Does the UE transmit an RRCSetupRequest message by setting LCID to 35? if (v_RRCSetupRequest_Received and v_LCID_Received) { f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 5"); } else { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 5"); } // Disable HARQ reception indication << MOVED CODE >> //f_NR_SS_SystemIndCtrlConfig(nr_Cell2, cds_NR_SystemIndCtrl_UL_HARQ(disable)); << MOVED CODE >> f_NR_SS_SystemIndCtrlConfig(nr_Cell2, cds_NR_SystemIndCtrl_UL_SRB0_LCID(disable)); // @siclog "Steps 8-13a1" siclog@ f_NR5GC_RRC_ConnectedState3N_Steps5_8(nr_Cell2, v_NasInd); } << SKIPPED CODE >>