3GPP TSG-RAN5 Meeting #2024-TTCN email	R5s240291

# Online, , 8th Dec 2023 - 31st Dec 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 34.229-3 | CR | 0945 | rev | - | Current version: | 17.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to IMS emergency call test cases 10.9 and 10.10 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2024-04-03 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | It may happen depending on UE configuration, that after IMS emergency call is released (SIP BYE sent by SS), UE will send immediately PDU release message. Current implementation of f_IMS_CC_ReleaseConnection adds 2.0 seconds delay by default to handle any further SIP message sent by the UE. In this case, SIP 200OK in response to BYE message has been already received (No IMS emergency registration), this delay may block the corrrect reception and handling of PDU release message and fail the test case. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Removed 2.0 seconds delay to handle PDU release message reception after SIP 200OK response to BYE has been sent. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may fail the test case. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 10.9.NR5GC, 10.10.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Overview	2

1	Corrections required:	2

1.1	Change to function f_TC_10_9_NR5GC_IMS2	2

1.2	Change to function f_TC_10_10_NR5GC_IMS2	3

# Overview

This document lists all the essential changes needed to correct problems in the iwd-TTCN3-B2022-09_D24wk12 related to the title of this CR.

Contact: 	Juan Garcia Martin

Juan.GarciaMartin@rohde-schwarz.com

- Corrections required:

  - Change to function f_TC_10_9_NR5GC_IMS2

| Function name | f_TC_10_9_NR5GC_IMS2 |
| --- | --- |
| Reason for change | Current implementation of f_IMS_CC_ReleaseConnection adds 2.0 seconds delay by default to handle any further SIP message sent by the UE. In this case, SIP 200OK in response to BYE message has been already received (No IMS emergency registration), this delay may block the corrrect reception and handling of PDU release message and fail the test case. |
| Summary of change | Removed 2.0 seconds delay to handle PDU release message reception after SIP 200OK response to BYE has been sent. |
| TTCN module | IMS_NR5GC_EmergencyCallTestcases |
| MCC160 Comment | Accepted |

Before Change:

<< SKIPPED CODE >> // Emergency call setup: 34.229-5 Annex A.6 f_IMS_EmergencyCallSetup_NR5GC_A_6_Step2ToEnd(v_IMS_DATA_REQ); f_IMS_PreliminaryPass(__FILE__, __LINE__, "Step 20"); //@siclog "Steps 21-22" siclog@ //SS sends BYE.(Step 1 of annex A.8) //UE sends 200 OK.(Step 1 of annex A.8) v_ContactUrlUE := f_IMS_PTC_ImsInfo_GetContactUrlUE(dialog); f_IMS_CallReleaseMT(v_ContactUrlUE); f_IMS_CC_ReleaseConnection(IPCAN_EmergencyCall_OptionalPDNRelease); //@sic R5s220609, R5-225362, R5s230677 sic@ f_IMS_TestBody_Set(false); f_IMS_IMS_SendCoOrdMsg(OtherPDN); // @sic R5s230677 sic@ << SKIPPED CODE >>

After Change:

<< SKIPPED CODE >> // Emergency call setup: 34.229-5 Annex A.6 f_IMS_EmergencyCallSetup_NR5GC_A_6_Step2ToEnd(v_IMS_DATA_REQ); f_IMS_PreliminaryPass(__FILE__, __LINE__, "Step 20"); //@siclog "Steps 21-22" siclog@ //SS sends BYE.(Step 1 of annex A.8) //UE sends 200 OK.(Step 1 of annex A.8) v_ContactUrlUE := f_IMS_PTC_ImsInfo_GetContactUrlUE(dialog); f_IMS_CallReleaseMT(v_ContactUrlUE); f_IMS_CC_ReleaseConnection(IPCAN_EmergencyCall_OptionalPDNRelease, -, 0.0); //@sic R5s220609, R5-225362, R5s230677 sic@ f_IMS_TestBody_Set(false); f_IMS_IMS_SendCoOrdMsg(OtherPDN); // @sic R5s230677 sic@ << SKIPPED CODE >>

  - Change to function f_TC_10_10_NR5GC_IMS2

| Function name | f_TC_10_10_NR5GC_IMS2 |
| --- | --- |
| Reason for change | Current implementation of f_IMS_CC_ReleaseConnection adds 2.0 seconds delay by default to handle any further SIP message sent by the UE. In this case, SIP 200OK in response to BYE message has been already received (No IMS emergency registration), this delay may block the corrrect reception and handling of PDU release message and fail the test case. |
| Summary of change | Removed 2.0 seconds delay to handle PDU release message reception after SIP 200OK response to BYE has been sent. |
| TTCN module | IMS_NR5GC_EmergencyCallTestcases |
| MCC160 Comment | Accepted |

Before Change:

<< SKIPPED CODE >> // Check for INVITE according to Table 10.10.3.3-2 f_IMS_PreliminaryPass(__FILE__, __LINE__, "Step 13"); //@siclog "Step 14 - 17" siclog@ //SS sends 100 Trying.(Step 2 of annex A.6) //SS sends 180 Ringing //SS sends 200 OK.(Step 4 of annex A.6) //UE sends ACK.(Step 5 of annex A.6) f_IMS_EmergencyCallSetup_NR5GC_A_6_Step2ToEnd(v_IMS_DATA_REQ); //@siclog "Steps 18-19" siclog@ // SS sends BYE.(Step 1 of annex A.8) //UE sends 200 OK.(Step 2 of annex A.8) v_ContactUrl := f_IMS_PTC_ImsInfo_GetContactUrlUE(dialog); f_IMS_CallReleaseMT(v_ContactUrl); f_IMS_CC_ReleaseConnection(IPCAN_EmergencyCall_OptionalPDNRelease); //@sic R5s220611, R5-225362, R5s230677 sic@ f_IMS_TestBody_Set(false); f_IMS_IMS_SendCoOrdMsg(OtherPDN); // @sic R5s230677 sic@ << SKIPPED CODE >>

After Change:

<< SKIPPED CODE >> // Check for INVITE according to Table 10.10.3.3-2 f_IMS_PreliminaryPass(__FILE__, __LINE__, "Step 13"); //@siclog "Step 14 - 17" siclog@ //SS sends 100 Trying.(Step 2 of annex A.6) //SS sends 180 Ringing //SS sends 200 OK.(Step 4 of annex A.6) //UE sends ACK.(Step 5 of annex A.6) f_IMS_EmergencyCallSetup_NR5GC_A_6_Step2ToEnd(v_IMS_DATA_REQ); //@siclog "Steps 18-19" siclog@ // SS sends BYE.(Step 1 of annex A.8) //UE sends 200 OK.(Step 2 of annex A.8) v_ContactUrl := f_IMS_PTC_ImsInfo_GetContactUrlUE(dialog); f_IMS_CallReleaseMT(v_ContactUrl); f_IMS_CC_ReleaseConnection(IPCAN_EmergencyCall_OptionalPDNRelease, -, 0.0); //@sic R5s220611, R5-225362, R5s230677 sic@ f_IMS_TestBody_Set(false); f_IMS_IMS_SendCoOrdMsg(OtherPDN); // @sic R5s230677 sic@ << SKIPPED CODE >>