3GPP TSG-RAN5 Meeting #2024-TTCN email	R5s240294

# Online, , 8th Dec 2023 - 31st Dec 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.523-3 | CR | 4808 | rev | - | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to NBIOT functions (for NTN) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI13_Test, NB_IOT-UEConTest |  |  |  |  |  | Date: |  |  | 2024-04-04 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | - In f_NBIOT_508Check_CP_MONonSMSDataTransfer the service accept messageg is sent and immediately followed by RRC Connection release procedure calls in test cases (for e.g 22.4.8/22.4.9), Due to the latency delays in transmitting the message and receiving RLC ACK, it is required to have a delay before proceeding to RRC Connection release procedure. - Due to Change 2.2 in R5s240266, In function f_NBIOT_RRC_ConnectionRelease_Local, v_NPDCCH_PeriodBundle is now passed as an incorrect value (10 for NTN and 1 for legacy), the parameter needs to be passed in msec (i.e 640msec for NTN and 64msec for legacy). |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Added delay for NTN condition - Introduced v_NPDCCH_Period and used v_NPDCCH_PeriodBundle*v_NPDCCH_Period to get the timing in msec and passed this as a parameter. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may fail the test case. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 22.4.8, 22.4.9 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Table of Contents

Table of Contents	2

1	Overview	2

2	Corrections required:	2

2.1	Correction to function f_NBIOT_508Check_CP_MONonSMSDataTransfer	2

2.2	Correction to function f_PreconfigureUE_ForNTN	4

# Overview

This document lists all the essential changes needed to correct problems in the iwd-TTCN3-B2022-09_D24wk12 related to the title of this CR.

Contact: 	Juan Garcia Martin

Juan.GarciaMartin@rohde-schwarz.com

# Corrections required:

## Correction to function f_NBIOT_508Check_CP_MONonSMSDataTransfer

| Function name | f_NBIOT_508Check_CP_MONonSMSDataTransfer |
| --- | --- |
| Reason for change | In this procedure the service accept messageg is sent and immediately followed by RRC Connection release procedure calls in test cases (for e.g 22.4.8/22.4.9), Due to the latency delays in transmitting the message and receiving RLC ACK, it is required to have a delay before proceeding to RRC Connection release procedure. |
| Summary of change | Added delay for NTN condition . |
| TTCN module | NBIOT_CommonProcedures |
| MCC160 Comment | Not needed, latency delay has been considered in f_NBIOT_RRC_ConnectionRelease_Common |

Before Change:

<< SKIPPED CODE >> // Step 3a3 SRB.receive(car_NB_SRB_NasPdu_IND(p_CellId, tsc_SRB1bis, cr_NAS_Indication(tsc_SHT_IntegrityProtected_Ciphered, cdr_CIOT_ActivateDefEpsBearerContextAccept(tsc_EpsDefaultBearerId)))); f_NBIOT_PreliminaryPass (__FILE__, __LINE__, "508Check CP MO NonSMS Data Transfer Step3a3"); if (pc_IP_PDN) { f_NBIOT_DelayForUserPlaneSignalling_CP(p_CellId); } // Step 3a4 SRB.receive(car_NB_SRB_NasPdu_IND(p_CellId, tsc_SRB1bis, cr_NAS_Indication(tsc_SHT_IntegrityProtected_Ciphered, cr_ESM_DATA_TRANSPORT (?, p_DataExpected)))); } // Step 4 SRB.send(cas_NB_SRB_NasPdu_REQ(p_CellId, tsc_SRB1bis, cs_TimingInfo_Now, cs_NAS_Request(tsc_SHT_IntegrityProtected_Ciphered, cs_SERVICE_ACCEPT))); } << SKIPPED CODE >>

After Change:

<< SKIPPED CODE >> // Step 3a3 SRB.receive(car_NB_SRB_NasPdu_IND(p_CellId, tsc_SRB1bis, cr_NAS_Indication(tsc_SHT_IntegrityProtected_Ciphered, cdr_CIOT_ActivateDefEpsBearerContextAccept(tsc_EpsDefaultBearerId)))); f_NBIOT_PreliminaryPass (__FILE__, __LINE__, "508Check CP MO NonSMS Data Transfer Step3a3"); if (pc_IP_PDN) { f_NBIOT_DelayForUserPlaneSignalling_CP(p_CellId); } // Step 3a4 SRB.receive(car_NB_SRB_NasPdu_IND(p_CellId, tsc_SRB1bis, cr_NAS_Indication(tsc_SHT_IntegrityProtected_Ciphered, cr_ESM_DATA_TRANSPORT (?, p_DataExpected)))); } // Step 4 SRB.send(cas_NB_SRB_NasPdu_REQ(p_CellId, tsc_SRB1bis, cs_TimingInfo_Now, cs_NAS_Request(tsc_SHT_IntegrityProtected_Ciphered, cs_SERVICE_ACCEPT))); if(pc_NB_ntn_only_Connectivity_EPC){ //added delay for NTN f_Delay(int2float(f_NBIOT_NTN_Get_T_NTNoffset(p_CellId))/500.0); } } << SKIPPED CODE >>

## Correction to function f_PreconfigureUE_ForNTN

| Function name | f_NBIOT_RRC_ConnectionRelease_Local |
| --- | --- |
| Reason for change | Due to Change 2.2 in R5s240266, v_NPDCCH_PeriodBundle (10 for NTN and 1 for legacy) is now passed as an incorrect value, the parameter needs to be passes in msec (i.e 640msec for NTN and 64msec for legacy). |
| Summary of change | Introduced v_NPDCCH_Period and used v_NPDCCH_PeriodBundle*v_NPDCCH_Period to get the timing in msec and passed this as a parameter. |
| TTCN module | Common4G5G_Functions |
| MCC160 Comment | Not needed, since R5s240266 has been implemented alternatively |

Before Change:

function f_NBIOT_RRC_ConnectionRelease_Local(NBIOT_CellId_Type p_CellId, IOT_STATE_Type p_IOT_State, SubFrameTiming_Type p_TimingAtT) runs on NBIOT_PTC { /* NOTE: the function schedules all steps to release and reconfigure the SRBs (and default DRB) * => this will in fact happen in the future */ var SubFrameTiming_Type v_Timing := p_TimingAtT; var template (value) TimingInfo_Type v_TimingInfo; var DRB_IdentityList_Type v_DrbIdList := f_NBIOT_SS_ActiveDRBs(p_CellId, p_IOT_State); // @sic R5s180547 sic@ var template (omit) integer v_HarqProcessNumber := f_NBIOT_CellInfo_GetDefaultHarqProcessNumber(p_CellId); /* @sic R5w180305: support of two HARQ processes sic@ */ var integer v_NPDCCH_PeriodBundle; // depending on NW type either NPDCCH period or NPDCCH bundle v_NPDCCH_PeriodBundle := f_NBIOT_GetNPDCCH_PeriodBundle(p_CellId); //@sic R5-235443, R5s230478 cl 2.7, R5-237436 sic@ /* Step 1 at T: disable the UL grant */ v_TimingInfo := cs_TimingInfo_EUTRA_NB(v_Timing); f_NBIOT_ULGrantTransmission(p_CellId, cs_NB_UL_GrantScheduling_Stop, v_TimingInfo); // @sic R5s220319 - MCC160 implementation: use of timing information according to 36.523-3 clause 7A.7 sic@ /* Step 2 at T+64ms */ v_Timing := f_SubFrameTiming_AddMilliSeconds(p_TimingAtT, v_NPDCCH_PeriodBundle); //@sic R5-s235443, R5-237436 sic@ v_TimingInfo := cs_TimingInfo_EUTRA_NB(v_Timing); if (p_IOT_State == USER_PLANE) { /* release security */ f_NBIOT_SS_RRC_ReleaseSecurity(p_CellId, v_TimingInfo); } f_NBIOT_ULGrantTransmission(p_CellId, cs_NB_UL_GrantScheduling_Start(cs_NB_PeriodicGrant(cs_NB_DciInfo_CcchDcchDtchUL(-, -, -, v_HarqProcessNumber), /* @sic R5w180305: support of two HARQ processes sic@ */ 4)), /* @sic R5-177097: give 4 sub-sequent UL grants to allow the UE to do RLC ACK sic@ */ v_TimingInfo); /* Step 3 at T+80ms: Release SRBs and DRBs */ v_Timing := f_SubFrameTiming_AddMilliSeconds(p_TimingAtT, 16 + v_NPDCCH_PeriodBundle); //@sic R5-s235443, R5-237436 sic@ v_TimingInfo := cs_TimingInfo_EUTRA_NB(v_Timing); f_NBIOT_SS_SRBs_DRBs_Release(p_CellId, p_IOT_State, v_TimingInfo, v_DrbIdList); // @sic R5s180547: v_DrbIdList sic@ /* Step 4 at T+96ms: Configure SRBs and DRBs */ v_Timing := f_SubFrameTiming_AddMilliSeconds(p_TimingAtT, 32 + v_NPDCCH_PeriodBundle); //@sic R5-s235443, R5-237436 sic@ v_TimingInfo := cs_TimingInfo_EUTRA_NB(v_Timing); f_NBIOT_SS_SRBs_DRBs_Config(p_CellId, p_IOT_State, v_TimingInfo, omit); }

After Change:

function f_NBIOT_RRC_ConnectionRelease_Local(NBIOT_CellId_Type p_CellId, IOT_STATE_Type p_IOT_State, SubFrameTiming_Type p_TimingAtT) runs on NBIOT_PTC { /* NOTE: the function schedules all steps to release and reconfigure the SRBs (and default DRB) * => this will in fact happen in the future */ var SubFrameTiming_Type v_Timing := p_TimingAtT; var template (value) TimingInfo_Type v_TimingInfo; var DRB_IdentityList_Type v_DrbIdList := f_NBIOT_SS_ActiveDRBs(p_CellId, p_IOT_State); // @sic R5s180547 sic@ var template (omit) integer v_HarqProcessNumber := f_NBIOT_CellInfo_GetDefaultHarqProcessNumber(p_CellId); /* @sic R5w180305: support of two HARQ processes sic@ */ var integer v_NPDCCH_PeriodBundle; // depending on NW type either NPDCCH period or NPDCCH bundle var integer v_NPDCCH_Period; // v_NPDCCH_Period := f_NBIOT_SearchSpace_GetPdcchPeriod(); // update to get in msec v_NPDCCH_PeriodBundle := f_NBIOT_GetNPDCCH_PeriodBundle(p_CellId); //@sic R5-235443, R5s230478 cl 2.7, R5-237436 sic@ /* Step 1 at T: disable the UL grant */ v_TimingInfo := cs_TimingInfo_EUTRA_NB(v_Timing); f_NBIOT_ULGrantTransmission(p_CellId, cs_NB_UL_GrantScheduling_Stop, v_TimingInfo); // @sic R5s220319 - MCC160 implementation: use of timing information according to 36.523-3 clause 7A.7 sic@ /* Step 2 at T+64ms */ // update to get in msec v_Timing := f_SubFrameTiming_AddMilliSeconds(p_TimingAtT, (v_NPDCCH_PeriodBundle*v_NPDCCH_Period)); //@sic R5-s235443, R5-237436 sic@ v_TimingInfo := cs_TimingInfo_EUTRA_NB(v_Timing); if (p_IOT_State == USER_PLANE) { /* release security */ f_NBIOT_SS_RRC_ReleaseSecurity(p_CellId, v_TimingInfo); } f_NBIOT_ULGrantTransmission(p_CellId, cs_NB_UL_GrantScheduling_Start(cs_NB_PeriodicGrant(cs_NB_DciInfo_CcchDcchDtchUL(-, -, -, v_HarqProcessNumber), /* @sic R5w180305: support of two HARQ processes sic@ */ 4)), /* @sic R5-177097: give 4 sub-sequent UL grants to allow the UE to do RLC ACK sic@ */ v_TimingInfo); /* Step 3 at T+80ms: Release SRBs and DRBs */ // update to get in msec v_Timing := f_SubFrameTiming_AddMilliSeconds(p_TimingAtT, 16 + (v_NPDCCH_PeriodBundle*v_NPDCCH_Period)); //@sic R5-s235443, R5-237436 sic@ v_TimingInfo := cs_TimingInfo_EUTRA_NB(v_Timing); f_NBIOT_SS_SRBs_DRBs_Release(p_CellId, p_IOT_State, v_TimingInfo, v_DrbIdList); // @sic R5s180547: v_DrbIdList sic@ /* Step 4 at T+96ms: Configure SRBs and DRBs */ //update to get in msec v_Timing := f_SubFrameTiming_AddMilliSeconds(p_TimingAtT, 32 + (v_NPDCCH_PeriodBundle*v_NPDCCH_Period)); //@sic R5-s235443, R5-237436 sic@ v_TimingInfo := cs_TimingInfo_EUTRA_NB(v_Timing); f_NBIOT_SS_SRBs_DRBs_Config(p_CellId, p_IOT_State, v_TimingInfo, omit); }