3GPP TSG-RAN5 Meeting #2024-TTCN email	R5s240295

# Online, , 8th Dec 2023 - 31st Dec 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.523-3 | CR | 4809 | rev | - | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to NBIOT-NTN testcase 22.4.30 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI17_Test, LTE_NBIOT_eMTC_NTN_plus_EPS-UEConTest |  |  |  |  |  | Date: |  |  | 2024-04-04 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | - Wrong System information combination type in function f_NBIOT_ModifySysinfoCombinationFromC1_To_C8. - Current implementation of TTCN( t_T317_min, t_T317_max, t_T318_min), makes it difficult to ensure that at Step 26A, the SystemInformationBlockType31-NB is broadcasted to the UE when UE starts reading SIBs after T318 timer expiry at the UE side, if the timer on the SS side are not aligned to the UE timers then we could result in test case failure. Add new timer of t_T317_Step24B, that takes the K-Offset into account (to align the start timer T-317 more accurately, and start after SS receive the UL Data(Step 24B) from UE, it is more accurate to sync up with timer (T317 and T318) in UE side. |  |  |  |  |  |  |  |  |
| Summary of change: |  | 1. Change system information combination form c1 to c8. 2. Added new timer of t_T317_Step24B (which takes K_Offset into account). Replaced the t_T317_min, t_T317_max with t_T317_Step24B. Replaced the t_T318_min with normal t_T318. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Conformant UE may failed the test case. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 22.4.30 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Table of Contents

1	Overview	3

2	Corrections required:	3

2.1	Correction to function f_NBIOT_ModifySysinfoCombinationFromC1_To_C8	3

2.2	Correction to function f_TC_22_4_30_NBIOT	4

# Overview

This document lists all the essential changes needed to correct problems in the iwd-TTCN3-B2022-09_D24wk12 related to the title of this CR.

Contact: 	Juan Garcia Martin

Juan.GarciaMartin@rohde-schwarz.com

# Corrections required:

## Correction to function f_NBIOT_ModifySysinfoCombinationFromC1_To_C8

| Function name | f_NBIOT_ModifySysinfoCombinationFromC1_To_C8 |
| --- | --- |
| Reason for change | Wrong System information combination type in function f_NBIOT_ModifySysinfoCombinationFromC1_To_C8. |
| Summary of change | Changed system information combination form c1 to c8. |
| TTCN module | NBIOT_CellInfo |
| MCC 160 Comments | Accepted |

Before Change:

function f_NBIOT_ModifySysinfoCombinationFromC1_To_C8(NBIOT_CellId_Type p_CellId, template (omit) SystemInformationBlockType31_NB_r17.servingSatelliteInfo_r17.ul_SyncValidityDuration_r17 p_UL_SyncValidityDuration_r17 := omit, boolean p_DoAttachWithoutPDN := true, //AllowAttachWithoutPDN = CONTROL_PLANE integer p_FrequencyBand := px_ePrimaryFrequencyBand, EUTRA_NAS_NumOfPLMN_Type p_CellsOnPLMN := LTE_AllCellsOnSamePLMN) runs on NBIOT_PTC { …… //Update scheduling info list v_SIB1.message_.c1.systemInformationBlockType1_r13.schedulingInfoList_r13 := v_SchedulingInfoList; v_SIB1.message_.c1.systemInformationBlockType1_r13.nonCriticalExtension := cs_SystemInformationBlockType1_NB_v1350_NTN_r17(v_TAC_CellId); v_CellBcchInfo := cs_BCCHInfo_NB_C1(v_MIB, v_SIB1, v_SI_List); f_NBIOT_CellInfo_SetBCCH_Info(p_CellId, v_CellBcchInfo); f_NBIOT_CellInfo_SetCombination(p_CellId, c1); }

After Change:

function f_NBIOT_ModifySysinfoCombinationFromC1_To_C8(NBIOT_CellId_Type p_CellId, template (omit) SystemInformationBlockType31_NB_r17.servingSatelliteInfo_r17.ul_SyncValidityDuration_r17 p_UL_SyncValidityDuration_r17 := omit, boolean p_DoAttachWithoutPDN := true, //AllowAttachWithoutPDN = CONTROL_PLANE integer p_FrequencyBand := px_ePrimaryFrequencyBand, EUTRA_NAS_NumOfPLMN_Type p_CellsOnPLMN := LTE_AllCellsOnSamePLMN) runs on NBIOT_PTC { //Update scheduling info list v_SIB1.message_.c1.systemInformationBlockType1_r13.schedulingInfoList_r13 := v_SchedulingInfoList; v_SIB1.message_.c1.systemInformationBlockType1_r13.nonCriticalExtension := cs_SystemInformationBlockType1_NB_v1350_NTN_r17(v_TAC_CellId); v_CellBcchInfo := cs_BCCHInfo_NB_C1(v_MIB, v_SIB1, v_SI_List); f_NBIOT_CellInfo_SetBCCH_Info(p_CellId, v_CellBcchInfo); f_NBIOT_CellInfo_SetCombination(p_CellId, c8); }

## Correction to function f_TC_22_4_30_NBIOT

| Function name | f_TC_22_4_30_NBIOT |
| --- | --- |
| Reason for change | Current implementation of TTCN( t_T317_min, t_T317_max, t_T318_min), makes it difficult to ensure that at Step 26A, the SystemInformationBlockType31-NB is broadcasted to the UE when UE starts reading SIBs after T318 timer expiry at the UE side, if the timer on the SS side are not aligned to the UE timers then we could result in test case failure. Add new timer of t_T317_Step24B, that takes the K-Offset into account (to align the start timer T-317 more accurately, and start after SS receive the UL Data(Step 24B) from UE, it is more accurate to sync up with timer (T317 and T318) in UE side. |
| Summary of change | Added new timer of t_T317_Step24B (which takes K_Offset into account). Replaced the t_T317_min, t_T317_max with t_T317_Step24B. Replaced the t_T318_min with normal t_T318. |
| TTCN module | NBIOT_RRC_NTN |
| MCC160 comments | As the timers were introduced in a submission CR R5s230708, it is proposed to keep them and to solve the problem alternatively, see the implementation below |

Before Change:

function f_TC_22_4_30_NBIOT() runs on NBIOT_PTC {// NB-IoT / NTN / Ephemeris information update / T317 Expiry / T318 Expiry @sic R5-235429, R5s230708 Ch2.1 sic@ var NAS_MSG_Indication_Type v_NAS_Ind; var O10_Type v_UserData; var SystemInformationBlockType2_NB_r13 v_SIB2; var DRB_IdentityList_Type v_DrbIdList; var integer v_SelectedPlmnIndex; timer t_T317 := 60.0; //Corresponds to the value (60sec) of ul_SyncValidityDuration_r17 in SIB31-NB acc. Table 22.4.30.3.3-2 timer t_T318 := 8.0; //Corresponds to the value (8000ms) of t318-r17 acc 36.508 Table 4.6.3-14 timer t_T317_min := f_NBIOT_SetTimerToleranceMin(rrcTimer,60.0); timer t_T317_max := f_NBIOT_SetTimerToleranceMax(rrcTimer,60.0); timer t_T318_min := f_NBIOT_SetTimerToleranceMin(rrcTimer,8.0); timer t_T301 := 2.5; timer t_T5 := 30.0; //System information combination 8 as defined in TS 36.508 [18] clause 8.1.4.3.1.1 is used. f_NBIOT_Init(c8, CONTROL_PLANE); //Table 22.4.30.3.3-2, Value of T317=60s f_NBIOT_CellInfo_ModifySIB31_UL_SyncValidityDuration_r17(nbiot_Cell1,s60); //Table 22.4.30.3.3-4 @sic R5s230708 Ch2.1.2 R5-237383 sic@ v_SIB2 := f_NBIOT_CellInfo_GetSIB2(nbiot_Cell1); v_SIB2.ue_TimersAndConstants_r13.t311_r13 := ms10000; v_SIB2.ue_TimersAndConstants_r13.t301_r13 := ms2500; v_SIB2.cp_Reestablishment_r14 := true_; f_NBIOT_CellInfo_SetSIB2 (nbiot_Cell1, v_SIB2); //Create and configure all cells f_NBIOT_CellConfig_Def(nbiot_Cell1, CONTROL_PLANE); f_UT_SwitchOnUE(UT, false); //The UE is in Automatic PLMN selection mode f_UT_AutomaticPLMN_Select_WithSwitchOnOff (UT); //The preconfigured UE location is defined in TS 36.508 [18] Clause 4.13. f_UT_ConfigureNTNPosition(UT); //Preamble: The UE is in State 1-NB switched off according to TS 36.508 [18]. f_UT_PowerOffUE (UT); f_NBIOT_SetCellPower(nbiot_Cell1, tsc_ServingCellRS_EPRE); f_NBIOT_TestBody_Set(true); //@siclog "Step 1" siclog@ //Power on UE. f_UT_SwitchOnUE(UT, false); //@siclog "Step 2" siclog@ //UE acquires SystemInformationBlockType31-NB and start T317 = 60 seconds. //@siclog "Steps 3-18b1" siclog@ //Steps 1 to 16b1 of the Attach, connected mode and test mode activated procedure described in TS 36.508 [18] subclause 8.1.5.2A.3 with TEST LOOP MODE A are performed on Ncell 1. // Acc to 36.508 Table 8.1.5.2A.3-1: NB-IoT UE Attach, connected mode and test mode activated procedure (state 2-NB to state 2A-NB) //Note: After receiving the message in step 4, SS start a timer T1 with value = T317. v_NAS_Ind := f_NBIOT_InitialRegistration_Step2_4(nbiot_Cell1, CONTROL_PLANE, PREAMBLE); //@sic R5230708 Ch2.1.3 sic@ t_T317.start; //T1 started f_NBIOT_InitialRegistration_Step5_14(nbiot_Cell1, CONTROL_PLANE, STATE2A_NB_TESTLOOP_ModeG, v_NAS_Ind); //@siclog "Steps 18A-18B" siclog@ //Steps 1 to 2 of the UE Test Loopback Activated procedure described in TS 36.508 [18] subclause 8.1.5.2B.3 are performed on Ncell 1. f_NBIOT_CloseUE_TestLoopModeGH (nbiot_Cell1, tsc_UE_TestLoopMode_TypeG, tsc_TestLoopModeGH_UplinkMode_NAS); //@sic R5230708 Ch2.1.4 sic@ //@siclog "Step 19" siclog@ //SS stops broadcasting SystemInformationBlockType31-NB f_NBIOT_ModifySysinfoCombinationTo_C1(nbiot_Cell1, true, false); //now only SIB2 will be broadcasted with cellBarred and DottachWithoutPDN = false for CONTROL_PLANE f_NBIOT_SS_ConfigureSysinfo(nbiot_Cell1, false); //@siclog "Step 19A" siclog@ //SS stops periodic grant allocation f_NBIOT_ULGrantTransmission(nbiot_Cell1, cs_NB_UL_GrantScheduling_Stop); //@siclog "Step 20" siclog@ //The SS transmits one IP packet embedded in an ESM DATA TRANSPORT and DLInformationTransfer-NB. (Note 1) v_UserData := f_GetN_OctetsFromPRBS(0, 10); SRB.send(cas_NB_SRB_NasPdu_REQ(nbiot_Cell1, tsc_SRB1bis, cs_TimingInfo_Now, cs_NAS_Request(tsc_SHT_IntegrityProtected_Ciphered, cs_ESM_DATA_TRANSPORT (tsc_EpsDefaultBearerId, tsc_PTI_Unassigned, cs_UserDataContainer(v_UserData))))); //@sic R5s230708 Ch.2.1.6 sic@ //@siclog "Steps 21-22" siclog@ //Void //@siclog "Step 23" siclog@ //Wait until expiry of T1. //Note: UE starts timer T318. SS start a timer T2 with value = T318. t_T317.timeout; //T1 expires t_T318.start; //T2 started //@siclog "Step 24" siclog@ //The SS starts the UL periodic grant transmission, allowing the UE to return the IP packet as received in step 20 f_NBIOT_ULGrantTransmission(nbiot_Cell1, cs_NB_UL_GrantScheduling_Start(cs_NB_ContinuousGrant)); //@sic R5s230708 Ch.2.1.7 sic@ //@siclog "Step 24A" siclog@ //SS resumes broadcasting SystemInformationBlockType31-NB. f_NBIOT_ModifySysinfoCombinationFromC1_To_C8(nbiot_Cell1, s60, false); //now SIB31-NB will be broadcasted again with T317=60s and DoAttachWithoutPDN=false for CONTROL_PLANE f_NBIOT_SS_ConfigureSysinfo(nbiot_Cell1, false); //Note: Timer T318 shall be stopped and T317 shall be started in UE side. SS starts a timer T3 with value = T317. t_T318.stop; //T2 stopped //@siclog "Step 24B" siclog@ //Check: Does the UE transmit one IP packet embedded in an ESM DATA TRANSPORT and ULInformationTransfer-NBcorresponding to grant in step 24? SRB.receive(car_NB_SRB_NasPdu_IND(nbiot_Cell1, tsc_SRB1bis, cr_NAS_Indication(tsc_SHT_IntegrityProtected_Ciphered, cr_ESM_DATA_TRANSPORT(?, cr_UserDataContainer)))); f_NBIOT_PreliminaryPass(__FILE__, __LINE__, "Step 24B"); t_T317_max.start; t_T317_min.start; //T3 started @sic R5s230708, R5-237383 sic@ //@siclog "Step 24C" siclog@ //SS stops broadcasting SystemInformationBlockType31-NB. f_NBIOT_ModifySysinfoCombinationTo_C1(nbiot_Cell1,true,false); //now only SIB2 will be broadcasted with cellBarred and DottachWithoutPDN = false for CONTROL_PLANE f_NBIOT_SS_ConfigureSysinfo(nbiot_Cell1, false); //@siclog "Step 24CA" siclog@ //SS stops periodic grant allocation f_NBIOT_ULGrantTransmission(nbiot_Cell1, cs_NB_UL_GrantScheduling_Stop); //@siclog "Step 24D" siclog@ //The SS transmits one IP packet embedded in an ESM DATA TRANSPORT and DLInformationTransfer-NB (Note 1). v_UserData := f_GetN_OctetsFromPRBS(10, 10); SRB.send(cas_NB_SRB_NasPdu_REQ(nbiot_Cell1, tsc_SRB1bis, cs_TimingInfo_Now, cs_NAS_Request(tsc_SHT_IntegrityProtected_Ciphered, cs_ESM_DATA_TRANSPORT (tsc_EpsDefaultBearerId, tsc_PTI_Unassigned, cs_UserDataContainer(v_UserData))))); //@sic R5s230708 Ch.2.1.6 sic@ //@siclog "Step 24E" siclog@ //Wait until expiry of T3. //Note: UE starts timer T318. SS starts a timer T4 with value = T318 t_T317_min.timeout; //T3 expired @sic R5s230708, R5-237383 sic@ t_T318_min.start; //T4 started t_T317_max.timeout; //@siclog "Step 24F" siclog@ //SS starts periodic UL grant transmission, allowing the UE to return the IP packet as received in step 24D. f_NBIOT_ULGrantTransmission(nbiot_Cell1, cs_NB_UL_GrantScheduling_Start(cs_NB_ContinuousGrant)); //@sic R5s230708 Ch.2.1.7 sic@ //@siclog "Step 25" siclog@ //Check: Does the UE transmit one IP packet embedded in an ESM DATA TRANSPORT and ULInformationTransfer-NB corresponding to grant in step 24F? alt{ [] SRB.receive(car_NB_SRB_NasPdu_IND(nbiot_Cell1, tsc_SRB1bis, cr_NAS_Indication(tsc_SHT_IntegrityProtected_Ciphered, cr_ESM_DATA_TRANSPORT(?, cr_UserDataContainer)))) { f_NBIOT_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 25"); } [] t_T318_min.timeout //T4 expires @sic R5s230708, R5-237383 sic@ { //@siclog "Step 26" siclog@ //Wait until expiry of T4. } } //@siclog "Step 26A" siclog@ //SS resumes broadcasting SystemInformationBlockType31-NB. f_NBIOT_ModifySysinfoCombinationFromC1_To_C8(nbiot_Cell1, s60, false); //now SIB31-NB will be broadcasted again with T317=60s and DoAttachWithoutPDN=false for CONTROL_PLANE f_NBIOT_SS_ConfigureSysinfo(nbiot_Cell1, false); //EXCEPTION: Steps 27-30 describe behaviour that depends on whether the UE supports RRC connection re-establishment. if(pc_RRC_re_establishment_CP_CIoT) {//@sic R5s230708 Ch2.1.9 R5-237383 sic@ // Locally release v_DrbIdList := f_NBIOT_SS_ActiveDRBs(nbiot_Cell1, CONTROL_PLANE); f_NBIOT_SS_SRBs_DRBs_Release(nbiot_Cell1, CONTROL_PLANE, -, v_DrbIdList); f_NBIOT_SS_SRBs_DRBs_Config(nbiot_Cell1, CONTROL_PLANE, -, omit); f_NBIOT_SS_ConfigRachProcedureWithNoMsg4(nbiot_Cell1); //@siclog "Step 27" siclog@ //Check: Does the UE transmit an RRCConnectionReestablishmentRequest-NB message on SRB0? f_NBIOT_RRC_RRCConnectionReestablishmentRequest_r14(nbiot_Cell1); f_NBIOT_PreliminaryPass(__FILE__, __LINE__, " Step 27"); //@siclog "Step 28" siclog@ //The SS does not respond for 2.5s (T301) and the SS shall handle any re-transmitted RRCConnectionReestablishmentRequest-NB message on Ncell 2 t_T301.start; alt { [] SRB.receive(car_NB_SRB0_RrcPdu_IND(nbiot_Cell1, cr_RRCConnectionReestablishmentRequest_NB_r14(?, otherFailure))) { repeat; } [] t_T301.timeout {} } //@siclog "Step 29" siclog@ //Void //@siclog "Step 30" siclog@ //Void f_NBIOT_SS_ConfigRachProcedureMsg4(nbiot_Cell1, f_NBIOT_508_RRCConnectionSetup()); } else { // Locally release the connection as UE will go to idle mode v_DrbIdList := f_NBIOT_SS_ActiveDRBs(nbiot_Cell1, CONTROL_PLANE); f_NBIOT_SS_SRBs_DRBs_Release(nbiot_Cell1, CONTROL_PLANE, -, v_DrbIdList); f_NBIOT_SS_SRBs_DRBs_Config(nbiot_Cell1, CONTROL_PLANE, -, omit); } //@siclog "Step 31" siclog@ t_T5.start; //@sic R5s230708, R5-237383 sic@ alt {//@sic R5s230708, R5-237383 sic@ //@siclog "Step 32a1" siclog@ [] SRB.receive(car_NB_SRB0_RrcPdu_IND(nbiot_Cell1, cr_RRCConnectionRequest_NB(?, ?))) { //@siclog "Step 32a2" siclog@ t_T5.stop; // switch on UL grant assignments f_NBIOT_ULGrantTransmission(nbiot_Cell1, cs_NB_UL_GrantScheduling_Start(cs_NB_ContinuousGrant)); f_NBIOT_RRC_ConnectionSetup_Def(); // UE shall establish SRB1 acc. to 36.331 cl. 5.3.3.4 and 5.3.9 // UE enters RRC_CONNECTED state //@siclog "Step 32a3" siclog@ // receive RRC Connection Setup Complete with piggy-backed NAS message on SRB1 (DCCH): v_SelectedPlmnIndex := f_NBIOT_CellInfo_GetSelectedPlmnIndex(nbiot_Cell1); f_NBIOT_RRCConnectionSetupComplete_Def(nbiot_Cell1, cr_508_RRCConnectionSetupComplete_NB(tsc_RRC_TI_Def, v_SelectedPlmnIndex), cr_NAS_IndicationWithPiggybacking (tsc_SHT_IntegrityProtected_PartiallyCiphered, cr_CONTROL_PLANE_SERVICE_REQUEST(?), cr_ESM_DATA_TRANSPORT(?, cr_UserDataContainer))); SRB.send(cas_NB_SRB_NasPdu_REQ(nbiot_Cell1, tsc_SRB1bis, cs_TimingInfo_Now, cs_NAS_Request(tsc_SHT_IntegrityProtected_Ciphered, cs_SERVICE_ACCEPT))); f_NBIOT_UE_Detach_SwitchOffUe(nbiot_Cell1, CONTROL_PLANE, RRC_CONNECTED); } [] t_T5.timeout { //@siclog "Step 32b1" siclog@ f_NBIOT_Postamble(nbiot_Cell1, CONTROL_PLANE, N1_IDLE); } } f_NBIOT_TestBody_Set(false); } }

After Change:

function f_TC_22_4_30_NBIOT() runs on NBIOT_PTC {// NB-IoT / NTN / Ephemeris information update / T317 Expiry / T318 Expiry @sic R5-235429, R5s230708 Ch2.1 sic@ var NAS_MSG_Indication_Type v_NAS_Ind; var O10_Type v_UserData; var SystemInformationBlockType2_NB_r13 v_SIB2; var DRB_IdentityList_Type v_DrbIdList; var integer v_SelectedPlmnIndex; var integer v_NTN_delay := 0; var float v_T317_Step24B := 0.0; timer t_T317 := 60.0; //Corresponds to the value (60sec) of ul_SyncValidityDuration_r17 in SIB31-NB acc. Table 22.4.30.3.3-2 timer t_T318 := 8.0; //Corresponds to the value (8000ms) of t318-r17 acc 36.508 Table 4.6.3-14 timer t_T317_Step24B; timer t_T301 := 2.5; timer t_T5 := 30.0; //System information combination 8 as defined in TS 36.508 [18] clause 8.1.4.3.1.1 is used. f_NBIOT_Init(c8, CONTROL_PLANE); //Table 22.4.30.3.3-2, Value of T317=60s f_NBIOT_CellInfo_ModifySIB31_UL_SyncValidityDuration_r17(nbiot_Cell1,s60); //Table 22.4.30.3.3-4 @sic R5s230708 Ch2.1.2 R5-237383 sic@ v_SIB2 := f_NBIOT_CellInfo_GetSIB2(nbiot_Cell1); v_SIB2.ue_TimersAndConstants_r13.t311_r13 := ms10000; v_SIB2.ue_TimersAndConstants_r13.t301_r13 := ms2500; v_SIB2.cp_Reestablishment_r14 := true_; f_NBIOT_CellInfo_SetSIB2 (nbiot_Cell1, v_SIB2); v_NTN_delay := f_NBIOT_NTN_Get_T_NTNoffset(nbiot_Cell1); v_T317_Step24B := 60.0 - int2float(v_NTN_delay)/1000.0; //Create and configure all cells f_NBIOT_CellConfig_Def(nbiot_Cell1, CONTROL_PLANE); f_UT_SwitchOnUE(UT, false); //The UE is in Automatic PLMN selection mode f_UT_AutomaticPLMN_Select_WithSwitchOnOff (UT); //The preconfigured UE location is defined in TS 36.508 [18] Clause 4.13. f_UT_ConfigureNTNPosition(UT); //Preamble: The UE is in State 1-NB switched off according to TS 36.508 [18]. f_UT_PowerOffUE (UT); f_NBIOT_SetCellPower(nbiot_Cell1, tsc_ServingCellRS_EPRE); f_NBIOT_TestBody_Set(true); //@siclog "Step 1" siclog@ //Power on UE. f_UT_SwitchOnUE(UT, false); //@siclog "Step 2" siclog@ //UE acquires SystemInformationBlockType31-NB and start T317 = 60 seconds. //@siclog "Steps 3-18b1" siclog@ //Steps 1 to 16b1 of the Attach, connected mode and test mode activated procedure described in TS 36.508 [18] subclause 8.1.5.2A.3 with TEST LOOP MODE A are performed on Ncell 1. // Acc to 36.508 Table 8.1.5.2A.3-1: NB-IoT UE Attach, connected mode and test mode activated procedure (state 2-NB to state 2A-NB) //Note: After receiving the message in step 4, SS start a timer T1 with value = T317. v_NAS_Ind := f_NBIOT_InitialRegistration_Step2_4(nbiot_Cell1, CONTROL_PLANE, PREAMBLE); //@sic R5230708 Ch2.1.3 sic@ t_T317.start; //T1 started f_NBIOT_InitialRegistration_Step5_14(nbiot_Cell1, CONTROL_PLANE, STATE2A_NB_TESTLOOP_ModeG, v_NAS_Ind); //@siclog "Steps 18A-18B" siclog@ //Steps 1 to 2 of the UE Test Loopback Activated procedure described in TS 36.508 [18] subclause 8.1.5.2B.3 are performed on Ncell 1. f_NBIOT_CloseUE_TestLoopModeGH (nbiot_Cell1, tsc_UE_TestLoopMode_TypeG, tsc_TestLoopModeGH_UplinkMode_NAS); //@sic R5230708 Ch2.1.4 sic@ //@siclog "Step 19" siclog@ //SS stops broadcasting SystemInformationBlockType31-NB f_NBIOT_ModifySysinfoCombinationTo_C1(nbiot_Cell1, true, false); //now only SIB2 will be broadcasted with cellBarred and DottachWithoutPDN = false for CONTROL_PLANE f_NBIOT_SS_ConfigureSysinfo(nbiot_Cell1, false); //@siclog "Step 19A" siclog@ //SS stops periodic grant allocation f_NBIOT_ULGrantTransmission(nbiot_Cell1, cs_NB_UL_GrantScheduling_Stop); //@siclog "Step 20" siclog@ //The SS transmits one IP packet embedded in an ESM DATA TRANSPORT and DLInformationTransfer-NB. (Note 1) v_UserData := f_GetN_OctetsFromPRBS(0, 10); SRB.send(cas_NB_SRB_NasPdu_REQ(nbiot_Cell1, tsc_SRB1bis, cs_TimingInfo_Now, cs_NAS_Request(tsc_SHT_IntegrityProtected_Ciphered, cs_ESM_DATA_TRANSPORT (tsc_EpsDefaultBearerId, tsc_PTI_Unassigned, cs_UserDataContainer(v_UserData))))); //@sic R5s230708 Ch.2.1.6 sic@ //@siclog "Steps 21-22" siclog@ //Void //@siclog "Step 23" siclog@ //Wait until expiry of T1. //Note: UE starts timer T318. SS start a timer T2 with value = T318. t_T317.timeout; //T1 expires t_T318.start; //T2 started //@siclog "Step 24" siclog@ //The SS starts the UL periodic grant transmission, allowing the UE to return the IP packet as received in step 20 f_NBIOT_ULGrantTransmission(nbiot_Cell1, cs_NB_UL_GrantScheduling_Start(cs_NB_ContinuousGrant)); //@sic R5s230708 Ch.2.1.7 sic@ //@siclog "Step 24A" siclog@ //SS resumes broadcasting SystemInformationBlockType31-NB. f_NBIOT_ModifySysinfoCombinationFromC1_To_C8(nbiot_Cell1, s60, false); //now SIB31-NB will be broadcasted again with T317=60s and DoAttachWithoutPDN=false for CONTROL_PLANE f_NBIOT_SS_ConfigureSysinfo(nbiot_Cell1, false); //Note: Timer T318 shall be stopped and T317 shall be started in UE side. SS starts a timer T3 with value = T317. t_T318.stop; //T2 stopped //@siclog "Step 24B" siclog@ //Check: Does the UE transmit one IP packet embedded in an ESM DATA TRANSPORT and ULInformationTransfer-NBcorresponding to grant in step 24? SRB.receive(car_NB_SRB_NasPdu_IND(nbiot_Cell1, tsc_SRB1bis, cr_NAS_Indication(tsc_SHT_IntegrityProtected_Ciphered, cr_ESM_DATA_TRANSPORT(?, cr_UserDataContainer)))); f_NBIOT_PreliminaryPass(__FILE__, __LINE__, "Step 24B"); t_T317_Step24B.start(v_T317_Step24B); //@siclog "Step 24C" siclog@ //SS stops broadcasting SystemInformationBlockType31-NB. f_NBIOT_ModifySysinfoCombinationTo_C1(nbiot_Cell1,true,false); //now only SIB2 will be broadcasted with cellBarred and DottachWithoutPDN = false for CONTROL_PLANE f_NBIOT_SS_ConfigureSysinfo(nbiot_Cell1, false); //@siclog "Step 24CA" siclog@ //SS stops periodic grant allocation f_NBIOT_ULGrantTransmission(nbiot_Cell1, cs_NB_UL_GrantScheduling_Stop); //@siclog "Step 24D" siclog@ //The SS transmits one IP packet embedded in an ESM DATA TRANSPORT and DLInformationTransfer-NB (Note 1). v_UserData := f_GetN_OctetsFromPRBS(10, 10); SRB.send(cas_NB_SRB_NasPdu_REQ(nbiot_Cell1, tsc_SRB1bis, cs_TimingInfo_Now, cs_NAS_Request(tsc_SHT_IntegrityProtected_Ciphered, cs_ESM_DATA_TRANSPORT (tsc_EpsDefaultBearerId, tsc_PTI_Unassigned, cs_UserDataContainer(v_UserData))))); //@sic R5s230708 Ch.2.1.6 sic@ //@siclog "Step 24E" siclog@ //Wait until expiry of T3. //Note: UE starts timer T318. SS starts a timer T4 with value = T318 t_T317_Step24B.timeout; //T3 expired t_T318.start; //@siclog "Step 24F" siclog@ //SS starts periodic UL grant transmission, allowing the UE to return the IP packet as received in step 24D. f_NBIOT_ULGrantTransmission(nbiot_Cell1, cs_NB_UL_GrantScheduling_Start(cs_NB_ContinuousGrant)); //@sic R5s230708 Ch.2.1.7 sic@ //@siclog "Step 25" siclog@ //Check: Does the UE transmit one IP packet embedded in an ESM DATA TRANSPORT and ULInformationTransfer-NB corresponding to grant in step 24F? alt{ [] SRB.receive(car_NB_SRB_NasPdu_IND(nbiot_Cell1, tsc_SRB1bis, cr_NAS_Indication(tsc_SHT_IntegrityProtected_Ciphered, cr_ESM_DATA_TRANSPORT(?, cr_UserDataContainer)))) { f_NBIOT_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 25"); } [] t_T318.timeout //T4 expires @sic R5s230708, R5-237383 sic@ { //@siclog "Step 26" siclog@ //Wait until expiry of T4. } } //@siclog "Step 26A" siclog@ //SS resumes broadcasting SystemInformationBlockType31-NB. f_NBIOT_ModifySysinfoCombinationFromC1_To_C8(nbiot_Cell1, s60, false); //now SIB31-NB will be broadcasted again with T317=60s and DoAttachWithoutPDN=false for CONTROL_PLANE f_NBIOT_SS_ConfigureSysinfo(nbiot_Cell1, false); //EXCEPTION: Steps 27-30 describe behaviour that depends on whether the UE supports RRC connection re-establishment. if(pc_RRC_re_establishment_CP_CIoT) {//@sic R5s230708 Ch2.1.9 R5-237383 sic@ // Locally release v_DrbIdList := f_NBIOT_SS_ActiveDRBs(nbiot_Cell1, CONTROL_PLANE); f_NBIOT_SS_SRBs_DRBs_Release(nbiot_Cell1, CONTROL_PLANE, -, v_DrbIdList); f_NBIOT_SS_SRBs_DRBs_Config(nbiot_Cell1, CONTROL_PLANE, -, omit); f_NBIOT_SS_ConfigRachProcedureWithNoMsg4(nbiot_Cell1); //@siclog "Step 27" siclog@ //Check: Does the UE transmit an RRCConnectionReestablishmentRequest-NB message on SRB0? f_NBIOT_RRC_RRCConnectionReestablishmentRequest_r14(nbiot_Cell1); f_NBIOT_PreliminaryPass(__FILE__, __LINE__, " Step 27"); //@siclog "Step 28" siclog@ //The SS does not respond for 2.5s (T301) and the SS shall handle any re-transmitted RRCConnectionReestablishmentRequest-NB message on Ncell 2 t_T301.start; alt { [] SRB.receive(car_NB_SRB0_RrcPdu_IND(nbiot_Cell1, cr_RRCConnectionReestablishmentRequest_NB_r14(?, otherFailure))) { repeat; } [] t_T301.timeout {} } //@siclog "Step 29" siclog@ //Void //@siclog "Step 30" siclog@ //Void f_NBIOT_SS_ConfigRachProcedureMsg4(nbiot_Cell1, f_NBIOT_508_RRCConnectionSetup()); } else { // Locally release the connection as UE will go to idle mode v_DrbIdList := f_NBIOT_SS_ActiveDRBs(nbiot_Cell1, CONTROL_PLANE); f_NBIOT_SS_SRBs_DRBs_Release(nbiot_Cell1, CONTROL_PLANE, -, v_DrbIdList); f_NBIOT_SS_SRBs_DRBs_Config(nbiot_Cell1, CONTROL_PLANE, -, omit); } //@siclog "Step 31" siclog@ t_T5.start; //@sic R5s230708, R5-237383 sic@ alt {//@sic R5s230708, R5-237383 sic@ //@siclog "Step 32a1" siclog@ [] SRB.receive(car_NB_SRB0_RrcPdu_IND(nbiot_Cell1, cr_RRCConnectionRequest_NB(?, ?))) { //@siclog "Step 32a2" siclog@ t_T5.stop; // switch on UL grant assignments f_NBIOT_ULGrantTransmission(nbiot_Cell1, cs_NB_UL_GrantScheduling_Start(cs_NB_ContinuousGrant)); f_NBIOT_RRC_ConnectionSetup_Def(); // UE shall establish SRB1 acc. to 36.331 cl. 5.3.3.4 and 5.3.9 // UE enters RRC_CONNECTED state //@siclog "Step 32a3" siclog@ // receive RRC Connection Setup Complete with piggy-backed NAS message on SRB1 (DCCH): v_SelectedPlmnIndex := f_NBIOT_CellInfo_GetSelectedPlmnIndex(nbiot_Cell1); f_NBIOT_RRCConnectionSetupComplete_Def(nbiot_Cell1, cr_508_RRCConnectionSetupComplete_NB(tsc_RRC_TI_Def, v_SelectedPlmnIndex), cr_NAS_IndicationWithPiggybacking (tsc_SHT_IntegrityProtected_PartiallyCiphered, cr_CONTROL_PLANE_SERVICE_REQUEST(?), cr_ESM_DATA_TRANSPORT(?, cr_UserDataContainer))); SRB.send(cas_NB_SRB_NasPdu_REQ(nbiot_Cell1, tsc_SRB1bis, cs_TimingInfo_Now, cs_NAS_Request(tsc_SHT_IntegrityProtected_Ciphered, cs_SERVICE_ACCEPT))); f_NBIOT_UE_Detach_SwitchOffUe(nbiot_Cell1, CONTROL_PLANE, RRC_CONNECTED); } [] t_T5.timeout { //@siclog "Step 32b1" siclog@ f_NBIOT_Postamble(nbiot_Cell1, CONTROL_PLANE, N1_IDLE); } } f_NBIOT_TestBody_Set(false); } }

MCC160 Alternative proposal:

function f_TC_22_4_30_NBIOT() runs on NBIOT_PTC {// NB-IoT / NTN / Ephemeris information update / T317 Expiry / T318 Expiry @sic R5-235429, R5s230708 Ch2.1 sic@ var NAS_MSG_Indication_Type v_NAS_Ind; var O10_Type v_UserData; var SystemInformationBlockType2_NB_r13 v_SIB2; var DRB_IdentityList_Type v_DrbIdList; var integer v_SelectedPlmnIndex; var integer v_NTN_delay; var float v_DelayValue; var float v_T317_min; var float v_T317_max; var float v_T318_min; timer t_T317 := 60.0; //Corresponds to the value (60sec) of ul_SyncValidityDuration_r17 in SIB31-NB acc. Table 22.4.30.3.3-2 timer t_T318 := 8.0; //Corresponds to the value (8000ms) of t318-r17 acc 36.508 Table 4.6.3-14 timer t_T317_min;// := f_NBIOT_SetTimerToleranceMin(rrcTimer,60.0); timer t_T317_max;// := f_NBIOT_SetTimerToleranceMax(rrcTimer,60.0); timer t_T318_min;// := f_NBIOT_SetTimerToleranceMin(rrcTimer,8.0); timer t_T301 := 2.5; timer t_T5 := 30.0; //System information combination 8 as defined in TS 36.508 [18] clause 8.1.4.3.1.1 is used. f_NBIOT_Init(c8, CONTROL_PLANE); //Table 22.4.30.3.3-2, Value of T317=60s f_NBIOT_CellInfo_ModifySIB31_UL_SyncValidityDuration_r17(nbiot_Cell1,s60); //Table 22.4.30.3.3-4 @sic R5s230708 Ch2.1.2 R5-237383 sic@ v_SIB2 := f_NBIOT_CellInfo_GetSIB2(nbiot_Cell1); v_SIB2.ue_TimersAndConstants_r13.t311_r13 := ms10000; v_SIB2.ue_TimersAndConstants_r13.t301_r13 := ms2500; v_SIB2.cp_Reestablishment_r14 := true_; f_NBIOT_CellInfo_SetSIB2 (nbiot_Cell1, v_SIB2); v_NTN_delay := f_NBIOT_NTN_Get_T_NTNoffset(nbiot_Cell1); v_DelayValue := 60.0 - int2float(v_NTN_delay)/1000.0; v_T317_min := f_NBIOT_SetTimerToleranceMin(rrcTimer,v_DelayValue); v_T317_max := f_NBIOT_SetTimerToleranceMax(rrcTimer,v_DelayValue); v_DelayValue := 8.0 - int2float(v_NTN_delay)/1000.0; v_T318_min := f_NBIOT_SetTimerToleranceMin(rrcTimer,v_DelayValue); //Create and configure all cells f_NBIOT_CellConfig_Def(nbiot_Cell1, CONTROL_PLANE); f_UT_SwitchOnUE(UT, false); //f_UT_AutomaticPLMN_Select_WithSwitchOnOff (UT); @sic R5s240281 Prose CR sic@ //The preconfigured UE location is defined in TS 36.508 [18] Clause 4.13. f_UT_ConfigureNTNPosition(UT); //Preamble: The UE is in State 1-NB switched off according to TS 36.508 [18]. f_UT_PowerOffUE (UT); f_NBIOT_SetCellPower(nbiot_Cell1, tsc_ServingCellRS_EPRE); f_NBIOT_TestBody_Set(true); //@siclog "Step 1" siclog@ //Power on UE. f_UT_SwitchOnUE(UT, false); //@siclog "Step 2" siclog@ //UE acquires SystemInformationBlockType31-NB and start T317 = 60 seconds. //@siclog "Steps 3-18b1" siclog@ //Steps 1 to 16b1 of the Attach, connected mode and test mode activated procedure described in TS 36.508 [18] subclause 8.1.5.2A.3 with TEST LOOP MODE A are performed on Ncell 1. // Acc to 36.508 Table 8.1.5.2A.3-1: NB-IoT UE Attach, connected mode and test mode activated procedure (state 2-NB to state 2A-NB) //Note: After receiving the message in step 4, SS start a timer T1 with value = T317. v_NAS_Ind := f_NBIOT_InitialRegistration_Step2_4(nbiot_Cell1, CONTROL_PLANE, PREAMBLE); //@sic R5230708 Ch2.1.3 sic@ t_T317.start; //T1 started f_NBIOT_InitialRegistration_Step5_14(nbiot_Cell1, CONTROL_PLANE, STATE2A_NB_TESTLOOP_ModeG, v_NAS_Ind); //@siclog "Steps 18A-18B" siclog@ //Steps 1 to 2 of the UE Test Loopback Activated procedure described in TS 36.508 [18] subclause 8.1.5.2B.3 are performed on Ncell 1. f_NBIOT_CloseUE_TestLoopModeGH (nbiot_Cell1, tsc_UE_TestLoopMode_TypeG, tsc_TestLoopModeGH_UplinkMode_NAS); //@sic R5230708 Ch2.1.4 sic@ //@siclog "Step 19" siclog@ //SS stops broadcasting SystemInformationBlockType31-NB f_NBIOT_ModifySysinfoCombinationTo_C1(nbiot_Cell1, true, false); //now only SIB2 will be broadcasted with cellBarred and DottachWithoutPDN = false for CONTROL_PLANE f_NBIOT_SS_ConfigureSysinfo(nbiot_Cell1, false); //@siclog "Step 19A" siclog@ //SS stops periodic grant allocation f_NBIOT_ULGrantTransmission(nbiot_Cell1, cs_NB_UL_GrantScheduling_Stop); //@siclog "Step 20" siclog@ //The SS transmits one IP packet embedded in an ESM DATA TRANSPORT and DLInformationTransfer-NB. (Note 1) v_UserData := f_GetN_OctetsFromPRBS(0, 10); SRB.send(cas_NB_SRB_NasPdu_REQ(nbiot_Cell1, tsc_SRB1bis, cs_TimingInfo_Now, cs_NAS_Request(tsc_SHT_IntegrityProtected_Ciphered, cs_ESM_DATA_TRANSPORT (tsc_EpsDefaultBearerId, tsc_PTI_Unassigned, cs_UserDataContainer(v_UserData))))); //@sic R5s230708 Ch.2.1.6 sic@ //@siclog "Steps 21-22" siclog@ //Void //@siclog "Step 23" siclog@ //Wait until expiry of T1. //Note: UE starts timer T318. SS start a timer T2 with value = T318. t_T317.timeout; //T1 expires t_T318.start; //T2 started //@siclog "Step 24" siclog@ //The SS starts the UL periodic grant transmission, allowing the UE to return the IP packet as received in step 20 f_NBIOT_ULGrantTransmission(nbiot_Cell1, cs_NB_UL_GrantScheduling_Start(cs_NB_ContinuousGrant)); //@sic R5s230708 Ch.2.1.7 sic@ //@siclog "Step 24A" siclog@ //SS resumes broadcasting SystemInformationBlockType31-NB. f_NBIOT_ModifySysinfoCombinationFromC1_To_C8(nbiot_Cell1, s60, false); //now SIB31-NB will be broadcasted again with T317=60s and DoAttachWithoutPDN=false for CONTROL_PLANE f_NBIOT_SS_ConfigureSysinfo(nbiot_Cell1, false); //Note: Timer T318 shall be stopped and T317 shall be started in UE side. SS starts a timer T3 with value = T317. t_T318.stop; //T2 stopped //@siclog "Step 24B" siclog@ //Check: Does the UE transmit one IP packet embedded in an ESM DATA TRANSPORT and ULInformationTransfer-NBcorresponding to grant in step 24? SRB.receive(car_NB_SRB_NasPdu_IND(nbiot_Cell1, tsc_SRB1bis, cr_NAS_Indication(tsc_SHT_IntegrityProtected_Ciphered, cr_ESM_DATA_TRANSPORT(?, cr_UserDataContainer)))); f_NBIOT_PreliminaryPass(__FILE__, __LINE__, "Step 24B"); v_NTN_delay := f_NBIOT_NTN_Get_T_NTNoffset(nbiot_Cell1); t_T317_max.start(v_T317_max); t_T317_min.start(v_T317_min); //T3 started @sic R5s230708, R5-237383 sic@ //@siclog "Step 24C" siclog@ //SS stops broadcasting SystemInformationBlockType31-NB. f_NBIOT_ModifySysinfoCombinationTo_C1(nbiot_Cell1,true,false); //now only SIB2 will be broadcasted with cellBarred and DottachWithoutPDN = false for CONTROL_PLANE f_NBIOT_SS_ConfigureSysinfo(nbiot_Cell1, false); //@siclog "Step 24CA" siclog@ //SS stops periodic grant allocation f_NBIOT_ULGrantTransmission(nbiot_Cell1, cs_NB_UL_GrantScheduling_Stop); //@siclog "Step 24D" siclog@ //The SS transmits one IP packet embedded in an ESM DATA TRANSPORT and DLInformationTransfer-NB (Note 1). v_UserData := f_GetN_OctetsFromPRBS(10, 10); SRB.send(cas_NB_SRB_NasPdu_REQ(nbiot_Cell1, tsc_SRB1bis, cs_TimingInfo_Now, cs_NAS_Request(tsc_SHT_IntegrityProtected_Ciphered, cs_ESM_DATA_TRANSPORT (tsc_EpsDefaultBearerId, tsc_PTI_Unassigned, cs_UserDataContainer(v_UserData))))); //@sic R5s230708 Ch.2.1.6 sic@ //@siclog "Step 24E" siclog@ //Wait until expiry of T3. //Note: UE starts timer T318. SS starts a timer T4 with value = T318 t_T317_min.timeout; //T3 expired @sic R5s230708, R5-237383 sic@ t_T318_min.start(v_T318_min); //T4 started t_T317_max.timeout; //@siclog "Step 24F" siclog@ //SS starts periodic UL grant transmission, allowing the UE to return the IP packet as received in step 24D. f_NBIOT_ULGrantTransmission(nbiot_Cell1, cs_NB_UL_GrantScheduling_Start(cs_NB_ContinuousGrant)); //@sic R5s230708 Ch.2.1.7 sic@ //@siclog "Step 25" siclog@ //Check: Does the UE transmit one IP packet embedded in an ESM DATA TRANSPORT and ULInformationTransfer-NB corresponding to grant in step 24F? alt{ [] SRB.receive(car_NB_SRB_NasPdu_IND(nbiot_Cell1, tsc_SRB1bis, cr_NAS_Indication(tsc_SHT_IntegrityProtected_Ciphered, cr_ESM_DATA_TRANSPORT(?, cr_UserDataContainer)))) { f_NBIOT_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 25"); } [] t_T318_min.timeout //T4 expires @sic R5s230708, R5-237383 sic@ { //@siclog "Step 26" siclog@ //Wait until expiry of T4. } } //@siclog "Step 26A" siclog@ //SS resumes broadcasting SystemInformationBlockType31-NB. f_NBIOT_ModifySysinfoCombinationFromC1_To_C8(nbiot_Cell1, s60, false); //now SIB31-NB will be broadcasted again with T317=60s and DoAttachWithoutPDN=false for CONTROL_PLANE f_NBIOT_SS_ConfigureSysinfo(nbiot_Cell1, false); //EXCEPTION: Steps 27-30 describe behaviour that depends on whether the UE supports RRC connection re-establishment. if(pc_RRC_re_establishment_CP_CIoT) {//@sic R5s230708 Ch2.1.9 R5-237383 sic@ // Locally release v_DrbIdList := f_NBIOT_SS_ActiveDRBs(nbiot_Cell1, CONTROL_PLANE); f_NBIOT_SS_SRBs_DRBs_Release(nbiot_Cell1, CONTROL_PLANE, -, v_DrbIdList); f_NBIOT_SS_SRBs_DRBs_Config(nbiot_Cell1, CONTROL_PLANE, -, omit); f_NBIOT_SS_ConfigRachProcedureWithNoMsg4(nbiot_Cell1); //@siclog "Step 27" siclog@ //Check: Does the UE transmit an RRCConnectionReestablishmentRequest-NB message on SRB0? f_NBIOT_RRC_RRCConnectionReestablishmentRequest_r14(nbiot_Cell1); f_NBIOT_PreliminaryPass(__FILE__, __LINE__, " Step 27"); //@siclog "Step 28" siclog@ //The SS does not respond for 2.5s (T301) and the SS shall handle any re-transmitted RRCConnectionReestablishmentRequest-NB message on Ncell 2 t_T301.start; alt { [] SRB.receive(car_NB_SRB0_RrcPdu_IND(nbiot_Cell1, cr_RRCConnectionReestablishmentRequest_NB_r14(?, otherFailure))) { repeat; } [] t_T301.timeout {} } //@siclog "Step 29" siclog@ //Void //@siclog "Step 30" siclog@ //Void f_NBIOT_SS_ConfigRachProcedureMsg4(nbiot_Cell1, f_NBIOT_508_RRCConnectionSetup()); } else { // Locally release the connection as UE will go to idle mode v_DrbIdList := f_NBIOT_SS_ActiveDRBs(nbiot_Cell1, CONTROL_PLANE); f_NBIOT_SS_SRBs_DRBs_Release(nbiot_Cell1, CONTROL_PLANE, -, v_DrbIdList); f_NBIOT_SS_SRBs_DRBs_Config(nbiot_Cell1, CONTROL_PLANE, -, omit); } //@siclog "Step 31" siclog@ t_T5.start; //@sic R5s230708, R5-237383 sic@ alt {//@sic R5s230708, R5-237383 sic@ //@siclog "Step 32a1" siclog@ [] SRB.receive(car_NB_SRB0_RrcPdu_IND(nbiot_Cell1, cr_RRCConnectionRequest_NB(?, ?))) { //@siclog "Step 32a2" siclog@ t_T5.stop; // switch on UL grant assignments f_NBIOT_ULGrantTransmission(nbiot_Cell1, cs_NB_UL_GrantScheduling_Start(cs_NB_ContinuousGrant)); f_NBIOT_RRC_ConnectionSetup_Def(); // UE shall establish SRB1 acc. to 36.331 cl. 5.3.3.4 and 5.3.9 // UE enters RRC_CONNECTED state //@siclog "Step 32a3" siclog@ // receive RRC Connection Setup Complete with piggy-backed NAS message on SRB1 (DCCH): v_SelectedPlmnIndex := f_NBIOT_CellInfo_GetSelectedPlmnIndex(nbiot_Cell1); f_NBIOT_RRCConnectionSetupComplete_Def(nbiot_Cell1, cr_508_RRCConnectionSetupComplete_NB(tsc_RRC_TI_Def, v_SelectedPlmnIndex), cr_NAS_IndicationWithPiggybacking (tsc_SHT_IntegrityProtected_PartiallyCiphered, cr_CONTROL_PLANE_SERVICE_REQUEST(?), cr_ESM_DATA_TRANSPORT(?, cr_UserDataContainer))); SRB.send(cas_NB_SRB_NasPdu_REQ(nbiot_Cell1, tsc_SRB1bis, cs_TimingInfo_Now, cs_NAS_Request(tsc_SHT_IntegrityProtected_Ciphered, cs_SERVICE_ACCEPT))); f_NBIOT_UE_Detach_SwitchOffUe(nbiot_Cell1, CONTROL_PLANE, RRC_CONNECTED); } [] t_T5.timeout { //@siclog "Step 32b1" siclog@ f_NBIOT_Postamble(nbiot_Cell1, CONTROL_PLANE, N1_IDLE); } } f_NBIOT_TestBody_Set(false); } } }