3GPP TSG-RAN5 Meeting #2024-TTCN email	R5s240296

# Online, , 8th Dec 2023 - 31st Dec 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 3417 | rev | - | Current version: | 17.10.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | NR5GC FR1: Re-verification of Rel-17 eNS test case 9.3.1.4 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ANRITSU LTD |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI17_Test, eNS_Ph2-UEConTest |  |  |  |  |  | Date: |  |  | 2024-04-05 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | To add test case 9.3.1.4 to the 5G/NR5GC ATS for FR1 path. |  |  |  |  |  |  |  |  |
| Summary of change: |  | This document lists all changes applied to test case 9.3.1.4 required for approval. See detailed change description for further information |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test case will not be added to ATS |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.3.1.4.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS/38.523-1 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

1.1	Verification Test Summary	3

2	Corrections required	3

2.1	Change 1	3

2.2	Change 2	5

2.3	Change 3	6

2.4	Change 4	14

2.5	Change 5	16

2.6	Change 6	20

3	Branches executed	21

4	Execution Log Files	21

4.1	Samsung Shannon S5E9945ERD	21

5	References	22

# Overview

This document lists all the essential changes needed to correct problems in the TTCN implementation of test case 9.3.1.4 which is part of the 5G NR5GC test suite.

With these changes applied the test case can be demonstrated to run with one or more UEs (see section 4). Execution log files are provided as evidence.

Contact:	Kalahasti, Narendra

Narendra.Kalahasti@anritsu.com

## Verification Test Summary

Test Case: 	9.3.1.4

ATS Version:	iwd-TTCN3-B2022-09_D24wk12

System Simulator used:	Anritsu Protocol Conformance Test System ME7834NR

UE used:	Samsung Shannon S5E9945ERD

Verification Status:	PASS

# Corrections required

## Change 1

| Function name | function fl_TC_9_3_1_4_NR5GC_TestBody () |
| --- | --- |
| Reason for change | - At step 12, Registration accept is performed with allowed NSSAI 3 to fulfil TP1 of this case. And by default, TTCN perform PDU session establishment on NSSAI 1. So, to keep NSSAI 1 at rejected NSSAI list of UE, PDU session establishment at Step 14a1 – 15 should be preformed on NSSAI 3. #draft proseCR shall be raised during RAN5#103 meeting |
| Summary of change | - Added cs_S_NSSAI_WithIEI, to perform PDU Session Establishment on NSSAI 3 on f_NR5GC_RRC_Idle_Steps16_20 |
| TTCN module | Mobility_Registration_NR5GC_NR |
| MCC160 Comment | Accepted subject to prose CR approval |

Before Change

function fl_TC_9_3_1_4_NR5GC_TestBody() runs on NR5GC_PTC { var NR_CellId_Type v_NGC_NASCellA := f_NR5GC_MapNASCell(ngc_CellA, NG_AllCellsOnSamePLMN); ---skip code--- //@siclog "Step 13" siclog@ f_NR5GC_Registration_Complete(v_NGC_NASCellA); //@siclog "Step 14a1 - 15" siclog@ f_NR5GC_RRC_Idle_Steps16_20(v_NGC_NASCellA, -, -, rrcConnectionRelease); // @sic R5-237428 R5s240165 sic@ //@siclog "Step 16" siclog@ ---skip code--- }

After Change

function fl_TC_9_3_1_4_NR5GC_TestBody() runs on NR5GC_PTC { var NR_CellId_Type v_NGC_NASCellA := f_NR5GC_MapNASCell(ngc_CellA, NG_AllCellsOnSamePLMN); ---skip code--- //@siclog "Step 13" siclog@ f_NR5GC_Registration_Complete(v_NGC_NASCellA); //@siclog "Step 14a1 - 15" siclog@ f_NR5GC_RRC_Idle_Steps16_20(v_NGC_NASCellA,-,-, rrcConnectionRelease,-,-,-,-,-,-,-,-,-,cs_S_NSSAI_WithIEI('22'O, cs_S_NSSAI('01'O, '03'O, omit))); // @sic R5-237428 R5s240165 sic@ //@siclog "Step 16" siclog@ ---skip code--- }

## Change 2

| Function name | function fl_TC_9_3_1_4_NR5GC_EUTRA_TestBody |
| --- | --- |
| Reason for change | - At step33, after checking rejected NSSAI from function f_UT_Check_NSSAI_RejectCauseMMI SS will fail the test unnecessarily as there is No pass condition after click on ‘Ok’ button from MMI proxy test will fail, and clicking ‘Not Ok’ will fail the case by MTC altstep a_MTC_ReceiveFromUT_SendToPTC() “ [] Ut.receive (car_UT_COMMON_CNF) -> value v_UT_Cnf { if (v_UT_Cnf.Result == false) { // If not received a successful confirmation, stop everything FatalError (__FILE__, __LINE__, "AT/MMI Command failed"); }” |
| Summary of change | - Implemented AT command instead MMI command. |
| TTCN module | Mobility_Registration_NR5GC_EUTRA |
| MCC160 Comment | Accepted. Also used AT command for check at step 20. |

Before Change

function fl_TC_9_3_1_4_NR5GC_EUTRA_TestBody() runs on EUTRA_Multi5GS_PTC { var B3_Type v_PDNType := f_GetPdnType(); var integer v_DRBId; var EPS_BearerIdentity v_EpsBearerId; var SRB_COMMON_IND v_ReceivedAsp; var template (value) NAS_ProtocolConfigOptions_Type v_ProtocolConfigOptions := {}; var NAS_MSG_Indication_Type v_NasInd; var charstring v_RejectedNSSAI; var template (value) PartialXtdRejectedNSSAI v_PartialXtdRejectedNSSAI_1_2 := f_BuildNR5GC_PartialXtdRejectedNSSAIType0({cs_XtdRejectedS_NSSAI('0001'B, '0011'B, '01'O, omit, omit, omit), cs_XtdRejectedS_NSSAI('0001'B, '0000'B, '02'O, omit, omit, omit)}); // @sic R5-235296 sic@ var template (value) PartialXtdRejectedNSSAI v_PartialXtdRejectedNSSAI_2 := f_BuildNR5GC_PartialXtdRejectedNSSAIType0({cs_XtdRejectedS_NSSAI('0001'B, '0000'B, '02'O, omit, omit, omit)}); // @sic R5-235296 sic@ var template (value) ExtdRejectedNSSAI v_ExtdRejectedNSSAI_1_2 := f_BuildNR5GC_ExtdRejectedNSSAI({v_PartialXtdRejectedNSSAI_1_2}); var template (value) ExtdRejectedNSSAI v_ExtdRejectedNSSAI_2 := f_BuildNR5GC_ExtdRejectedNSSAI({v_PartialXtdRejectedNSSAI_2}); // @siclog "Step 16" siclog@ -----skip code--- //@siclog "Step 33" siclog@ //@sic R5-237428 sic@ v_RejectedNSSAI := f_Convert_ListOfExtdRejectedNSSAI_ForMMICommand (v_ExtdRejectedNSSAI_2); // @sic R5-235296 sic@ if (f_UT_Check_NSSAI_RejectCauseMMI(UT, v_RejectedNSSAI)) { f_EUTRA_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 33"); } }

After Change

function fl_TC_9_3_1_4_NR5GC_EUTRA_TestBody() runs on EUTRA_Multi5GS_PTC { var B3_Type v_PDNType := f_GetPdnType(); var integer v_DRBId; var EPS_BearerIdentity v_EpsBearerId; var SRB_COMMON_IND v_ReceivedAsp; var template (value) NAS_ProtocolConfigOptions_Type v_ProtocolConfigOptions := {}; var NAS_MSG_Indication_Type v_NasInd; var charstring v_RejectedNSSAI; var template (value) PartialXtdRejectedNSSAI v_PartialXtdRejectedNSSAI_1_2 := f_BuildNR5GC_PartialXtdRejectedNSSAIType0({cs_XtdRejectedS_NSSAI('0001'B, '0011'B, '01'O, omit, omit, omit), cs_XtdRejectedS_NSSAI('0001'B, '0000'B, '02'O, omit, omit, omit)}); // @sic R5-235296 sic@ var template (value) PartialXtdRejectedNSSAI v_PartialXtdRejectedNSSAI_2 := f_BuildNR5GC_PartialXtdRejectedNSSAIType0({cs_XtdRejectedS_NSSAI('0001'B, '0000'B, '02'O, omit, omit, omit)}); // @sic R5-235296 sic@ var template (value) ExtdRejectedNSSAI v_ExtdRejectedNSSAI_1_2 := f_BuildNR5GC_ExtdRejectedNSSAI({v_PartialXtdRejectedNSSAI_1_2}); var template (value) ExtdRejectedNSSAI v_ExtdRejectedNSSAI_2 := f_BuildNR5GC_ExtdRejectedNSSAI({v_PartialXtdRejectedNSSAI_2}); var charstring v_AT_Response; var NAS_PlmnId v_PLMN := f_Asn2Nas_PlmnId(cs_HPLMN_Def); // @siclog "Step 16" siclog@ ----skip code--- //@siclog "Step 33" siclog@ //@sic R5-237428 sic@ /* v_RejectedNSSAI := f_Convert_ListOfExtdRejectedNSSAI_ForMMICommand (v_ExtdRejectedNSSAI_2); // @sic R5-235296 sic@ if (f_UT_Check_NSSAI_RejectCauseMMI(UT, v_RejectedNSSAI)) { f_EUTRA_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 33"); }*/ v_AT_Response := f_UT_ReadXtdRejectedNSSAI(UT, f_ConvertPLMNtoString(v_PLMN)); // @sic R5s230702 sic@ if (f_MatchXtdRejectedNSSAI_WithATCommand({cs_XtdRejectedS_NSSAI('0001'B, '0000'B, '02'O, omit, omit, omit)}, v_AT_Response)) { f_EUTRA_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 33"); } else { f_EUTRA_PreliminaryPass(__FILE__, __LINE__, "Step 33"); } }

#### MCC160 Implementation

function fl_TC_9_3_1_4_NR5GC_EUTRA_TestBody() runs on EUTRA_Multi5GS_PTC

{

var B3_Type v_PDNType := f_GetPdnType();

var integer v_DRBId;

var EPS_BearerIdentity v_EpsBearerId;

var SRB_COMMON_IND v_ReceivedAsp;

var template (value) NAS_ProtocolConfigOptions_Type v_ProtocolConfigOptions := {};

var NAS_MSG_Indication_Type v_NasInd;

var charstring v_AT_Response;

var NAS_PlmnId v_PLMN := f_Asn2Nas_PlmnId(f_EUTRA_CellInfo_GetGutiPLMN(eutra_CellA));

// @siclog "Step 16" siclog@

f_EUTRA_NR_WaitForCoOrd_Trigger(NR);

f_EUTRA38_SetCellPower(eutra_CellA, tsc_ServingCellRS_EPRE, tsc_ServingCellRS_EPRE_FR2);

// @siclog "Step 17" siclog@

f_EUTRA38_TrackingAreaUpdateFromN1 (eutra_CellA, tsc_RRC_TI_Def, omit);     // @sic R5s240165 sic@

f_EUTRA_PreliminaryPass(__FILE__, __LINE__, "Step 17");

// @siclog "Step 18" siclog@

//void

// @sic R5-237428 sic@

//@siclog "Step 20" siclog@

// Check: Are S-NSSAI=1 and S-NSSAI=2 in the Rejected NSSAI list PLMN using AT/MMI command +C5GNSSAIRDP - FFS: This AT command doesn't support Extended Rejected NSSAI!

// @sic R5-237428 sic@

v_AT_Response := f_UT_ReadXtdRejectedNSSAI(UT, f_ConvertPLMNtoString(v_PLMN)); // @sic R5s240296 sic@

if (f_MatchXtdRejectedNSSAI_WithATCommand({cs_XtdRejectedS_NSSAI('0001'B, '0011'B, '01'O, omit, omit, omit),

cs_XtdRejectedS_NSSAI('0001'B, '0000'B, '02'O, omit, omit, omit)},

v_AT_Response)) {

f_EUTRA_PreliminaryPass(__FILE__, __LINE__, "Step 20");

} else {

f_EUTRA_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 20");

}

// @siclog "Step 23-24 void" siclog@

// @sic R5-237428 sic@

//@siclog "Step 25" siclog@

// Cause the UE to request connectivity to an additional PDN.

// Get the next EPS Bearer Id - which is DRB id + 4

v_DRBId := f_EUTRA38_GetNextDRBId(Internet_DNN);

f_UT_RequestAdditionalPDNConnectivity(UT, (v_DRBId + 4), tsc_RandomAPN);  // @sic R5s240165 sic@

// @siclog "Step 25A-25E" siclog@

// @sic R5-237428 sic@

//receive RRC Connection Setup Complete with piggy-backed Service Request on SRB1 (DCCH):

v_NasInd := f_EUTRA_RRC_ConnEst_DefWithNas(eutra_CellA,              // @sic R5s240165 sic@

tsc_RRC_TI_Def,

?,

cr_NAS_Indication(tsc_SHT_IntegrityProtected,

cr_508_SERVICE_REQUEST(f_EUTRA_SecurityKSIasme_Get())));

//RRC Security is activated and RRC Connection Reconfiguration sent to configure SRB2 and 1 AM DRB

f_EUTRA38_MultiPDN_Activate_SRB2_DRB_SendRrcMsg(eutra_CellA, tsc_RRC_TI_Def, v_NasInd.SecurityProtection.NasCount);    // @sic R5s240165 sic@

SRB.receive(car_SRB1_RrcPdu_IND(eutra_CellA, cr_508_RRCConnectionReconfigurationComplete(tsc_RRC_TI_Def)));

// @sic R5s240165 sic@

//@siclog "Step 26" siclog@

// The UE transmits a PDN CONNECTIVITY REQUEST message as specified to request an additional PDN.

SRB.receive(car_SRB2_NasPdu_IND(eutra_CellA,

cr_NAS_Indication(tsc_SHT_IntegrityProtected_Ciphered,

cr_PDNConnReqAdditionalPDN(v_PDNType, ?)))) -> value v_ReceivedAsp;

//@siclog "Step 27-29" siclog@

v_ProtocolConfigOptions := {cs_ProtocolContainer('001B'O, '02'O & f_Asn2Nas_PlmnId (f_EUTRA_CellInfo_GetGutiPLMN (eutra_CellA)))};   // @sic R5-235296 sic@

v_EpsBearerId := f_EUTRA38_MultiPDN_EstablishAdditionalPDN_Step2 (eutra_CellA, v_ReceivedAsp.Signalling.Nas[0].Pdu.Msg.pDN_CONNECTIVITY_REQUEST, -, -, -, -, v_ProtocolConfigOptions);

//@siclog "Step 28" siclog@

a_EUTRA_IdleUpdated_ConnectToAdditionalPDN_Step3(eutra_CellA);

//@siclog "Step 29" siclog@

a_EUTRA38_MultiPDN_ConnectToAdditionalPDN_Step4(eutra_CellA, v_EpsBearerId);

f_EUTRA_PreliminaryPass(__FILE__, __LINE__, "Step 29");

f_EUTRA_DelayForUserPlaneSignalling (dontWaitForIMS);  // @sic R5s240165 sic@

//@siclog "Step 30" siclog@

f_EUTRA_RRC_ConnectionRelease(eutra_CellA);

//@siclog "Step 31 void" siclog@

// @sic R5-237428 sic@

//@siclog "Step 33" siclog@

//@sic R5-237428 sic@

v_AT_Response := f_UT_ReadXtdRejectedNSSAI(UT, f_ConvertPLMNtoString(v_PLMN)); // @sic R5s240296 sic@

if (f_MatchXtdRejectedNSSAI_WithATCommand({cs_XtdRejectedS_NSSAI('0001'B, '0000'B, '02'O, omit, omit, omit)},

v_AT_Response)) {

f_EUTRA_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 33")

} else {

f_EUTRA_PreliminaryPass(__FILE__, __LINE__, "Step 33");

}

}

# Branches executed

The test case was executed on NR band n78 using NR ciphering nea2 and integrity algorithm nia2

# Execution Log Files

## Samsung Shannon S5E9945ERD

The Samsung Shannon S5E9945ERD UE passed this test case on Anritsu Protocol Conformance Test System ME7834NR. The documentation below is enclosed as evidence of the successful test case run [1]:

- Test case execution log file:

TC_9_3_1_4_LOG.html

- PICS/PIXIT parameter file:

TC_9_3_1_4_PIXIT.xml

# References

| [1] | R5s240297: NR5GC FR1: Supporting information for re-verification of Rel-17 eNS test case 9.3.1.4 |
| --- | --- |