3GPP TSG-RAN5 Meeting #2024-TTCN email	R5s240298

# Online, , 8th Dec 2023 - 31st Dec 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 3418 | rev | - | Current version: | 17.10.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction of Multi-PDU handling in f_NR5GC_508RRC_InterRAT_HO_ToNR |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies UK |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2024-04-10 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The IRAT test cases may fail in Multi PDU config as TE is sending wrong SDAP config with both PDU session 1 and 2 being mapped to DRB1 as default DRB. As a result, UE is showing validation failure for MobilityFromEutraCommand message. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added the PDU type as Internet if it is not of type IMS. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may fail the test case |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.1.6.2.3.NR5GC, 8.1.4.2.2.1.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

2	Corrections required.	3

2.1	Change 1	3

# Overview

This document lists all the changes needed to correct issues in the ATS iwd-TTCN3-B2022-09_D24wk12 related to the title of this CR.

Contact:	Mohit Kanchan
	mohit.kanchan@keysight.com

# Corrections required.

## Change 1

| Function name | function f_NR5GC_508RRC_InterRAT_HO_ToNR() |
| --- | --- |
| Reason for change | The IRAT test cases may fail in Multi PDU config as TE is sending wrong SDAP config with both PDU session 1 and 2 being mapped to DRB1 as default DRB. As a result, UE is showing validation failure for MobilityFromEutraCommand message. |
| Summary of change | Added the PDU type as Internet if it is not of type IMS. |
| TTCN module | NR5GC_IRAT_CommonFunctions.ttcn |
| MCC160 Comment | Accepted in principle. The PDU type should not be hardcoded, but updated for each PDU session. See below. Same change made in f_NR5GC_IRAT_S1ToN1_SessionModification |

Before Change:

function f_NR5GC_508RRC_InterRAT_HO_ToNR(NR_CellId_Type p_TargetCellId, EUTRA_NR_PduSessionInfoList_Type p_PduSessionInfoList, EUTRA_NR_CoOrd_Security_Type p_EUTRASecurityParams, RNTI_Value p_RNTI_Value := tsc_C_RNTI_Value3, template (omit) ReconfigurationWithSync.rach_ConfigDedicated p_RACH_ConfigDedicated := omit, template (present) UL_DCCH_Message p_RRCConnReconfigComplete := cr_38508_RRCReconfigurationComplete, template (omit) boolean p_RACHCnfFlag := omit ) runs on NR5GC_PTC { //@sic R5s221238: added p_RACHCnfFlag sic@ var NR_PhysicalParameters_Type v_NR_PhysicalParameters := f_NR_CellInfo_GetPhysicalParameters(p_TargetCellId); var template (value) NR_RachProcedureConfig_Type v_RachProcedureConfig := cs_NR_RachProcedureConfig_CRNTI_HO(v_NR_PhysicalParameters, p_RNTI_Value); //CBRA var template (value) RRCReconfiguration v_RRCReconfiguration; var template (value) PDUSessionInfoList_Type v_SessionInfoList; var integer v_NoOfPduSessions := lengthof(p_PduSessionInfoList); /***************Skipping the code *********************/ v_NASContainer.mac := v_CalculatedMac; v_NASContainerString := encvalue(v_NASContainer); v_MasterKeyUpdate := cs_MasterKeyUpdate(true, v_NCC, bit2oct(v_NASContainerString)); for (i := 0; i < v_NoOfPduSessions; i := i + 1) { if (p_PduSessionInfoList[i].IsIMS) { // @sic R5-206296 sic@ v_PDU_Type := IMS_DNN; } v_SessionInfoList[i] := cs_GSM_MobilityInfo(int2oct(p_PduSessionInfoList[i].PDU_SessionId, 1), -, v_PDU_Type); f_NR5GC_MobileInfo_AddPDUSession(valueof(v_SessionInfoList[i])); }/***************Skipping the code *********************/ }

After Change:

function f_NR5GC_508RRC_InterRAT_HO_ToNR(NR_CellId_Type p_TargetCellId, EUTRA_NR_PduSessionInfoList_Type p_PduSessionInfoList, EUTRA_NR_CoOrd_Security_Type p_EUTRASecurityParams, RNTI_Value p_RNTI_Value := tsc_C_RNTI_Value3, template (omit) ReconfigurationWithSync.rach_ConfigDedicated p_RACH_ConfigDedicated := omit, template (present) UL_DCCH_Message p_RRCConnReconfigComplete := cr_38508_RRCReconfigurationComplete, template (omit) boolean p_RACHCnfFlag := omit ) runs on NR5GC_PTC { //@sic R5s221238: added p_RACHCnfFlag sic@ var NR_PhysicalParameters_Type v_NR_PhysicalParameters := f_NR_CellInfo_GetPhysicalParameters(p_TargetCellId); var template (value) NR_RachProcedureConfig_Type v_RachProcedureConfig := cs_NR_RachProcedureConfig_CRNTI_HO(v_NR_PhysicalParameters, p_RNTI_Value); //CBRA var template (value) RRCReconfiguration v_RRCReconfiguration; var template (value) PDUSessionInfoList_Type v_SessionInfoList; var integer v_NoOfPduSessions := lengthof(p_PduSessionInfoList); /***************Skipping the code *********************/ v_NASContainer.mac := v_CalculatedMac; v_NASContainerString := encvalue(v_NASContainer); v_MasterKeyUpdate := cs_MasterKeyUpdate(true, v_NCC, bit2oct(v_NASContainerString)); for (i := 0; i < v_NoOfPduSessions; i := i + 1) { if (p_PduSessionInfoList[i].IsIMS) { // @sic R5-206296 sic@ v_PDU_Type := IMS_DNN; } else{ v_PDU_Type := Internet_DNN; } v_SessionInfoList[i] := cs_GSM_MobilityInfo(int2oct(p_PduSessionInfoList[i].PDU_SessionId, 1), -, v_PDU_Type); f_NR5GC_MobileInfo_AddPDUSession(valueof(v_SessionInfoList[i])); }/***************Skipping the code *********************/ }

#### MCC160 Implementation

...

v_MasterKeyUpdate := cs_MasterKeyUpdate(true, v_NCC, bit2oct(v_NASContainerString));

for (i := 0; i < v_NoOfPduSessions; i := i + 1) {

v_SessionInfoList[i] := cs_GSM_MobilityInfo(int2oct(p_PduSessionInfoList[i].PDU_SessionId, 1), -, f_GetMultiPdnTypeFromIndex(p_PduSessionInfoList[i].PdnIndex));

f_NR5GC_MobileInfo_AddPDUSession(valueof(v_SessionInfoList[i]));

}

f_BuildRRCReconfigParamsNewDRBs (v_RadioBearerConfig,

v_CellGroupConfig,

v_RadioBearerList,

v_SDAP_ConfigurationList,

p_TargetCellId,

valueof(v_SessionInfoList),

NONE);

...