3GPP TSG-RAN5 Meeting #2024-TTCN email	R5s240301

# Online, , 8th Dec 2023 - 31st Dec 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 37.571-4 | CR | 0202 | rev | - | Current version: | 17.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction of GNSS 15s test cases for BDS |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies UK Ltd |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_pos_enh-UEConTest |  |  |  |  |  | Date: |  |  | 2024-04-15 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | - Based on CR R5-241577, GNSS-AuxiliaryInformation should also be provided in case of BDS also. Currently passing ‘omit’ in the template cs_GenericAssistDataReqElement_BDS for GNSS-AuxiliaryInformation while requesting GNSS-GenericAssistData depending on supported GNSSs. - Required code missing to handle and building IE GNSS_ID_BDS_r16 for all satellites in the list. Please refer Prose CR R5-241577 - Currently the function fl_Build_cs_GNSS_AuxiliaryInformation does not provide support to build IE GNSS_AuxiliaryInformation for all Satellites in the list in case of BDS |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Removed ‘omit’ for GNSS-AuxiliaryInformation from the template cs_GenericAssistDataReqElement_BDS as it is needed. - Added a function and template to set IE GNSS_ID_BDS_r16 for all satellites in the list in case of BDS. - Added support in the function fl_Build_cs_GNSS_AuxiliaryInformation to build IE GNSS_AuxiliaryInformation for BDS as well. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may fail these test cases |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.3.4.1, 9.3.4.2, 9.3.4.3, 9.3.4.4, 9.3.1.2, 7.3.4.1, 7.3.4.2, 7.3.4.3, 7.3.4.4, 7.3.5.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	3

1	Overview	4

2	Corrections required.	4

2.1	Change 1	4

2.2	Change 2	7

# Overview

This document lists all the changes needed to correct issues in the ATS iwd-TTCN3-B2022-09_D24wk12 related to the title of this CR.

Contact:	Mohit Kanchan
	mohit.kanchan@keysight.com

# Corrections required.

## Change 1

| Function name | LPP_Templates.ttcn |
| --- | --- |
| Reason for change | - Based on CR R5-241577, GNSS-AuxiliaryInformation should also be provided in case of BDS also. Currently passing ‘omit’ in the template cs_GenericAssistDataReqElement_BDS for GNSS-AuxiliaryInformation while requesting GNSS-GenericAssistData depending on supported GNSSs. - Required code missing to handle and building IE GNSS_ID_BDS_r16 for all satellites in the list. Please refer Prose CR R5-241577 - Currently the function fl_Build_cs_GNSS_AuxiliaryInformation does not provide support to build IE GNSS_AuxiliaryInformation for all Satellites in the list in case of BDS |
| Summary of change | - Removed ‘omit’ for GNSS-AuxiliaryInformation from the template cs_GenericAssistDataReqElement_BDS as it is needed. - Added a function and template to set IE GNSS_ID_BDS_r16 for all satellites in the list in case of BDS. - Added support in the function fl_Build_cs_GNSS_AuxiliaryInformation to build IE GNSS_AuxiliaryInformation for BDS as well. |
| TTCN module | LPP_Templates.ttcn |
| MCC160 Comment | Accepted. |

Before change

<<SKIPPED CODE>>

template (value) GNSS_GenericAssistDataReqElement cs_GenericAssistDataReqElement_BDS :=

/* @status    APPROVED (POS) */

cs_GNSS_GenericAssistDataReqElement(cs_GNSS_ID_Bds,

cs_GNSS_TimeModelListReq(cs_GNSS_TimeModelElementReq(1, true)), // GPS

cs_GNSS_NavigationModelReq_ReqNavList(tsc_SvID_1_2_7_18_21_27, false),

cs_GNSS_AcquisitionAssistanceReq(0),

cs_GNSS_AlmanacReq,

-,

omit);

<<SKIPPED CODE>>

// templates for GNSS-AuxiliaryInformation:

template (value) GNSS_ID_GPS_SatElement cs_GNSS_ID_GPS_SatElement(integer p_SatelliteID,

GNSS_ID_GPS_SatElement p_ProvidedSatElement) :=

{ /* @status    APPROVED (POS) */

svID := cs_SV_ID(p_SatelliteID),

signalsAvailable := p_ProvidedSatElement.signalsAvailable

};

/*

* @desc      Builds IE GNSS_ID_GPS for all satellites in the list

* @param     p_SatelliteIdList

* @param     p_ProvidedGNSS_ID_GPS

* @return    template (value) GNSS_ID_GPS

* @status    APPROVED (POS)

*/

function fl_Build_cs_GNSS_ID_GPS(IntegerList_Type p_SatelliteIdList,

GNSS_ID_GPS p_ProvidedGNSS_ID_GPS) return template (value) GNSS_ID_GPS

{

var template (value) GNSS_ID_GPS v_GNSS_ID_GPS;

var integer i;

for (i := 0; i < 6; i:= i + 1) {

v_GNSS_ID_GPS[i] := cs_GNSS_ID_GPS_SatElement(p_SatelliteIdList[i], p_ProvidedGNSS_ID_GPS[i]);

}

return v_GNSS_ID_GPS;

}

template (value) GNSS_ID_GLONASS_SatElement cs_GNSS_ID_GLONASS_SatElement(integer p_SatelliteID,

GNSS_ID_GLONASS_SatElement p_ProvidedSatElement) :=

{ /* @status    APPROVED (POS) */

svID := cs_SV_ID(p_SatelliteID),

signalsAvailable := p_ProvidedSatElement.signalsAvailable,

channelNumber := p_ProvidedSatElement.channelNumber

};

/*

* @desc      Builds IE GNSS_ID_GLONASS for all satellites in the list

* @param     p_SatelliteIdList

* @param     p_ProvidedGNSS_ID_GLONASS

* @return    template (value) GNSS_ID_GLONASS

* @status    APPROVED (POS)

*/

function fl_Build_cs_GNSS_ID_GLONASS(IntegerList_Type p_SatelliteIdList,

GNSS_ID_GLONASS p_ProvidedGNSS_ID_GLONASS) return template (value) GNSS_ID_GLONASS

{

var template (value) GNSS_ID_GLONASS v_GNSS_ID_GLONASS;

var integer i;

for (i := 0; i < 6; i:= i + 1) {

v_GNSS_ID_GLONASS[i] := cs_GNSS_ID_GLONASS_SatElement(p_SatelliteIdList[i], p_ProvidedGNSS_ID_GLONASS[i]);

}

return v_GNSS_ID_GLONASS;

}

/*

* @desc      Builds the IE GNSS_AuxiliaryInformation for all Satellites in the list

* @param     p_SatelliteIdList

* @param     p_ProvidedGNSS_AuxiliaryInformation

* @return    template (value) GNSS_AuxiliaryInformation

* @status    APPROVED (POS)

*/

function fl_Build_cs_GNSS_AuxiliaryInformation(IntegerList_Type p_SatelliteIdList,

GNSS_AuxiliaryInformation p_ProvidedGNSS_AuxiliaryInformation) return template (value) GNSS_AuxiliaryInformation

{

var template (value) GNSS_AuxiliaryInformation v_GNSS_AuxiliaryInformation;

if (ischosen(p_ProvidedGNSS_AuxiliaryInformation.gnss_ID_GPS)) {

v_GNSS_AuxiliaryInformation.gnss_ID_GPS := fl_Build_cs_GNSS_ID_GPS(p_SatelliteIdList, p_ProvidedGNSS_AuxiliaryInformation.gnss_ID_GPS);

}

else {

v_GNSS_AuxiliaryInformation.gnss_ID_GLONASS := fl_Build_cs_GNSS_ID_GLONASS(p_SatelliteIdList, p_ProvidedGNSS_AuxiliaryInformation.gnss_ID_GLONASS);

}

return v_GNSS_AuxiliaryInformation;

}

<<SKIPPED CODE>>

After change

<<SKIPPED CODE>>

template (value) GNSS_GenericAssistDataReqElement cs_GenericAssistDataReqElement_BDS :=

/* @status    APPROVED (POS) */

cs_GNSS_GenericAssistDataReqElement(cs_GNSS_ID_Bds,

cs_GNSS_TimeModelListReq(cs_GNSS_TimeModelElementReq(1, true)), // GPS

cs_GNSS_NavigationModelReq_ReqNavList(tsc_SvID_1_2_7_18_21_27, false),

cs_GNSS_AcquisitionAssistanceReq(0),

cs_GNSS_AlmanacReq,

-);

<<SKIPPED CODE>>

// templates for GNSS-AuxiliaryInformation:

template (value) GNSS_ID_GPS_SatElement cs_GNSS_ID_GPS_SatElement(integer p_SatelliteID,

GNSS_ID_GPS_SatElement p_ProvidedSatElement) :=

{ /* @status    APPROVED (POS) */

svID := cs_SV_ID(p_SatelliteID),

signalsAvailable := p_ProvidedSatElement.signalsAvailable

};

/*

* @desc      Builds IE GNSS_ID_GPS for all satellites in the list

* @param     p_SatelliteIdList

* @param     p_ProvidedGNSS_ID_GPS

* @return    template (value) GNSS_ID_GPS

* @status    APPROVED (POS)

*/

function fl_Build_cs_GNSS_ID_GPS(IntegerList_Type p_SatelliteIdList,

GNSS_ID_GPS p_ProvidedGNSS_ID_GPS) return template (value) GNSS_ID_GPS

{

var template (value) GNSS_ID_GPS v_GNSS_ID_GPS;

var integer i;

for (i := 0; i < 6; i:= i + 1) {

v_GNSS_ID_GPS[i] := cs_GNSS_ID_GPS_SatElement(p_SatelliteIdList[i], p_ProvidedGNSS_ID_GPS[i]);

}

return v_GNSS_ID_GPS;

}

template (value) GNSS_ID_GLONASS_SatElement cs_GNSS_ID_GLONASS_SatElement(integer p_SatelliteID,

GNSS_ID_GLONASS_SatElement p_ProvidedSatElement) :=

{ /* @status    APPROVED (POS) */

svID := cs_SV_ID(p_SatelliteID),

signalsAvailable := p_ProvidedSatElement.signalsAvailable,

channelNumber := p_ProvidedSatElement.channelNumber

};

/*

* @desc      Builds IE GNSS_ID_GLONASS for all satellites in the list

* @param     p_SatelliteIdList

* @param     p_ProvidedGNSS_ID_GLONASS

* @return    template (value) GNSS_ID_GLONASS

* @status    APPROVED (POS)

*/

function fl_Build_cs_GNSS_ID_GLONASS(IntegerList_Type p_SatelliteIdList,

GNSS_ID_GLONASS p_ProvidedGNSS_ID_GLONASS) return template (value) GNSS_ID_GLONASS

{

var template (value) GNSS_ID_GLONASS v_GNSS_ID_GLONASS;

var integer i;

for (i := 0; i < 6; i:= i + 1) {

v_GNSS_ID_GLONASS[i] := cs_GNSS_ID_GLONASS_SatElement(p_SatelliteIdList[i], p_ProvidedGNSS_ID_GLONASS[i]);

}

return v_GNSS_ID_GLONASS;

}

template (value) GNSS_ID_BDS_SatElement_r16  cs_GNSS_ID_BDS_SatElement(integer p_SatelliteID,

GNSS_ID_BDS_SatElement_r16 p_ProvidedSatElement) :=

{

svID_r16 := cs_SV_ID(p_SatelliteID),

satType_r16 := p_ProvidedSatElement.satType_r16

};

function fl_Build_cs_GNSS_ID_BDS_r16(IntegerList_Type p_SatelliteIdList,

GNSS_ID_BDS_r16 p_ProvidedGNSS_ID_BDS) return template (value) GNSS_ID_BDS_r16

{

var template (value) GNSS_ID_BDS_r16 v_GNSS_ID_BDS;

var integer i;

for (i := 0; i < 6; i:= i + 1) {

v_GNSS_ID_BDS[i] := cs_GNSS_ID_BDS_SatElement(p_SatelliteIdList[i], p_ProvidedGNSS_ID_BDS[i]);

}

return v_GNSS_ID_BDS;

}

/*

* @desc      Builds the IE GNSS_AuxiliaryInformation for all Satellites in the list

* @param     p_SatelliteIdList

* @param     p_ProvidedGNSS_AuxiliaryInformation

* @return    template (value) GNSS_AuxiliaryInformation

* @status    APPROVED (POS)

*/

function fl_Build_cs_GNSS_AuxiliaryInformation(IntegerList_Type p_SatelliteIdList,

GNSS_AuxiliaryInformation p_ProvidedGNSS_AuxiliaryInformation) return template (value) GNSS_AuxiliaryInformation

{

var template (value) GNSS_AuxiliaryInformation v_GNSS_AuxiliaryInformation;

if (ischosen(p_ProvidedGNSS_AuxiliaryInformation.gnss_ID_GPS)) {

v_GNSS_AuxiliaryInformation.gnss_ID_GPS := fl_Build_cs_GNSS_ID_GPS(p_SatelliteIdList, p_ProvidedGNSS_AuxiliaryInformation.gnss_ID_GPS);

}

else if (ischosen(p_ProvidedGNSS_AuxiliaryInformation.gnss_ID_GLONASS)){

v_GNSS_AuxiliaryInformation.gnss_ID_GLONASS := fl_Build_cs_GNSS_ID_GLONASS(p_SatelliteIdList, p_ProvidedGNSS_AuxiliaryInformation.gnss_ID_GLONASS);

}

else{

v_GNSS_AuxiliaryInformation.gnss_ID_BDS_r16 := fl_Build_cs_GNSS_ID_BDS_r16(p_SatelliteIdList, p_ProvidedGNSS_AuxiliaryInformation.gnss_ID_BDS_r16);

}

return v_GNSS_AuxiliaryInformation;

}

<<SKIPPED CODE>>

## Change 2

| Function name | f_POS_SelectTemplate_GenericAssistDataElement_BDS() |
| --- | --- |
| Reason for change | This SatelliteIdList simulated for Glonass so incorrect SatelliteIdList is using for BDS |
| Summary of change | Corrected SatelliteIdList in the function f_POS_SelectTemplate_GenericAssistDataElement_BDS |
| TTCN module | CommonPOS_Functions.ttcn |
| MCC160 Comment | Accepted. |

Before change

function f_POS_SelectTemplate_GenericAssistDataElement_BDS(GNSS_GenericAssistData p_RetrievedGenericAssistData,

boolean p_UEB) return template (value) GNSS_GenericAssistDataElement

{

var template (value) GNSS_GenericAssistDataElement v_GenericAssistDataElement;

var integer i := f_POS_FindIndexForGNSSMethod(p_RetrievedGenericAssistData, cs_GNSS_ID_Bds); //Indicates the position of the BDS data in the retrieved data

if (i == -1) {//BDS element was not found in the list

FatalError (__FILE__, __LINE__, "Invalid CellId"); // This function is only called if PICS indicate BDS support. If BDS assistance data is not there, it is a FatalError

}

else {

//Include all possible elements in the template

// @sic R5-240554 sic@

v_GenericAssistDataElement := cs_GNSS_GenericAssistDataElement(cs_GNSS_ID_Bds,

cs_GNSS_TimeModelList(tsc_GNSS_TO_ID_GPS, p_RetrievedGenericAssistData[i].gnss_TimeModels[0]),

cs_NavigationModel_Bds(p_RetrievedGenericAssistData[i].gnss_NavigationModel),

cs_GNSS_AcquisitionAssistance(cs_GNSS_SignalID(fl_DetermineBDS_SignalID_GNSS_AcquAssist()), // Beidou B1I or B1C

fl_Build_cs_GNSS_AcquisitionAssistList(tsc_IntList_38_40_42_43_59_60, //@sic R5-215711 sic@

p_RetrievedGenericAssistData[i].gnss_AcquisitionAssistance.gnss_AcquisitionAssistList),

p_RetrievedGenericAssistData[i].gnss_AcquisitionAssistance.confidence_r10),

cs_GNSS_Almanac_Bds(p_RetrievedGenericAssistData[i].gnss_Almanac),

-,

fl_Build_cs_GNSS_AuxiliaryInformation(tsc_IntList_3_4_5_10_18_19,  //@sic R5-241577 sic@

p_RetrievedGenericAssistData[i].gnss_AuxiliaryInformation));

//Now remove those elements not needed depending on the case

if (not (pc_A_GPS_L1C_A or pc_A_GPS_L1C_A_MGPS)) { //Only present if GPS is also supported

v_GenericAssistDataElement.gnss_TimeModels := omit;

}

//@sic R5-206424 sic@

if (not pc_BDS_B1C or pc_BDS_B2a or pc_BDS_B3I) { // UE does not supports multiple BDS signals -> Do not include Auxiliary Information (37.571-2, Table 5.4.1.1-1) @sic R5-230336 sic@

v_GenericAssistDataElement.gnss_AuxiliaryInformation := omit;

}

if (p_UEB) { // UE-Based case

v_GenericAssistDataElement.gnss_AcquisitionAssistance := omit;

v_GenericAssistDataElement.gnss_Almanac := omit;

}

else { //UE-Assisted case

if (pc_GNSS_AA) { // UE supports GNSS Acquisition Assistance

v_GenericAssistDataElement.gnss_NavigationModel := omit;

v_GenericAssistDataElement.gnss_Almanac := omit;

}

else { // UE does not supports GNSS Acquisition Assistance

v_GenericAssistDataElement.gnss_AcquisitionAssistance := omit;

}

}

}

return v_GenericAssistDataElement;

}

After change

function f_POS_SelectTemplate_GenericAssistDataElement_BDS(GNSS_GenericAssistData p_RetrievedGenericAssistData,

boolean p_UEB) return template (value) GNSS_GenericAssistDataElement

{

var template (value) GNSS_GenericAssistDataElement v_GenericAssistDataElement;

var integer i := f_POS_FindIndexForGNSSMethod(p_RetrievedGenericAssistData, cs_GNSS_ID_Bds); //Indicates the position of the BDS data in the retrieved data

if (i == -1) {//BDS element was not found in the list

FatalError (__FILE__, __LINE__, "Invalid CellId"); // This function is only called if PICS indicate BDS support. If BDS assistance data is not there, it is a FatalError

}

else {

//Include all possible elements in the template

// @sic R5-240554 sic@

v_GenericAssistDataElement := cs_GNSS_GenericAssistDataElement(cs_GNSS_ID_Bds,

cs_GNSS_TimeModelList(tsc_GNSS_TO_ID_GPS, p_RetrievedGenericAssistData[i].gnss_TimeModels[0]),

cs_NavigationModel_Bds(p_RetrievedGenericAssistData[i].gnss_NavigationModel),

cs_GNSS_AcquisitionAssistance(cs_GNSS_SignalID(fl_DetermineBDS_SignalID_GNSS_AcquAssist()), // Beidou B1I or B1C

fl_Build_cs_GNSS_AcquisitionAssistList(tsc_IntList_38_40_42_43_59_60, //@sic R5-215711 sic@

p_RetrievedGenericAssistData[i].gnss_AcquisitionAssistance.gnss_AcquisitionAssistList),

p_RetrievedGenericAssistData[i].gnss_AcquisitionAssistance.confidence_r10),

cs_GNSS_Almanac_Bds(p_RetrievedGenericAssistData[i].gnss_Almanac),

-,

fl_Build_cs_GNSS_AuxiliaryInformation(tsc_IntList_38_40_42_43_59_60,  //@sic R5-241577 sic@

p_RetrievedGenericAssistData[i].gnss_AuxiliaryInformation));

//Now remove those elements not needed depending on the case

if (not (pc_A_GPS_L1C_A or pc_A_GPS_L1C_A_MGPS)) { //Only present if GPS is also supported

v_GenericAssistDataElement.gnss_TimeModels := omit;

}

//@sic R5-206424 sic@

if (not pc_BDS_B1C or pc_BDS_B2a or pc_BDS_B3I) { // UE does not supports multiple BDS signals -> Do not include Auxiliary Information (37.571-2, Table 5.4.1.1-1) @sic R5-230336 sic@

v_GenericAssistDataElement.gnss_AuxiliaryInformation := omit;

}

if (p_UEB) { // UE-Based case

v_GenericAssistDataElement.gnss_AcquisitionAssistance := omit;

v_GenericAssistDataElement.gnss_Almanac := omit;

}

else { //UE-Assisted case

if (pc_GNSS_AA) { // UE supports GNSS Acquisition Assistance

v_GenericAssistDataElement.gnss_NavigationModel := omit;

v_GenericAssistDataElement.gnss_Almanac := omit;

}

else { // UE does not supports GNSS Acquisition Assistance

v_GenericAssistDataElement.gnss_AcquisitionAssistance := omit;

}

}

}

return v_GenericAssistDataElement;

}