3GPP TSG-RAN5 Meeting #2024-TTCN email	R5s240303

# Online, , 8th Dec 2023 - 31st Dec 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 3421 | rev | - | Current version: | 17.10.0 |  |
| For HELPon using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to NR5GC testcase 9.1.4.1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Starpoint |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2024-04-07 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Testcase will not stop even error occurred when using function f_AT_ResponseCheck. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add self.kill at the end of the function |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Testcase will not stop even error occurred. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.1.4.1.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	2

2	Corrections required	2

2.1	f_AT_ResponseCheck	2

# Overview

This document lists all the essential changes needed to correct problems in the iwd-TTCN3-B2022-09_D24wk12 related to the title of this CR.

Contact:	Ting Gao

gaoting@starpointcomm.com

# Corrections required

## f_AT_ResponseCheck

| Functionname | f_AT_ResponseCheck |
| --- | --- |
| Reason for change | Testcase will not stop even error occurred when using function f_AT_ResponseCheck. |
| Summary of change | Add self.kill at the end of the function |
| TTCN module | NAS_AuxiliaryDefsAndFunctions.ttcn |
| MCC160 Comment | This function is used several times in a test case to check the same and different AT responses. If the test stops after the first failure, the results of the later checks will not be known. Alternative solution implemented: returned the result of the match and stop the test case, if any fail, once all checks have been performed. |

Before Change:

function f_AT_ResponseCheck(charstring p_AT_Response, charstring p_ExpectedString, charstring p_Text := "", charstring p_Expression := "*[""]([^""]+)[""]*", integer p_Group := 0) { /* p_Expression := "*[""]([^""]+)[""]*"; TTCN-3 regular expression: anything + quote + no quote (at least one character) + quote + anything; * => the text between the most inner quotes is returned */ var charstring v_Expression := p_Expression; var charstring v_Result; var template charstring v_ExpectedString := pattern p_ExpectedString; var boolean v_MatchResult; var charstring v_CRLF := oct2char('0D'O) & oct2char('0A'O); v_Result := regexp(p_AT_Response, v_Expression, p_Group); v_MatchResult := match(v_Result, v_ExpectedString); if (v_MatchResult == true) { f_SetVerdict(pass,__FILE__, __LINE__, p_Text); } else { f_ErrorLog(__FILE__, __LINE__, v_CRLF & "Value expected: " & p_ExpectedString & v_CRLF & "Value received: " & v_Result); // if anything went wrong, show it f_SetVerdict(fail,__FILE__, __LINE__, p_Text); } }

After Change:

function f_AT_ResponseCheck(charstring p_AT_Response, charstring p_ExpectedString, charstring p_Text := "", charstring p_Expression := "*[""]([^""]+)[""]*", integer p_Group := 0) { /* p_Expression := "*[""]([^""]+)[""]*"; TTCN-3 regular expression: anything + quote + no quote (at least one character) + quote + anything; * => the text between the most inner quotes is returned */ var charstring v_Expression := p_Expression; var charstring v_Result; var template charstring v_ExpectedString := pattern p_ExpectedString; var boolean v_MatchResult; var charstring v_CRLF := oct2char('0D'O) & oct2char('0A'O); v_Result := regexp(p_AT_Response, v_Expression, p_Group); v_MatchResult := match(v_Result, v_ExpectedString); if (v_MatchResult == true) { f_SetVerdict(pass,__FILE__, __LINE__, p_Text); } else { f_ErrorLog(__FILE__, __LINE__, v_CRLF & "Value expected: " & p_ExpectedString & v_CRLF & "Value received: " & v_Result); // if anything went wrong, show it f_SetVerdict(fail,__FILE__, __LINE__, p_Text); self.kill; // => test case will be terminated immediately } }

#### MCC160 Implementation:

var boolean v_Result;

...

//@siclog "Steps 14" siclog@

v_Result := f_AT_ResponseCheck(v_AT_Response,

v_CurrentYearStrExt& "/12/31,14:[0-9]#(2):[0-9]#(2)", // TZ GMT+1, dst (1 char) ignored, check date and hour, ignore mins, secs @sic R5s200903, R5-221451, R5s220502 sic@

"Step 14 (UT and local time zone)",

v_Expression_CTZR,

2);

v_Result := v_Result and f_AT_ResponseCheck(v_AT_Response,

"1", // DST=1

"Step 14  DST",

v_Expression_CTZR,

1);

v_AT_Response := f_UT_ChkOperatorName (UT, LONG_NAME);

v_Result := v_Result and f_AT_ResponseCheck (v_AT_Response,

v_FullNameString,

"Step 14 (long network name)");

v_AT_Response := f_UT_ChkOperatorName (UT, SHORT_NAME);

v_Result := v_Result and f_AT_ResponseCheck (v_AT_Response,

v_ShortNameString,

"Step 14 (short network name)");

v_AT_Response := f_UT_ChkTimeZone (UT);

v_Result := v_Result and f_AT_ResponseCheck(v_AT_Response,

"*[0-9]#(2)/[0-9]#(2)/[0-9]#(2),[0-9]#(2):[0-9]#(2):[0-9]#(2)\+04*", // only TZ to be checked

"Step 14 (local time zone)");

f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 14");

...

function f_AT_ResponseCheck(charstring p_AT_Response,

charstring p_ExpectedString,

charstring p_Text := "",

charstring p_Expression := "*[""]([^""]+)[""]*",

integer p_Group := 0) return boolean

{ /* p_Expression := "*[""]([^""]+)[""]*";    TTCN-3 regular expression: anything + quote + no quote (at least one character) + quote + anything;

*                                          => the text between the most inner quotes is returned */

var charstring v_Expression := p_Expression;

var charstring v_Result;

var template charstring v_ExpectedString := pattern p_ExpectedString;

var boolean v_MatchResult;

var charstring v_CRLF := oct2char('0D'O) & oct2char('0A'O);

v_Result := regexp(p_AT_Response, v_Expression, p_Group);

v_MatchResult := match(v_Result, v_ExpectedString);

if (v_MatchResult == true) {

f_SetVerdict(pass,__FILE__, __LINE__, p_Text);

} else  {

f_ErrorLog(__FILE__, __LINE__, v_CRLF & "Value expected: " & p_ExpectedString & v_CRLF & "Value received: " & v_Result);  // if anything went wrong, show it

f_SetVerdict(fail,__FILE__, __LINE__, p_Text);

}

return v_MatchResult;

}