3GPP TSG-RAN5 Meeting #2024-TTCN email	R5s240305

# Online, , 8th Dec 2023 - 31st Dec 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 3422 | rev | - | Current version: | 17.10.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to NR5GC testcase 10.1.2.1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2024-04-08 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The TTCN implementation for the preamble calls the function f_NR5GC_Preamble (v_NGC_NASCellA, STATE_CONNECTED_3A, TEST_LOOPModeB_ON);, which calls Activate RB test Mode, and Close UE Test Loop Mode B. Depending on whether the first PDU session established is for IMS, or not the TTCN implementation calls : if (not (isvalue(v_DRBInfo))) { f_UT_RequestPDUSession(UT, 2, pc_APN_ID_Internet); f_NR5GC_PDUSessionEstablishment (v_NGC_NASCellA, 1); f_NR5GC_Activate_TestMode (v_NGC_NASCellA, tsc_UE_TestLoopMode_TypeB, tsc_NR_RbId_SRB2); // activate - SEE CHANGE 3 if(not pc_IP_Ping) { f_NR5GC_CloseUE_TestLoopModeB (v_NGC_NASCellA, '00'O); } } This results in the second call for Activate RB Test Mode, and Close UE Test Loop Mode B. This second call can be ignored by the UEs, because of the first call and there is no Open UE Test Loop ModeB function call before the second call. This causes testcase failure at Step 4, because the UE does not loop back the data that is received at Step 3. This needs to be corrected. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Updated the TTCN implementation to ensure that Activate RB Test Mode and Close UE Test Loop is called only once after the non-IMS PDU session is established. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Conformant UE may failed the test case. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 10.1.2.1.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	3

1	Overview	4

2	Corrections required:	4

2.1	Correction to function f_TC_10_1_2_1_NR5GC	4

# Overview

This document lists all the essential changes needed to correct problems in the iwd-TTCN3-B2022-09_D24wk12 related to the title of this CR.

Contact: 	Juan Garcia Martin

Juan.GarciaMartin@rohde-schwarz.com

# Corrections required:

## Correction to function f_TC_10_1_2_1_NR5GC

| Function name | f_TC_10_1_2_1_NR5GC |
| --- | --- |
| Reason for change | The TTCN implementation for the preamble calls the function f_NR5GC_Preamble (v_NGC_NASCellA, STATE_CONNECTED_3A, TEST_LOOPModeB_ON);, which calls Activate RB test Mode, and Close UE Test Loop Mode B. Depending on whether the first PDU session established is for IMS, or not the TTCN implementation calls : if (not (isvalue(v_DRBInfo))) { f_UT_RequestPDUSession(UT, 2, pc_APN_ID_Internet); f_NR5GC_PDUSessionEstablishment (v_NGC_NASCellA, 1); f_NR5GC_Activate_TestMode (v_NGC_NASCellA, tsc_UE_TestLoopMode_TypeB, tsc_NR_RbId_SRB2); // activate - SEE CHANGE 3 if(not pc_IP_Ping) { f_NR5GC_CloseUE_TestLoopModeB (v_NGC_NASCellA, '00'O); } } This results in the second call for Activate RB Test Mode, and Close UE Test Loop Mode B. This second call can be ignored by the UEs, because of the first call and there is no Open UE Test Loop ModeB function call before the second call. This causes testcase failure at Step 4, because the UE does not loop back the data that is received at Step 3. This needs to be corrected. |
| Summary of change | Updated the TTCN implementation to ensure that Activate RB Test Mode and Close UE Test Loop is called only once after the non-IMS PDU session is established. |
| TTCN module | GSM_NR5GC.ttcn |
| MCC160 Comment | Accepted |

Before Change:

function f_TC_10_1_2_1_NR5GC() runs on NR5GC_PTC { //Network-requested PDU session modification / Accepted / Reactivation var NR_CellId_Type v_NGC_NASCellA := f_NR5GC_MapNASCell(ngc_CellA, NG_AllCellsOnSamePLMN); var integer v_DRBId; // @sic R5s230095 sic@ var template (omit) NR_DRBInfo_Type v_DRBInfo; // @sic R5s230095 sic@ f_NR5GC_Init_NAS(NR_1); //Create and configure cells f_NR_CellConfig_Def(v_NGC_NASCellA); //Preamble //The UE is in 5GS state 3N-A f_NR5GC_Preamble (v_NGC_NASCellA, STATE_CONNECTED_3A, TEST_LOOPModeB_ON); // @sic R5-198972 sic@ // check if internet PDU session established in preamble // @sic R5s230095 sic@ v_DRBId := f_NR_GetDefaultDRB_ForFirstNonIMSPDUSession(); v_DRBInfo := f_NR_GetDRBInfo_FromId(v_DRBId); if (not (isvalue(v_DRBInfo))) { f_UT_RequestPDUSession(UT, 2, pc_APN_ID_Internet); f_NR5GC_PDUSessionEstablishment (v_NGC_NASCellA, 1); f_NR5GC_Activate_TestMode (v_NGC_NASCellA, tsc_UE_TestLoopMode_TypeB, tsc_NR_RbId_SRB2); // activate - SEE CHANGE 3 if(not pc_IP_Ping) { f_NR5GC_CloseUE_TestLoopModeB (v_NGC_NASCellA, '00'O); } } f_NR_TestBody_Set(true); // @sic R5s220796 sic@ //Main test body fl_TC_10_1_2_1_Body(); f_NR_TestBody_Set(false); // @sic R5s220796 sic@ // Finish //f_NR5GC_OpenUE_TestLoopMode_Deactivate_TestMode(v_NGC_NASCellA); // @sic R5s190944 sic@ //@sic R5s230130 sic@ f_NR_Postamble(v_NGC_NASCellA, STATE_CONNECTED_3A); }

After Change:

function f_TC_10_1_2_1_NR5GC() runs on NR5GC_PTC { //Network-requested PDU session modification / Accepted / Reactivation var NR_CellId_Type v_NGC_NASCellA := f_NR5GC_MapNASCell(ngc_CellA, NG_AllCellsOnSamePLMN); var integer v_DRBId; // @sic R5s230095 sic@ var template (omit) NR_DRBInfo_Type v_DRBInfo; // @sic R5s230095 sic@ f_NR5GC_Init_NAS(NR_1); //Create and configure cells f_NR_CellConfig_Def(v_NGC_NASCellA); //Preamble //The UE is in 5GS state 3N-A f_NR5GC_Preamble (v_NGC_NASCellA, STATE_CONNECTED_3A, TESTModeB_ON); // check if internet PDU session established in preamble // @sic R5s230095 sic@ v_DRBId := f_NR_GetDefaultDRB_ForFirstNonIMSPDUSession(); v_DRBInfo := f_NR_GetDRBInfo_FromId(v_DRBId); if (not (isvalue(v_DRBInfo))) { f_UT_RequestPDUSession(UT, 2, pc_APN_ID_Internet); f_NR5GC_PDUSessionEstablishment (v_NGC_NASCellA, 1); } if(not pc_IP_Ping) { f_NR5GC_CloseUE_TestLoopModeB (v_NGC_NASCellA, '00'O); } f_NR_TestBody_Set(true); // @sic R5s220796 sic@ //Main test body fl_TC_10_1_2_1_Body(); f_NR_TestBody_Set(false); // @sic R5s220796 sic@ // Finish //f_NR5GC_OpenUE_TestLoopMode_Deactivate_TestMode(v_NGC_NASCellA); // @sic R5s190944 sic@ //@sic R5s230130 sic@ f_NR_Postamble(v_NGC_NASCellA, STATE_CONNECTED_3A); }