3GPP TSG-RAN5 Meeting #2024-TTCN email	R5s240306

# Online, , 8th Dec 2023 - 31st Dec 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 3423 | rev | - | Current version: | 17.10.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to NR5GC testcase 6.3.2.4 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI17_Test, eCPSOR_CON-UEConTest |  |  |  |  |  | Date: |  |  | 2024-04-10 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | SOR transparent container IE’s IEI and IE length part (first 3 octets) should not be included in the Payload container IE, according to 24.501 9.11.3.39. Current implementation is encoding the IE length (2 octets) inside cs_NG_SORTransparentContainerValue. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Set IE length to omit in call to template cs_NG_SORTransparentContainerValue. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may fail the test case. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.3.2.4.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This issue was already raised and accepted in R5s240027 but seems that changes were not included in 24wk12. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Table of Contents

1	Overview	2

2	Corrections required:	2

2.1	Correction to function fl_TC_6_3_2_4_TestBody	2

- Overview

This document lists all the essential changes needed to correct problems in the iwd-TTCN3-B2022-09_D24wk12 related to the title of this CR.

Contact: 	Juan Garcia Martin

Juan.GarciaMartin@rohde-schwarz.com

- Corrections required:

  - Correction to function fl_TC_6_3_2_4_TestBody

| Function name | fl_TC_6_3_2_4_TestBody |
| --- | --- |
| Reason for change | SOR transparent container IE’s IEI and IE length part (first 3 octets) should not be included in the Payload container IE, according to 24.501 9.11.3.39. Current implementation is encoding the IE length (2 octets) inside cs_NG_SORTransparentContainerValue. |
| Summary of change | Set IE length to omit in call to template cs_NG_SORTransparentContainerValue. |
| TTCN module | SOR_CMCI_NR5GC |
| MCC160 Comment | Accepted. |

Before Change:

function fl_TC_6_3_2_4_TestBody() runs on NR5GC_PTC { << SKIPPED CODE >> timer t_Sorcm:=60.0; timer t_Wait:=49.0; //@siclog "Step 1" siclog@ //Void //@sic R5-233459 sic@ //@siclog "Steps 2-21a1" siclog@ //Void//@sic R5-233459 sic@ //@siclog "Steps 22" siclog@ //The SS transmits an DLInformationTransfer message containing steering of roaming information indicating list of preferred PLMN/access technology combination provided with acknowledgment requested from the UE for successful reception. v_Data := v_Length_PlmnID_AccessTechId & v_PlmnID & v_AccessTechId & v_SORInfo & v_Length_SOR_CMCI_contents & v_Length_SOR_CMCI_rule_contents & v_TsorcmTimer & v_CriterionType & v_CriterionValue;//@sic R5s230605 sic@ v_NAS_SecurityParams := f_NR5GC_Security_Get(); v_Mac_A17 :=f_NG_Authentication_A17(bit2oct(encvalue(v_Header)), v_NAS_SecurityParams.KAUSF, v_CounterSOR, v_Data, v_NAS_SecurityParams.KDF); v_Mac := bit2oct(v_Mac_A17); v_NAS_TxMsgContainer_SOR_CMCI := bit2oct(encvalue(cs_NG_SORTransparentContainerValue(v_Header, v_Mac, v_CounterSOR, v_Data, omit, int2oct(lengthof(v_Data) + 19, 2)))); v_PduToSend := f_Get_NG_DLNASTransport (cs_PayloadContainerType(tsc_PayloadContainerSORMsg), omit, cs_PayloadContainer(omit, v_NAS_TxMsgContainer_SOR_CMCI)); SRB.send(cas_NR_SRB_NasPdu_REQ(nr_Cell12, tsc_NR_RbId_SRB2, cs_TimingInfo_Now, cs_NG_NAS_Request(tsc_SHT_IntegrityProtected_Ciphered,v_PduToSend.Msg))); << SKIPPED CODE >> }

After Change:

function fl_TC_6_3_2_4_TestBody() runs on NR5GC_PTC { << SKIPPED CODE >> timer t_Sorcm:=60.0; timer t_Wait:=49.0; //@siclog "Step 1" siclog@ //Void //@sic R5-233459 sic@ //@siclog "Steps 2-21a1" siclog@ //Void//@sic R5-233459 sic@ //@siclog "Steps 22" siclog@ //The SS transmits an DLInformationTransfer message containing steering of roaming information indicating list of preferred PLMN/access technology combination provided with acknowledgment requested from the UE for successful reception. v_Data := v_Length_PlmnID_AccessTechId & v_PlmnID & v_AccessTechId & v_SORInfo & v_Length_SOR_CMCI_contents & v_Length_SOR_CMCI_rule_contents & v_TsorcmTimer & v_CriterionType & v_CriterionValue;//@sic R5s230605 sic@ v_NAS_SecurityParams := f_NR5GC_Security_Get(); v_Mac_A17 :=f_NG_Authentication_A17(bit2oct(encvalue(v_Header)), v_NAS_SecurityParams.KAUSF, v_CounterSOR, v_Data, v_NAS_SecurityParams.KDF); v_Mac := bit2oct(v_Mac_A17); v_NAS_TxMsgContainer_SOR_CMCI := bit2oct(encvalue(cs_NG_SORTransparentContainerValue(v_Header, v_Mac, v_CounterSOR, v_Data, omit, omit /* int2oct(lengthof(v_Data) + 19, 2)*/))); v_PduToSend := f_Get_NG_DLNASTransport (cs_PayloadContainerType(tsc_PayloadContainerSORMsg), omit, cs_PayloadContainer(omit, v_NAS_TxMsgContainer_SOR_CMCI)); SRB.send(cas_NR_SRB_NasPdu_REQ(nr_Cell12, tsc_NR_RbId_SRB2, cs_TimingInfo_Now, cs_NG_NAS_Request(tsc_SHT_IntegrityProtected_Ciphered,v_PduToSend.Msg))); << SKIPPED CODE >> }