3GPP TSG-RAN5 Meeting #2024-TTCN email	R5s240309

# Online, , 8th Dec 2023 - 31st Dec 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 3424 | rev | - | Current version: | 17.10.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | NR5GC FR1: Addition of Rel-17 eCPSOR test case 6.3.2.5 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ANRITSU LTD |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI17_Test, eCPSOR_CON-UEConTest |  |  |  |  |  | Date: |  |  | 2024-04-15 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | To add test case 6.3.2.5 to the 5G/NR5GC ATS for FR1 path. |  |  |  |  |  |  |  |  |
| Summary of change: |  | This document lists all changes applied to test case 6.3.2.5 required for approval. See detailed change description for further information |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test case will not be added to ATS |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.3.2.5.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS/TR ... CR…38.523-1 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Table of Contents

Table of Contents	2

1	Overview	3

1.1	Verification Test Summary	3

2	Corrections required	3

2.1	Change 1	3

3	Branches executed	10

4	Execution Log Files	11

4.1	Mediatek MT6815	11

5	References	11

- Overview

This document lists all the essential changes needed to correct problems in the TTCN implementation of test case 6.3.2.5 which is part of the 5G NR5GC test suite.

With these changes applied the test case can be demonstrated to run with one or more UEs (see section 4). Execution log files are provided as evidence.

Contact:	Kalahasti, Narendra

Narendra.Kalahasti@anritsu.com

  - Verification Test Summary

Test Case: 	6.3.2.5

ATS Version:	iwd-TTCN3-B2022-09_D24wk12

System Simulator used:	Anritsu Protocol Conformance Test System ME7834NR

UE used:	Mediatek MT6815

Verification Status:	PASS

- Corrections required

This CR have changes on top of R5s230554, So Changes 2 and 3 of R5s230554 is also part of this CR.

  - Change 1

| Function name | fl_TC_6_3_2_4_TestBody () |
| --- | --- |
| Reason for change | - AT step 22, Current TTCN is not implemented SOR-CMCI container correctly. As per 24.501 Figure: 9.11.3.51.2A it should be like below 8; 7; 6; 5; 4; 3; 2; 1. SOR transparent container IEI; octet 1. Length of SOR transparent container contents; octet 2 octet 3. SOR header; octet 4. SOR-MAC-IAUSF; octet 5 octet 20. CounterSOR; octet 21 octet 22. Length of PLMN ID and access technology list; octet 23*. PLMN ID and access technology list; octet 24* octet m*. 0 Spare; 0 Spare; 0 Spare; 0 Spare; 0 Spare; SSSI; SSCMI; SI; octet o. SOR-CMCI 12oct; octet (o+1)* octet p*. SOR-CMCI is as below Length of SOR-CMCI contents 2oct; octet o+1 octet o+2. SOR-CMCI rule 1 Length of SOR-CMCI rule contents 2oct; octet q+1 octet q+2. Tsor-cm timer value 1oct 21; octet q+3. Criterion type 1oct 01; octet q+4. Criterion value 6oct; octet (q+5)* octet r*...; octet (o+1)* octet p*. SOR-SNPN-SI; octet (p+1)* octet u. Figure 9.11.3.51.2A: SOR transparentcontainer information element for list type with value "1", SOR datatype with value "0", additional parameters with value "1" So updated variables and v_data variable according to figure Figure 9.11.3.51.2A. - AT step22, SOR transparent container IE’s IEI and IE length part (first 3 octets) should not be included in the Payload container IE, according to 24.501 9.11.3.39. Current implementation is encoding the IE length (2 octets) inside cs_NG_SORTransparentContainerValue. - At step26, Swicth on command is required after dereg step at step 25a1-25b1. - After switching on at step26, there is no Tsor-cm timer started for any PDU session or service. While one or more Tsor-cm timers are running, the UE shall check the newly established PDU session or service for a matching criterion in the SOR-CMCI. To ensure it, a SOR-CM should be sent after the power on registration procedure. So added 2 additional steps DL NAS Transport Message Step44AA and UL NAS Transport message Step44D. Additional information is present in prose draft. #draftProsePR would be raised during RAN5#103 meeting |
| Summary of change | - Added variables for Length of PLMN ID and access technology list, Length of SOR-CMCI contents , Length of SOR-CMCI rule contents and modified v_Data - Set IE length to omit in call to template cs_NG_SORTransparentContainerValue. - Added function f_UT_SwitchOnUE . - Add steps 44AA for (NR RRC: DLInformationTransfer 5GMM: DL NAS TRANSPORT) and modified step 44D for (NR RRC: ULInformationTransfer 5GMM:UL NAS TRANSPORT) with same message contents as steps 22 & 24. |
| TTCN module | SOR_CMCI_NR5GC |
| MCC160 Comment | - Accepted. - Accepted. - Accepted. - Postponed, the changes need to be further discussed in RAN5#103. |

Before Change

function fl_TC_6_3_2_5_TestBody() runs on NR5GC_PTC { var octetstring v_Data; var NAS_PlmnId v_PlmnID := f_NR_Asn2Nas_PlmnId(f_NR_CellInfo_GetPLMN (nr_Cell11)); var O2_Type v_AccessTechId :='0800'O; var O1_Type v_SORInfo :='03'O;//SSCMI var octetstring v_NAS_TxMsgContainer_SOR_CMCI; var O16_Type v_Mac; var B128_Type v_Mac_A17; var NG_NAS_SecurityParams_Type v_NAS_SecurityParams; var octetstring v_CriterionType:= '01'O; var octetstring v_CriterionValue:= f_DomainName_Encode(pc_APN_ID_Internet); var O1_Type v_Length :=int2oct(3+lengthof(v_CriterionValue),1); var octetstring v_TsorcmTimer:= '21'O; var O2_Type v_CounterSOR :='0001'O; var template (value) SOR_Header v_Header := cs_NG_SOR_Header('1'B,'1'B,'1'B,'0'B,'1'B);//@sic R5-233460 sic@ var template (value) NG_NAS_DL_Pdu_Type v_PduToSend; var NR_SRB_COMMON_IND v_ReceivedAsp; var NG_NAS_MSG_Indication_Type v_ReceivedMsg; timer t_Sorcm:=60.0; timer t_Tmax:=f_NR_SetTimerToleranceMax(nr_Cell12, nonProtocolTimer, 660.0); //@siclog "Step 1" siclog@ //Void //@sic R5-233460 sic@ //@siclog "Steps 2-21a1" siclog@ //Void//@sic R5-233460 sic@ //@siclog "Step 22" siclog@ //The SS transmits an DLInformationTransfer message containing steering of roaming information indicating list of preferred PLMN/access technology combination provided with acknowledgment requested from the UE for successful reception v_Data := v_PlmnID & v_AccessTechId & v_SORInfo& v_Length & v_TsorcmTimer & v_CriterionType & v_CriterionValue; v_NAS_SecurityParams := f_NR5GC_Security_Get(); v_Mac_A17 :=f_NG_Authentication_A17(bit2oct(encvalue(v_Header)), v_NAS_SecurityParams.KAUSF, v_CounterSOR, v_Data, v_NAS_SecurityParams.KDF); v_Mac := bit2oct(v_Mac_A17); v_NAS_TxMsgContainer_SOR_CMCI := bit2oct(encvalue(cs_NG_SORTransparentContainerValue(v_Header, v_Mac, v_CounterSOR, v_Data, omit, int2oct(lengthof(v_Data) + 19, 2)))); v_PduToSend := f_Get_NG_DLNASTransport (cs_PayloadContainerType(tsc_PayloadContainerSORMsg), omit, cs_PayloadContainer(omit, v_NAS_TxMsgContainer_SOR_CMCI)); SRB.send(cas_NR_SRB_NasPdu_REQ(nr_Cell12, tsc_NR_RbId_SRB2, cs_TimingInfo_Now, cs_NG_NAS_Request(tsc_SHT_IntegrityProtected_Ciphered,v_PduToSend.Msg))); //@siclog "Step 23" siclog@ //The SS starts timer of Tsor-cm timer. t_Sorcm.start; //@siclog "Step 24" siclog@ //The UE transmits an ULInformationTransfer message carrying acknowledgement of successful reception of the steering of roaming information. SRB.receive(car_NR_SRB_NasPdu_IND(nr_Cell12, tsc_NR_RbId_SRB2, cr_NG_NAS_Ind(tsc_SHT_IntegrityProtected_Ciphered))) -> value v_ReceivedAsp; // @sic R5s201387 Baseline Moving sic@ @sic R5s210026 sic@ if (not f_Check_NG_ULNASTransport(v_ReceivedAsp.Signalling.Nas[0].Pdu.Msg, cr_PayloadContainerType(tsc_PayloadContainerSORMsg), omit)) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "UL NAS Transport Message Failed"); } //@siclog "Step 25a1-25b1" siclog@ //Steps 5a1 to 5b1 of Switch off procedure described in TS 38.508-1 [4] Table 4.9.6.3-1 are performed on NR Cell 12 before Tsor-cm timer expires. f_NR_UE_DeRegisterOnSwitchOff (nr_Cell12, STATE_CONNECTED_3A); t_Sorcm.stop; //@siclog "Step 26" siclog@ //Power on the UE or intert USIM (Note 1) //switch on command is missing here //@siclog "Step 27-39" siclog@ //Steps 1 to 13 of the registration procedure described in TS 38.508-1 [4] subclause 4.5.2.2-2 are performed on NR Cell 12. //@siclog "Step 40" siclog@ //The SS transmits an DLInformationTransfer message and an REGISTRATION ACCEPT message//@sic R5-237423 sic@ //@siclog "Step 41" siclog@ //Void //@sic R5-235284 sic@ //@siclog "Step 42" siclog@ //Void //@sic R5-235284 sic@ //@siclog "Step 43" siclog@//@sic R5-237423 sic@ //The UE transmits an ULInformationTransfer message and REGISTRATION COMPLETE message. f_NR5GC_Registration(nr_Cell12); //@siclog "Step 44a1" siclog@ //Void //@sic R5-235284 sic@ //@siclog "Step 44A" siclog@ // The generic procedure for UE-requested PDU session establishment, specified in subclause 4.5A.2, takes place performing establishment of UE-requested PDU session(s) with ExpectedNumberOfNewPDUSessions = pc_noOf_PDUsSameConnection. if (pc_noOf_PDUsSameConnection > 0 ){//@sic R5-235284 sic@ f_NR5GC_PDUSessionEstablishment (nr_Cell12,-,tsc_NR_RbId_SRB1);//@sic R5s210576 sic@ } else if (pc_noOf_PDUsNewConnection >0 ){ f_NR_RRCRelease(nr_Cell12);//prose CR needed. f_NR5GC_RRC_Idle_Extension(nr_Cell12); // @sic R5-236305 sic@ } // Step 44AA introduced here //@siclog "Step 44B" siclog@ //The SS starts timer of Tsor-cm timer. t_Sorcm.start; //@siclog "Step 44C" siclog@ t_Tmax.start(f_NR_SetTimerToleranceMax(nr_Cell12, nonProtocolTimer, 660.0)); // Step 44D introduced here //@siclog "Step 45" siclog@ //Check: Does the UE transmit a DEREGISTRATION REQUEST after Tsor-cm timer expires? t_Sorcm.timeout; SRB.receive(car_NR_SRB_NasPdu_IND(nr_Cell12, tsc_NR_RbId_SRB2, cr_NG_NAS_Ind(tsc_SHT_IntegrityProtected_Ciphered))) -> value v_ReceivedAsp; v_ReceivedMsg := v_ReceivedAsp.Signalling.Nas[0]; if(not f_Check_NG_DeregistrationReqMOMsg(v_ReceivedMsg.Pdu.Msg, crs_DeregisterType('0'B, '0'B, '01'B))) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "De-registration Request Message Failed"); } else { f_NR_PreliminaryPass (__FILE__, __LINE__, "Step 45 pass"); } //@siclog "Step 46" siclog@ //The SS transmits a DEREGISTRATION ACCEPT message. SRB.send(cas_NR_SRB_NasPdu_REQ(nr_Cell12,-, -,cs_NG_NAS_Request(tsc_SHT_IntegrityProtected_Ciphered, cs_NG_DEREGISTRATION_ACCEPT))); // Need to 'reset' all stored lists as all DRBs and PDU sessions no longer exist in UE f_NR5GC_MobileInfo_Init(); f_IMS_IPCAN_SendCoOrdMsg(IMS[f_PDN_PdnIndex2Integer(PDN_1)], cms_IPCAN_IMS_Reset); //@siclog "Step 47" siclog@ //Check: Does the UE transmits an RRCSetupRequest on NR Cell 11 before tmax expires? alt { [] SRB.check(receive(car_NR_SRB0_RrcPdu_IND(nr_Cell11, cr_38508_RRCSetupRequest(cr_NR_EstablishmentCause_Any)))) { t_Tmax.stop; f_NR_PreliminaryPass (__FILE__, __LINE__, "Step 47 pass"); } [] t_Tmax.timeout { f_NR_SetVerdictFailOrInconc (__FILE__, __LINE__, "Step 47 Fail"); } } //@siclog "Step 48-49" siclog@ //Steps 3 to 4 of the registration procedure described in TS 38.508-1 subclause 4.5.2.2-2 are performed on NR Cell 11. //@siclog "Steps 51-66a1" siclog@ f_NR5GC_RRC_IdleState1N_Def(nr_Cell11, TESTMode_OFF); // @siclog "Steps 50a1 - 50a16a1" siclog@ //Void // @siclog "Steps 50b1 - 50b3a1" siclog@ //Void }//End of fl_TC_6_3_2_5_TestBody

After Change

function fl_TC_6_3_2_5_TestBody() runs on NR5GC_PTC { var octetstring v_Data; var NAS_PlmnId v_PlmnID := f_NR_Asn2Nas_PlmnId(f_NR_CellInfo_GetPLMN (nr_Cell11)); var O2_Type v_AccessTechId :='0800'O; var O1_Type v_SORInfo :='03'O;//SSCMI var octetstring v_NAS_TxMsgContainer_SOR_CMCI; var O16_Type v_Mac; var B128_Type v_Mac_A17; var NG_NAS_SecurityParams_Type v_NAS_SecurityParams; var octetstring v_CriterionType:= '01'O; var octetstring v_CriterionValue:= f_DomainName_Encode(pc_APN_ID_Internet); var O2_Type v_Length_SOR_CMCI_rule_contents :=int2oct(2+lengthof(v_CriterionValue),2); var O2_Type v_Length_SOR_CMCI_contents :=int2oct(4+lengthof(v_CriterionValue),2); var octetstring v_TsorcmTimer:= '21'O; var O2_Type v_CounterSOR :='0001'O; var template (value) SOR_Header v_Header := cs_NG_SOR_Header('1'B,'1'B,'1'B,'0'B,'1'B);//@sic R5-233460 sic@ var template (value) NG_NAS_DL_Pdu_Type v_PduToSend; var NR_SRB_COMMON_IND v_ReceivedAsp; var NG_NAS_MSG_Indication_Type v_ReceivedMsg; timer t_Sorcm:=60.0; timer t_Tmax:=f_NR_SetTimerToleranceMax(nr_Cell12, nonProtocolTimer, 660.0); var O1_Type v_Length_PlmnID_AccessTechId := int2oct(lengthof(v_PlmnID & v_AccessTechId),1); //@siclog "Step 1" siclog@ //Void //@sic R5-233460 sic@ //@siclog "Steps 2-21a1" siclog@ //Void//@sic R5-233460 sic@ //@siclog "Step 22" siclog@ //The SS transmits an DLInformationTransfer message containing steering of roaming information indicating list of preferred PLMN/access technology combination provided with acknowledgment requested from the UE for successful reception v_Data := v_Length_PlmnID_AccessTechId & v_PlmnID & v_AccessTechId & v_SORInfo & v_Length_SOR_CMCI_contents & v_Length_SOR_CMCI_rule_contents & v_TsorcmTimer & v_CriterionType & v_CriterionValue; v_NAS_SecurityParams := f_NR5GC_Security_Get(); v_Mac_A17 :=f_NG_Authentication_A17(bit2oct(encvalue(v_Header)), v_NAS_SecurityParams.KAUSF, v_CounterSOR, v_Data, v_NAS_SecurityParams.KDF); v_Mac := bit2oct(v_Mac_A17); v_NAS_TxMsgContainer_SOR_CMCI := bit2oct(encvalue(cs_NG_SORTransparentContainerValue(v_Header, v_Mac, v_CounterSOR, v_Data, omit, omit))); v_PduToSend := f_Get_NG_DLNASTransport (cs_PayloadContainerType(tsc_PayloadContainerSORMsg), omit, cs_PayloadContainer(omit, v_NAS_TxMsgContainer_SOR_CMCI)); SRB.send(cas_NR_SRB_NasPdu_REQ(nr_Cell12, tsc_NR_RbId_SRB2, cs_TimingInfo_Now, cs_NG_NAS_Request(tsc_SHT_IntegrityProtected_Ciphered,v_PduToSend.Msg))); //@siclog "Step 23" siclog@ //The SS starts timer of Tsor-cm timer. t_Sorcm.start; //@siclog "Step 24" siclog@ //The UE transmits an ULInformationTransfer message carrying acknowledgement of successful reception of the steering of roaming information. SRB.receive(car_NR_SRB_NasPdu_IND(nr_Cell12, tsc_NR_RbId_SRB2, cr_NG_NAS_Ind(tsc_SHT_IntegrityProtected_Ciphered))) -> value v_ReceivedAsp; // @sic R5s201387 Baseline Moving sic@ @sic R5s210026 sic@ if (not f_Check_NG_ULNASTransport(v_ReceivedAsp.Signalling.Nas[0].Pdu.Msg, cr_PayloadContainerType(tsc_PayloadContainerSORMsg), omit)) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "UL NAS Transport Message Failed"); } //@siclog "Step 25a1-25b1" siclog@ //Steps 5a1 to 5b1 of Switch off procedure described in TS 38.508-1 [4] Table 4.9.6.3-1 are performed on NR Cell 12 before Tsor-cm timer expires. f_NR_UE_DeRegisterOnSwitchOff (nr_Cell12, STATE_CONNECTED_3A); t_Sorcm.stop; //@siclog "Step 26" siclog@ //Power on the UE or intert USIM (Note 1) f_UT_SwitchOnUE(UT); //@siclog "Step 27-39" siclog@ //Steps 1 to 13 of the registration procedure described in TS 38.508-1 [4] subclause 4.5.2.2-2 are performed on NR Cell 12. //@siclog "Step 40" siclog@ //The SS transmits an DLInformationTransfer message and an REGISTRATION ACCEPT message//@sic R5-237423 sic@ //@siclog "Step 41" siclog@ //Void //@sic R5-235284 sic@ //@siclog "Step 42" siclog@ //Void //@sic R5-235284 sic@ //@siclog "Step 43" siclog@//@sic R5-237423 sic@ //The UE transmits an ULInformationTransfer message and REGISTRATION COMPLETE message. f_NR5GC_Registration(nr_Cell12); //@siclog "Step 44a1" siclog@ //Void //@sic R5-235284 sic@ //@siclog "Step 44A" siclog@ // The generic procedure for UE-requested PDU session establishment, specified in subclause 4.5A.2, takes place performing establishment of UE-requested PDU session(s) with ExpectedNumberOfNewPDUSessions = pc_noOf_PDUsSameConnection. if (pc_noOf_PDUsSameConnection > 0 ){//@sic R5-235284 sic@ f_NR5GC_PDUSessionEstablishment (nr_Cell12,-,tsc_NR_RbId_SRB1);//@sic R5s210576 sic@ } else if (pc_noOf_PDUsNewConnection >0 ){ f_NR_RRCRelease(nr_Cell12);//prose CR needed. f_NR5GC_RRC_Idle_Extension(nr_Cell12); // @sic R5-236305 sic@ } //@siclog "Step 44AA" siclog@ v_Data := v_PlmnID & v_AccessTechId ; v_Header:= cs_NG_SOR_Header('1'B,'1'B,'1'B,'0'B);//@sic R5-233460 sic@ v_NAS_SecurityParams := f_NR5GC_Security_Get(); v_Mac_A17 :=f_NG_Authentication_A17(bit2oct(encvalue(v_Header)), v_NAS_SecurityParams.KAUSF, v_CounterSOR, v_Data, v_NAS_SecurityParams.KDF); v_Mac := bit2oct(v_Mac_A17); v_NAS_TxMsgContainer_SOR_CMCI := bit2oct(encvalue(cs_NG_SORTransparentContainerValue(v_Header, v_Mac, v_CounterSOR, v_Data, omit, omit))); v_PduToSend := f_Get_NG_DLNASTransport (cs_PayloadContainerType(tsc_PayloadContainerSORMsg), omit, cs_PayloadContainer(omit, v_NAS_TxMsgContainer_SOR_CMCI)); SRB.send(cas_NR_SRB_NasPdu_REQ(nr_Cell12, tsc_NR_RbId_SRB2, cs_TimingInfo_Now, cs_NG_NAS_Request(tsc_SHT_IntegrityProtected_Ciphered,v_PduToSend.Msg))); //@siclog "Step 44B" siclog@ //The SS starts timer of Tsor-cm timer. t_Sorcm.start; //@siclog " Step 44C " siclog@ t_Tmax.start(f_NR_SetTimerToleranceMax(nr_Cell12, nonProtocolTimer, 660.0)); //@siclog " Step 44D " siclog@ SRB.receive(car_NR_SRB_NasPdu_IND(nr_Cell12, tsc_NR_RbId_SRB2, cr_NG_NAS_Ind(tsc_SHT_IntegrityProtected_Ciphered))) -> value v_ReceivedAsp; // @sic R5s201387 Baseline Moving sic@ @sic R5s210026 sic@ if (not f_Check_NG_ULNASTransport(v_ReceivedAsp.Signalling.Nas[0].Pdu.Msg, cr_PayloadContainerType(tsc_PayloadContainerSORMsg), omit)) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "UL NAS Transport Message Failed"); } //@siclog "Step 45" siclog@ //Check: Does the UE transmit a DEREGISTRATION REQUEST after Tsor-cm timer expires? t_Sorcm.timeout; SRB.receive(car_NR_SRB_NasPdu_IND(nr_Cell12, tsc_NR_RbId_SRB2, cr_NG_NAS_Ind(tsc_SHT_IntegrityProtected_Ciphered))) -> value v_ReceivedAsp; v_ReceivedMsg := v_ReceivedAsp.Signalling.Nas[0]; if(not f_Check_NG_DeregistrationReqMOMsg(v_ReceivedMsg.Pdu.Msg, crs_DeregisterType('0'B, '0'B, '01'B))) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "De-registration Request Message Failed"); } else { f_NR_PreliminaryPass (__FILE__, __LINE__, "Step 45 pass"); } //@siclog "Step 46" siclog@ //The SS transmits a DEREGISTRATION ACCEPT message. SRB.send(cas_NR_SRB_NasPdu_REQ(nr_Cell12,-, -,cs_NG_NAS_Request(tsc_SHT_IntegrityProtected_Ciphered, cs_NG_DEREGISTRATION_ACCEPT))); // Need to 'reset' all stored lists as all DRBs and PDU sessions no longer exist in UE f_NR5GC_MobileInfo_Init(); f_IMS_IPCAN_SendCoOrdMsg(IMS[f_PDN_PdnIndex2Integer(PDN_1)], cms_IPCAN_IMS_Reset); //@siclog "Step 47" siclog@ //Check: Does the UE transmits an RRCSetupRequest on NR Cell 11 before tmax expires? alt { [] SRB.check(receive(car_NR_SRB0_RrcPdu_IND(nr_Cell11, cr_38508_RRCSetupRequest(cr_NR_EstablishmentCause_Any)))) { t_Tmax.stop; f_NR_PreliminaryPass (__FILE__, __LINE__, "Step 47 pass"); } [] t_Tmax.timeout { f_NR_SetVerdictFailOrInconc (__FILE__, __LINE__, "Step 47 Fail"); } } //@siclog "Step 48-49" siclog@ //Steps 3 to 4 of the registration procedure described in TS 38.508-1 subclause 4.5.2.2-2 are performed on NR Cell 11. //@siclog "Steps 51-66a1" siclog@ f_NR5GC_RRC_IdleState1N_Def(nr_Cell11, TESTMode_OFF); // @siclog "Steps 50a1 - 50a16a1" siclog@ //Void // @siclog "Steps 50b1 - 50b3a1" siclog@ //Void }//End of fl_TC_6_3_2_5_TestBody

- Branches executed

The test case was executed on NR band n78 using NR ciphering nea2 and integrity algorithm nia2

- Execution Log Files

  - Mediatek MT6815

The Mediatek MT6815 UE passed this test case on Anritsu Protocol Conformance Test System ME7834NR. The documentation below is enclosed as evidence of the successful test case run [1]:

- Test case execution log file:

TC_6_3_2_5_LOG.html

- PICS/PIXIT parameter file:

TC_6_3_2_5_PIXIT.xml

- References

| [1] | R5s240310: NR5GC FR1: Supporting information for addition of Rel-17 eCPSOR test case 6.3.2.5 |
| --- | --- |