3GPP TSG-RAN5 Meeting #2024-TTCN email	R5s240311

# Online, , 8th Dec 2023 - 31st Dec 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.523-3 | CR | 4812 | rev | - | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to NBIOT testcase 22.4.13 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI13_Test, NB_IOT-UEConTest |  |  |  |  |  | Date: |  |  | 2024-04-15 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The current TTCN has missed implementing a path outlined in agreed Prose CR R5-20566 which suggests that the IE pdcp-Parameter-r13 is Not Present when both (pc_User_plane_CIoT_Optimisation OR pc_S1_U_DataTransfer) are set to FALSE. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added the code to check if both (pc_User_plane_CIoT_Optimisation OR pc_S1_U_DataTransfer) are set to FALSE than IE pdcp-Parameter-r13 is not present. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may fail the test case |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 22.4.13 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	2

2	Corrections required	2

2.1	function f_TC_22_4_13_NBIOT ()	2

# Overview

This document lists all the essential changes needed to correct problems in the iwd-TTCN3-B2022-09_D24wk12 related to the title of this CR.

Contact: 	Parikshit Bhise

Parikshit.bhise@rohde-schwarz.com

# Corrections required

## function f_TC_22_4_13_NBIOT ()

| Function name | function f_TC_22_4_13_NBIOT () |
| --- | --- |
| Reason for change | The current TTCN has missed implementing a path outlined in agreed Prose CR R5-20566 which suggests that the IE pdcp-Parameter-r13 is Not Present when both (pc_User_plane_CIoT_Optimisation OR pc_S1_U_DataTransfer) are set to FALSE |
| Summary of change | Added the code to check if both (pc_User_plane_CIoT_Optimisation OR pc_S1_U_DataTransfer) are set to FALSE than IE pdcp-Parameter-r13 is not present |
| TTCN module | NBIOT_RRC_ControlPlane.ttcn |
| MCC160 Comment | Accepted in principle but as this field is specified as to be not present in this case, it shall be set to omit |

Before Change:

<Skip Code> function f_TC_22_4_13_NBIOT() runs on NBIOT_PTC { var template PDCP_Parameters_NB_r13 v_PDCP_Parameters := omit; // if CP, IE not present var NB_SRB_COMMON_IND v_ReceivedAspForUeCapabilityInfo; var UECapabilityInformation_NB_r13_IEs v_UECapabilityInformation_r13; var UE_Capability_NB_r13 v_UECapability; var template (present) UE_Capability_NB_r13 v_UE_Capability_NB; f_NBIOT_Init(c1); //Create and configure all cells f_NBIOT_CellConfig_Def(nbiot_Cell1, CONTROL_PLANE); //Bring UE to initial state f_NBIOT_Preamble(nbiot_Cell1, CONTROL_PLANE); f_NBIOT_TestBody_Set(true); //@siclog "Step 1" siclog@ SRB.send(cas_NB_SRB_RrcPdu_REQ(nbiot_Cell1, tsc_SRB1bis, cs_TimingInfo_Now, cs_508_UeCapabilityEnquiry_NB(tsc_RRC_TI_Def))); if (pc_NB_User_Plane_CIoT_Optimisation or pc_S1_U_DataTransfer) { // Check contents if UP @sic R5-170962, R5-173076, R5-203070, R5-205668 sic@ if (not(pc_ROHC_profile0x0002 or pc_ROHC_profile0x0003 or pc_ROHC_profile0x0004 or pc_ROHC_profile0x0006 or pc_ROHC_profile0x0102 or pc_ROHC_profile0x0103 or pc_ROHC_profile0x0104)){ v_UE_Capability_NB := cr_UE_Capability_NB; } else{ v_PDCP_Parameters := cr_PDCP_Parameters_NB(pc_ROHC_profile0x0002, pc_ROHC_profile0x0003, pc_ROHC_profile0x0004, pc_ROHC_profile0x0006, pc_ROHC_profile0x0102, pc_ROHC_profile0x0103, pc_ROHC_profile0x0104); v_UE_Capability_NB := cr_UE_Capability_NB(v_PDCP_Parameters); } } //@siclog "Step 2" siclog@ SRB.receive(car_NB_SRB_RrcPdu_IND(nbiot_Cell1, tsc_SRB1bis, cr_508_UeCapabilityInformation_NB(tsc_RRC_TI_Def, v_UE_Capability_NB, // @sic R5-205668 sic@ cr_UE_RadioPagingInfo_NB(nb1)))) -> value v_ReceivedAspForUeCapabilityInfo; <Skip Code>

After Change:

<Skip Code> function f_TC_22_4_13_NBIOT() runs on NBIOT_PTC { var template PDCP_Parameters_NB_r13 v_PDCP_Parameters := omit; // if CP, IE not present var NB_SRB_COMMON_IND v_ReceivedAspForUeCapabilityInfo; var UECapabilityInformation_NB_r13_IEs v_UECapabilityInformation_r13; var UE_Capability_NB_r13 v_UECapability; var template (present) UE_Capability_NB_r13 v_UE_Capability_NB; f_NBIOT_Init(c1); //Create and configure all cells f_NBIOT_CellConfig_Def(nbiot_Cell1, CONTROL_PLANE); //Bring UE to initial state f_NBIOT_Preamble(nbiot_Cell1, CONTROL_PLANE); f_NBIOT_TestBody_Set(true); //@siclog "Step 1" siclog@ SRB.send(cas_NB_SRB_RrcPdu_REQ(nbiot_Cell1, tsc_SRB1bis, cs_TimingInfo_Now, cs_508_UeCapabilityEnquiry_NB(tsc_RRC_TI_Def))); if (not(pc_NB_User_Plane_CIoT_Optimisation or pc_S1_U_DataTransfer)){ //WA#WI=1275666 v_UE_Capability_NB := cr_UE_Capability_NB; } if (pc_NB_User_Plane_CIoT_Optimisation or pc_S1_U_DataTransfer) { // Check contents if UP @sic R5-170962, R5-173076, R5-203070, R5-205668 sic@ if (not(pc_ROHC_profile0x0002 or pc_ROHC_profile0x0003 or pc_ROHC_profile0x0004 or pc_ROHC_profile0x0006 or pc_ROHC_profile0x0102 or pc_ROHC_profile0x0103 or pc_ROHC_profile0x0104)){ v_UE_Capability_NB := cr_UE_Capability_NB; } else{ v_PDCP_Parameters := cr_PDCP_Parameters_NB(pc_ROHC_profile0x0002, pc_ROHC_profile0x0003, pc_ROHC_profile0x0004, pc_ROHC_profile0x0006, pc_ROHC_profile0x0102, pc_ROHC_profile0x0103, pc_ROHC_profile0x0104); v_UE_Capability_NB := cr_UE_Capability_NB(v_PDCP_Parameters); } } //@siclog "Step 2" siclog@ SRB.receive(car_NB_SRB_RrcPdu_IND(nbiot_Cell1, tsc_SRB1bis, cr_508_UeCapabilityInformation_NB(tsc_RRC_TI_Def, v_UE_Capability_NB, // @sic R5-205668 sic@ cr_UE_RadioPagingInfo_NB(nb1)))) -> value v_ReceivedAspForUeCapabilityInfo; <Skip Code>

#### MCC160 Implementation

if (pc_NB_User_Plane_CIoT_Optimisation or pc_S1_U_DataTransfer) { // Check contents if UP @sic R5-170962, R5-173076, R5-203070, R5-205668 sic@

if (not(pc_ROHC_profile0x0002 or pc_ROHC_profile0x0003 or pc_ROHC_profile0x0004 or pc_ROHC_profile0x0006 or pc_ROHC_profile0x0102 or pc_ROHC_profile0x0103 or pc_ROHC_profile0x0104)){

v_UE_Capability_NB := cr_UE_Capability_NB;

}

else{

v_PDCP_Parameters := cr_PDCP_Parameters_NB(pc_ROHC_profile0x0002,

pc_ROHC_profile0x0003,

pc_ROHC_profile0x0004,

pc_ROHC_profile0x0006,

pc_ROHC_profile0x0102,

pc_ROHC_profile0x0103,

pc_ROHC_profile0x0104);

v_UE_Capability_NB := cr_UE_Capability_NB(v_PDCP_Parameters);

}

} else {

v_UE_Capability_NB := cr_UE_Capability_NB(omit);

}