3GPP TSG-RAN5 Meeting #2024-TTCN email	R5s240313

# Online, , 8th Dec 2023 - 31st Dec 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 3425 | rev | - | Current version: | 17.10.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to NR MAC testcase 7.1.1.3.8.1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2024-04-16 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | In TS 38.523-1 Table 7.1.1.3.8.1.3.2-1: Main behaviour Step 9: SS adjusts cell levels according to row T1 of Table 7.1.1.3.8.3.1.2-0/0A Row T1 suggests that cell power of NR Cell 1 is dropped to -99 dBm and NR Cell 3 remains at -88 dBm According to TS 38.331, Sec 7.1.1: Timer T310 is started as soon as the PHY layer detects physical layer problems for the SpCell (NR Cell 1) i.e. upon receiving N310 consecutive out-of-sync indications from lower layers (Because power of NR Cell 1 has dropped), T310 timer is configured for 1000ms. Upon expiry of T310 timer, UE initiates the connection re-establishment procedure to NR Cell 1. The issue here is the phr-ProhibitTimer is also configured for sf1000 which would mean 1000 subframes, 1 subframe = 1ms. Hence, this timer is also for 1000ms Step 11: Check: After prohibitPHR-Timer after step 7, does the UE transmit a MAC PDU containing Multiple Entry PHR MAC CE containing Type 1 PH of NR SpCell and Scell? Note 3 Hence, for the UE, when T310 timer expires, it tries to perform re-attach to NR Cell 1 by initiating a RACH procedure and sending an RRC Re-Establishment Request which leads to TC failure. |  |  |  |  |  |  |  |  |
| Summary of change: |  | It is recommended to decrease the value of the prohibitPHR-Timer from sf1000 to sf500 so that this timer expires before T310 timer expires and the check at Step 11 can be fulfilled. At Step 12: SS adjusts cell levels according to row T2 of Table 7.1.1.3.8.1.3.2-0/0A Row T2 suggests that cell power of NR Cell 1 is brought back to -88 dBm and NR Cell 3 remains at -88 dBm. This would mean that UE would again be in-sync with NR Cell 1 because it would have received N311 consecutive in-sync indications from lower layers for the SpCell (NR Cell 1) and would stop timer T310. NOTE: A Prose CR would be raised at the next RAN5 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may fail the test case. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.1.1.3.8.1.ENDC, 7.1.1.3.8.1.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  | x |  | Test specifications |  |  |  | 38.523-1 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	3

1	Overview	4

2	Corrections required	4

2.1	Change 1	4

2.2	Change 2	5

# Overview

This document lists all the changes needed to correct issues in the ATS iwd-TTCN3-B2022-09_D24wk12 related to the title of this CR.

Contact:  Bhise Parikshit
parikshit.bhise@rohde-schwarz.com

# Corrections required

## Change 1

| Template name | cs_PHR_Config_71138 |
| --- | --- |
| Reason for change | In TS 38.523-1 Table 7.1.1.3.8.1.3.2-1: Main behaviour Step 9: SS adjusts cell levels according to row T1 of Table 7.1.1.3.8.3.1.2-0/0A Row T1 suggests that cell power of NR Cell 1 is dropped to -99 dBm and NR Cell 3 remains at -88 dBm According to TS 38.331, Sec 7.1.1: Timer T310 is started as soon as the PHY layer detects physical layer problems for the SpCell (NR Cell 1) i.e. upon receiving N310 consecutive out-of-sync indications from lower layers (Because power of NR Cell 1 has dropped), T310 timer is configured for 1000ms. Upon expiry of T310 timer, UE initiates the connection re-establishment procedure to NR Cell 1. The issue here is the phr-ProhibitTimer is also configured for sf1000 which would mean 1000 subframes, 1 subframe = 1ms. Hence, this timer is also for 1000ms Step 11: Check: After prohibitPHR-Timer after step 7, does the UE transmit a MAC PDU containing Multiple Entry PHR MAC CE containing Type 1 PH of NR SpCell and Scell? Note 3 Hence, for the UE, when T310 timer expires, it tries to perform re-attach to NR Cell 1 by initiating a RACH procedure and sending an RRC Re-Establishment Request which leads to TC failure. |
| Summary of change | It is recommended to decrease the value of the prohibitPHR-Timer from sf1000 to sf500 so that this timer expires before T310 timer expires and the check at Step 11 can be fulfilled. At Step 12: SS adjusts cell levels according to row T2 of Table 7.1.1.3.8.1.3.2-0/0A Row T2 suggests that cell power of NR Cell 1 is brought back to -88 dBm and NR Cell 3 remains at -88 dBm. This would mean that UE would again be in-sync with NR Cell 1 because it would have received N311 consecutive in-sync indications from lower layers for the SpCell (NR Cell 1) and would stop timer T310. NOTE: A Prose CR would be raised at the next RAN5 |
| TTCN module | MAC_TC_Common_NR.ttcn |
| MCC160 Comment | Accepted Conditional to Prose CR agreement |

Before Change:

template (value) PHR_Config cs_PHR_Config_71138 := //As per 38.523-1 Table 7.1.1.3.8.1.3.3-5: PHR-Config /* @status APPROVED (ENDC, NR5GC) */ cs_PHR_Config_Params(-, true, -, sf1000, dB3);

After Change:

template (value) PHR_Config cs_PHR_Config_71138 := //As per 38.523-1 Table 7.1.1.3.8.1.3.3-5: PHR-Config /* @status APPROVED (ENDC, NR5GC) */ cs_PHR_Config_Params(-, true, -, sf500, dB3); //WA#

## Change 2

| Template name | f_TC_7_1_1_3_8_NR_TestBody |
| --- | --- |
| Reason for change | In TS 38.523-1 Table 7.1.1.3.8.1.3.2-1: Main behaviour Step 9: SS adjusts cell levels according to row T1 of Table 7.1.1.3.8.3.1.2-0/0A Row T1 suggests that cell power of NR Cell 1 is dropped to -99 dBm and NR Cell 3 remains at -88 dBm According to TS 38.331, Sec 7.1.1: Timer T310 is started as soon as the PHY layer detects physical layer problems for the SpCell (NR Cell 1) i.e. upon receiving N310 consecutive out-of-sync indications from lower layers (Because power of NR Cell 1 has dropped), T310 timer is configured for 1000ms. Upon expiry of T310 timer, UE initiates the connection re-establishment procedure to NR Cell 1. The issue here is the phr-ProhibitTimer is also configured for sf1000 which would mean 1000 subframes, 1 subframe = 1ms. Hence, this timer is also for 1000ms Step 11: Check: After prohibitPHR-Timer after step 7, does the UE transmit a MAC PDU containing Multiple Entry PHR MAC CE containing Type 1 PH of NR SpCell and Scell? Note 3 Hence, for the UE, when T310 timer expires, it tries to perform re-attach to NR Cell 1 by initiating a RACH procedure and sending an RRC Re-Establishment Request which leads to TC failure. |
| Summary of change | It is recommended to decrease the value of the prohibitPHR-Timer from sf1000 to sf500 so that this timer expires before T310 timer expires and the check at Step 11 can be fulfilled. At Step 12: SS adjusts cell levels according to row T2 of Table 7.1.1.3.8.1.3.2-0/0A Row T2 suggests that cell power of NR Cell 1 is brought back to -88 dBm and NR Cell 3 remains at -88 dBm. This would mean that UE would again be in-sync with NR Cell 1 because it would have received N311 consecutive in-sync indications from lower layers for the SpCell (NR Cell 1) and would stop timer T310. NOTE: A Prose CR would be raised at the next RAN5 |
| TTCN module | MAC_TC_Common_NR.ttcn |
| MCC160 Comment | Accepted Conditional to Prose CR agreement |

Before Change:

function f_TC_7_1_1_3_8_NR_TestBody(NR_CellId_Type p_SPCell, NR_CellId_Type p_SCell, template (present) NR_MAC_CE_MultiEntryPHR_Type p_NR_MAC_CE_MultiEntryPHR, SCellIndex p_SCellIndex := tsc_NR_SCellIndex_1) runs on NR_BASE_PTC { //UE power headroom reporting / SCell activation / DL pathloss change reporting/ var NR_SYSTEM_IND v_NR_SYSTEM_IND; var SubFrameTiming_Type v_TimingInfo1; var SubFrameTiming_Type v_TimingInfo2; var SubFrameTiming_Type v_TimingInfo3; var SubFrameTiming_Type v_TimingInfo4; var SubFrameTiming_Type v_TimingInfo5; var float v_TimerMax :=f_NR_SetTimerToleranceMax (p_SPCell, l2Timer, 1.000); var float v_TimerMin :=f_NR_SetTimerToleranceMin (p_SPCell, l2Timer, 1.000); var float v_TimerDuration ; <<SKIPPED CODE>>

After Change:

function f_TC_7_1_1_3_8_NR_TestBody(NR_CellId_Type p_SPCell, NR_CellId_Type p_SCell, template (present) NR_MAC_CE_MultiEntryPHR_Type p_NR_MAC_CE_MultiEntryPHR, SCellIndex p_SCellIndex := tsc_NR_SCellIndex_1) runs on NR_BASE_PTC { //UE power headroom reporting / SCell activation / DL pathloss change reporting/ var NR_SYSTEM_IND v_NR_SYSTEM_IND; var SubFrameTiming_Type v_TimingInfo1; var SubFrameTiming_Type v_TimingInfo2; var SubFrameTiming_Type v_TimingInfo3; var SubFrameTiming_Type v_TimingInfo4; var SubFrameTiming_Type v_TimingInfo5; var float v_TimerMax :=f_NR_SetTimerToleranceMax (p_SPCell, l2Timer, 0.500); //WA# var float v_TimerMin :=f_NR_SetTimerToleranceMin (p_SPCell, l2Timer, 0.500); //WA# var float v_TimerDuration ; <<SKIPPED CODE>>