3GPP TSG-RAN5 Meeting #2024-TTCN email	R5s240317

# Online, , 8th Dec 2023 - 31st Dec 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 3427 | rev | - | Current version: | 17.10.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to NR5GC test case 11.4.5 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies UK Ltd |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2024-04-18 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | As per the NOTE 4 in the test procedure, UE shall send the registration request with security header type ‘0000’ Plain 5GS NAS message or ‘0001’ integrity protected NAS message. However current TTCN only allows registration request with security header type ‘0000’ Plain 5GS NAS message which can cause test case failure. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Allowed both security header type ‘0000’ Plain 5GS NAS message or ‘0001’ integrity protected NAS message |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may fail the test case |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 11.4.5.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

- Overview

This document lists all the changes needed to correct issues in the ATS iwd-TTCN3-B2022-09_D24wk12 related to the title of this CR.

Contact:	Mohit Kanchan
	mohit.kanchan@keysight.com

- Corrections required.

  - Change 1

| Function name | fl_TC_11_4_5_NR5GC_TestBody |
| --- | --- |
| Reason for change | As per the NOTE 4 in the test procedure, UE shall send the registration request with security header type ‘0000’ Plain 5GS NAS message or ‘0001’ integrity protected NAS message. However current TTCN only allows registration request with security header type ‘0000’ Plain 5GS NAS message which can cause test case failure. |
| Summary of change | Allowed both security header type ‘0000’ Plain 5GS NAS message or ‘0001’ integrity protected NAS message |
| TTCN module | Emergency_NR5GC.ttcn |
| MCC160 Comment | Accepted. |

Before Change:

function fl_TC_11_4_5_NR5GC_TestBody() runs on NR5GC_PTC { var NG_NAS_MSG_Indication_Type v_ReceivedMsg; var GMM_MobilityInfo_Type v_GMM_MobilityInfo; var NR_SecurityParams_Type v_RRC_SecurityParams; var TypeOfRegistration_Type v_TypeOfRegistration; var NR_SRB_COMMON_IND v_ReceivedAsp; var template (omit) GSM_MobilityInfo_Type v_GSM_MobilityInfo_IMS; var GSM_MobilityInfo_Type v_GSM_MobilityInfo; var PDUSessionInfoList_Type v_PDUSessionInfo; var O1_Type v_PDUSessionId; var PDN_Index_Type v_PDNIndex;//@sic R5-240509 R5s240045 sic@ var NG_NAS_UL_Pdu_Type v_RxdMsg; var template (value) NG_NAS_DL_Pdu_Type v_PiggyBackedMsgToSend;//@sic R5-240509 R5s240045 sic@ var boolean v_FinishedIMS := false;//@sic R5-240509 R5s240045 sic@ ---- ---- //@siclog "Step 14" siclog@ //Check: Does the UE perform on NR Cell 1 the Registration procedure for initial registration as specified in TS 38.508-1 [4] subclause 4.5.2.2-2, 'connected without release'? SRB.check(receive(car_NR_SRB0_RrcPdu_IND(nr_Cell1, cr_38508_RRCSetupRequest(cr_NR_EstablishmentCause_Any)))); f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 14 passed"); f_NR5GC_MobileInfo_Init();//@sic R5s210875 sic@ f_NR5GC_Security_Init();//@sic R5s210875 sic@ v_RRC_SecurityParams := f_NR_Security_Get();//@sic R5s210875 sic@ v_RRC_SecurityParams.AS_Integrity.Algorithm := px_NR_IntegrityProtAlgorithm;//@sic R5s210875 sic@ v_RRC_SecurityParams.AS_Ciphering.Algorithm := px_NR_CipheringAlgorithm;//@sic R5s210875 sic@ f_NR_Security_Set(v_RRC_SecurityParams);//@sic R5s210875 sic@ v_TypeOfRegistration := f_NR5GC_InitialOrMobilityRegistrationRequest (v_GMM_MobilityInfo, v_ReceivedMsg, nr_Cell1);//@sic R5-225410 sic@ //@siclog "17a1-17b3a1" siclog@ @sic R5-202600 sic@ //NOTE: The 5GS registration type shall be only set as Mobility Registration for R16 UEs according to TS 24.501 subclause 5.2.3.2.5 specified in Release 16. The EXCEPTION description applies only to R15 UEs. if (v_TypeOfRegistration == Initial_NoSecurity) { //@siclog "17a1-17a16a1" siclog@ //IF 5GS registration type is set as Initial Registration in step 22, THEN steps 5 to 20a1 of the registration procedure described in TS 38.508-1 [4] Table 4.5.2.2-2 are performed on NR Cell 12. f_NR5GC_RRC_Idle_Steps5_20(v_GMM_MobilityInfo, nr_Cell1, v_ReceivedMsg, TESTMode_OFF,-, v_TypeOfRegistration); //@sic R5-225410 sic@ } else if (v_TypeOfRegistration == Mobility) { //@siclog "17b1-17b3a1" siclog@ // IF 5GS registration type is set as Mobility Registration in step 22, THEN steps 5 to 6a1 of the generic test procedure in TS 38.508-1 [4] Table 4.9.5.2.2-1 are performed on NR Cell 11. f_NR5GC_MobilityRegistration_Steps4_6(nr_Cell1, v_GMM_MobilityInfo); //@sic R5-225410 sic@ } }

After Change:

function fl_TC_11_4_5_NR5GC_TestBody() runs on NR5GC_PTC { var NG_NAS_MSG_Indication_Type v_ReceivedMsg; var GMM_MobilityInfo_Type v_GMM_MobilityInfo; var NR_SecurityParams_Type v_RRC_SecurityParams; var TypeOfRegistration_Type v_TypeOfRegistration; var NR_SRB_COMMON_IND v_ReceivedAsp; var template (omit) GSM_MobilityInfo_Type v_GSM_MobilityInfo_IMS; var GSM_MobilityInfo_Type v_GSM_MobilityInfo; var PDUSessionInfoList_Type v_PDUSessionInfo; var O1_Type v_PDUSessionId; var PDN_Index_Type v_PDNIndex;//@sic R5-240509 R5s240045 sic@ var NG_NAS_UL_Pdu_Type v_RxdMsg; var template (value) NG_NAS_DL_Pdu_Type v_PiggyBackedMsgToSend;//@sic R5-240509 R5s240045 sic@ var boolean v_FinishedIMS := false;//@sic R5-240509 R5s240045 sic@ ---- ---- //@siclog "Step 14" siclog@ //Check: Does the UE perform on NR Cell 1 the Registration procedure for initial registration as specified in TS 38.508-1 [4] subclause 4.5.2.2-2, 'connected without release'? SRB.check(receive(car_NR_SRB0_RrcPdu_IND(nr_Cell1, cr_38508_RRCSetupRequest(cr_NR_EstablishmentCause_Any)))); f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 14 passed"); f_NR5GC_MobileInfo_Init();//@sic R5s210875 sic@ f_NR5GC_Security_Init();//@sic R5s210875 sic@ v_RRC_SecurityParams := f_NR_Security_Get();//@sic R5s210875 sic@ v_RRC_SecurityParams.AS_Integrity.Algorithm := px_NR_IntegrityProtAlgorithm;//@sic R5s210875 sic@ v_RRC_SecurityParams.AS_Ciphering.Algorithm := px_NR_CipheringAlgorithm;//@sic R5s210875 sic@ f_NR_Security_Set(v_RRC_SecurityParams);//@sic R5s210875 sic@ v_TypeOfRegistration := f_NR5GC_InitialOrMobilityRegistrationRequest (v_GMM_MobilityInfo, v_ReceivedMsg, nr_Cell1);//@sic R5-225410 sic@ //@siclog "17a1-17b3a1" siclog@ @sic R5-202600 sic@ //NOTE: The 5GS registration type shall be only set as Mobility Registration for R16 UEs according to TS 24.501 subclause 5.2.3.2.5 specified in Release 16. The EXCEPTION description applies only to R15 UEs. if ((v_TypeOfRegistration == Initial_NoSecurity) or (v_TypeOfRegistration == Initial_Secure)) { //@siclog "17a1-17a16a1" siclog@ //IF 5GS registration type is set as Initial Registration in step 22, THEN steps 5 to 20a1 of the registration procedure described in TS 38.508-1 [4] Table 4.5.2.2-2 are performed on NR Cell 12. f_NR5GC_RRC_Idle_Steps5_20(v_GMM_MobilityInfo, nr_Cell1, v_ReceivedMsg, TESTMode_OFF,-, v_TypeOfRegistration); //@sic R5-225410 sic@ } else if (v_TypeOfRegistration == Mobility) { //@siclog "17b1-17b3a1" siclog@ // IF 5GS registration type is set as Mobility Registration in step 22, THEN steps 5 to 6a1 of the generic test procedure in TS 38.508-1 [4] Table 4.9.5.2.2-1 are performed on NR Cell 11. f_NR5GC_MobilityRegistration_Steps4_6(nr_Cell1, v_GMM_MobilityInfo); //@sic R5-225410 sic@ } }