3GPP TSG-RAN5 Meeting #2024-TTCN email	R5s240319

# Online, , 8th Dec 2023 - 31st Dec 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.523-3 | CR | 4816 | rev | - | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to LTE-IRAT testcase 11.2.11 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI9_Test |  |  |  |  |  | Date: |  |  | 2024-04-18 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Similar to TDD, when UEA0 is used cipheringOnOff also needs to be updated to false |  |  |  |  |  |  |  |  |
| Summary of change: |  | Renamed the function and added vc_UTRAN_Global.Security.cipheringOnOff to be updated |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may fail the test case |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 11.2.11 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  | x |  | Test specifications |  |  |  | 36.523-1 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Table of Contents

Table of Contents	2

1	Overview	2

2	Corrections required to test case 11.2.11	2

2.1	f_UTRAN_Security_SetCipheringAlgorithm	2

2.2	f_TC_11_2_11_UTRAN	3

# Overview

This document lists all the essential changes needed to correct problems in the iwd-TTCN3-B2022-09_D24wk12 related to the title of this CR.

Contact: 	Parikshit Bhise

Parikshit.bhise@rohde-schwarz.com

# Corrections required to test case 11.2.11

## f_UTRAN_Security_SetCipheringAlgorithm

| Template name | f_UTRAN_Security_SetCipheringAlgorithm |
| --- | --- |
| Reason for change | Similar to TDD, when UEA0 is used cipheringOnOff also needs to be updated to false |
| Summary of change | Renamed the function and added vc_UTRAN_Global.Security.cipheringOnOff to be updated |
| TTCN module | UTRAN_CellInfo |
| MCC160 Comment | Not needed. Solved as per R5-243474 and R5-243475. |

Before Change:

function f_UTRAN_Security_SetCipheringAlgorithm (CipheringAlgorithm_r7 p_Alg) runs on UTRAN_PTC { vc_UTRAN_Global.Security.cipheringAlgorithm := p_Alg; }

After Change:

function f_UTRAN_Security_SetCiphering (CipheringAlgorithm_r7 p_Alg, boolean p_cipheringOnOff) runs on UTRAN_PTC { // updated to add second parameter p_cipheringOnOff vc_UTRAN_Global.Security.cipheringAlgorithm := p_Alg; vc_UTRAN_Global.Security.cipheringOnOff := p_cipheringOnOff }

## f_TC_11_2_11_UTRAN

| Template name | f_TC_11_2_11_UTRAN |
| --- | --- |
| Reason for change | Similar to TDD, when UEA0 is used cipheringOnOff also needs to be updated to false |
| Summary of change | Used new name of the function and updated the second parameter to false. |
| TTCN module | IMS_EmergencyCall_UGC_UTRAN |
| MCC160 Comment | Not needed. Solved as per R5-243474 and R5-243475. |

Before Change:

function f_TC_11_2_11_UTRAN() runs on UTRAN_IRAT_PTC { var integer v_T0_CPICH_Cell5 := tsc_AttenuationCellOFF; // @sic R5s140949 sic@ var integer v_T1_CPICH_Cell5 := -64; var integer v_T0_PCCPCH_Cell5 := tsc_AttenuationCellOFF; // @sic R5s140949 sic@ var integer v_T1_PCCPCH_Cell5 := -64; // Initialise all cells f_UTRAN_Init(EUTRA_UTRAN); f_UTRAN_Security_SetCipheringAlgorithm(uea0); // @sic R5s230373 sic@

After Change:

function f_TC_11_2_11_UTRAN() runs on UTRAN_IRAT_PTC { var integer v_T0_CPICH_Cell5 := tsc_AttenuationCellOFF; // @sic R5s140949 sic@ var integer v_T1_CPICH_Cell5 := -64; var integer v_T0_PCCPCH_Cell5 := tsc_AttenuationCellOFF; // @sic R5s140949 sic@ var integer v_T1_PCCPCH_Cell5 := -64; // Initialise all cells f_UTRAN_Init(EUTRA_UTRAN); f_UTRAN_Security_SetCiphering(uea0,false); // @sic R5s230373 sic@