3GPP TSG-RAN5 Meeting #2024-TTCN email	R5s240356

# Online, , 8th Dec 2023 - 31st Dec 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 3431 | rev | - | Current version: | 17.10.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to NR RRC testcases 8.1.5.8.2.x |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2024-04-29 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The default OnSR UL Grant config should be stopped before the initiation of the RRC procedure Delay procedure because the UE would try to obtain an UL grant via the SR procedure as soon as it receives the RRC Reconfiguration message at Step 3 or the RRC Resume message at Step 6a5 instead of waiting for the periodic UL grant transmissions as the TC intends for the calculation of the RRC procedure Delay. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Stopped the default OnSR UL Grant config before the function f_NR5GC_RRC_ProcedureDelay and function f_NR5GC_RRC_ProcedureDelayWithRACH |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may fail the test case. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.1.5.8.2.1.NR5GC, 8.1.5.8.2.2,NR5GC, 8.1.5.8.2.3.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

2	Corrections required	3

2.1	Change 1	3

# Overview

This document lists all the changes needed to correct issues in the ATS iwd-TTCN3-B2022-09_D24wk12 related to the title of this CR.

Contact:  Bhise Parikshit
parikshit.bhise@rohde-schwarz.com

# Corrections required

## Change 1

| Template name | f_TC_8_1_5_8_2_x_NR5GC_TestBody |
| --- | --- |
| Reason for change | The default OnSR UL Grant config should be stopped before the initiation of the RRC procedure Delay procedure because the UE would try to obtain an UL grant via the SR procedure as soon as it receives the RRC Reconfiguration message at Step 3 or the RRC Resume message at Step 6a5 instead of waiting for the periodic UL grant transmissions as the TC intends for the calculation of the RRC procedure Delay. |
| Summary of change | Stopped the default OnSR UL Grant config before the function f_NR5GC_RRC_ProcedureDelay and function f_NR5GC_RRC_ProcedureDelayWithRACH |
| TTCN module | RRC_Others_CA_NR5GC.ttcn |
| MCC160 | Accepted |

Before Change:

function f_TC_8_1_5_8_2_x_NR5GC_TestBody(NR_CellId_Type p_PCell, NR_CellId_Type p_SCell) runs on NR5GC_PTC { <<SKIPPED CODE>> // @siclog "Steps 1-3" siclog@ // The SS transmits an RRCReconfiguration message with sCellToAddModList to add SCell(Cell 3) // The SS starts transmitting a periodic DCI from (16*2microsec+1+TDL) slots after step 1 to schedule PUSCH. // Check: Does the UE transmit an RRCReconfigurationComplete message within (16*2microsec+1+TDL+K2) slots after successful completion of step 1? (Note 1, Note 2, Note 3) //@sic R5s210842 change 1 sic@ v_TimingInfo := f_NR_GetNextSendOccasion_DL(p_PCell,500); if (f_NR_CellInfo_GetSCS(p_PCell) != kHz15) { //@sic R5s230375 R5s230510 sic@ v_TimingInfo2 := f_SubFrameTiming_AddMilliSeconds(v_TimingInfo, 5); // 5 ms later } else { //Allow more time for FDD/TDD SCS=15kHz - k1=7 - reconfigure SS after UL HARQ feedback is received by SS v_TimingInfo2 := f_SubFrameTiming_AddMilliSeconds(v_TimingInfo, 10); // 10 ms later } f_NR_ConfigPCell_AddRelSCells(p_PCell, {p_SCell}, tsc_PCellIndex_0, cs_TimingInfo_NR(v_TimingInfo2), f_NR_CellInfo_GetRNTI(p_PCell)); f_NR_ConfigSCell(p_SCell,v_SCGConfigCA, cs_TimingInfo_NR(v_TimingInfo2)); //@sic R5s220365 sic@ f_NR5GC_RRC_ProcedureDelay(p_PCell, cas_NR_SRB1_RrcPdu_REQ(p_PCell, cs_TimingInfo_NR(v_TimingInfo), cs_38508_RRCReconfiguration(tsc_NR_RRC_TI_Def, -, -, -, v_V1530Ext), crs_MacBearerRouting(p_PCell)), //@sic R5s221132 sic@ car_NR_SRB_RrcPdu_IND(p_PCell, tsc_NR_RbId_SRB1, cr_38508_RRCReconfigurationComplete, - , crs_MacBearerRouting(p_PCell)), 16, //converted in slots inside the function "Step 3"); f_NR_ULGrantConfiguration_Start(p_PCell); //@sic R5s220365 sic@ <<SKIPPED CODE>> //@sic R5s210620 R5s210842 change 1 sic@ v_TimingInfo := f_NR_GetNextSendOccasion_DL(p_PCell,500); if (f_NR_CellInfo_GetSCS(p_PCell) != kHz15) { //@sic R5s230510 sic@ v_TimingInfo2 := f_SubFrameTiming_AddMilliSeconds(v_TimingInfo, 5); // 5 ms later } else { //Allow more time for FDD/TDD SCS=15kHz - k1=7 - reconfigure SS after UL HARQ feedback is received by SS v_TimingInfo2 := f_SubFrameTiming_AddMilliSeconds(v_TimingInfo, 10); // 10 ms later } f_NR_ConfigPCell_AddRelSCells(p_PCell, {p_SCell}, tsc_PCellIndex_0, cs_TimingInfo_NR(v_TimingInfo2), f_NR_CellInfo_GetRNTI(p_PCell)); f_NR_ConfigSCell(p_SCell,v_SCGConfigCA, cs_TimingInfo_NR(v_TimingInfo2)); // @siclog "Steps 6a5 - 6a7" siclog@ //The SS transmits an RRCResume message with sCellToAddModList to add SCell(Cell 3) //The SS starts transmitting a periodic DCI from (16*2micros+1+TDL) slots after step 6a5 to schedule PUSCH.(Note 1, Note 2) //Check: Does the UE transmit an RRCResumeComplete message within (16*2micros+1+TDL+K2) slots after successful completion of step 6a5? (Note 1, Note 2, Note 3) //@sic R5-235291 R5-237424 sic@ f_NR5GC_RRC_ProcedureDelayWithRACH(p_PCell, cas_NR_SRB1_RrcPdu_REQ(p_PCell, cs_TimingInfo_NR(v_TimingInfo), cs_38508_RRCResume(tsc_NR_RRC_TI_Def, bit2oct(encvalue(v_SCGConfig)), v_RadioBearerConfig), crs_MacBearerRouting(p_PCell)), car_NR_SRB1_RrcPdu_IND(p_PCell, cr_38508_RRCResumeComplete, -, crs_MacBearerRouting(p_PCell)), 16, //converted in slots inside the function "Step 6a7"); f_NR_ULGrantConfiguration_Start(p_PCell); //@sic R5s220365 sic@ <<SKIPPED CODE>>

After Change:

function f_TC_8_1_5_8_2_x_NR5GC_TestBody(NR_CellId_Type p_PCell, NR_CellId_Type p_SCell) runs on NR5GC_PTC { <<SKIPPED CODE>> // @siclog "Steps 1-3" siclog@ // The SS transmits an RRCReconfiguration message with sCellToAddModList to add SCell(Cell 3) // The SS starts transmitting a periodic DCI from (16*2microsec+1+TDL) slots after step 1 to schedule PUSCH. // Check: Does the UE transmit an RRCReconfigurationComplete message within (16*2microsec+1+TDL+K2) slots after successful completion of step 1? (Note 1, Note 2, Note 3) //@sic R5s210842 change 1 sic@ v_TimingInfo := f_NR_GetNextSendOccasion_DL(p_PCell,500); if (f_NR_CellInfo_GetSCS(p_PCell) != kHz15) { //@sic R5s230375 R5s230510 sic@ v_TimingInfo2 := f_SubFrameTiming_AddMilliSeconds(v_TimingInfo, 5); // 5 ms later } else { //Allow more time for FDD/TDD SCS=15kHz - k1=7 - reconfigure SS after UL HARQ feedback is received by SS v_TimingInfo2 := f_SubFrameTiming_AddMilliSeconds(v_TimingInfo, 10); // 10 ms later } f_NR_ConfigPCell_AddRelSCells(p_PCell, {p_SCell}, tsc_PCellIndex_0, cs_TimingInfo_NR(v_TimingInfo2), f_NR_CellInfo_GetRNTI(p_PCell)); f_NR_ConfigSCell(p_SCell,v_SCGConfigCA, cs_TimingInfo_NR(v_TimingInfo2)); f_NR_ULGrantConfiguration_Stop(p_PCell); //WA# //@sic R5s220365 sic@ f_NR5GC_RRC_ProcedureDelay(p_PCell, cas_NR_SRB1_RrcPdu_REQ(p_PCell, cs_TimingInfo_NR(v_TimingInfo), cs_38508_RRCReconfiguration(tsc_NR_RRC_TI_Def, -, -, -, v_V1530Ext), crs_MacBearerRouting(p_PCell)), //@sic R5s221132 sic@ car_NR_SRB_RrcPdu_IND(p_PCell, tsc_NR_RbId_SRB1, cr_38508_RRCReconfigurationComplete, - , crs_MacBearerRouting(p_PCell)), 16, //converted in slots inside the function "Step 3"); f_NR_ULGrantConfiguration_Start(p_PCell); //@sic R5s220365 sic@ <<SKIPPED CODE>> //@sic R5s210620 R5s210842 change 1 sic@ v_TimingInfo := f_NR_GetNextSendOccasion_DL(p_PCell,500); if (f_NR_CellInfo_GetSCS(p_PCell) != kHz15) { //@sic R5s230510 sic@ v_TimingInfo2 := f_SubFrameTiming_AddMilliSeconds(v_TimingInfo, 5); // 5 ms later } else { //Allow more time for FDD/TDD SCS=15kHz - k1=7 - reconfigure SS after UL HARQ feedback is received by SS v_TimingInfo2 := f_SubFrameTiming_AddMilliSeconds(v_TimingInfo, 10); // 10 ms later } f_NR_ConfigPCell_AddRelSCells(p_PCell, {p_SCell}, tsc_PCellIndex_0, cs_TimingInfo_NR(v_TimingInfo2), f_NR_CellInfo_GetRNTI(p_PCell)); f_NR_ConfigSCell(p_SCell,v_SCGConfigCA, cs_TimingInfo_NR(v_TimingInfo2)); f_NR_ULGrantConfiguration_Stop(p_PCell); //WA# // @siclog "Steps 6a5 - 6a7" siclog@ //The SS transmits an RRCResume message with sCellToAddModList to add SCell(Cell 3) //The SS starts transmitting a periodic DCI from (16*2micros+1+TDL) slots after step 6a5 to schedule PUSCH.(Note 1, Note 2) //Check: Does the UE transmit an RRCResumeComplete message within (16*2micros+1+TDL+K2) slots after successful completion of step 6a5? (Note 1, Note 2, Note 3) //@sic R5-235291 R5-237424 sic@ f_NR5GC_RRC_ProcedureDelayWithRACH(p_PCell, cas_NR_SRB1_RrcPdu_REQ(p_PCell, cs_TimingInfo_NR(v_TimingInfo), cs_38508_RRCResume(tsc_NR_RRC_TI_Def, bit2oct(encvalue(v_SCGConfig)), v_RadioBearerConfig), crs_MacBearerRouting(p_PCell)), car_NR_SRB1_RrcPdu_IND(p_PCell, cr_38508_RRCResumeComplete, -, crs_MacBearerRouting(p_PCell)), 16, //converted in slots inside the function "Step 6a7"); f_NR_ULGrantConfiguration_Start(p_PCell); //@sic R5s220365 sic@ <<SKIPPED CODE>>