3GPP TSG-RAN5 Meeting #2024-TTCN email	R5s240361

# Online, , 8th Dec 2023 - 31st Dec 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 3437 | rev | - | Current version: | 17.10.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to NR5GC Idle mode test case 6.1.1.5 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies UK Ltd |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2024-05-07 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The current TTCN implementation pass a hardcoded IMSI value to the function f_NR5GC_MobileInfo_SetDefaultIMSI which is not correct. The SIM profile used in this test case only fix the MCC and MNC part of IMSI. The MSIN part is chosen by the test house as per TS 34.108, clause 8.3.2.2. The current TTCN implemention will fail the testcase if different MSIN is used. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Removed the function SetDefaultIMSI which was used to set the IMSI including MCC, MNC and MSIN. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may fail the test case |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.1.1.5.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Overview

This document lists all the changes needed to correct issues in the ATS iWD-TTCN3-B2020-09_D24wk12 related to the title of this CR.

Contact:	Mohit Kanchan
	mohit.kanchan@keysight.com

# Corrections required.

## Change 1

| Function name | function f_TC_6_1_1_5_NR5GC() runs on NR5GC_PTC |
| --- | --- |
| Reason for change | The current TTCN implementation pass a hardcoded IMSI value to the function f_NR5GC_MobileInfo_SetDefaultIMSI which is not correct. The SIM profile used in this test case only fix the MCC and MNC part of IMSI. The MSIN part is chosen by the test house as per TS 34.108, clause 8.3.2.2. The current TTCN implemention will fail the testcase if different MSIN is used. |
| Summary of change | Removed the function SetDefaultIMSI which was used to set the IMSI including MCC, MNC and MSIN. |
| TTCN module | Idle_PLMNSelection_NR5GC.ttcn |
| MCC160 Comment | Alternative solution implemented: updated value of stored IMSI with PLMN of special USIM |

Before Change:

function f_TC_6_1_1_5_NR5GC() runs on NR5GC_PTC {// PLMN selection of RPLMN, HPLMN/EHPLMN, UPLMN and OPLMN / Automatic mode / User reselection f_NR5GC_Init(NR_1); // Set Sys info and cell info for cell 3& 13 to be used // @sic R5-203367 sic@ f_NR_CellInfo_SetPLMN(nr_Cell3, tsc_NR_HPLMN_003_21);//PLMN3 f_NR_CellInfo_SetPLMN(nr_Cell13, tsc_NR_HPLMN_004_31);//PLMN4 // Create and configure cell f_NR_CellConfig_Def(nr_Cell1); f_NR_CellConfig_Def(nr_Cell3); // @sic R5-203367 sic@ f_NR_CellConfig_Def(nr_Cell12); f_NR_CellConfig_Def(nr_Cell13); // Equip UE with the USIM configuration 6 specified in table 6.4.1-6 TS 38.508-1 f_UT_USIM_Insert(UT, "USIM configuration 6 specified in table 6.4.1-6 TS 38.508-1"); f_NR5GC_MobileInfo_SetDefaultIMSI('003210123456063'H); // @sic R5s240248 sic@ f_UT_AutomaticPLMN_Select_WithSwitchOnOff (UT); // Preamble: The UE is in state Registered, Idle Mode (state 1N-A) on NR Cell 1 according to TS 38.508-1 f_NR5GC_Preamble(nr_Cell1, STATE_IDLE_1A); f_NR_TestBody_Set(true); f_TC_6_1_1_5_NR5GC_TestBody(); f_NR_TestBody_Set(false); // Switch/Power off UE f_NR_Postamble(nr_Cell3, STATE_IDLE_1A); // @sic R5-203367 sic@ }

After Change:

function f_TC_6_1_1_5_NR5GC() runs on NR5GC_PTC {// PLMN selection of RPLMN, HPLMN/EHPLMN, UPLMN and OPLMN / Automatic mode / User reselection f_NR5GC_Init(NR_1); // Set Sys info and cell info for cell 3& 13 to be used // @sic R5-203367 sic@ f_NR_CellInfo_SetPLMN(nr_Cell3, tsc_NR_HPLMN_003_21);//PLMN3 f_NR_CellInfo_SetPLMN(nr_Cell13, tsc_NR_HPLMN_004_31);//PLMN4 // Create and configure cell f_NR_CellConfig_Def(nr_Cell1); f_NR_CellConfig_Def(nr_Cell3); // @sic R5-203367 sic@ f_NR_CellConfig_Def(nr_Cell12); f_NR_CellConfig_Def(nr_Cell13); // Equip UE with the USIM configuration 6 specified in table 6.4.1-6 TS 38.508-1 f_UT_USIM_Insert(UT, "USIM configuration 6 specified in table 6.4.1-6 TS 38.508-1"); //Removed_f_NR5GC_MobileInfo_SetDefaultIMSI('003210123456063'H); // @sic R5s240248 sic@ f_UT_AutomaticPLMN_Select_WithSwitchOnOff (UT); // Preamble: The UE is in state Registered, Idle Mode (state 1N-A) on NR Cell 1 according to TS 38.508-1 f_NR5GC_Preamble(nr_Cell1, STATE_IDLE_1A); f_NR_TestBody_Set(true); f_TC_6_1_1_5_NR5GC_TestBody(); f_NR_TestBody_Set(false); // Switch/Power off UE f_NR_Postamble(nr_Cell3, STATE_IDLE_1A); // @sic R5-203367 sic@ }

## Change 2

| Function name | function f_TC_6_1_1_5_NR5GC_TestBody() runs on NR5GC_PTC |
| --- | --- |
| Reason for change | The current TTCN implementation pass a hardcoded IMSI value to the function f_NR5GC_MobileInfo_SetDefaultIMSI which is not correct. The SIM profile used in this test case only fix the MCC and MNC part of IMSI. The MSIN part is chosen by the test house as per TS 34.108, clause 8.3.2.2. The current TTCN implemention will fail the testcase if different MSIN is used. |
| Summary of change | Removed the function SetDefaultIMSI which was used to set the IMSI including MCC, MNC and MSIN . |
| TTCN module | Idle_PLMNSelection_NR5GC.ttcn |
| MCC160 Comment | Alternative solution implemented: updated value of stored IMSI with PLMN of special USIM |

Before Change:

function f_TC_6_1_1_5_NR5GC_TestBody() runs on NR5GC_PTC {// @sic R5-201127, R5s200165, R5-203367 sic@ var template (value) NR_CellPowerList_Type v_CellPowerList_AtT1; var template (value) NR_CellPowerList_Type v_CellPowerList_AtT2; var template (value) NR_CellPowerList_Type v_CellPowerList_AtT3; var NG_NAS_MSG_Indication_Type v_ReceivedMsg; var GMM_MobilityInfo_Type v_GMM_MobilityInfo; var float v_TimerValue1 := 90.0; …. … // @siclog "Step 14" siclog@ // Does the UE send an RRCSetupRequest on NR Cell 1? f_NR_RRC_SetupRequest_Def(nr_Cell1); f_NR_PreliminaryPass(__FILE__, __LINE__, "Test Case 6.1.1.5 Step 14"); // @siclog "Step 14A-14B" siclog@ // Steps 3-4 of Table 4.5.2.2-2 of the generic procedure in TS 38.508-1 [4] are performed f_NR_RRCSetup_Def (nr_Cell1); v_ReceivedMsg := f_NR_RRCSetupComplete_Def(nr_Cell1, cr_38508_RRCSetupComplete(tsc_NR_RRC_TI_Def, ?), (tsc_SHT_NoSecurityProtection, tsc_SHT_IntegrityProtected)); if (f_Check_NG_RegistrationReqMsg (v_GMM_MobilityInfo, v_ReceivedMsg.Pdu, Mobility)) { // @siclog "Step 15b1-15b2 Void" siclog@ // @siclog "Step 15b3-15b5a1" siclog@ // IF 5GS registration type is set as Mobility Registration in step 14B, THEN Steps 4 to 6a1 of the generic test procedure in TS 38.508-1 Table 4.9.5.2.2-1 are performed on NR Cell 1 f_NR5GC_MobilityRegistration_Steps4_6(nr_Cell1, v_GMM_MobilityInfo); } else if (f_Check_NG_RegistrationReqMsg (v_GMM_MobilityInfo, v_ReceivedMsg.Pdu, Initial_Secure)) { // @siclog "Step 15a1-15a16a1" siclog@ // IF 5GS registration type is set as Initial Registration in step 14B, THEN Steps 5 to 20a1of the generic test procedure in TS 38.508-1 Table 4.5.2.2-2 are performed on NR Cell 1 f_NR5GC_MobileInfo_Init(); // @sic R5s230085 sic@ f_NR5GC_MobileInfo_SetDefaultIMSI('003210123456063'H); // @sic R5s240248 sic@ f_NR5GC_RRC_Idle_Steps5_20(v_GMM_MobilityInfo, nr_Cell1, v_ReceivedMsg, TESTMode_OFF, -, Initial_Secure); } else { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Registration Request Message Failed"); } //@siclog "Step 16 - 19b1" siclog@ // Void // @siclog "Step 20" siclog@ // The SS changes the power level setting according to the row T3 in table 6.1.1.5.3.2-1/2. f_NR_SetCellPowerList(v_CellPowerList_AtT3); // @siclog "Step 21" siclog@ // Cause the UE in Automatic network selection mode to initiate user reselection and registration onto an available PLMN. f_UT_TriggerUserReselection(UT); // @siclog "Step 22" siclog@ // Does the UE send an RRCSetupRequest on NR Cell 13? f_NR_RRC_SetupRequest_Def(nr_Cell3); f_NR_PreliminaryPass(__FILE__, __LINE__, "Test Case 6.1.1.5 Step 22"); // @siclog "Step 22A-22B" siclog@ // Steps 3-4 of Table 4.5.2.2-2 of the generic procedure in TS 38.508-1 [4] are performed f_NR_RRCSetup_Def (nr_Cell3); v_ReceivedMsg := f_NR_RRCSetupComplete_Def(nr_Cell3, cr_38508_RRCSetupComplete(tsc_NR_RRC_TI_Def, ?), (tsc_SHT_NoSecurityProtection, tsc_SHT_IntegrityProtected)); if (f_Check_NG_RegistrationReqMsg (v_GMM_MobilityInfo, v_ReceivedMsg.Pdu, Mobility)) { // @siclog "Step 23b1-23b2 Void" siclog@ // @siclog "Step 23b3-23b5a1" siclog@ // IF 5GS registration type is set as Mobility Registration in step 22B, THEN Steps 4 to 6a1 of the generic test procedure in TS 38.508-1 Table 4.9.5.2.2-1 are performed on NR Cell 3 f_NR5GC_MobilityRegistration_Steps4_6(nr_Cell3, v_GMM_MobilityInfo); } else if (f_Check_NG_RegistrationReqMsg (v_GMM_MobilityInfo, v_ReceivedMsg.Pdu, Initial_Secure)) { // @siclog "Step 23a1-23a16a1" siclog@ // IF 5GS registration type is set as Initial Registration in step 22B, THEN Steps 5 to 20a1of the generic test procedure in TS 38.508-1 Table 4.5.2.2-2 are performed on NR Cell 13 f_NR5GC_MobileInfo_Init(); // @sic R5s230085 sic@ f_NR5GC_MobileInfo_SetDefaultIMSI('003210123456063'H); // @sic R5s240248 sic@ f_NR5GC_RRC_Idle_Steps5_20(v_GMM_MobilityInfo, nr_Cell3, v_ReceivedMsg, TESTMode_OFF, -, Initial_Secure); } else { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Registration Request Message Failed"); } //@siclog "Step 24 - 27b1" siclog@ // Void }

After Change:

function f_TC_6_1_1_5_NR5GC_TestBody() runs on NR5GC_PTC {// @sic R5-201127, R5s200165, R5-203367 sic@ var template (value) NR_CellPowerList_Type v_CellPowerList_AtT1; var template (value) NR_CellPowerList_Type v_CellPowerList_AtT2; var template (value) NR_CellPowerList_Type v_CellPowerList_AtT3; var NG_NAS_MSG_Indication_Type v_ReceivedMsg; var GMM_MobilityInfo_Type v_GMM_MobilityInfo; var float v_TimerValue1 := 90.0; … … // @siclog "Step 14" siclog@ // Does the UE send an RRCSetupRequest on NR Cell 1? f_NR_RRC_SetupRequest_Def(nr_Cell1); f_NR_PreliminaryPass(__FILE__, __LINE__, "Test Case 6.1.1.5 Step 14"); // @siclog "Step 14A-14B" siclog@ // Steps 3-4 of Table 4.5.2.2-2 of the generic procedure in TS 38.508-1 [4] are performed f_NR_RRCSetup_Def (nr_Cell1); v_ReceivedMsg := f_NR_RRCSetupComplete_Def(nr_Cell1, cr_38508_RRCSetupComplete(tsc_NR_RRC_TI_Def, ?), (tsc_SHT_NoSecurityProtection, tsc_SHT_IntegrityProtected)); if (f_Check_NG_RegistrationReqMsg (v_GMM_MobilityInfo, v_ReceivedMsg.Pdu, Mobility)) { // @siclog "Step 15b1-15b2 Void" siclog@ // @siclog "Step 15b3-15b5a1" siclog@ // IF 5GS registration type is set as Mobility Registration in step 14B, THEN Steps 4 to 6a1 of the generic test procedure in TS 38.508-1 Table 4.9.5.2.2-1 are performed on NR Cell 1 f_NR5GC_MobilityRegistration_Steps4_6(nr_Cell1, v_GMM_MobilityInfo); } else if (f_Check_NG_RegistrationReqMsg (v_GMM_MobilityInfo, v_ReceivedMsg.Pdu, Initial_Secure)) { // @siclog "Step 15a1-15a16a1" siclog@ // IF 5GS registration type is set as Initial Registration in step 14B, THEN Steps 5 to 20a1of the generic test procedure in TS 38.508-1 Table 4.5.2.2-2 are performed on NR Cell 1 f_NR5GC_MobileInfo_Init(); // @sic R5s230085 sic@ //Removed_f_NR5GC_MobileInfo_SetDefaultIMSI('003210123456063'H); // @sic R5s240248 sic@ f_NR5GC_RRC_Idle_Steps5_20(v_GMM_MobilityInfo, nr_Cell1, v_ReceivedMsg, TESTMode_OFF, -, Initial_Secure); } else { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Registration Request Message Failed"); } //@siclog "Step 16 - 19b1" siclog@ // Void // @siclog "Step 20" siclog@ // The SS changes the power level setting according to the row T3 in table 6.1.1.5.3.2-1/2. f_NR_SetCellPowerList(v_CellPowerList_AtT3); // @siclog "Step 21" siclog@ // Cause the UE in Automatic network selection mode to initiate user reselection and registration onto an available PLMN. f_UT_TriggerUserReselection(UT); // @siclog "Step 22" siclog@ // Does the UE send an RRCSetupRequest on NR Cell 13? f_NR_RRC_SetupRequest_Def(nr_Cell3); f_NR_PreliminaryPass(__FILE__, __LINE__, "Test Case 6.1.1.5 Step 22"); // @siclog "Step 22A-22B" siclog@ // Steps 3-4 of Table 4.5.2.2-2 of the generic procedure in TS 38.508-1 [4] are performed f_NR_RRCSetup_Def (nr_Cell3); v_ReceivedMsg := f_NR_RRCSetupComplete_Def(nr_Cell3, cr_38508_RRCSetupComplete(tsc_NR_RRC_TI_Def, ?), (tsc_SHT_NoSecurityProtection, tsc_SHT_IntegrityProtected)); if (f_Check_NG_RegistrationReqMsg (v_GMM_MobilityInfo, v_ReceivedMsg.Pdu, Mobility)) { // @siclog "Step 23b1-23b2 Void" siclog@ // @siclog "Step 23b3-23b5a1" siclog@ // IF 5GS registration type is set as Mobility Registration in step 22B, THEN Steps 4 to 6a1 of the generic test procedure in TS 38.508-1 Table 4.9.5.2.2-1 are performed on NR Cell 3 f_NR5GC_MobilityRegistration_Steps4_6(nr_Cell3, v_GMM_MobilityInfo); } else if (f_Check_NG_RegistrationReqMsg (v_GMM_MobilityInfo, v_ReceivedMsg.Pdu, Initial_Secure)) { // @siclog "Step 23a1-23a16a1" siclog@ // IF 5GS registration type is set as Initial Registration in step 22B, THEN Steps 5 to 20a1of the generic test procedure in TS 38.508-1 Table 4.5.2.2-2 are performed on NR Cell 13 f_NR5GC_MobileInfo_Init(); // @sic R5s230085 sic@ //Removed_f_NR5GC_MobileInfo_SetDefaultIMSI('003210123456063'H); // @sic R5s240248 sic@ f_NR5GC_RRC_Idle_Steps5_20(v_GMM_MobilityInfo, nr_Cell3, v_ReceivedMsg, TESTMode_OFF, -, Initial_Secure); } else { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Registration Request Message Failed"); } //@siclog "Step 24 - 27b1" siclog@ // Void }

#### MCC160 Implementation:

function f_TC_6_1_1_5_NR5GC() runs on NR5GC_PTC

{// PLMN selection of RPLMN, HPLMN/EHPLMN, UPLMN and OPLMN / Automatic mode / User reselection

var hexstring v_PLMN3 := '00321'H;

var hexstring v_IMSI;

var hexstring v_SpecialUSIM_IMSI;

f_NR5GC_Init(NR_1);

// Set Sys info and cell info for cell 3& 13 to be used

// @sic R5-203367 sic@

f_NR_CellInfo_SetPLMN(nr_Cell3, tsc_NR_HPLMN_003_21);//PLMN3

f_NR_CellInfo_SetPLMN(nr_Cell13, tsc_NR_HPLMN_004_31);//PLMN4

// Create and configure cell

f_NR_CellConfig_Def(nr_Cell1);

f_NR_CellConfig_Def(nr_Cell3); // @sic R5-203367 sic@

f_NR_CellConfig_Def(nr_Cell12);

f_NR_CellConfig_Def(nr_Cell13);

// Equip UE with the USIM configuration 6 specified in table 6.4.1-6 TS 38.508-1

f_UT_USIM_Insert(UT, "USIM configuration 6 specified in table 6.4.1-6 TS 38.508-1");

v_IMSI := f_NR5GC_MobileInfo_GetDefaultIMSI();

v_SpecialUSIM_IMSI := v_PLMN3 & substr(v_IMSI, 0, 5);

f_NR5GC_MobileInfo_SetDefaultIMSI(v_SpecialUSIM_IMSI);

f_UT_AutomaticPLMN_Select_WithSwitchOnOff (UT);

// Preamble: The UE is in state Registered, Idle Mode (state 1N-A) on NR Cell 1 according to TS 38.508-1

f_NR5GC_Preamble(nr_Cell1, STATE_IDLE_1A);

f_NR_TestBody_Set(true);

f_TC_6_1_1_5_NR5GC_TestBody(v_SpecialUSIM_IMSI);

f_NR_TestBody_Set(false);

// Switch/Power off UE

f_NR_Postamble(nr_Cell3, STATE_IDLE_1A); // @sic R5-203367 sic@

}

/*

* @desc      REFERENCE TS 38.523-1 clause 6.1.1.5

* @status    APPROVED (NR5GC)

*/

function f_TC_6_1_1_5_NR5GC_TestBody(hexstring p_SpecialUSIM_IMSI) runs on NR5GC_PTC

{// @sic R5-201127, R5s200165, R5-203367 sic@

var template (value) NR_CellPowerList_Type v_CellPowerList_AtT1;

var template (value) NR_CellPowerList_Type v_CellPowerList_AtT2;

var template (value) NR_CellPowerList_Type v_CellPowerList_AtT3;

var NG_NAS_MSG_Indication_Type v_ReceivedMsg;

var GMM_MobilityInfo_Type v_GMM_MobilityInfo;

var float v_TimerValue1 := 90.0;

v_CellPowerList_AtT1:={

cs_NR_CellPower(nr_Cell1, tsc_NR_ServingCellSSS_EPRE_FR1, tsc_NR_ServingCellSSS_EPRE_FR2),

cs_NR_CellPower(nr_Cell12, tsc_NR_ServingCellSSS_EPRE_FR1, tsc_NR_ServingCellSSS_EPRE_FR2),

cs_NR_CellPower(nr_Cell13, tsc_NR_ServingCellSSS_EPRE_FR1, tsc_NR_ServingCellSSS_EPRE_FR2)

};

v_CellPowerList_AtT2:={

cs_NR_CellPower(nr_Cell1, tsc_NR_NonSuitableOffCellSSS_EPRE, tsc_NR_NonSuitableOffCellSSS_EPRE),

cs_NR_CellPower(nr_Cell13, tsc_NR_NonSuitableOffCellSSS_EPRE, tsc_NR_NonSuitableOffCellSSS_EPRE)

};

v_CellPowerList_AtT3:={

cs_NR_CellPower(nr_Cell1, tsc_NR_ServingCellSSS_EPRE_FR1, tsc_NR_ServingCellSSS_EPRE_FR2),

cs_NR_CellPower(nr_Cell3, tsc_NR_ServingCellSSS_EPRE_FR1, tsc_NR_ServingCellSSS_EPRE_FR2),

cs_NR_CellPower(nr_Cell12, tsc_NR_NonSuitableOffCellSSS_EPRE, tsc_NR_NonSuitableOffCellSSS_EPRE),

cs_NR_CellPower(nr_Cell13, tsc_NR_ServingCellSSS_EPRE_FR1, tsc_NR_ServingCellSSS_EPRE_FR2)

};

// @siclog "Step 1" siclog@

// The SS changes the power level setting according to the row T1 in table 6.1.1.5.3.2-1/2

f_NR_SetCellPowerList(v_CellPowerList_AtT1);

// @siclog "Step 2" siclog@

// Cause the UE in Automatic network selection mode to initiate user reselection and registration onto an available PLMN

f_UT_TriggerUserReselection(UT);

// @siclog "Step 3" siclog@

// Does the UE send an RRCSetupRequest on NR Cell 12?

f_NR_RRC_SetupRequest_Def(nr_Cell12);

f_NR_PreliminaryPass(__FILE__, __LINE__, "Test Case 6.1.1.5 Step 3");

// @siclog "Step 3A-3B" siclog@

// Steps 3-4 of Table 4.5.2.2-2 of the generic procedure in TS 38.508-1 [4] are performed

f_NR_RRCSetup_Def (nr_Cell12);

v_ReceivedMsg := f_NR_RRCSetupComplete_Def(nr_Cell12,

cr_38508_RRCSetupComplete(tsc_NR_RRC_TI_Def, ?),

(tsc_SHT_NoSecurityProtection, tsc_SHT_IntegrityProtected));

if (f_Check_NG_RegistrationReqMsg (v_GMM_MobilityInfo, v_ReceivedMsg.Pdu, Mobility)) {

// @siclog "Step 4b1-4b2 Void" siclog@

// @siclog "Step 4b3-4b5a1" siclog@

// IF 5GS registration type is set as Mobility Registration in step 3B, THEN Steps 4 to 6a1 of the generic test procedure in TS 38.508-1 Table 4.9.5.2.2-1 are performed on NR Cell 12

f_NR5GC_MobilityRegistration_Steps4_6(nr_Cell12, v_GMM_MobilityInfo);

}

else if (f_Check_NG_RegistrationReqMsg (v_GMM_MobilityInfo, v_ReceivedMsg.Pdu, Initial_Secure)) {

// @siclog "Step 4a1-4a16a1" siclog@

// IF 5GS registration type is set as Initial Registration in step 3B, THEN Steps 5 to 20a1of the generic test procedure in TS 38.508-1 Table 4.5.2.2-2 are performed on NR Cell 12

f_NR5GC_MobileInfo_Init(); // @sic R5s230085 sic@

f_NR5GC_MobileInfo_SetDefaultIMSI(p_SpecialUSIM_IMSI);

f_NR5GC_RRC_Idle_Steps5_20(v_GMM_MobilityInfo, nr_Cell12, v_ReceivedMsg, TESTMode_OFF, -, Initial_Secure);

}

else {

f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Registration Request Message Failed");

}

//@siclog "Step 5 - 8b1" siclog@

// Void

// @siclog "Step 9" siclog@

// The SS changes the power level setting according to the row T2 in table 6.1.1.5.3.2-1/2

f_NR_SetCellPowerList(v_CellPowerList_AtT2);

// @siclog "Step 10" siclog@

// Cause the UE in Automatic network selection mode to initiate user reselection and registration onto an available PLMN

f_UT_TriggerUserReselection(UT);

// @siclog "Step 11" siclog@

// Does the UE send an RRCSetupRequest on NR Cell 12 within 90 s?

f_NR_CheckNoRRCSetupReq(nr_Cell12, v_TimerValue1, -, "Step 11: UE sent an RRCSetupRequest on NR Cell 12 within 90s"); //@sic R5s201387 sic@

// @siclog "Step 12" siclog@

// The SS changes the power level setting according to the row T1 in table 6.1.1.5.3.2-1/2.

f_NR_SetCellPowerList(v_CellPowerList_AtT1);

// @siclog "Step 13" siclog@

// Cause the UE in Automatic network selection mode to initiate user reselection and registration onto an available PLMN.

f_UT_TriggerUserReselection(UT);

// @siclog "Step 14" siclog@

// Does the UE send an RRCSetupRequest on NR Cell 1?

f_NR_RRC_SetupRequest_Def(nr_Cell1);

f_NR_PreliminaryPass(__FILE__, __LINE__, "Test Case 6.1.1.5 Step 14");

// @siclog "Step 14A-14B" siclog@

// Steps 3-4 of Table 4.5.2.2-2 of the generic procedure in TS 38.508-1 [4] are performed

f_NR_RRCSetup_Def (nr_Cell1);

v_ReceivedMsg := f_NR_RRCSetupComplete_Def(nr_Cell1,

cr_38508_RRCSetupComplete(tsc_NR_RRC_TI_Def, ?),

(tsc_SHT_NoSecurityProtection, tsc_SHT_IntegrityProtected));

if (f_Check_NG_RegistrationReqMsg (v_GMM_MobilityInfo, v_ReceivedMsg.Pdu, Mobility)) {

// @siclog "Step 15b1-15b2 Void" siclog@

// @siclog "Step 15b3-15b5a1" siclog@

// IF 5GS registration type is set as Mobility Registration in step 14B, THEN Steps 4 to 6a1 of the generic test procedure in TS 38.508-1 Table 4.9.5.2.2-1 are performed on NR Cell 1

f_NR5GC_MobilityRegistration_Steps4_6(nr_Cell1, v_GMM_MobilityInfo);

}

else if (f_Check_NG_RegistrationReqMsg (v_GMM_MobilityInfo, v_ReceivedMsg.Pdu, Initial_Secure)) {

// @siclog "Step 15a1-15a16a1" siclog@

// IF 5GS registration type is set as Initial Registration in step 14B, THEN Steps 5 to 20a1of the generic test procedure in TS 38.508-1 Table 4.5.2.2-2 are performed on NR Cell 1

f_NR5GC_MobileInfo_Init(); // @sic R5s230085 sic@

f_NR5GC_MobileInfo_SetDefaultIMSI(p_SpecialUSIM_IMSI); // @sic R5s240248 sic@

f_NR5GC_RRC_Idle_Steps5_20(v_GMM_MobilityInfo, nr_Cell1, v_ReceivedMsg, TESTMode_OFF, -, Initial_Secure);

}

else {

f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Registration Request Message Failed");

}

//@siclog "Step 16 - 19b1" siclog@

// Void

// @siclog "Step 20" siclog@

// The SS changes the power level setting according to the row T3 in table 6.1.1.5.3.2-1/2.

f_NR_SetCellPowerList(v_CellPowerList_AtT3);

// @siclog "Step 21" siclog@

// Cause the UE in Automatic network selection mode to initiate user reselection and registration onto an available PLMN.

f_UT_TriggerUserReselection(UT);

// @siclog "Step 22" siclog@

// Does the UE send an RRCSetupRequest on NR Cell 13?

f_NR_RRC_SetupRequest_Def(nr_Cell3);

f_NR_PreliminaryPass(__FILE__, __LINE__, "Test Case 6.1.1.5 Step 22");

// @siclog "Step 22A-22B" siclog@

// Steps 3-4 of Table 4.5.2.2-2 of the generic procedure in TS 38.508-1 [4] are performed

f_NR_RRCSetup_Def (nr_Cell3);

v_ReceivedMsg := f_NR_RRCSetupComplete_Def(nr_Cell3,

cr_38508_RRCSetupComplete(tsc_NR_RRC_TI_Def, ?),

(tsc_SHT_NoSecurityProtection, tsc_SHT_IntegrityProtected));

if (f_Check_NG_RegistrationReqMsg (v_GMM_MobilityInfo, v_ReceivedMsg.Pdu, Mobility)) {

// @siclog "Step 23b1-23b2 Void" siclog@

// @siclog "Step 23b3-23b5a1" siclog@

// IF 5GS registration type is set as Mobility Registration in step 22B, THEN Steps 4 to 6a1 of the generic test procedure in TS 38.508-1 Table 4.9.5.2.2-1 are performed on NR Cell 3

f_NR5GC_MobilityRegistration_Steps4_6(nr_Cell3, v_GMM_MobilityInfo);

}

else if (f_Check_NG_RegistrationReqMsg (v_GMM_MobilityInfo, v_ReceivedMsg.Pdu, Initial_Secure)) {

// @siclog "Step 23a1-23a16a1" siclog@

// IF 5GS registration type is set as Initial Registration in step 22B, THEN Steps 5 to 20a1of the generic test procedure in TS 38.508-1 Table 4.5.2.2-2 are performed on NR Cell 13

f_NR5GC_MobileInfo_Init(); // @sic R5s230085 sic@

f_NR5GC_MobileInfo_SetDefaultIMSI(p_SpecialUSIM_IMSI); // @sic R5s240248 sic@

f_NR5GC_RRC_Idle_Steps5_20(v_GMM_MobilityInfo, nr_Cell3, v_ReceivedMsg, TESTMode_OFF, -, Initial_Secure);

}

else {

f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Registration Request Message Failed");

}

//@siclog "Step 24 - 27b1" siclog@

// Void

}