3GPP TSG-RAN5 Meeting #2024-TTCN email	R5s240365

# Online, , 8th Dec 2023 - 31st Dec 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.523-3 | CR | 4823 | rev | - | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections to NBIOT testcase 22.3.1.2 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI13_Test, NB_IOT-UEConTest |  |  |  |  |  | Date: |  |  | 2024-05-07 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | In Testcase 22.3.1.2 at step 31/32 a MAC PDU is sent with timing info based on the timing at step 21. This is incorrect as it places the time the MAC PDU should be sent in the past. This can cause a fail for a conformant UE depending on implementation. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Changed the timing info variable to increment from to be the timing info at step 24 as this ensures the MAC PDU is sent at the correct time in the TC. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may fail the test cases. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 22.3.1.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  | x |  | Test specifications |  |  |  | 36.523-1 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  | R5-242524 raised for RAN5#103 |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Table of Contents

Table of Contents	2

1	Overview	3

2	Corrections required	4

# Overview

This document lists all the changes needed to correct issues in the ATS iwd-TTCN3-B2022-09_D24wk12 related to the title of this CR.

Contact:  Bhise Parikshit
parikshit.bhise@rohde-schwarz.com

# Corrections required

  - Correction to function

| Function name | f_TC_22_3_1_2_NBIOT |
| --- | --- |
| Reason for change | At step 31/32 a Mac PDU is sent “in the 88th period (9th for NTN) NPDCCH period bundle after the transmission at step 21” which is incorrect and schedules the MAC PDU to be sent in the past at this point in the TC (the 88th period after step 21 happens at step 26). This can cause the TC to fail at step 33 if the implementation sends the Mac PDU at step 32 immediately due to TA timer not expirying at that point. |
| Summary of change | Changed the variable to be incrementing for the timing info at step 31/32 to be from step 24 as this puts the timing correctly inline with the rest of the testcase. |
| TTCN module | NBIOT_MAC_Testcases.ttcn |
| MCC160 Comment | Accepted |

Before:

>>> skipped code <<<

//@siclog "Step 30" siclog@

f_NBIOT_MAC_CPMode_RlcStatusPdu_ReceiveAndCheck(v_RLC_CountsInfoListByRef, v_CellId);

f_NBIOT_PreliminaryPass(__FILE__, __LINE__, "Step 30");

t_Timer.start(2.0);  /* at step 30 we are in the 59th NPDCCH cycle after sending the TA command at step 24;

UL grant at step 34 is scheduled for the 89th NPDCCH cycle => 2s should be sufficient */

//@siclog "Step 31" siclog@

//In the 88th (9th for NTN) NPDCCH period bundle after the transmission at step 21 SS transmits a downlink assignment to C-RNTI assigned to the UE on NPDCCH and transmits in the indicated downlink assignment a RLC PDU with polling field 'P' set to '0' in a MAC PDU on NPDSCH. (NOTE 10, NOTE 13, , NOTE 4)

if (v_NtnEnvironment)

{//@sic R5-241564 sic@

v_TimingInfo := cs_TimingInfo_SubFrameFull(f_NBIOT_IncrementSearchSpace(v_TimingStart_Step21, int2float(9 * v_NpdcchPeriodBundle)));

}

else

{

v_TimingInfo := cs_TimingInfo_SubFrameFull(f_NBIOT_IncrementSearchSpace(v_TimingStart_Step21, int2float(88 * v_NpdcchPeriodBundle)));

}

//@siclog "Step 32" siclog@

v_IncrementVTS := 1;    /* @sic R5-185020: DL RLC Sequence number incremented after step 32 sic@ */

v_MAC_SDUList := {

fl_NBIOT_MAC_EncodedRlcAmPdu_TX(v_RLC_CountsInfoListByRef, fl_ESM_DATA_TRANSPORT_UserData(v_RlcSduSizeDL), tsc_NoPoll, v_IncrementVTS)  /* @sic R5s170813 change 1.3: polling field set to 0 sic@

@sic R5s180063 change 2.1: v_IncrementVTS := 0 sic@

@sic R5-185020: v_IncrementVTS := 1 sic@ */

};

v_MAC_PDU := f_NBIOT_MAC_PDU_WithSDUs_TX(tsc_LchId_SIB1bis, v_MAC_SDUList);

f_NBIOT_MAC_CPMode_SendMacPdu(v_CellId, v_TimingInfo, v_MAC_PDU);

//@siclog "Step 34" siclog@

//In the 90th (10th for NTN) NPDCCH period bundle after the transmission at step 24 the SS allocates an UL grant for the UE.(NOTE 3, NOTE 4)

if (v_NtnEnvironment)

{//@sic R5-241564 sic@

v_TimingInfo := cs_TimingInfo_SubFrameFull(f_NBIOT_IncrementSearchSpace(v_TimingStart_Step24, int2float(10 * v_NpdcchPeriodBundle)));

}

else

{

v_TimingInfo := cs_TimingInfo_SubFrameFull(f_NBIOT_IncrementSearchSpace(v_TimingStart_Step24, int2float(90 * v_NpdcchPeriodBundle)));

}

f_NBIOT_ULGrantTransmission(v_CellId, cs_NB_UL_GrantScheduling_Start(cs_NB_SingleGrant), v_TimingInfo);

alt {

//@siclog "Step 33" siclog@

[] SYSIND.receive(car_NB_UL_HARQ_IND(v_CellId)) {

f_NBIOT_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 33");

}

//@siclog "Step 35" siclog@

[] L2DATA.receive(car_NB_L2_DATA_IND(cr_NB_IndAspCommonPart_RB(?, ?), ?)) {

f_NBIOT_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 35");

}

[] t_Timer.timeout {

}

}

>>> skipped code <<<

After:

>>> skipped code <<<

//@siclog "Step 30" siclog@

f_NBIOT_MAC_CPMode_RlcStatusPdu_ReceiveAndCheck(v_RLC_CountsInfoListByRef, v_CellId);

f_NBIOT_PreliminaryPass(__FILE__, __LINE__, "Step 30");

t_Timer.start(2.0);  /* at step 30 we are in the 59th NPDCCH cycle after sending the TA command at step 24;

UL grant at step 34 is scheduled for the 89th NPDCCH cycle => 2s should be sufficient */

//@siclog "Step 31" siclog@

//In the 88th (9th for NTN) NPDCCH period bundle after the transmission at step 24 SS transmits a downlink assignment to C-RNTI assigned to the UE on NPDCCH and transmits in the indicated downlink assignment a RLC PDU with polling field 'P' set to '0' in a MAC PDU on NPDSCH. (NOTE 10, NOTE 13, , NOTE 4)

if (v_NtnEnvironment)

{//@sic R5-241564 sic@

v_TimingInfo := cs_TimingInfo_SubFrameFull(f_NBIOT_IncrementSearchSpace(v_TimingStart_Step24, int2float(9 * v_NpdcchPeriodBundle)));

}

else

{

v_TimingInfo := cs_TimingInfo_SubFrameFull(f_NBIOT_IncrementSearchSpace(v_TimingStart_Step24, int2float(88 * v_NpdcchPeriodBundle)));

}

//@siclog "Step 32" siclog@

v_IncrementVTS := 1;    /* @sic R5-185020: DL RLC Sequence number incremented after step 32 sic@ */

v_MAC_SDUList := {

fl_NBIOT_MAC_EncodedRlcAmPdu_TX(v_RLC_CountsInfoListByRef, fl_ESM_DATA_TRANSPORT_UserData(v_RlcSduSizeDL), tsc_NoPoll, v_IncrementVTS)  /* @sic R5s170813 change 1.3: polling field set to 0 sic@

@sic R5s180063 change 2.1: v_IncrementVTS := 0 sic@

@sic R5-185020: v_IncrementVTS := 1 sic@ */

};

v_MAC_PDU := f_NBIOT_MAC_PDU_WithSDUs_TX(tsc_LchId_SIB1bis, v_MAC_SDUList);

f_NBIOT_MAC_CPMode_SendMacPdu(v_CellId, v_TimingInfo, v_MAC_PDU);

//@siclog "Step 34" siclog@

//In the 90th (10th for NTN) NPDCCH period bundle after the transmission at step 24 the SS allocates an UL grant for the UE.(NOTE 3, NOTE 4)

if (v_NtnEnvironment)

{//@sic R5-241564 sic@

v_TimingInfo := cs_TimingInfo_SubFrameFull(f_NBIOT_IncrementSearchSpace(v_TimingStart_Step24, int2float(10 * v_NpdcchPeriodBundle)));

}

else

{

v_TimingInfo := cs_TimingInfo_SubFrameFull(f_NBIOT_IncrementSearchSpace(v_TimingStart_Step24, int2float(90 * v_NpdcchPeriodBundle)));

}

f_NBIOT_ULGrantTransmission(v_CellId, cs_NB_UL_GrantScheduling_Start(cs_NB_SingleGrant), v_TimingInfo);

alt {

//@siclog "Step 33" siclog@

[] SYSIND.receive(car_NB_UL_HARQ_IND(v_CellId)) {

f_NBIOT_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 33");

}

//@siclog "Step 35" siclog@

[] L2DATA.receive(car_NB_L2_DATA_IND(cr_NB_IndAspCommonPart_RB(?, ?), ?)) {

f_NBIOT_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 35");

}

[] t_Timer.timeout {

}

}

>>> skipped code <<<