3GPP TSG-RAN5 Meeting #2024-TTCN email	R5s240378

# Online, , 8th Dec 2023 - 31st Dec 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 3446 | rev | - | Current version: | 17.10.0 |  |
| For HELPon using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to NR5GC testcase 7.1.1.1.2 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Starpoint |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2024-05-22 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In agreed R5s190842 change 4.1.3, searchSpaceSIB1 is set to omit. But after that, step0AA is added in the test procedure and SearchSpaceSIB1 is set to 0 in SIB1.Same configuration need to be set in RRCReconfiguration message in step0A. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Delete the incorrect configuration. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may fail the testcase |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.1.1.1.2.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	2

2	Corrections required	2

2.1	f_TC_7_1_1_1_2_NR5GC	2

# Overview

This document lists all the essential changes needed to correct problems in the iwd-TTCN3-B2022-09_D24wk12 related to the title of this CR.

Contact:	Ting Gao

gaoting@starpointcomm.com

# Corrections required

## f_TC_7_1_1_1_2_NR5GC

| Functionname | f_TC_7_1_1_1_2_NR5GC |
| --- | --- |
| Reason for change | In agreed R5s190842 change 4.1.3, searchSpaceSIB1 is set to omit. But after that, step0AA is added in the test procedure and SearchSpaceSIB1 is set to 0 in SIB1.Same configuration need to be set in RRCReconfiguration message in step0A. |
| Summary of change | Delete the incorrect configuration. |
| TTCN module | MAC_NR5GC.ttcn |
| MCC160 Comment | Accepted |

Before Change:

function f_TC_7_1_1_1_2_NR5GC () runs on NR5GC_PTC { // Random access procedure / Successful / C-RNTI Based / Preamble selected by MAC itself var DRB_Identity v_DRBId ; var template (value) RACH_ConfigGeneric v_RACH_ConfigGeneric ; var template (value) MAC_CellGroupConfig v_MAC_CellGroupConfig; var template (value) CellGroupConfig v_CellGroupConfig; var template (value) SpCellConfig v_SpCellConfig ; var template (value) RACH_ConfigCommon.ssb_perRACH_OccasionAndCB_PreamblesPerSSB v_Ssb_perRACH_OccasionAndCB_PreamblesPerSSB := { one := n32 }; var template (omit) RRCReconfiguration_v1530_IEs v_V1530Ext; var NR_DuplexMode_Type v_NR_DuplexMode ; var boolean v_IsFR1; var template (value) NR_UplinkBWP_List_Type v_NR_UplinkBWP_List; //@sic R5s190842 sic@ var NR_DRBInfoList_Type v_ExistingDRBs; var integer i :=0; //@sic R5s201454 sic@ var NR_PhysicalParameters_Type v_NR_PhysicalParameters; var template (value) NR_RachProcedureConfig_Type v_NR_RachProcedure; //Init Cell parameters f_NR5GC_Init(NR_1); v_MAC_CellGroupConfig := f_NR_CellInfo_GetMAC_CellGroupConfig(nr_Cell1); v_MAC_CellGroupConfig.tag_Config.tag_ToAddModList[0].timeAlignmentTimer := ms750; f_NR_CellInfo_SetMAC_CellGroupConfig(nr_Cell1, v_MAC_CellGroupConfig); //Create and configure NR cell1 f_NR_CellConfig_Def(nr_Cell1); // Get NR capabilities sent during EUTRA preamble f_NR5GC_Preamble (nr_Cell1, STATE_IDLE_1A, TESTModeA_ON); //Set null cipher Algorithm f_NR_AS_CipheringAlgorithm_Set(nea0); //Retrieve DRB Id of the first PDU session v_DRBId := f_NR_GetDefaultDRB_ForFirstPDUSession(); //@sic R5s190842 sic@ v_ExistingDRBs:= f_NR5GC_MobileInfo_GetDRBInfoList(); for(i :=0;i<lengthof(v_ExistingDRBs);i :=i+1) { v_ExistingDRBs[i].SS_Config.Config.AddOrReconfigure.Sdap:={None :=true}; } f_NR5GC_MobileInfo_SetDRBInfoList(v_ExistingDRBs); //Bring UE to State 3A f_NR5GC_RRC_ConnectedState3N_Def(nr_Cell1, TEST_LOOPModeA_ON, -, -, //p_IpPduDelayTime -, -, //MAC_CellGroupConfig -, - ); f_NR_TestBody_Set(true); //@siclog "Steps 0A-0B" siclog@ v_NR_DuplexMode := f_NR_CellInfo_GetDuplexMode(nr_Cell1); v_IsFR1 := f_NR_CellInfo_GetIsFR1(nr_Cell1); if (v_IsFR1) { if (ischosen(v_NR_DuplexMode.TDD)) //FR1 TDD { v_RACH_ConfigGeneric := cs_RACH_ConfigGeneric71112_FR1_TDD; } else if (f_NR_CellInfo_GetHalfDuplexFDD(nr_Cell1) ) //Half Duplex FR1 FDD { //@sic R5-233408 sic@ v_RACH_ConfigGeneric := cs_RACH_ConfigGeneric71112_FR1_HDFDD; } else //FR1 FDD { v_RACH_ConfigGeneric := cs_RACH_ConfigGeneric71112_FR1_FDD; } } else { // FR2 v_RACH_ConfigGeneric := cs_RACH_ConfigGeneric71112_FR2; } v_SpCellConfig := f_NR_GetReconfigurationWithSyncDef_MCG(nr_Cell1); //@sic R5s200043 sic@ v_NR_UplinkBWP_List := f_NR_CellInfo_GetUplinkBWP_List(nr_Cell1); v_SpCellConfig.reconfigurationWithSync.t304 := ms2000; v_SpCellConfig.reconfigurationWithSync.rach_ConfigDedicated := omit; //@sic R5s190842 sic@ v_SpCellConfig.reconfigurationWithSync.spCellConfigCommon.uplinkConfigCommon.initialUplinkBWP.rach_ConfigCommon.setup.rach_ConfigGeneric := v_RACH_ConfigGeneric; v_NR_UplinkBWP_List[0].Common.R15.rach_ConfigCommon.setup.rach_ConfigGeneric := valueof(v_RACH_ConfigGeneric); v_SpCellConfig.reconfigurationWithSync.spCellConfigCommon.uplinkConfigCommon.initialUplinkBWP.rach_ConfigCommon.setup.totalNumberOfRA_Preambles := 42; v_NR_UplinkBWP_List[0].Common.R15.rach_ConfigCommon.setup.totalNumberOfRA_Preambles := 42; v_SpCellConfig.reconfigurationWithSync.spCellConfigCommon.uplinkConfigCommon.initialUplinkBWP.rach_ConfigCommon.setup.ssb_perRACH_OccasionAndCB_PreamblesPerSSB := { one := n32 }; v_NR_UplinkBWP_List[0].Common.R15.rach_ConfigCommon.setup.ssb_perRACH_OccasionAndCB_PreamblesPerSSB := { one := n32 }; v_SpCellConfig.reconfigurationWithSync.spCellConfigCommon.uplinkConfigCommon.initialUplinkBWP.rach_ConfigCommon.setup.groupBconfigured := cs_GroupBConfigured_Def; v_NR_UplinkBWP_List[0].Common.R15.rach_ConfigCommon.setup.groupBconfigured := valueof(cs_GroupBConfigured_Def); v_SpCellConfig.reconfigurationWithSync.spCellConfigCommon.uplinkConfigCommon.initialUplinkBWP.rach_ConfigCommon.setup.ra_ContentionResolutionTimer := sf48; v_NR_UplinkBWP_List[0].Common.R15.rach_ConfigCommon.setup.ra_ContentionResolutionTimer := sf48; v_SpCellConfig.reconfigurationWithSync.spCellConfigCommon.downlinkConfigCommon.initialDownlinkBWP.pdcch_ConfigCommon.setup.searchSpaceSIB1 := omit; //@sic Step 0AA //Update the same in Sys info //@sic R5s200883 sic@ f_NR_CellInfo_SetUplinkConfigCommon(nr_Cell1, v_RACH_ConfigGeneric, 42, v_Ssb_perRACH_OccasionAndCB_PreamblesPerSSB, cs_GroupBConfigured_Def, sf48); f_NR_CellInfo_SetUplinkBWP_List(nr_Cell1, v_NR_UplinkBWP_List); f_NR_SS_ConfigureSysinfo(nr_Cell1, - , f_NR_InitialiseBcchToPdschConfig(nr_Cell1)); v_CellGroupConfig := cs_38508_CellGroupConfig (tsc_NR_CellGroupId_MCG , omit, omit, omit, omit, v_SpCellConfig //SpCellConfig ); v_V1530Ext := cs_NR_RRCReconfiguration_v1530_IEs(bit2oct(encvalue(v_CellGroupConfig))); // Configure SS to handle CBRA with UL Grant as a result of Reconfiguration with Sync //@sic R5s220673 R5-223419 sic@ v_NR_PhysicalParameters := f_NR_CellInfo_GetPhysicalParameters(nr_Cell1); v_NR_RachProcedure := cs_NR_RachProcedureConfig_71112_2(v_NR_PhysicalParameters); f_NR_SS_RachProcedureConfig(nr_Cell1, v_NR_RachProcedure); // Send message to UE //@sic R5s190842 sic@ f_NR_SendRRCReconfiguration (nr_Cell1, -,-, v_V1530Ext,-,-,-,false); f_NR_SS_CommonCellConfig(nr_Cell1, cads_NR_UplinkBWP_ConfigCommon_REQ(nr_Cell1, -, v_NR_UplinkBWP_List)); //@sic R5s200549 sic@ SRB.receive(car_NR_SRB_RrcPdu_IND(nr_Cell1, tsc_NR_RbId_SRB1, cr_38508_RRCReconfigurationComplete)); f_TC_7_1_1_1_2_NR_TestBody1(v_DRBId); //@sic Step 17A-17B //@sic R5s201454 sic@ v_NR_PhysicalParameters := f_NR_CellInfo_GetPhysicalParameters(nr_Cell1); v_NR_RachProcedure := cs_NR_RachProcedureConfig_71112_2(v_NR_PhysicalParameters); f_NR_SS_RachProcedureConfig(nr_Cell1, v_NR_RachProcedure); f_NR_SendRRCReconfiguration (nr_Cell1, -,-, v_V1530Ext); f_TC_7_1_1_1_2_NR_TestBody2(v_DRBId); f_NR_TestBody_Set(false); //Postamble f_NR_ULGrantConfiguration_Start(nr_Cell1); // Def UL Grant Config //@sic R5s230130 sic@ //f_NR5GC_OpenUE_TestLoopMode_Deactivate_TestMode(nr_Cell1); f_NR_Postamble(nr_Cell1, STATE_CONNECTED_3A); };

After Change:

function f_TC_7_1_1_1_2_NR5GC () runs on NR5GC_PTC { // Random access procedure / Successful / C-RNTI Based / Preamble selected by MAC itself var DRB_Identity v_DRBId ; var template (value) RACH_ConfigGeneric v_RACH_ConfigGeneric ; var template (value) MAC_CellGroupConfig v_MAC_CellGroupConfig; var template (value) CellGroupConfig v_CellGroupConfig; var template (value) SpCellConfig v_SpCellConfig ; var template (value) RACH_ConfigCommon.ssb_perRACH_OccasionAndCB_PreamblesPerSSB v_Ssb_perRACH_OccasionAndCB_PreamblesPerSSB := { one := n32 }; var template (omit) RRCReconfiguration_v1530_IEs v_V1530Ext; var NR_DuplexMode_Type v_NR_DuplexMode ; var boolean v_IsFR1; var template (value) NR_UplinkBWP_List_Type v_NR_UplinkBWP_List; //@sic R5s190842 sic@ var NR_DRBInfoList_Type v_ExistingDRBs; var integer i :=0; //@sic R5s201454 sic@ var NR_PhysicalParameters_Type v_NR_PhysicalParameters; var template (value) NR_RachProcedureConfig_Type v_NR_RachProcedure; //Init Cell parameters f_NR5GC_Init(NR_1); v_MAC_CellGroupConfig := f_NR_CellInfo_GetMAC_CellGroupConfig(nr_Cell1); v_MAC_CellGroupConfig.tag_Config.tag_ToAddModList[0].timeAlignmentTimer := ms750; f_NR_CellInfo_SetMAC_CellGroupConfig(nr_Cell1, v_MAC_CellGroupConfig); //Create and configure NR cell1 f_NR_CellConfig_Def(nr_Cell1); // Get NR capabilities sent during EUTRA preamble f_NR5GC_Preamble (nr_Cell1, STATE_IDLE_1A, TESTModeA_ON); //Set null cipher Algorithm f_NR_AS_CipheringAlgorithm_Set(nea0); //Retrieve DRB Id of the first PDU session v_DRBId := f_NR_GetDefaultDRB_ForFirstPDUSession(); //@sic R5s190842 sic@ v_ExistingDRBs:= f_NR5GC_MobileInfo_GetDRBInfoList(); for(i :=0;i<lengthof(v_ExistingDRBs);i :=i+1) { v_ExistingDRBs[i].SS_Config.Config.AddOrReconfigure.Sdap:={None :=true}; } f_NR5GC_MobileInfo_SetDRBInfoList(v_ExistingDRBs); //Bring UE to State 3A f_NR5GC_RRC_ConnectedState3N_Def(nr_Cell1, TEST_LOOPModeA_ON, -, -, //p_IpPduDelayTime -, -, //MAC_CellGroupConfig -, - ); f_NR_TestBody_Set(true); //@siclog "Steps 0A-0B" siclog@ v_NR_DuplexMode := f_NR_CellInfo_GetDuplexMode(nr_Cell1); v_IsFR1 := f_NR_CellInfo_GetIsFR1(nr_Cell1); if (v_IsFR1) { if (ischosen(v_NR_DuplexMode.TDD)) //FR1 TDD { v_RACH_ConfigGeneric := cs_RACH_ConfigGeneric71112_FR1_TDD; } else if (f_NR_CellInfo_GetHalfDuplexFDD(nr_Cell1) ) //Half Duplex FR1 FDD { //@sic R5-233408 sic@ v_RACH_ConfigGeneric := cs_RACH_ConfigGeneric71112_FR1_HDFDD; } else //FR1 FDD { v_RACH_ConfigGeneric := cs_RACH_ConfigGeneric71112_FR1_FDD; } } else { // FR2 v_RACH_ConfigGeneric := cs_RACH_ConfigGeneric71112_FR2; } v_SpCellConfig := f_NR_GetReconfigurationWithSyncDef_MCG(nr_Cell1); //@sic R5s200043 sic@ v_NR_UplinkBWP_List := f_NR_CellInfo_GetUplinkBWP_List(nr_Cell1); v_SpCellConfig.reconfigurationWithSync.t304 := ms2000; v_SpCellConfig.reconfigurationWithSync.rach_ConfigDedicated := omit; //@sic R5s190842 sic@ v_SpCellConfig.reconfigurationWithSync.spCellConfigCommon.uplinkConfigCommon.initialUplinkBWP.rach_ConfigCommon.setup.rach_ConfigGeneric := v_RACH_ConfigGeneric; v_NR_UplinkBWP_List[0].Common.R15.rach_ConfigCommon.setup.rach_ConfigGeneric := valueof(v_RACH_ConfigGeneric); v_SpCellConfig.reconfigurationWithSync.spCellConfigCommon.uplinkConfigCommon.initialUplinkBWP.rach_ConfigCommon.setup.totalNumberOfRA_Preambles := 42; v_NR_UplinkBWP_List[0].Common.R15.rach_ConfigCommon.setup.totalNumberOfRA_Preambles := 42; v_SpCellConfig.reconfigurationWithSync.spCellConfigCommon.uplinkConfigCommon.initialUplinkBWP.rach_ConfigCommon.setup.ssb_perRACH_OccasionAndCB_PreamblesPerSSB := { one := n32 }; v_NR_UplinkBWP_List[0].Common.R15.rach_ConfigCommon.setup.ssb_perRACH_OccasionAndCB_PreamblesPerSSB := { one := n32 }; v_SpCellConfig.reconfigurationWithSync.spCellConfigCommon.uplinkConfigCommon.initialUplinkBWP.rach_ConfigCommon.setup.groupBconfigured := cs_GroupBConfigured_Def; v_NR_UplinkBWP_List[0].Common.R15.rach_ConfigCommon.setup.groupBconfigured := valueof(cs_GroupBConfigured_Def); v_SpCellConfig.reconfigurationWithSync.spCellConfigCommon.uplinkConfigCommon.initialUplinkBWP.rach_ConfigCommon.setup.ra_ContentionResolutionTimer := sf48; v_NR_UplinkBWP_List[0].Common.R15.rach_ConfigCommon.setup.ra_ContentionResolutionTimer := sf48; //v_SpCellConfig.reconfigurationWithSync.spCellConfigCommon.downlinkConfigCommon.initialDownlinkBWP.pdcch_ConfigCommon.setup.searchSpaceSIB1 := omit; // del //@sic Step 0AA //Update the same in Sys info //@sic R5s200883 sic@ f_NR_CellInfo_SetUplinkConfigCommon(nr_Cell1, v_RACH_ConfigGeneric, 42, v_Ssb_perRACH_OccasionAndCB_PreamblesPerSSB, cs_GroupBConfigured_Def, sf48); f_NR_CellInfo_SetUplinkBWP_List(nr_Cell1, v_NR_UplinkBWP_List); f_NR_SS_ConfigureSysinfo(nr_Cell1, - , f_NR_InitialiseBcchToPdschConfig(nr_Cell1)); v_CellGroupConfig := cs_38508_CellGroupConfig (tsc_NR_CellGroupId_MCG , omit, omit, omit, omit, v_SpCellConfig //SpCellConfig ); v_V1530Ext := cs_NR_RRCReconfiguration_v1530_IEs(bit2oct(encvalue(v_CellGroupConfig))); // Configure SS to handle CBRA with UL Grant as a result of Reconfiguration with Sync //@sic R5s220673 R5-223419 sic@ v_NR_PhysicalParameters := f_NR_CellInfo_GetPhysicalParameters(nr_Cell1); v_NR_RachProcedure := cs_NR_RachProcedureConfig_71112_2(v_NR_PhysicalParameters); f_NR_SS_RachProcedureConfig(nr_Cell1, v_NR_RachProcedure); // Send message to UE //@sic R5s190842 sic@ f_NR_SendRRCReconfiguration (nr_Cell1, -,-, v_V1530Ext,-,-,-,false); f_NR_SS_CommonCellConfig(nr_Cell1, cads_NR_UplinkBWP_ConfigCommon_REQ(nr_Cell1, -, v_NR_UplinkBWP_List)); //@sic R5s200549 sic@ SRB.receive(car_NR_SRB_RrcPdu_IND(nr_Cell1, tsc_NR_RbId_SRB1, cr_38508_RRCReconfigurationComplete)); f_TC_7_1_1_1_2_NR_TestBody1(v_DRBId); //@sic Step 17A-17B //@sic R5s201454 sic@ v_NR_PhysicalParameters := f_NR_CellInfo_GetPhysicalParameters(nr_Cell1); v_NR_RachProcedure := cs_NR_RachProcedureConfig_71112_2(v_NR_PhysicalParameters); f_NR_SS_RachProcedureConfig(nr_Cell1, v_NR_RachProcedure); f_NR_SendRRCReconfiguration (nr_Cell1, -,-, v_V1530Ext); f_TC_7_1_1_1_2_NR_TestBody2(v_DRBId); f_NR_TestBody_Set(false); //Postamble f_NR_ULGrantConfiguration_Start(nr_Cell1); // Def UL Grant Config //@sic R5s230130 sic@ //f_NR5GC_OpenUE_TestLoopMode_Deactivate_TestMode(nr_Cell1); f_NR_Postamble(nr_Cell1, STATE_CONNECTED_3A); };