3GPP TSG-RAN5 Meeting #2024-TTCN email	R5s240380

# Online, , 8th Dec 2023 - 31st Dec 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 3448 | rev | - | Current version: | 17.10.0 |  |
| For HELPon using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to NR5GC testcase 8.1.5.7.1.x |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Starpoint |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2024-05-22 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | If StartPeriodicCyclicGrant for SCell in step8A effective before DRB send in step8,UE will not send SR to PCell after receiving uplink grant in SCell, UE may not loops back the PDCP PDU on PCell and lead to test case error. Current ttcn implementation can not make sure step8 effective first every time which will lead to test case unstable. |  |  |  |  |  |  |  |  |
| Summary of change: |  | 1 Add v_TimingUL2 for step8A. 2 Modify the timing for DRB send in step8. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may fail the test cases |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.1.5.7.1.1.NR5GC,8.1.5.7.1.2.NR5GC,8.1.5.7.1.3.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	2

2	Corrections required	2

2.1	f_TC_8_1_5_7_1_X_NR5GC_TestBody	2

# Overview

This document lists all the essential changes needed to correct problems in the iwd-TTCN3-B2022-09_D24wk12 related to the title of this CR.

Contact:	Ting Gao

gaoting@starpointcomm.com

# Corrections required

## f_TC_8_1_5_7_1_X_NR5GC_TestBody

| Functionname | f_TC_8_1_5_7_1_X_NR5GC_TestBody |
| --- | --- |
| Reason for change | If StartPeriodicCyclicGrant for SCell in step8A effective before DRB send in step8,UE will not send SR to PCell after receiving uplink grant in SCell, UE may not loops back the PDCP PDU on PCell and lead to test case error. Current ttcn implementation can not make sure step8 effective first every time which will lead to test case unstable. |
| Summary of change | 1 Add v_TimingUL2 for step8A. 2 Modify the timing for DRB send in step8. |
| TTCN module | RRC_Others_CA_NR5GC.ttcn |
| MCC160 Comment | Accepted as improvement |

Before Change:

function f_TC_8_1_5_7_1_X_NR5GC_TestBody(NR_CellId_Type p_PCell, NR_CellId_Type p_SCell) runs on NR5GC_PTC { var template (value) CellGroupConfig v_MCG_Config; var template (value) UL_AM_RLC v_UL_AM_RLC; var template (value) RadioBearerConfig v_RadioBearerConfig; var template (value) RRCReconfiguration_v1530_IEs v_RRCReconfiguration_v1530; var template (omit) NR_RLC_BearerToAddModList_Type v_RLC_Bearer_Config; var IntegerList_Type v_ListOfDRBs := f_NR_GetListOfDRBs_ForFirstPDUSession(1); var octetstring v_PDCP_Data := f_IPv4IPv6_IcmpEchoReply(f_LoopbackModeB_IP_Address_NW()); var template (value) NR_RLC_TestModeConfig_Type v_NR_RLC_TestModeConfig := {Info := {AckProhibit := Prohibit}}; //@sic R5s220944 sic@ var template (value) NR_RLC_TestModeConfig_Type v_NR_RLC_TestModeConfig_continue := {Info := {AckProhibit := Continue}}; //@sic R5s220944 sic@ var integer v_DRBj_PCell := f_NR_GetDefaultDRB_ForFirstPDUSession(); var integer v_DRBj_SCell := v_ListOfDRBs[0]; //DRB is used ONLY for SS configuration! @sic R5-223444 sic@ var NR_UplinkBWP_Type v_NR_UplinkBWP := f_NR_CellInfo_GetUplinkBWP(p_SCell, tsc_NR_BWP_Id); var NR_ResourceAllocation_Type v_NR_ResourceAllocation; var template (value) NR_RadioBearerList_Type v_SS_Drb_ConfigList_NotAck := { f_NR_GetRadioBearerConfig_FullAM(p_PCell, v_DRBj_PCell, -, cs_NR_PDCP_RbConfig_TransparentMode, v_NR_RLC_TestModeConfig), f_NR_GetRadioBearerConfig_FullAM(p_PCell, v_DRBj_SCell, -, cs_NR_PDCP_RbConfig_TransparentMode, v_NR_RLC_TestModeConfig)}; var template (value) NR_RadioBearerList_Type v_SS_Drb_ConfigList_Ack := { f_NR_GetRadioBearerConfig_FullAM(p_PCell, v_DRBj_PCell, -, cs_NR_PDCP_RbConfig_TransparentMode, v_NR_RLC_TestModeConfig_continue), f_NR_GetRadioBearerConfig_FullAM(p_PCell, v_DRBj_SCell, -, cs_NR_PDCP_RbConfig_TransparentMode, v_NR_RLC_TestModeConfig_continue)}; var template (value) NR_RadioBearerList_Type v_SS_Drb_ConfigList := { f_NR_GetRadioBearerConfig_FullAM(p_PCell, v_DRBj_SCell, -, cs_NR_PDCP_RbConfig_TransparentMode, -, cs_SDAP_Configuration_None )}; //@sic R5s220944 sic@ var SubFrameTiming_Type v_TimingUL; //@siclog Steps 1-2 siclog@ //The SS transmits an RRCReconfiguration message to add SCell for the UE f_NR5GC_AddMod_1SCell( p_PCell, p_SCell,-, true, -, false); //CA UL enabled //@sic R5-223424 sic@ //Reconfigure SCell in SS //The UE transmits an RRCReconfigurationComplete message //@siclog Steps 2A siclog@ //The SS transmits a SCell Activation/Deactivation MAC-CE to activate SCell (NR Cell 3). f_SS_CellConfig_ActivateSCell_CA(p_PCell); // @sic R5s220669 R5s220818 sic@ // Dummy DRB configured on SS f_NR_SS_CommonRadioBearerConfig(p_PCell, v_SS_Drb_ConfigList, cs_TimingInfo_Now); //@siclog Steps 3-4 siclog@ //The SS transmits an RRCReconfiguration message to establish DRB #j for the UE v_RadioBearerConfig := cs_38508_RadioBearerConfigDef(-,{cs_38508_NRDRB_ToAddMod( //@sic R5s220944 sic@ f_NR_GetSDAP_InDRBList(v_DRBj_PCell), v_DRBj_PCell, cs_NR_PDCP_Config_Split(cs_38508_PDCP_Config_DRB, f_NR_LogicalChannelId_DRB(v_DRBj_PCell), tsc_NR_CellGroupId_MCG, true))}, -, -); //Configure RLC-BearerConfig-1 v_UL_AM_RLC := f_NR_UL_AM_RLC(f_NR_CellInfo_GetIsFR1(p_PCell)); v_UL_AM_RLC.maxRetxThreshold := t32; v_RLC_Bearer_Config[0] := cs_38508_RLC_BearerConfig_DRB_LCID( f_NR_LogicalChannelId_DRB(v_DRBj_PCell), v_DRBj_PCell, cs_38508_RLC_Config_AM_DRB(v_UL_AM_RLC, f_NR_DL_AM_RLC(f_NR_CellInfo_GetIsFR1(p_PCell))), cs_38508_LogicalChannelConfig_ASC(f_NR_LogicalChannelId_DRB(v_DRBj_PCell),-,-,-,-,-,{0})) ; //Configure RLC-BearerConfig21 v_UL_AM_RLC.maxRetxThreshold := t1; v_RLC_Bearer_Config[1] := cs_38508_RLC_BearerConfig_DRB_LCID( f_NR_LogicalChannelId_DRB(v_DRBj_SCell), v_DRBj_PCell, cs_38508_RLC_Config_AM_DRB(v_UL_AM_RLC, f_NR_DL_AM_RLC(f_NR_CellInfo_GetIsFR1(p_PCell))), cs_38508_LogicalChannelConfig_ASC(f_NR_LogicalChannelId_DRB(v_DRBj_SCell),-,-,-,-,-,{1})) ; v_MCG_Config := cs_NR_CellGroupConfigDef(tsc_NR_CellGroupId_MCG, v_RLC_Bearer_Config,omit, omit, omit, omit,omit, omit); v_RRCReconfiguration_v1530 := cs_NR_RRCReconfiguration_v1530_IEs(bit2oct(encvalue(v_MCG_Config))); f_NR_SendRRCReconfiguration(p_PCell,-,v_RadioBearerConfig,v_RRCReconfiguration_v1530, -, -, -, -, -, crs_MacBearerRouting(p_PCell)); //@sic R5-213516 sic@ //@siclog "Step 5-6" siclog@ // void @sic R5s220944 sic@ //@siclog "Step 7" siclog@ //The SS stops sending MCG RLC acknowledgments on PCell (NR Cell 1) and SCell (NR Cell 3) f_NR_SS_CommonRadioBearerConfig(p_PCell, v_SS_Drb_ConfigList_NotAck); //@sic R5s220944 sic@ //@siclog "Step 8A" siclog@ //@sic R5-223424 sic@ //SS transmits a periodic UL Grant on SCell (as PUCCH is not configured in the SCell), allowing the UE to loop back the PDCP PDU on SCell. v_NR_ResourceAllocation := f_NR_ResourceAllocation_UplinkGrant(4096, v_NR_UplinkBWP); //@sic R5s220458 sic@ v_TimingUL := f_NR_GetNextSendOccasion_UL(p_SCell); //@sic R5s220944 sic@ f_NR_ULGrantConfiguration_StartPeriodicCyclicGrant(p_SCell, cs_TimingInfo_NR(v_TimingUL), v_NR_ResourceAllocation, 20, 6); //@sic R5s221101 sic@ //@siclog "Step 8" siclog@ //The SS transmits one PDCP PDU to the UE on DRB #j. DRB.send(cas_NR_DRB_COMMON_REQ_PDCP_PDUList(p_PCell, v_DRBj_PCell, cs_TimingInfo_Now, {cs_NR_PDCP_PDU_18B(0, v_PDCP_Data)}, -, -, crs_MacBearerRouting(p_PCell))); interleave{ //@siclog "Step 9" siclog@ //The UE loops back the PDCP PDU on PCell on DRB #j [] DRB.receive(car_NR_DRB_COMMON_IND_PDCP_PDUList_DC(p_PCell,v_DRBj_PCell,cs_RlcBearerRouting_NR(p_PCell), -, {cs_NR_PDCP_PDU_18B(0, v_PDCP_Data)}, crs_MacBearerRouting(p_PCell))){} //@siclog "Step 9A" siclog@ //The UE loops back the PDCP PDU on SCell on DRB #j [] DRB.receive(car_NR_DRB_COMMON_IND_PDCP_PDUList_DC(p_PCell, v_DRBj_SCell, cs_RlcBearerRouting_NR(p_PCell), -,{cs_NR_PDCP_PDU_18B(0, v_PDCP_Data)}, crs_MacBearerRouting(p_SCell))){} //@sic R5s220944 sic@ } //@siclog "Step 10" siclog@ //Check: Does the UE transmit a FailureInformation message with failureType set to rlc-failure? /* @sic R5s201387 BASELINE MOVING 2020 sic@ */ SRB.receive(car_NR_SRB_RrcPdu_IND(p_PCell, tsc_NR_RbId_SRB1, cr_FailureInformation(tsc_NR_CellGroupId_MCG, f_NR_LogicalChannelId_DRB(v_DRBj_SCell),rlc_failure), -, crs_MacBearerRouting(p_PCell))); //@sic R5-225281 R5s221101 sic@ f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 10"); //@siclog "Step 10A" siclog@ f_NR_SS_CommonRadioBearerConfig(p_PCell, v_SS_Drb_ConfigList_Ack); //@sic R5s220944 sic@ //@siclog "Steps 11 - 12" siclog@ //The SS transmits an RRCReconfigurationmessage containing an sCellToReleaseList with SCell NR Cell 3. //Check: Does the UE transmit an RRCReconfigurationComplete message? f_NR5GC_Rel_1SCell(p_PCell, p_SCell); //@sic R5-214617 sic@ }

After Change:

function f_TC_8_1_5_7_1_X_NR5GC_TestBody(NR_CellId_Type p_PCell, NR_CellId_Type p_SCell) runs on NR5GC_PTC { var template (value) CellGroupConfig v_MCG_Config; var template (value) UL_AM_RLC v_UL_AM_RLC; var template (value) RadioBearerConfig v_RadioBearerConfig; var template (value) RRCReconfiguration_v1530_IEs v_RRCReconfiguration_v1530; var template (omit) NR_RLC_BearerToAddModList_Type v_RLC_Bearer_Config; var IntegerList_Type v_ListOfDRBs := f_NR_GetListOfDRBs_ForFirstPDUSession(1); var octetstring v_PDCP_Data := f_IPv4IPv6_IcmpEchoReply(f_LoopbackModeB_IP_Address_NW()); var template (value) NR_RLC_TestModeConfig_Type v_NR_RLC_TestModeConfig := {Info := {AckProhibit := Prohibit}}; //@sic R5s220944 sic@ var template (value) NR_RLC_TestModeConfig_Type v_NR_RLC_TestModeConfig_continue := {Info := {AckProhibit := Continue}}; //@sic R5s220944 sic@ var integer v_DRBj_PCell := f_NR_GetDefaultDRB_ForFirstPDUSession(); var integer v_DRBj_SCell := v_ListOfDRBs[0]; //DRB is used ONLY for SS configuration! @sic R5-223444 sic@ var NR_UplinkBWP_Type v_NR_UplinkBWP := f_NR_CellInfo_GetUplinkBWP(p_SCell, tsc_NR_BWP_Id); var NR_ResourceAllocation_Type v_NR_ResourceAllocation; var template (value) NR_RadioBearerList_Type v_SS_Drb_ConfigList_NotAck := { f_NR_GetRadioBearerConfig_FullAM(p_PCell, v_DRBj_PCell, -, cs_NR_PDCP_RbConfig_TransparentMode, v_NR_RLC_TestModeConfig), f_NR_GetRadioBearerConfig_FullAM(p_PCell, v_DRBj_SCell, -, cs_NR_PDCP_RbConfig_TransparentMode, v_NR_RLC_TestModeConfig)}; var template (value) NR_RadioBearerList_Type v_SS_Drb_ConfigList_Ack := { f_NR_GetRadioBearerConfig_FullAM(p_PCell, v_DRBj_PCell, -, cs_NR_PDCP_RbConfig_TransparentMode, v_NR_RLC_TestModeConfig_continue), f_NR_GetRadioBearerConfig_FullAM(p_PCell, v_DRBj_SCell, -, cs_NR_PDCP_RbConfig_TransparentMode, v_NR_RLC_TestModeConfig_continue)}; var template (value) NR_RadioBearerList_Type v_SS_Drb_ConfigList := { f_NR_GetRadioBearerConfig_FullAM(p_PCell, v_DRBj_SCell, -, cs_NR_PDCP_RbConfig_TransparentMode, -, cs_SDAP_Configuration_None )}; //@sic R5s220944 sic@ var SubFrameTiming_Type v_TimingUL; var SubFrameTiming_Type v_TimingUL2; //@siclog Steps 1-2 siclog@ //The SS transmits an RRCReconfiguration message to add SCell for the UE f_NR5GC_AddMod_1SCell( p_PCell, p_SCell,-, true, -, false); //CA UL enabled //@sic R5-223424 sic@ //Reconfigure SCell in SS //The UE transmits an RRCReconfigurationComplete message //@siclog Steps 2A siclog@ //The SS transmits a SCell Activation/Deactivation MAC-CE to activate SCell (NR Cell 3). f_SS_CellConfig_ActivateSCell_CA(p_PCell); // @sic R5s220669 R5s220818 sic@ // Dummy DRB configured on SS f_NR_SS_CommonRadioBearerConfig(p_PCell, v_SS_Drb_ConfigList, cs_TimingInfo_Now); //@siclog Steps 3-4 siclog@ //The SS transmits an RRCReconfiguration message to establish DRB #j for the UE v_RadioBearerConfig := cs_38508_RadioBearerConfigDef(-,{cs_38508_NRDRB_ToAddMod( //@sic R5s220944 sic@ f_NR_GetSDAP_InDRBList(v_DRBj_PCell), v_DRBj_PCell, cs_NR_PDCP_Config_Split(cs_38508_PDCP_Config_DRB, f_NR_LogicalChannelId_DRB(v_DRBj_PCell), tsc_NR_CellGroupId_MCG, true))}, -, -); //Configure RLC-BearerConfig-1 v_UL_AM_RLC := f_NR_UL_AM_RLC(f_NR_CellInfo_GetIsFR1(p_PCell)); v_UL_AM_RLC.maxRetxThreshold := t32; v_RLC_Bearer_Config[0] := cs_38508_RLC_BearerConfig_DRB_LCID( f_NR_LogicalChannelId_DRB(v_DRBj_PCell), v_DRBj_PCell, cs_38508_RLC_Config_AM_DRB(v_UL_AM_RLC, f_NR_DL_AM_RLC(f_NR_CellInfo_GetIsFR1(p_PCell))), cs_38508_LogicalChannelConfig_ASC(f_NR_LogicalChannelId_DRB(v_DRBj_PCell),-,-,-,-,-,{0})) ; //Configure RLC-BearerConfig21 v_UL_AM_RLC.maxRetxThreshold := t1; v_RLC_Bearer_Config[1] := cs_38508_RLC_BearerConfig_DRB_LCID( f_NR_LogicalChannelId_DRB(v_DRBj_SCell), v_DRBj_PCell, cs_38508_RLC_Config_AM_DRB(v_UL_AM_RLC, f_NR_DL_AM_RLC(f_NR_CellInfo_GetIsFR1(p_PCell))), cs_38508_LogicalChannelConfig_ASC(f_NR_LogicalChannelId_DRB(v_DRBj_SCell),-,-,-,-,-,{1})) ; v_MCG_Config := cs_NR_CellGroupConfigDef(tsc_NR_CellGroupId_MCG, v_RLC_Bearer_Config,omit, omit, omit, omit,omit, omit); v_RRCReconfiguration_v1530 := cs_NR_RRCReconfiguration_v1530_IEs(bit2oct(encvalue(v_MCG_Config))); f_NR_SendRRCReconfiguration(p_PCell,-,v_RadioBearerConfig,v_RRCReconfiguration_v1530, -, -, -, -, -, crs_MacBearerRouting(p_PCell)); //@sic R5-213516 sic@ //@siclog "Step 5-6" siclog@ // void @sic R5s220944 sic@ //@siclog "Step 7" siclog@ //The SS stops sending MCG RLC acknowledgments on PCell (NR Cell 1) and SCell (NR Cell 3) f_NR_SS_CommonRadioBearerConfig(p_PCell, v_SS_Drb_ConfigList_NotAck); //@sic R5s220944 sic@ //@siclog "Step 8A" siclog@ //@sic R5-223424 sic@ //SS transmits a periodic UL Grant on SCell (as PUCCH is not configured in the SCell), allowing the UE to loop back the PDCP PDU on SCell. v_NR_ResourceAllocation := f_NR_ResourceAllocation_UplinkGrant(4096, v_NR_UplinkBWP); //@sic R5s220458 sic@ v_TimingUL := f_NR_GetNextSendOccasion_UL(p_SCell); //@sic R5s220944 sic@ v_TimingUL2 := f_SubFrameTiming_AddMilliSeconds(v_TimingUL, 40); f_NR_ULGrantConfiguration_StartPeriodicCyclicGrant(p_SCell, cs_TimingInfo_NR(v_TimingUL2), v_NR_ResourceAllocation, 20, 6); //@sic R5s221101 sic@ //@siclog "Step 8" siclog@ //The SS transmits one PDCP PDU to the UE on DRB #j. DRB.send(cas_NR_DRB_COMMON_REQ_PDCP_PDUList(p_PCell, v_DRBj_PCell, cs_TimingInfo_NR(v_TimingUL), {cs_NR_PDCP_PDU_18B(0, v_PDCP_Data)}, -, -, crs_MacBearerRouting(p_PCell))); interleave{ //@siclog "Step 9" siclog@ //The UE loops back the PDCP PDU on PCell on DRB #j [] DRB.receive(car_NR_DRB_COMMON_IND_PDCP_PDUList_DC(p_PCell,v_DRBj_PCell,cs_RlcBearerRouting_NR(p_PCell), -, {cs_NR_PDCP_PDU_18B(0, v_PDCP_Data)}, crs_MacBearerRouting(p_PCell))){} //@siclog "Step 9A" siclog@ //The UE loops back the PDCP PDU on SCell on DRB #j [] DRB.receive(car_NR_DRB_COMMON_IND_PDCP_PDUList_DC(p_PCell, v_DRBj_SCell, cs_RlcBearerRouting_NR(p_PCell), -,{cs_NR_PDCP_PDU_18B(0, v_PDCP_Data)}, crs_MacBearerRouting(p_SCell))){} //@sic R5s220944 sic@ } //@siclog "Step 10" siclog@ //Check: Does the UE transmit a FailureInformation message with failureType set to rlc-failure? /* @sic R5s201387 BASELINE MOVING 2020 sic@ */ SRB.receive(car_NR_SRB_RrcPdu_IND(p_PCell, tsc_NR_RbId_SRB1, cr_FailureInformation(tsc_NR_CellGroupId_MCG, f_NR_LogicalChannelId_DRB(v_DRBj_SCell),rlc_failure), -, crs_MacBearerRouting(p_PCell))); //@sic R5-225281 R5s221101 sic@ f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 10"); //@siclog "Step 10A" siclog@ f_NR_SS_CommonRadioBearerConfig(p_PCell, v_SS_Drb_ConfigList_Ack); //@sic R5s220944 sic@ //@siclog "Steps 11 - 12" siclog@ //The SS transmits an RRCReconfigurationmessage containing an sCellToReleaseList with SCell NR Cell 3. //Check: Does the UE transmit an RRCReconfigurationComplete message? f_NR5GC_Rel_1SCell(p_PCell, p_SCell); //@sic R5-214617 sic@ }