3GPP TSG-RAN5 Meeting #2024-TTCN email	R5s240381

# Online, , 8th Dec 2023 - 31st Dec 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 3449 | rev | - | Current version: | 17.10.0 |  |
| For HELPon using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed changeaffects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to NR5GC testcase 7.1.1.4.1.x and 7.1.1.6.1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Starpoint, TDIA |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2024-05-28 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | According to TS38.523-1, the configuration for testcase 7.1.1.4.1.x and 7.1.1.6.1 are not correct. |  |  |  |  |  |  |  |  |
| Summary of change: |  | 1 Use correct configuration for the test cases 2 Correct the notes |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may fail the test cases. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.1.1.4.1.1.NR5GC, 7.1.1.4.1.3.NR5GC, 7.1.1.4.1.4.NR5GC, 7.1.1.4.1.5.NR5GC, 7.1.1.6.1.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

2	Corrections required	3

2.1	Change 1	3

2.2	Change 2	3

# Overview

This document lists all the changes needed to correct issues in the ATS iwd-TTCN3-B2022-09_D24wk12 related to the title of this CR.

Contact:	Ting Gao
	gaoting@starpointcomm.com

# Corrections required

## Change 1

| Function name | Template cs_NR_PDSCH_TimeDomainResourceAllocationList_71141x |
| --- | --- |
| Reason for change | According to TS38.523-1Table 7.1.1.4.1.0-1, the configuration and notes are not correct. |
| Summary of change | 1 Use Table 7.1.1.4.1.0-1 for 71141x 2 Correct the startSymbolAndLength |
| TTCN module | MAC_TC_Common_NR.ttcn |
| MCC160 Comment | Accepted |

Before Change:

template (value) PDSCH_TimeDomainResourceAllocationList cs_NR_PDSCH_TimeDomainResourceAllocationList_71141x := { /* @status APPROVED (ENDC, NR5GC) */ // Acc to 38.523-1 Table 7.1.1.6.1.3.3-5E: PDSCH-TimeDomainResourceAllocationList //@sic R5-190677 sic@ cs_NR_PDSCH_TimeDomainResourceAllocation(omit, typeA, 53), // S=2, L=12 cs_NR_PDSCH_TimeDomainResourceAllocation(omit, typeA, 53) // S=2, L=7 };

After Change:

template (value) PDSCH_TimeDomainResourceAllocationList cs_NR_PDSCH_TimeDomainResourceAllocationList_71141x := { /* @status APPROVED (ENDC, NR5GC) */ // Acc to 38.523-1 7.1.1.4.1.0-1: PDSCH-TimeDomainResourceAllocationList //@sic R5-190677 sic@ cs_NR_PDSCH_TimeDomainResourceAllocation(omit, typeA, 53), // S=2, L=12 cs_NR_PDSCH_TimeDomainResourceAllocation(omit, typeA, 86) // S=2, L=7 };

## Change 2

| Function name | Template cs_NR_PDSCH_TimeDomainResourceAllocationList_71161 |
| --- | --- |
| Reason for change | According to TS38.523-1 Table 7.1.1.6.1.3.3-5E, the configuration and notes are not correct. |
| Summary of change | 1 Use Table 7.1.1.6.1.3.3-5E for 71161 2 Correct the startSymbolAndLength |
| TTCN module | MAC_TC_Common_NR.ttcn |
| MCC160 Comment | - Accepted. - Not applicable |

Before Change:

template (value) PDSCH_TimeDomainResourceAllocationList cs_NR_PDSCH_TimeDomainResourceAllocationList_71161 := { /* @status APPROVED (ENDC, NR5GC) */ // Acc to 38.523-1 Table 7.1.1.4.1.0-1: PDSCH-TimeDomainResourceAllocationList //@sic R5-297339 sic@ cs_NR_PDSCH_TimeDomainResourceAllocation(0, typeA, 53), // S=2, L=12 cs_NR_PDSCH_TimeDomainResourceAllocation(4, typeA, 53) // S=2, L=12 };

After Change:

template (value) PDSCH_TimeDomainResourceAllocationList cs_NR_PDSCH_TimeDomainResourceAllocationList_71161 := { /* @status APPROVED (ENDC, NR5GC) */ // Acc to 38.523-1 Table 7.1.1.6.1.3.3-5E: PDSCH-TimeDomainResourceAllocationList //@sic R5-297339 sic@ cs_NR_PDSCH_TimeDomainResourceAllocation(0, typeA, 53), // S=2, L=12 cs_NR_PDSCH_TimeDomainResourceAllocation(4, typeA, 53) // S=2, L=12 };