3GPP TSG-RAN5 Meeting #2024-TTCN email	R5s240387

# Online,  , 8th Dec 2023 - 31st Dec 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.579-5 | CR | 0113 | rev | - | Current version: | 17.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections to MCX common functions |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI14_Test, MCImp-UEConTest |  |  |  |  |  | Date: |  |  | 2024-06-04 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | 1, In current implementation of f_XPATH_ExpressionForAnyNamespaces(), Escape Backslash (\) is needed to escape special regex characters “[“ and “]” 2, As RFC 3830 clause 5.2: "Calculate the MAC/signature over the entire MIKEY message, except the MAC/Signature field, and add the MAC/signature in the field In current TTCN implementation of f_MIKEY_SAKKE_Signature_Validate, the code ONLY removes the signature string field (MIKEY_Message.sign.s), where it should actually remove the whole sign (including sType, and sLen fields) |  |  |  |  |  |  |  |  |
| Summary of change: |  | 1, Added Escape Backslash for special regex characters 2, Added code to remove the extra 2bytes (16 bits), i.e sType (B4_Type) and sLen (B12_Type) |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE will fail the test case |  |  |  |  |  |  |  |  |
| Clauses affected: |  | f_XPATH_ExpressionForAnyNamespaces, f_MIKEY_SAKKE_Signature_Validate |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

2	Corrections required	3

2.1	Change 1	3

2.2	Change 2	4

# Overview

This document lists all the changes needed to correct issues in the ATS iwd-TTCN3-B2022-09_D24wk12 related to the title of this CR.

Contact:	Mohit Kanchan
	mohit.kanchan@keysight.com

# Corrections required

## Change 1

| Function name | f_XPATH_ExpressionForAnyNamespaces() |
| --- | --- |
| Reason for change | Escape Backslash (\) is needed to escape special regex characters “[“ and “]” |
| Summary of change | Added Escape Backslash for special regex characters |
| TTCN module | MCX_XML_Encryption.ttcn |
| MCC160 Comment | Rejected, as not needed According to the TTCN-3 standard ES 201 873-1, "[[]" and "[]]" are valid expressions. In annex B.1.5.1 it is stated: "An empty list and an empty negated list are not allowed. Therefore a closing square bracket "]" immediately following an opening square bracket "[" or a caret following the opening square bracket "[" and immediately followed by a closing square bracket "]" shall be processed as literal characters." => "[]]" should be allowed and match ']'. And further down in NOTE 1: "Embedded lists are not allowed. For example in pattern "[ab[r-z]]" the second "[" denotes a literal "[", the first "]" closes the list and the second "]" retains its literal value as no related opening bracket precedes it in the pattern. The pattern will match character strings containing two elements, with the first element equal to "a", "b", "[" or anything in the range "r"-"z" and the second character equal to "]"." => "[][" should be allowed and match '['. The proposed change is just an alternative way to express the same regular expression. Both notations are equivalent and shall be processable by any compliant TTCN-3 compiler. |

Before Change:

function f_XPATH_ExpressionForAnyNamespaces(charstring p_Expression) return charstring { var charstring v_String := regexp(p_Expression, "//(*)", 0); var CharStringList_Type v_ElementList; var charstring v_Element; var charstring v_Expression; var charstring v_IndexExpr; var charstring v_RegExp; var integer i; if (v_String == "") { v_Expression := p_Expression; } else { v_ElementList := f_StringSplit(v_String, {"/"}); for (i := 0; i < lengthof(v_ElementList); i := i + 1) { v_Element := v_ElementList[i]; v_RegExp := "(*)([[][0-9]+[]])"; v_IndexExpr := regexp(v_Element, v_RegExp, 1); if (v_IndexExpr != "") { v_Element := regexp(v_Element, v_RegExp, 0); } v_ElementList[i] := "*[local-name()='" & v_Element & "']" & v_IndexExpr; } v_Expression := "//" & f_StringJoin(v_ElementList, "/"); } return v_Expression; }

After Change:

function f_XPATH_ExpressionForAnyNamespaces(charstring p_Expression) return charstring { var charstring v_String := regexp(p_Expression, "//(*)", 0); var CharStringList_Type v_ElementList; var charstring v_Element; var charstring v_Expression; var charstring v_IndexExpr; var charstring v_RegExp; var integer i; if (v_String == "") { v_Expression := p_Expression; } else { v_ElementList := f_StringSplit(v_String, {"/"}); for (i := 0; i < lengthof(v_ElementList); i := i + 1) { v_Element := v_ElementList[i]; v_RegExp := "(*)([\[][0-9]+[\]])"; v_IndexExpr := regexp(v_Element, v_RegExp, 1); if (v_IndexExpr != "") { v_Element := regexp(v_Element, v_RegExp, 0); } v_ElementList[i] := "*[local-name()='" & v_Element & "']" & v_IndexExpr; } v_Expression := "//" & f_StringJoin(v_ElementList, "/"); } return v_Expression; }

## Change 2

| - Function name | f_MIKEY_SAKKE_Signature_Validate |
| --- | --- |
| Reason for change | As RFC 3830 clause 5.2: "Calculate the MAC/signature over the entire MIKEY message, except the MAC/Signature field, and add the MAC/signature in the field In current TTCN implementation, the code ONLY removes the signature string field (MIKEY_Message.sign.s), where it should actually remove the whole sign (including sType, and sLen fields) |
| Summary of change | Added code to remove the extra 2bytes (16 bits), i.e sType (B4_Type) and sLen (B12_Type) from the octetstring v_Message |
| TTCN module | MCX_MIKEY_Messages.ttcn |
| MCC160 Comment | Rejected According to RFC 3830 clause 5.2: "The process of generating a MIKEY message consists of the following steps: ... * As a last step (...), create and concatenate the MAC/signature payload without the MAC/signature field filled in (...). * Calculate the MAC/signature over the entire MIKEY message, except the MAC/Signature field, and add the MAC/signature in the field. ..." => RFC 3830 distinguishes "MAC/signature payload" and "MAC/Signature field" - the payload is included in the calculation of the signature, the field is not included. This is clarified in 33.180 E.1.2: "The signature shall then be calculated over the entire MIKEY message (including the S type field and Signature length field of the Signature payload) followed by concatenation of the signature to the end of the MIKEY message". sType and sLen need to be kept in the octetstring used for calculation of the signature. |

Before Change:

function f_MIKEY_SAKKE_Signature_Validate(MCX_SecurityParameters_Type p_SecurityParameters, MIKEY_Message p_MIKEY_Message) return boolean { var MIKEY_Message v_MIKEY_Message := p_MIKEY_Message; var charstring v_IDRiURI := f_MIKEY_Message_GetIDRIdentity(v_MIKEY_Message, tsc_MIKEY_IDR_Role_IDRi); // Initiator's URI var octetstring v_UID := f_MCX_SecurityParameters_GetUID(p_SecurityParameters, v_IDRiURI); var octetstring v_Signature := p_MIKEY_Message.sign.s; var octetstring v_Message; v_MIKEY_Message.sign.s := ''O; /* RFC 3830 clause 5.2: "Calculate the MAC/signature over the entire MIKEY message, except the MAC/Signature field, and add the MAC/signature in the field" According to clause 6.5 the MAC/Signature field is the field of the Signature payload. => generation of the signature includes the Signature payload's type and length field */ v_Message := bit2oct(encvalue(v_MIKEY_Message)); return fx_ECCSI_VerifySignature(v_Message, v_Signature, p_SecurityParameters.ECCSI.KPAK, v_UID); }

After Change:

function f_MIKEY_SAKKE_Signature_Validate(MCX_SecurityParameters_Type p_SecurityParameters, MIKEY_Message p_MIKEY_Message) return boolean { var MIKEY_Message v_MIKEY_Message := p_MIKEY_Message; var charstring v_IDRiURI := f_MIKEY_Message_GetIDRIdentity(v_MIKEY_Message, tsc_MIKEY_IDR_Role_IDRi); // Initiator's URI var octetstring v_UID := f_MCX_SecurityParameters_GetUID(p_SecurityParameters, v_IDRiURI); var octetstring v_Signature := p_MIKEY_Message.sign.s; var octetstring v_Message; v_MIKEY_Message.sign.s := ''O; /* RFC 3830 clause 5.2: "Calculate the MAC/signature over the entire MIKEY message, except the MAC/Signature field, and add the MAC/signature in the field" According to clause 6.5 the MAC/Signature field is the field of the Signature payload. => generation of the signature includes the Signature payload's type and length field */ v_Message := bit2oct(encvalue(v_MIKEY_Message)); if (ispresent(v_MIKEY_Message.sign)) { v_Message := substr(v_Message, 0, lengthof(v_Message)-2); } return fx_ECCSI_VerifySignature(v_Message, v_Signature, p_SecurityParameters.ECCSI.KPAK, v_UID); }