3GPP TSG-RAN5 Meeting #2024-TTCN email	R5s240414

# Online, , 8th Dec 2023 - 31st Dec 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 3460 | rev | - | Current version: | 17.10.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction for Test case 8.1.4.1.7.X |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ANRITSU LTD |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2024-06-09 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | TS 38.523-3 cl. 7.3.5.3.4, step 9 enquires about PDCP count but only SRB count is received but not DRB. This is because that DRB is configured but not yet activated. As a result of having receieved only count for SRB, cipehing is not applied for DRB. For this reason, we have proposed to apply the ciphering on DRB otherwise this leads to test failure due to data not being looped back. Therefore, the 8.1.4.1.7.x test body is modified to reactivate security for all RBs after handover is complete to ensure that all added RBs are included in the process. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Modified f_TC_8_1_4_1_7_x_NR5GC_TestBody reactivate security for all RBs without key refresh after handover is complete to ensure that all configured RBs are included and ciphered properly. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may fail the test case |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.1.4.1.7.1, 8.1.4.1.7.2, 8.1.4.1.7.3.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

2	Corrections required	3

2.1	Change 1	3

# Overview

This document lists all the changes needed to correct issues in the ATS iwd-TTCN3-B2022-09_D24wk12 related to the title of this CR.

Contact:	Kalahasti, Narendra

Narendra.Kalahasti@anritsu.com

# Corrections required

## Change 1

| Function name | function f_TC_8_1_4_1_7_x_NR5GC_TestBody |
| --- | --- |
| Reason for change | TS 38.523-3 cl. 7.3.5.3.4, step 9 enquires about PDCP count but only SRB count is received but not DRB. This is because that DRB is configured but not yet activated. As a result of having receieved only count for SRB, cipehing is not applied for DRB. For this reason, we have proposed to apply the ciphering on DRB otherwise this leads to test failure due to data not being looped back. Therefore, the 8.1.4.1.7.x test body is modified to reactivate security for all RBs after handover is complete to ensure that all added RBs are included in the process. |
| Summary of change | Modified f_TC_8_1_4_1_7_x_NR5GC_TestBody reactivate security for all RBs without key refresh after handover is complete to ensure that all configured RBs are included and ciphered properly. |
| TTCN module | RRC_NR_CA_IntraNR_Handover_NR5GC.ttcn |
| MCC160 Comment | Accepted in principle but implemented differently. The PDCP count calculated in step 6 is returned and used in step 9 to avoid the security function getting PDCP count again with timing Now. See below. |

Before Change

function f_TC_8_1_4_1_7_x_NR5GC_TestBody(NR_CellId_Type p_SourcePCell, NR_CellId_Type p_SourceSCell, NR_CellId_Type p_TargetPCell, NR_CellId_Type p_TargetSCell) runs on NR5GC_PTC { var template (value) NR_CellPowerList_Type v_CellPowerList_AtT0 :={cs_NR_CellPower(p_SourcePCell, -88, -91), //@sic R5-237355 sic@ cs_NR_CellPower(p_SourceSCell, -88, -91), //@sic R5-237355 sic@ cs_NR_CellPower(p_TargetPCell, tsc_NR_NonSuitableOffCellSSS_EPRE, tsc_NR_NonSuitableOffCellSSS_EPRE), //@sic R5s210079 sic@ cs_NR_CellPower(p_TargetSCell, -88, -91) //@sic R5s210079, R5-237355 sic@ }; var template (value) NR_CellPowerList_Type v_CellPowerList_AtT1 :={cs_NR_CellPower(p_SourcePCell, tsc_NR_NonSuitableOffCellSSS_EPRE, tsc_NR_NonSuitableOffCellSSS_EPRE), cs_NR_CellPower(p_SourceSCell, -88, -91), //@sic R5-237355 sic@ cs_NR_CellPower(p_TargetPCell, -82, -82), //@sic R5s211333, R5-237355 sic@ cs_NR_CellPower(p_TargetSCell, -88, -91) //@sic R5-237355 sic@ }; var NR_CellId_Type v_SourcePCell; var NR_CellId_Type v_SourceSCell; var NR_CellId_Type v_TargetPCell; var integer v_DRBId; var PDUSessionInfoList_Type v_PDUSessionInfoList; //@siclog "Step 1" siclog@ //The SS changes Cell parameters according to the row "T0" in table 8.1.4.1.7.1.3.2-1/2. f_NR_SetCellPowerList(v_CellPowerList_AtT0); //@siclog "Steps 2-3" siclog@ //The SS transmits an RRCReconfiguration message including a sCellToAddModList to add NR Cell 3 as a SCell. //The UE transmits the RRCReconfigurationComplete message? f_NR5GC_AddMod_1SCell(p_SourcePCell, p_SourceSCell); // NR Cell1, NR Cell3/10 //@siclog "Steps 4-5" siclog@ //The SS transmits an RRCReconfiguration message including a reconfigurationWithSync for handover to NR Cell 3 and sCellToAddModList to add NR Cell 2 as a SCell. //Check: Does UE transmit the RRCReconfigurationComplete message? v_TargetPCell := p_SourceSCell; // NR Cell3/10 f_NR5GC_508RRC_IntraNR_CA_HO_InterCell(p_SourcePCell, p_SourceSCell, v_TargetPCell, p_TargetSCell, -, -, cs_MasterKeyUpdate(false, tsc_NR_38508_NextHopChainingCount));// NR Cell1, NR Cell3/10, NR Cell3/10, NR Cell2 @sic R5s201249 sic@ //@siclog "Step 5A" siclog@ //Check: Does the test result of generic test procedure in TS 38.508-1 Table 4.9.1-1A indicate that the UE is capable of exchanging IP data on DRB #n associated with the first PDU session on Cell2? f_NR5GC_CheckDataPath_CA(v_TargetPCell, p_TargetSCell, "Step 5A", true); //@sic R5-203665 sic@ //@siclog "Step 6" siclog@ //The SS changes Cell parameters according to the row "T1" in table 8.2.4.2.1.1.3.2-1/2. f_NR_SetCellPowerList(v_CellPowerList_AtT1); //@siclog "Steps 7-8" siclog@ //The SS transmits an RRCReconfiguration message including a reconfigurationWithSync to change PCell to NR Cell 23 and including sCellToReleaseList with a sCellIndex of NR Cell 2. //Check: Does the UE transmit the RRCReconfigurationComplete message? v_SourcePCell := p_SourceSCell; // NR Cell3/10 v_SourceSCell := p_TargetSCell; // NR Cell2 f_NR5GC_508RRC_IntraNR_CA_HO_InterCell(v_SourcePCell, v_SourceSCell, p_TargetPCell, omit, -, -, cs_MasterKeyUpdate(false, tsc_NR_38508_NextHopChainingCount)); // NR Cell3/10, NR Cell2, NR Cell23/30 @sic R5s201249 sic@ //@siclog "Step 9" siclog@ //Check: Does the test result of generic test procedure in TS 38.508-1 subclause 4.9.1 indicate that the UE is capable of exchanging IP data on DRB #n associated with the first PDU session on Cell23/30? v_DRBId := f_NR_GetDefaultDRB_ForFirstPDUSession(); v_PDUSessionInfoList := f_NR5GC_MobileInfo_GetSessionInfoList(); f_NR5GC_CheckDataPath(p_TargetPCell, //NR Cell23/30 f_NR5GC_GetPdnIndex(v_PDUSessionInfoList[0]), dedicatedBearer, v_DRBId, "Step 9"); }

After Change

function f_TC_8_1_4_1_7_x_NR5GC_TestBody(NR_CellId_Type p_SourcePCell, NR_CellId_Type p_SourceSCell, NR_CellId_Type p_TargetPCell, NR_CellId_Type p_TargetSCell) runs on NR5GC_PTC { var template (value) NR_CellPowerList_Type v_CellPowerList_AtT0 :={cs_NR_CellPower(p_SourcePCell, -88, -91), //@sic R5-237355 sic@ cs_NR_CellPower(p_SourceSCell, -88, -91), //@sic R5-237355 sic@ cs_NR_CellPower(p_TargetPCell, tsc_NR_NonSuitableOffCellSSS_EPRE, tsc_NR_NonSuitableOffCellSSS_EPRE), //@sic R5s210079 sic@ cs_NR_CellPower(p_TargetSCell, -88, -91) //@sic R5s210079, R5-237355 sic@ }; var template (value) NR_CellPowerList_Type v_CellPowerList_AtT1 :={cs_NR_CellPower(p_SourcePCell, tsc_NR_NonSuitableOffCellSSS_EPRE, tsc_NR_NonSuitableOffCellSSS_EPRE), cs_NR_CellPower(p_SourceSCell, -88, -91), //@sic R5-237355 sic@ cs_NR_CellPower(p_TargetPCell, -82, -82), //@sic R5s211333, R5-237355 sic@ cs_NR_CellPower(p_TargetSCell, -88, -91) //@sic R5-237355 sic@ }; var NR_CellId_Type v_SourcePCell; var NR_CellId_Type v_SourceSCell; var NR_CellId_Type v_TargetPCell; var integer v_DRBId; var PDUSessionInfoList_Type v_PDUSessionInfoList; //@siclog "Step 1" siclog@ //The SS changes Cell parameters according to the row "T0" in table 8.1.4.1.7.1.3.2-1/2. f_NR_SetCellPowerList(v_CellPowerList_AtT0); //@siclog "Steps 2-3" siclog@ //The SS transmits an RRCReconfiguration message including a sCellToAddModList to add NR Cell 3 as a SCell. //The UE transmits the RRCReconfigurationComplete message? f_NR5GC_AddMod_1SCell(p_SourcePCell, p_SourceSCell); // NR Cell1, NR Cell3/10 //@siclog "Steps 4-5" siclog@ //The SS transmits an RRCReconfiguration message including a reconfigurationWithSync for handover to NR Cell 3 and sCellToAddModList to add NR Cell 2 as a SCell. //Check: Does UE transmit the RRCReconfigurationComplete message? v_TargetPCell := p_SourceSCell; // NR Cell3/10 f_NR5GC_508RRC_IntraNR_CA_HO_InterCell(p_SourcePCell, p_SourceSCell, v_TargetPCell, p_TargetSCell, -, -, cs_MasterKeyUpdate(false, tsc_NR_38508_NextHopChainingCount));// NR Cell1, NR Cell3/10, NR Cell3/10, NR Cell2 @sic R5s201249 sic@ f_NR_SS_AS_ActivateSecurity_NoRefresh(v_TargetPCell,-); //@siclog "Step 5A" siclog@ //Check: Does the test result of generic test procedure in TS 38.508-1 Table 4.9.1-1A indicate that the UE is capable of exchanging IP data on DRB #n associated with the first PDU session on Cell2? f_NR5GC_CheckDataPath_CA(v_TargetPCell, p_TargetSCell, "Step 5A", true); //@sic R5-203665 sic@ //@siclog "Step 6" siclog@ //The SS changes Cell parameters according to the row "T1" in table 8.2.4.2.1.1.3.2-1/2. f_NR_SetCellPowerList(v_CellPowerList_AtT1); //@siclog "Steps 7-8" siclog@ //The SS transmits an RRCReconfiguration message including a reconfigurationWithSync to change PCell to NR Cell 23 and including sCellToReleaseList with a sCellIndex of NR Cell 2. //Check: Does the UE transmit the RRCReconfigurationComplete message? v_SourcePCell := p_SourceSCell; // NR Cell3/10 v_SourceSCell := p_TargetSCell; // NR Cell2 f_NR5GC_508RRC_IntraNR_CA_HO_InterCell(v_SourcePCell, v_SourceSCell, p_TargetPCell, omit, -, -, cs_MasterKeyUpdate(false, tsc_NR_38508_NextHopChainingCount)); // NR Cell3/10, NR Cell2, NR Cell23/30 @sic R5s201249 sic@ //@siclog "Step 9" siclog@ //Check: Does the test result of generic test procedure in TS 38.508-1 subclause 4.9.1 indicate that the UE is capable of exchanging IP data on DRB #n associated with the first PDU session on Cell23/30? v_DRBId := f_NR_GetDefaultDRB_ForFirstPDUSession(); v_PDUSessionInfoList := f_NR5GC_MobileInfo_GetSessionInfoList(); f_NR5GC_CheckDataPath(p_TargetPCell, //NR Cell23/30 f_NR5GC_GetPdnIndex(v_PDUSessionInfoList[0]), dedicatedBearer, v_DRBId, "Step 9"); }

MCC TF160 proposed implementation

function f_NR_SS_PdcpCount_Handover(NR_CellId_Type                              p_SourceCellId,

NR_CellId_Type                              p_TargetCellId,

template( value) NR_RadioBearerList_Type    p_RadioBearerList,

boolean                                     p_RBConfig_KeyChange := true,

template (value) TimingInfo_Type            p_TimingInfo := cs_TimingInfo_Now,

boolean                                     p_IncrementSRB1 := true //@sic R5-216260 sic@

) runs on NR_BASE_PTC return NR_PdcpCountInfoList_Type

{

var NR_PdcpCountInfoList_Type v_NR_PdcpCountInfoList := f_NR_GetPdcpCountInfoListIntraNR_HO(p_SourceCellId, p_RadioBearerList, p_RBConfig_KeyChange, p_IncrementSRB1);

f_NR_SS_PdcpCount_Set(p_TargetCellId, v_NR_PdcpCountInfoList, p_TimingInfo);

return v_NR_PdcpCountInfoList;

}

function f_NR_SS_AS_ActivateSecurity_NoRefresh( NR_CellId_Type p_NR_CellId,

template (value) TimingInfo_Type p_TimingInfo := cs_TimingInfo_Now,

template (omit) NR_PdcpCountInfoList_Type p_PdcpCountInfoList:= omit

) runs on NR_BASE_PTC

{

var NR_SecurityParams_Type v_RRC_SecurityParams := f_NR_Security_Get();

v_RRC_SecurityParams.AS_Ciphering.ActTimeList := f_NR_RRC_CipherActTime_GetCurrent(p_NR_CellId, false, -, p_PdcpCountInfoList);

f_NR_SS_RRC_EnableIntProt_CiphULandDL(p_NR_CellId,

v_RRC_SecurityParams.AS_Integrity,

v_RRC_SecurityParams.AS_Ciphering,

p_TimingInfo);

f_NR_Security_Set(v_RRC_SecurityParams);

}

function f_NR5GC_508RRC_IntraNR_CA_HO_InterCell_Common(NR_CellId_Type                               p_SourcePCellId,

NR_CellId_Type                               p_SourceSCellId,

NR_CellId_Type                               p_TargetPCellId,

template (omit) NR_CellId_Type                p_TargetSCellId,

template (omit) CellGroupConfig               p_SCGConfig_SCell,

RNTI_Value                                   p_RNTI_Value :=  tsc_C_RNTI_Value3,

boolean                                      p_RBConfig_KeyChange := true,

template (value) DL_DCCH_Message             p_RRCReconfiguration,

template (value) NR_RachProcedureConfig_Type p_RachProcedureConfig,

template (value) NR_DRBInfoList_Type         p_DrbInfoList,

template (value) UL_GrantConfig_Type         p_UL_GrantConfig_SourceCell := cs_UL_GrantConfig_OnSR,

template (value) UL_GrantConfig_Type         p_UL_GrantConfig_TargetCell := cs_UL_GrantConfig_OnSR,

template (present) UL_DCCH_Message           p_RRCConnReconfigComplete := cr_38508_RRCReconfigurationComplete,

NextHopChainingCount                         p_NCC := tsc_NR_38508_NextHopChainingCount,

template (omit) ReconfigurationWithSync.rach_ConfigDedicated p_RACH_ConfigDedicated := omit

) runs on NR_BASE_PTC

{ //@sic R5s201249: added p_SCGConfig_SCell R5-213516: added MacBearerRouting sic@

var SubFrameTiming_Type v_Timing;

var SubFrameTiming_Type v_Timing1;

var SubFrameTiming_Type v_Timing2;

var template (value) NR_RadioBearerList_Type v_RadioBearerList := f_NR5GC_GetRadioBearerList(p_DrbInfoList);

var IntegerList_Type v_DrbIdList:= f_NR_GetActiveDRBs(p_SourcePCellId);

var template (value) TimingInfo_Type v_TimingInfo;  //@sic R5s201387 sic@

var NR_PdcpCountInfoList_Type v_PdcpCountInfoList := {}; //@sic R5s220816 sic@

v_Timing := f_NR_GetNextSendOccasion_DL(p_SourcePCellId, 700); // 700 ms // Time for schedule of DL RRC message

v_Timing1 := f_SubFrameTiming_AddMilliSeconds (v_Timing, 10);  // time for Target SCell-> PCell configuration SS configuration

v_Timing2 := f_SubFrameTiming_AddMilliSeconds (v_Timing, 20);  // time for Target PCell SS configuration

// 1. Source PCell: Stop periodic TA.

// NOTE 1: Unless explicitly specified UL grant configuration keeps configured as per default at the source cell.

f_NR_ULGrantConfiguration_Common(p_SourcePCellId, cs_TimingInfo_NR(v_Timing), -, cs_NR_UplinkTimeAlignment_Stop, p_UL_GrantConfig_SourceCell);

// 2. Target PCell: Configure target PCell for no RACH response transmission.

//Calculate T in the TargetPCell

v_TimingInfo := f_NR_TimingOtherCell(p_SourcePCellId, cs_TimingInfo_NR(v_Timing), p_TargetPCellId);

f_NR_SS_RachProcedureConfig(p_TargetPCellId, cs_NR_RachProcedureConfig_NoResponse, v_TimingInfo);

// 3. Source PCell: Schedule the transmission of RRCReconfiguration message to UE requesting Handover to target PCell and SCell.

SRB.send(cas_NR_SRB1_RrcPdu_REQ(p_SourcePCellId, cs_TimingInfo_NR(v_Timing), p_RRCReconfiguration, crs_MacBearerRouting(p_SourcePCellId))); //@sic R5-213516 sic@

// At time T1 if target PCell is the same as source SCell, else at time "Now", steps 4-6:

if (p_TargetPCellId == p_SourceSCellId) { //@sic R5-223443 R5s220590 sic@

//Calculate T1 in the TargetPCell

v_TimingInfo := f_NR_TimingOtherCell(p_SourcePCellId, cs_TimingInfo_NR(v_Timing1), p_TargetPCellId);

// 4.1 Source PCell: Configure from PCell to normal cell: remove source SCell from the SCell list in the source PCell

// 5. Target PCell: Configuration of DRBs

f_NR_ReconfigurePCellToNormalCell( p_SourcePCellId, tsc_PCellIndex_0, v_TimingInfo, v_DrbIdList, true);  //@sic R5s220816 sic@

// 4.2 Target PCell: Configure from SCell to PCell.

f_NR_ReconfigSCellToNormalCell(p_TargetPCellId, v_RadioBearerList, v_TimingInfo);

} else {

// At time Now, steps 5-6:

// 5. Target PCell: Configuration of DRBs

f_NR_SS_CommonRadioBearerConfig(p_TargetPCellId, v_RadioBearerList); //@sic R5s201249 sic@

}

//6. Transfer of the PDCP Count for AM DRBs and SRBs (if applied) from source to target PCell:

//   a) Source PCell: Get PDCP COUNT.

//   b) Target PCell: Set PDCP COUNT.

//   NOTE 2: No further sending/receiving of DRB data before the HO is done.

//   NOTE 3: For AM DRBs the PDCP count is maintained, for UM DRBs the PDCP count is reset. For SRBs, the PDCP count is maintained or reset depending on the RRCReconfiguration message content.

if (p_TargetPCellId == p_SourceSCellId) {

v_PdcpCountInfoList := f_NR_SS_PdcpCount_Handover(p_SourcePCellId, p_TargetPCellId, v_RadioBearerList, p_RBConfig_KeyChange, cs_TimingInfo_NR(v_Timing1));

} else {

v_PdcpCountInfoList := f_NR_SS_PdcpCount_Handover(p_SourcePCellId, p_TargetPCellId, v_RadioBearerList, p_RBConfig_KeyChange); //@sic R5s201249 sic@

}

//At time T2, steps 7-12:

//7. Target PCell: Inform the SS about the HO and about the source PCell id.

f_NR_SS_PdcpHandoverCtrl(p_TargetPCellId, cas_NR_PdcpHandoverInit_REQ(p_SourcePCellId, p_TargetPCellId, cs_TimingInfo_NR(v_Timing2)));

//8. Target PCell: Configure RACH procedure either dedicated or C-RNTI based.

// NOTE 4: The FollowOnFlag is set to true in the ASP reconfiguring C-RNTI

f_NR_CellInfo_SetRNTI(p_TargetPCellId, p_RNTI_Value);

//Calculate T2 in the TargetPCell

v_TimingInfo := f_NR_TimingOtherCell(p_SourcePCellId, cs_TimingInfo_NR(v_Timing2), p_TargetPCellId);

if (isvalue(p_TargetSCellId)) { //@sic R5s210079 sic@

//@sic R5s201249 sic@

f_NR_CellInfo_SetRNTI(valueof(p_TargetSCellId), p_RNTI_Value);

f_NR_SS_C_RNTI_Config(valueof(p_TargetSCellId), p_RNTI_Value, cs_TimingInfo_NR(v_Timing2)); //@sic R5s210389 sic@

}

f_NR_SS_C_RNTI_Config(p_TargetPCellId, p_RNTI_Value, cs_TimingInfo_NR(v_Timing2), tsc_NoCnfReq, tsc_FollowOnFlag);  //@sic R5-206369, R5s210389 sic@

f_NR_SS_RachProcedureConfigHO(p_TargetPCellId, p_RachProcedureConfig, v_TimingInfo, p_RACH_ConfigDedicated);

//9. Target PCell: Activate security.

if ((p_NCC > tsc_NR_38508_NextHopChainingCount) or p_RBConfig_KeyChange) { // @sic R5s190795 R5s200159 sic@

f_NR_SS_AS_ActivateSecurity_Current(p_TargetPCellId, p_NCC, v_TimingInfo, -, v_PdcpCountInfoList);

} else {

f_NR_SS_AS_ActivateSecurity_NoRefresh(p_TargetPCellId, v_TimingInfo, PdcpCountInfoList);  //@sic R5s200102 sic@

}