3GPP TSG-RAN5 Meeting #2024-TTCN email	R5s240423

# Online, , 8th Dec 2023 - 31st Dec 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.523-3 | CR | 4832 | rev | - | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to function fl_NBIOT_Common_Init () |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies UK Ltd |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI17_Test, LTE_NBIOT_eMTC_NTN_plus_EPS-UEConTest |  |  |  |  |  | Date: |  |  | 2024-06-18 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The value of NtnCellType should be configured to identify the different NCell with index i. By default, value of NtnCellType is set as 1 for all cells including Cell 1. NtnCellType is updated for neighbour cells in function fl_NBIOT_Common_Init. According to current implementation of TTCN the value of NtnCellType shall be set to 1 for the first neighbour cell as the value of i will be 0. This will conflict with NtnCellType of the Main cell. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Modified the function to also consider the current value of the NtnCellType |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may fail the NB-IoT multi-cell test cases |  |  |  |  |  |  |  |  |
| Clauses affected: |  | All NB-IoT multi-cell test cases |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Table of Contents

Table of Contents	2

1	Overview	3

2	Corrections required	3

2.1	Change 1	3

# Overview

This document lists all the changes needed to correct issues in the ATS iwd-TTCN3-B2022-09_D24wk24 related to the title of this CR.

Contact:	Mohit Kanchan
	mohit.kanchan@keysight.com

# Corrections required

## Change 1

| Function name | fl_NBIOT_Common_Init () |
| --- | --- |
| Reason for change | The value of NtnCellType should be configured to identify the different NCell with index i. By default, value of NtnCellType is set as 1 for all cells including Cell 1. NtnCellType is updated for neighbour cells in function fl_NBIOT_Common_Init. According to current implementation of TTCN the value of NtnCellType shall be set to 1 for the first neighbour cell as the value of i will be 0. This will conflict with NtnCellType of the Main cell. |
| Summary of change | Modified the function to also consider the current value of the NtnCellType |
| TTCN module | NBIOT_CellInfo.ttcn |
| MCC160 Comment | Accepted |

Before Change:

function fl_NBIOT_Common_Init(NB_InitParams_Local_Type p_Params, boolean p_DoAttachWithoutPDN, template (omit)NBIOT_CellId_List_Type p_Neighbour_NTN_Cells := omit) runs on NBIOT_PTC { var integer i; var integer v_Index; var NTN_Satellite_Type v_NTN_Scenario := f_NBIOT_MobileInfo_GetNTNSatelliteType(); //@sic R5w240103 sic@ vc_NBIOT_Global.Ctrl.Default := null; vc_NBIOT_Global.Ctrl.TestBody := false; vc_NBIOT_Global.Ctrl.VerdictCtrl := normal; vc_NBIOT_Global.Ctrl.HarqErrorHandling := ignore; vc_NBIOT_Global.Ctrl.LoopbackIsActivated := false; // Initialise security parameters vc_NBIOT_Global.Security := f_NBIOT_Security_Init(); // Initialise Cell array vc_NBIOT_Global.CellArray := f_NBIOT_CellArray_Init(p_Params.SysinfoCombination, p_DoAttachWithoutPDN); //@sic R5-235468, R5s230706 sic@ vc_NBIOT_Global.CellArray := f_NBIOT_NAS_CellArray_Init(vc_NBIOT_Global.CellArray, p_Params.CellsOnSamePLMN, p_Params.SysinfoCombination, p_DoAttachWithoutPDN, p_Params.CellSpecialFreq); if(ispresent(p_Neighbour_NTN_Cells) and v_NTN_Scenario != ntn_None) {//@sic R5-235468, R5s230706, R5-237126 sic@ for(i:=0; i<lengthof(p_Neighbour_NTN_Cells); i:=i+1) { v_Index := f_NBIOT_CellInfo_GetIndex(valueof(p_Neighbour_NTN_Cells[i])); if(v_NTN_Scenario == ntn_GSO){ f_NBIOT_CellInfo_SetSIB31(valueof(p_Neighbour_NTN_Cells[i]), cs_SystemInformationBlockType31_NB_r17_GSO_Array[i+1]); } else{//@sic R5w240103 sic@ f_NBIOT_CellInfo_SetSIB31(valueof(p_Neighbour_NTN_Cells[i]), cs_SystemInformationBlockType31_NB_r17_NGSO_Array[i+1]); } vc_NBIOT_Global.CellArray[v_Index].NtnInfo.NtnCellType := i+1; //@sic R5s240270, R5-237126 sic@ } } //Initialise Mobile Info vc_NBIOT_Global.MobileInfo.Tmsi := '00000000'O; vc_NBIOT_Global.MobileInfo.P_Tmsi := '00000000'O; vc_NBIOT_Global.MobileInfo.PDNs := omit; f_NBIOT_ActivateDefault(StandardDefault); }

After Change:

function fl_NBIOT_Common_Init(NB_InitParams_Local_Type p_Params, boolean p_DoAttachWithoutPDN, template (omit)NBIOT_CellId_List_Type p_Neighbour_NTN_Cells := omit) runs on NBIOT_PTC { var integer i; var integer v_Index; var NTN_Satellite_Type v_NTN_Scenario := f_NBIOT_MobileInfo_GetNTNSatelliteType(); //@sic R5w240103 sic@ vc_NBIOT_Global.Ctrl.Default := null; vc_NBIOT_Global.Ctrl.TestBody := false; vc_NBIOT_Global.Ctrl.VerdictCtrl := normal; vc_NBIOT_Global.Ctrl.HarqErrorHandling := ignore; vc_NBIOT_Global.Ctrl.LoopbackIsActivated := false; // Initialise security parameters vc_NBIOT_Global.Security := f_NBIOT_Security_Init(); // Initialise Cell array vc_NBIOT_Global.CellArray := f_NBIOT_CellArray_Init(p_Params.SysinfoCombination, p_DoAttachWithoutPDN); //@sic R5-235468, R5s230706 sic@ vc_NBIOT_Global.CellArray := f_NBIOT_NAS_CellArray_Init(vc_NBIOT_Global.CellArray, p_Params.CellsOnSamePLMN, p_Params.SysinfoCombination, p_DoAttachWithoutPDN, p_Params.CellSpecialFreq); if(ispresent(p_Neighbour_NTN_Cells) and v_NTN_Scenario != ntn_None) {//@sic R5-235468, R5s230706, R5-237126 sic@ for(i:=0; i<lengthof(p_Neighbour_NTN_Cells); i:=i+1) { v_Index := f_NBIOT_CellInfo_GetIndex(valueof(p_Neighbour_NTN_Cells[i])); if(v_NTN_Scenario == ntn_GSO){ f_NBIOT_CellInfo_SetSIB31(valueof(p_Neighbour_NTN_Cells[i]), cs_SystemInformationBlockType31_NB_r17_GSO_Array[i+1]); } else{//@sic R5w240103 sic@ f_NBIOT_CellInfo_SetSIB31(valueof(p_Neighbour_NTN_Cells[i]), cs_SystemInformationBlockType31_NB_r17_NGSO_Array[i+1]); } vc_NBIOT_Global.CellArray[v_Index].NtnInfo.NtnCellType := vc_NBIOT_Global.CellArray[v_Index].NtnInfo.NtnCellType + i+1; //@sic R5s240270, R5-237126 sic@ } } //Initialise Mobile Info vc_NBIOT_Global.MobileInfo.Tmsi := '00000000'O; vc_NBIOT_Global.MobileInfo.P_Tmsi := '00000000'O; vc_NBIOT_Global.MobileInfo.PDNs := omit; f_NBIOT_ActivateDefault(StandardDefault); }