3GPP TSG-RAN5 Meeting #2024-TTCN email	R5s240425

# Online, , 8th Dec 2023 - 31st Dec 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 3463 | rev | - | Current version: | 17.10.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to NR5GC testcase 9.1.6.1.1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2024-06-19 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | In current TTCN implementation, at Step 13a3, USIM is removed, and the TTCN expects a De-registraiton REQ. This does not accommodate a possibility of the UE releasing the PDU session that has been established in Step 13a2 prior to this. This needs to be accommodated |  |  |  |  |  |  |  |  |
| Summary of change: |  | Implemented the TTCN to handle the PDU Session release after Step 13a3. Kindle see detailed change below. A Prose CR will be raised for this. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE can fail the test cases |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.1.6.1.1.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  | x |  | Test specifications |  |  |  | 38.523-1 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	2

2	Corrections required	2

2.1	fl_TC_9_1_6_1_1_TestBody	2

# Overview

This document lists all the essential changes needed to correct problems in the iwd-TTCN3-B2022-09_D24wk24 TTCN-3 related to the title of this CR.

Contact: 	Parikshit Bhise

Parikshit.bhise@rohde-schwarz.com

# Corrections required

## fl_TC_9_1_6_1_1_TestBody

| Function name | fl_TC_9_1_6_1_1_TestBody |
| --- | --- |
| Reason for change | In current TTCN implementation, at Step 13a3, USIM is removed, and the TTCN expects a De-registraiton REQ. This does not accommodate a possibility of the UE releasing the PDU session that has been established in Step 13a2 prior to this. This needs to be accommodated |
| Summary of change | Implemented the TTCN to handle the PDU Session release after Step 13a3. Kindle see detailed change below. A Prose CR will be raised for this. |
| TTCN module | Deregistration_NR5GC.ttcn |
| MCC160 Comment | Rejected Because the associated prose CR R5-245752 was set as not pursued at RAN5#104. |

Before Change:

<<SKIPPED CODE>> if (pc_USIM_Removal) { // The UE is switched on. //@siclog "Step 13a1" siclog@ f_NR5GC_SwitchOnAndResetIMS(); //@sic R5s220733 sic@ //The UE performs Registration procedure as specified in TS 38.508-1 [4] subclause 4.5.2 with 'connected without release'. //@siclog "step13a2" siclog@ f_NR5GC_RRC_IdleState1N_Def(v_NGC_NASCellA, -, noRrcConnectionRelease); //Cause removal of USIM from the UE without powering down. //@siclog "step13a3" siclog@ f_UT_USIM_Remove (UT); //Check: Does the UE transmit a DEREGISTRATION REQUEST with the De-registration type IE indicating "switch off"? //@siclog "step13a4" siclog@ SRB.receive(car_NR_SRB_NasPdu_IND(v_NGC_NASCellA, tsc_NR_RbId_SRB2, cr_NG_NAS_Ind(tsc_SHT_IntegrityProtected_Ciphered))) -> value v_ReceivedAsp; // @sic R5s190109 sic@ v_ReceivedMsg := v_ReceivedAsp.Signalling.Nas[0]; if(not f_Check_NG_DeregistrationReqMOMsg(v_ReceivedMsg.Pdu.Msg, crs_DeregisterType('1'B, '0'B, '01'B))) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "De-registration Request Message Failed"); } else { f_NR_PreliminaryPass (__FILE__, __LINE__, "Step 13"); } } <<SKIPPED CODE>>

After Change:

<<SKIPPED CODE>> if (pc_USIM_Removal) { // The UE is switched on. //@siclog "Step 13a1" siclog@ f_NR5GC_SwitchOnAndResetIMS(); //@sic R5s220733 sic@ //The UE performs Registration procedure as specified in TS 38.508-1 [4] subclause 4.5.2 with 'connected without release'. //@siclog "step13a2" siclog@ f_NR5GC_RRC_IdleState1N_Def(v_NGC_NASCellA, -, noRrcConnectionRelease); //Cause removal of USIM from the UE without powering down. //@siclog "step13a3" siclog@ f_UT_USIM_Remove (UT); //Check: Does the UE transmit a DEREGISTRATION REQUEST with the De-registration type IE indicating "switch off"? //@siclog "step13a4" siclog@ SRB.receive(car_NR_SRB_NasPdu_IND(v_NGC_NASCellA, tsc_NR_RbId_SRB2, cr_NG_NAS_Ind(tsc_SHT_IntegrityProtected_Ciphered))) -> value v_ReceivedAsp; // @sic R5s190109 sic@ v_ReceivedMsg := v_ReceivedAsp.Signalling.Nas[0]; v_DetachReq := f_NR5GC_OptionalPDUsReleasedOnSwitchOff (v_NGC_NASCellA, v_ReceivedAsp.Signalling.Nas[0]); //WA#9_1_6_1_1 if(not f_Check_NG_DeregistrationReqMOMsg(v_DetachReq.Pdu.Msg /*WA#9_1_6_1_1 v_ReceivedMsg.Pdu.Msg*/, crs_DeregisterType('1'B, '0'B, '01'B))) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "De-registration Request Message Failed"); } else { f_NR_PreliminaryPass (__FILE__, __LINE__, "Step 13"); } } <<SKIPPED CODE>>