3GPP TSG-RAN5 Meeting #2024-TTCN email	R5s240426

# Online, , 8th Dec 2023 - 31st Dec 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 3464 | rev | - | Current version: | 17.10.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to NR5GC test case 9.1.5.1.4 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies UK Ltd |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2024-06-20 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The required implementation to execute the NR5GC test case 9.1.5.1.4 is missing from the NR5GC_Testsuite module. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added the missing implementation to enable execution of this test case. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | 9.1.5.1.4 test case can not be executed. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.1.5.1.4.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

2	Corrections required.	3

2.1	Change 1	3

# Overview

This document lists all the changes needed to correct issues in the ATS iwd-TTCN3-B2022-09_D24wk24 related to the title of this CR.

Contact:	Mohit Kanchan
	mohit.kanchan@keysight.com

# Corrections required.

## Change 1

| Function name | module NR5GC_Testsuite |
| --- | --- |
| Reason for change | The required implementation to execute the NR5GC test case 9.1.5.1.4 is missing from the NR5GC_Testsuite module. |
| Summary of change | Added the missing implementation to enable execution of this test case. |
| TTCN module | NR5GC_Testsuite.ttcn |
| MCC160 Comment | Accepted. In addition, one further update is needed to set the SNPN Only attribute in the test. See additional MCC160 implementation below. |

Before Change:

module NR5GC_Testsuite { SKIPPED TEXT //---------------------------------------------------------------------------- testcase TC_9_1_5_1_3a_NR5GC() runs on MTC_NR5GC system SYSTEM_NR5GC { // @purpose // Initial registration / 5GS services / NSSAI handling / NSSAI storage var NR5GC_PTC v_NR5GC := null; var IMS_PTC v_IMS1 := null; var IMS_PTC v_IMS2 := null; timer t_GuardTimer := int2float(900); v_NR5GC := NR5GC_PTC.create("NR5GC") alive; f_MTC_ConnectPTCs_NR5GC(system, v_NR5GC, v_IMS1, v_IMS2); v_NR5GC.start(f_TC_9_1_5_1_3a_NR5GC()); t_GuardTimer.start; f_MTC_MainLoop_NR5GC(t_GuardTimer); } //---------------------------------------------------------------------------- //---------------------------------------------------------------------------- //---------------------------------------------------------------------------- //---------------------------------------------------------------------------- testcase TC_9_1_5_1_5_NR5GC() runs on MTC_NR5GC system SYSTEM_NR5GC { // @purpose // Initial registration / Abnormal / Change of cell into a new tracking area / Failure after 5 attempts var NR5GC_PTC v_NR5GC := null; var IMS_PTC v_IMS1 := null; var IMS_PTC v_IMS2 := null; timer t_GuardTimer := int2float(1000); v_NR5GC := NR5GC_PTC.create("NR5GC") alive; f_MTC_ConnectPTCs_NR5GC(system, v_NR5GC, v_IMS1, v_IMS2); v_NR5GC.start(f_TC_9_1_5_1_5_NR5GC()); t_GuardTimer.start; f_MTC_MainLoop_NR5GC(t_GuardTimer); } SKIPPED TEXT //---------------------------------------------------------------------------- control { if (f_NR_TestcaseApplicability(NR_C39A, "TC_10_1_1_1_NR5GC")) { execute(TC_10_1_1_1_NR5GC()); } if (f_NR_TestcaseApplicability(NR_C39A, "TC_10_1_1_2_NR5GC")) { execute(TC_10_1_1_2_NR5GC()); } if (f_NR_TestcaseApplicability(NR_C21, "TC_10_1_2_1_NR5GC")) { execute(TC_10_1_2_1_NR5GC()); } SKIPPED TEXT if (f_NR_TestcaseApplicability(NR_C21, "TC_9_1_5_1_3_NR5GC")) { execute(TC_9_1_5_1_3_NR5GC()); } if (f_NR_TestcaseApplicability(NR_C21, "TC_9_1_5_1_3a_NR5GC")) { execute(TC_9_1_5_1_3a_NR5GC()); } if (f_NR_TestcaseApplicability(NR_C21, "TC_9_1_5_1_5_NR5GC")) { execute(TC_9_1_5_1_5_NR5GC()); } SKIPPED TEXT }

After Change:

module NR5GC_Testsuite { SKIPPED TEXT //---------------------------------------------------------------------------- testcase TC_9_1_5_1_3a_NR5GC() runs on MTC_NR5GC system SYSTEM_NR5GC { // @purpose // Initial registration / 5GS services / NSSAI handling / NSSAI storage var NR5GC_PTC v_NR5GC := null; var IMS_PTC v_IMS1 := null; var IMS_PTC v_IMS2 := null; timer t_GuardTimer := int2float(900); v_NR5GC := NR5GC_PTC.create("NR5GC") alive; f_MTC_ConnectPTCs_NR5GC(system, v_NR5GC, v_IMS1, v_IMS2); v_NR5GC.start(f_TC_9_1_5_1_3a_NR5GC()); t_GuardTimer.start; f_MTC_MainLoop_NR5GC(t_GuardTimer); } //---------------------------------------------------------------------------- //---------------------------------------------------------------------------- testcase TC_9_1_5_1_4_NR5GC() runs on MTC_NR5GC system SYSTEM_NR5GC { // @purpose // Initial registration / 5GS services / MICO mode / TAI list handling var NR5GC_PTC v_NR5GC := null; var IMS_PTC v_IMS1 := null; var IMS_PTC v_IMS2 := null; timer t_GuardTimer := int2float(300); v_NR5GC := NR5GC_PTC.create("NR5GC") alive; f_MTC_ConnectPTCs_NR5GC(system, v_NR5GC, v_IMS1, v_IMS2); v_NR5GC.start(f_TC_9_1_5_1_4_NR5GC()); t_GuardTimer.start; f_MTC_MainLoop_NR5GC(t_GuardTimer); } //---------------------------------------------------------------------------- //---------------------------------------------------------------------------- testcase TC_9_1_5_1_5_NR5GC() runs on MTC_NR5GC system SYSTEM_NR5GC { // @purpose // Initial registration / Abnormal / Change of cell into a new tracking area / Failure after 5 attempts var NR5GC_PTC v_NR5GC := null; var IMS_PTC v_IMS1 := null; var IMS_PTC v_IMS2 := null; timer t_GuardTimer := int2float(1000); v_NR5GC := NR5GC_PTC.create("NR5GC") alive; f_MTC_ConnectPTCs_NR5GC(system, v_NR5GC, v_IMS1, v_IMS2); v_NR5GC.start(f_TC_9_1_5_1_5_NR5GC()); t_GuardTimer.start; f_MTC_MainLoop_NR5GC(t_GuardTimer); } SKIPPED TEXT //---------------------------------------------------------------------------- control { if (f_NR_TestcaseApplicability(NR_C39A, "TC_10_1_1_1_NR5GC")) { execute(TC_10_1_1_1_NR5GC()); } if (f_NR_TestcaseApplicability(NR_C39A, "TC_10_1_1_2_NR5GC")) { execute(TC_10_1_1_2_NR5GC()); } if (f_NR_TestcaseApplicability(NR_C21, "TC_10_1_2_1_NR5GC")) { execute(TC_10_1_2_1_NR5GC()); } SKIPPED TEXT if (f_NR_TestcaseApplicability(NR_C21, "TC_9_1_5_1_3_NR5GC")) { execute(TC_9_1_5_1_3_NR5GC()); } if (f_NR_TestcaseApplicability(NR_C21, "TC_9_1_5_1_3a_NR5GC")) { execute(TC_9_1_5_1_3a_NR5GC()); } if (f_NR_TestcaseApplicability(NR_C313, "TC_9_1_5_1_4_NR5GC")) { execute(TC_9_1_5_1_4_NR5GC()); } if (f_NR_TestcaseAp4licability(NR_C21, "TC_9_1_5_1_5_NR5GC")) { execute(TC_9_1_5_1_5_NR5GC()); } SKIPPED TEXT }

Additional MCC160 implementation

module TestcaseProperties

function f_GetTestcaseAttrib_SNPN_Only(charstring p_Testcase) return boolean { select (p_Testcase) { case ("TC_10_1_1_1_NR5GC") { return true; } ... SKIPPED TEXT ... case ("TC_9_1_5_1_3_NR5GC") { return true; } case ("TC_9_1_5_1_3a_NR5GC") { return true; } case ("TC_9_1_5_1_4_NR5GC") { return true; } case ("TC_9_1_5_1_5_NR5GC") { return true; } case ("TC_9_1_5_1_9_NR5GC") { return true; } case ("TC_9_1_5_2_1_NR5GC") { return true; } case ("TC_9_1_5_2_2_NR5GC") { return true; } case ("TC_9_1_5_2_4_NR5GC") { return true; } case ("TC_9_1_5_2_7_NR5GC") { return true; } case ("TC_9_1_5_2_8_NR5GC") { return true; } case ("TC_9_1_6_1_1_NR5GC") { return true; } case ("TC_9_1_6_1_2_NR5GC") { return true; } case ("TC_9_1_6_1_3_NR5GC") { return true; } case ("TC_9_1_6_2_1_NR5GC") { return true; } case ("TC_9_1_6_2_2_NR5GC") { return true; } case ("TC_9_1_7_1_NR5GC") { return true; } case ("TC_9_1_7_2_NR5GC") { return true; } case ("TC_9_1_8_1_NR5GC") { return true; } case ("TC_9_1_8_2_NR5GC") { return true; } } return false; }