3GPP TSG-RAN5 Meeting #2024-TTCN email	R5s240427

# Online, , 8th Dec 2023 - 31st Dec 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 3465 | rev | - | Current version: | 17.10.0 |  |
| For HELPon using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed changeaffects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to NSA testcase 7.1.1.1.2 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Starpoint |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2024-06-21 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Different UE takes different time to access to NR, if TTCN code runs fast,in function f_TC_7_1_1_1_2_NR_TestBody1, rach will configured for SS,which may lead to error then handle msg4 for UE access to NR. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add delay before f_TC_7_1_1_1_2_NR_TestBody1 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE can fail the test cases. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.1.1.1.2.ENDC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

2	Corrections required	3

2.1	Change 1	3

# Overview

This document lists all the changes needed to correct issues in the ATS iwd-TTCN3-B2022-09_D24wk24 related to the title of this CR.

Contact:	Ting Gao
	gaoting@starpointcomm.com

# Corrections required

## Change 1

| Function name | f_TC_7_1_1_1_2_ENDC_NR() |
| --- | --- |
| Reason for change | Different UE takes different time to access to NR, if TTCN code runs fast,in function f_TC_7_1_1_1_2_NR_TestBody1, rach will configured for SS,which may lead to error then handle msg4 for UE access to NR. |
| Summary of change | Add delay before f_TC_7_1_1_1_2_NR_TestBody1. |
| TTCN module | MAC_ENDC_NR.ttcn |
| MCC160 Comment | Accepted as improvement for Stability and repeatability of test case. |

Before Change:

function f_TC_7_1_1_1_2_ENDC_NR() runs on ENDC_NR_PTC { // Random access procedure / Successful / C-RNTI Based / Preamble selected by MAC itself var template (value) CellGroupConfig v_SCGConfig; var template (value) SpCellConfig v_SpCellConfig ; var template (value) RACH_ConfigGeneric v_RACH_ConfigGeneric ; var template (value) MAC_CellGroupConfig v_MAC_CellGroupConfig; var NR_DuplexMode_Type v_NR_DuplexMode ; var boolean v_IsFR1 ; var template (value) NR_UplinkBWP_List_Type v_NR_UplinkBWP_List; //@sic R5s190837 sic@ //@sic R5s201316 sic@ var NR_PhysicalParameters_Type v_NR_PhysicalParameters; var template (value) NR_RachProcedureConfig_Type v_NR_RachProcedure; //Init Cell parameters f_ENDC_NR_Init(); //Create and configure NR cell1 v_MAC_CellGroupConfig := f_NR_CellInfo_GetMAC_CellGroupConfig(nr_Cell1); v_MAC_CellGroupConfig.tag_Config.tag_ToAddModList[0].timeAlignmentTimer := ms750; f_NR_CellInfo_SetMAC_CellGroupConfig(nr_Cell1, v_MAC_CellGroupConfig); //Create and configure NR cell1 f_NR_CellConfig_Def(nr_Cell1); // Get NR capabilities sent during EUTRA preamble f_NR_ENDC_PreambleOnEUTRA(nr_Cell1, MCG_Only); f_NR_AS_CipheringAlgorithm_Set(nea0); //Configure on SS the SCG DRB in test mode f_NR_ENDC_ConfigAM_UM(nr_Cell1, 1, 0, -, -, -, -, -, -); // Wait for Trigger from EUTRA indicating start of Test Body f_EUTRA_NR_WaitForCoOrd_Trigger(EUTRA); f_NR_TestBody_Set(true); //@siclog "Steps 0A-0B" siclog@ v_NR_DuplexMode := f_NR_CellInfo_GetDuplexMode(nr_Cell1); v_IsFR1 := f_NR_CellInfo_GetIsFR1(nr_Cell1); if (v_IsFR1) { if (ischosen(v_NR_DuplexMode.TDD)) //FR1 TDD { v_RACH_ConfigGeneric := cs_RACH_ConfigGeneric71112_FR1_TDD; } else if (f_NR_CellInfo_GetHalfDuplexFDD(nr_Cell1) ) //Half Duplex FR1 FDD { //@sic R5-233408 sic@ v_RACH_ConfigGeneric := cs_RACH_ConfigGeneric71112_FR1_HDFDD; } else //FR1 FDD { v_RACH_ConfigGeneric := cs_RACH_ConfigGeneric71112_FR1_FDD; } } else { // FR2 v_RACH_ConfigGeneric := cs_RACH_ConfigGeneric71112_FR2; } v_SpCellConfig := f_NR_GetSpCellConfigDef(nr_Cell1); v_NR_UplinkBWP_List := f_NR_CellInfo_GetUplinkBWP_List(nr_Cell1); v_SpCellConfig.reconfigurationWithSync.t304 := ms2000; v_SpCellConfig.reconfigurationWithSync.rach_ConfigDedicated := omit; //@sic R5s190837 sic@ v_SpCellConfig.reconfigurationWithSync.spCellConfigCommon.uplinkConfigCommon.initialUplinkBWP.rach_ConfigCommon.setup.rach_ConfigGeneric := v_RACH_ConfigGeneric; v_NR_UplinkBWP_List[0].Common.R15.rach_ConfigCommon.setup.rach_ConfigGeneric := valueof(v_RACH_ConfigGeneric); v_SpCellConfig.reconfigurationWithSync.spCellConfigCommon.uplinkConfigCommon.initialUplinkBWP.rach_ConfigCommon.setup.totalNumberOfRA_Preambles := 42; v_NR_UplinkBWP_List[0].Common.R15.rach_ConfigCommon.setup.totalNumberOfRA_Preambles := 42; v_SpCellConfig.reconfigurationWithSync.spCellConfigCommon.uplinkConfigCommon.initialUplinkBWP.rach_ConfigCommon.setup.ssb_perRACH_OccasionAndCB_PreamblesPerSSB := { one := n32 }; v_NR_UplinkBWP_List[0].Common.R15.rach_ConfigCommon.setup.ssb_perRACH_OccasionAndCB_PreamblesPerSSB := { one := n32 }; v_SpCellConfig.reconfigurationWithSync.spCellConfigCommon.uplinkConfigCommon.initialUplinkBWP.rach_ConfigCommon.setup.groupBconfigured := cs_GroupBConfigured_Def; v_NR_UplinkBWP_List[0].Common.R15.rach_ConfigCommon.setup.groupBconfigured := valueof(cs_GroupBConfigured_Def); v_SpCellConfig.reconfigurationWithSync.spCellConfigCommon.uplinkConfigCommon.initialUplinkBWP.rach_ConfigCommon.setup.ra_ContentionResolutionTimer := sf48; v_NR_UplinkBWP_List[0].Common.R15.rach_ConfigCommon.setup.ra_ContentionResolutionTimer := sf48; f_NR_SS_CommonCellConfig(nr_Cell1, cads_NR_UplinkBWP_ConfigCommon_REQ(nr_Cell1, -, v_NR_UplinkBWP_List)); // @sic R5s200549 sic@ v_SCGConfig := cs_38508_CellGroupConfig (tsc_NR_CellGroupId_SCG, omit, omit, omit, omit, v_SpCellConfig //SpCellConfig ); f_NR_SendRRCReconfigurationContentsToEUTRA (omit, v_SCGConfig,-,-,-,reconfigurationWithSync_CheckRACH); //@sic R5s190541 sic@ // Now send octetstrings to EUTRA f_TC_7_1_1_1_2_NR_TestBody1(tsc_NR_DRB2); //@sic R5s201316 sic@ v_NR_PhysicalParameters := f_NR_CellInfo_GetPhysicalParameters(nr_Cell1); v_NR_RachProcedure := cs_NR_RachProcedureConfig_71112_2(v_NR_PhysicalParameters); f_NR_SS_RachProcedureConfig(nr_Cell1, v_NR_RachProcedure); f_NR_SendRRCReconfigurationContentsToEUTRA (omit, v_SCGConfig,-,-,-,reconfigurationWithSync_CheckRACH); //@sic R5s190541 sic@ // Now send octetstrings to EUTRA f_TC_7_1_1_1_2_NR_TestBody2(tsc_NR_DRB2); f_NR_TestBody_Set(false); // Send Trigger to EUTRA indicating Finish of Test Body f_EUTRA_NR_SendCoOrd(EUTRA, cms_EUTRA_NR_Trigger); //Wait from coordination AFTER the connection is released in E-UTRA f_EUTRA_NR_WaitForCoOrd_Trigger(EUTRA, "Postamble"); //Postamble f_NR_ReleaseAllCells(); }

After Change:

function f_TC_7_1_1_1_2_ENDC_NR() runs on ENDC_NR_PTC { // Random access procedure / Successful / C-RNTI Based / Preamble selected by MAC itself var template (value) CellGroupConfig v_SCGConfig; var template (value) SpCellConfig v_SpCellConfig ; var template (value) RACH_ConfigGeneric v_RACH_ConfigGeneric ; var template (value) MAC_CellGroupConfig v_MAC_CellGroupConfig; var NR_DuplexMode_Type v_NR_DuplexMode ; var boolean v_IsFR1 ; var template (value) NR_UplinkBWP_List_Type v_NR_UplinkBWP_List; //@sic R5s190837 sic@ //@sic R5s201316 sic@ var NR_PhysicalParameters_Type v_NR_PhysicalParameters; var template (value) NR_RachProcedureConfig_Type v_NR_RachProcedure; //Init Cell parameters f_ENDC_NR_Init(); //Create and configure NR cell1 v_MAC_CellGroupConfig := f_NR_CellInfo_GetMAC_CellGroupConfig(nr_Cell1); v_MAC_CellGroupConfig.tag_Config.tag_ToAddModList[0].timeAlignmentTimer := ms750; f_NR_CellInfo_SetMAC_CellGroupConfig(nr_Cell1, v_MAC_CellGroupConfig); //Create and configure NR cell1 f_NR_CellConfig_Def(nr_Cell1); // Get NR capabilities sent during EUTRA preamble f_NR_ENDC_PreambleOnEUTRA(nr_Cell1, MCG_Only); f_NR_AS_CipheringAlgorithm_Set(nea0); //Configure on SS the SCG DRB in test mode f_NR_ENDC_ConfigAM_UM(nr_Cell1, 1, 0, -, -, -, -, -, -); // Wait for Trigger from EUTRA indicating start of Test Body f_EUTRA_NR_WaitForCoOrd_Trigger(EUTRA); f_NR_TestBody_Set(true); //@siclog "Steps 0A-0B" siclog@ v_NR_DuplexMode := f_NR_CellInfo_GetDuplexMode(nr_Cell1); v_IsFR1 := f_NR_CellInfo_GetIsFR1(nr_Cell1); if (v_IsFR1) { if (ischosen(v_NR_DuplexMode.TDD)) //FR1 TDD { v_RACH_ConfigGeneric := cs_RACH_ConfigGeneric71112_FR1_TDD; } else if (f_NR_CellInfo_GetHalfDuplexFDD(nr_Cell1) ) //Half Duplex FR1 FDD { //@sic R5-233408 sic@ v_RACH_ConfigGeneric := cs_RACH_ConfigGeneric71112_FR1_HDFDD; } else //FR1 FDD { v_RACH_ConfigGeneric := cs_RACH_ConfigGeneric71112_FR1_FDD; } } else { // FR2 v_RACH_ConfigGeneric := cs_RACH_ConfigGeneric71112_FR2; } v_SpCellConfig := f_NR_GetSpCellConfigDef(nr_Cell1); v_NR_UplinkBWP_List := f_NR_CellInfo_GetUplinkBWP_List(nr_Cell1); v_SpCellConfig.reconfigurationWithSync.t304 := ms2000; v_SpCellConfig.reconfigurationWithSync.rach_ConfigDedicated := omit; //@sic R5s190837 sic@ v_SpCellConfig.reconfigurationWithSync.spCellConfigCommon.uplinkConfigCommon.initialUplinkBWP.rach_ConfigCommon.setup.rach_ConfigGeneric := v_RACH_ConfigGeneric; v_NR_UplinkBWP_List[0].Common.R15.rach_ConfigCommon.setup.rach_ConfigGeneric := valueof(v_RACH_ConfigGeneric); v_SpCellConfig.reconfigurationWithSync.spCellConfigCommon.uplinkConfigCommon.initialUplinkBWP.rach_ConfigCommon.setup.totalNumberOfRA_Preambles := 42; v_NR_UplinkBWP_List[0].Common.R15.rach_ConfigCommon.setup.totalNumberOfRA_Preambles := 42; v_SpCellConfig.reconfigurationWithSync.spCellConfigCommon.uplinkConfigCommon.initialUplinkBWP.rach_ConfigCommon.setup.ssb_perRACH_OccasionAndCB_PreamblesPerSSB := { one := n32 }; v_NR_UplinkBWP_List[0].Common.R15.rach_ConfigCommon.setup.ssb_perRACH_OccasionAndCB_PreamblesPerSSB := { one := n32 }; v_SpCellConfig.reconfigurationWithSync.spCellConfigCommon.uplinkConfigCommon.initialUplinkBWP.rach_ConfigCommon.setup.groupBconfigured := cs_GroupBConfigured_Def; v_NR_UplinkBWP_List[0].Common.R15.rach_ConfigCommon.setup.groupBconfigured := valueof(cs_GroupBConfigured_Def); v_SpCellConfig.reconfigurationWithSync.spCellConfigCommon.uplinkConfigCommon.initialUplinkBWP.rach_ConfigCommon.setup.ra_ContentionResolutionTimer := sf48; v_NR_UplinkBWP_List[0].Common.R15.rach_ConfigCommon.setup.ra_ContentionResolutionTimer := sf48; f_NR_SS_CommonCellConfig(nr_Cell1, cads_NR_UplinkBWP_ConfigCommon_REQ(nr_Cell1, -, v_NR_UplinkBWP_List)); // @sic R5s200549 sic@ v_SCGConfig := cs_38508_CellGroupConfig (tsc_NR_CellGroupId_SCG, omit, omit, omit, omit, v_SpCellConfig //SpCellConfig ); f_NR_SendRRCReconfigurationContentsToEUTRA (omit, v_SCGConfig,-,-,-,reconfigurationWithSync_CheckRACH); //@sic R5s190541 sic@ // Now send octetstrings to EUTRA f_Delay(0.01); f_TC_7_1_1_1_2_NR_TestBody1(tsc_NR_DRB2); //@sic R5s201316 sic@ v_NR_PhysicalParameters := f_NR_CellInfo_GetPhysicalParameters(nr_Cell1); v_NR_RachProcedure := cs_NR_RachProcedureConfig_71112_2(v_NR_PhysicalParameters); f_NR_SS_RachProcedureConfig(nr_Cell1, v_NR_RachProcedure); f_NR_SendRRCReconfigurationContentsToEUTRA (omit, v_SCGConfig,-,-,-,reconfigurationWithSync_CheckRACH); //@sic R5s190541 sic@ // Now send octetstrings to EUTRA f_TC_7_1_1_1_2_NR_TestBody2(tsc_NR_DRB2); f_NR_TestBody_Set(false); // Send Trigger to EUTRA indicating Finish of Test Body f_EUTRA_NR_SendCoOrd(EUTRA, cms_EUTRA_NR_Trigger); //Wait from coordination AFTER the connection is released in E-UTRA f_EUTRA_NR_WaitForCoOrd_Trigger(EUTRA, "Postamble"); //Postamble f_NR_ReleaseAllCells(); }