3GPP TSG-RAN5 Meeting #2024-TTCN email	R5s240428

# Online, , 8th Dec 2023 - 31st Dec 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 3466 | rev | - | Current version: | 17.10.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction of NR5GC eCall over IMS testcase 11.5.5 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI16_Test, NR_EIEI-UEConTest |  |  |  |  |  | Date: |  |  | 2024-06-21 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Current implementation of eCall 11.5.5 test case will fail as variables v_PDUSessionInfoList and v_PDUSessionId are assigned before UE has Registered to NR cell, assiging no values to v_PDUSessionInfoList and causing a runtime error when converting int2oct v_PDUSessionId. This issue can be corrected doing the assigments after UE has registered in NR cell and PDUs have been established. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Assigned values to variables v_PDUSessionInfoList and v_PDUSessionId after f_NR5GC_EmergencyCallLimitedService function has been called. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE can fail this test case |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 11.5.5.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	2

2	Corrections required	2

2.1	Correction to function fl_TC_11_5_5_TestBody	2

- Overview

This document lists all the essential changes needed to correct problems in the iwd-TTCN3-B2022-09_D24wk24 related to the title of this CR.

Contact: 	Juan Garcia Martin

Juan.GarciaMartin@rohde-schwarz.com

- Corrections required

  - Correction to function fl_TC_11_5_5_TestBody

| Function name | fl_TC_11_5_5_TestBody |
| --- | --- |
| Reason for change | Current implementation of eCall 11.5.5 test case will fail as variables v_PDUSessionInfoList and v_PDUSessionId are assigned before UE has Registered to NR cell, assiging no values to v_PDUSessionInfoList and causing a runtime error when converting int2oct v_PDUSessionId. |
| Summary of change | Assigned values to variables v_PDUSessionInfoList and v_PDUSessionId after f_NR5GC_EmergencyCallLimitedService function has been called. |
| TTCN module | NR5GC\11.5\ECall_NR5GC.ttcn |
| MCC160 Comment | Accepted |

Before Change:

function fl_TC_11_5_5_TestBody() runs on NR5GC_PTC { var PDUSessionInfoList_Type v_PDUSessionInfoList := f_NR5GC_MobileInfo_GetSessionInfoList(); var O1_Type v_PDUSessionId := int2oct(f_Get_PDUSessionIdForDNNType(v_PDUSessionInfoList, Emergency_PDN), 1); var NR_SRB_COMMON_IND v_ReceivedAsp; var template (value) NG_NAS_DL_Pdu_Type v_MsgToSend; var default v_MyDefaultVar := null; var float v_Wait:= 5.0; // @sic R5-243594 sic@ //@siclog "Step 1" siclog@ f_NR5GC_SwitchOnAndResetIMS(); //@siclog "Step 2" siclog@ f_Delay(60.0); // @sic R5-227244 R5s240050 sic@ //@siclog "Step 3" siclog@ f_UT_InitiateECall(UT, MANUAL); //@siclog "Step 4A-4R" siclog@ f_NR5GC_EmergencyCallLimitedService(nr_Cell1); // @sic R5-227244 sic@ //@siclog "Step 5-5A" siclog@ f_IMS_IPCAN_SendCoOrdMsg(IMS[tsc_Index_IMS2]); v_MyDefaultVar := activate(a_NR5GC_Deregistration_REQ(nr_Cell1)); // @sic R5-243594 sic@ IMS[tsc_Index_IMS2].receive(cmr_IMS_IPCAN_Trigger ("IMS Call Release Msgs Sent")); // @sic R5s220039, R5-225362 sic@ sic@ // @siclog "Step 5B" siclog@ v_MsgToSend := f_Get_NG_PDUSessionReleaseCommand(v_PDUSessionId, cs_GMM_GSM_Cause (omit, '00100100'B)); SRB.send(cas_NR_SRB_NasPdu_REQ(nr_Cell1, tsc_NR_RbId_SRB2, -, cs_NG_NAS_RequestWithPiggybacking(tsc_SHT_IntegrityProtected_Ciphered, v_MsgToSend))); // @siclog "Step 5C" siclog@ SRB.receive(car_NR_SRB_NasPdu_IND(nr_Cell1, tsc_NR_RbId_SRB2, cr_NG_NAS_IndWithPiggybacking(tsc_SHT_IntegrityProtected_Ciphered))) -> value v_ReceivedAsp; if (not f_Check_NG_PDUSessionReleaseComplete (v_PDUSessionInfoList, v_ReceivedAsp.Signalling.Nas[0].Pdu, v_PDUSessionId)) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "PDU Session Release Complete failed"); } //@siclog "Step 7" siclog@ f_NR_RRCRelease(nr_Cell1); // @sic R5-227244 sic@ deactivate(v_MyDefaultVar); //@sic R5-243594 sic@ //@siclog "Step 8" siclog@ f_Delay(120.0); // @sic R5-243594 sic@ //@siclog "Step 9" siclog@ f_UT_InitiateECall(UT, TEST, -, LOCAL_CNF_REQUIRED); // @sic R5s230789 sic@ //@siclog "Step 10" siclog@ //Check: Does UE send an RRCConnectionRequest on NGC Cell A within 5 seconds? f_NR_CheckNoRRCSetupReq(nr_Cell1, v_Wait, -, "Step 10: RRCSetupRequest received"); // @sic R5-227244 sic@ }

After Change:

function fl_TC_11_5_5_TestBody() runs on NR5GC_PTC { var PDUSessionInfoList_Type v_PDUSessionInfoList; var O1_Type v_PDUSessionId; var NR_SRB_COMMON_IND v_ReceivedAsp; var template (value) NG_NAS_DL_Pdu_Type v_MsgToSend; var default v_MyDefaultVar := null; var float v_Wait:= 5.0; // @sic R5-243594 sic@ //@siclog "Step 1" siclog@ f_NR5GC_SwitchOnAndResetIMS(); //@siclog "Step 2" siclog@ f_Delay(60.0); // @sic R5-227244 R5s240050 sic@ //@siclog "Step 3" siclog@ f_UT_InitiateECall(UT, MANUAL); //@siclog "Step 4A-4R" siclog@ f_NR5GC_EmergencyCallLimitedService(nr_Cell1); // @sic R5-227244 sic@ //@siclog "Step 5-5A" siclog@ f_IMS_IPCAN_SendCoOrdMsg(IMS[tsc_Index_IMS2]); v_MyDefaultVar := activate(a_NR5GC_Deregistration_REQ(nr_Cell1)); // @sic R5-243594 sic@ IMS[tsc_Index_IMS2].receive(cmr_IMS_IPCAN_Trigger ("IMS Call Release Msgs Sent")); // @sic R5s220039, R5-225362 sic@ sic@ // @siclog "Step 5B" siclog@ v_PDUSessionInfoList := f_NR5GC_MobileInfo_GetSessionInfoList(); v_PDUSessionId := int2oct(f_Get_PDUSessionIdForDNNType(v_PDUSessionInfoList, Emergency_PDN), 1); v_MsgToSend := f_Get_NG_PDUSessionReleaseCommand(v_PDUSessionId, cs_GMM_GSM_Cause (omit, '00100100'B)); SRB.send(cas_NR_SRB_NasPdu_REQ(nr_Cell1, tsc_NR_RbId_SRB2, -, cs_NG_NAS_RequestWithPiggybacking(tsc_SHT_IntegrityProtected_Ciphered, v_MsgToSend))); // @siclog "Step 5C" siclog@ SRB.receive(car_NR_SRB_NasPdu_IND(nr_Cell1, tsc_NR_RbId_SRB2, cr_NG_NAS_IndWithPiggybacking(tsc_SHT_IntegrityProtected_Ciphered))) -> value v_ReceivedAsp; if (not f_Check_NG_PDUSessionReleaseComplete (v_PDUSessionInfoList, v_ReceivedAsp.Signalling.Nas[0].Pdu, v_PDUSessionId)) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "PDU Session Release Complete failed"); } //@siclog "Step 7" siclog@ f_NR_RRCRelease(nr_Cell1); // @sic R5-227244 sic@ deactivate(v_MyDefaultVar); //@sic R5-243594 sic@ //@siclog "Step 8" siclog@ f_Delay(120.0); // @sic R5-243594 sic@ //@siclog "Step 9" siclog@ f_UT_InitiateECall(UT, TEST, -, LOCAL_CNF_REQUIRED); // @sic R5s230789 sic@ //@siclog "Step 10" siclog@ //Check: Does UE send an RRCConnectionRequest on NGC Cell A within 5 seconds? f_NR_CheckNoRRCSetupReq(nr_Cell1, v_Wait, -, "Step 10: RRCSetupRequest received"); // @sic R5-227244 sic@ }