3GPP TSG-RAN5 Meeting #2024-TTCN email	R5s240435

# Online, , 8th Dec 2023 - 31st Dec 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.523-3 | CR | 4836 | rev | - | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to NBIOT-NTN testcase 22.3.1.13 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI17_Test, LTE_NBIOT_eMTC_NTN_plus_EPS-UEConTest |  |  |  |  |  | Date: |  |  | 2024-06-28 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | - In order to ensure that the RLC ACK is sent before going into transparent it is proposed to adde a delay. - Current TTCN is calling f_NBIOT_L2_CPMode_RestoreSRBs with activation time but the follow up functions are using immediate activation and this results in UE Capability information message being sent out before normal mode is restored. - At the end of the test case TTCN incorrectly assigns an inconclusive verdict. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Added f_Delay(int2float(f_NBIOT_NTN_Get_T_NTNoffset(nbiot_Cell1))/500.0) , after f_NBIOT_InitialRegistration_Step5_14 (Step 18A-18J) - Added a delay after updating the Koffset inorder to ensure that the MAC CE is sent out and update of Koffset is also configured in SS side. Called f_NBIOT_L2_CPMode_RestoreSRBs with immediate activation time - Remove the call for inconclusive verdict and assigned a pass verdict. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A Conformant UE may fail the test case |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 22.3.1.13 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Table of Contents

Table of Contents	3

1	Overview	3

2	Corrections required	3

2.1	Change to the f_TC_22_3_1_13_NBIOT	3

# Overview

This document lists all the essential changes needed to correct problems in the iwd-TTCN3-B2022-09_D24wk24 related to the title of this CR.

Contact: 	Parikshit Bhise

Parikshit.bhise@rohde-schwarz.com

# Corrections required

## Change to the f_TC_22_3_1_13_NBIOT

| Function name | f_TC_22_3_1_13_NBIOT |
| --- | --- |
| Reason for change | - In order to ensure that the RLC ACK is sent before going into transparent it is proposed to adde a delay. - Current TTCN is calling f_NBIOT_L2_CPMode_RestoreSRBs with activation time but the follow up functions are using immediate activation and this results in UE Capability information message being sent out before normal mode is restored. - At the end of the test case TTCN incorrectly assigns an inconclusive verdict. |
| Summary of change | - Added f_Delay(int2float(f_NBIOT_NTN_Get_T_NTNoffset(nbiot_Cell1))/500.0) , after f_NBIOT_InitialRegistration_Step5_14 (Step 18A-18J) - Added a delay after updating the Koffset inorder to ensure that the MAC CE is sent out and update of Koffset is also configured in SS side. Called f_NBIOT_L2_CPMode_RestoreSRBs with immediate activation time - Remove the call for inconclusive verdict and assigned a pass verdict. |
| TTCN module | NBIOT_MAC_NTN |
| MCC160 Comment | - Accepted - Accepted - Accepted |

Before Change:

function f_TC_22_3_1_13_NBIOT() runs on NBIOT_PTC {//NB-IoT / NTN / UE specific TA report / UE specific Koffset //@sic R5-241563, R5-234494 sic@ var template (value) SystemInformationBlockType2_NB_r13 v_SIB2; var NAS_MSG_Indication_Type v_NAS_Ind; var NB_RLC_CountsInfoList_Type v_RLC_CountsInfoListByRef; // RLC status to be maintained by Send/Receive functions var template (value) TimingInfo_Type v_TimingInfo; var SubFrameTiming_Type v_TimingStart; var NB_SYSTEM_IND v_NB_SYSTEM_IND; var NB_SRB_COMMON_IND v_NB_SRB_COMMON_IND; var template (value) DL_CCCH_Message_NB v_RrcConnectionSetup; var float v_ModificationPeriod_s; var integer v_Koffset; … … … … //@siclog "Step 17" siclog@ //The SS transmits an RRCConnectionSetup-NB message to configure the offsetThresholdTA, Table 22.3.1.13.3.3-2 //@siclog "Step 18" siclog@ //Check: Does the UE transmit an RRCConnectionSetupComplete-NB message with one Timing Advance Report MAC CE? interleave { [] SRB.receive(car_NB_SRB_RrcNasPdu_IND(nbiot_Cell1, tsc_SRB1bis, cr_508_RRCConnectionSetupComplete_NB(tsc_RRC_TI_Def,?),?)) -> value v_NB_SRB_COMMON_IND {} //@sic R5s240246 sic@ [] SYSIND.receive(car_NB_TARreport_IND(nbiot_Cell1, ?)) -> value v_NB_SYSTEM_IND {} } if (v_NB_SRB_COMMON_IND.Common.TimingInfo != v_NB_SYSTEM_IND.Common.TimingInfo) { f_NBIOT_SetVerdictFailOrInconc(__FILE__, __LINE__, "TA report is not received in the same MAC PDU as RRCConnectionSetupComplete-NB"); } else { f_NBIOT_PreliminaryPass(__FILE__, __LINE__, "Step 18") } v_NAS_Ind := v_NB_SRB_COMMON_IND.Signalling.Nas[0]; //@siclog "Step 18A-18J" siclog@ //Steps 5 to 14b1 of the Attach, connected mode procedure described in TS 36.508 [18] subclause 8.1.5.2.3 are performed on Ncell 1. f_NBIOT_InitialRegistration_Step5_14 (nbiot_Cell1, CONTROL_PLANE, STATE2_NB_CONNECTED_UPDATED, v_NAS_Ind); //switch to L2 test mode v_RLC_CountsInfoListByRef := f_NBIOT_L2_CPMode_StopULGrant_EnableSrbTestMode(nbiot_Cell1, TransparentMode_RLC_MAC); //@siclog "Step 19" siclog@ //The SS transmits a MAC PDU containing a Differential Koffset MAC CE with differential Koffset set to 5 ms. f_NBIOT_MAC_CPMode_SendMacPdu(nbiot_Cell1, -, cs_MAC_PDU_DifferentialKoffset(int2bit(5,6))); //@sic R5s240246, ch 2.1.9 sic@ v_TimingStart := f_NBIOT_GetNextSearchSpace(nbiot_Cell1); v_TimingInfo := cs_TimingInfo_SubFrameFull(v_TimingStart); v_Koffset := f_NBIOT_CellInfo_Get_Koffset(nbiot_Cell1) - 5; //@sic R5-241556 sic@ f_NBIOT_SS_CommonCellConfig(nbiot_Cell1, cas_NB_CellConfig_Update_NTNConfig(nbiot_Cell1, v_TimingInfo, -, cs_NB_NTN_Config(cs_NB_NTN_TimingOffset (v_Koffset)))); //@sic R5s240246, ch 2.1.9, R5-241556 sic@ //leave L2 test mode v_TimingInfo := cs_TimingInfo_SubFrameFull(f_NBIOT_IncrementSearchSpace(v_TimingStart, 2.0)); //@sic R5s240246, ch 2.1.10 sic@ f_NBIOT_L2_CPMode_RestoreSRBs(nbiot_Cell1, v_RLC_CountsInfoListByRef.AM_VTS, v_RLC_CountsInfoListByRef.AM_VRR, v_TimingInfo); //@sic R5s240246, ch 2.1.10 sic@ f_NBIOT_ULGrantTransmission(nbiot_Cell1, cs_NB_UL_GrantScheduling_Start); //@sic R5s240246 sic@ //@siclog "Steps 20-21" siclog@ //The SS sends UECapabilityEnquiry-NB message to the UE //Check: Does UE send a UECapabilityInformation-NB message after applying differential Koffset of 5ms received at step 19? f_NBIOT_Capability(nbiot_Cell1, CONTROL_PLANE); //Note: Successful reception of this message at SS confirms that differential Koffset is applied at UE. f_NBIOT_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 21"); f_NBIOT_TestBody_Set(false); //------------------------------------ f_NBIOT_Postamble(nbiot_Cell1, CONTROL_PLANE, N2_CONNECTED); }

After Change:

function f_TC_22_3_1_13_NBIOT() runs on NBIOT_PTC {//NB-IoT / NTN / UE specific TA report / UE specific Koffset //@sic R5-241563, R5-234494 sic@ var template (value) SystemInformationBlockType2_NB_r13 v_SIB2; var NAS_MSG_Indication_Type v_NAS_Ind; var NB_RLC_CountsInfoList_Type v_RLC_CountsInfoListByRef; // RLC status to be maintained by Send/Receive functions var template (value) TimingInfo_Type v_TimingInfo; var SubFrameTiming_Type v_TimingStart; var NB_SYSTEM_IND v_NB_SYSTEM_IND; var NB_SRB_COMMON_IND v_NB_SRB_COMMON_IND; var template (value) DL_CCCH_Message_NB v_RrcConnectionSetup; var float v_ModificationPeriod_s; var integer v_Koffset; … … … … //@siclog "Step 17" siclog@ //The SS transmits an RRCConnectionSetup-NB message to configure the offsetThresholdTA, Table 22.3.1.13.3.3-2 //@siclog "Step 18" siclog@ //Check: Does the UE transmit an RRCConnectionSetupComplete-NB message with one Timing Advance Report MAC CE? interleave { [] SRB.receive(car_NB_SRB_RrcNasPdu_IND(nbiot_Cell1, tsc_SRB1bis, cr_508_RRCConnectionSetupComplete_NB(tsc_RRC_TI_Def,?),?)) -> value v_NB_SRB_COMMON_IND {} //@sic R5s240246 sic@ [] SYSIND.receive(car_NB_TARreport_IND(nbiot_Cell1, ?)) -> value v_NB_SYSTEM_IND {} } if (v_NB_SRB_COMMON_IND.Common.TimingInfo != v_NB_SYSTEM_IND.Common.TimingInfo) { f_NBIOT_SetVerdictFailOrInconc(__FILE__, __LINE__, "TA report is not received in the same MAC PDU as RRCConnectionSetupComplete-NB"); } else { f_NBIOT_PreliminaryPass(__FILE__, __LINE__, "Step 18") } v_NAS_Ind := v_NB_SRB_COMMON_IND.Signalling.Nas[0]; //@siclog "Step 18A-18J" siclog@ //Steps 5 to 14b1 of the Attach, connected mode procedure described in TS 36.508 [18] subclause 8.1.5.2.3 are performed on Ncell 1. f_NBIOT_InitialRegistration_Step5_14 (nbiot_Cell1, CONTROL_PLANE, STATE2_NB_CONNECTED_UPDATED, v_NAS_Ind); f_Delay(int2float(f_NBIOT_NTN_Get_T_NTNoffset(nbiot_Cell1))/500.0); // added delay to ensure RLC ACK is sent before moving to transparent mode //switch to L2 test mode v_RLC_CountsInfoListByRef := f_NBIOT_L2_CPMode_StopULGrant_EnableSrbTestMode(nbiot_Cell1, TransparentMode_RLC_MAC); //@siclog "Step 19" siclog@ //The SS transmits a MAC PDU containing a Differential Koffset MAC CE with differential Koffset set to 5 ms. f_NBIOT_MAC_CPMode_SendMacPdu(nbiot_Cell1, -, cs_MAC_PDU_DifferentialKoffset(int2bit(5,6))); //@sic R5s240246, ch 2.1.9 sic@ v_TimingStart := f_NBIOT_GetNextSearchSpace(nbiot_Cell1); v_TimingInfo := cs_TimingInfo_SubFrameFull(v_TimingStart); v_Koffset := f_NBIOT_CellInfo_Get_Koffset(nbiot_Cell1) - 5; //@sic R5-241556 sic@ f_NBIOT_SS_CommonCellConfig(nbiot_Cell1, cas_NB_CellConfig_Update_NTNConfig(nbiot_Cell1, v_TimingInfo, -, cs_NB_NTN_Config(cs_NB_NTN_TimingOffset (v_Koffset)))); //@sic R5s240246, ch 2.1.9, R5-241556 sic@ // added delay and remove activation time, as the sequence of events have to happen with immediate activation time for Step 20-21 to be sent immediately after restoring SRBs to normal mode f_Delay(2.0); //leave L2 test mode //v_TimingInfo := cs_TimingInfo_SubFrameFull(f_NBIOT_IncrementSearchSpace(v_TimingStart, 2.0)); //@sic R5s240246, ch 2.1.10 sic@ f_NBIOT_L2_CPMode_RestoreSRBs(nbiot_Cell1, v_RLC_CountsInfoListByRef.AM_VTS, v_RLC_CountsInfoListByRef.AM_VRR); //@sic R5s240246, ch 2.1.10 sic@ f_NBIOT_ULGrantTransmission(nbiot_Cell1, cs_NB_UL_GrantScheduling_Start); //@sic R5s240246 sic@ //@siclog "Steps 20-21" siclog@ //The SS sends UECapabilityEnquiry-NB message to the UE //Check: Does UE send a UECapabilityInformation-NB message after applying differential Koffset of 5ms received at step 19? f_NBIOT_Capability(nbiot_Cell1, CONTROL_PLANE); //Note: Successful reception of this message at SS confirms that differential Koffset is applied at UE. //f_NBIOT_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 21"); f_NBIOT_PreliminaryPass(__FILE__, __LINE__, "Step 21"); // pass verdict needs to be assigned f_NBIOT_TestBody_Set(false); //------------------------------------ f_NBIOT_Postamble(nbiot_Cell1, CONTROL_PLANE, N2_CONNECTED); }