3GPP TSG-RAN5 Meeting #2024-TTCN email	R5s240436

# Online, , 8th Dec 2023 - 31st Dec 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 3470 | rev | - | Current version: | 17.10.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to NR5GC Redcap MAC test case 7.1.1.8.3 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies UK Ltd |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI17_Test, NR_redcap_plus_ARCH-UEConTest, |  |  |  |  |  | Date: |  |  | 2024-06-30 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The current TTCN implementation of Step 5-7 doesn’t disable Harq Ack/Nack Reporting even after receiving Expected Harq Indication from device. In the current implementation Harq Ack/Nack reporting is only stopped if Harq Ack is received already before RRCSetupComplete is handled by TTCN. However, due to race conditions, different events could arrive in different order, hence it is not always guaranteed that expected Harq Ack will arrive TTCN always before RRCConnectionSetupComplete message. This could lead to Test case failure after step 7, since unexpcted Harq Ack/Nack shall be reported to TTCN at Steps 8-13a. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added call to f_NR_SS_SystemIndCtrlConfig to disable UL Harq reporting after Step 7. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may fail the test case |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.1.1.8.3.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

2	Corrections required	3

2.1	Change 1	3

# Overview

This document lists all the changes needed to correct issues in the ATS iwd-TTCN3-B2022-09_D24wk24 related to the title of this CR.

Contact:	Mohit Kanchan
	mohit.kanchan@keysight.com

# Corrections required

## Change 1

| Function name | f_TC_7_1_1_8_3_NR_TestBody () |
| --- | --- |
| Reason for change | The current TTCN implementation of Step 5-7, doesn’t disable Harq Ack/Nack Reporting even after receiving Expected Harq Indication from device. In the current implementation Harq Ack/Nack reporting is only stopped if Harq Ack is received already before RRCSetupComplete is handled by TTCN. However, due to race conditions, different events could arrive in different order, hence it is not always guaranteed that expected Harq Ack will arrive TTCN always before RRCConnectionSetupComplete message. This could lead to Test case failure after step 7, since unexpcted Harq Ack/Nack shall be reported to TTCN at Steps 8-13a. |
| Summary of change | Added call to f_NR_SS_SystemIndCtrlConfig to disable UL Harq reporting after Step 7. |
| TTCN module | MAC_BWP_NR5GC.ttcn |
| MCC160 | Accepted. To avoid disabling twice when the UL HARD is received before the RRCSetupComplete, it is proposed to add a variable – see MCC 160 proposed implementation below. |

Before Change

function f_TC_7_1_1_8_3_NR_TestBody() runs on NR5GC_PTC { //@sic R5s240142 sic@ var template (value) BWP_DownlinkCommon v_BWP_DownlinkCommon; var template (value) NR_DownlinkBWP_Type v_NR_DownlinkBWP; var template (value) NR_UplinkBWP_Type v_NR_UplinkBWP; //@sic R5s240142 sic@ var template (value) NR_SearchSpace_NrofCandidates_Type v_NrofCandidates; var template (omit) NR_PcchConfig_Type v_PcchConfig:= f_NR_InitialisePcchConfig(nr_Cell2); var NG_NAS_GutiParameters_Type v_GutiParameters := f_NR5GC_CellInfo_GetGutiParameters(nr_Cell2); var NG_5G_S_TMSI v_NG_5G_S_TMSI := f_NR_Get5G_S_TMSI_FromGuti(v_GutiParameters); var NG_NAS_MSG_Indication_Type v_NasInd; var boolean v_RRCSetupRequest_Received := false; var boolean v_RRCSetupRequestComplete_Received := false; var boolean v_LCID_Received := false; var boolean v_UL_HARQ_Received := false; var NR_SRB_COMMON_IND v_SRB_COMMON_IND; var NR_SYSTEM_IND v_SYSTEM_IND; var SubFrameTiming_Type v_TimingRRCSetupRequest; var SubFrameTiming_Type v_Timing_UL_HARQ; //Initialise RedCap specific Initial UL BWP in SIB1 //@sic R5s240142 sic@ v_NR_UplinkBWP := f_GetInitialUplinkBWP_RedCap(nr_Cell2, cs_PUCCH_Config_Def(omit, enable, omit, -90, omit, fromLowerEdge, 1, n2), omit); f_NR_CellInfo_SetSIB1_InitialUplinkBWP_RedCap(nr_Cell2, v_NR_UplinkBWP.Common.R15); //Initialise RedCap specific Initial DL BWP //Initialise RedCap initial DL BWP in SIB1 if (f_NR_CellInfo_GetIsFR1(nr_Cell2)) { //FR1 v_NrofCandidates := cs_38508_NrofCandidates_Def; } else { //FR2 v_NrofCandidates := cds_38508_NrofCandidates_FR2 } v_BWP_DownlinkCommon := fl_Get_InitialDownlinkBWP_RedCap(nr_Cell2,cs_PDCCH_ConfigCommon_71183(v_NrofCandidates)); // Add RedCap-specific initial DL BWP in SIB1 in the Cell Array f_NR_CellInfo_SetSIB1_InitialDownlinkBWP_RedCap(nr_Cell2, v_BWP_DownlinkCommon); //Initialise RedCap initialBWP for SS // The RedCap-Specific initial downlink BWP information is provided in the cell reconfiguration ASP with: // BWP Id 0 // DL SearchSpace Array for: cssType0 (SIB1), cssType1 (Msg2/Msg4) after initial access, cssType2 (paging), uss v_NR_DownlinkBWP := fl_GetInitialNR_DownlinkBWP_RedCap(nr_Cell2, v_NrofCandidates); // The AssignedBWPs are reconfigured from InitialBWP to InitialBWP_RedCap cell reconfiguration ASP with: SIB1, RandomAccessResponse, ContentionResolution(Msg4_Based), PcchConfig v_PcchConfig.SearchSpaceAndDci.AssignedBWPs := cs_NR_AssignedBWPs_RedCapInitialBWP; // @siclog "Steps 1-2" siclog@ // The SS changes the SIB1 of NR Cell 1 2 to configure initialDownlinkBWP-RedCap and initialUplinkBWP-RedCap. // The SS transmits a Short message on PDCCH using P-RNTI indicating a systemInfoModification. f_NR_ModifySysinfo(nr_Cell2); // @siclog "Step 3" siclog@ // Wait for 2.1* modification period second for the UE to receive new system information. f_NR_WaitModificationPeriods(nr_Cell2); //Configure SS to report UL_SRB0_LCID and UL HARQ feedback f_NR_SS_SystemIndCtrlConfig(nr_Cell2, cds_NR_SystemIndCtrl_UL_SRB0_LCID(enable)); f_NR_SS_SystemIndCtrlConfig(nr_Cell2, cds_NR_SystemIndCtrl_UL_HARQ(enable)); // Reconfigure SS - RedCap initial DL BWP //Acc to 38.331 cl initialDownlinkBWP-RedCap: If the locationAndBandwidth of this BWP contains the entire CORESET#0, the UE applies the locationAndBandwidth upon reception of this field (e.g. to determine the frequency position of signals described in relation to this locationAndBandwidth) but it keeps CORESET#0 until after reception of RRCSetup/RRCResume/RRCReestablishment. // RACH Msg2/Msg4 to be reconfigured after initial access - not needed in this TC //@sic R5s240142 sic@ f_NR_SS_CommonCellConfig(nr_Cell2, cads_NR_BWP_CONFIG_DL_RedCap_REQ(nr_Cell2, -, // cs_TimingInfo_Now v_NR_DownlinkBWP, omit, //v_BcchToPdschConfig, v_PcchConfig, omit)); // Reconfigure SS - RedCap initial UL BWP f_NR_SS_CommonCellConfig(nr_Cell2, cads_NR_BWP_CONFIG_UL_RedCap_REQ(nr_Cell2, -, // cs_TimingInfo_Now, v_NR_UplinkBWP )); // @siclog "Step 4" siclog@ //The SS transmits a PagingPaging message within the BWP configured by initialDownlinkBWP-RedCap. f_NR_Paging(nr_Cell2, cs_PagingRecordList_OneRecord(cs_PagingUE_Identity_5G_S_TMSI(v_NG_5G_S_TMSI)), rf128, 2); //@sic R5s240142 sic@ f_NR_ULGrantConfiguration_Start(nr_Cell2, -, -, -, cs_NR_UplinkTimeAlignment_Stop, -, -, tsc_NoCnfReq ); // @siclog "Step 5" siclog@ // Check: Does the UE transmit an RRCSetupRequest message by setting LCID to 35 // @siclog "Step 6" siclog@ // The SS transmits an RRCSetup message. // SS configured to do it // @siclog "Step 7" siclog@ // Check: Does the UE transmit HARQ ACK for step 6 on PUCCH configured by initialUplinkBWP-RedCap? alt { // RRCSetupRequest [not v_RRCSetupRequest_Received] SRB.receive (car_NR_SRB0_RrcPdu_IND(nr_Cell2, cr_38508_RRCSetupRequest)) -> value v_SRB_COMMON_IND { v_RRCSetupRequest_Received := true; v_TimingRRCSetupRequest := valueof(v_SRB_COMMON_IND.Common.TimingInfo.SubFrame); if (not (v_RRCSetupRequestComplete_Received and v_LCID_Received and v_UL_HARQ_Received)) { //Other events not all received repeat; // => wait for other events } } //RRCSetupComplete [not v_RRCSetupRequestComplete_Received] SRB.receive (car_NR_SRB1_RrcNasPdu_IND(nr_Cell2, cr_38508_RRCSetupComplete(tsc_NR_RRC_TI_Def), cr_NG_NAS_IndWithPiggybacking(tsc_SHT_IntegrityProtected), omit)) -> value v_SRB_COMMON_IND { v_RRCSetupRequestComplete_Received := true; if(v_UL_HARQ_Received) { //@sic R5s240286 sic@ f_NR_SS_SystemIndCtrlConfig(nr_Cell2, cds_NR_SystemIndCtrl_UL_HARQ(disable)); } v_NasInd := valueof (v_SRB_COMMON_IND.Signalling.Nas[0]); if (not (v_RRCSetupRequest_Received and v_LCID_Received and v_UL_HARQ_Received)) { //Other events not all received repeat; // => wait for other events } } // SysInd LCID [not v_LCID_Received] SYSIND.receive (cr_NR_SystemIndicationUL_SRB0_LCID_IND(nr_Cell2, cr_TimingInfo_Any, 35)) -> value v_SYSTEM_IND { v_LCID_Received := true; if (not (v_RRCSetupRequest_Received and v_RRCSetupRequestComplete_Received and v_UL_HARQ_Received)) { //Other events not all received repeat; // => wait for other events } } // SysInd UL HARQ [not v_UL_HARQ_Received] SYSIND.receive (car_NR_SYSTEM_UL_HARQ_IND(nr_Cell2, ack)) -> value v_SYSTEM_IND { v_UL_HARQ_Received := true; v_Timing_UL_HARQ := valueof(v_SYSTEM_IND.Common.TimingInfo.SubFrame); //Check: Does the UE transmit HARQ ACK for step 6 on PUCCH configured by initialUplinkBWP-RedCap? f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 7"); if (not (v_RRCSetupRequest_Received and v_RRCSetupRequestComplete_Received and v_LCID_Received)) { //Other events not all received repeat; // => wait for other events } } } //Check timing of events f_NR_RacingCond_CheckTwoEvents(nr_Cell2, v_TimingRRCSetupRequest, v_Timing_UL_HARQ); //Check: Does the UE transmit an RRCSetupRequest message by setting LCID to 35? if (v_RRCSetupRequest_Received and v_LCID_Received) { f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 5"); } else { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 5"); } // Disable SRB0 LCID indication // @sic R5s240286 sic@ f_NR_SS_SystemIndCtrlConfig(nr_Cell2, cds_NR_SystemIndCtrl_UL_SRB0_LCID(disable)); [New Change Here] // @siclog "Steps 8-13a1" siclog@ f_NR5GC_RRC_ConnectedState3N_Steps5_8(nr_Cell2, v_NasInd); }

After Change

function f_TC_7_1_1_8_3_NR_TestBody() runs on NR5GC_PTC { //@sic R5s240142 sic@ var template (value) BWP_DownlinkCommon v_BWP_DownlinkCommon; var template (value) NR_DownlinkBWP_Type v_NR_DownlinkBWP; var template (value) NR_UplinkBWP_Type v_NR_UplinkBWP; //@sic R5s240142 sic@ var template (value) NR_SearchSpace_NrofCandidates_Type v_NrofCandidates; var template (omit) NR_PcchConfig_Type v_PcchConfig:= f_NR_InitialisePcchConfig(nr_Cell2); var NG_NAS_GutiParameters_Type v_GutiParameters := f_NR5GC_CellInfo_GetGutiParameters(nr_Cell2); var NG_5G_S_TMSI v_NG_5G_S_TMSI := f_NR_Get5G_S_TMSI_FromGuti(v_GutiParameters); var NG_NAS_MSG_Indication_Type v_NasInd; var boolean v_RRCSetupRequest_Received := false; var boolean v_RRCSetupRequestComplete_Received := false; var boolean v_LCID_Received := false; var boolean v_UL_HARQ_Received := false; var NR_SRB_COMMON_IND v_SRB_COMMON_IND; var NR_SYSTEM_IND v_SYSTEM_IND; var SubFrameTiming_Type v_TimingRRCSetupRequest; var SubFrameTiming_Type v_Timing_UL_HARQ; //Initialise RedCap specific Initial UL BWP in SIB1 //@sic R5s240142 sic@ v_NR_UplinkBWP := f_GetInitialUplinkBWP_RedCap(nr_Cell2, cs_PUCCH_Config_Def(omit, enable, omit, -90, omit, fromLowerEdge, 1, n2), omit); f_NR_CellInfo_SetSIB1_InitialUplinkBWP_RedCap(nr_Cell2, v_NR_UplinkBWP.Common.R15); //Initialise RedCap specific Initial DL BWP //Initialise RedCap initial DL BWP in SIB1 if (f_NR_CellInfo_GetIsFR1(nr_Cell2)) { //FR1 v_NrofCandidates := cs_38508_NrofCandidates_Def; } else { //FR2 v_NrofCandidates := cds_38508_NrofCandidates_FR2 } v_BWP_DownlinkCommon := fl_Get_InitialDownlinkBWP_RedCap(nr_Cell2,cs_PDCCH_ConfigCommon_71183(v_NrofCandidates)); // Add RedCap-specific initial DL BWP in SIB1 in the Cell Array f_NR_CellInfo_SetSIB1_InitialDownlinkBWP_RedCap(nr_Cell2, v_BWP_DownlinkCommon); //Initialise RedCap initialBWP for SS // The RedCap-Specific initial downlink BWP information is provided in the cell reconfiguration ASP with: // BWP Id 0 // DL SearchSpace Array for: cssType0 (SIB1), cssType1 (Msg2/Msg4) after initial access, cssType2 (paging), uss v_NR_DownlinkBWP := fl_GetInitialNR_DownlinkBWP_RedCap(nr_Cell2, v_NrofCandidates); // The AssignedBWPs are reconfigured from InitialBWP to InitialBWP_RedCap cell reconfiguration ASP with: SIB1, RandomAccessResponse, ContentionResolution(Msg4_Based), PcchConfig v_PcchConfig.SearchSpaceAndDci.AssignedBWPs := cs_NR_AssignedBWPs_RedCapInitialBWP; // @siclog "Steps 1-2" siclog@ // The SS changes the SIB1 of NR Cell 1 2 to configure initialDownlinkBWP-RedCap and initialUplinkBWP-RedCap. // The SS transmits a Short message on PDCCH using P-RNTI indicating a systemInfoModification. f_NR_ModifySysinfo(nr_Cell2); // @siclog "Step 3" siclog@ // Wait for 2.1* modification period second for the UE to receive new system information. f_NR_WaitModificationPeriods(nr_Cell2); //Configure SS to report UL_SRB0_LCID and UL HARQ feedback f_NR_SS_SystemIndCtrlConfig(nr_Cell2, cds_NR_SystemIndCtrl_UL_SRB0_LCID(enable)); f_NR_SS_SystemIndCtrlConfig(nr_Cell2, cds_NR_SystemIndCtrl_UL_HARQ(enable)); // Reconfigure SS - RedCap initial DL BWP //Acc to 38.331 cl initialDownlinkBWP-RedCap: If the locationAndBandwidth of this BWP contains the entire CORESET#0, the UE applies the locationAndBandwidth upon reception of this field (e.g. to determine the frequency position of signals described in relation to this locationAndBandwidth) but it keeps CORESET#0 until after reception of RRCSetup/RRCResume/RRCReestablishment. // RACH Msg2/Msg4 to be reconfigured after initial access - not needed in this TC //@sic R5s240142 sic@ f_NR_SS_CommonCellConfig(nr_Cell2, cads_NR_BWP_CONFIG_DL_RedCap_REQ(nr_Cell2, -, // cs_TimingInfo_Now v_NR_DownlinkBWP, omit, //v_BcchToPdschConfig, v_PcchConfig, omit)); // Reconfigure SS - RedCap initial UL BWP f_NR_SS_CommonCellConfig(nr_Cell2, cads_NR_BWP_CONFIG_UL_RedCap_REQ(nr_Cell2, -, // cs_TimingInfo_Now, v_NR_UplinkBWP )); // @siclog "Step 4" siclog@ //The SS transmits a PagingPaging message within the BWP configured by initialDownlinkBWP-RedCap. f_NR_Paging(nr_Cell2, cs_PagingRecordList_OneRecord(cs_PagingUE_Identity_5G_S_TMSI(v_NG_5G_S_TMSI)), rf128, 2); //@sic R5s240142 sic@ f_NR_ULGrantConfiguration_Start(nr_Cell2, -, -, -, cs_NR_UplinkTimeAlignment_Stop, -, -, tsc_NoCnfReq ); // @siclog "Step 5" siclog@ // Check: Does the UE transmit an RRCSetupRequest message by setting LCID to 35 // @siclog "Step 6" siclog@ // The SS transmits an RRCSetup message. // SS configured to do it // @siclog "Step 7" siclog@ // Check: Does the UE transmit HARQ ACK for step 6 on PUCCH configured by initialUplinkBWP-RedCap? alt { // RRCSetupRequest [not v_RRCSetupRequest_Received] SRB.receive (car_NR_SRB0_RrcPdu_IND(nr_Cell2, cr_38508_RRCSetupRequest)) -> value v_SRB_COMMON_IND { v_RRCSetupRequest_Received := true; v_TimingRRCSetupRequest := valueof(v_SRB_COMMON_IND.Common.TimingInfo.SubFrame); if (not (v_RRCSetupRequestComplete_Received and v_LCID_Received and v_UL_HARQ_Received)) { //Other events not all received repeat; // => wait for other events } } //RRCSetupComplete [not v_RRCSetupRequestComplete_Received] SRB.receive (car_NR_SRB1_RrcNasPdu_IND(nr_Cell2, cr_38508_RRCSetupComplete(tsc_NR_RRC_TI_Def), cr_NG_NAS_IndWithPiggybacking(tsc_SHT_IntegrityProtected), omit)) -> value v_SRB_COMMON_IND { v_RRCSetupRequestComplete_Received := true; if(v_UL_HARQ_Received) { //@sic R5s240286 sic@ f_NR_SS_SystemIndCtrlConfig(nr_Cell2, cds_NR_SystemIndCtrl_UL_HARQ(disable)); } v_NasInd := valueof (v_SRB_COMMON_IND.Signalling.Nas[0]); if (not (v_RRCSetupRequest_Received and v_LCID_Received and v_UL_HARQ_Received)) { //Other events not all received repeat; // => wait for other events } } // SysInd LCID [not v_LCID_Received] SYSIND.receive (cr_NR_SystemIndicationUL_SRB0_LCID_IND(nr_Cell2, cr_TimingInfo_Any, 35)) -> value v_SYSTEM_IND { v_LCID_Received := true; if (not (v_RRCSetupRequest_Received and v_RRCSetupRequestComplete_Received and v_UL_HARQ_Received)) { //Other events not all received repeat; // => wait for other events } } // SysInd UL HARQ [not v_UL_HARQ_Received] SYSIND.receive (car_NR_SYSTEM_UL_HARQ_IND(nr_Cell2, ack)) -> value v_SYSTEM_IND { v_UL_HARQ_Received := true; v_Timing_UL_HARQ := valueof(v_SYSTEM_IND.Common.TimingInfo.SubFrame); //Check: Does the UE transmit HARQ ACK for step 6 on PUCCH configured by initialUplinkBWP-RedCap? f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 7"); if (not (v_RRCSetupRequest_Received and v_RRCSetupRequestComplete_Received and v_LCID_Received)) { //Other events not all received repeat; // => wait for other events } } } //Check timing of events f_NR_RacingCond_CheckTwoEvents(nr_Cell2, v_TimingRRCSetupRequest, v_Timing_UL_HARQ); //Check: Does the UE transmit an RRCSetupRequest message by setting LCID to 35? if (v_RRCSetupRequest_Received and v_LCID_Received) { f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 5"); } else { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 5"); } // Disable SRB0 LCID indication // @sic R5s240286 sic@ f_NR_SS_SystemIndCtrlConfig(nr_Cell2, cds_NR_SystemIndCtrl_UL_SRB0_LCID(disable)); f_NR_SS_SystemIndCtrlConfig(nr_Cell2, cds_NR_SystemIndCtrl_UL_HARQ(disable)); // @siclog "Steps 8-13a1" siclog@ f_NR5GC_RRC_ConnectedState3N_Steps5_8(nr_Cell2, v_NasInd); }

MCC160 proposed implementation

function f_TC_7_1_1_8_3_NR_TestBody() runs on NR5GC_PTC

{ //@sic R5s240142 sic@

var template (value) BWP_DownlinkCommon v_BWP_DownlinkCommon;

var template (value) NR_DownlinkBWP_Type v_NR_DownlinkBWP;

var template (value) NR_UplinkBWP_Type v_NR_UplinkBWP;     //@sic R5s240142 sic@

var template (value) NR_SearchSpace_NrofCandidates_Type v_NrofCandidates;

var template (omit) NR_PcchConfig_Type v_PcchConfig:= f_NR_InitialisePcchConfig(nr_Cell2);

var NG_NAS_GutiParameters_Type v_GutiParameters := f_NR5GC_CellInfo_GetGutiParameters(nr_Cell2);

var NG_5G_S_TMSI v_NG_5G_S_TMSI := f_NR_Get5G_S_TMSI_FromGuti(v_GutiParameters);

var NG_NAS_MSG_Indication_Type v_NasInd;

var boolean v_RRCSetupRequest_Received := false;

var boolean v_RRCSetupRequestComplete_Received := false;

var boolean v_LCID_Received := false;

var boolean v_UL_HARQ_Received := false;

var NR_SRB_COMMON_IND v_SRB_COMMON_IND;

var NR_SYSTEM_IND v_SYSTEM_IND;

var SubFrameTiming_Type v_TimingRRCSetupRequest;

var SubFrameTiming_Type v_Timing_UL_HARQ;

var boolean v_UL_HarqReportingDisabled := false;  //@sic R5s240436 sic@

//Initialise RedCap specific Initial UL BWP in SIB1

//@sic R5s240142 sic@

v_NR_UplinkBWP := f_GetInitialUplinkBWP_RedCap(nr_Cell2, cs_PUCCH_Config_Def(omit, enable, omit, -90, omit, fromLowerEdge, 1, n2), omit);

f_NR_CellInfo_SetSIB1_InitialUplinkBWP_RedCap(nr_Cell2, v_NR_UplinkBWP.Common.R15);

…

// @siclog "Step 5" siclog@

// Check: Does the UE transmit an RRCSetupRequest message by setting LCID to 35

// @siclog "Step 6" siclog@

// The SS transmits an RRCSetup message.

// SS configured to do it

// @siclog "Step 7" siclog@

// Check: Does the UE transmit HARQ ACK for step 6 on PUCCH configured by initialUplinkBWP-RedCap?

alt {

// RRCSetupRequest

[not v_RRCSetupRequest_Received] SRB.receive (car_NR_SRB0_RrcPdu_IND(nr_Cell2, cr_38508_RRCSetupRequest)) -> value v_SRB_COMMON_IND

{

v_RRCSetupRequest_Received := true;

v_TimingRRCSetupRequest := valueof(v_SRB_COMMON_IND.Common.TimingInfo.SubFrame);

if (not (v_RRCSetupRequestComplete_Received and v_LCID_Received and v_UL_HARQ_Received)) { //Other events not all received

repeat;        // => wait for other events

}

}

//RRCSetupComplete

[not v_RRCSetupRequestComplete_Received] SRB.receive (car_NR_SRB1_RrcNasPdu_IND(nr_Cell2, cr_38508_RRCSetupComplete(tsc_NR_RRC_TI_Def), cr_NG_NAS_IndWithPiggybacking(tsc_SHT_IntegrityProtected), omit)) -> value v_SRB_COMMON_IND

{

v_RRCSetupRequestComplete_Received := true;

if(v_UL_HARQ_Received) { //@sic R5s240286 sic@

f_NR_SS_SystemIndCtrlConfig(nr_Cell2, cds_NR_SystemIndCtrl_UL_HARQ(disable));

v_UL_HarqReportingDisabled := true;

}

v_NasInd  := valueof (v_SRB_COMMON_IND.Signalling.Nas[0]);

if (not (v_RRCSetupRequest_Received and v_LCID_Received and v_UL_HARQ_Received)) { //Other events not all received

repeat;        // => wait for other events

}

}

// SysInd LCID

[not v_LCID_Received] SYSIND.receive (cr_NR_SystemIndicationUL_SRB0_LCID_IND(nr_Cell2, cr_TimingInfo_Any, 35)) -> value v_SYSTEM_IND

{

v_LCID_Received := true;

if (not (v_RRCSetupRequest_Received and v_RRCSetupRequestComplete_Received and v_UL_HARQ_Received)) { //Other events not all received

repeat;        // => wait for other events

}

}

// SysInd UL HARQ

[not v_UL_HARQ_Received] SYSIND.receive (car_NR_SYSTEM_UL_HARQ_IND(nr_Cell2, ack)) -> value v_SYSTEM_IND

{

v_UL_HARQ_Received := true;

v_Timing_UL_HARQ := valueof(v_SYSTEM_IND.Common.TimingInfo.SubFrame);

//Check: Does the UE transmit HARQ ACK for step 6 on PUCCH configured by initialUplinkBWP-RedCap?

f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 7");

if (not (v_RRCSetupRequest_Received and v_RRCSetupRequestComplete_Received and v_LCID_Received)) { //Other events not all received

repeat;        // => wait for other events

}

}

}

//Check timing of events

f_NR_RacingCond_CheckTwoEvents(nr_Cell2, v_TimingRRCSetupRequest, v_Timing_UL_HARQ);

//Check: Does the UE transmit an RRCSetupRequest message by setting LCID to 35?

if (v_RRCSetupRequest_Received and v_LCID_Received) {

f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 5");

} else {

f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 5");

}

//Disable UL HARQ ind reporting (in case it wasn't done in the alt due to v_UL_HARQ_Received=false)

if (not(v_UL_HarqReportingDisabled)) {

f_NR_SS_SystemIndCtrlConfig(nr_Cell2, cds_NR_SystemIndCtrl_UL_HARQ(disable));  //@sic R5s240436 sic@

}

// Disable SRB0 LCID indication

// @sic R5s240286 sic@

f_NR_SS_SystemIndCtrlConfig(nr_Cell2, cds_NR_SystemIndCtrl_UL_SRB0_LCID(disable));

// @siclog "Steps 8-13a1" siclog@

f_NR5GC_RRC_ConnectedState3N_Steps5_8(nr_Cell2, v_NasInd);

}