3GPP TSG-RAN5 Meeting #2024-TTCN email	R5s240439

# Online, , 8th Dec 2023 - 31st Dec 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 3472 | rev | - | Current version: | 17.10.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to NR5GC testcase 11.4.12 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2024-07-01 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | In the current TTCN implementation, at Step 4 the variable v_UENWCap is set as IfPresent. If this is not present, then the argument valueof(v_UENWCap) will cause a runtime error. This needs to be addressed. In the current TTCN implementation the template used for capturing the TAU REQ at Step 22a4 expects the IE ueStatus to be present. This is fine for a case when the UE is transitioning from N1S1. However, in the current case the UE has already been on Eutra cell for the emergency call, and after releasing the call the UE performs a TAU to gain full services on the Eutra cell. Thus, this is not a N1S1 transition, and hence the IE ueStatus may be omitted by a conformant UE. This needs to be accommodated. After the SS receives TAU Complete at Step 22a6, according to the Prose SS should release the rrc connection. This is not implemented, and causes failure in postamble. This needs to be corrected. |  |  |  |  |  |  |  |  |
| Summary of change: |  | In the call to function f_EUTRA38_TAUReqFromN1, instead of passing valueof(v_UENWCap) passed v_UENWCap Created a new template cr_TAU_Request_11_4_12 and used it instead of cr_TAU_Request_N1ToS1. Please see below. Called the function f_EUTRA_RRC_ConnectionRelease after Step 22a6. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE can fail the test cases |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 11.4.12.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	3

1	Overview	3

2	Corrections required	3

2.1 	Change 1	3

2.1	Change 2	9

# Overview

This document lists all the essential changes needed to correct problems in the iwd-TTCN3-B2022-09_D24wk24 related to the title of this CR.

Contact: 	Parikshit Bhise

Parikshit.bhise@rohde-schwarz.com

# Corrections required

## 2.1 Change 1

| Function name | fl_TC_11_4_12_NR5GC_EUTRA_TestBody |
| --- | --- |
| Reason for change | In the current TTCN implementation, at Step 4 the variable v_UENWCap is set as IfPresent. If this is not present, then the argument valueof(v_UENWCap) will cause a runtime error. This needs to be addressed. In the current TTCN implementation the template used for capturing the TAU REQ at Step 22a4 expects the IE ueStatus to be present. This is fine for a case when the UE is transitioning from N1S1. However, in the current case the UE has already been on Eutra cell for the emergency call, and after releasing the call the UE performs a TAU to gain full services on the Eutra cell. Thus, this is not a N1S1 transition, and hence the IE ueStatus may be omitted by a conformant UE. This needs to be accommodated. After the SS receives TAU Complete at Step 22a6, according to the Prose SS should release the rrc connection. This is not implemented, and causes failure in postamble. This needs to be corrected. |
| Summary of change | In the call to function f_EUTRA38_TAUReqFromN1, instead of passing valueof(v_UENWCap) passed v_UENWCap Created a new template cr_TAU_Request_11_4_12 and used it instead of cr_TAU_Request_N1ToS1, where the IE ueStatus is set as IfPresent. Please see below and also Change 2. Called the function f_EUTRA_RRC_ConnectionRelease after Step 22a6. |
| TTCN module | Emergency_IRAT_NR5GC_EUTRA.ttcn |
| MCC160 Comments | 1. Accepted but also changed if statement in f_EUTRA38_TAUReqFromN1 from ‘isvalue’ to ‘ispresent’ 2. Alternative solution implemented: passed this value as a parameter to the existing template. Also implemented in 11.1.7. 3. Accepted. Also implemented in 11.1.7. |

Before Change:

function fl_TC_11_4_12_NR5GC_EUTRA_TestBody() runs on EUTRA_Multi5GS_PTC { var NAS_MSG_Indication_Type v_NasInd; var NasCount_Type v_NasCount; var template UE_AdditionalSecurityCapability v_UEAddSecurityCap; // @sic R5s230881 R5-241528 sic@ var template(present) UE_NetworkCap v_UENWCap := cdr_NR5GC_UENetworkCap('58'O); // @sic R5-243593 sic@ timer t_T1 := f_EUTRA_SetTimerToleranceMin (eutra_Cell1, nasTimer, 60.0); //@siclog "Step 0" siclog@ //@sic R5-223438 sic@ //Set the power levels according to T0 as per Table 11.4.12.3.1-1/2 f_EUTRA38_SetCellPower(eutra_Cell1, -91, -91); //@siclog "Step 1" siclog@ //Make the UE initiate an Emergency call. f_UT_RequestIMSEmergencyCall(UT); //@siclog "Step 2 - 4" siclog@ //Check: Does the UE transmit an RRCConnectionRequest message with 'establishmentCause' set to 'emergency'? //SS transmits an RRCConnectionSetup message. //Check: Does the UE transmit an RRCConnectionSetupComplete message to confirm the successful completion of the connection establishment and a TRACKING AREA UPDATE REQUEST message is sent to update the registration of the actual tracking area. //For some consequences related to the content of the TRACKING AREA UPDATE REQUEST message see the Notes in TS.36.508 [7], Table 4.5A.2.1-1. if (pc_EN_DC == true ) { // @sic R5s230881 R5-241528 sic@ v_UEAddSecurityCap := cr_UE_AddSecurityCap_Any; } else { v_UEAddSecurityCap := omit; } v_UENWCap.cIoT_V2X := (? length (2) & '1'B & ? length (5)) ifpresent; // @sic R5-243593 sic@ v_NasInd := f_EUTRA38_TAUReqFromN1(eutra_Cell1, tsc_RRC_TI_Def, -, MAPPED_CXT, -, emergency, -, v_UEAddSecurityCap,// @sic R5s230881 R5-241528 sic@ valueof(v_UENWCap) // @sic R5-243593 sic@ ); f_EUTRA_PreliminaryPass(__FILE__, __LINE__, "Step 2"); f_EUTRA_PreliminaryPass(__FILE__, __LINE__, "Step 4"); v_NasCount := v_NasInd.SecurityProtection.NasCount; //@siclog "Steps 5-8" siclog@ //Steps 5-8 from the Generic Test Procedure for IMS Emergency call establishment in EUTRA: Normal Service as specified in TS 36.508 [7], Table 4.5A.4.3-1 are performed. f_EUTRA38_MultiPDN_Activate_SRB2_DRB_SendRrcMsg(eutra_Cell1, tsc_RRC_TI_Def, v_NasCount); f_EUTRA_Activate_SRB2_DRB_ReceiveRrcAndNasMsg(eutra_Cell1, tsc_RRC_TI_Def); //@siclog "Step 9" siclog@ //SS responds with TRACKING AREA UPDATE ACCEPT message. //@siclog "Step 10" siclog@ //The UE transmits a TRACKING AREA UPDATE COMPLETE message. f_EUTRA38_TrackingAreaUpdateFromN1_WithoutReq(eutra_Cell1, v_NasInd, MAPPED_CXT, noRrcConnectionRelease, -, true, -, true); //@siclog "Steps 11-17" siclog@ //Steps 9-15 from the Generic Test Procedure for IMS Emergency call establishment in EUTRA: Normal Service as specified in TS 36.508 [7], Table 4.5A.4.3-1 are performed. f_EUTRA38_MultiPDN_EstablishAdditionalPDNs(eutra_Cell1, 1, -, -, -, '100'B, omit, true); // @sic R5s211357 R5-231416 sic@ f_EUTRA38_MultiPDN_ActivateDedicatedEpsBearer_Speech(eutra_Cell1, Emergency_PDN); // @siclog "Step 18" siclog@ // The SS waits 1 second. f_Delay(1.0); // @siclog "Step 18A" siclog@ // Set the power levels according to "T1" as per Table 11.4.12.3.1-1/2. f_EUTRA_NR_SendCoOrd (NR, cms_EUTRA_NR_Trigger("Change Power Level")); // @sic R5-241692 sic@ // @siclog "Step 19" siclog@ // Release IMS Call as specified in the generic procedure in TS 34.229-1 [35] subclause C.32. // Tell IMS PTC to release the call f_IMS_IPCAN_SendCoOrdMsg(IMS[tsc_Index_IMS2]); // @sic R5s221058 sic@ // Emergency call release in IMS f_IMS_IPCAN_WaitForTrigger(IMS[tsc_Index_IMS2], "Call Released"); // The SS deactivates the EPS bearer. [This is the rest of C.32] f_EUTRA_DeactivateEPS_BearerContext (eutra_Cell1, int2hex(valueof(f_Get_EPSBearerIdForAPNType(Emergency_PDN)), 1)); // @sic R5s211357 R5s221058 sic@ // @siclog "Step 20" siclog@ // The SS releases the RRC connection. f_EUTRA_RRC_ConnectionRelease(eutra_Cell1);// @sic R5-241692 sic@ // @siclog "Step 21" siclog@ t_T1.start; // @sic R5-241692 sic@ alt {// @sic R5-241692 sic@ // @siclog "Step 22a1" siclog@ // The UE transmits an RRCConnectionRequest. [] SRB.check(receive(car_SRB0_RrcPdu_IND(eutra_Cell1, cr_RRCConnectionRequest(cr_EstablishmentCause_Any)))) { // @siclog "Step 22a2" siclog@ // Stop Timer T1 t_T1.stop; // @siclog "Steps 22a3-22a7" siclog@ // Steps 2-6 of 36.508 Table 6.4.2.7-1 are executed.// @sic R5-243593 sic@ v_NasInd := f_EUTRA_RRC_ConnEst_DefWithNas(eutra_Cell1, tsc_RRC_TI_Def, ?, cr_NAS_Indication(tsc_SHT_IntegrityProtected, cr_TAU_Request_N1ToS1(('000'B, '001'B, '010'B), ?, cr_DRXparameter_Any ('5C'O) ifpresent, f_GetAdditionalUpdateType(NORMAL), -, -, -, ?, -, -, cr_UeRadioCap ifpresent, -, -, cr_UE_AddSecurityCap_Any ifpresent))); f_EUTRA_TrackingAreaUpdate_Accept_Complete(eutra_Cell1, f_EUTRA_CellInfo_GetGuti(eutra_Cell1), f_GetEPSTAUType(NORMAL), v_NasInd.Pdu.Msg.tRACKING_AREA_UPDATE_REQUEST.epsBearerContextStatus, v_NasInd.Pdu.Msg.tRACKING_AREA_UPDATE_REQUEST.addUpdateType, NORMAL); } // @siclog "Step 22b1" siclog@ // Timer T1 expires. [] t_T1.timeout {} } }

After Change:

function fl_TC_11_4_12_NR5GC_EUTRA_TestBody() runs on EUTRA_Multi5GS_PTC { var NAS_MSG_Indication_Type v_NasInd; var NasCount_Type v_NasCount; var template UE_AdditionalSecurityCapability v_UEAddSecurityCap; // @sic R5s230881 R5-241528 sic@ var template(present) UE_NetworkCap v_UENWCap := cdr_NR5GC_UENetworkCap('58'O); // @sic R5-243593 sic@ timer t_T1 := f_EUTRA_SetTimerToleranceMin (eutra_Cell1, nasTimer, 60.0); //@siclog "Step 0" siclog@ //@sic R5-223438 sic@ //Set the power levels according to T0 as per Table 11.4.12.3.1-1/2 f_EUTRA38_SetCellPower(eutra_Cell1, -91, -91); //@siclog "Step 1" siclog@ //Make the UE initiate an Emergency call. f_UT_RequestIMSEmergencyCall(UT); //@siclog "Step 2 - 4" siclog@ //Check: Does the UE transmit an RRCConnectionRequest message with 'establishmentCause' set to 'emergency'? //SS transmits an RRCConnectionSetup message. //Check: Does the UE transmit an RRCConnectionSetupComplete message to confirm the successful completion of the connection establishment and a TRACKING AREA UPDATE REQUEST message is sent to update the registration of the actual tracking area. //For some consequences related to the content of the TRACKING AREA UPDATE REQUEST message see the Notes in TS.36.508 [7], Table 4.5A.2.1-1. if (pc_EN_DC == true ) { // @sic R5s230881 R5-241528 sic@ v_UEAddSecurityCap := cr_UE_AddSecurityCap_Any; } else { v_UEAddSecurityCap := omit; } v_UENWCap.cIoT_V2X := (? length (2) & '1'B & ? length (5)) ifpresent; // @sic R5-243593 sic@ v_NasInd := f_EUTRA38_TAUReqFromN1(eutra_Cell1, tsc_RRC_TI_Def, -, MAPPED_CXT, -, emergency, -, v_UEAddSecurityCap,// @sic R5s230881 R5-241528 sic@ v_UENWCap //WA#11_4_12 valueof(v_UENWCap) // @sic R5-243593 sic@ ); f_EUTRA_PreliminaryPass(__FILE__, __LINE__, "Step 2"); f_EUTRA_PreliminaryPass(__FILE__, __LINE__, "Step 4"); v_NasCount := v_NasInd.SecurityProtection.NasCount; //@siclog "Steps 5-8" siclog@ //Steps 5-8 from the Generic Test Procedure for IMS Emergency call establishment in EUTRA: Normal Service as specified in TS 36.508 [7], Table 4.5A.4.3-1 are performed. f_EUTRA38_MultiPDN_Activate_SRB2_DRB_SendRrcMsg(eutra_Cell1, tsc_RRC_TI_Def, v_NasCount); f_EUTRA_Activate_SRB2_DRB_ReceiveRrcAndNasMsg(eutra_Cell1, tsc_RRC_TI_Def); //@siclog "Step 9" siclog@ //SS responds with TRACKING AREA UPDATE ACCEPT message. //@siclog "Step 10" siclog@ //The UE transmits a TRACKING AREA UPDATE COMPLETE message. f_EUTRA38_TrackingAreaUpdateFromN1_WithoutReq(eutra_Cell1, v_NasInd, MAPPED_CXT, noRrcConnectionRelease, -, true, -, true); //@siclog "Steps 11-17" siclog@ //Steps 9-15 from the Generic Test Procedure for IMS Emergency call establishment in EUTRA: Normal Service as specified in TS 36.508 [7], Table 4.5A.4.3-1 are performed. f_EUTRA38_MultiPDN_EstablishAdditionalPDNs(eutra_Cell1, 1, -, -, -, '100'B, omit, true); // @sic R5s211357 R5-231416 sic@ f_EUTRA38_MultiPDN_ActivateDedicatedEpsBearer_Speech(eutra_Cell1, Emergency_PDN); // @siclog "Step 18" siclog@ // The SS waits 1 second. f_Delay(1.0); // @siclog "Step 18A" siclog@ // Set the power levels according to "T1" as per Table 11.4.12.3.1-1/2. f_EUTRA_NR_SendCoOrd (NR, cms_EUTRA_NR_Trigger("Change Power Level")); // @sic R5-241692 sic@ // @siclog "Step 19" siclog@ // Release IMS Call as specified in the generic procedure in TS 34.229-1 [35] subclause C.32. // Tell IMS PTC to release the call f_IMS_IPCAN_SendCoOrdMsg(IMS[tsc_Index_IMS2]); // @sic R5s221058 sic@ // Emergency call release in IMS f_IMS_IPCAN_WaitForTrigger(IMS[tsc_Index_IMS2], "Call Released"); // The SS deactivates the EPS bearer. [This is the rest of C.32] f_EUTRA_DeactivateEPS_BearerContext (eutra_Cell1, int2hex(valueof(f_Get_EPSBearerIdForAPNType(Emergency_PDN)), 1)); // @sic R5s211357 R5s221058 sic@ // @siclog "Step 20" siclog@ // The SS releases the RRC connection. f_EUTRA_RRC_ConnectionRelease(eutra_Cell1);// @sic R5-241692 sic@ // @siclog "Step 21" siclog@ t_T1.start; // @sic R5-241692 sic@ alt {// @sic R5-241692 sic@ // @siclog "Step 22a1" siclog@ // The UE transmits an RRCConnectionRequest. [] SRB.check(receive(car_SRB0_RrcPdu_IND(eutra_Cell1, cr_RRCConnectionRequest(cr_EstablishmentCause_Any)))) { // @siclog "Step 22a2" siclog@ // Stop Timer T1 t_T1.stop; // @siclog "Steps 22a3-22a7" siclog@ // Steps 2-6 of 36.508 Table 6.4.2.7-1 are executed.// @sic R5-243593 sic@ v_NasInd := f_EUTRA_RRC_ConnEst_DefWithNas(eutra_Cell1, tsc_RRC_TI_Def, ?, cr_NAS_Indication(tsc_SHT_IntegrityProtected, cr_TAU_Request_11_4_12(('000'B, '001'B, '010'B), //WA#11_4_12 ?, cr_DRXparameter_Any ('5C'O) ifpresent, f_GetAdditionalUpdateType(NORMAL), -, -, -, ?, -, -, cr_UeRadioCap ifpresent, -, -, cr_UE_AddSecurityCap_Any ifpresent))); f_EUTRA_TrackingAreaUpdate_Accept_Complete(eutra_Cell1, f_EUTRA_CellInfo_GetGuti(eutra_Cell1), f_GetEPSTAUType(NORMAL), v_NasInd.Pdu.Msg.tRACKING_AREA_UPDATE_REQUEST.epsBearerContextStatus, v_NasInd.Pdu.Msg.tRACKING_AREA_UPDATE_REQUEST.addUpdateType, NORMAL); f_EUTRA_RRC_ConnectionRelease(eutra_Cell1); //WA#11_4_12 } // @siclog "Step 22b1" siclog@ // Timer T1 expires. [] t_T1.timeout {} } }

## Change 2

| Function name | cr_TAU_Request_11_4_12 |
| --- | --- |
| Reason for change | In the current TTCN implementation the template used for capturing the TAU REQ at Step 22a4 expects the IE ueStatus to be present. This is fine for a case when the UE is transitioning from N1S1. However, in the current case the UE has already been on Eutra cell for the emergency call, and after releasing the call the UE performs a TAU to gain full services on the Eutra cell. Thus, this is not a N1S1 transition, and hence the IE ueStatus may be omitted by a conformant UE. This needs to be accommodated. |
| Summary of change | New template |
| TTCN module | Eutra38_NR5GCProcedures.ttcn |
| MCC160 Comments | Alternative solution implemented: added this as a parameter to the existing template |

template (present) NAS_UL_Message_Type cr_TAU_Request_11_4_12 (template (present) EPS_UpdateTypeValue p_EpsUpdate_TypeValue, template (present) NAS_KsiValue p_KsiValue, template DRXparameter p_DRXparameter, template AdditionalUpdateType p_AdditionalUpdateType, template (present) MobileIdentity p_MobileId := cr_MobileIdAny, template TrackingAreaId p_TAI := cr_TAI_Any ifpresent, template (present) B1_Type p_ActiveFlag := ?, // @sic R5-202667, R5-211371 sic@ template (present) B1_Type p_TSC := tsc_NasKsi_NativeSecurityContext, template NAS_KeySetIdentifier p_NonCurrentKSI := omit, template MobileIdentity p_AdditionalGUTI := cr_MobileIdAny ('50'O) ifpresent, // @sic R5-202563 sic@ template UERadioCapInfoUpdateNeeded p_UeRadioCap := cr_UeRadioCap, template EPS_BearerContextStatus p_EPS_BearerContextStatus := cr_EPS_BearerContextStatus_Any, template (present) UE_NetworkCap p_UENWCap := cdr_NR5GC_UENetworkCap('58'O), template UE_AdditionalSecurityCapability p_AddSecurityCap := cr_UE_AddSecurityCap_Any) := { /* @status APPROVED (NR5GC_IRAT) */ tRACKING_AREA_UPDATE_REQUEST := { securityHeaderType := tsc_SHT_NoSecurityProtection, protocolDiscriminator := tsc_PD_EMM, messageType := tsc_MT_TauRequest, asmeNasKeySetId := cr_NAS_KeySetIdentifier(p_KsiValue, p_TSC), // @sic R5s200509 sic@ epsUpdateType := cr_EPS_UpdateType(p_ActiveFlag, p_EpsUpdate_TypeValue), // @sic R5-202667 sic@ oldGuti := p_MobileId, nonCurrentNativeNasKeySetId := p_NonCurrentKSI, /* @sic R5s100135, R5s200509 sic@ */ gprsCipheringKeySeqNum := cr_CiphKeySeqNum('8'H, ?) ifpresent, /* @sic R5s100135, R5-135073 sic@ */ oldPtmsiSignature := omit, additionalGuti := p_AdditionalGUTI, // @sic R5-201215 sic@ nonce := omit, ueNetworkCapability := p_UENWCap, // @sic R5s211353 sic@ lastVisitedRegisteredTai := p_TAI, drxParameter := p_DRXparameter, // @sic R5s100660, R5s120210 sic@ ueRadioCapNeeded := p_UeRadioCap, // @sic R5s200583 sic@ epsBearerContextStatus := p_EPS_BearerContextStatus, // @sic R5s200760 sic@ msNetworkCapability := cr_MS_NetworkCap_Any ifpresent, oldLai := cr_LAI_Any ifpresent, tmsiStatus := cr_TMSI_Status ifpresent, msClassmark2 := cr_MS_Clsmk2_Any_tlv ifpresent, // @sic R5s100662 sic@ msClassmark3 := cr_MSCLSMK3_Any ifpresent, supportedCodecList := cr_CodecListAny ifpresent, addUpdateType := p_AdditionalUpdateType, /* @sic R5-103681 sic@ */ voiceDomainPref := cr_VoiceDomainPref_Any ifpresent, /* @sic R5s110176 Baseline Moving sic@ */ oldGUTI_Type := cr_GUTI_Type_Native, deviceProperties := cr_DeviceProperties('D'H) ifpresent, /* cl. 9.9.2.0A O TV 1 IEI=0xD- @sic R5s120178 Baseline Moving sic@*/ msNetworkFeatureSupport := cr_MS_NetworkFeatureSupport('C'H) ifpresent, /* cl. 9.9.2.0A O TV 1 IEI=0xC- @sic R5s120178 Baseline Moving sic@*/ tmsiBasedNRIContainer := cr_NwkResourceIdAny ifpresent, // @sic R5s130195 Baseline Moving sic@ t3324Value := cr_GPRS_Timer2_3('6A'O) ifpresent, // @sic R5s150329 Baseline Moving sic@ t3412ExtdValue := cr_GPRS_Timer2_3 ('5E'O) ifpresent, // @sic R5s150329 Baseline Moving sic@ extendedDRXParams := cr_ExtdDRXParamsAny ifpresent, // @sic R5s160711 Baseline Moving sic@ ueAddSecurityCap := p_AddSecurityCap, // @sic R5-241528 sic@ ueStatus := cr_UEStatus_N1 ifpresent, //WA#11_4_12 addInfoReq := cr_AdditionalInfoReqAny ifpresent, /* cl. 9.9.3.55 O TLV 2 IEI=0x17 @sic R5s180552 Baseline Moving 2018 Phase 2 sic@ */ n1UENetworkCap := cr_N1_UENetworkCapAny ifpresent, /* cl. 9.9.3.57 O TLV 3-15 IEI=0x32 @sic R5s201386 Baseline Moving sic@ */ ueRadioCapIdAvailability := cr_UERadioCapIdAvailabilityAny ifpresent, /* cl. 9.9.3.58 O TV 3 IEI=0x34 @sic R5s201386 Baseline Moving sic@ */ requestedWUSAssistInfo := cr_WUSAssistInfoAny ifpresent, /* cl. 9.9.3.62 O TLV 3-n IEI=0x35 @sic R5s201386 Baseline Moving sic@ */ drxInNB_S1 := cr_DRXParamCommonAny ('36'O) ifpresent, /* cl. 9.9.3.63 O TLV 3 IEI=0x36 @sic R5s201386 Baseline Moving sic@ */ requestedIMSI_Offset := cr_IMSI_Offset ifpresent, /* cl. 9.9.3.64 O TLV 4 IEI=0x38 Sep22 @sic R5s221178 Baseline Moving sic@ */ ueRequestType := cr_UE_RequestType ifpresent, /* cl. 9.9.3.65 O TLV 3 IEI=0x29 Sep22 @sic R5s221178 Baseline Moving sic@ */ pagingRestriction := cr_PagingRestriction ifpresent /* cl. 9.9.3.66 O TLV 3-5 IEI=0x28 Sep22 @sic R5s221178 Baseline Moving sic@ */ } };

#### MCC160 Implementation:

template (present) NAS_UL_Message_Type cr_TAU_Request_N1ToS1(template (present) EPS_UpdateTypeValue p_EpsUpdate_TypeValue,

template (present) NAS_KsiValue p_KsiValue,

template DRXparameter p_DRXparameter,

template AdditionalUpdateType p_AdditionalUpdateType,

template (present) MobileIdentity p_MobileId := cr_MobileIdAny,

template TrackingAreaId p_TAI := cr_TAI_Any ifpresent,

template (present) B1_Type p_ActiveFlag := ?,            // @sic R5-202667, R5-211371 sic@

template (present) B1_Type  p_TSC := tsc_NasKsi_NativeSecurityContext,

template NAS_KeySetIdentifier p_NonCurrentKSI := omit,

template MobileIdentity p_AdditionalGUTI := cr_MobileIdAny ('50'O) ifpresent, // @sic R5-202563 sic@

template UERadioCapInfoUpdateNeeded p_UeRadioCap := cr_UeRadioCap,

template EPS_BearerContextStatus p_EPS_BearerContextStatus := cr_EPS_BearerContextStatus_Any,

template (present) UE_NetworkCap p_UENWCap := cdr_NR5GC_UENetworkCap('58'O),

template UE_AdditionalSecurityCapability p_AddSecurityCap := cr_UE_AddSecurityCap_Any,

template UE_Status p_UE_Status := cr_UEStatus_N1) :=

{ /* @status    APPROVED (NR5GC_IRAT) */

tRACKING_AREA_UPDATE_REQUEST := {

securityHeaderType         := tsc_SHT_NoSecurityProtection,

protocolDiscriminator      := tsc_PD_EMM,

messageType                := tsc_MT_TauRequest,

asmeNasKeySetId            := cr_NAS_KeySetIdentifier(p_KsiValue, p_TSC),  // @sic R5s200509 sic@

epsUpdateType              := cr_EPS_UpdateType(p_ActiveFlag, p_EpsUpdate_TypeValue), // @sic R5-202667 sic@

oldGuti                    := p_MobileId,

nonCurrentNativeNasKeySetId  := p_NonCurrentKSI,   /* @sic R5s100135, R5s200509 sic@ */

gprsCipheringKeySeqNum     := cr_CiphKeySeqNum('8'H, ?) ifpresent,        /* @sic R5s100135, R5-135073 sic@ */

oldPtmsiSignature          := omit,

additionalGuti             := p_AdditionalGUTI,  // @sic R5-201215 sic@

nonce                      := omit,

ueNetworkCapability        := p_UENWCap,  // @sic R5s211353 sic@

lastVisitedRegisteredTai   := p_TAI,

drxParameter               := p_DRXparameter, // @sic R5s100660, R5s120210 sic@

ueRadioCapNeeded           := p_UeRadioCap, // @sic R5s200583 sic@

epsBearerContextStatus     := p_EPS_BearerContextStatus,  // @sic R5s200760 sic@

msNetworkCapability        := cr_MS_NetworkCap_Any ifpresent,

oldLai                     := cr_LAI_Any ifpresent,

tmsiStatus                 := cr_TMSI_Status ifpresent,

msClassmark2               := cr_MS_Clsmk2_Any_tlv ifpresent,  // @sic R5s100662 sic@

msClassmark3               := cr_MSCLSMK3_Any ifpresent,

supportedCodecList         := cr_CodecListAny ifpresent,

addUpdateType              := p_AdditionalUpdateType,         /* @sic R5-103681 sic@ */

voiceDomainPref            := cr_VoiceDomainPref_Any ifpresent, /* @sic R5s110176 Baseline Moving sic@ */

oldGUTI_Type               := cr_GUTI_Type_Native,

deviceProperties           := cr_DeviceProperties('D'H) ifpresent,  /* cl. 9.9.2.0A O TV 1 IEI=0xD- @sic R5s120178 Baseline Moving sic@*/

msNetworkFeatureSupport    := cr_MS_NetworkFeatureSupport('C'H) ifpresent,   /* cl. 9.9.2.0A O TV 1 IEI=0xC- @sic R5s120178 Baseline Moving sic@*/

tmsiBasedNRIContainer      := cr_NwkResourceIdAny ifpresent, // @sic R5s130195 Baseline Moving sic@

t3324Value                 := cr_GPRS_Timer2_3('6A'O) ifpresent, // @sic R5s150329 Baseline Moving sic@

t3412ExtdValue             := cr_GPRS_Timer2_3 ('5E'O) ifpresent, // @sic R5s150329 Baseline Moving sic@

extendedDRXParams          := cr_ExtdDRXParamsAny ifpresent,  // @sic R5s160711 Baseline Moving sic@

ueAddSecurityCap           := p_AddSecurityCap,             // @sic R5-241528 sic@

ueStatus                   := p_UE_Status,

addInfoReq                 := cr_AdditionalInfoReqAny ifpresent,     /* cl. 9.9.3.55 O TLV 2    IEI=0x17 @sic R5s180552 Baseline Moving 2018 Phase 2 sic@ */

n1UENetworkCap             := cr_N1_UENetworkCapAny ifpresent,  /* cl. 9.9.3.57 O TLV 3-15 IEI=0x32 @sic R5s201386 Baseline Moving sic@ */

ueRadioCapIdAvailability   := cr_UERadioCapIdAvailabilityAny ifpresent,  /* cl. 9.9.3.58 O TV 3 IEI=0x34 @sic R5s201386 Baseline Moving sic@ */

requestedWUSAssistInfo     := cr_WUSAssistInfoAny ifpresent,  /* cl. 9.9.3.62 O TLV 3-n IEI=0x35 @sic R5s201386 Baseline Moving sic@ */

drxInNB_S1                 := cr_DRXParamCommonAny ('36'O) ifpresent,   /* cl. 9.9.3.63 O TLV 3 IEI=0x36 @sic R5s201386 Baseline Moving sic@ */

requestedIMSI_Offset       := cr_IMSI_Offset ifpresent,  /* cl. 9.9.3.64 O TLV 4 IEI=0x38 Sep22 @sic R5s221178 Baseline Moving sic@ */

ueRequestType              := cr_UE_RequestType ifpresent,     /* cl. 9.9.3.65 O TLV 3 IEI=0x29 Sep22 @sic R5s221178 Baseline Moving sic@ */

pagingRestriction          := cr_PagingRestriction ifpresent   /* cl. 9.9.3.66 O TLV 3-5 IEI=0x28 Sep22 @sic R5s221178 Baseline Moving sic@ */

}

};

function fl_TC_11_4_12_NR5GC_EUTRA_TestBody() runs on EUTRA_Multi5GS_PTC

{

...

// @siclog "Step 21" siclog@

t_T1.start; // @sic R5-241692 sic@

alt {// @sic R5-241692 sic@

// @siclog "Step 22a1" siclog@

// The UE transmits an RRCConnectionRequest.

[] SRB.check(receive(car_SRB0_RrcPdu_IND(eutra_Cell1, cr_RRCConnectionRequest(cr_EstablishmentCause_Any))))

{

// @siclog "Step 22a2" siclog@

// Stop Timer T1

t_T1.stop;

// @siclog "Steps 22a3-22a7" siclog@

// Steps 2-6 of 36.508 Table 6.4.2.7-1 are executed.// @sic R5-243593 sic@

v_NasInd := f_EUTRA_RRC_ConnEst_DefWithNas(eutra_Cell1,

tsc_RRC_TI_Def,

?,

cr_NAS_Indication(tsc_SHT_IntegrityProtected,

cr_TAU_Request_N1ToS1(('000'B, '001'B, '010'B),

?,

cr_DRXparameter_Any ('5C'O) ifpresent,

f_GetAdditionalUpdateType(NORMAL),

-, -, -,

?,

-, -,

cr_UeRadioCap ifpresent,

-, -,

cr_UE_AddSecurityCap_Any ifpresent,

cr_UEStatus_Any ifpresent)));

f_EUTRA_TrackingAreaUpdate_Accept_Complete(eutra_Cell1,

f_EUTRA_CellInfo_GetGuti(eutra_Cell1),

f_GetEPSTAUType(NORMAL),

v_NasInd.Pdu.Msg.tRACKING_AREA_UPDATE_REQUEST.epsBearerContextStatus,

v_NasInd.Pdu.Msg.tRACKING_AREA_UPDATE_REQUEST.addUpdateType,

NORMAL);

}

// @siclog "Step 22b1" siclog@

// Timer T1 expires.

[] t_T1.timeout {}

}

}

function f_EUTRA38_TAUReqFromN1(EUTRA_CellId_Type p_CellId,

RRC_TransactionIdentifier p_RRC_TI,

template MobileIdentity p_MobileId := omit,

N1ToS1_SecurityContext_Type p_CxtType := MAPPED_CXT,

template EUTRA_CellId_Type p_CellIdPreviouslyCampedOn := eutra_Cell_NonSpecific,

template (present) EstablishmentCause p_EstablishmentCause := mo_Signalling,

boolean p_FirstN1_S1Transition := true,

template UE_AdditionalSecurityCapability p_AddSecurityCap := cr_UE_AddSecurityCap_Any,

template (omit) UE_NetworkCap p_UE_NetworkCap := omit) runs on EUTRA_Multi5GS_PTC return NAS_MSG_Indication_Type

{

var template AdditionalUpdateType v_AdditionalUpdateType := f_GetAdditionalUpdateType (NORMAL);

var NAS_MSG_Indication_Type v_NasInd;

var EUTRA_SecurityParams_Type v_SecurityParams;

var EUTRA_NR_CoOrd_Security_Type v_SecurityRx;

var template TrackingAreaId v_TAI:= *; // @sic R5s220225 sic@

var EUTRA_CellId_Type v_OldCellId;

var GutiParameters_Type v_GutiParams;

var NAS_PlmnId v_PLMN;

var TrackingAreaCode v_Tac;

var template UERadioCapInfoUpdateNeeded v_UeRadioCap:= cr_UeRadioCap;

var EUTRA_NR_PduSessionInfoList_Type v_PduSessionInfoList := f_EUTRA38_WaitForPDUSessionInfoList(p_CellId); // @sic R5s210149 sic@

var integer v_NoOfDRBs := f_EUTRA38_MobileInfo_GetNoOfDRBs();

var EUTRA38_DRBInfoList_Type v_ExistingDRBs := f_EUTRA38_MobileInfo_GetDRBInfoList(); // @sic R5s201363 sic@

var template (value) EUTRA38_DRBInfoList_Type v_NewDRBInfoList := {};

var integer v_NewDRBId;

var integer i;

var integer j := 0;

var template (present) MobileIdentity v_MobileId := cr_MobileIdAny;

var EUTRA_NR_CoOrd_GUTI_Type v_CoOrd_Guti := f_EUTRA_NR_WaitForCoOrd_GutiParams(NR);

var template (value) MobileIdentity v_MappedGUTI := f_EUTRA38_NR_Convert5G_CoOrdToEPS_GUTI (v_CoOrd_Guti, omit);

var EPS_BearerContextStatus v_EPS_BearerContextStatus;

var IntegerList_Type v_TransferredBearers;

var template (present) UE_NetworkCap v_UENWCap;

if (isvalue(p_MobileId)) { // @sic R5s200564 sic@

v_MobileId := p_MobileId; // compiler warning can be ignored due to previous check

}

if (p_CxtType == MAPPED_CXT) { // @sic R5s200564 sic@

v_MobileId := v_MappedGUTI;

}

if (isvalue(p_CellIdPreviouslyCampedOn)) {

v_OldCellId := valueof(p_CellIdPreviouslyCampedOn);

if (v_OldCellId == eutra_Cell_NonSpecific) {

v_OldCellId := p_CellId;

}

v_GutiParams:= f_EUTRA_CellInfo_GetGuti(v_OldCellId);

v_PLMN := f_Asn2Nas_PlmnId(v_GutiParams.PLMN_Identity);

v_Tac := f_EUTRA_CellInfo_GetTAC(v_OldCellId);

v_TAI := cr_TAI(v_PLMN, bit2oct(v_Tac));

}

// If we need a new algorithm, have to store the new values before calling this

// tell NAS EMU to check next message with 5G integrity

f_EUTRA_NB_SS_NAS_SecurityConfig(NASCTRL, cas_NasSecurity_5GIntegrity_REQ(true));

if (not p_FirstN1_S1Transition) { // @sic R5s200583 sic@

v_UeRadioCap := cr_UeRadioCap ifpresent;

}

if (ispresent(p_UE_NetworkCap)) { // @sic R5-243593 sic@

v_UENWCap := p_UE_NetworkCap;

} else {

v_UENWCap := cdr_NR5GC_UENetworkCap('58'O);

if (valueof(p_EstablishmentCause) == emergency) { // @sic R5-217714 sic@

v_UENWCap.cIoT_proSeCap := ?; // @sic R5-220644 sic@

v_UENWCap.cIoT_V2X := ?

}

}

/* Steps 1-3 */

//Receive RRC Connection Setup Complete with piggy-backed NAS message on SRB1 (DCCH):

v_NasInd := f_EUTRA_RRC_ConnEst_DefWithNas(p_CellId,

p_RRC_TI,

p_EstablishmentCause,

cr_NAS_Indication(tsc_SHT_IntegrityProtected,

cr_TAU_Request_N1ToS1(('000'B, '001'B, '010'B), // @sic R5-201171, R5-202563, R5-211371 sic@

?,

cr_DRXparameter_Any ('5C'O) ifpresent,

v_AdditionalUpdateType,

v_MobileId,  // @sic R5-204399 sic@

v_TAI,

-, -, -,  // @sic R5s200509 sic@

cr_MobileIdAny ('50'O) ifpresent, // @sic R5202563 sic@

v_UeRadioCap,

-,

v_UENWCap,

p_AddSecurityCap))); // @sic R5-241528 sic@

...