3GPP TSG-RAN5 Meeting #2024-TTCN email	R5s240440

# Online, , 8th Dec 2023 - 31st Dec 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 3473 | rev | - | Current version: | 17.10.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to NR5GC testcase 6.4.1.1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2024-07-01 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | - After receiving the RRCResume message, there may be time difference between UE receiving RRCResume and UE applying the new RRC config. carried in RRCResume. Some UE implementations transmit the RLC Ack (for the rrcResume) using RACH as soon as the rrcResume is received without applying the configuration received within rrcResume and before transmitting rrcResumeComplete. Proposal is to make the rrcResume cases flexible to receive the resume complete either using UL DCI Grant or RAR grant. So, SS is required to configure a short UL periodic to consider this Random Access procedure successfully completed by UE if it gets into below clause of TS 38.321, section 5.1.5 Contention Resolution. i.e., 1> if notification of a reception of a PDCCH transmission of the SpCell is received from lower layers: 2> if the C-RNTI MAC CE was included in Msg3: 3> if the Random Access procedure was initiated for SpCell beam failure recovery or for beam failure recovery of both BFD-RS sets of SpCell (as specified in clause 5.17) and the PDCCH transmission is addressed to the C-RNTI; or 3> if the Random Access procedure was initiated by a PDCCH order and the PDCCH transmission is addressed to the C-RNTI; or 3> if the Random Access procedure was initiated by the MAC sublayer itself or by the RRC sublayer and the PDCCH transmission is addressed to the C-RNTI and contains a UL grant for a new transmission: 4> consider this Contention Resolution successful; 4> stop ra-ContentionResolutionTimer; 4> discard the TEMPORARY_C-RNTI; 4> consider this Random Access procedure successfully completed. 2> else if the CCCH SDU was included in Msg3 and the PDCCH transmission is addressed to its TEMPORARY_C-RNTI: 2. RRCResumeComplete has been received so SS can switch back to SR based UL Grant configurations now. |  |  |  |  |  |  |  |  |
| Summary of change: |  | 1. Added a flag p_StartGrantPeriodic to enable Periodic grant configuration before sending rrcResume in function f_NR_SendRRCResume_Def. 2. Added a flag p_SRGrantEnable to control the UL Grant configuration as SR based in function f_NR_RRCResumeComplete_Def. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may fail the testcase |  |  |  |  |  |  |  |  |
| Clauses affected: |  | .6.4.1.1.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  | Other NR5GC rrcResume test cases like 8.1.1.4.1, 8.1.1.4.2, 8.1.5.9.3, 8.1.6.1.4.2, 11.3.3 etc are impacted as well. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	3

1	Overview	4

2	Corrections required	4

2.1	Change 1	4

2.2	Change 2	6

# Overview

This document lists all the essential changes needed to correct problems in the iwd-TTCN3-B2022-09_D24wk24 TTCN-3 related to the title of this CR.

Contact: 	Parikshit Bhise

Parikshit.bhise@rohde-schwarz.com

# Corrections required

## Change 1

| Template name | f_NR_SendRRCResume_Def |
| --- | --- |
| Reason for change | After receiving the RRCResume message, there may be time difference between UE receiving RRCResume and UE applying the new RRC configuraition received in in RRCResume. Some UE implementations transmit the RLC Ack (for the rrcResume) using RACH as soon as the rrcResume is received without applying the configuration received within rrcResume and before transmitting rrcResumeComplete. Proposal is to make the rrcResume cases flexible to receive the resume complete either using UL DCI Grant or RAR grant. So, SS is required to configure a short UL periodic grant to consider this Random Access procedure successfully completed by UE if it gets into below clause of TS 38.321, section 5.1.5 Contention Resolution. i.e., 1> if notification of a reception of a PDCCH transmission of the SpCell is received from lower layers: 2> if the C-RNTI MAC CE was included in Msg3: 3> if the Random Access procedure was initiated for SpCell beam failure recovery or for beam failure recovery of both BFD-RS sets of SpCell (as specified in clause 5.17) and the PDCCH transmission is addressed to the C-RNTI; or 3> if the Random Access procedure was initiated by a PDCCH order and the PDCCH transmission is addressed to the C-RNTI; or 3> if the Random Access procedure was initiated by the MAC sublayer itself or by the RRC sublayer and the PDCCH transmission is addressed to the C-RNTI and contains a UL grant for a new transmission: 4> consider this Contention Resolution successful; 4> stop ra-ContentionResolutionTimer; 4> discard the TEMPORARY_C-RNTI; 4> consider this Random Access procedure successfully completed. 2> else if the CCCH SDU was included in Msg3 and the PDCCH transmission is addressed to its TEMPORARY_C-RNTI: |
| Summary of change | Added a flag p_StartGrantPeriodic to enable Periodic grant configuration before sending rrcResume in function f_NR_SendRRCResume_Def. |
| TTCN module | NR5GC\NR5GC_RRCSteps.ttcn |
| MCC160 | Accepted in principle. As the issue is related to the random access procedure, it is cleaner to modify the random access in the SS: before sending RRCResume, reconfigure RACH to a RACH procedure with contention-based with C-RNTI based contention resolution. Then after RRCResumeComplete, reconfigure RACH procedure as for initial access. See MCC160 proposed implementation below. Same change done in f_TC_8_1_5_8_2_x_NR5GC_TestBody f_TC_8_1_5_8_1_NR5GC_TestBody f_TC_8_2_6_2_4_NR5GC_TestBody fl_TC_8_2_7_2_1_TestBody |

Before Change:

function f_NR_SendRRCResume_Def( NR_CellId_Type p_NR_CellId, template (value) TimingInfo_Type p_TimingInfo := cs_TimingInfo_Now, RRC_TransactionIdentifier p_RRC_TI := tsc_NR_RRC_TI_Def, template (omit) CellGroupConfig p_CellGroupConfig := omit, template (omit) RadioBearerConfig p_RadioBearerConfig := omit, template (omit) MeasConfig p_MeasConfig := omit, template (omit) RRCResume_v1560_IEs p_RRCResume_v1560_IEs := omit ) runs on NR5GC_PTC { //@sic R5-201646: added p_RadioBearerConfig sic@ //@sic R5-204441: added p_MeasConfig sic@ //@sic R5-241544: for SDT added v_RrcResume sic@ var NR_DRBInfoList_Type v_ExistingDRBs := f_NR5GC_MobileInfo_GetDRBInfoList(); var template (value) NR_RadioBearerList_Type v_SS_Config := f_NR5GC_GetRadioBearerList(v_ExistingDRBs); var template (value) DL_DCCH_Message v_RrcResume := f_NR_38508_Get_RRCResume_Def(p_NR_CellId, p_RRC_TI, p_CellGroupConfig, p_RadioBearerConfig, p_MeasConfig, p_RRCResume_v1560_IEs); // Switch on UL grant assignments f_NR_ULGrantConfiguration_Start(p_NR_CellId); // Refresh security keys and activate security => moved to the function bringing UE to Inactive state to give time to SS to configure security parameters //@sic R5s210578 sic@ //Reconfigure DRB in SS - existing DRBs are not reconfigured during release f_NR_SS_CommonRadioBearerConfig(p_NR_CellId, v_SS_Config, p_TimingInfo); //Activate security on newly configured DRB @sic R5s200102 sic@ f_NR_SS_AS_ActivateSecurity_NoRefresh(p_NR_CellId, p_TimingInfo); //@sic R5s210578 sic@ SRB.send(cas_NR_SRB1_RrcPdu_REQ(p_NR_CellId, p_TimingInfo, v_RrcResume)); //@sic R5-204441, R5s191051: added p_CellGroupConfig as a temporary solution sic@ }

After Change:

function f_NR_SendRRCResume_Def( NR_CellId_Type p_NR_CellId, template (value) TimingInfo_Type p_TimingInfo := cs_TimingInfo_Now, RRC_TransactionIdentifier p_RRC_TI := tsc_NR_RRC_TI_Def, template (omit) CellGroupConfig p_CellGroupConfig := omit, template (omit) RadioBearerConfig p_RadioBearerConfig := omit, template (omit) MeasConfig p_MeasConfig := omit, template (omit) RRCResume_v1560_IEs p_RRCResume_v1560_IEs := omit, boolean p_StartGrantPeriodic := true //WA#WI=1271929 ) runs on NR5GC_PTC { //@sic R5-201646: added p_RadioBearerConfig sic@ //@sic R5-204441: added p_MeasConfig sic@ //@sic R5-241544: for SDT added v_RrcResume sic@ var NR_DRBInfoList_Type v_ExistingDRBs := f_NR5GC_MobileInfo_GetDRBInfoList(); var template (value) NR_RadioBearerList_Type v_SS_Config := f_NR5GC_GetRadioBearerList(v_ExistingDRBs); var template (value) DL_DCCH_Message v_RrcResume := f_NR_38508_Get_RRCResume_Def(p_NR_CellId, p_RRC_TI, p_CellGroupConfig, p_RadioBearerConfig, p_MeasConfig, p_RRCResume_v1560_IEs); //WA#WI=1271929 - Periodic grant var SubcarrierSpacing v_SCS := f_NR_CellInfo_GetSCS(p_NR_CellId); if(p_StartGrantPeriodic) { if (v_SCS == kHz30) { f_NR_ULGrantConfiguration_StartPeriodicGrant(p_NR_CellId, cs_TimingInfo_SubFrame(2), valueof (cs_NR_ResourceAllocationDef), 10); // periodic grant every 5ms (80% of 12ms the Inactivity Timer) } else { f_NR_ULGrantConfiguration_StartPeriodicGrant(p_NR_CellId, cs_TimingInfo_SubFrame(5), valueof (cs_NR_ResourceAllocationDef), 10); // periodic grant every 5ms (80% of 12ms the Inactivity Timer) } }else { // Switch on UL grant assignments f_NR_ULGrantConfiguration_Start(p_NR_CellId); } // Refresh security keys and activate security => moved to the function bringing UE to Inactive state to give time to SS to configure security parameters //@sic R5s210578 sic@ //Reconfigure DRB in SS - existing DRBs are not reconfigured during release f_NR_SS_CommonRadioBearerConfig(p_NR_CellId, v_SS_Config, p_TimingInfo); //Activate security on newly configured DRB @sic R5s200102 sic@ f_NR_SS_AS_ActivateSecurity_NoRefresh(p_NR_CellId, p_TimingInfo); //@sic R5s210578 sic@ SRB.send(cas_NR_SRB1_RrcPdu_REQ(p_NR_CellId, p_TimingInfo, v_RrcResume)); //@sic R5-204441, R5s191051: added p_CellGroupConfig as a temporary solution sic@ }

MCC160 proposed implementation

The same change is done inside:

-

function f_NR_SS_ReconfigureRACH_CBRA_CrntiBased(NR_CellId_Type                       p_NR_CellId

) runs on NR_BASE_PTC

{ //@sic R5s240440 sic@

var template (value) NR_RachProcedureConfig_Type v_RachProcedureConfig;

var NR_PhysicalParameters_Type v_NR_PhysicalParameters := f_NR_CellInfo_GetPhysicalParameters(p_NR_CellId);

f_NR_SS_RachProcedureConfig(nr_Cell1, cs_NR_RachProcedureConfig_CBRA_CrntiBased(v_NR_PhysicalParameters));

}

function f_NR_SendRRCResume_Def( NR_CellId_Type                    p_NR_CellId,

template (value) TimingInfo_Type  p_TimingInfo := cs_TimingInfo_Now,

RRC_TransactionIdentifier         p_RRC_TI := tsc_NR_RRC_TI_Def,

template (omit) CellGroupConfig   p_CellGroupConfig := omit,

template (omit) RadioBearerConfig p_RadioBearerConfig := omit,

template (omit) MeasConfig        p_MeasConfig := omit,

template (omit) RRCResume_v1560_IEs p_RRCResume_v1560_IEs := omit

) runs on NR5GC_PTC

{ //@sic R5-201646: added p_RadioBearerConfig sic@

//@sic R5-204441: added p_MeasConfig sic@

//@sic R5-241544: for SDT added v_RrcResume sic@

var NR_DRBInfoList_Type v_ExistingDRBs := f_NR5GC_MobileInfo_GetDRBInfoList();

var template (value) NR_RadioBearerList_Type v_SS_Config := f_NR5GC_GetRadioBearerList(v_ExistingDRBs);

var template (value) DL_DCCH_Message v_RrcResume := f_NR_38508_Get_RRCResume_Def(p_NR_CellId, p_RRC_TI, p_CellGroupConfig, p_RadioBearerConfig, p_MeasConfig, p_RRCResume_v1560_IEs);

// Reconfigure SS - set DcchDtch on CSS with DCI format 1_0 to send RRCResume

//@sic R5s240461 sic@

f_NR_SS_ReconfigSStoCSS_DCI1_0(p_NR_CellId);

// Reconfigure Random Access procedure to CBRA C-RNTI based in case Ue uses RACH to send RLC ACK

//@sic R5s240440 sic@

f_NR_SS_ReconfigureRACH_CBRA_CrntiBased(p_NR_CellId);

// Switch on UL grant assignments

f_NR_ULGrantConfiguration_Start(p_NR_CellId);

// Refresh security keys and activate security => moved to the function bringing UE to Inactive state to give time to SS to configure security parameters

//@sic R5s210578 sic@

//Reconfigure DRB in SS - existing DRBs are not reconfigured during release

f_NR_SS_CommonRadioBearerConfig(p_NR_CellId, v_SS_Config, p_TimingInfo);

//Activate security on newly configured DRB @sic R5s200102 sic@

f_NR_SS_AS_ActivateSecurity_NoRefresh(p_NR_CellId, p_TimingInfo);

//@sic R5s210578 sic@

SRB.send(cas_NR_SRB1_RrcPdu_REQ(p_NR_CellId, p_TimingInfo, v_RrcResume));  //@sic R5-204441, R5s191051: added p_CellGroupConfig as a temporary solution sic@

}

## Change 2

| Template name | f_NR_RRCResumeComplete_Def |
| --- | --- |
| Reason for change | 1. RRCResumeComplete has been received so SS can switch back to SR based UL Grant configurations now. |
| Summary of change | 1. Added a flag p_SRGrantEnable to control the UL Grant configuration as SR based in function f_NR_RRCResumeComplete_Def. |
| TTCN module | NR5GC\NR5GC_RRCSteps.ttcn |
| MCC160 | See change 1 - reconfigure RACH procedure as for initial access Same change done in fl_TC_8_1_1_1_1_TestBody f_TC_8_1_5_8_2_x_NR5GC_TestBody f_TC_8_1_5_8_1_NR5GC_TestBody f_TC_8_1_5_9_3_NR5GC_TestBody f_TC_8_2_6_2_4_NR5GC_TestBody fl_TC_8_2_7_2_1_TestBody fl_TC_11_3_1a_TestBody fl_TC_11_3_3_TestBody fl_TC_11_3_6and6a_TestBody fl_TC_11_3_11_TestBody |

Before Change:

function f_NR_RRCResumeComplete_Def(NR_CellId_Type p_NR_CellId, template(present) UL_DCCH_Message p_ExpectedRRCResumeComplete := cr_38508_RRCResumeComplete) runs on NR_BASE_PTC return template (omit) NG_NAS_MSG_Indication_Type { var template (omit) NG_NAS_MSG_Indication_Type v_DedicatedNAS_Message := omit; //@sic R5s191051 change 4 sic@ var NR_SRB_COMMON_IND v_ReceivedAsp; //The UE transmits an RRCResumeComplete message. alt { // @sic R5s200403 sic@ [] SRB.receive(car_NR_SRB1_RrcPdu_IND(p_NR_CellId, p_ExpectedRRCResumeComplete)) -> value v_ReceivedAsp{} [] SRB.receive(car_NR_SRB1_RrcNasPdu_IND(p_NR_CellId, p_ExpectedRRCResumeComplete, ?)) -> value v_ReceivedAsp{ v_DedicatedNAS_Message := v_ReceivedAsp.Signalling.Nas[0]; } } return v_DedicatedNAS_Message; }

After Change:

* p_SRGrantEnable = Set to false if periodic Grant was off in f_NR_SendRRCResume_Def. //WA#WI=1271929 */ function f_NR_RRCResumeComplete_Def(NR_CellId_Type p_NR_CellId, template(present) UL_DCCH_Message p_ExpectedRRCResumeComplete := cr_38508_RRCResumeComplete, boolean p_SRGrantEnable := true) //WA#WI=1271929 runs on NR_BASE_PTC return template (omit) NG_NAS_MSG_Indication_Type { var template (omit) NG_NAS_MSG_Indication_Type v_DedicatedNAS_Message := omit; //@sic R5s191051 change 4 sic@ var NR_SRB_COMMON_IND v_ReceivedAsp; //The UE transmits an RRCResumeComplete message. alt { // @sic R5s200403 sic@ [] SRB.receive(car_NR_SRB1_RrcPdu_IND(p_NR_CellId, p_ExpectedRRCResumeComplete)) -> value v_ReceivedAsp{} [] SRB.receive(car_NR_SRB1_RrcNasPdu_IND(p_NR_CellId, p_ExpectedRRCResumeComplete, ?)) -> value v_ReceivedAsp{ v_DedicatedNAS_Message := v_ReceivedAsp.Signalling.Nas[0]; } } if(p_SRGrantEnable) //WA#WI=1271929 { // The SS starts the UL default(SR Based) grant transmission. f_NR_ULGrantConfiguration_Start(p_NR_CellId); } return v_DedicatedNAS_Message; }

MCC160 proposed implementation

function f_NR_RRCResumeComplete_Def(NR_CellId_Type p_NR_CellId,

template(present) UL_DCCH_Message p_ExpectedRRCResumeComplete := cr_38508_RRCResumeComplete)

runs on NR_BASE_PTC return  template (omit) NG_NAS_MSG_Indication_Type

{

var template (omit) NG_NAS_MSG_Indication_Type v_DedicatedNAS_Message := omit; //@sic R5s191051 change 4 sic@

var NR_SRB_COMMON_IND v_ReceivedAsp;

//The UE transmits an RRCResumeComplete message.

alt { // @sic R5s200403 sic@

[] SRB.receive(car_NR_SRB1_RrcPdu_IND(p_NR_CellId, p_ExpectedRRCResumeComplete)) -> value v_ReceivedAsp{}

[] SRB.receive(car_NR_SRB1_RrcNasPdu_IND(p_NR_CellId, p_ExpectedRRCResumeComplete, ?)) -> value v_ReceivedAsp{

v_DedicatedNAS_Message := v_ReceivedAsp.Signalling.Nas[0];

}

}

// Reconfigure SS - set DcchDtch on USS with DCI format 1_1

//@sic R5s240461 sic@

f_NR_SS_ReconfigSStoUSS_DCI1_1(p_NR_CellId);

// Reconfigure RACH procedure as for initial access.  @sic R5s240440 sic@

f_NR_SS_RachProcedureConfig_Def(p_NR_CellId);

return v_DedicatedNAS_Message;

}